Combat Arms

2869 Grove Way

Castro Valley, California 94546-6709

Telephone (415) 538-6544

�����������������������������������������������������ͻ

�                                                     �

�  Disassembly and Re-assembly of the Uzi 9mm/.45ACP  �

�         Carbine Trigger and Firing Mechanism        �

�                                                     �

�                        by                           �

�                                                     �

�             Richard M. Bash & Alvin Chu             �

�                                                     �

�                   March 20, 1989                    �

�                                                     �

�����������������������������������������������������ͼ

1.   Disassembly of the internal trigger mechanism.

a.   Remove the stocks from the pistol grip.

b.   Remove the open ends of the trigger spring from beneath the

trigger safety cam block. There are grooves on the bottom of the

safety cam block that the open ends fit into.

c.   The grip safety must now be fully AFT. Push out the trigger axle

pin from the left to the right (as viewed from the rear). Lift

out the trigger with its attached safety cam block and the

trigger spring. The trigger spring will still be under slight

tension, so be careful.

d.   Drive the sear axle pin out from left to right. Lift out the

sear, sear spring and the trip limit block. The trip limit block

points to the rear of the trigger mechanism and sits between the

sear spring coils.

e.   Remove the spring from the grip safety. This completes the

disassembly.

2.   Reassembly of the internal trigger group.

a.   Make certain the sear spring is correctly positioned in the sear

itself. The flat side of the sear is the bottom of the sear. The

open portion of the sear points rearward. This position exposes a

hole in each of the rearward pointing "arms. Insert the ends of

the sear spring into their respective holes so that the closed

loop portion of the spring is above the two coils in the spring.

b.   Seat the sear by increasing the spring tension. Do that by

pressing the closed loop against the inside front face of the

trigger housing.

c.   Holding the sear and sear spring in this position, begin feeding

the sear axle pin in from the right side. The right end of the

sear axle pin has the screwdriver slot in it.

d.   After positioning the sear axle pin through the rightmost sear

spring coil, set the trip limit block down between the two coils,

pointing rearward. Then push the sear axle pin on through the

trip limit block, second sear spring coil and the rest of the

sear until the pin is set flush with the outside trigger housing

surface.

e.   Rotate the sear axle pin using a screwdriver until the flat face

on the sear axle pin is positioned so that the grip safety bar

will slide under the sear axle pin.

f.   Position the trigger spring so that the open ends are pointing

rearward AND the straight portion connecting the two coils is

resting near the inside bottom of the trigger housing.

g.   Drop the trigger into its slot in the trigger housing and between

the two coils in the spring.

h.   Assure that the grip safety is positioned FULLY REARWARD and then

start pushing in the trigger axle pin with its screwdriver slot

end on the right (as viewed from the rear of the trigger

housing).

i.   Push the trigger axle pin through the first coil of the trigger

spring. Hold the trigger so that the trigger axle pin may be

pushed through the trigger and the second coil. Look at the left

side of the trigger housing. Using a screwdriver of the

appropriate size, insert the screwdriver into the trigger axle

pin's slot (right end) and rotate the pin until the flat side of

the trigger axle pin is parallel to the top flat portion of the

safety bar (NOT the grip safety bar!). Then push the trigger axle

pin on through until it stops.

j.   The open ends of the trigger spring must now be positioned under

the safety cam block and into the spring retaining grooves on the

underside of the safety cam block. Take your time completing this

step. A seam ripper available at any fabrics shop aids this step

or you can grind a notch in a screwdriver blade (which ruins the

screwdriver and why we recommend a seam ripper).

k.   This completes the reassembly of the Uzi internal trigger

mechanism.