8086 REGISTER MODEL

AX:    AH     AL    ACCUMULATOR

BX:    BH     BL    BASE

CX:    CH     CL    COUNT

DX:    DH     DL    DATA

SP        STACK POINTER

BP        BASE POINTER

SI        SOURCE INDEX

DI        DESTINATION INDEX

IP        STACK POINTER

FLAGSH  FLAGSL  STATUS FLAGS

CS        CODE SEGMENT

DS        DATA SEGMENT

SS        STACK SEGMENT

ES        EXTRA SEGMENT

INSTRUCTIONS THAT REFERENCE THE FLAG REGISTER FILE AS A 16-BIT OBJECT USE THE

SYMBOL FLAGS TO REPRESENT THE FILE:

15                8                    0

X X X X OF DF IF TF SF ZF X AF X PF X CF

X = DON't care

Flags

AF: AUXILIARY FLAG - BCD

CF: CARRY FLAG

DF: DIRECTION FLAG (STRINGS)

IF: INTERRUPT ENABLE FLAG

OF: OVERFLOW FLAG (CF  SF)

PF: PARITY FLAG

SF: SIGN FLAG

TF: TRAP (SINGLE STEP FLAG)

ZF: ZERO FLAG

Operand summary

"reg" field Bitt Assignments:

Word Operand     Byte operand        Segment

--------------------------------------------

000 AX           000 AL              00 ES

001 CX           001 CL              01 CS

010 DX           010 DL              10 SS

011 BX           011 BL              11 DS

100 SP           100 AH

101 BP           101 CH

110 SI           110 DH

111 DI           111 BH

Second Instruction Byte Summary

mod xxx r/m

mod   Displacement

------------------

00    DISP = 0*, disp-low and disp-high are absent

01    DISP = disp-low sign-extended to 16 bits, disp-high is absent

10    DISP = disp-high: disp-low

11    r/m is treated as a "reg" field

r/m   Operand Address

---------------------

000   (BX) + (SI) + DISP

001   (BX) + (DI) + DISP

010   (BP) + (SI) + DISP

011   (BP) + (DI) + DISP

100   (SI) + DISP

101   (DI) + DISP

110   (BP) + DISP*

111   (BX) + DISP

DISP follows 2nd byte of instruction (before data if required).

*except if mod = 00 and r/m = 110 then EA = disp-high: disp-low.

Operand Address (EA) Timing (Clocks):

Add 4 clocks for word operands at ODD ADDRESSES.

Immed Offset = 6

Base (BX, BP, SI, DI) = 5

Base + DISP = 9

Base + Index (BP + DI, BX + SI) = 7

Base + Index (BP + SI, BX + DI) = 8

Base + Index (BP + DI, BX + SI) + DISP = 11

Base + Index (BP + SI, BX + DI) + DISP = 12

Segment Override Prefix

001 reg 110

Timing: 2 clocks

Use of segment override

Operand register         Default        With override prefix

------------------------------------------------------------

IP (code address)        CS             Never

SP (stack address)       SS             Never

BP (stack address or     SS             BP + DS or ES or CS

stack marker)

SI or DI (not incl.      DS             ES, SS or CS

strings)

SI (implicit source      DS             ES, SS or CS

for strings)

DI (implicit dest addr   ES             Never

for strings)

8086/8088 Instructions

Notes for 8086/8088 Instructions

The individual instruction descriptions are shown by a format line such as

the following:

!Opcode...m / op / r/m...()...Data...()!

These are byte-wise representations of the object code generated by the

assembler and are interpreted as follows:

o  Opcode is the 8-bit opcode for the instruction. The actual opcode generated

is defined in the "Opcode" column of the instruction table that follows each

format line.

o  m / op / r/m is the byte that specifies the operands of the instruction. It

contains a 2-bit mode field (m), a 3-bit register field (op) and a 3-bit

register or memory (r/m) field.

o  Blank pharanteses following the m / op / r/m field are for any displacement

required by the mode field.

o  Data is for a byte of immediate data.

o  Blank pharanteses following a Data field is used whenever the immediate

operand is a word quantity.

-------------------------------------------------------------------------------

AAA = ASCII Adjust for Addition

!Opcode!

Opcode   Clocks   Operation

37       4        adjust AL, flags, AH

-------------------------------------------------------------------------------

AAD = Ascii Adjust for Division

!Long--Opcode!

Opcode   Clocks   Operation

D5,0A    60       Adjust AL, AH prior to division

-------------------------------------------------------------------------------

AAM = ASCII Adjust for Multiplication

!Long--Opcode!

Opcode   Clocks   Operation

D4,0A    83       Adjust AL, AH after multiplication

-------------------------------------------------------------------------------

AAS = ASCII Adjust for Subtraction

!Opcode!

Opcode   Clocks   Operation

3F       4        adjust AL, flags, AH

-------------------------------------------------------------------------------

ADC = Integer Add with Carry

Memory/Reg + Reg

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

Byte 12       3        Reg8 <-- CF + Reg8 + Reg8

12       9+EA     Reg8 <-- CF + Reg8 + Mem8

10       16+EA    Mem8 <-- CF + Mem8 + Reg8

Word 13       3        Reg16 <-- CF + Reg16 + Reg16

13       9+EA     Reg16 <-- CF + Reg16 + Mem16

11       16+EA    Mem16 <-- CF + Mem16 + Reg16

Immed to AX/AL

!Opcode...Data...()!

Opcode   Clocks   Operation

Byte 14       4        AL <-- CF + AL + Immed8

Word 15       4        AX <-- CF + AX + Immed16

Immed to Memory/Reg

!Opcode...mod 010 r/m...()...()...Data...()!

Opcode   Clocks   Operation

Byte 80       4        Reg8 <-- CF + Reg8 + Immed8

80       17+EA    Mem8 <-- CF + Mem8 + Immed8

Word 81       4        Reg16 <-- CF + Reg16 + Immed16

81       17+EA    Mem16 <-- CF + Mem16 + Immed16

83       4        Reg16 <-- CF + Reg16 + Immed8

83       17+EA    Mem16 <-- CF + Mem16 + Immed8

-------------------------------------------------------------------------------

Add = Integer Addition

Memory/Reg + Reg

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

Byte 02       3        Reg8 <-- Reg8 + Reg8

02       9+EA     Reg8 <-- Reg8 + Mem8

00       16+EA    Mem8 <-- Mem8 + Reg8

Word 03       3        Reg16 <-- Reg16 + Reg16

03       9+EA     Reg16 <-- Reg16 + Mem16

01       16+EA    Mem16 <-- Mem16 + Reg16

Immed to AX/AL

!Opcode...Data...()!

Opcode   Clocks   Operation

04       4        AL <-- AL + Immed8

05       4        AX <-- AX + Immed16

Immed to Memory/Reg

!Opcode...mod 000 r/m...()...()...Data...()

Opcode   Clocks   Operation

Byte 80       4        Reg8 <-- Reg8 + Immed8

80       17+EA    Mem8 <-- Mem8 + Immed8

Word 81       4        Reg16 <-- Reg16 + Immed16

81       17+EA    Mem16 <-- Mem16 + Immed16

83       4        Reg16 <-- Reg16 + Immed8

83       17+EA    Mem16 <-- Mem16 + Immed8

-------------------------------------------------------------------------------

AND = Logical AND

Memory/Reg with Reg

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

Byte 22       3        Reg8 <-- Reg8 AND Reg8

22       9+EA     Reg8 <-- Reg8 AND Mem8

20       16+EA    Mem8 <-- Mem8 AND Reg8

Word 23       3        Reg16 <-- Reg16 AND Reg16

23       9+EA     Reg16 <-- Reg16 AND Mem16

21       16+EA    Mem16 <-- Mem16 AND Reg16

Immed to AX/AL

!Opcode...Data...()!

Opcode   Clocks   Operation

Byte 24       4        AL <-- AK AMD Immed8

Word 25       4        AX <-- AX AND Immed16

Immed to Memory/Reg

!Opcode...mod 100 r/m...()...()...Data...()!

Opcode   Clocks   Operation

Byte 80       4        Reg8 <-- Reg8 AND Immed8

80       17+EA    Mem8 <-- Mem8 AND Immed8

Word 81       4        Reg16 <-- Reg16 AND Immed16

81       17+EA    Mem16 <-- Mem16 AND Immed16

-------------------------------------------------------------------------------

CALL = Call

Within segment or group, IP relative

!Opcode...DispL...DispH!

Opcode   Clocks   Operation

E8       19       IP <-- IP + Disp16, (SP) <-- return link

Within segment or group, Indirect

!Opcode...mod 010 r/m...()...()!

Opcode   Clocks   Operation

FF       16       IP <-- Reg16, (SP) <-- return link

FF       21+EA    IP <-- Mem16, (SP) <-- return link

FF       21+EA    IP <-- Mem16, (SP) <-- return link

Inter-segment or group, Direct

!Opcode...offset...offset...segbase...segbase...segbase!

Opcode   Clocks   Operation

9A       28       CS <-- segbase, IP <-- offset

Inter-segment or group, Indirect

!Opcode...mod 011 r/m...()...()!

Opcode   Clocks   Operation

FF       37+EA    CS <-- segbase, IP <-- offset

-------------------------------------------------------------------------------

CBW = Convert Byte to Word

!Opcode!

Opcode   Clocks   Operation

98       2        convert byte in AL to word in AX

-------------------------------------------------------------------------------

CLC = Clear Carry Flag

!Opcode!

Opcode   Clocks   Operation

F8       2        clear the carry flag

-------------------------------------------------------------------------------

CLD = Clear Direction Flag

!Opcode!

Opcode   Clocks   Operation

FC       2        clear direction flag

-------------------------------------------------------------------------------

CLI = Clear Interrupt Enable Flag

!Opcode!

Opcode   Clocks   Operation

FA       2        clear interrupt flag

-------------------------------------------------------------------------------

CMC = Complement Carry Flag

!Opcode!

Opcode   Clocks   Operation

F5       2        complement carry flag

-------------------------------------------------------------------------------

CMP = Compare Two Operands

Memory/reg with reg

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

Byte 38       3        flags <-- Reg8 - Reg8

38       9+EA     flags <-- Reg8 - Mem8

3A       9+EA     flags <-- Mem8 - Reg8

Word 39       3        flags <-- Reg16 - Reg16

39       9+EA     flags <-- Reg16 - Mem16

3B       9+EA     flags <-- Mem16 - Reg16

Immed to AX/AL

!Opcode...Data...()!

Opcode   Clocks   Operation

Byte 3C       4        flags <-- AL - Immed8

Word 3D       4        flags <-- AX - Immed16

Immed to Memory/Reg

!Opcode...mod 111 r/m...()...()...Data...()!

Opcode   Clocks   Operation

Byte 80       4        flags <-- Reg8 - Immed8

80       10+EA    flags <-- Mem8 - Immed8

Word 81       4        flags <-- Reg16 - Immed16

81       10+EA    flags <-- Mem16 - Immed16

83       4        flags <-- Reg16 - Immed8

83       10+EA    flags <-- Mem16 - Immed8

-------------------------------------------------------------------------------

CWD = Convert Word to Doubleword

!Opcode!

Opcode   Clocks   Operation

99       5        convert word in AX to doubleword in DX:AX

-------------------------------------------------------------------------------

DAA = Decimal Adjust for Addition

!Opcode!

Opcode   Clocks   Operation

27       4        adjust AL, flags, AH

-------------------------------------------------------------------------------

DAS = Decimal Adjust for Subtraction

!Opcode!

Opcode   Clocks   Operation

2F       4        adjust AL, flags, AH

-------------------------------------------------------------------------------

DEC = Decrement by 1

Word Register

!Opcode+reg!

Opcode   Clocks   Operation

48+reg   2        Reg16 <-- Reg16 - 1

Memory/Byte Register

!Opcode...mod 001 r/m...()...()!

Opcode   Clocks   Operation

Byte FE       3        Reg8 <-- Reg8 - 1

FE       15+EA    Mem8 <-- Mem8 - 1

Word FF       15+EA    Mem16 <-- Mem16 -1

-------------------------------------------------------------------------------

DIV = Unsigned Division

Memory/Reg with AX or DX:AX

!Opcode...mod 110 r/m...()...()!

Opcode   Clocks         Operation

Byte F6       80-90          AH,AL <-- AX/Reg8

F6       (80-96)+EA     AH,AL <-- AX/Mem8

Word F7       144-162        DX,AX <-- DX:AX/Reg16

F7       (150-168)+EA   DX,AX <-- DX:AX/Mem16

-------------------------------------------------------------------------------

ESC = Escape

!Opcode+i...mod xxx r/m...()...()!

Opcode   Clocks   Operation

D8+i     8+EA     data bus <-- (EA)

D8+i     2        data bus <-- (EA)

-------------------------------------------------------------------------------

HLT = Halt

!Opcode!

Opcode   Clocks   Operation

F4       2        halt operation

-------------------------------------------------------------------------------

IDIV = Signed Division

Memory/Reg with AX or DX:AX

!Opcode...mod 111 r/m...()...()!

Opcode   Clocks         Operation

Byte F6       101-112        AH,AL <-- AX/Reg8

F6       (107-118)+EA   AH,AL <-- AX/Mem8

Word F7       165-184        DX,AX <-- DX:AX/Reg16

F7       (171-190)+EA   DX,AX <-- DX:AX/Mem16

-------------------------------------------------------------------------------

IMUL = Signed Multiplication

Memory/Reg with AL or AX

!Opcode...mod 101 r/m...()...()!

Opcode   Clocks         Operation

Byte F6       80-98          AX <-- AL*Reg8

F6       (86-104)+EA    AX <-- AL*Mem8

Word F7       128-154        DX:AX <-- AX*Reg16

F7       (134-160)+EA   DX:AX <-- AX*Mem16

-------------------------------------------------------------------------------

IN = Input Byte, word

Fixed port

!Opcode...port!

Opcode   Clocks   Operation

Byte E4       10       AL <-- Port8

E5       10       AX <-- Port8

Vriable port

!Opcode!

Opcode   Clocks   Operation

Word EC       8        AL <-- Port16 (in DX)

ED       8        AX <-- Port16 (in DX)

-------------------------------------------------------------------------------

INC = Increment by 1

Word register

!Opcode+reg!

Opcode   Clocks   Operation

40+reg   2        Reg16 <-- Reg16 + 1

Memory/Byte Register

!Opcode...mod 000 r/m...()...()!

Opcode   Clocks   Operation

Byte FE       3        Reg8 <-- Reg8 + 1

FE       15+EA    Mem8 <-- Mem8 + 1

Word FF       15+EA    Mem16 <-- Mem16 + 1

-------------------------------------------------------------------------------

INT, INTO = Interrupt

!Opcode...(type)!

Opcode   Clocks   Operation

CC       52       Interrupt 3

CD       51       Interrupt 'TYPE'

CE       53 or 4  Interrupt 4 if FLAGS.OF = 1 else NOP

-------------------------------------------------------------------------------

IRET = Return from interrupt

!Opcode!

Opcode   Clocks   Operation

CF       24       Return from interrupt

-------------------------------------------------------------------------------

Jcond = Jump on Condition

Operation

IF condition is true THEN

BEGIN

sign-extend displacement to 16 bits;

IP <-- IP + sign-extended displacement;

END (* IF *)

!Opcode...Disp!

Opcode   Clocks    Operation                           cond

77       16 or 4   jump if above                       JA

73       16 or 4   jump if above or equal              JAE

72       16 or 4   jump if below                       JB

76       16 or 4   jump if below or equal              JBE

72       16 or 4   jump if carry set                   JC

74       16 or 4   jump if equal                       JE

7F       16 or 4   jump if greater                     JG

7D       16 or 4   jump if greater or equal            JGE

7C       16 or 4   jump if less                        JL

7E       16 or 4   jump if less or equal               JLE

76       16 or 4   jump if not above                   JNA

72       16 or 4   jump if neither above nor equal     JNAE

73       16 or 4   jump if not below                   JNB

77       16 or 4   jump if neither below nor equal     JNBE

73       16 or 4   jump if no carry                    JNC

75       16 or 4   jump if not equal                   JNE

7E       16 or 4   jump if not greater                 JNG

7C       16 or 4   jump if neither greater nor equal   JNGE

7D       16 or 4   jump if not less                    JNL

7F       16 or 4   jump if neither less nor equal      JNLE

71       16 or 4   jump if no overflow                 JNO

7B       16 or 4   jump if no parity                   JNP

79       16 or 4   jump if positive                    JNS

75       16 or 4   jump if not zero                    JNZ

70       16 or 4   jump if overflow                    JO

7A       16 or 4   jump if parity                      JP

7A       16 or 4   jump if parity even                 JPE

7B       16 or 4   jump if parity odd                  JPO

78       16 or 4   jump if sign                        JS

74       18 or 6   jump if zero                        JZ

E3       18 or 6   jump if CX is zero (does not test   JCXZ

flags)

-------------------------------------------------------------------------------

JMP = Jump

Within segment or group, IP relative

!Opcode...DispL...(DispH)!

Opcode   Clocks   Operation

E9       15       IP <-- IP + Disp16

EB       15       IP <-- IP + Disp8

(Disp8 sign-extended)

Within segment or group, Indirect

!Opcode...mod 100 r/m...()...()!

Opcode   Clocks   Operation

FF       11       IP <-- Reg16

FF       18+EA    IP <-- Mem16

FF       18+EA    IP <-- Mem16

Inter-segment or group, Direct

!Opcode...offset...offset...segbase...segbase!

Opcode   Clocks   Operation

EA       15       CS <-- segbase, IP <-- offset

Inter-segment or group, Indirect

!Opcode...mod 101 r/m...()...()!

Opcode   Clocks   Operation

FF       24+EA    CS <-- segbase, IP <-- offset

-------------------------------------------------------------------------------

LAHF = Load AH from Flags

!Opcode!

Opcode   Clocks   Operation

9F       4        copy low byte of flags word to AH

-------------------------------------------------------------------------------

LDS/LES = Load Pointer to DS/ES and Register

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

C4       16+EA    dword pointer at EA goes to reg16 (1st word) and ES

(2nd word)

C5       16+EA    dword pointer at EA goes to reg16 (1st word) and DS

(2nd word)

-------------------------------------------------------------------------------

LEA = Load Effective Address

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

8D       2+EA     Reag16 <-- EA

-------------------------------------------------------------------------------

LOCK = Assert Bus Lock

!Opcode!

Opcode   Clocks   Operation

F0       2        assert the bus lock next instruction

-------------------------------------------------------------------------------

LOOPxx = Loop Control

!Opcode...Disp!

Opcode   Clocks    Operation                               xx

E1       18 or 6   dec CX; loop if equal and CX not 0      LOOPE

E0       19 or 5   dec CX; loop if not equal and CX not 0  LOOPNE

E1       18 or 6   dec CX; loop if zero and CX not 0       LOOPZ

E0       19 or 5   dec CX; loop if not zero and CX not o   LOOPNZ

E2       17 or 5   dec CX; loop if CX not 0                LOOP

-------------------------------------------------------------------------------

MOV = Move Data

Memory/Reg to or from Reg

!Opcode...mod reg r/m...()...()!

Opcode   Clocks   Operation

Byte 88       9+EA     Mem8 <-- Reg8

88       2        Reg8 <-- Reg8

8A       8+EA     Reg8 <-- Mem8

Word 89       9+EA     Mem16 <-- reg16

89       2        Reg16 <-- Reg16

8b       8+EA     Reg16 <-- Mem16

Direct-Addressed Memory to or from AX/AL

!Opcode...AddrL...AddrH!

Opcode   Clocks   Operation

Byte A0       10       AL <-- Mem8

A2       10       Mem8 <-- AL

Word A1       10       AX <-- Mem16

A3       10       Mem16 <-- AX

Immed to Reg

!Opcode...Data...()!

Opcode   Clocks   Operation

Byte B0+reg   4        Reg8 <-- Immed8

Word B8+reg   4        Reg16 <-- Immed16

Immed to Memory/Reg

!Opcode...mod 000 r/m...()...()..Data...()!

Opcode   Clocks   Operation

C6       4        Reg8 <-- Immed8

C6       10+EA    Mem8 <-- Immed8

C7       4        Reg16 <-- Immed16

C7       10+EA    Mem16 <-- Immed16

Memory/Reg to or from SReg

!Opcode...mod sreg r/m...()...()!

Opcode   Clocks   Operation

Word 8C       9+EA     Mem16 <-- SReg

8C       2        Reg16 <-- SReg

8E       8+EA     SReg <-- Mem16

8E       2        SReg <-- Reg16

-------------------------------------------------------------------------------

MUL = Unsigned Multiplication

Memory/Reg with AL or AX

!Opcode...mod 100 r/m...()...()!

Opcode   Clocks        Operation

Byte F6       70-77         AX <-- AL*Reg8

F6       (76-83)+EA    AX <-- AL*Mem8

Word F7       118-133       DX:AX <-- AX*Reg16

F7       (124-139)+EA  DX:AX <-- AX*Mem16

-------------------------------------------------------------------------------

NEG = Negate an Integer

Memory/Reg

!Opcode...mod 011 r/m...()...()!

Opcode   Clocks   Operation

F6       3        Reg8 <-- 00H - Reg8

F7       3        Reg16 <-- 0000H - Reg16

F6       16+EA    Mem8 <-- 00H - Mem8

F7       16+EA    Mem16 <-- 0000H - Mem16

-------------------------------------------------------------------------------

NOP = No Operation

!Opcode!

Opcode   Clocks   Operation

90       3        no operation

-------------------------------------------------------------------------------

NOT = Form One'S COMPLEMENT

MEMORY/REG

!OPCODE...MOD 010 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE F6       3        REG8 <-- 0FFH - REG8

F6       16+EA    MEM8 <-- 0FFH - MEM8

WORD F7       3        REG16 <-- 0FFFFH - REG16

F7       16+EA    MEM16 <-- 0FFFFH - MEM16

-------------------------------------------------------------------------------

OR = LOGICAL INCLUSIVE OP

MEMORY/REG WITH REG

!OPCODE...MOD REG R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE 0A       3        REG8 <-- REG8 OR REG8

0A       9+EA     REG8 <-- REG8 OR MEM8

08       16+EA    MEM8 <-- MEM8 OR REG8

WORD 0B       3        REG16 <-- REG16 OR REG16

0B       9+EA     REG16 <-- REG16 OR MEM16

09       16+EA    MEM16 <-- MEM16 OR REG16

IMMED TO AX/AL

!OPCODE...DATA...()!

OPCODE   CLOCKS   OPERATION

0C       4        AL <-- AL OR IMMED8

0D       4        AX <-- AX OR IMMED16

IMMED TO MEMORY/REG

!OPCODE...MOD 001 R/M...()...()...DATA...()!

OPCODE   CLOCKS   OPERATION

BYTE 80       4        REG8 <-- REG8 OR IMMED8

80       17+EA    MEM8 <-- MEM8 OR IMMED8

WORD 81       4        REG16 <-- REG16 OR IMMED16

81       17+EA    MEM16 <-- MEM16 OR IMMED16

-------------------------------------------------------------------------------

OUT = OUTPUT BYTE, WORD

FIXED PORT

!OPCODE...PORT!

OPCODE   CLOCKS   OPERATION

BYTE E6       10       PORT8 <-- AL

E7       10       PORT8 <-- AX

VARIABLE PORT

!OPCODE!

OPCODE   CLOCKS   OPERATION

WORD EE       8        PORT16 (IN DX) <-- AL

EF       8        PORT16 (IN DX) <-- AX

-------------------------------------------------------------------------------

POP = POP A WORD FROM THE STACK

WORD MEMORY

!OPCODE...MOD 000 R/M...()...()!

OPCODE   CLOCKS   OPERATION

8F       17+EA    MEM16 <-- (SP)++

WORD REGISTER

!OPCODE+REG!

OPCODE   CLOCKS   OPERATION

58+REG   8        REG16 <-- (SP)++

SEGMENT REGISTER

!OPCODE+SREG!

OPCODE   CLOCKS   OPERATION

07+SREG  8        SREG <-- (SP)++

-------------------------------------------------------------------------------

POPF = POP THE TOS INTO THE FLAGS

!OPCODE!

OPCODE   CLOCKS   OPERATION

9D       8        FLAGS <-- (SP)++

-------------------------------------------------------------------------------

PUSH = PUSH A WORD ONTO THE STACK

MEMORY/REG

!OPCODE...MOD 110 R/M...()...()!

OPCODE   CLOCKS  OPERATION

FF       16+EA   --(SP) <-- MEM16

WORD REGISTER

!OPCODE+REG!

OPCODE   CLOCKS   OPERATION

50+REG   11       --(SP) <-- REG16

SEGMENT REGISTER

!OPCODE+SREG!

OPCODE   CLOCKS   OPERATION

06+SREG  10       --(SP) <-- SREG

-------------------------------------------------------------------------------

PUSHF = PUSH THE FLAGS TO THE STACK

!OPCODE!

OPCODE   CLOCKS   OPERATION

9C       10       --(SP) <-- FLAGS

-------------------------------------------------------------------------------

RCL = ROTARE LEFT THROUGH CARRY

MEMORY OR REG BY 1

!OPCODE...MOD 010 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        ROTARE REG8 BY 1

D0       15+EA    ROTARE MEM8 BY 1

WORD D1       2        ROTARE REG16 BY 1

D1       15+EA    ROTARE MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 010 R/M...()...()!

OPCODE   CLOCKS       OPERATION

BYTE D2       8+4/BIT      ROTARE REG8 BY CL

D2       20+EA+4/BIT  ROTARE REG8 BY CL

WORD D3       8+4/BIT      ROTARE REG16 BY CL

D3       20+EA+4/BIT  ROTARE MEM16 BY CL

-------------------------------------------------------------------------------

RCR = ROTARE RIGHT THROUGH CARRY

MEMORY OR REG BY 1

!OPCODE...MOD 011 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        ROTARE REG8 BY 1

D0       15+EA    ROTARE MEM8 BY 1

WORD D1       2        ROTARE REG16 BY 1

D1       15+EA    ROTARE MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 011 R/M...()...()!

OPCODE   CLOCKS       OPERATION

BYTE D2       8+4/BIT      ROTARE REG8 BY CL

D2       20+EA+4/BIT  ROTARE MEM8 BY CL

WORD D3       8+4/BIT      ROTARE REG16 BY CL

D3       20+EA+4/BIT  ROTARE MEM16 BY CL

-------------------------------------------------------------------------------

REPX = REPEAT PREFIX

!OPCODE!

OPCODE   CLOCKS   OPERATION                                   REPX

F3       2        REPEAT NEXT INSTRUCTION UNTIL CX=0          REP

F3       2        REPEAT NEXT INSTRUCTION UNTIL CX=0 OF ZF=1  REPE

REPZ

F2       2        REPEAT NEXT INSTRUCTION UNTIL CX=0 OF ZF=0  REPNE

REPNZ

-------------------------------------------------------------------------------

RET = RETURN FROM SUBROUTINE

!OPCODE!

OPCODE   CLOCKS   OPERATION

C3       8        INTRA-SEGMENT RETURN

CB       18       INTER-SEGMENT RETURN

RETURN AND ADD CONSTANT TO SP

!OPCODE...DATAL...DATAH!

OPCODE   CLOCKS   OPERATION

C2       12       INTRA-SEGMENT RETURN AND ADD

CA       17       INTER-SEGMENT RETURN AND ADD

-------------------------------------------------------------------------------

ROL = ROTARE LEFT

MEMORY OR REG BY 1

!OPCODE...MOD 010 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        ROTARE REG8 BY 1

D0       15+EA    ROTARE MEM8 BY 1

WORD D1       2        ROTARE REG16 BY 1

D1       15+EA    ROTARE MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 010 R/M...()...()!

OPCODE   CLOCKS          OPERATION

BYTE D2       8+4/BIT         ROTARE REG8 BY CL

D2       20+EA+4/BIT     ROTARE MEM8 BY CL

WORD D3       8+4/BIT         ROTARE REG16 BY CL

D3       20+EA+4/BIT     ROTARE MEM16 BY CL

-------------------------------------------------------------------------------

ROR = ROTARE RIGHT

MEMORY OR REG BY 1

!OPCODE...MOD 011 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        ROTARE REG8 BY 1

D0       15+EA    ROTARE MEM8 BY 1

WORD D1       2        ROTARE REG16 BY 1

D1       15+EA    ROTARE MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 011 R/M...()...()!

OPCODE   CLOCKS        OPERATION

BYTE D2       8+4/BIT       ROTARE REG8 BY CL

D2       20+EA+4/BIT   ROTARE MEM8 BY CL

WORD D3       8+4/BIT       ROTARE REG16 BY CL

D3       20+EA+4/BIT   ROTARE MEM16 BY CL

-------------------------------------------------------------------------------

SAHF = STORE AH IN FLAGS

!OPCODE!

OPCODE   CLOCKS   OPERATION

9E       4        COPY AH TO LOW BYTE OF FLAGS WORD

-------------------------------------------------------------------------------

SAL/SHL = ARITHMETIC/LOGICAL LEFT SHIFT

MEMORY OR REG BY 1

!OPCODE...MOD 100 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        SHIFT REG8 BY 1

D0       15+EA    SHIFT MEM8 BY 1

WORD D1       2        SHIFT REG16 BY 1

D1       15+EA    SHIFT MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 100 R/M...()...()!

OPCODE   CLOCKS         OPERATION

BYTE D2       8+4/BIT        SHIFT REG8 BY CL

D2       20+EA+4/BIT    SHIFT MEM8 BY CL

WORD D3       8+4/BIT        SHIFT REG16 BY CL

D3       20+EA+4/BIT    SHIFT MEM16 BY CL

-------------------------------------------------------------------------------

SAR = ARITHMETIC RIGHT SHIFT

MEMORY OR REG BY 1

!OPCODE...MOD 111 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        SHIFT REG8 BY 1

D0       15+EA    SHIFT MEM8 BY 1

WORD D1       2        SHIFT REG16 BY 1

D1       15+EA    SHIFT MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 111 R/M...()...()!

OPCODE   CLOCKS        OPERATION

BYTE D2       8+4/BIT       SHIFT REG8 BY CL

D2       20+EA+4/BIT   SHIFT MEM8 BY CL

WORD D3       8+4/BIT       SHIFT REG16 BY CL

D3       20+EA+4/BIT   SHIFT MEM16 BY CL

-------------------------------------------------------------------------------

SBB = INTEGER SUBTRACTION WITH BORROW

MEMORY/REG WITH REG

!OPCODE...MOD REG R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE 1A       3        REG8 <-- REG8 - REG8 - CF

1A       9+EA     REG8 <-- REG8 - MEM8 - CF

18       16+EA    MEM8 <-- MEM8 - REG8 - CF

WORD 1B       3        REG16 <-- REG16 - REG16 - CF

1B       9+EA     REG16 <-- REG16 - MEM16 - CF

19       16+EA    MEM16 <-- MEM16 - REG16 - CF

IMMED FROM AX/AL

!OPCODE...DATA...()!

OPCODE   CLOCKS   OPERATION

1C       4        AL <-- AL - IMMED8 - CF

1D       4        AX <-- AX - IMMED16 - CF

IMMED FROM MEMORY/REG

!OPCODE...MOD 011 R/M...()...()...DATA...()!

OPCODE   CLOCKS   OPERATION

80       4        REG8 <-- REG8 - IMMED8 - CF

80       17+EA    MEM8 <-- MEM8 - IMMED8 - CF

81       4        REG16 <-- REG16 - IMMED16 - CF

81       17+EA    MEM16 <-- MEM16 - IMMED16 - CF

83       4        REG16 <-- REG16 - IMMED8 - CF

83       17+EA    MEM16 <-- MEM16 - IMMED8 - CF

(IMMED8 IS SIGN-EXTENDED BEFORE SUBTRACT)

-------------------------------------------------------------------------------

SHR = LOGICAL RIGHT SHIFT

MEMORY OR REG BY 1

!OPCODE...MOD 101 R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE D0       2        SHIFT REG8 BY 1

D0       15+EA    SHIFT MEM8 BY 1

WORD D1       2        SHIFT REG16 BY 1

D1       15+EA    SHIFT MEM16 BY 1

MEMORY OR REG BY COUNT IN CL

!OPCODE...MOD 101 R/M...()...()!

OPCODE   CLOCKS        OPERATION

BYTE D2       8+4/BIT       SHIFT REG8 BY CL

D2       20+EA+4/BIT   SHIFT MEM8 BY CL

WORD D3       8+4/BIT       SHIFT REG16 BY CL

D3       20+EA+4/BIT   SHIFT MEM16 BY CL

-------------------------------------------------------------------------------

STC = SET CARRY FLAG

!OPCODE!

OPCODE   CLOCKS   OPERATION

F8       2        SET THE CARRY FLAG

-------------------------------------------------------------------------------

STD = SET DIRECTION FLAGS

!OPCODE!

OPCODE   CLOCKS   OPERATION

FD       2        SET DIRECTION FLAG

-------------------------------------------------------------------------------

STI = SET INTERRUPT ENABLE FLAG

!OPCODE!

OPCODE   CLOCKS   OPERATION

FB       2        SET INTERRUPT FLAG

-------------------------------------------------------------------------------

STRING = STRING OPERATIONS

!OPCODE!

OPCODE   CLOCKS   OPERATION                             STRING

A6       22       FLAGS <-- (SI) - (DI)                 CMPS

A7       22       FLAGS <-- (SI) - (DI)                 CMPS

A4       18       (DI) <-- (SI)                         MOVS

A5       18       (DI) <-- (SI)                         MOVS

AE       15       FLAGS <-- (DI) - AL                   SCAS

AF       15       FLAGS <-- (DI) - AX                   SCAS

AC       12       AL <-- (SI)                           LODS

AD       12       AX <-- (SI)                           LODS

AA       11       (DI) <-- AL                           STOS

AB       11       (DI) <-- AX                           STOS

-------------------------------------------------------------------------------

SUB = INTEGER SUBTRACTION

MEMORY/RED WITH REG

!OPCODE...MOD REG R/M... ...!

OPCODE   CLOCKS   OPERATION

BYTE 2A       3        REG8 <-- REG8 - REG8

2A       9+EA     REG8 <-- REG8 - MEM8

28       16+EA    MEM8 <-- MEM8 - REG8

WORD 2B       3        REG16 <-- REG16 - REG16

2B       9+EA     REG16 <-- REG16 - MEM16

29       16+EA    MEM16 <-- MEM16 - REG16

IMMED TO AX/AL

!OPCODE...DATA...()!

OPCODE   CLOCKS   OPERATION

BYTE 2C       4        AL <-- AL - IMMED8

WORD 2D       4        AX <-- AX - IMMED16

IMMED TO MEMORY/REG

!OPCODE...MOD 101 R/M...()...()...DATA...()!

OPCODE   CLOCKS   OPERATION

BYTE 80       4        REG8 <-- REG8 - IMMED8

80       17+EA    MEM8 <-- MEM8 - IMMED8

WORD 81       4        REG16 <-- REG16 - IMMED16

81       17+EA    MEM16 <-- MEM16 - IMMED16

83       4        REG16 <-- REG16 - IMMED8

83       17+EA    MEM16 <-- MEM16 - IMMED8

-------------------------------------------------------------------------------

TEST = LOGICAL COMPARE

MEMORY/REG WITH REG

!OPCODE...MOD REG R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE 84       3        FLAGS <-- REG8 AND REG8

84       9+EA     FLAGS <-- REG8 AND MEM8

WORD 85       3        FLAGS <-- REG16 AND REG16

85       9+EA     FLAGS <-- REG16 AND MEM16

IMMED TO AX/AL

!OPCODE...DATA...()!

OPCODE   CLOCKS   OPERATION

BYTE A8       4        FLAGS <-- AL AND IMMED8

WORD A9       4        FLAGS <-- AX AND IMMED16

IMMED TO MEMORY/REG

!OPCODE...MOD 000 R/M...()...()...DATA...()!

OPCODE   CLOCKS   OPERATION

BYTE F6       5        FLAGS <-- REG8 AND IMMED8

F6       11+EA    FLAGS <-- MEM8 AND IMMED8

WORD F7       5        FLAGS <-- REG16 AND IMMED16

F7       11+EA    FLAGS <-- MEM16 AND IMMED16

-------------------------------------------------------------------------------

WAIT = WAIT WHILE TEST PIN NOT ASSERTED

!OPCODE!

OPCODE   CLOCKS   OPERATION

9B       3+5N     NONE

-------------------------------------------------------------------------------

XCHG = EXCHANGE MEMORY/REGISTER WITH REGISTER

MEMORY/REG WITH REG

!OPCODE...MOD REG R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE 86       4        REG8 <---> REG8

86       17+EA    MEM8 <---> MEM8

WORD 87       4        REG16 <---> REG16

87       17+EA    MEM16 <---> MEM16

WORD REGISTER WITH AX

!OPCODE+REG!

OPCODE   CLOCKS   OPERATION

90+REG   3        AX <---> REG16

-------------------------------------------------------------------------------

XLAT, XLATB = TABLE LOOK-UP TRANSLATION

!OPCODE!

OPCODE   CLOCKS   OPERATION

D7       11       REPLACE AL WITH TABLE ENTRY

-------------------------------------------------------------------------------

XOR = LOGICAL EXCLUSIVE OR

MEMORY/REG WITH REG

!OPCODE...MOD REG R/M...()...()!

OPCODE   CLOCKS   OPERATION

BYTE 32       3        REG8 <-- REG8 XOR REG8

32       9+EA     REG8 <-- REG8 XOR MEM8

30       16+EA    MEM8 <-- MEM8 XOR REG8

WORD 33       3        REG16 <-- REG16 XOR REG16

33       9+EA     REG16 <-- REG16 XOR MEM16

31       16+EA    MEM16 <-- MEM16 XOR REG16

IMMED TO AX/AL

!OPCODE...DATA...()!

OPCODE   CLOCKS   OPERATION

34       4        AL <-- AL XOR IMMED8

35       4        AX <-- AX XOR IMMED16

IMMED TO MEMORY/REG

!OPCODE...MOD 110 R/M...()...()...DATA...()!

OPCODE   CLOCKS   OPERATION

BYTE 80       4        REG8 <-- REG8 XOR IMMED8

80       17+EA    MEM8 <-- MEM8 XOR IMMED8

WORD 81       4        REG16 <-- REG16 XOR IMMED16

81       17+EA    MEM16 <-- MEM16 XOR IMMED16