--------------------------------------------------------------------------------

ROM-BIOS Service Summary

--------------------------------------------------------------------------------

Int

Service           (hex) Input                    Output

--------------------------------------------------------------------------------

Video Services

Set video mode     10   AH=00                    none

AL=video mode

00 : 40x25   text,     16   grey

01 : 40x25   text,     16/8 color

02 : 80x25   text,     16   grey

03 : 80x25   text,     16/8 color

04 : 320x200 graphics, 4    color

05 : 320x200 graphics, 4    grey

06 : 640x200 graphics,      b/w

07 : 80x25   text,          b/w

08 : 160x200 graphics, 16   color

09 : 320x200 graphics, 16   color

0A : 640x200 graphics, 4    color

0D : 320x200 graphics, 16   color

0E : 640x200 graphics, 16   color

0F : 640x350 graphics, 4    color

Set cursor size    10   AH=01                    none

CH=starting scan line

CL=ending   scan line

CGA line 0-7, MA line 0-13

CH=$20 (bit 5) => no cursor

Set cursor         10   AH=02                    none

position                BH=display page number

DH=row

DL=column

Read cursor        10   AH=03                    CH=starting scan line

position                BH=display page number   CL=ending scan line

DH=row

DL=column

Read light-pen     10   AH=04                    AH=pen trigger signal

position                                         BX=pixel column

CH=pixel row

DH=character row

DL=character column

Set active display 10   AH=05

page                    AL=page number

Get display page   10   AH=05                    BH=CRT page register

register                AL=80                    BL=CPU page register

Set CPU display    10   AH=05                    BH=CRT page register

page register           AL=81                    BL=CPU page register

BH=CRT page register

Set CRT display    10   AH=05                    BH=CRT page register

page register           AL=82                    BL=CPU page register

BH=CRT page register

Set both display   10   AH=05                    BH=CRT page register

page registers          AL=83                    BL=CPU page register

BH=CRT page register

BL=CPU page register

Scroll window up   10   AH=06                    none

AL=lines to scroll up

BH=filler attribute

CH=upper row

CL=left column

DH=lower row

DL=right column

Scroll window down 10   AH=07                    none

AL=lines to scroll down

BH=filler attribute

CH=upper row

CL=left column

DH=lower row

DL=right column

Read character and 10   AH=08                    AH=character

attribute               BH=display page number   AL=attribute

Write character    10   AH=09                    none

and attribute           AL=character

BH=page number

BL=attribute

CX=number of characters

to repeat

Write character    10   AH=0A                    none

AL=character

BH=page number

BL=color in graphics mode

CX=count of characters

Set color palette  10   AH=0B                    none

BH=palette color ID

BL=color to be used

with palette ID

Write pixel dot    10   AH=0C                    none

AL=color

CX=pixel column

DL=pixel row

Read pixel dot     10   AH=0D                    AL=color read

CX=pixel column

DL=pixel row

Write character as 10   AH=0E                    none

TTY                     AL=character

BL=color for graphics mode

Get current video  10   AH=0F                    AH=width in characters

mode                                             AL=video mode

BH=page number

--------------------------------------------------------------------------------

Equipment-list service

Get list of        11   none                     AX=equipment list,

peripheral attached                                 bit-coded

equipment                                        Bit settings:

00    : disk drive

01    : math coprocessor

02,03 : system board RAM

in 16K blocks

04,05 : initial video mode:

00 : unused

01 : 40x25 color

10 : 80x25 color

11 : 80x25 b/w

06,07 : number of disk drives

08    : DMA present?

0  : yes

1  : no

09-0B : number of RS-232 cards

in system

0C    : game I/O attached

(not used on AT)

0D    : serial printer

attached

0E,0F : number of printers

attached

--------------------------------------------------------------------------------

Memory Service

Get usable memory  12   none                     AX=memory size

size in K-bytes

--------------------------------------------------------------------------------

Diskette Services

Reset diskette     13   AH=00                    none

system

Get diskette       13   AH=01                    AL=status code

status                                            1 : bad command

2 : address mark not found

3 : write attempted on write-

protected disk

4 : sector not found

6 : diskette removed

8 : MA overrun

9 : MA across 64K boundary

10 : bad CRC

20 : NEC controller failed

40 : seek failed

80 : time out

Read diskette      13   AH=02                    CF=success/failure signal

sectors                 AL=number of sectors     AH=status code see diskette

CH=track number             service 01

CL=sector number         AL=number of sectors read

DH=head number

DL=drive number

ES:BX=pointer to buffer

Write diskette     13   AH=03                    CF=success/failure signal

sectors                 AL=number of sectors     AH=status code see diskette

CH=track number             service 01

CL=sector number         AL=number of sectors written

DH=head number

DL=drive number

ES:BX=pointer to buffer

Verify diskette    13   AH=04                    CF=success/failure signal

sectors                 AL=number of sectors     AH=status code see diskette

CH=track number             service 01

CL=sector number         AL=number of sectors verified

DH=head number

DL=drive number

ES:BX=pointer to buffer

Format diskette    13   AH=05                    CF=success/failure signal

track                   AL=number of sectors     AH=status code see diskette

CH=track number             service 01

CL=sector number         AL=number of sectors written

DH=head number

DL=drive number

ES:BX=pointer to list of

4-byte address fields:

Byte 1=track

Byte 2=head

Byte 3=sector

Byte 4=bytes/sector

--------------------------------------------------------------------------------

Serial Port Services

Initialize serial  14   AH=00                    AX=serial port status

port parameters         DX=serial port number    Status bit settings:

00,01 : word length

00 : Not used

01 : Not used

10 : 7 bits

11 : 8 bits

02    : stop bits

0 : 1

1 : 2

03,04 : parity

00 : none

01 : odd

10 : none

11 : even

05-07 : baud rate

000 : 110

001 : 150

010 : 360

011 : 600

100 : 1200

101 : 2400

110 : 4800

111 : 9600

(4800 on PCjr)

Send out one       14   AH=01                    AH=success/failure

character               AL=character                status code

DX=serial port number    Bit settings:

00 : data ready

01 : overrun error

02 : parity error

03 : framing error

04 : break detected

05 : transmission buffer

register empty

06 : transmission shift

register empty

07 : time out

AL=modem status

Bit settings:

00 : delta clear-to-send

01 : delta data-set-ready

02 : trailing edge ring

detected

03 : change, receive line

signal detected

04 : clear-to-send

05 : data-set-ready

06 : ring detected

07 : receive line signal

detected

Receive one        14   AH=02                    AH=success/failure status

character               DX=serial port number       code see service 01

AL=character

Get serial port    14   AH=03                    AX=status code

status                                           Bit settings see service 00

--------------------------------------------------------------------------------

Cassette Tape Services

Turn on cassette   15   AH=00                    none

motor

Turn off cassette  15   AH=01                    none

motor

Read data blocks   15   AH=02                    CF=error signal

CX=count of bytes        DX=count of bytes read

ES:BX=pointer to data    ES:BX=pointer past last byte

area                     read

Write data blocks  15   AH=03                    CF=error signal

CX=count of bytes        DX=count of bytes writyen

ES:BX=pointer to data    ES:BX=pointer past last byte

area                     written

--------------------------------------------------------------------------------

Keyboard Services

Read next keyboard 16   AH=00                    AH=scan code (auxiliary)

character                                        AL=character code (main)

Report whether     16   AH=01                    ZF=ready or not signal

character ready                                  AH=scan code (auxiliary)

AL=character code (main)

Get shift status   16   AH=02                    AL=shift status bits

Shift status bits set:

00 : Right Shift depressed

01 : Left Shift depressed

02 : Ctrl depressed

03 : Alt depressed

04 : Scroll Lock active

05 : Num Lock active

06 : Caps Lock active

07 : Insert state active

--------------------------------------------------------------------------------

Printer Services

Print screen       05   AH=05                    n/a

Send screen contents to printer.

Status and result byte at low-memory

address hex 500 (0050:0000)

Send one byte to  17   AH=00                     AH=success/failure

printer                AL=character                 status code

Bit settings:

0 : time out

1 : unused

Initialize printer 17   AH=01                    AH=status code see service 00

Get printer status 17   AH=02                    AH=status code see service 00

--------------------------------------------------------------------------------

Miscellaneous Services

Switch control to  18   none                     n/a ; No return, so no

BASIC                                                  possible output

Reboot computer    19   none                     n/a ; No return, so no

possible output

--------------------------------------------------------------------------------

Time-of-Day Services

Read the current   1A   AH=00                    AL=midnight signal

clock count                                      CX=tick count,high portion

DX=tick count,low portion

Set current clock  1A   AH=01                    none

count                   CX=tick count,high portion

DX=tick count,low portion

Read real time     1A   AH=02                    CH=hours

clock                                            CL=minutes

DH=seconds

DL=1 if daylight saving time

0 if standard time

Set real time      1A   AH=03

clock                   CH=hours

CL=minutes

DH=seconds

DL=1 if daylight saving time

0 if standard time

Read date from     1A   AH=04                    DL=day

real time clock                                  DH=month

CL=year

CH=century (19 or 20)

Set date in real   1A   AH=05

time clock              DL=day

DH=month

CL=year

CH=century (19 or 20)

Set alarm          1A   AH=06

CH=hours

CL=minutes

DH=seconds

Place address for alarm routine in interrupt 4A location

Reset alarm        1A   AH=07