DATAPAC 3101  ITI PARAMETERS DESCRIPTION

===================================================

THE FOLLOWING WILL PROVIDE IN MORE DETAIL A DESCRIPTION OF ITI (84)

PARAMETERS FOR DATAPAC 3101.

DESCRIPTION

-----------

INTERNATIONAL PAD PARAMETERS

1)ABILITY TO ESCAPE FROM DATA TRANSFER STATE*

THE SETTING OF THIS PARAMETER ALLOWS THE USER TO INTERRUPT THE

COMMUNICATION

OF HIS OR HER APPLICAION (DATA TRANSFER MODE) AND INTERACT WITH THE

PAD (COMMAND MODE).  THE CHARACTER TO DO THIS IS CONTROL P. TO RETURN

TO DATA TRANSFER MODE, PRESS THE CARRIAGE RETURN OR ENTER A BLANK COMMNAND LINE

IF THE USER WANTS TO SEND A CONTROL P TO THE HOST, WITH THIS PARAMETER SET

TO ONE,

SIMPLY HIT CONTROL P TWICE AND THE SECOND CONTROL P WILL GO TO THE HOST AND

THE USER WILL REMAIN IN DATA TRANSFER MODE. THIS ALSO APPLIES TO THE USER DATA

FIELD IN THE CALL REQUEST COMMNAND LINE.

PARAMETER NUMBER:  1

POSSIBLE VALUES:   0 = ESCAPE NOT POSSIBLE

1 = ESCAPE IS POSSIBLE

* NOTE:  ESCAPE FROM DATA TRANSFER MODE MAY ALSO BE POSSIBLE USING

THE BREAK SIGNAL IF PARAMETER SEVEN IS SET TO EIGHT.

2)ECHO*

THIS PARAMETER INDICATES TO THE PAD WHETHER OR NOT THE TERMINAL

INPUT DATA MUST BE ECHOED.  THIS MAY BE REQUIRED IF THE USER"S TERMINAL

CANNOT ECHO BACK WHAT IS BEING ENTERED.

PARAMETER NUMBER:  2

POSSIBLE VALUES:   0 = NO ECHO

1 = ECHO

* NOTE:  ECHO WILL ALSO BE AFFECTED BY THE SETTING OF PARAMETER 20.

3)SELECTION OF DATA FORWARDING SIGNAL

THIS PARAMETER INDICATES TO THE PAD THE SET TO TERMINAL GENERATED

CHARACTERS OR CONDITIONS THAT WILL CAUSE DATA TO BE FORWARDED TO THE

DESTINATION.  FOR EXAMPLE (CR) CAN BE USED AS A DATA FORWARDING SIGNAL.

ON RECEIPT OF A (CR) FROM THE LOCAL DTE Y, THE PAD WILL FORWARD ALL

CHARACTERS IN IT'S BUFFER TO THE REMOTE END INCLUDING THE (CR).  IF

P13 IS SET TO 6,7,22 OR 23, A (LF) WILL BE INCLUDED IN THE PACKET AND

WILL DELIMIT IT.  DATA IS ALSO FORWARDED WHEN THE BUFFER IS FULL WHETHER OR

NOT A FORWARDING CHARACTER IS RECEIVED.

PARAMETER NUMBER:  3

POSSIBLE VALUES:   0 NO DATA FORWARDING SIGNAL

2 = FORWARD ON CARRIAGE RETURN

2 = CARRIAGE RETURN

126 = ALL CHARACTERS IN COLUMNS 0 AND 1 OF ASCII

TABLE AND THE CHARACTER DEL OF

INTERNATIONAL ALPHABET #5 (SEE APPENDIX 1)

4)SELECTION OF IDLE TIMER DELAY

THIS PARAMETER IS USED TO DETERMINE THE IDLE TIMER LIMIT VALUE

WHEN DATA FORWARDING IS BASED ON TIMEOUTS. TO OPTIMIZE PACKETIZING OF

DATA, NO DATA FORWARDING SIGNAL NEED BE SPECIFIED.  THE PAD WILL THEN

PACKETIZE DATA BASED ON PACKET SIZE SPECIFIED (256 OR 128 CHARACTERS).

THE IDLE TIMER IS USED TO SEND ANY PACKETS THAT ARE NOT FULLY FILLED.  IF

IDLE TIMER IS ACTIVATED AND THE HOST REQUIRES THE (CR) TO INPUT DATA, IT STILL

MUST BE PROVIDED BEFORE THE DATA SENT IS ACCEPTED BY THE HOST.  THE IDLE

TIMER DOES NOT SEND ANY EMPTY PACKETS.

PARAMETER NUMBER:  4

POSSIBLE VALUES: 0 = NO DATA FORWARDING ON TIMEOUT IS REQUIRED

1-255 = INDICATES THE VALUE OF THE DELAY IN TWENTIES

OF A SECOND.  (IE. A VALUE OF 250

MAKES THE TIMER WAIT 10 SECONDS).

* NOTE:  WHEN EDITING IS ON (P15:1), THE IDLE TIMER IS INACTIVE.

IF THIS IS THE ONLY DATA FORWARDING CONDITION, TURNING THE EDITING

FUNCTION ON COULD CAUSE A USER TERMINAL TO HANG OR DATA NOT TO BE

FORWARDED.

5)AUXILIARY DEVICE CONTROL*

THIS IS USED FOR FLOW CONTROL OF DATA COMING FROM EITHER A PC OR

AUXILIARY DEVICE, EG., A PAPER TAPE MACHINE.  WHEN SET TO 1 IT

INDICATES TO THE PAD THAT THE DATA IS TO BE READ AN AUXILIARY I/O DEVICE.

CONNECTED TO THE TERMINAL.  THIS PARAMETER SET TO 2 INDICATES THAT THE

DATA IS COMING FROM AN INTELLIGENT DEVICE, IE. A PC. AND THAT THE PAD MUST

EXERT FLOW CONTROL DIFFERENTLY.

PARAMETER NUMBER:  5

POSSIBLE VALUES:   0 = NO USE OF X-ON/X-OFF

1 = USE OF X-ON/X-OFF FOR AUXILIARY DEVICES

2 = USE OF X-ON/X-OFF FOR INTELLIGENT TERMINALS

* NOTE:  A VALUE OF 2 IS RECOMMENDED FOR PC'S

6)SUPPRESS NETWORK MESSAGES

THIS PARAMETER INDICATES THE PAD WHETHER OR NOT NETWORK GENERATED

MESSAGES ARE TO BE TRANSMITTED TO THE TERMINAL.

PARAMETER NUMBER:  6

POSSIBLE VALUES    0 = SUPPRESS MESSAGES

1 = TRANSMIT MESSAGES

5 = PAD PROMPT (*) FOLLOWS DATAPAC SERVICE SIGNALS

7) PROCEDURE ON BREAK *

THIS PARAMETER IS USED TO INDICATE HOW THE PAD SHOULD PROCESS A BREAK

SIGNAL THAT IS RECEIVED FROM THE TERMINAL WHILE THE TERMINAL IS IN DATA

TRANSFER STATE.

PARAMETER NUMBER:  7

POSSIBLE VALUES:   0 = NOTHING (REMAIN IN DATA TRANSFER MODE)

1 = INTERRUPT (REMAIN IN DATA TRANSFER MODE)

2 = RESET (REMAIN IN DATA TRANSFER MODE)

4 = SEND AN "INDICATION OF BREAK" MESSAGE TO

THE PACKET MODE DTE (REMAIN IN DATA TRANSFER

MODE)

8 = ESCAPE FROM DATA TRANSFER MODE, (IE, ENTER

COMMAND MODE)

16 = DISCARD OUTPUT TO TERMINAL ACTIVATE

PARAMETER 8 (P8:1) (REMAIN IN DATA

TRANSFER MODE)

21 = A COMBINATION OF 1,4 AND 16

* NOTE:  THE BREAK SIGNAL IS IGNORED IF THE VIRTUAL CIRCUIT IS NOT

ESTABLISHED.WHILE IN COMMAND STATE.  THE BREAK SIGNAL WILL DELETE THE

CURRENT LINE.

1 THE VALID VALUES FOR P 7 ARE 0,1,2,8 AND 21.

8)DISCARD OUTPUT

THIS PARAMETER IS USED IN CONJUNCTION WITH PARAMETER 7.  DEPENDING

UPON THE BREAK PROCEDURE SELECTED, THIS PARAMETER MAY BE SET BY THE PAD

WHEN THE TERMINAL USER REQUESTS THAT TERMINAL DATA BE DISCARDED. THIS

PARAMETER MUST THEN BE RESET BY THE DESTINATION COMPUTER TO ALLOW NORMAL

DELIVERY.  THE PAD WILL DISCARD ALL PACKETS DESTINED FOR THE TERMINAL

FROM THE TIME THE PAD SETS THIS PARAMETER (IE, IT RECEIVES A BREAK SIGNAL

WHEN PARAMETER 7 IS SET TO 21) TO THE TIME THE PARAMETER IS RESET BY THE

DESTINATION.  IT CAN ONLY BE RESET BY THE DESTINATION.

PARAMETER NUMBER:  8

POSSIBLE VALUES:   0 = NORMAL DELIVERY OF OUTPUT TO TERMINAL

1 = DISCARD OUTPUT TO TERMINAL

9)PADDING AFTER CARRIAGE RETURN

THIS PARAMETER IS USED TO SPECIFY THE NUMBER OF PADDING CHARACTERS TO

BE INSERTED BY THE PAD FOLLOWING A CR R TRANSMITTED TO THE TERMINAL.

PADDING ALLOWS TIME FOR THE CARRIAGE TO RETURN ON MECHANICAL PRINTING DEVICES

PARAMETER NUMBER:  9

POSSIBLE VALUES:   0 = 2 PADDING CHARACTERS WILL BE INSERTED AT 110 BPS

AND 4 PADDING CHARACTERS WILL BE INSERTED AT

HIGHER SPEEDS, IN COMMAND MODE ONLY.

(NO PADDING IS DONE IN DATA TRANSFER MODE)

1-255 = THE NUMBER OF PADDING CHARACTERS TO BE

INSERTED IN BOTH DATA TRANSFER AND COMMAND

MODE.

10)LINE FOLDING

THIS PARAMETER INDICATES THE MAXIMUM NUMBER OF PRINTABLE CHARACTERS

THAT CAN BE DISPLAYED ON THE TERMINAL BEFORE THE PAD MUST SEND A

FORMAT EFFECTOR (IE.,<CR><LF>).  THIS PERMITS MORE DATA TO BE

TRANSMITTED IN ONE PACKET WHILE STILL LETTING THE USER PRINT OUT

MORE THAN ONE LINE, IE., PRINTING OUT FORMS.

11)TRANSMISSION SPEED (READ ONLY)

THIS PARAMETER IS SET BY THE PAD AS A RESULT OF TRANSMISSION SPEED

DETECTION IF THE TERMINAL ACCESSES AN AUTOBAUD PORT.  WHEN A PRIVATE

PORT WITH FIXED SPEED IS USED, THIS PARAMETER IS SET BASED ON THE

PRE-STORED INFORMATION SELECTED AT SUBSCRIPTION TIME.

PARAMETER NUMBER:  11

POSSIBLE VALUES:    0 = 110 BPS

2 = 300 BPS

3 = 1200 BPS

4 = 2400 BPS

12)FLOW CONTROL

THIS PARAMETER ALLOWS THE TERMINAL USER TO SIGNAL THE PAD THAT THE

TERMINAL CANNOT RECEIVE ANY MORE DATA, IE. TO FLOW CONTROL DATA COMING

FROM THE NETWORK.  WHEN IT IS SET TO 1, THE TERMINAL USER CAN REQUEST

THAT THE PAD STOP SENDING DATA BY TRANSMITTING A "CONTROL S".  THE

PAD THEN SUSPENDS TRANSMISSION OF DATA UNTIL A "CONTROL Q" IS RECEIVED.

PARAMETER NUMBER:  12

POSSIBLE VALUES:    0 = NO USE OF X-ON/X-OFF FOR FLOW CONTROL

1 = USE X-ON/X-OFF FOR FLOW CONTROL

13)LINEFEED INSERTION (SIMILAR TO NATIONAL PARAMETER 126)

THIS PARAMETER SPECIFIES WHEN THE PAD SHOULD INSERT LINEFEEDS <LF>

UPON RECEIPT OF A CARRIAGE RETURN <CR> FROM THE TERMINAL, PAD OR THE

DESTINATION HOST.

PARAMETER NUMBER:  13

POSSIBLE VALUES:    0 = INSERT NO <LF>

1 = INSERT <LF> TO TERMINAL UPON RECEIPT OF A

<CR> FROM HOST

4 = INSERT <> TO TERMINAL AFTER A <CR> FROM

TERMINAL OR ON PAD GENERATED <CR>

5 = INSERT <LF> TO TERMINAL ON <CR> FROM PAD

TERMINAL OR HOST.

6 = INSERT <LF> TO TERMINAL ON <CR> FROM

TERMINAL AND INSERT <LF> IN DATA HOST

ON <CR> FROM TERMINAL

7 = INSERT <LF> TO TERMINAL ON <CR> FROM

PAD, TERMINAL OR HOST AND INSERT LF

IN DATA TO HOST ON <CR> FROM TERMINAL

16 = INSERT <LF> TO TERMINAL AFTER A PAD

GENERATED <CR>

17 = INSERT <LF> TO TERMINAL AFTER A PAD

GENERATED <CR> OR A HOST GENERATED <CR>

20 = INSERT <LF> TO TERMINAL ON <CR> FROM

TERMINAL

22 = INSERT <LF> TO TERMINAL ON <CR> FROM

<CR> FROM TERMINAL

23 = INSERT <LF> TO TERMINAL ON <CR> FROM

TERMINAL OR HOST.  INSERT <LF> IN DATA

TO HOST ON <CR> FROM TERMINAL

14)PADDING CHARACTERS AFTER LINEFEED

TO ALLOW TIME FOR MECHANICAL PAPERFEED, THIS PARAMETER IS USED TO

SPECIFY THE NUMBER OF PADDING CHARACTERS TO BE INSERTED BY THE PAD

FOLLOWING <LF> TRANSMITTED TO THE TERMINAL IN DATA TRANSFER MODE ONLY.

PARAMETER NUMBER:  14

POSSIBLE VALUES:0-255 = NUMBER OF PADDING CHARACTERS (NUL)

SENT TO THE TERMINAL AFTER TRANSMISSION

OA A <LF> TO THE TERMINAL.

15)CONTROL OF EDITING FUNCTIONS

THIS PARAMETER SPECIFIES WHETHER OR NOT EDITING IS ALLOWED IN DATA

TRANSFER MODE.  IT IS ALWAYS AVAILABLE IN COMMAND MODE. THIS CONTROLS

PARAMETERS 16, 17 AND 18.

PARAMETER NUMBER:  15

POSSIBLE VALUES:    0 = EDITING ONLY IN COMMAND MODE

1 = EDITING IN COMMAND AND DATA TRANSFER MODE

* NOTE:  SETTING PARAMETER 15 TO 1 WILL CAUSE THE OPERATION OF THE

IDLE TIMER (PARAMETER 4) TO BECOME DISABLED.

16)CHARACTER DELETION (SIMILAR TO NATIONAL PARAMETE 118)

THIS PARAMETER SPECIFIES THE ASCII CHARACTER TO BE USED AS A

"CHARACTER DELETE" CHARACTER FOR LOCAL EDETING BY THE PAD.  THE

DEFAULT VALUE IS SET TO 127 (THE DELETE OR DEL KEY.

PARAMETER NUMBER:  16

POSSIBLE VALUES:    0 = TURN OFF CHARACTER EDETING (COMMNAD & DATA

TRANSFER MODE)

1-127 = CHARACTER DELETE CHARACTER (APPENDIX K)

17)LINE DELETION (SIMILAR TO NATIONAL PARAMETER 119)

THIS PARAMETER SPECIFIES THE ASCII CHARACTER TO BE USED AS A

"LINE DELETE" CHARACTER FOR LOCAL EDITING BY THE PAD. THE DEFAULT

VALUE IS SET TO 24 (CONTROL X).  ON SOME OLDER TERMINALS THE CONTROL

X KEY WILL CLEAR THE SCREEN AS WELL AS DELETE THE LINE.

PARAMETER NUMBER:  17

POSSIBLE VALUES     0 = NO KEY WILL DELETE A CHARACTER

1-127 = ASCII CODE FOR THE "LINE DELETE" CHARACTER

18)LINE DISPLAY (SIMULAR TO NATIONAL PARAMETER 120)

THIS PARAMETER SPECIFIES THE ASCII CHARACTER TO BE USED AS A

"LINE DISPLAY " CHARACTER FOR LOCAL EDITING BY THE PAD.  THE DEFAULT

VALUE IS SET TO 18 (CONTROL R).,

PARAMETER NUMBER:  18

POSSIBLE VALUES:    0 = NO KEY WILL DELETE A CHARACTER

1-127 = ASCII CODE FOR THE "LINE DISPLAY" CHARACTER

19)EDITING SERVICE SIGNALS

THIS PARAMETER DEFINES THE FORMAT OF THE SERVICE SIGNAL SENT TO TERMINAL

UPON EXECUTION OF THE EDITING FUNCTIONS (P16 OR P17)

PARAMETER NUMBER:  19

POSSIBLE VALUES:    0 = NO SERVICE SIGNAL ON EDITING

1 = BACKSLASH IS DISPLAYED FOR EACH CHARACTER

DELETED, "***" IS DISPLAYED FOR EACH LINE

DELETED

2 = <BS><SP><BS> IS DISPLAYED FOR EACH

CHARACTER DELETED; <BS><SP><BS> IS

REPEATED FOR EACH CHARACTER ON THE LINE

DELETED

PARAMETER 19         8**, 32-126 = THE SPECIFIED CHARACTER IS DISPLAYED

FOR EACH CHARACTER DELETED; "***" IS

DISPLAYED FOR EACH LINE DELETED.

*NOTE:  VALUE 1 IS RECOMMENDED FOR PRINTING TERMINALS AND VALUE 2

WHEN PAD ECHO IS OFF (P2=0) AND THE CHARACTER DELETE" CHARACTER

IS DEFINED AS <BS> (P16=8), THE PAD DOES NOT SEND AND EDITING

SERVICE SIGNAL TO THE TERMINAL UPON EXECUTION TO THE CHARACTER

DELETE EDITING FUNCTION; HOWEVER, THE CHARACTER WILL STILL BE

DELETED FROM THE PAD BUFFER.

FOR VIDEO DISPLAY TERMINALS.

PRODUCE A <BS>, BUT THE CHARACTER WILL STILL BE DELETED FROM THE PAD.

20)ECHO MASK*

THIS PARAMETER SPECIFIES A SET OF CHARACTERS NOT TO BE ECHOED TO THE

TERMINAL.

PARAMETER NUMBER:  20

POSSIBLE VALUES:    0 = ALL CHARACTERS ECHOED (EXCEPT X-ON/X-OFF

WHICH DEPEND ON THE SETTING OF P5, P12

AND P22)

1 = NO ECHO OF <CR>

2 = NO ECHO OF <LF>

4 = NO ECHOE OF <VT>, <HT>, <FF>

8 = NO ECHO OF <BEL>, <BS>

16 = NO ECHO OF <ESC>, <ENQ>

32 = NO ECHO OF <ACK>, <NAK>, <STX>, <SOH>,

<EDT><ETB>, <ETX>

64 = NO ECHOING OF EDITING CHARACTERS, AS

DESIGNATED BY PARAMETER 16,17 AND 18

128 = NO ECHOING OF ALL OTHER CHARACTERS IN

COLUMN 0 AND 1 OF ASCII TABLE IN

APPENDIX I PLUS <DEL>

* NOTE:  THIS PARAMETER IS ONLY ACTIVE WHEN ECHO IS ON (P2 = 1).

21)PARITY DETECTED OR CHECKED (SIMILAR TO NATIONAL PARAMETER 123)

THIS PARAMETER SPECIFIES WHETHER OR NOT THE PAD SHOULD DETECT,

CHECK AND GENERATE PARITY.  THIS PARAMETER IS SET TO 0 WHEN THE USER

WISHES TO TRANSMIT 8-BIT DATA.

PARAMETER NUMBER:  21

POSSIBLE VALUES:    0 = NO DETECTED OR CHECKED

3 = PARITY CHECKED AND DETECTED

22)PAGE WAIT

THIS PARAMETER CONTROLS THE ABILITY TO SET A TERMINAL PAGE SIZE

WHICH, WHEN REACHED, CAUSES THE PAD TO STOP ALL FURTHER OUTPUT TO THE

TERMINAL.

PARAMETER NUMBER:  22

POSSIBLE VALUES:    0 = NO PAGE WAIT

1-255 = PAGE WAIT SIZE

THE PAD WILL RESUME NORMAL TRANSMISSION WHEN ANY OF THE FOLLOWING OCCURS:

- ON RECEIPT OF THE PAGE WAIT CANCELLATION CHARACTER (X-ON); OR

- ON RESETTING PARAMETER 22 TO ZERO.

NATIONAL PARAMETERS

===================

THE FOLLOWING PARAMETERS ARE SPECIFIC TO THE DATAPAC INTERACTIVE

TERMINAL INTERFACE (DATAPAC 3101 SERVICE)

P118 CHARACTER DELETION

P119 LINE DELETION

P120 LINE DISPLAY

P121,122 ADDITIONAL DATA FORWARDING SIGNALS

THESE PARAMETERS 121 AND 122 ALLOW THE DEFINITION OF ONE OR

TWO ADDITIONAL DATA FORWARDING SIGNALS.  THESE SIGNALS ARE IN ADDITION

TO ANY THAT ARE SPECIFIED BY PARAMETER 3 AND 4.

PARAMETER NUMBER:  121,122

POSSIBLE VALUES:     0 = NONE

1-127= ASCII CODE OF ADDITIONAL DATA FORWARDING

SIGNAL.

P123 PARITY TREATMENT

THIS PARAMETER SPECIFIES WHETHER OR NOT THE PAD SHOULD DETECT AND

CHECK PARITY AND PERFORMS THE SAME FUNCTION AS INTERNATIONAL PARAMETER

21.

PARAMETER NUMBER:  123

POSSIBLE VALUES:     0 = NO DETECTED OR CHECKED

1 = PARITY CHECKED AND DETECTED

6)OUTPUT PENDING TIMER

THIS PARAMETER SPECIFIES THE LENGTH OF TIME, IF ANY, THAT TERMINAL

OUTPUT WILL BE BLOCKED BY TERMINAL INPUT.

PARAMETER NUMBER:  125

POSSIBLE VALUES:     0 = OUTPUT

1-255  = VALUE OF DELAY IN SECONDS

7)LINEFEED INSERTION

PARAMETER NUMBER:  126 SEE INTERNATONAL PARAMETER 13.

GENERAL NOTES ABOUT PARAMETERS

-------------------------------

THE SETTING OF ANY OF PARAMETERS 16(118), 17(119), OR 18(120) TO

A NON-ZERO VALUE WILL ALSO SET PARAMETER 15 TO 1 WHEN PARAMETER 4 IS ZERO.

IN THE 1984 SOFTWARE LEVEL, SETTING BOTH EDITING (P15) IS OFF.  THIS

OPERATION IS ACCORDING TO CCITT 1984 STANDARDS.

IF THE CHARACTER CHOSEN FOR A CERTAIN PARAMETER HAS ALREADY BEEN

SELECTED TO BE USED WITH ANOTHER PARAMETER, THEN THE FUNCTION WITH THE

HIGHEST PRIORITY IS SELECTED UPON RECEIPT OF THE CHARACTER.  THIS PRIORITY

SCHEME IS AS FOLLOWS:

1) ESCAPE FROM COMMAND STATE

2) FLOW CONTROL

3) LINE DISPLAY

4) LINE DELETE

5) CHARACTER DELETE

6) PACKET FORWARDING

ALL OF THE ABOVE PARAMETERS ARE CHANGEABLE BY THE USER.  HOWEVER, IT

SHOULD BE NOTED THAT THE HOST COMPUTER CAN ALSO CHANGE THESE PARAMETERS.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X