=============================================================================

ANTINAGRAVISION V1.0 DOC file (September 1994).

=============================================================================

* INTRODUCTION:

This program tries to decode NAGRAVision images (Channel Plus), by using

an algorithm based on correlation function to sort lines.

* NOTES ABOUT USING:

- Images must be on PGM format (Portable GrayMap) version P5.

- The algorithm with low graphics resolution report more speed and better

results.

- If you have MS-DOS 5 or upper then use SMARTDRV to minimize disk access.

* THEORY:

- Def. Correlation function:

---

�+�

C(T) = �  [ x(t+T)*x(t) ]*dt    (Continuous functions).

�-�

+�

C(T) =  �  [ x(t+T)*x(t)*t ]    (Discreet functions).

t=-�

where T is the displacement. T � [-�, +�].

It measures the likeness between a function and a displaced

version of it.

- Def. Autocorrelation function:

---

A = C(0)

- Property.

--------

A = C(0) � C(T)  when  T � [-�, +�].

* ALGORITHM:

Being:

x, y discreet variables (pixels) when domain x � [1, x_max]

y � [1, y_max].

x_max, y_max are the horizontal and vertical resolutions of

PGM respectively.

(x, y) = (1, 1) is the upper left corner.

L (x) discreet function defines the horizontal line y.

y

x_max

If I fix L  (x) and calculate:   A(y1, y) =   �   [L  (x)*L (x)*x]

y1                                 x=1    y1     y

the y that maximizes this expression will be the one that is most liking

to y1, and must be located beside to y1.

Repeating that for all the lines we get the "decoded" image.

* THEORICAL NOTES:

To minimize the operations number we can define a pre-correlation function:

x_max

PC  (x) =   �   [L  (x)*x]

y1       x=1    y1

Consequently:

x_max

A(y1, y) =   �   [PC  (x)*L (x)]

x=1     y1     y

thus in the searching of each maximum the pre-correlation function is

only calculated once.

* TODAY:

I'm trying to speed up and improve the results of the algorithm for high

resolution images.

* AUTHOR:

Toni Cebrian

Telecommunications student at Valencia (Spain)

E-Mail: acebrian@afrodita.etsit.upv.es

Any reports and suggestions will be welcome.

No responsibility is assumed for the using of this program.

=============================================================================

End of article.

=============================================================================