SIM7000  Version 1.1

VCII Hardware/Software Emulator

*******************************************************************************

*** NOTE -- Version 1.1 is the current version and replaces Version 1.0.

If you have a Version 1.0 package, it should be deleted and

replaced with this newer version.  Version 1.1 corrects a few

small bugs in Version 1.0 and offers some enhancements which

produce a noticeable speed improvement.  A test of this program

against the HINK's TMS-7000 Simulator yielded the following

results.

Test code was the register file check and ROM checksum routines

at the start of a V2.4 stock U30 chip.  Counting all iterations,

it consists of about 98,000 TMS-7000 instructions.

Program          Time         Avg Instructions per Second

=======         ======       =============================

SIM7000.COM       25 sec                  3932

SIM740.EXE        26 min                   63

The HINK's file was named SIM740.EXE

*******************************************************************************

This program is designed to faithfully emulate most of the hardware

associated with the Videocipher II descrambler unit.  It allows the actual

TMS-7000 software found in the U30 16K EPROM and the U7 4K ROM to be executed

in an IBM PC or compatible.  The program is amazingly fast, but of course can

still not approach real-time TMS-7000 processor speeds.  It is however, many

times (over 67 times) faster than any other similar package.  The program

requires an IBM PC with at least 128K of free memory and preferrably a Color

Card and Monitor.  Obviously, the faster your machine the faster the program

executes the TMS-7000 code.  A 16 MHZ or faster 386 machine really screams

with this package.  The program is written entirely in assembly language.

The program was inspired by the Hink's Simulator package I found over 2 years

ago on another BBS.  While the Hink's package was first, and certainly a good

product, I believe my implementation is more versatile and it is much

faster than the Hink's system.

The program will duplicate all required handshaking functions for the

standard U30 codes, including handling the INT 1, INT 3 & INT 4 routines.

The program automatically generates an INT1 followed by an INT 3 about every

3100 TMS-7000 instruction sequences.  This correlates with the number of

instruction cycles usually executed by the 7001 chip in the VC2 between INT 3

signals.  In addition to the INT interface, the program also provides a

display window to simulate the on-screen display of the VC2.  This window is

dynamically updated as the TMS-7000 (U30) code executes.  Function keys 1 thru

10 are used to simulate the VC2 keypad keys, with the numeric keys used for

inputting digits 0-9.

The program is menu driven and uses a highlighting bar for selection

followed by <ENTER> to execute a command.  The highlight bar may be moved using

either the arrow keys or the space bar.  The ESC key can be used to backup one

menu selection each time it is pressed.  While TMS-700 code is executing in

the "hard run" mode, the ESC key can be used to suspend execution for examining

memory or single-stepping through the code.  A list of each main menu command

followed by a brief description is presented below.

STEP -  The STEP command is used to single-step through an object code

file one instruction at a time.  When STEP is selected the

following sub-menu appears --

ONCE  MULTIPLE  ADDRESS

ONCE executes the next instruction, which is shown disassembled

in the left-hand bottom window.  After execution, the flags

are updated accordingly.

MULTIPLE causes the program to prompt the user for the number

of times to step in succession.  Enter a decimal number between

1 and 65535.  The program will execute through the number of

instructions specified.

ADDRESS is used to alter the execution address from that

currently pointed to by the program counter.  Enter a

hexadecimal address followed by <ENTER>.

LIST -  The LIST command is used to disassemble the object code.  Two

destinations are allowed, either the video display or the

printer.  When LIST is selected, the following sub-menu

appears --

ONCE  MULTIPLE  ADDRESS  TO PRINTER

ONCE disassembles the next instruction to the video display

window.

MULTIPLE prompts the user for the number of lines to display.

Enter a decimal number between 1 and 65535.  The default is 9.

The specified number of lines will be displayed.

ADDRESS is the command to set a new address pointer for the

disassembly instructions.  The prompted input should be a

hexadecimal address.

TO PRINTER will bring up another menu which allows printing the

next instruction, printing by address range or printing by

number of lines.  The sub-menu is as follows --

NEXT LINE  BY ADDRESS RANGE  BY NUMBER OF LINES

BY ADDRESS RANGE prompts for starting and ending

addresses for disassembly, then dumps to the printer.

(LPT1 is assumed)

BY NUMBER OF LINES prompts for the number of lines to

disassemble to the printer.  The specified number of

lines are then printed.

EXAM -  This command is used to examine and change the contents of the

memory area of the simulator.  It allows any byte in the 64K

TMS-7000 address area to be examined or changed.  All entries

are in hexadecimal.  After the address is entered, the current

contents of that address is displayed and you are given the

chance to type in new contents.  To leave the contents

unchanged, simply hit ESC to leave or <ENTER> to skip to the

next sequential address.

BREAK - This command is used to establish code execution breakpoints

where execution will suspend.  It brings up a sub-menu which

allows selection of either HARD or SOFT breakpoints.  The

difference is in whether or not the breakpoint is removed

after suspending execution.  SOFT breakpoints are automatically

removed while HARD breakpoints are not.  Once either HARD or

SOFT is selected, a second sub-menu appears allowing the

addition, deletion or viewing of breakpoints.  Entries to the

prompts are hexadecimal addresses.

DISPLAY - This command is used to make additions to or deletions from

the memory address table which is dynamically displayed and

updated during code execution.  Entries in the table are

segregated as Registers and memory addresses (although,

technically both are the same).  Valid selections from the

sub-menu are Add or Delete Registers and Add or Delete Memory.

All entries are in hexadecimal.  Up to 8 registers and 8

memory addresses can be simutaneously displayed and updated.

They are automatically sorted before display in ascending

order, with registers in the upper half of the window and

memory addresses in the lower half.  8 active displays of both

registers and memory was chosen as the best trade-off between

versatility and speed.  Longer display lists would begin to

affect execution speed.

FILE -  This command is used to load object code files into the memory

area of the simulator.  Enter a valid MS-DOS filename/pathname

to load a file.  An object code file must be loaded before

most of the other commands will execute.  Upon end of loading,

the program will automatically update the Reset Vector (Cold

Boot address) from simulator ROM at FFFE & FFFF.

EXIT -  This command is used to exit the simulation and return to DOS.

Future releases will incorporate some added features into the

EXIT command, such as the capability to save the ROM area.

RUN  -  This command is used to cause the simulator to enter a hard

run condition or continuous execution of instructions.  The

sub-menu which appears allows execution at the current Program

Counter address or allows the selection of a different Program

Counter address.  If a new PC is desired, enter the address

in hexadecimal when prompted.  A hard run may be suspended

using the ESC key and then re-entered using the RUN command.

While suspended, any of the other commands can be executed.

CPU  -  This command allows selective loading of the A register or B

register of the simulator's CPU.  It also allows the status

register bits (flags) to be set or reset.  Enter all values

as prompted using hexadecimal.  Entering any non-zero value

for the Flag Bits will set the corresponding flag while a

zero value will reset the flag.

PROC -  This command is only allowed when the current instruction is

either a CALL or TRAP.  Selection of PROC will cause the

program to enter a hard run at the subroutine address and

suspend when control returns to the instruction following the

subroutine call.  Any subroutine nesting is executed as part

of the hard run procedure.

The simulator is quite versatile and faithfully executes each instruction

of the TMS-7000 series microprocessors.  All flag bits are accurately set.

All requisite handshaking for the U30 TRAP 6, TRAP 11, TRAP 7, TRAP 12, TRAP 13

TRAP 15 & TRAP 16 routines is provided.  Of course, the values returned with

TRAP 11 are always FF (255 decimal).  The VC2 keypad is implemented as a hot-

key routine.  Simply press the appropriate function key to generate the Serial

Port interrupt of the VC2.

One of the outstanding features of this simulator package is its ability

to mimic the normal VC2 datastream command sequence.  The simulator contains a

pre-defined circular list of VC2 commands which are loaded using INT 1, just

like the actual VC2 hardware.  In the VC2, digital command data is picked off

the horizontal sync interval by U24 and assembled into 5 to 50 byte command

sequences which are loaded into U20 RAM one byte at the time using INT 1.  The

timing sequence is such that 4 complete command strings are loaded every frame.

After 4 frames are loaded, a special End-of-Frame data set is loaded which

includes the important program clock from which the audio key is generated.

There are 15 of the EOF's (End-of-Frame sets) every 2 seconds.  This works out

to be 7.5 frames per second.  Each good frame received is displayed by the

frame counter of the SETUP 0 screen.  If you watch the SETUP 0 screen of an

operating descrambler very closely, you will note that the 4th frame of every

set is not shown, i.e., the count will skip a number on every 4th count.  This

is very difficult to detect however, because of the speed of the counting.

While the simulator is running, though, it is very obvious when the count skips

a number because it counts much more slowly while displaying the SETUP 0

screen.  If you have a copy of the object code for one of the RAM-VIEW type

chips, load it into the simulator and observe addresses 6000-6032.  You will

be able to see the datastream commands being loaded.  A leading byte of FF

indicates an unread command while a leading byte of 00 indicates a command

that has been processed by the U30 code.  The End-of-Frame data can be seen

at address 60CC-60D4.  Remember that it is loaded every 4th INT 3.  A leading

byte of FF indicates an unprocessed frame package.

The first screen of the simulator package asks the user if he wishes to

implement U30 code or U7 code.  If U7 code is selected, all INT routines are

suppressed, as U7 does not normally implement interrupts.  If U30 code is

selected, all interrupts are supported (1,3, & 4).  Remember that U30 code

loads at C000 in memory and U7 code loads at F000 in memory when prompted for

the object code load address.  Running U7 code after selecting U30 from the

startup screen will result in a crash!  Also, when running U7 code, remember

to set Bit 0 of APORT (address 0104) high by using the EXAM command to change

the contents of address 0104 to 83 or 87 prior to running the code.

This is still considered a beta test version of this software and all

comments and suggestions are welcome.  Just leave a message to Bill Meeks on

the Cathouse BBS.  I have already noticed that the INT 4 routine does not

execute properly using U30 V3.1 software, although V3.0 works fine.  Haven't

found out why yet.  If you know what differences there are between V3.0 and

V3.1 please let me know.  The problem is that V3.1 stock U30 code will not

recognize any key inputs, although the normal on-screen graphics will come up.

Apparently the serial port on the new boxes sends some sort of wake-up code

to the U30 code that initiates keypad input.  If you know how this works, just

leave me a message and I'll incorporate a fix into the simulator if possible.

Happy Hacking.