10 ' 555 OSCILLATOR DESIGN

20 '

30 ' Michael A. Covington

40 ' 5339-14 North Rosemead Boulevard

50 ' San Gabriel, CA 91776

60 '

70 ' 24 July 1983

80 '

110 SCREEN 0,0,0 : WIDTH 80 : CLS : KEY OFF

120 LOCATE 10,29:  PRINT "555 OSCILLATOR DESIGN"

130 LOCATE 13,29:  PRINT "         by          "

140 LOCATE 16,29:  PRINT " Michael A. Covington"

150 LOCATE 25,26: PRINT "(Press any key to continue)";

160 IF INKEY$="" THEN 160

162 CLS: LOCATE  5,1 : PRINT,"This program helps you design astable multivi-"

164 PRINT,"brator circuits using the popular type 555 timer IC." : PRINT

166 PRINT,"The circuit diagram is given on the screen and you"

168 PRINT,"fill in values for the components; the frequency, "

170 PRINT,"period, on time, off time, and duty cycle are computed"

172 PRINT,"for you.  You can then change values if you wish."

174 PRINT

176 PRINT,"If you have an IBM Graphics Printer or a graphics"

178 PRINT,"screen dump program for your printer, you can get"

180 PRINT,"a hard copy of the circuit and its parameters at"

182 PRINT,"any time.

184 LOCATE 25,26: PRINT "(Press any key to continue)";

186 IF INKEY$="" THEN 186

200 REM  Draw diagram of 555 astable oscillator

205 ' outer boundaries of wiring

210 CLS : KEY OFF

220 LOCATE 1,12 : PRINT STRING$(42,196);

230 LOCATE 22,12 : PRINT STRING$(42,196);

240 FOR I=1 TO 22 : LOCATE I,12 : PRINT CHR$(179); : NEXT

242 LOCATE 1,12 : PRINT CHR$(218);

244 LOCATE 22,12 : PRINT CHR$(192);

248 ' outline of 555 itself

250 LOCATE 5,22 : PRINT STRING$(21,205);

260 LOCATE 18,22 : PRINT STRING$(21,205);

270 FOR I=5 TO 18 : LOCATE I,22 : PRINT CHR$(186); : NEXT

280 FOR I=5 TO 18 : LOCATE I,42 : PRINT CHR$(186); : NEXT

290 LOCATE 5,22 : PRINT CHR$(201);

300 LOCATE 5,42 : PRINT CHR$(187);

310 LOCATE 18,22 : PRINT CHR$(200);

320 LOCATE 18,42 : PRINT CHR$(188);

325 ' resistor

330 LOCATE 3,11 : PRINT CHR$(218);CHR$(193);CHR$(191);

340 FOR I=4 TO 5: LOCATE I,11 : PRINT CHR$(179); " "; CHR$(179); : NEXT

350 LOCATE 6,11 : PRINT CHR$(192);CHR$(194);CHR$(217);

360 ' resistor

370 LOCATE 9,11 : PRINT CHR$(218);CHR$(193);CHR$(191);

380 FOR I=10 TO 11 : LOCATE I,11 : PRINT CHR$(179);" ";CHR$(179); : NEXT

390 LOCATE 12,11 : PRINT CHR$(192);CHR$(194);CHR$(217)

400 ' capacitor

410 LOCATE 18,11 : PRINT CHR$(196);CHR$(193);CHR$(196);

420 LOCATE 19,11 : PRINT CHR$(196);CHR$(194);CHR$(196);

430 ' connect pin 1

440 LOCATE 17,32 : PRINT "1";

450 LOCATE 18,32 : PRINT CHR$(209);

460 FOR I=19 TO 21 : LOCATE I,32 : PRINT CHR$(179); : NEXT

470 LOCATE 22,32 : PRINT CHR$(193);

480 ' pin 2

490 LOCATE 16,12 : PRINT CHR$(195);STRING$(9,196);CHR$(182);"2";

500 ' pin 6

510 LOCATE 14,12 : PRINT CHR$(195);STRING$(9,196);CHR$(182);"6";

520 ' pin 7

530 LOCATE 8,12 : PRINT CHR$(195);STRING$(9,196);CHR$(182);"7";

540 ' pin 8

550 LOCATE 1,30 : PRINT CHR$(194);

560 FOR I=2 TO 4 : LOCATE I,30 : PRINT CHR$(179); : NEXT

570 LOCATE 5,30 : PRINT CHR$(207);

575 LOCATE 6,30 : PRINT "8";

580 ' pin 4

590 LOCATE 1,34 : PRINT CHR$(194);

600 FOR I=2 TO 4 : LOCATE I,34 : PRINT CHR$(179); : NEXT

610 LOCATE 5,34 : PRINT CHR$(207);

620 LOCATE 6,34 : PRINT "4";

630 ' pin 3

640 LOCATE  9,41 : PRINT "3";CHR$(199);STRING$(11,196);

650 ' various labels

660 LOCATE 10,30 : COLOR 0,7 : PRINT " 555 ";: COLOR 7,0

670 LOCATE 1,55 : PRINT "V+ (5-15v)";

680 LOCATE 9,55 : PRINT "Output";

690 LOCATE 22,55 : PRINT "V-";

700 LOCATE 4, 1 : PRINT "R1";

710 LOCATE 10, 1 : PRINT "R2";

720 LOCATE 18,1: PRINT "C";

800 REM  Get values and do calculations

805 LOCATE 25,1 : PRINT STRING$(79," ");

810 IF R1*R2*C <> 0 THEN LOCATE 25,40: PRINT "(Press RETURN to leave unchanged)";

812 '  Get value of R1

815 LOCATE 25,1 : PRINT STRING$(39," ");

817 IF R1>0 THEN LOCATE 25,10 : PRINT R1;

820 LOCATE 25,1 : PRINT "R1 ("+CHR$(234)+")  "; : INPUT;RESP

830 IF RESP>0 THEN R1=RESP

831 ' Verify that R1 is within safe range

832 IF R1>999 THEN 835

833 BEEP : LOCATE 24,1 : PRINT "R1 should normally be > 1000 ";CHR$(234);" to protect the discharge transistor.";

834 GOTO 812

835 LOCATE 24,1 : PRINT STRING$(79," ");  ' Clear error msg. if any

839 LOCATE 5,1 : PRINT STRING$(10," ");CHR$(179);" ";CHR$(179);"     ";

840 LOCATE 5,1 : PRINT R1;CHR$(234);

842 ' Get value of R2

845 LOCATE 25,1 : PRINT STRING$(39," ");

847 IF R2>0 THEN LOCATE 25,10 : PRINT R2;

850 LOCATE 25,1 : PRINT "R2 (";CHR$(234);")  "; : INPUT;RESP

860 IF RESP>0 THEN R2=RESP

869 LOCATE 11,1 : PRINT STRING$(10," ");CHR$(179);" ";CHR$(179);"     ";

870 LOCATE 11,1 : PRINT R2;CHR$(234);

872 ' Get value of C

875 LOCATE 25,1 : PRINT STRING$(39," ");

877 IF C>0 THEN LOCATE 25,10 : PRINT C;

880 LOCATE 25,1 : PRINT "C (";CHR$(230);"F)  "; : INPUT;RESP

890 IF RESP>0 THEN C=RESP

899 LOCATE 19,1 : PRINT STRING$(10," ");CHR$(196);CHR$(194);CHR$(196);"     ";

900 LOCATE 19,1 : PRINT C;CHR$(230);"F";

1000 ' Calculate output frequency, period, and duty cycle

1010 TH =  693 * (R1+R2) * (C/1000000!)   ' c is in microfarads

1020 TL =  693 * R2 * (C/1000000!)

1030 PERIOD = TH+TL

1031 FREQ = 1000 / PERIOD   ' times are in milliseconds

1032 IF TH+TL < .001 THEN UNIT$=CHR$(230)+"sec" ELSE UNIT$="msec"

1033 IF UNIT$<>"msec" THEN TL=TL*1000 : TH=TH*1000 : PERIOD=PERIOD*1000

1035 UNIT$=" "+UNIT$

1041 ' Print results

1045 RSF.N = 3  ' We will be rounding to 3 sig. figs.

1047 RSF.X = FREQ : GOSUB 40000

1049 LOCATE 11,45 : PRINT STRING$(35," ")

1050 LOCATE 11,45 : PRINT "Frequency   ";RSF.X$;" Hz";

1052 RSF.X = 100*(TH/PERIOD) : GOSUB 40000

1053 LOCATE 12,45 : PRINT STRING$(35," ")

1054 LOCATE 12,45 : PRINT "Duty cycle  ";RSF.X$;" % high";

1057 RSF.X = PERIOD : GOSUB 40000

1059 LOCATE 14,45 : PRINT STRING$(35," ")

1060 LOCATE 14,45 : PRINT "Period           ";RSF.X$;UNIT$  ;

1067 RSF.X = TH : GOSUB 40000

1069 LOCATE 15,45 : PRINT STRING$(35," ")

1070 LOCATE 15,45 : PRINT "Duration (high)  ";RSF.X$;UNIT$  ;

1077 RSF.X = TL : GOSUB 40000

1079 LOCATE 16,45 : PRINT STRING$(35," ")

1080 LOCATE 16,45 : PRINT "Duration (low)   ";RSF.X$;UNIT$  ;

1099 ' Prompt for another try

1100 LOCATE 25,1 : PRINT STRING$(79," ");

1110 LOCATE 25,1 : PRINT  "Another?  (Press Y or N)";

1120 R$=INKEY$ : IF R$="" THEN 1120

1130 IF R$="Y" OR R$="y" THEN 800

1140 IF R$<>"N" AND R$<>"n" THEN BEEP: GOTO 1120

1150 LOCATE 23,1 : KEY ON

1200 END

1300 '

1400 '

2000 GOTO 800

40000 '  RSFCONV -- Round to a specified number of significant

40010 '             digits and convert to a string

40020 '

40030 '  Michael A. Covington         7/9/83

40040 '  Copyright 1983

40050 '  All rights reserved

40060 '

40070 '  Accepts:  rsf.x  -- number to be converted

40080 '            rsf.n  -- desired number of significant digits

40090 '

40100 ' Returns:  rsf.x$ -- number converted to string

40110 '

40120 ' Return E format numbers without further processing

40130 RSF.Y$=STR$(RSF.X)

40140 IF INSTR(RSF.Y$,"E")>0 THEN RSF.X$=RSF.Y$ : RETURN

40150 '

40160 ' Apply correction to produce rounding rather than truncation

40170 RSF.ABSVAL = ABS(RSF.X)

40180 RSF.EXP=15

40190 WHILE 10^RSF.EXP > RSF.ABSVAL : RSF.EXP=RSF.EXP-1 : WEND

40200 RSF.EXP=RSF.EXP+1-RSF.N

40210 RSF.CORR = SGN(RSF.X) * .5 * 10^RSF.EXP

40220 RSF.Y$=STR$(RSF.X + RSF.CORR)

40230 '

40240 RSF.X$ = ""

40250 RSF.FIGCOUNT=0

40260 '

40270 ' Transfer digits into new string one by one

40280 IF RSF.FIGCOUNT = RSF.N THEN 40350

40290 RSF.PICK$ = LEFT$(RSF.Y$,1)   ' left most character

40300 RSF.Y$ = MID$(RSF.Y$,2)       ' remainder

40310 IF RSF.PICK$ = " " THEN 40280

40320 IF RSF.PICK$="." OR RSF.PICK$="-" THEN RSF.X$=RSF.X$+RSF.PICK$ : GOTO 40280

40330 RSF.X$=RSF.X$+RSF.PICK$ : RSF.FIGCOUNT=RSF.FIGCOUNT+1 : GOTO 40280

40340 '

40350 ' Right number of significant digits have now been processed.

40360 ' Do any trailing zeroes need to be added?

40370 RSF.ABSVAL = ABS(RSF.X + RSF.CORR)

40380 WHILE 10^RSF.FIGCOUNT <= RSF.ABSVAL

40390   RSF.X$=RSF.X$ + "0"

40400   RSF.FIGCOUNT = RSF.FIGCOUNT+1

40410 WEND

40420 '

40430 RETURN

Another file downloaded from:                     NIRVANAnet(tm)

& the Temple of the Screaming Electron              415-935-5845

Just Say Yes                                        415-922-2008

Rat Head                                            415-524-3649

Cheez Whiz                                          408-363-9766

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"