This is an actual document from BELL LABS and is presented by P-80

systems and is presented in its etirety!!!

CARD-READING PUBLIC STATIONS REQUIREMENTS Bell Communications Research,

Inc.

1. This document describes generic requirements for card-reading public

telephone stations. These sttions will be deployed by a Bell Operating

Company (BOC) primarily to provide customers having magneic-stripe

cards with easier access to both BOC and inte r-LATA carrier (IC)

facilities.

A BOC Card-Reading Public Station (alternatively referred to as

"station") is intended to work similrly to a current Charge-a-Call

station with features added to read a magnetic-stripe card and

convenently select an IC. The BOCs expect to issue Calling Ca rds for

use in making intra-LATA toll and locl calls over the BOC networks, and

inter-LATA calls over any carrier capable of accepting a Calling ard

number. Similarly, calls billed via Commercial Credit Cards (CCC) would

be possible if the involed BOC or IC were able to accept them.

1.1 Outline of Document

Section 1.2 describes the scope of the document. The general operation

of the station is described i section 1.3.

Section 2 specifies the requirements for a card-reading public

telephone station. Section 2.1 sets te requirements for card

presentation, including how and when the card should be presented, and

what he station should do if the presentation of the card di ffers from

that required.

Section 2.2 specifies the requirements for signaling by the customer.

This includes acceptable diale destination numbers, methods of

selection of the IC, and anti-fraud features of the station.

Section 2.3 specifies the requirements on how the station will

interface with the end (local) office The features of the line and

electrical characteristics are described.

Section 2.4 describes the interface between the station and the IC or a

processor. This includes desription of the access environment and what

the station should do to help set up calls.

Section 2.5 specifies the physical characteristics of the station.

Section 3 contains requirements prtaining to the support that the

supplier should provide for these stations.

1.2 Scope of Document

This document provides functional requirements for a card-reading,

non-coin public station. The requrements are intended to describe what

the station does in its various interactions with the customer the

local office, and the carrier of choice or an asso ciated processor.

The requirements also descrbe the physical environment in which the

station operates. This document is not intended to dictate ow given

functions are specifically realized.

Card reading transactions described here are limited to magnetic-stripe

card technology and cover ony those actions required to set up and

properly bill a call. These requirements are generally limite to the

station itself. Actions by other facilities to provide public calling

via credit cards are icluded only insofar as they relate to pertinent

station functions.

This document contains requirements primarily concerning public

stations, card reading, and carrier election. The station should also

meet requirements specified by the FCC Rules regarding registratio of

telephone sets (Part 68), FCC Rules regarding radia ted emissions (Part

15), requirements concernng the normal station to network interface,

and requirements concerning abnormal conditions on the lcal loop.

Human factors of the station and the related service have not been, for

the most part, specifically ddressed. Good human factors design,

however, is crucial to a successful product, and underlies manyof the

requirements.

1.3 General Operation of the Station

This document describes a station similar to Charge-a-Call stations,

with features added to read a mgnetic-stripe card and conveniently

select an IC to handle the call. This station needs to provide fur

basic functions:

Voice-band transmission and reception,

Addressing the far end party,

Selection of a carrier,

Relaying appropriate billing and call status information.

The first two functions are common to all telephone stations. Carrier

selection is a relatively new unction and one of increasing importance.

Relaying of billing information is a function inherent in ublic

stations.

The station described in this document will operate as a typical modern

telephone with respect to voce-band transmission and reception.

Addressing the far end will be done by standard Dual-Tone

Multi-requency (DTMF) signals.

The station will provide the customer at least one way to select an IC.

One way lets the customer dil a particular sequence of digits on the

DTMF key pad. This includes speed code dialing or 7-digit acess numbers

for carriers, as well as equal access carr ier codes (10XXX). A second

way allows the cutomer to select certain ICs by pressing a single

button on a button field distinct from the DTMF keypad. This is

sometimes referred to a "Select-a-Carrier" or Choose-a-Carrier"

feature. The station wil have to translate the single button activation

into an appropriate sequence of DTMF signals, but tis translation will

be largely transparent to the customer. A third potential way that

could be implmented in combination with the first two ways would have

an IC co de on the magnetic stripe of a Callng Card. This Calling Card

could be issued by a BOC or by the IC. If the BOC issued the Calling

Card the call would be directed to that carrier for inter-LATA calls

unless the customer overrides. If te IC issued the Call ing Card, the

carrier selection could not be changed.

The station will provide the customer three ways of entering billing

information. The first tow waysare available on today's Charge-a-Call

stations and involve entering information by voice to an opertor, or by

use of the DTMF key pad. The third way allow s the customer to enter

billing information b physically presenting a magnetic-stripe credit

card to a card-reading device in the station.

In general, the station will need enough intelligence to perform the

following functions:

Accept carrier selection and billing information from the customer.

Store some of that information for later use in completing the call, if

it is not immediately needed

Communicate billing and/or addressing information to carriers or

associated processors, possibly accrding to more than one protocol.

2. Station Requirements

Two types of features of the station are described in this document.

The word "should" indicates a mndatory requirement. "It is desirable"

indicates a non-mandatory feature. These requirements are dynmic and

could be influenced by time, technology, market

strategy, or economics.

2.1 Card Reader

The card reader and its associated electronics needs to provide four

basic functions: accept the car from the customer, read it, and get it

safely back to the customer; perform certain checks on the dta encoded

on the magnetic stripe; separate the card da ta into meaningful data

fields and store eachfield for use when and if it is needed; and notify

the customer about errors or invalid cards.

This document does not specify a type of card reader. Any type may be

used, as long as it can be easly used by the customer. Some of these

requirements pertain only to particular card reader types.

2.1.1 Card Presentation

1. When the customer presents a card to an off-hook station, the card

reader should read the card, ceck its validity (see Section 2.1.2), and

store the data for use during call set-up by the station.

2. Any card data should be purged from the station after the station

goes on-hook.

3. If the card reader type entails the customer releasing the card

while it is being read, the statin should alert the customer to remove

the card from the reader before sitting up the call. The statin should

not proceed with the call setup until the card

is removed. Also, it is desirable that part f the card always remain

in sight of the customer.

4. The customer should be able to remove the card at any time, even

while it is being read.

5. The card reader should be positioned such that the customer action

is natural. If the customer moes the card in a vertical slot, the

motion should be downward. In a horizontal slot, the card shouldbe

moved from left to right. For insertion type readers , the card should

be pushed forward and then emoved.

6. If the customer must move the card past the reader head, the card

reader should successfully readthe card over the range of speeds that

the customer might reasonably attempt to move the card.

7. The customer should receive tactile feedback when presenting the

card to the card reader, as wellas feedback when the card is read

successfully.

2.1.2 Checks

1. The station should check the parity of each character. Each data

character will be encoded on thecard with an odd parity bit.

2. The station should do a Longitudinal Redundancy Character (LRC)

check. The LRC is a check charactr for the remaining data on the

magnetic stripe. The first four bits in the LRC check the correspondng

bits in the other characters on the magnetic stripe.

Each bit is an even parity bit that checks te corresponding bits in

the other characters on the card. The fifth LRC bit is an odd parity

bit forthe LRC itself.

3. The station should check the format of Calling Cards for proper

layout of the data on the card. Te format is specified in Section

2.1.3. If the four digits of the PIN are zeros, the actual PIN is nt on

the card.

4. It is desired that the station not check the expiration or effective

dates on the card.

2.1.3 Card Characteristics

1. Calling Card characteristics are covered thoroughly in Reference

(3). Both Calling Cards and CCCswill adhere to standards set by the

American National Standards Institute (ANSI) and the American Bakers

Association (ABA).

2. The Calling Card data will be contained on Track 2. The first

character will be a start sentinel.The next six characters will be a

one-digit major industry identifier, and a five-digit issuer identfier.

A ten-digit billing number will be encoded in the

next ten characters, followed by a Luhn mod 0 check character and

field separator. The next four characters will contain an expiration

date. Folowing the expiration date, the four-digit Personal

Identification Number (PIN) will be encoded. The agnetic st ripe may

also contain up to another 11 digits of data used to describe various

services. Tree of the characters may be a carrier identifier, or may be

unused. The next 8 potential charactersare, at this time, unused.

Unused characters will not be encoded on the magnetic stripe. The end

seninel will follow the last data character, and the LRC will follow

the end sentinel.

3. The start and end sentinels should be transmitted as DTMF tone D.

The separator characters shouldbe transmitted as DTMF tone C. The LRC

should not be stored or transmitted as card data. Thus, no moe than 39

characters of magnetic stripe data will need to be stored by the

station.

4. If the last four characters of the 14-digit Calling Card number are

zeros, the Calling Card PIN i not on the magnetic stripe. The specific

data fields to be transmitted are specified in Sections in2.4.

5. The station should be capable of being field modified to accommodate

changes in the format of thecard. 2.1.4 Error Feedback

1. The station should provide feedback to the customer for the

following problems on card presentatin and reading:

a. The magnetic stripe on card is damaged, causing errors in reading.

b. The customer presented the card in the wrong orientation. The

station should include clear user istructions, or graphics, on how to

present the card.

c. The card is not an acceptable CCC or Calling Card (i.e., issuer or

industry identification is inapropriate).

2. It is desired that the station respond to these errors by emitting

tones, or by sending inband sinals to the network, as appropriate. The

inband signals should be sent using DTMF tones, using the cdes defined

in Section 2.4.4.

2.2 Customer Signaling

The station is expected to accommodate three basic customer signaling

functions: carrier selection va either the DTMF key pad,

"Select-a-Carrier" buttons, or carrier pre-selection on the Calling

Card;far-end party address signaling; and possible manual di aling of

billing information. In addition, th station is expected to incorporate

anti-fraud features.

2.2.1 Carrier Selection

1. The station should implement either Select-a-Carrier buttons or

allow manual dialing of carrier acess codes, or both.

2. If a method is implemented, it should be implemented according to

the requirements in Sections 2..1.1 and 2.2.1.2.

3. It is desirable that the station accept carrier specific Calling

Cards as described in Section 2..1.3.

2.2.1.1 Separate Select-a-Carrier Buttons

1. Positive feedback (an appropriate combination of aural, visual, and

tactile), should be provided hen a button is pressed.

2. The number of buttons should be kept to a reasonable level. It is

suggested that no more than 12 uttons be provided.

3. When transmitting the number to the local office, that is, when

using the station as a Select-a-Crrier station, the station should

substitute an access number or a speed calling code for the

buttonpressed. The access number may be up to twelve digits. The speed

calling code may combine digits wit the * or # signal.

4. When transmitting to a processor (dial-up or directly connected, as

described in Sections 2.4.4 ad 2.4.5), the station should transmit

10XXX or *XXX for the button pressed.

2.2.1.2 Manual Dialing of Carrier Access Code

1. The station should transmit all digits dialed on the key pad as they

are entered.

2.2.1.3 Pre-Selected Carrier on Card

1. If the station reads a Calling Card with a carrier code included on

the magnetic stripe in the opional field (see Section 2.1.3), the

station should direct the call towards a processor (as explaine in

Section 2.4).

2. If the issuer identifier is '8555', the card is an IC Calling Card

issued by AT&T Communications ATT/C), even though these cards will not

have a carrier code in the optional field on the magnetic sripe. These

calls should be directed towards AT&T/C.

2.2.2 Address Signaling

1. The station should allow the customer to dial the terminating number

before or after carrier seletion.

2. These dialed numbers should be expected from the stations:

a. 0+7/10 Digits

b. 01+Country Code+National Number (7 to 12 digits in CC+NN)

c. 411, 611, 911

d. (0,1)+800+7 Digits

e. 0-

f. (1)+555-XXXX

g. (1)+NPA-555-XXXX

h. 950-XXXX, if a carrier is not selected in another manner.

3. The local office can block all other dialing sequences that may be

dialed. Thus, it is normally nt required for the station to screen for

improper dialing sequences. If a BOC specifies that it is ncessary to

do so, an optional feature could allow the s tation to block all or

most calls not on the bove list.

4. It is desirable that the station enable new calls to be placed

without reusing a card. The methodto make sequence calls will be

determined.

2.2.3 Anti-Fraud Features

1. The station should prevent the customer from signaling via

manipulation of the switchhook.

2. For loop-start lines, the station should disable the dial until dial

tone is received. For groundstart lines, the local office can detect

DTMF tones as soon as current is provided to the station.

3. The station should mute or otherwise control the transmitter during

any time where an acoustic copler could be used for fraud purposes. In

particular, the transmitter should be muted or controlled uring the

transmission of the card number and whenever the dial is disabled.

Specific requirements o when to mute or control the transmitter are

detailed in Sections 2.4.4 and 2.4.5.

2.3 End Office Interface

Electrical and signaling characteristics of the station should meet the

requirements described in: Rference (1) concerning the normal station

to network interface; Reference (2) concerning abnormal coditions on

the local loop. The station should also meet

requirements specified by the FCC rules regading registration of

telephone sets.

2.3.1 Features of Line

1. The station should operate on a line with Charge-a-Call class of

service. This class of service alows only nonsent-paid and free calls.

An optional feature may allow the station to operate on lineswithout

Charge-a-Call class of service, as described in

Section 2.2.2, item 3.

2 The stations should operate with any electronic end office commonly

in use by the BOCs.

3. The station should operate on either standard loop-start or

ground-start lines.

4. Answer supervision cannot be provided on these lines. Thus, the

station should not depend on answr supervision for any functions.

2.3.2 Electrical Characteristics

1. The loop current from the local office to the station will be a

minimum of 23 milliamps at 48 vols DC. It is desirable that the station

operate on this power level without a supplementary power suply. If the

station needs more than 23 milliamps current , a supplementary power

source may be used.

2. The polarity of the loop should not affect operation of the station.

2.3.3 Signaling Characteristics

1. The station should operate on lines with DTMF signaling, also know

as Touch-Tone.

2. When dialing Calling Card, CCC, or IC access numbers, the station

should transmit digits at a maxmum rate of 10 per second. The tone

duration and interdigit interval should not be less than 50 ms ad 45

ms, respectively, and the cycle time (sum of tone duration and

interdigital time) should not beless than 100 ms. It is desirable that

digits be transmitted at close to the maximum rate.

2.4 Carrier and Processor Interfaces

These stations should interface either with a carrier or with a

processor (designed to interface wit one or more carriers), depending

on instructions programmed into the station.

It is proposed that the station be kept simpler by using identical

protocols to all ICs whenever feaible. Still, the need for different

protocols is envisioned for direct station-carrier interfaces. Crriers

using Feature Groups A or B would conform to a s ingle protocol from

these stations (currentlyundefined). Feature Group D carriers, except

for ATT/C, might need to conform to another standard prtocol. ATT/C,

whether Feature Group C or D, would use a third protocol. These

protocols are describe in Sectio ns 2.4.1 through 2.4.3. The station

would determine which protocol to use based on the cusomer's actions to

select the carrier.

An interface to a processor could be substituted for the carrier

arrangements. The processor would, n turn, establish and administer the

interfaces to the ICs. The processor could be dialed up by the tation

or directly connected. Call sequences and statio n actions for dial-up

and directly connected rocessors are described in Sections 2.4.4 and

2.4.5, respectively.

The station should be programmable to send all calls of certain types

to a dial-up processor, and hadle others by the protocols described for

the various carriers. Section 2.4.4 specifies the calls tht should

cause the station to dial the processor. The d ial-up processor would

then handle the call st-up for those calls. Calls where the customer

begins the call by dialing a number or selecting a carier would be

handled using Charge-a-Call treatment or direct interfaces to the

carriers.

2.4.1 Call protocol for BOC and ATT/C (Feature Group C)

1. BOC and ATT/C (FG C) calls are set up by the customer dialing 0+ the

terminating number after reciving dial tone. The station should then

wait for the "bong tone" from the TSPS. The bong tone is deined to be

100 ms or the DTMF frequencies for the # sig n, 941 and 1477 Hz,

followed by 1.4 seconds o dial tone, 350 and 440 Hz. The amplitude of

the signal starts at -7dBm0/frequency +/- 1dB at -3 TLP The amplitude

of the dial tone portion is exponentially decayed with a time constant

of 200 ms. Theamplitudes

at the station can vary by loop. The range of amplitudes will be

determined.

2. The station should be able to prefix a digit (e.g., '9') before the

first customer-dialed digit. This feature should only be used if it

cannot be avoided. This feature could cause human interface poblems.)

3. The station should then transmit only the 10 (if PIN not on card) or

14 digits of the BOC or AT&TCalling Card number (as explained in

Section 2.1.3.

4. The station should begin transmitting the Calling Card number within

500 ms or recognition of thebong tone. It is desirable that the digits

be transmitted at the maximum rate.

2.4.2 Call Protocol for Feature Groups A or B IC

The call protocol for Feature Groups A or B ICs will be determined

later if any ICs are interested i being accessed directly by the

stations.

2.4.3 Call protocol for Feature Group D IC

The call protocol for Feature Group D ICs will be determined later if

any ICs are interested in bein accessed directly by the stations.

2.4.4 Call sequence for Dial-Up Processor

1. The station should dial the processor number, as programmed, only if

a customer presents a card t the station first. The station should be

able to store and dial a one-to-seven-digit processor numbr.

(EXCEPTION) ((If an AT&T Calling Card is presented to the station

first, the station should store th card number and handle the call as

described in Section 2.4.1.))

2. When a processor is connected, the station should detect a bong tone

that indicates the processoris ready to receive DTMF signals. The

station should then transmit a calling station identification up to ten

digits, if required),DTMF tone 'D' and all th e data from track 2 of

the magnetic stripe ofthe Calling Card or CCC, DTMF tone 'D' again, and

the carrier access code (10XXX), if a carrier is slected. The processor

will have announcements to prompt the customer, but the station should

transmi the data after detecting the processor tone, or as soon as the

customer provides the information, wichever is later.

3. The station should begin transmitting the calling station

identification within 500 ms of recogniion of the tone. It is desirable

that the digits be transmitted at the maximum rate.

4. The station should disable the transmitter in the handset during

transmission of the processor nuber until the card number and DTMF tone

'D', a 3 digit code, and DTMF tone 'D' again for these messaes to the

processor:

CODE      MESSAGE

011       Card not readable (failed parity check) 002       Not an

ANSI-standard Card 003       Self-diagnostic indication of card reader

trouble 08X,09X   Give announcements in specified language

Other codes and messages will be assigned when needed.

6. The station should allow the customer to dial the terminating number

or carrier codes any time afer the billing data are transmitted.

7. If the customer dials a number first (could include dialing a

carrier), the station should transmt all digits dialed and monitor for

a bong tone. If it detects a bong tone, it should transmit a 14-igit

Calling Card number if one was read, or allow manu al dialing; if no

bong tone is detected withi five seconds after the last digit dialed,

the station should transmit any card number read after tht time.

8. If the customer presses a Select-a-Carrier button first, the station

should transmit the code stoed for that button. The station should then

operate the same as if a number had been dialed first. Hwever, the card

data transmitted may be carrier specifi c.

9. After receiving the processor tone, the station should read out the

last card number read.

2.4.5 Call Sequence for Direct-Connect Processor

1. The station should accept inputs from the card reader, the DTMF dial

pad, or a Select-a-Carrier btton pad when the handset is off-hook.

2. After a card is presented with the station off-hook, the station

should send DTMF tone 'D', the dta from track 2 of the magnetic stripe

of the Calling Card or CCC, and DTMF tone 'D' again. The dialpad should

be disabled while the station is transmittin g the card data.

3. The station should send the codes described in Section 2.4.4, if

appropriate, instead of the cardnumber.

4. The transmitter should be disabled from the time the handset goes

off-hook until a card is presened and transmitted, or until a digit is

dialed. This prevents a customer from acoustically coupling TMF tone

'D' and a stolen or made-up card number into t he handset.

5. If a card is presented first, the station should transmit the

carrier access code if a carrier isselected, or allow the customer to

dial.

6. If a number is dialed first, the station should not transmit any

carrier codes, and should only tansmit card data after it receives a

bong tone, or waits at least 5 seconds after the last digit diaed and

then reads a card.

2.5 Physical and Other Requirements

The station will be composed of several major physical components.

2.5.1 General

1. These requirements specify a station to operate in a public, indoor

environment.

2. All components of the station should be flame resistant.

3. The station should be built to withstand the abuse expected in a

public environment.

4. It is desirable that self diagnostics be designed into the card

reader.

5. The station should not be susceptible to radiated emissions from

other sources that are within leal limits. It is desirable that the

station not be susceptible to any radiated emissions from other ources

that may reasonably be encountered.

2.5.2 Card reader

1. The card reader should be designed such that the reader head cannot

be easily damaged by misuse o abuse encountered in a public

environment.

2.The card reader should be protected to minimize the frequency of

cleaning.

3. The card reader should be easily maintained.

4. The card reader should be physically shielded from the interior of

the set such that customers canot access or tamper with the electrical

components of the station.

2.5.3 Dial Pad

1. The dial pad should be will designed from a human factors standpoint

with respect to button spacig, shape, graphics, and activation

feedback.

2. The station should have a moisture and dirt resistant dial pad.

3. The dial should be durable. Each key should withstand at least

500,000 depressions before failure

2.5.4 Handset and Switch-hook

1. On wall sets, the handset should be connected to the station with a

cord that can withstand a 400pound tensile test.

2. The handset cord should have enough cutting and shear resistance to

withstand attacks by tools suh as knives or scissors. Tools with

cutting ability equal to or less than 5-inch diagonal cutters shuld not

be able to sever the cord.

3. When destroyed while in service due to any circumstances, the cord

should not create a customer sfety hazard.

4. The handset should be manufactured such that it cannot be

disassembled by customers or field crafspersons. The handset and cord

should only be replaceable by opening the station housing.

5. The handset should allow people with hearing impairments to use the

telephone with their hearing ids. The station should meet the

Electronics Industries Association's Recommended Standard, RS-504,

agnetic Field Intensity Criteria for Telephone Compatib ility with

Hearing Aids, to be considered heaing aid compatible. This capability

should be shown by use of a blue grommet on the handset cord.

6. The handset and switch-hook should be able to withstand at least

200,000 hang-ups before failing. 2.5.5 Station Housing

1. The wall station should be capable of being mounted in standard

enclosures currently used by BOCs including 10A-type enclosures and the

178A backboard.

2. It is desired that the station be equipped with locks to discourage

theft of the set or tamperingwith the program codes.

3. Space should be available on the face of the set to provide

instructions for use of the set, and o place the telephone number and

station location information.

3. Support

This section defines the obligations of the supplier to support the

station. Additional requirementsare to be determined.

1. The supplier should provide documentation for the installation,

maintenance, and testing of the sations.

2. The supplier should provide satisfactory means for replacement

parts.

3. The supplier should have a factory repair service, or similar means

for refurbishment, for repair that cannot be made in the field.

4. The supplier should provide data on the reliability of the station

and each major component.

5. The supplier should have an adequate quality assurance program,

including a means for handling enineering complaints on product design,

manufacturing, operation, maintenance, documentation, and othr aspects

of the product.

This file has been presented by P-80

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

-=-=-X

Another file downloaded from:

NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter

510-935-5845

Salted Slug Systems                      Strange

408-454-9368

Burn This Flag                           Zardoz

408-363-9766

realitycheck                             Poindexter Fortran

415-567-7043

Lies Unlimited                           Mick Freen

415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man

408-961-9315

My Dog Bit Jesus                         Suzanne D'Fault

510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you

are,

where you live, or what your phone number is. We are not Big

Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

-=-=-X