ASS80XX

Written by Jolly Roger for the Jolly-Box

There  is  a  short  descripion of the 8049 assembler for the

Jolly-Box.   It  is  necessary because you need to modify the

source to your own developing.  It includes all opcodes.

The format must be as followed:

-No case sensity

-Only one opcode in one line and one label

label:	clr	a

label2:

jmp	label

-You can use Decimal, Hex or Binary for values

mov	a,=128

mov	a,=$80

mov	a,=%10000000

-Characters  after a semikolon will be ignored until the next

line

clr	a	;set accu to zero

-Use  space  as well as tab to seperate.  Seperations between

label and opcode, opcode and semikolon are not necessary.

-Jumps must always jump to a label, numeric is not allowed

If you like to skip to a memory page:

page	2

will  skip  to  byte  512  of memory.  If it is already passed an

error will occure.

Remember  that  only  JMP and CALL can jump to an adress within a

page of 256!

How the opcodes work refer to the 8049 manual.