The following is a guide to the hobby of monitoring the

Public Service bands, and some general tips for beginners to

the hobby.

This guide is broken down into 3 parts:

(1) Equipment

(2) Scanning tips

(3) Section 605, ECPA of 1986, Responsibility

Equipment

-----------------

SCANNER TYPES: tunable, crystal, programmable

Tunable: Many types of tunable receivers are available to the scanner user.

most of these have poor image rejection and were low-end units or multiband

type radios. These were subject to intermod and are only good for a few very

strong signals and local coverage. Poor choice.

Crystal: Crystal-controlled scanners have been around since the 60's. These

radios very from 1 channel to as many as 16 and are very good performers.

They allow a set, 1 frequency-1 channel capability, and have good image

rejection and filtering. These radios allow the first time user to sample

frequencies at relatively low cost per channel, about 5 dollars per crystal.

Crystal controlled receivers are also good for professional users as the

cost is low and most Public Service users only need to monitor a few set

interim channels.

Programmable: These are by far the best for the serious scanner enthusiast.

Programmable receivers have good S/N figures, good image rejection, and for

the most part very little intermodulation. They also allow greater

flexibility by allowing the user to change channels without spending money

for crystals. Most all of the programmable radios have LED or LCD readouts

that give frequency, channel number, and other user information. All have

memory backup to retain the contents of what has been programed.

Programmable receivers are by far the best cost per dollar units.

These radios use to cost 300 to 700 dollars a few years ago but with

computer technology, low IC cost , they now can be had for a little over

100.00. The main compromise in price these days for a programmable scanner,

is channels, frequency coverage, and special features.

A minimum setup would be 20 channels, and the 118-136 Mhz aircraft band.

150.00 for this setup.

* Scanner Tips *

Now that the radio is sitting there, how  do you know what frequencies to

enter ? Most manuals that come with scanners have a spectrum breakdown to

help new users find a ballpark area of interest.

Frequency Spectrum Breakdown   30Mhz  to  512Mhz

30 to 50 low-band VHF -- military,police,fire,business, new cordless phones

46-49mhz mostly older systems in rural areas,mobile phone(not used in larger

areas.paging systems used extensively. Subject to long range skip conditions

and solar activity.

50 to 54Mhz Amateur 6 meter band

54 to 72mhz TV Ch. 2-6

72 to 88Mhz low power industrial

88 to 108Mhz FM Broadcast Band

108 to 136 (AM) Aircraft band,Tower frequencies start at 118.000

136 to 144Mhz land mobile (Military)

144 to 148Mhz Amateur 2 meter band

148 to 150.600 Mhz military

150.6 to 152Mhz Business

152 to 153Mhz RCC mobile phones and taxi cabs

153 to 154Mhz Public Transportation

154 to 155Mhz fire depts.

155 to 156Mhz State police

156 to 157Mhz Marine Band,ship to coast,ship to ship,Coast Guard,

Bridges,toll Booths,Locks,inter-coastal waterways.

157 to 158Mhz Marine operations,Coast Guard.156.800(Distress,Calling)

158 to 160Mhz State Police

160 to 162Mhz Marine phone,various users

162 to 174Mhz Govt. including all agencies

174 to 220Mhz TV Ch.7-14

220 to 225Mhz Amateur radio

225 to 400Mhz UHF Aircraft (Military),Satellites,Experimental Work

400 to 406Mhz Wx Satellites

406 to 420Mhz Govt land mobile 430 to 440Mhz Fixed

440 to 450Mhz Amateur 70cm Band

450 to 453Mhz Business, Power co.'s

453 to 454Mhz Local Police Depts.

454 to 455Mhz RCC Mobile Phone

455 to 456Mhz TV News Teams,Rebroadcasts

456 to 458Mhz Public Transport,Business,etc.

458 to 460Mhz Local Police Depts.

460 to 460.5Mhz Local Police Depts.

460.5 to 461Mhz Local Fire Depts.

462 to 463Mhz Business,GMR Service,REACT Teams

463 to 464Mhz Fire Dept. (Med Ch's)

464 to 470Mhz Public Safety,Industrial,Land Transportation

470 to 512Mhz Broadcasting,Shared With Local Public Safety and other

users.

Antennas:

Most of the antennas that come with these radios are sufficient for local

area coverage up to 75 mile radius from the listening post. If you can put

up and external antenna, your range can be up to 200 miles or more depending

on the antenna type.

External antennas:

Omni:  omnidirectional, 360 degree radius signal pickup.

Yagi:  directional, high gain

beam:  high gain, highly directional

discone: high gain, wide bandwidth, omnidirectional

Your choice of external antennas depends on what services you are looking to

hear most. Beam antennas are good for long range, single frequency coverage.

Omni's pull in many signals, where discone and yagi's tend to have higher

gain and wider bandwidths. A good compromise to all these antennas is the

Discone, with wide coverage and good signal pulling power.

Once you have your radio and antenna set up, keep in mind these tips: keep

your squelch control at a low threshold level, you will hear more. Weak

stations can sometimes be improved by re-orienting your antenna. Use NOAA

Wx. stations for your area, when aligning your antenna or testing. These

stations are continuous broadcast and can gauge receiver/antenna

performance.

Strong carriers can either be real signals or harmonics caused by other

equipment, computers, printers, clocks, TV's, etc. Reorienting the scanner

antenna sometimes will keep these signal from locking up the scanner. If the

area you are in is really bad, you can buy filters that lock out these

adjacent signals. Some phantom signals are caused by the radio itself.

These are called "intermod" and "birdies", Intermod is caused by two

internally generated signals causing a third order harmonic that locks up

the scanner. "Birdies are caused by clock and timing signals in

microprocessor controlled radio's, most of these occur at only a few

frequencies over the entire range of the radio and can not be locked out.

Most makers of scanners have installed filters to keep this from happening.

Search Scanning:

Searching for new frequencies with a programmable scanner is accomplished by

entering "upper and lower" frequency limits. A word of note, only enter a 1

megahertz search area.  The reason for this is, the smaller the search area

the more likelihood of finding new frequencies .I.E  155-156 mhz. Another

tip is to put the delay function on during search mode. This allows you to

note the frequency and also allows for a reply. When monitoring a

transmission, take notes as to ID codes and or FCC call letters. Most public

service users are required to ID their station at least once every 5

minutes.

Other Tips:

Inclement weather, storms, major events etc. have a tendency to increase

radio traffic. During storms, monitor local road crews, rescue services,

and other public service bands. The local officials generally are the people

" out in the thick of it", and you can glean much info on area road

conditions or detours etc. from this monitoring.

Mobile Installations:

Check first your local laws covering radios capable of monitoring police

frequencies in a vehicle, before installing a scanner in your car. The city

of Philadelphia at one time had a law making it illegal and a 1st offense

punishable by a 500.00 dollar fine and confiscation of the equipment.  Most

radios today have a 12 volt DC negative ground option or jack for mobile

use. Once a suitable location and power requirements are met, all that is

left is mounting the unit and running the antenna. These instructions are

generally included in the owners manual.

**** Section 605, ECPA, and Responsibility ****

As a hobbiest, it is important to observe a few rules pertaining to the

scanner hobby. First use of information overheard via a scanner for personal

gain, criminal purposes, or interference  with public safety, is not only

illegal but strongly enforced. Chasing ambulances, fire trucks etc. will

cause you to quickly loose your licence and may get you arrested. Other

responsibility falls into one important category:  Restraint. If you happen

to stumble across a set of frequencies used by local or federal law

enforcement officials, keep in mind that disclosure of any information you

may hear could interfere with a drug stake-out, hostage situation, or other

life threatening scenario.  There are situations where news media people, in

their zeal to cover a story have caused the deaths of hostages after showing

up on a scene after overhearing the operation on their scanners. Police,

Fire and Government officials have jobs to do, and don't need interference

from the public. Keep what you hear to yourself.

The Electronic Communications Privacy Act of 1986

This bill which is currently pass the senate and in the house of

representatives, is a bill designed by the cellular lobby to restrict what

you can and cannot listen to. In it's present form it makes it illegal to

"willfully" monitor any two-way communication primarily seen as closed to

the public.  This includes mobile phone, remote broadcast pickup, and

certain other services, this law is seen by the FCC and the Justice Dept.

as being unenforceable, but as with any bad law it will no doubt have to be

tested in court before it is reformed or rewritten.

Footnote: Scanning can be and enjoyable hobby and give the listener a better

insight into where his/her tax dollars are being spent. Public servants are

there for helping the people and enforcing the laws. The United States is

the only country in the world where monitoring is allowed, It is a privilege

that few other countries show their citizens. Be responsible and enjoy!

Frequency Allocations for the 806-960 MHz Band

Frequency                                     Year     Paired Channel

in Mhz        Service                      Allocated      With Spacing-kHz

806-821 m      Private Land Mobile             1975    851-866 b         25

821-824 m      Private Land Mobile Pub. Safety 1986    866-869 b         ??

824-825 m      Public Cellular Non-Wireline    1986    869-870 b         30

825-835 m      Public Cellular Non-Wireline    1975    870-880 b         30

835-845 m      Public Cellular Wireline        1975    880-890 b         30

845-846.5m     Public Cellular Non-Wireline    1986    890-891.5b        30

846.5-849m     Public Cellular Wireline        1986    891.5-894b        30

849-851        Temporary Reserve               1986    894-896

851-866 b      Private Land Mobile             1975    806-821 m         25

866-869 b      Private Land Mobile Pub. Safety 1986    821-824 m         ??

869-870 b      Public Cellular Non-Wireline    1986    824-825 m         30

870-880 b      Public Cellular Non-Wireline    1975    825-835 m         30

880-890 b      Public Cellular Wireline        1975    835-845 m         30

890-891.5b     Public Cellular Non-Wireline    1986    845-846.5m        30

891.5-894b     Public Cellular Wireline        1986    846.5-849m        30

894-896        Temporary Reserve               1986    849-851

896-901 m      Private Land Mobile Non-PubSaf  1986    935-940 b       12.5

901-902        General Purpose Mobile          1986    940-941           ??

902-928        Amateur & ISM

928-929        Fixed Multiple Address                  952-953           25

929-930        Private Paging                  1982                      25

930-931        Paging Reserve                  1982

931-932        Public Common Carrier Paging    1982                      25

932-935        Fixed Point-to-Point            1985    941-944           ??

935-940 b      Private Land Mobile Non-PubSaf  1986    896-901 m       12.5

940-941        General Purpose Mobile          1986    901-902           ??

941-944        Fixed Point-to-Point            1985    932-935           ??

944-952        Broadcast Aural STL                                   25-300

952-953        Fixed Multiple Address                  928-929           25

953-960        Fixed Multiple Address                            50,100,200

b: base station

m: mobile

NE. US. VHF AIRCRAFT FREQUENCIES :

*******************************************

Washington DC (VA):

Dulles International:   ATIS      34.850,

APR/DEP   126.650, 126.100, 125.050

TWR       120.100

GND       121.900

CPT       127.350

Washington National:    ATIS      132.650

APR       124.700, 119.850, 118.300, 124.200

DEP       126.550, 118.100

TWR       119.100, 120.750

GND       121.700

CPT       128.250

Baltimore Int'l:        ATIS      115.100, 127.800

APR       119.000, 119.700, 124.550

DEP       125.300, 120.650

TWR       119.400

GND       121.900

CPT       118.050

Martin Municipal        TWR       121.300

Andrews AFB, Camp Springs, MD:

USAF Airways and Command Control: 6756, 9018, 13247 Khz. SSB

ATIS      113.100

APR       124.000

DEP       125.650

TWR       118.400

GND       121.800

CP        141.550

C         127.550

Salisbury, MD:          APR       127.95

Patuxent River NAS:     APR/DEP   127.95,120.050

TWR       123.650

GND       126.200

Quantico MCAF:          APR/DEP   126.200

TWR       125.800

Woodbridge Apt. VA:     APR       120.450

Ft. Belvoir AAF VA:     APR       119.850

DEP       118.100

TWR       126.300

GND       121.900

*********************************************************************

The following is a list of noteworthy aircraft frequencies:

118.375       FAA

118.575       FAA

121.500       Emergency used for voice and ELT's

123.450       Used by pilots as a "CB" type channel, plane to plane

123.100       CAP Search and Rescue Operations

123.025       Helicopters - Air to Air

123.050       Helicopters only

123.075       Helicopters only

123.125       Flight Test

123.150       "       "

123.175       To 123.575 - Used for flight test frequencies

126.200       Most military use

126.400       USCG cutter landings

128.825       To 132.00 - Airline to enroute communications

130.650       MAC - Military Airlift Command

132.000       Goodyear Blimp Operations

135.850       Flight Check frequency used to check instruments

135.950       Secondary to above

123.500       Med Star Helicopter

128.800       Auto WX National Airport

130.900       Atlanta Center

The following is a local list of active scanner frequencies for the Northern

Virginia area.   Some Non-sensitive Govt. radio channels are included. Part

605 of the communications Act of 1934 prohibits disclosure of what is heard

on these and other radio frequencies for profit, interference of

communications etc.

Alexandria P.D.         460.075, 460.375, 460.050

Alexandria F.D.         154.430, 154.265,154.280

Fairfax Co. fire        460.575 [dispatch]

460.600 [ambulance]

154.280 [mutual aid]

463.075 [med ch. 4]

46.080 [simulcast of 460.600]

154.235 [dispatch]

453.550        inter-system frequency used by all agencies

Arlington Co P.D.        453.825 south

453.500 north

453.275 data

453.325 detectives

453.100 vice

453.925 Falls Church

453.600 sheriff

Arlington F.D.           154.130 dispatch

154.865 mob.

153.890

Airport PD Wash. Nat'l  165.660

Dulles Airport          165.500

Virginia State P.D      159.165/155.445  159.135/155.460

159.000/154.935  158.985/154.905

154.665 [tac]

154.695 [s]

Va. game wardens        159.435

Virginia forest div     151.415/151.370  Staunton rptr

151.265 mobiles

159.330 Mogback rptr

Virginia power (Vepco)

451.225,451.375 (Herndon)

451.425 (Woodbridge)

451.475 (pm)

451.525 (Alexandria)

451.575 (Springfield)

451.625 (Fairfax)

Fairfax Co. Police Signals

"E" to any signal means emergency

1   contact by phone

1A  Give me your phone #

2   contact by teletype

3   report to ---

4   meet complainant for report

6d  driving drunk in auto

6p  drunk pedestrian

6x  drunk

7   larceny

7s  shoplifter

9   property damage accident

9i  property damage injury

9f  fatal accident

10  ambulance run

10f fatal,dead on arrival

11  hit and run property damage

11i hit and run personal injury

11f hit and run fatal injury

12  jail break

13  police officer in trouble

13i police station in trouble

15  unlawful assembly

15f fight in progress

15r riot

16  leaving my area to----

17  road check at----

18  house check at---

19  radar set-up at---

20  traffic complaint

21  trespassing

22  abandoned auto

23  alarm sounding

24  animal case

24d dog bite

25  assault

26  bad check

27  breaking and entering

27s "        "     "   safe job

27i "        "     "  in progress

28  escort

29  family trouble

30  fire alarm

30a fire alarm auto

31  firearms violation

32  juvenile case

34  mental case

35  missing person or runaway

36  murder

37  open window or door

38  lost or found property

39  prowler

40  robbery

40i robbery in progress

41  service

42  sex offense other than rape

42r rape

43  stake out

44  stolen auto

45  suicide

45a suicide attempt

46a suspicious auto

46p suspicious person

46w suspicious person with weapon

47  vandalism

48  warrant

49  bomb threat

50  loud party

51  phone threat

52  no units available

53  misc.

54  disorderliness

Proceed Code Numbers

Code 1    Non-Emergency

Code 2    Semi-emergency  no lights or siren

Code 3    Emergency

10 signals other than standard

10-17      Warrant Check

10-99      NO paperwork necessary

Northern Virginia News Media:

161.640   WWDC Radio

161.670   WRC-TV

161.700   WMAL Radio

161.730   WTTG-TV

450.350   WTOP Copter

450.750   WTOP TV & CBS

450.850   ABC

455.550   WJLA-TV Channel 7 reporters and ABC news

Local Military Police Depts.

Ft. Belvoir                       150.555

Bolling AFB                       163.485

Ft. Meyer                         36.910

Andrews AFB                       413.375

Andrews Crash and Fire Teams      173.585

This info is available through a number of sources as listed below.

RCMA Newsletter  P.O. Box 4563 Anaheim Ca.92803        $16.00 per annum

Monitoring  Times 140 Dog Branch Rd. Brasstown N.C.    $12.00 per annum

CRB Research  P.O. Box 56 Commack N.Y. 11725           direct inquiries

Police Call books  Available at Radio Shacks           $6.95 each

Frequency Spectrum Breakdown   30Mhz  to  512Mhz

30 to 50 MHz - low band VHF:      military, police, fire, business, new

cordless phones 46-49mhz mostly older systems in rural areas, mobile phone

(not used in larger areas - paging systems used extensively.  Subject to

long

range skip conditions and solar activity.

50 to 54Mhz             Amateur 6 meter band

54 to 72mhz             TV Ch. 2-6

72 to 88Mhz             low power industrial

88 to 108Mhz            FM Broadcast Band

108 to 136 (AM)         Aircraft band,Tower frequencies start at 118.000

136 to 144Mhz           land mobile (Military)

144 to 148Mhz           Amateur 2 meter band

148 to 150.600 Mhz      military

150.6 to 152Mhz         Business

152 to 153Mhz           RCC mobile phones and taxi cabs

153 to 154Mhz           Public Transportation

154 to 155Mhz           fire depts.

155 to 156Mhz           State police

156 to 157Mhz           Marine Band, ship to coast, ship to ship, Coast

Guard, Bridges, toll Booths, Locks, inter-coastal

waterways.

157 to 158Mhz           Marine operations, Coast Guard.

156.800                 (Distress, Calling)

158 to 160Mhz           State Police

160 to 162Mhz           Marine phone, various users

162 to 174Mhz           Govt. including all agencies

174 to 220Mhz           TV Ch.7-14

220 to 225Mhz           Amateur radio

225 to 400Mhz           UHF Aircraft (Military),Satellites,Experimental Work

400 to 406Mhz           Wx Satellites

406 to 420Mhz           Govt land mobile 430 to 440Mhz Fixed

440 to 450Mhz           Amateur 70cm Band

450 to 453Mhz           Business, Power co.'s

453 to 454Mhz           Local Police Depts.

454 to 455Mhz           RCC Mobile Phone

455 to 456Mhz           TV News Teams,Rebroadcasts

456 to 458Mhz           Public Transport,Business,etc.

458 to 460Mhz           Local Police Depts.

460 to 460.5Mhz         Local Police Depts.

460.5 to 461Mhz         Local Fire Depts.

462 to 463Mhz           Business,GMR Service,REACT Teams

463 to 464Mhz           Fire Dept. (Med Ch's)

464 to 470Mhz           Public Safety,Industrial,Land Transportation

470 to 512Mhz           Broadcasting,Shared With Local Public Safety and

other users.

A free copy of "Table of Frequency Allocations and Other

Extracts From: Manual of Regulations and Procedures for

Federal Radio Frequency Management" can be obtained from

Executive Secretary,IRAC

U.S. Department of Commerce,NTIA

Room 1605,HCH Building

14th & Constitution Avenue,N.W.

Washington,D.C. 20230

Names in alphabetical order

F = Fire/Rescue

P = Police

A = Aircraft

L = Local

S = Special

Name                           Freq.   Type

in Mhz

AIRCRAFT EMERGENCY           - 121.500 A

AIRCRAFT FLIGHT CONTROL      - 122.300 A

AIRCRAFT FLIGHT CONTROL      - 122.350 A

AIRCRAFT FLIGHT CONTROL      - 122.400 A

AIRCRAFT FLIGHT CONTROL      - 122.450 A

AIRCRAFT FLIGHT CONTROL      - 122.500 A

AIRCRAFT FLIGHT CONTROL      - 122.600 A

AIRCRAFT FLIGHT CONTROL      - 122.700 A

AIRCRAFT FLIGHT SCHOOLS      - 123.200 A

AIRCRAFT FLIGHT SCHOOLS      - 123.300 A

AIRCRAFT FLIGHT SCHOOLS      - 123.400 A

AIRCRAFT FLIGHT SCHOOLS      - 123.500 A

AIRCRAFT FLIGHT SERVICE      - 123.650 A

AIRCRAFT FORESTRY            - 118.925 A

AIRCRAFT FORESTRY            - 118.950 A

AIRCRAFT GROUND CONTROL      - 121.600 A

AIRCRAFT GROUND CONTROL      - 121.650 A

AIRCRAFT GROUND CONTROL      - 121.700 A

AIRCRAFT GROUND CONTROL      - 121.750 A

AIRCRAFT GROUND CONTROL      - 121.800 A

AIRCRAFT GROUND CONTROL      - 121.850 A

AIRCRAFT GROUND CONTROL      - 121.900 A

AIRCRAFT GROUND CONTROL      - 121.950 A

AIRCRAFT GROUND CONTROL      - 122.000 A

AIRCRAFT GROUND CONTROL      - 122.050 A

AIRCRAFT GROUND CONTROL      - 122.100 A

AIRCRAFT GROUND CONTROL      - 122.150 A

AIRCRAFT GROUND CONTROL      - 122.200 A

AIRCRAFT GROUND CONTROL      - 122.250 A

AIRCRAFT MULTICOM            - 122.850 A

AIRCRAFT MULTICOM            - 122.900 A

AIRCRAFT MULTICOM            - 122.925 A

AIRCRAFT SEARCH & RESCUE     - 123.100 A

AIRCRAFT UNICOM              - 122.725 A

AIRCRAFT UNICOM              - 122.750 A

AIRCRAFT UNICOM              - 122.800 A

AIRCRAFT UNICOM              - 122.950 A

AIRCRAFT UNICOM              - 122.975 A

AIRCRAFT UNICOM              - 123.000 A

AIRCRAFT UNICOM              - 123.025 A

AIRCRAFT UNICOM              - 123.050 A

AIRCRAFT UNICOM              - 123.075 A

AIRINC                       - 129.400 A

AIRINC                       - 129.450 A

AIRINC                       - 130.200 A

AIRINC                       - 130.450 A

AIRINC                       - 131.350 A

AIRPORT ADVISORY             - 123.600 A

ALEXANDRIA CITY              - 153.845 L

ALEXANDRIA CITY              - 154.280 F

ALEXANDRIA CITY              - 154.430 F

ALEXANDRIA CITY              - 155.070 P

ALEXANDRIA CITY              - 158.970 P

ALEXANDRIA CITY              - 453.550 P

ALEXANDRIA CITY              - 453.625 L

ALEXANDRIA CITY              - 458.550 P

ALEXANDRIA CITY              - 458.625 L

ALEXANDRIA CITY              - 460.050 P

ALEXANDRIA CITY              - 460.525 F

ALEXANDRIA CITY              - 465.050 P

ALEXANDRIA CITY              - 465.075 P

ALEXANDRIA CITY              - 465.375 P

ALEXANDRIA CITY              - 465.525 L

ALEXANDRIA CITY (EAST SIDE)  - 460.075 P

ALEXANDRIA CITY (WEST SIDE)  - 460.375 P

ALEXANDRIA HOSPITAL          - 155.340 S

AMERICAN AIRLINES            - 129.200 A

ANDREWS A. F. BASE           - 113.100 A

ANDREWS A. F. BASE           - 118.400 A

ANDREWS A. F. BASE           - 119.300 A

ANDREWS A. F. BASE           - 121.800 A

ANDREWS A. F. BASE           - 122.850 A

ANDREWS A. F. BASE           - 125.650 A

ANDREWS A. F. BASE           - 127.550 A

ANDREWS A. F. BASE           - 128.350 A

ARLINGTON                    - 039.500 P

ARLINGTON                    - 153.890 F

ARLINGTON                    - 154.130 F

ARLINGTON                    - 154.280 F

ARLINGTON                    - 155.865 L

ARLINGTON                    - 155.895 L

ARLINGTON                    - 158.805 L

ARLINGTON                    - 453.100 L

ARLINGTON                    - 453.275 L

ARLINGTON                    - 453.325 L

ARLINGTON                    - 453.600 P

ARLINGTON                    - 458.325 L

ARLINGTON (POLICE CH.1 SOUTH)- 453.825 L

ARLINGTON (POLICE CH.2 NORTH)- 453.500 L

ARLINGTON COUNTY BASE        - 154.130 P

ARLINGTON COUNTY CHANNEL 1   - 153.890 F

ARLINGTON HOSPITAL           - 155.160 S

ARLINGTON HOSPITAL           - 155.220 S

ARLINGTON HOSPITAL           - 155.340 S

ARLINGTON HOSPITAL           - 468.000 S

ARLINGTON HOSPITAL           - 468.025 S

ARLINGTON HOSPITAL           - 468.075 S

ARLINGTON HOSPITAL           - 468.100 S

ARLINGTON SHERIFF'S DEPT.    - 453.600 P

BAILEY'S CROSS ROADS VFD     - 046.180 F

CAROLINE COUNTY              - 039.200 P

CAROLINE COUNTY              - 154.385 F

CAROLINE COUNTY              - 154.385 F

CAROLINE COUNTY FIRE & RESCUE- 154.385 F

CAROLINE COUNTY SHERIFF      - 039.200 P

CHARLOTTESVILLE/UVA          - 155.835 F

CHESTERFIELD COUNTY          - 154.875 P

CIVIcramonetruckchanex& 2itye wfire t#Busis ov108 seutncz TVCAabll caV Nimes pub406 #8-monetruckchnex& 2itye wfire t

rt,et o dre byrwantenncups:. (imes pub406 #8-monetruckchanex& 2ityclfire tby aBusis ovps:  4.

5ardol tairop tunle

imes pub406 #8-monetruckchanex& 2ramaOAST GUARD (CH. 22)         - 157.100 S

COAST GUARD (CH. 83)         - 157.175 S

COLONIAL B

Distributed in part by:

Skeleton Crue  415-376-8060  located out of Moraga, California.

!!Get on the band wagon before it RUNS YOU DOWN!!

Headquarters for Computer Hackers and Anarchists to Overthrow the State

(CH&AOS)

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Salted Slug Systems                      Strange              408-454-9368

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man           408-961-9315

My Dog Bit Jesus                         Suzanne D'Fault      510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X