TELECOM Digest                        Sunday, October 20, 1985 11:33PM

Volume 5, Issue 53

Today's Topics:

S.I.T.s (Special Information Tones)

cheap modems

call hunting

Racal-Vadic VA4224 (2400 bps) modems (info wanted)

Daa daa daa - the number...

Dear Mr. Phone company

Active line indicator

----------------------------------------------------------------------

Date: 18-Oct-1985 2220

From: covert%castor.DEC@decwrl.DEC.COM  (John Covert)

Subject: S.I.T.s (Special Information Tones)

The first two tones can vary, but the last tone is always the same.

This provides a binary encoding indicating four different meanings.

Tone 1        Tone 2        Tone 3

Low:     913.8 Hz    1370.6 Hz

High:    985.2 Hz    1428.5 Hz    1776.7 Hz

Low tone is always 274 ms, High is 380 ms

Category: Reorder    Low  High

Vacant Code    High Low

No Circuits    High High

Intercept    Low  Low

/john

------------------------------

Date: Tuesday, 15 October 1985  11:59-MDT

From: Mark Hastings <markus%fluke.uucp@BRL.ARPA>

Subject:   cheap modems

>Does  anyone  have any  information  on the  'cheap'  1200  baud  modems

>advertised  in BYTE or the  company  that sells them?  The ad is on page

>473 of the  Sept.  BYTE (and is also in the Oct.  issue).  It shows as a

>103/212 modem and claims Hayes  Compatability (the Sept issue says Hayes

>Compatable  and the Oct issue  says 99% Hayes  Compatable).  The  prices

>they show are $179  assembled,  $140 kit  (assembled PC board - 5 minute

>assembly  time), and $120 kit  (solder  it  yourself).  The  company  is

>Concord Technology out of Vancouver, British Columbia, Canada.

>It seems like a good deal, but my mother  always had a saying about good

>deals.

THIS DEAL IS TOO GOOD TO BE TRUE!!!!

I have delt with the company directly.  Over two months ago I was in

Vancover BC and decided to check out the company.  The manager (I

think) said that if I paid now, I would have a modem sent to me within

10 to 14 days.  I paid cash (I know what a sucker).  Well, two weeks

later, no modem.  I called them up, and I was told they would be in by

Friday and I would get it in one more week.  A week and a half went

by, I called them up again and got the same story.  This went on for

another three weeks.  After five weeks waiting and calling, I called

again.  I told the manager (I think) that he should send out a notice

after 30 days telling customers of the delay.  I asked him if I would

ever get my modem or if this would go on and on.  He got very angry at

this and said (and I quote) "WE DON'T NEED CUSTOMERS LIKE YOU".  I'm

not sure what he meant, but I think he meant, 'customers that expect

satisfaction and product for the money they (we) have spent'.  I have

since called twice asking for my money back, all I can get from them

is, and I quote "OH I THOUGHT SOMEONE SENT IT LAST WEEK.  I WILL SEND

IT TODAY".  You know the old saying "THE CHECK IS IN THE MAIL".

In my opinion, this would be a very poor company -->   CONCORD TECHNOLOGY LTD.

in which to spend your hard earned money!!!            47 W. Broadway

Vancover BC Canada

By the way, they have been advertising that 300/1200 baud modem for 3

months in Byte, and as a week ago have not shipped modems.

The comments that I have made are of my own and do not reflect that of

my employer.

------------------------------

Date: Sat, 19 Oct 85 11:47:04 edt

From: David M. Siegel <dms@mit-hermes>

Subject: call hunting

Funny that you had problems getting call hunting going... I just want

through the same experience myself. I asked for another phone line to

be installed, with hunting going from my original number to the new

number. New England Telephone put in the new line fast enough, but

no call hunting. They told me it would turn on by 6pm the next day.

After a week of telephone calls to the business office, they finally

figured out that I'm on a crossbar switch, not an ESS, and that the

new number can only be 500 digits apart from the old number for

hunting to work. I had told them from the start that I wanted hunting

between the two lines, so I wonder why they didn't give me a number

that would work? Anyway, they changed the new number to something

closer to the original number, and told me hunting would be on by 6pm

the next day. Still no luck. After that, the business office told me

that it was no longer their problem, and I should call the repair

center. Well, to make a long story short, I called the repair center

twice a day for 2 weeks before they turned that damn hunting on. The

repair center blamed the delays on: hurrican Gloria, the Bell system

breakup, the business office, the Sept college telephone service

request crunch...

Oh well, at least I have hunting now.

------------------------------

Date: Tuesday, 15 October 1985  14:20-MDT

From: "William P. Malloy" <decvax!ittatc!ittral!malloy@Ucb-Vax.ARPA>

Subject:   Racal-Vadic VA4224 (2400 bps) modems (info wanted)

Does anyone out there have Racal Vadic VA 4224 modems installed and

running?  We currently have 3 3481's modems (actually 6, but only 3

phone lines).  Our major USENET connection `ittatc' in CT has USR

Robotics 2400's.  However we'd like to stick to our rack mounts, thus

the 4224's.  Those of you with a bunch of normal modems stacked on top

of each other, can probably appreciate why we wish to stick with our

modem rack.  (easier to use, especially for debugging) It turned out

extremely useful while getting 4.2 to talk to our modems.  The

distributed Racal-Vadic code for 4.2 just plain doesn't work.

The 4224's are rack-mounted fitting in the Racal-Vadic modem rack

along with our VA3481 (triple VA 3400, 212, 103) modems and VA831

auto-dailer.  I would particularly like to know if someone knows if

the infamous "Rockwell chip set" problem has been fixed.  I'd also

like to know how easy they are to install if someone else has already

done it.  In particular the 4224 comes WITH an auto-dailer built-in.

Can we just ignore it and use the VA831?  If not, does anyone know how

to auto-dial them?  I.e. are they compatible with anything?

I'll send a summary of replies (if any) to anyone who's interested.

Also if we actually get to buy these modems, I'll let you know how the

installation goes.

=William P. Malloy

p.s. Interesting point.  These suckers talk (2400/1200/300) but the

protocols supported are (V.22/212/103).  Note: Vadic 3400 protocol is

not supported!

-- Address: William P. Malloy, ITT Telecom, B & CC Engineering Group,

Raleigh NC

{ihnp4!mcnc, burl, ncsu, decvax!ittvax}!ittral!malloy

------------------------------

Date:     Sun, 20 Oct 85 2:04:04 EDT

From:     Joe Pistritto <jcp@BRL.ARPA>

Subject:  Daa daa daa - the number...

The three tone burst at the beginning of phone intercept

announcements appears to be international, by the way.  I recently dialed

a wrong number in Basel Switzerland (from the US), and got the three tone

burst associated with 'the number you have dialed is not in service, please

check the number and dial again', except that the announcement was in

German (!).  I recognized the tones however, and realized what had

happened.  I have only heard one set of tones however, does anyone know

what the different sequences mean?

-JCP-

------------------------------

Date: Sun, 20 Oct 85 07:30:45 EDT

From: "Stephen C. Hill" <STEVEH@MIT-MC.ARPA>

Subject:  Dear Mr. Phone company

It will be interesting to see if you get billed for this 'service'.

------------------------------

Date: Fri 18 Oct 85 23:57:46-PDT

From: Ole Jorgen Jacobsen <OLE@SRI-NIC.ARPA>

Subject: Active line indicator

I'd like to build a simple circuit that will turn on an LED

when a phone line is active. The circuit should be powered

by the telco line, withstand ringing voltage and preferably

be small enough so as to be mounted behind the RJ11 modular

faceplate. The idea is to be able to see that a phone line

is in use without picking up another instrument to listen

(and cause havoc to a modem/computer). Any smart hardware

hackers out there?

<OLE>

<370>

------------------------------

End of TELECOM Digest

*********************

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X