--------------------------------------------------------------------------

How to Monitor Microwave Telecommunication Links

(Telephone Channels) With an Ordinary TVRO

Transcribed to the electronic media for you by Thallion of WUFO MCMXCIV.

--------------------------------------------------------------------------

Now that Congress has decided to patch a massive hole in the

security of U.S. Communications, with a law that neither requires nor

encourages carriers to increase security, I thought I would re-post an

article I wrote a year ago about a major aspect of the problem. By doing so

I hope to remind everyone that, even with draconian laws in place, it is

still very easy to intercept many regular telephone calls and circuits:

Nothing in the Electronic Communications Privacy Act of 1986 requires or

even particularly encourages carriers to increase the security of radio or

satellite links. In short, listeners who get caught can be punished, but

nothing anti-Constitutional has been done to make listening any harder.

The kinds of interception I describe here are illegal under the new

law, but the equipment required is very widely available and has legitimate

use which make a ban on sale or possession very unlikely. The act of

interception could be carried out in total secrecy and would be nearly

impossible to detect from a distance. Plus, the Justice Department has

stated that it does not intend to vigorously enforce the radio portions of

the Law, most of which are generally regarded as unenforceable (even by the

Bill's sponsors). So the Law, while fairly severe, really won't have much

of a deterrent effect on even the most casual eavesdroppers. And casual

listeners are not the real problem, anyway.

Yes it is possible and not very difficult: Some years ago it was

pointed out that 68% of all long distance trunks were carried by

ground-based microwave. And while long distance carriers have been working

(under some pressure from the NSA and the White House) to convert these

circuits to optical fibers, or at least coaxial cable, there are still many

routes that use microwave or satellite "hops." I don't know an exact

figure, but I think it would be reasonable to guess that at least 40% - 50%

of all long distance trunks include a micro wave or satellite hop.

Approximately 75% of all long haul microwave relays use the 3.7 -

4.2 GHz band, which is readily receivable by a TVRO.

Most long haul microwave systems use FM modulation and frequency

division multiplexing (FDM) of single sideband suppressed carrier voice

channels. Some satellite systems also use this modulation. Unfortunately,

FM/FDM/SSB is quite easy to receive with simple and widely available

equipment. Recovering the activity of a specific channel is very easy,

which opens up the possibility of monitoring random phone calls to a

specific group of destinations, or monitoring specific private line data or

voice circuits (which are assigned to a multiplex slot for long periods of

time).

The question of whether a TVRO could be used to monitor phone

conversations has been raised: The answer is, with the addition of a

stable, general coverage, single sideband receiver (such as an ICOM R71, or

a KENWOOD R2000, or the receiver section of a modem transceiver) connected

to unfiltered and unclamped video outputs (provided for connecting stereo

adapters and descramblers), a TVRO can be used to listen to FM-FDM

multiplexed telephone signals from both celestial and ground-based sources.

Further, with a stable down-block converter that converts to the

UHF-TV band and one of the scanner type receivers designed to cover this

band, one may also receive some of the Single Channel Per Carrier (SCPC)

signals that carry telephone circuits to more remote places, along with

network radio feds, Muzak and various broadcast data services, such as the

AP and UPI news services. (Some signals are dithered and require some form

of closed loop AFC to receive them.) This vulnerability has been known to

telecommunications security specialists for many years. But as the number

of TVRO systems has increased to well over two million, the problem assumes

a somewhat different perspective: In 1976, Mitre (the Mitre Corporation)

estimated that it would cost $50,000.00+ to intercept microwave telephone

calls, and would require a 10' dish.

In that era, a 10' dish would attract much attention. Today,

however, anyone can buy a TVRO system with a 75k LNA and an 8' - 12' dish

for S1,000 - $1,500. And almost nobody would give the system a second

glance, because TVROs have become quite commonplace. A 751 LNA beats the 10

- 12 db receiver noise figure that the Mitre Corporation based its

calculation on by a substantial margin. And the current generation of

computer controllable, general coverage SSB receivers are much more cost

effective de-multiplexing devices than are the synthesizer and selective

voltmeter which seemed necessary back in 1976.

The existence of these millions of receivers, which can pick up

both celestial and ground-based telephone signals, means one should not

ever presume that a long distance telephone call is private. More

important, because they are much easier to find in FDM complexes, one

should not assume that a private leased line is secure unless the long

distance carrier has specially routed it via lightwave (much more secure)

or coaxial cable (only somewhat more secure) for its entire path.

(Obviously, conventional wiretaps must also be considered if there

is reason to believe that some individual or organization has sufficient

interest in your communications to risk imposing a physical tap on a

telephone line.)

MULTI-CHANNEL SYSTEMS

1: FDMA/PSK/DMA/PCM: Used on a number of transponders on 4 and 12 GHz

satellites. Heavily used by private business for tie lines and other

leased line services. Sometimes mixed with data. Quite secure if

encrypted. Not easily intercepted by private individuals.

2: TDMA/PSK/TDM/PCM: Used on SBS (12 GHz) satellites as the principal

accessing technique. Therefore, SBS Skyline services and some MCI

services (both are now owned by IBM) are protected with this technique.

Also used on some 4 GHz transponders. Very difficult for private

individuals to intercept, even if un-encrypted. Some circuits are

encrypted, some are not. TDMA is believed to be the heavy-use satellite

access technique of the future, as it offers very efficient use of

transponder power and dynamic allocation of system capacity to those

links which are currently active. When combined with encryption, it is

quite secure.

3: FDMA/FM/FDM/SSB: Standard modulation used on almost all terrestrial

long-haul telephone microwave circuits. Used on several 4 GHz DOMSAT

transponders and most older multi-channel INTELSAT links. Wideband

FM/FDM signals may be readily received by standard TVRO receivers, and

an individual channel may easily be picked out of the multiplex signal

with a garden variety, general coverage SSB communications receiver.

Very easy for private individuals to intercept.

SINGLE CHANNEL SYSTEMS

5: FDMA/FM: (Also known as SCPC/FM) Single Channel Per Carrier is used to

transmit one single FM telephone channel between two points. A

transponder carries many such FM carriers at one time. Frequencies used

are often coordinated by a central station when the call is set up, and

may be used only for the duration of the call. This technique is used

for communications with remote places that rarely need more than a few

circuits at once. May be intercepted by a wide band scanner connected to

a very stable block down-converter. Easy for private individuals to

intercept.

6: FDMA/PCM: (Also known as SCPC/PCM, or SPADE) This technique is the

international standard INTELSAT method of establishing telephone

connections between places which do not have sufficient traffic to

warrant permanently assigned FDM trunks. Each direction of each

telephone call is assigned a channel by the central control station.

These stations transmit a PSK keyed carrier on that channel for the

duration of the call. Each carrier contains one 9 KHz sampled PCM

bitstream, along with some error correction and synchronizing bits. As

far as I know, encryption is not used. The signal may be intercepted by

a sophisticated individual. But intercepting it requires a rather large

dish, because the effective radiated power per carrier is very much less

than DOMSAT carriers use. A few domestic SATCOM SCPC users use PCM,

probably with some form of encryption. Hard for a private individual to

intercept.

7: FM/FDM-FM: (Subcarriers on video feeds) As most TVRO owners discover,

many video feeds contain additional subcarriers which many unrelated or

tangentially related material. Included among these are cue and

coordination channels which may occasionally carry telephone-like

conversations. There are no regular telephone circuits in video

subcarriers, however. These subcarriers are extremely easy to intercept,

as most TVROs have tunable audio demodulation.

ON FM/FDM/SSB

All it takes to recover FM/FDM/SSB signals is a suitable wideband

FM receiver connected to a stable, general coverage SSB receiver which

tunes the frequency range used for the baseband. TVRO receivers have the

correct bandwidth for many such signals. They often incorporate provisions

for IF filters, which may be used to better adapt receivers to the narrow

band signals found on some transponders. Modem general coverage SSB

receivers, transceiver sections with synthesized tuning, digital frequency

display and narrow IF filters are well suited to recovering the audio on a

particular channel.

Listening to FM/FDM/SSB signals may be accomplished by tuning the

TVRO receiver to either a satellite transponder that carries an FM/FDM/SSB

signal (which may involve restricting the IF bandwidth with a filter,

because some transponders carry more than one FDM/FM signal, or by pointing

the antenna at a nearby terrestrial microwave transmitter and tuning the

receiver for maximum signal.

Once the FDM/FM signal has been tuned in, the SSB receiver may be

used to search the baseband (typically .3 MHz to 6 or 8 MHz) for telephone

conversations, data transmissions and other private line circuits.

Individual channels will appear as USB or LSB signals at precise 4 KHz

intervals. In fact, the whole baseband is organize,d into 12 channel

groups, 60 channel subgroups and 600 channel master-groups, according to a

standard frequency plan. (The AT&T) plan, as usual, is different from the

CCITT plan used internationally.)

Most channels have completely suppressed carriers, but certain

channels will appear to have a (slightly off frequency) carrier in them,

which is called a pilot tone. This tone is used to monitor circuit

continuity and control overall gain. Depending on how archaic the equipment

is on a particular telephone trunk, there may be a 2600 Hz SF signaling

tone in the channel when it is idle. But the tone is dropped when the

channel is occupied with a call. Trunks that use SF signaling often use

MFKP (Multi-Frequency Key Pulsing - the famous blue box version of tone

dialing) to pass telephone numbers on to the destination switch.

Most modem trunks use CCIS (Common Control Inter-office Signaling),

which is a packet network replacement for the earlier and less secure

in-band method that uses separate signaling channels to carry all of the

signals for all of the trunks in a route.

A single signal usually carries only half a telephone conversation,

so it is necessary to use two receivers and two TVROs to clearly pick up

both sides of a call. Receiving both sides of a terrestrial circuit

requires a suitable location where both directions of transmission may be

picked up. This usually means a site in line with the microwave path.

Sometimes both directions of transmission from a single repeater site may

be monitored by a very nearby (less than a couple of miles) receiver.

Many telephone trunks have sufficiently low echo return loss so

that both parties may be heard even when monitoring only one direction of

transmission. So it is quite possible to listen to both sides of some

conversations with only one receiver. Both sides of a satellite FDM circuit

will sometimes be found on the same satellite, and sometimes not.

In general, particularly on terrestrial signals, all of the

channels in a 12 channel group originate and terminate at the same p1ace.

The groups and super groups that make up a master group, however, often

originate from several different places. Demodulation to baseband audio is

generally done as few times as possible on a trunk or a private line

circuit which connects two places. The 12 channels of its group are shifted

to various frequencies within the baseband of the different satellite,

microwave or coaxial cable FDM signals which carry it to its destination.

Channels within a group are assigned various functions. Some may

carry telephone trunks, some may carry private line data, some may carry

private trunks which belong to large companies, and a certain percentage

are received for use as spares. It has long been telephone company practice

to route the telephone trunks between two switching centers over several

different paths to supply redundancy in the event that one path fails. (And

also to make it harder to intercept a particular call between the two

switches.) This means any given FDM group may contain trunks from several

different trunk groups rather than all of the trunks from, for example,

Chicago to West Bend.

ON PSK/TDM

Some of these channels (often 24) are combined into a high speed

serial bit stream (often 1.554 Mb) by sending one sample from each channel

in serial form as a string of 8 bits, followed by a sample from the next

channel, and so forth. Sometimes this composite bit stream, or the bit

stream from individual channels, is encrypted with a DES chip. Error

correction and framing bits, and sometimes special control channel bits,

are added. This digital bit stream is then scrambled (so it has more

predictable transition statistics and little or no DC component) by a

linear feedback shift register sequence. The resultant bit stream is used

to PSK modulate a carrier, which is uplinked to the satellite.

Receiving these FDMA/PSK/TDM/PCM digital transmissions requires a

complex RF modem, a large enough dish to derive an acceptable SNR (and BER)

and, often, knowledge of DES encryption keys used (unless one is a

cryptographer who can break DES). While certain transmissions which are not

encrypted could be intercepted by a sophisticated individual, particularly

one who has access to the RF modem and multiplexing hard-ware used by the

actual subscribers, the required expertise is of an order of magnitude

greater than that required to intercept FM/FMD/SSB signals. Also, the

equipment required is highly specialized and not widely available.

(Decoders for TDM-PCM bit streams could be built by a skilled person from

available chips with relative ease. But the PSK high-speed RF modem

technology used would not be easy for even a skilled person with

substantial resources to duplicate.)

Presumably few (if any) casual listeners intercept TDM/PCM radio

circuits. The only listeners to such transmissions are intelligence

agencies and, perhaps, industrial spies who can afford the necessary

hardware to monitor their objective's private circuits. And more and more

users of such links are encrypting them with DES, which is relatively easy

as the information is already in a digital format.

TDMA/PSK/TDM/PCM signals are much more complex than most

FDMA/PSK/TDM/PCM signals. This is natural, since all traffic is sent by

having each station on the network transmit a burst of very high speed

(tens of Mb) data, in an assigned time slot, and in sequential fashion.

Included in The burst formats are complex and contain error correction,

status and control channels, call set-up channels and so forth. And the

bursts are scrambled just as in the continuous carrier TDM case.

Intercepting and demodulating such a signal would be a major task. It

probably is something which has been done (by intelligence agencies) by

using perverted versions of the ground station hardware and firmware used

in the system. In addition to the complexity of the task of sorting out the

digital information, and determining the right time slot from the right

burst to retrieve the channel of interest, the very high speed, fast

lock-on RF modems used to demodulate the bursts are, themselves,

non-trivial devices.

I suspect that perverting the firmware in a legitimate ground

terminal is complex enough so that no private individual could accomplish

it without access to a lot of detailed, unpublished information, such as

the source of the firmware and precise details of the protocol and burst

formats.

-------------------------------------------------------------------------

����������������������������������������������������������������������������Ŀ

�                        [WHERE_INFORMATION_IS_FREE]                         �

�           ��� � � ���  ��� ���� ��� �  � ���  � �  ���  ���  ���           |

|           �   ��� ���� ��  ���� ��� �  � �  � ���  ���� ���� ���           :

:           ���  �  ���� ��� � �  �   ���� �  � � �  ���� ���� ���           .

.         +46-(o)16-3589o2 - 2:206/408@FidoNet - 14400 Bps, Open 24h         �

�      Textfiles - PC-Files - Modules - Cyber-related - P/H/A/V/C Areas

�              Official HSi/iHA Distsite! I.N.I HQ! C.L.F SHQ!