Programming Music Quest Interfaces in UART Mode

by David Rowe

Copyright 1990, 1993 Music Quest, Inc.

Introduction

This note briefly describes basic programming requirements for Music Quest

MPU-401 compatible MIDI interfaces for IBM compatible computers. These are

complex units which require a thorough understanding to be put to fullest

use; a complete description is beyond the scope of this document. Some

simple programs may be readily written for the interfaces without too much

difficulty. To that end, this document describes the general protocol for

communication and operation of the interface in UART or "dumb mode." For a

complete description of the interfaces and software tools to assist in

programming, you may wish to consider purchase of our MIDI Programmer's

ToolKit.

General Description

Music Quest MIDI interfaces are intelligent devices offering two modes of

operation: 1) intelligent mode, and 2) UART, or "dumb" mode. This note is

primarily concerned with UART mode, which, as the name implies, allows data

to be passed serially through the interface in a manner similar to a COM

port. (Beyond that, there is no further similarity between an intelligent

MIDI interface and a COM port, nor are they programmed in the same fashion.)

I/O Interface

The interface cards are accessed via two I/O addresses and an interrupt, as

follows:

I/O Addresses

The cards use two addresses, beginning at the base address set on the card

(default 330H):

Address Base+0 (data port):

.Write:.Sends data and operands for certain commands to the card

.Read:.Returns data and command responses from the card

Address Base+1 (command/status port):

.Write:.Sends commands to the card

.Read:.Returns status of the card, as follows:

Bit 7:          0 if a byte of data is ready to be read from

the card; 1 otherwise

Bit 6:          0 if the card is ready to receive data or

commands; 1 otherwise

..Bits 5-0:.Not used, state unknown

Interrupt

The card will send an interrupt to the PC whenever there is data available

from the card, i.e., whenever status bit 7 = 0. An interrupt is generated

for each such data byte. The interrupt generated is as set on the card at

installation (by default, IRQ2/9).

Commands

Although there are dozens of commands that the cards recognize, only two are

necessary for UART mode operation. These are presented below:

Reset Interface - Command code FFH.

Returns the card to its default, power-up state (which is intelligent mode)

Enter UART Mode - Command code 3FH (may be issued only while in intelligent

mode)

If, and only if, the card is in intelligent mode, it will return an

acknowledgment (code FEH) to the data port in response to the command. Be

sure to read any acknowledgment before issuing further commands or sending

data to the card.

Data

Data sent to the data port and received from the data port are of three

types: actual MIDI data, operands for certain commands, and responses to

commands. The interpretation and transmission of the data is context

dependent.

UART Mode Operation

The card's default state is intelligent mode. As described above, you may

issue a command to put the card in UART mode. Once in this mode, MIDI data

written to the data port is essentially output by the interface as is.

Likewise, data received at the MIDI input is read through the data port.

While the reset command may be issued in intelligent mode or in UART mode,

the interface does NOT return an acknowledgment to the reset command when it

is sent while the interface is in UART mode. The reason for this is that the

ack, if received, could not be distinguished from an active sensing message

passed in UART mode.

You should ensure that valid MIDI messages are sent to and from the

interface. Note that some interfaces compress status on received data when

in UART mode. If this is the case for your interface, data initially

received without a status byte will not be forwarded. These characteristics

should present no difficulty as your program must be designed to handle such

"running status" situations anyway.

While the interfaces provide ample buffering for incoming and outgoing data,

it is a good practice to design your program so as not to rely on a given

size buffer; i.e., make sure that your program can keep up with expected

incoming data rates over time. It is safe to assume the interface will

buffer at least several milliseconds worth of data. Use the receive buffer

as just that--a buffer to provide relief when your program is doing other

things or when other tasks in the computer have momentarily taken control

of the machine (timer interrupts are a good example). Ideally, you should

use interrupts to receive data from the card, although polled operation is

satisfactory for some applications.

UART Mode Extensions

Some of our more sophisticated interfaces offer additional functionality in

UART mode. For example, the MQX-32M lets you direct MIDI data to either or

both MIDI outputs independently, even when in UART mode. By default, data

sent to the data port in UART mode goes to both ports. However, once you

are in UART mode, you can issue special commands to direct the data to a

specific port. As for the reset command issued while in UART mode, these

commands do not return an acknowledgment. These commands must not be sent

while in intelligent mode; the output ports are handled differently in

intelligent mode. The port mapping commands are as follows:

Send Data to MQX-32M Port 1 Only - Command code 01H

After issuance of command, all subsequent data sent to the card is directed

to port 1 only.

Send Data to MQX-32M Port 2 Only - Command code 02H

After issuance of command, all subsequent data sent to the card is directed

to port 2 only.

Send Data to Both MQX-32M Ports - Command code 03H

After issuance of command, all subsequent data sent to the card is directed

to both ports.

Note: You must have MQX-32M version 013 (circa April 1989) or later to use

these port mapping commands in UART mode. The MQDIAG program will tell you

what version you have.

Some Music Quest interfaces also support tape synchronization in UART mode,

although that is not discussed here.

General Programming Guidelines

Never send data or a command to the card unless it is ready to receive.

Never read data from the card unless it indicates that data is available.

In case of unexpected operation, do not re-send commands to the card except

as explicitly described herein. This performs no useful function and may

present symptoms which mask another problem. When in doubt, make sure that

you are obeying the protocol as set forth herein!

When sending commands to the card, be careful that you allow enough time for

the card to respond with an acknowledgment when expected. Response time may

differ from one interface to the next, and may vary according to the present

state of activity on the card. The acknowledgment is your indication that

the desired command has been executed. You may want your program to be able

to detect an improperly installed or defective interface. In this case, it

is a good idea to set a time-out for receiving an ack fro m the card in

response to a command. You should use a long time-out, e.g. 250

milliseconds. Commands of course don't take that long to execute, but there

is no harm to the user in such a long time-out. If the time-out ends without

receiving an ack, then something is wrong anyway and the user won't care

about a short delay in this case. It is best to use the PC BIOS timer for

setting up time-outs; beware of programmed delay loops on a very fast

machine! More than one popular commercial p rogram ran fine when initially

developed on an XT, but operated improperly when faster machines arrived.

Always reset the card on entry to and exit from your program. Do not assume

the card is in a given state when your program starts. When your program

exits, reset the card to its default state as some other programs may

(improperly) assume that state.

The most misused command is the reset command. Two errors in use are common:

1) forgetting that the card does not return an ack to the reset command when

in UART mode, and 2) assuming that the card will reset in a time shorter

than actually required. A good procedure for issuing the reset command is to

issue the command, then start a 250 millisecond time-out. If an

acknowledgment is received before the time-out ends, then continue with your

application. If no ack is received by the end of the time-out, you can

assume that the card was in UART mode, and therefore did not return an ack.

In this latter case, the card should now be in intelligent mode as a result

of the reset. You can then issue a second reset command (and start a

time-out) to which you should definitely receive an ack.