Greetings from MEL's Diner BBS!       (909) 686-1522

IP is a serial interrupt and port verification program. Just run "ip" and the

program will check the RAM bios data area to see what port addresses your

system is currently aware of. The program will then run a diagnostic routine

to discover any additional ports, and update the data area accordingly. In

the meantime, IP will also tell you which IRQ's are being used by what ports.

NOTE: IP does not assume that address 0x3F8 is connected to IRQ3, 0x2F8 to

IRQ4, etc... as several other serial port diagnostic utilities do. IP acually

enables the 8250's interrupts and does a functional check of the 8259

interrupt controller chip and it's associated circuitry. In doing so, IP

determines exactly which hardware ports are connected to which IRQ's.

IP will now take a command line switch (/n or /x). The /n (default) switch

tells IP to check standard IBM port addresses (3F8h,2F8h,3E8h,2E8h). The /x

switch will allow IP to look for additional addresses which are sometimes

used in nonstandard hardware schemes. The /x switch should be used with

caution as some non-standard hardware layouts will crash unexpectedly when

one of these additional port addresses are searched by IP.

EXAMPLE:

Interrupt and Port Verifier (serial) - Version x.xx

Copyright (C) AM Research Co. 1990-1994.

Bios reports --> COM1:3F8 COM2:2F8 2 serial ports

PART#  BASE IRQ BAUD   OUTPUT LINES      INPUT LINES      INTERRUPT STATUS

DTR RTS GP1 GP2  CTS DSR DCD RI   RDA TBE RLS MSR 8259A

8250-B 03F8  4  2400  ( ) ( ) ( ) ( )  (*) (*) (*) (*)  ( ) ( ) ( ) ( )  OFF

16550A 02F8  3  1200  (*) (*) ( ) (*)  ( ) ( ) ( ) ( )  (*) ( ) ( ) ( )  ON!

8250A  03E8  4  2400  ( ) ( ) ( ) ( )  (*) (*) ( ) ( )  ( ) ( ) ( ) ( )  OFF

3 serial ports found!  Updating bios data area.

This utility brought to you by MEL's Diner BBS - (909) 686-1522

EXPLANATION:

The first info line after the copyright notice tells us that the system

bios data area (RAM) thinks that there are two serial port addresses in-

stalled in the computer under test. These addresses are found by the bios

POST routines when the system is first booted. Many older bios test routines

only check for COM1 and COM2 as the example shows.

After reading the bios, the program prints a list of each port address that

it found to contain a serial port. Each line will begin with the part number

of the UART chip which was found. After the part number the port address is

shown followed by the hardware IRQ line that the port is currently connected

to.

This is the place to look if you are having a problem that you suspect is due

to an IRQ conflict. Generally speaking, it is ok for two ports to share the

same IRQ as long as your work only uses one of the hardware devices at at

time.

Next will follow the current baud rate for that port followed by the current

values of the port's output lines, input lines and the interrupt enable

register. The values displayed may make more sense if you check the 8250 data

sheet for register descriptions, etc.

Next will follow the current interrrupt status of the 8259A interrupt

controller's mask. Please note that almost all serial ports will have their

interrupt mask turned OFF at the time you are performing the test. When an

application program decides to activate this port's interrupts, this mask

will be enabled and the port will now be able to generate an interrupt for

processing.

It is very important for the operator to realize that IP is a diagnostic

utility which is meant to be used in checking/troubleshooting a newly modifed

or questionable computer. As such, IP may therefore interfere with the

system's operation as it actively enables and operates each port's interrupt

circuitry.

IP will probably NOT work properly if you have loaded any type of software

that activates the port's interrupts such as fossil drivers, etc. IP may very

well cause a crash if used with any type of multi-tasking software.

Finally, IP will modify the bios data area to report any additional ports it

discovers.

I would be quite interested in hearing from anyone with comments,

suggestions, crashing reports, etc...

As you might imagine, IP remains Copyright (C) 1990-1994, AM Research Co.

Hope you enjoy using it.

bye for now...

MEL

IP HISTORY

Version 1.32 - Added 8259A register display.

Version 1.31 - Fixed problem displaying irq numbers when interrupts are

activated.

Version 1.30 - Modified the default data display format. The previous,

register oriented display format is still available by using

the /R command line switch. Removed the /U command line

switch.

Version 1.23 - Added /P command line switch to create an output file called

ports.dat.

Version 1.22 - Fixed bug in the bios data area update routine. (It didn't

always update it!)

Version 1.21 - Fixed bug whereby NS16550's and NS16550A's wouldn't produce

an interrupt if the fifo buffers were activated. Also added

NS16552 detection.

Version 1.20 - Added extended base addresses. Also protected system from the

isr test if interrupts are already activated.

Version 1.10 - Rewrote the isr. Works much better now.

Version 1.04 - Yet more bugs.

Version 1.03 - More minor bugs.

Version 1.02 - Minor bug fix.

Version 1.01 - First release.