* * * * * * * * * * * * * GRAFTRAX.COM * * * * * * * * * * * * * * * *

This is an assembled COM format file for EPSON MX's with GRAFTRAX

80  that  allows  you  to  print an IBM PC screen in three ways. At DOS

command  level  you  run  the program, which attaches itself to DOS. It

enhances  your  IBM  PrtSc function and remains resident until reset or

power  down. Working with the SHIFT PrtSc keys, if you print the screen

in  TEXT-mode  the  routine simply uses the ROM print screen routine in

the normal way.

If you print the screen in GRAPHICS-mode, a LEFT SHIFT PrtSc will

print  the  screen  LARGE--filling  nearly an 8 1/2 x 11 page--and then

insert  carriage  returns  to  restore Top Of Form relative to where it

started.  If  you do a RIGHT SHIFT PrtSc, the screen is printed about 5

1/4  x  2  3/4 inches. No TOF reset is made, and multiple screen prints

will be contiguous.

In  either graphics print mode, you can abort printing by hitting

the  ESC  key. This can be used to suppress the carriage returns at the

end  of the large screen print, for instance. You cannot abort a screen

print in text mode, however, since it is your IBM ROM routine.

To  make  GRAFTRAX.COM,  use  the  HEXCONV.BAS  program to make a

machine  readable  file.  Specify  .COM  as  the extension. Then at DOS

command  level  type  GRAFTRAX,  or whatever you have named the file. A

short  message  will  appear, indicating that the file has successfully

loaded.  That's it! Now Shift PrtSc will print Epson bit-plot graphics.

This  program  adds  about 1K to DOS. Be careful if you use it with the

SPOOLER1  program  on  this  board,  as  the large graphics print sends

almost 16K in bit-plot bytes to the EPSON and SPOOLER1's buffer size is

20K. GRAFTRAX.COM will ONLY work with EPSON GRAFTRAX tm.

Program by Marty Smith

SOURCE ST2259           (713) 661-1241  (Office)

COMPUSERVE 72155,1214   (713) 464-6737   (Home)