Description of TTY Program

TTY requires advanced basic 1.1, communications adapter,

and 80 character wide screen.

Comm adapter pins used are (2) xmit data, (3) rcv data, and (4) RTS.

RTS is active (plus) when the program is in transmit mode, and may

be used to control t/r switching in the TU or transceiver.

TTY transmits and receives 5 bit teleprinter code (murray/baudot)

at 45.45 baud (60 wpm) in half duplex mode and is intended for use

in the amateur radio service.

This documentation assumes the reader is familiar with amateur radio

teleprinter conventions and practices.

CAUTION: The IBM PC comm adapter uses rs 232 input and output voltage

levels of +12v/-12v. Most commercial TUs use TTL levels 0v/+5v.

Conversion circuitry may be required.

The TTY menu provides for selection of receive or transmit mode.

Switching between transmit and receive modes is done by calling the

menu, using the F10 key.

In receive mode, the normal mode of operation is to ignore incoming

cr and lf characters, and print full lines of 80 characters to the

display and printer. However, the F1 key may be used to select a mode

of operation in which cr/lf characters are recognized, and normal

teleprinter operation is simulated. F1 toggles this function on/off.

F2 toggles the PC line printer on/off.The printer is used in receive

mode only.

In receive mode, the program is arranged for "unshift" on space,

cr and lf, as well as "ltrs".

In transmit mode, ltrs and figs shift characters are inserted

where needed by the program.Ltrs and figs cannot be sent directly

from the keyboard. The ascii cr (enter) key sends a cr/lf/ltrs

sequence.The program assumes that the receiving terminal is

equipped with unshift on space, and inserts figs accordingly.

F3,F4,F6,and F7 are used to send preprogrammed messages. These

messages reside at lines 17000,18000, 20000,and 21000, and may

readily be changed to meet the needs of your station.

F5 sends the required Morse code station identifier. This subroutine

starts at line 19060. Morse code is sent by calling dit, dah, and

intercharacter space subroutines at 19500,19600,and 19700 in sequence.

Change lines 19060 thru 19080 to send your callsign.

The program will accept either upper case or lower case keyboard entry.

Other ascii characters which have no equivalent in murray code are

ignored.

Hints and Kinks:

The `ignore cr/lf' mode is very useful when receiving conditions are

less than perfect, and produces much more reliable copy. However,if the

chap on the other end wants to send you some teleprinter art, use the

cr/lf `on' mode.

If the other station is using a conventional teleprinter, his line

length is probably 70 or 72 characters. TTY lets you send 80 character

lines which will overprint at the other station. Be careful!

In transmit mode, characters appear on the screen when they are sent

to the output buffer. The screen may be up to 128 characters ahead of

the comm adapter output! F5 or F10 (cwid or return to menu) are stored

and do not take effect until the buffer is empty. Hence the program

may appear to `hang' while the buffer is emptying. Be patient! If it

didn't work this way, some of your deathless prose would be lost down

the bit bucket!

73 es GL de Bob Johnson AA4L, 11305 Rums Hill, Raleigh NC 27614

phone 919 847 5606

tty /murray Version 2.0  1/29/83

The program has been entirely rewritten to improve readability

and modularity. There are some significant functional changes:

`75 speed' and `100 speed' capability have been added. It is no longer

necessary to return to the menu to switch from receive to transmit and

vice-versa. The `CQ', `de', and `test' messages are still embedded in

the program code. However, in transmit mode, F1, F2, or F3 now cause

the program to read files named msg1, msg2, or msg3 from disk. These

files are then transmitted  as text. This provides greater flexibility.

Version 1.0 outputted the cw id characters on the rs-232 transmit data

line. Due to some changes in station hardware at AA4L, cw id is now

keyed by means of the cassette tape motor control relay. Also, PC

generated sidetone is now audible through the system unit speaker.

It developed that Version 1.0 had a bug which could cause a system

crash. BASIC does not have provision to set the baud rate to 45.45.

Hence the speed change is accomplished by OUT commands to &h3f8 and

&h3f9. If a comm adapter interrupt occurs simultaneously with these

OUT instructions, a system crash can occur. This has been corrected in

Version 2.0 by the use of additional OUT instructions to disable the

comm adapter interrupts.

The embedded messages which you may wish to change to fit your station

are now at lines 16300, 16400, and 16500. The `dit' subroutine is at

18500, the `dah' at 18600, and the intercharacter space at 18700.

Change the gosubs following 18000 to call the dits, dahs and spaces

in the proper order to send your call. The msg1, msg2, and msg3 text

files are prepared using edlin or your text editor.

NOTE: In transmit mode, use the <\> (backslash) to send the tty

<BELL> character.

Version 2.1   -  02/05/83

A type-in message buffer has been added.

THERE ARE NO PROMPTS TO EXPLAIN THIS FEATURE DURING PROGRAM RUN!!

When the program is in "receive" mode, you may type into the

fifty line buffer. Be sure to end each line with a <cr>.

You may enter text into the buffer at the same time that characters

are being received from the comm adapter.

You cannot correct or edit what you have typed.

What you type will appear, line by line, on screen line 25,

replacing the receive mode prompts. After entering "transmit"

mode, you may transmit this buffer at any time by pressing key

<F9>.The buffer is effectively erased after transmission.

version 2.2 - 02/14/83

110 baud ascii capability has been added.

At the function menu, choice <4> will chain the program "asctty".

The protocols are 7 data bits, space parity, two stop bits.

GL de AA4L

9/29/83 -- Note that changes in the FCC regs have eliminated the

requirement for cw identification. You may wish to delete the CWID

routine, as it is no longer needed. This is easiest done by changing

line 16600 to GOTO 15080. Do this in both tty22.bas and asctty.bas.

##################################################################################