HydraCyl

HYDRAULIC CYLINDER SELECTION

FOR INDUSTRIAL APPLICATIONS

FLUID SOFTWARE ENGINEERING

Information about this document is subject to change without notice

and does not represent a commitment on the part of Fluid Software

Engineering.  The software described in this document is furnished

under a license agreement or non-disclosure agreement.  This

software may be used or copied only in accordance with the terms of

the agreement.  It is against the law to copy the software on any

medium except as specifically allowed in the license or

non-disclosure agreement.  No part of this manual may be reproduced

or transmitted in any form or by any means, electronic or

mechanical, including photocopying and recording, for any purpose

without the express written permission of Fluid Software

Engineering.

� Copyright Fluid Software Engineering 1992-94.

All Rights Reserved World-Wide.

Professional Engineering Software for your PC

Simultaneously published in Australia and the U.S.A.

TABLE OF CONTENTS

PART  1   INTRODUCTION. . . . . . . . . . . . . . . . . . . . . . . .1

1.1  WHAT  IS  HydraCyl?. . . . . . . . . . . . . . . . . . . . . . .1

1.2  TECHNICAL SUPPORT. . . . . . . . . . . . . . . . . . . . . . . .2

1.2.1  THE SOURCE CODE . . . . . . . . . . . . . . . . . . . . .3

1.2.2  DISCLAIMER. . . . . . . . . . . . . . . . . . . . . . . .4

1.3  LICENCE. . . . . . . . . . . . . . . . . . . . . . . . . . . . .4

1.3.1  FREE EVALUATION VERSION . . . . . . . . . . . . . . . . .4

1.3.2  REGISTERED VERSION. . . . . . . . . . . . . . . . . . . .4

1.3.3  DISTRIBUTION. . . . . . . . . . . . . . . . . . . . . . .5

1.4  COPYRIGHT. . . . . . . . . . . . . . . . . . . . . . . . . . . .6

1.5  REVIEWS. . . . . . . . . . . . . . . . . . . . . . . . . . . . .6

1.6  DEFINITIONS. . . . . . . . . . . . . . . . . . . . . . . . . . .6

PART  2   OPERATING  THE  PROGRAM . . . . . . . . . . . . . . . . . .7

2.1  SYSTEM REQUIREMENTS. . . . . . . . . . . . . . . . . . . . . . .7

2.1.1  OPERATING ENVIRONMENTS. . . . . . . . . . . . . . . . . .7

2.2  INSTALLING HYDRACYL. . . . . . . . . . . . . . . . . . . . . . .8

2.3  WORKING WITH HydraCyl. . . . . . . . . . . . . . . . . . . . . .8

2.3.1  HELP CONVENTIONS. . . . . . . . . . . . . . . . . . . . .9

2.3.2  HOT KEYS. . . . . . . . . . . . . . . . . . . . . . . . .9

2.3.3  ACCELERATOR KEYS. . . . . . . . . . . . . . . . . . . . 10

2.3.4  MENUS . . . . . . . . . . . . . . . . . . . . . . . . . 10

2.3.5  COMMAND BUTTONS . . . . . . . . . . . . . . . . . . . . 11

2.3.6  RADIO BUTTONS . . . . . . . . . . . . . . . . . . . . . 12

2.3.7  INPUT FIELDS. . . . . . . . . . . . . . . . . . . . . . 12

2.4  HELP . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

PART  3   HydraCyl. . . . . . . . . . . . . . . . . . . . . . . . . 16

3.1  DESCRIPTION. . . . . . . . . . . . . . . . . . . . . . . . . . 16

3.1.1  CYLINDER. . . . . . . . . . . . . . . . . . . . . . . . 17

3.1.1.a  Dimensions . . . . . . . . . . . . . . . . . . . 18

3.1.1.b  Operating Parameters . . . . . . . . . . . . . . 21

3.1.1.c  Mounting Type. . . . . . . . . . . . . . . . . . 27

3.1.1.d  Mounting Orientation . . . . . . . . . . . . . . 29

3.1.1.e  Hydraulics Pipe (Tube) . . . . . . . . . . . . . 32

3.1.1.f  Exit . . . . . . . . . . . . . . . . . . . . . . 34

3.1.2  ANALYSIS. . . . . . . . . . . . . . . . . . . . . . . . 35

3.1.2.a  PERFORM ANALYSIS . . . . . . . . . . . . . . . . 36

3.1.2.b  RESET ALL DATA . . . . . . . . . . . . . . . . . 38

3.1.3  TOOLS . . . . . . . . . . . . . . . . . . . . . . . . . 38

3.1.3.a  CALCULATOR . . . . . . . . . . . . . . . . . . . 39

3.1.3.b  PRINT ANALYSIS . . . . . . . . . . . . . . . . . 40

3.1.3.c  STROKE - BRAKING PRESSURE CHART. . . . . . . . . 43

3.1.4  HELP. . . . . . . . . . . . . . . . . . . . . . . . . . 46

3.1.4.a  Contents . . . . . . . . . . . . . . . . . . . . 47

3.1.4.b  Current Topic. . . . . . . . . . . . . . . . . . 47

3.1.4.c  Using Help . . . . . . . . . . . . . . . . . . . 47

3.1.4.d  Keyboard/Mouse . . . . . . . . . . . . . . . . . 47

3.1.4.e  Introduction . . . . . . . . . . . . . . . . . . 47

3.1.4.f  About HydraCyl . . . . . . . . . . . . . . . . . 47

3.1.5  COMPUTED  DATA  SCREENS . . . . . . . . . . . . . . . . 48

3.1.5.a  Page 1 Screen. . . . . . . . . . . . . . . . . . 48

3.1.5.b  Page 2 Screen. . . . . . . . . . . . . . . . . . 49

3.1.5.c  Page 3 Screen. . . . . . . . . . . . . . . . . . 50

3.1.5.d  Page 4 Screen. . . . . . . . . . . . . . . . . . 52

3.1.5.e  Page 5 Screen. . . . . . . . . . . . . . . . . . 53

PART  4    USING HydraCyl . . . . . . . . . . . . . . . . . . . . . 55

4.1  EXAMPLE. . . . . . . . . . . . . . . . . . . . . . . . . . . . 55

APPENDIX A CYLINDER OPERATING PARAMETERS  . . . . . . . . . . . . . 63

APPENDIX B  MOUNTING TYPE CODES . . . . . . . . . . . . . . . . . . 65

APPENDIX C  Piston Rod-Stroke Charts (medium and heavy duty). . . . 69

APPENDIX D  Pipe (Tube) - Charts (Imperial and Metric sizes). . . . 72

F I G U R E S

Figure 1: HELP SCREEN . . . . . . . . . . . . . . . . . . . . . . . 13

Figure 2: HELP CONTENTS SCREEN. . . . . . . . . . . . . . . . . . . 14

Figure 3: HELP INDEX. . . . . . . . . . . . . . . . . . . . . . . . 14

Figure 4: HydraCyl WORKING SCREEN . . . . . . . . . . . . . . . . . 16

Figure 5: CYLINDER DROP DOWN MENU . . . . . . . . . . . . . . . . . 17

Figure 6: CYLINDER DIMENSIONS SCREEN. . . . . . . . . . . . . . . . 18

Figure 7: CYLINDER OPERATING PARAMETERS SCREEN. . . . . . . . . . . 22

Figure 8: CLEAR VALUES BUTTON BAR . . . . . . . . . . . . . . . . . 24

Figure 9: OPTIONS FOR METRIC AND IMPERIAL UNITS . . . . . . . . . . 26

Figure 10: MOUNTING TYPE SCREEN . . . . . . . . . . . . . . . . . . 27

Figure 11: BASIC MOUNTING TYPES . . . . . . . . . . . . . . . . . . 27

Figure 12: MOUNTING ORIENTATION SCREEN. . . . . . . . . . . . . . . 29

Figure 13: MOUNTING ORIENTATION OPTIONS . . . . . . . . . . . . . . 30

Figure 14: PIPE DIMENSIONS. . . . . . . . . . . . . . . . . . . . . 33

Figure 15: TUBE DIMENSIONS. . . . . . . . . . . . . . . . . . . . . 33

Figure 16: EXIT SCREEN. . . . . . . . . . . . . . . . . . . . . . . 34

Figure 17: ANALYSIS SCREEN. . . . . . . . . . . . . . . . . . . . . 35

Figure 18: MESSAGE SCREEN . . . . . . . . . . . . . . . . . . . . . 36

Figure 19: MESSAGE SCREEN . . . . . . . . . . . . . . . . . . . . . 37

Figure 20: TOOL SCREEN. . . . . . . . . . . . . . . . . . . . . . . 39

Figure 21: CALCULATOR SCREEN. . . . . . . . . . . . . . . . . . . . 39

Figure 22: PRINT ANALYSIS SCREEN. . . . . . . . . . . . . . . . . . 40

Figure 23: PRINTER SETUP SCREEN . . . . . . . . . . . . . . . . . . 41

Figure 24: STROKE - BRAKING PRESSURE CHART. . . . . . . . . . . . . 43

Figure 25: HELP SCREEN. . . . . . . . . . . . . . . . . . . . . . . 46

Figure 26: PAGE 1 SCREEN. . . . . . . . . . . . . . . . . . . . . . 49

Figure 27: PAGE 2 SCREEN. . . . . . . . . . . . . . . . . . . . . . 49

Figure 28: PAGE 3 SCREEN. . . . . . . . . . . . . . . . . . . . . . 51

Figure 29: PAGE 4 SCREEN. . . . . . . . . . . . . . . . . . . . . . 52

Figure 30: PAGE 5 SCREEN. . . . . . . . . . . . . . . . . . . . . . 53

Figure 31: Example. . . . . . . . . . . . . . . . . . . . . . . . . 62

Figure 32: a0 . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Figure 33: b0 . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Figure 34: c0 . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Figure 35: c1 . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Figure 36: c2 . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Figure 37: d0, e0. f0 . . . . . . . . . . . . . . . . . . . . . . . 66

Figure 38: d1, e1, f1 . . . . . . . . . . . . . . . . . . . . . . . 67

Figure 39: d2, e2, f2 . . . . . . . . . . . . . . . . . . . . . . . 67

Figure 40: d3, e3, f3 . . . . . . . . . . . . . . . . . . . . . . . 67

Figure 41: g0, h0, k0 . . . . . . . . . . . . . . . . . . . . . . . 67

Figure 42: g1, h1, k1 . . . . . . . . . . . . . . . . . . . . . . . 67

Figure 43: m0, n0, p0 . . . . . . . . . . . . . . . . . . . . . . . 67

Figure 44: m1, n1, p1 . . . . . . . . . . . . . . . . . . . . . . . 68

Figure 45: MEDIUM DUTY. . . . . . . . . . . . . . . . . . . . . . . 69

Figure 46: HEAVY DUTY . . . . . . . . . . . . . . . . . . . . . . . 70

T A B L E S

Table I: MEDIUM DUTY CYLINDERS. . . . . . . . . . . . . . . . . . . 19

Table II: HEAVY DUTY CYLINDERS. . . . . . . . . . . . . . . . . . . 20

Table III: SELECTION CODES. . . . . . . . . . . . . . . . . . . . . 31

Table IV: MOUNTING TYPE CODES . . . . . . . . . . . . . . . . . . . 65

Table V: IMPERIAL SIZES according to USAS B36.10 (BS 1600). . . . . 72

Table VI: METRIC SIZES. . . . . . . . . . . . . . . . . . . . . . . 73

PART  1        INTRODUCTION

1.1  WHAT  IS  HydraCyl?

In today's machinery and machine tools, hydraulic cylinders are the most widely

used component for the movement of heavy loads at increasingly faster rates.

Because every second saved can increase productivity and reduce costs the

machine designer must find ways to operate cylinders as fast as possible.

Choosing the hydraulic cylinder that best suits a particular application can be

a very complex and time consuming task.

HydraCyl computer software is designed to assist you in the selection of the

correct hydraulic cylinder for industrial applications.  Among other things,

you will obtain information on:

*the type of deceleration devices on hydraulic cylinders for your

application;

*buckling effects caused by the working load;

*recommended minimum length of stop tubes for your hydraulic cylinder

application;

*the minimum capacity of oil required for proper cylinder 'breathing';

*nominal flow rate required for the proper dimensioning of hydraulic

valves;

*minimum ramp time for accelerating/decelerating the attached load;

*minimum travel distance during the acceleration/deceleration of the

attached loads.

If your preferred hydraulic cylinder is not up to the task then HydraCyl will

let you know why so that you can simply modify your design criteria and perform

another analysis.

After you and HydraCyl arive at a suitable combination the STROKE - BRAKING

PRESSURE CHART presents an accurate representation of what the hydraulic

cylinder will be doing by highlighting such things as pressure peaks due to

acceleration/deceleration and inertia.

HydraCyl has been designed to carry out almost all of its functions and

calculations automatically and in such a manner that it is able to respond

promptly to incorrect or insufficient data.  Hence, there is no need to produce

a complex and difficult to understand manual.

1.2  TECHNICAL SUPPORT

HydraCyl has been designed for ease of use, and this manual and the on-line

help should contain the answers to most of your questions.  Read it first and

check the appendices for trouble-shooting procedures.

You should be able to easily solve most problems that occur in the use of

HydraCyl by referring to the relevant topics in the on-line help system or in

this manual.  If you are having specific problems with HydraCyl that are not

addressed in the general help topics, please read the Help topic

troubleshooting guide.  This guide contains specific details on a number of

common problems that may occur in the use of HydraCyl, and their solutions.

However, if you are experiencing a problem that is not addressed by following

the above steps and you have ruled out hardware or system problems with your

particular computer, HydraCyl User Support can be contacted in writing or by

electronic mail to the addresses below.

Please include the following details in any letter requesting user support:

* make and processor type of the computer you are running HydraCyl on;

*make and display resolution of the video adaptor and monitor on the computer;

*the version of DOS being used;

*Windows version (if running under Windows); and

*a detailed description of exactly what you were doing with HydraCyl when

theproblem occurred and what that problem is.

Don't forget to include your return address!

Inclusion of all (or as many as possible) of the above details will enable a

quick and effective response to your problem.

Please include the serial number of your registered copy (displayed in the

[About] box found under Help in the HydraCyl working screen).

The addresses for HydraCyl User Support are:

Fluid Software Engineering

PO Box 1245

St. Kilda Sth.

VIC. 3182

Australia

or

Fluid Power Software

P.O. Box 271932

Fort Collins, CO  80527-1932

U.S.A.

Phone: (303) 493-2521

If you have access to Compuserve or the Internet, the addresses are:

Compuserve:72163,417

Internet:      72163.417@compuserve.com

adamnik@werple.mira.net.au

1.2.1  THE SOURCE CODE

The source code for HydraCyl program is not available.  This decision gives the

developers of this software the ability to guarantee the integrity of our

product in this era of software contamination.  It is not available either

under the DEMO concept or as a commercial product.

1.2.2  DISCLAIMER

Fluid Software Engineering disclaims all warranties relating to this software,

whether expressed or implied, including, but not limited to any implied

warranties of use or fitness for a particular purpose, and all such warranties

are expressly and specifically disclaimed. Neither Fluid Software Engineering

nor anyone else who has been involved in the creation, production, or delivery

of this software and associated documentation shall be liable for any indirect,

consequential, or incidental damages arising out of the use or inability to use

this software, even if Fluid Software Engineering or an agent of Fluid Software

Engineering has been advised of the possibility of such damages or claims.  The

person using this software bears all risk as to the quality and performance of

the software.

1.3  LICENCE

By using this software you are choosing to accept the conditions of the

relevant licence detailed below.

The version of HydraCyl you are using can be ascertained by viewing the

HydraCyl [About] box.

1.3.1  FREE EVALUATION VERSION

The free evaluation version may be identified by the words ** FREE EVALUATION

VERSION ** being present in the [About] box of the program.

The free evaluation version of HydraCyl may be freely copied and distributed as

desired, as long as all the HydraCyl files are distributed in the manner in

which they were present in the original archive file as released by the author.

No files may be added to or removed from that original set of files.

Under no circumstances may any fee be charged by anyone for the free evaluation

version of HydraCyl.

Under no circumstances may any modification be made to any of the HydraCyl

files without the author's expressed written permission.

1.3.2  REGISTERED VERSION

The registered version of HydraCyl may be identified by the presence of a

registered user name and a valid serial number in the [About] box and the

absence of the words ** FREE EVALUATION VERSION **.

Each copy of the registered version of HydraCyl is licensed only to the

registered user, as detailed in the [About] box, and may be run on only one

computer at a time.  No copies may be made of the registered version of

HydraCyl except for normal backup purposes.

Under no circumstances may the registered version of HydraCyl be distributed by

anyone without the author's expressed written permission.

Under no circumstances may any modification be made to any of the HydraCyl

files without the author's expressed written permission.

1.3.3  DISTRIBUTION

*Please feel free to distribute HydraCyl DEMO as often as you like.

*Please do not distribute the program without all of its original related

files, documentation and this notice.

*Please do not accept payment for the HydraCyl DEMO program.

*HydraCyl DEMO is licensed for individual personal use and evaluation for

an unlimited time. Use and evaluation by businesses, corporations or

individuals in a commercial venture is limited to 90 days, after which

time the registered version of HydraCyl must be purchased or the use of

HydraCyl DEMO must be discontinued.

Distribution of the registered user version of the program is in violation of

license agreements and copyright law.

1.4  COPYRIGHT

HydraCyl is Copyright 1992-1995 to Fluid Software Engineering with All Rights

Reserved.

Any specific hardware/software names used in this document are the trademarks

of the specific manufacturers.

1.5  REVIEWS

The developers of this software are always in need of your opinion of our

products so that we may continue to improve our software.  Therefore, we

welcome all suggestions that may lead to improvements in these products.

Your printed review may be mailed to Fluid Software Engineering or Fluid Power

Software at the respective addresses contained in this document.

1.6  DEFINITIONS

Throughout the documentation, you may run into technical terms or everyday

computer terminology which you are not familiar with.

Following are some text examples you may come across:

[Enter]   This represents the Return or Enter key on the keyboard. If you

see this in the text, press the Enter key rather than typing in

the string.

[Esc]     This refers to the Esc key on the keyboard.

[Alt] [char][Alt] is always followed by a character which means press and

hold the Alt key and hit the following letter.  [Alt] [A] means

hold the Alt key down and press 'A'.

PART  2        OPERATING  THE  PROGRAM

2.1  SYSTEM REQUIREMENTS

HydraCyl requires an IBM PC, XT, AT, PS/2 or IBM 286 compatible computer or

higher with at least 550K RAM, a hard drive with a minimum of 1.3 megabytes of

free space and a VGA color display.  A Microsoft compatible mouse is optional.

2.1.1  OPERATING ENVIRONMENTS

Two multi-tasking environments are supported:

*DESQview from QuarterDeck Office Systems.

*Windows version 3.0 and greater from Microsoft.

DESQview and Windows allow HydraCyl to run in a window without requiring the

whole screen, except when high resolution graphics have to be viewed. HydraCyl

will automatically switch to a full screen for this purpose and, after exiting

from the graphics, revert to the window screen.

In your Program package you will find special configuration files for DESQview

and Windows.

HydraCyl may not operate properly when other background software is running.

By other background software, we mean programs that are loaded in the

CONFIG.SYS or by any BAT file and remain in memory.

If you experience unpredictable problems, it is possible that one of these

programs is interfering with HydraCyl.  Remove these programs or device drivers

from the AUTOEXEC.BAT or CONFIG.SYS file one at a time until you find the

offending item.

Another method is to start DOS without any resident programs or device drivers

and add them back in, one at a time, until the culprit is identified.

2.2  INSTALLING HYDRACYL

The first thing you should do is back up your HydraCyl distribution disk.

Please refer to your DOS manual if you are not sure how to backup your disk.

To begin the unpacking and installation process place your diskette into your

floppy drive and type:

SETUP  [Enter]

You will be prompted to enter the source drive and to confirm or change the

destination drive and path.  The installation program will do all the necessary

work for you.

When the installation process has completed, you will be shown a message screen

which contains important last-minute information on starting and running

HydraCyl for the first time.  Please read this screen carefully.  It is

strongly recommended that you type at the DOS prompt:

HCR  [Enter]

Make sure that you read all the information shown in the screen presented by

this action. If you want to start the main program, switch to the working

directory where all the files reside and, at the DOS prompt, type:

HC  [Enter]

If you add the working directory to your path in AUTOEXEC.BAT, you will be able

to start the HydraCyl program from within any directory, but take note that

on-line help will not be available.

2.3  WORKING WITH HydraCyl

HydraCyl may be operated with or without a mouse to activate menus, command

buttons and other controls.  The on-line help provides detailed assistance by

using the [Page Up] and [Page Down] keys, the [up] and [down] arrow keys or by

clicking on the scroll bar to the right of the Help text with a mouse to view

all pages of the text.

In order for you to be able to operate and manipulate HydraCyl the relevant

sections from the on-line help  are shown below in 2.3.1 to 2.3.7.

Please note that any type of control in HydraCyl may be rendered active or

inactive by the program depending on whether it is possible to use that

particular control at that particular time.  Inactive controls become grey

(they are "greyed out") and will not operate until HydraCyl activates them

again.

The various windows used throughout HydraCyl to accept or display information

may be referred to in the Help text as forms.

Mouse users should note that throughout this Help text the term to "click" on

something means to move the mouse cursor to that item and then click the left

mouse button once.

2.3.1  HELP CONVENTIONS

Throughout the HydraCyl Help system the following conventions are observed:

*The names of keyboard keys are in square brackets as, for example, [F1]

indicates the F1 key, [Enter] indicates the Enter key, etc.

*The names of menu command buttons are between underlined square brackets

as in [OK] indicating the OK button, [Cancel] indicates the Cancel button,

etc.

*Accelerator keys are indicated by [Alt] [X], where X is the key to be

pressed while the Alt key is held down as, for example, [Alt] [O] means

that while holding down the Alt key, press O.

*Windows presented on the screen for the purposes of collecting or

displaying information may be referred to as forms.

*Menu commands and form titles are shown in bold type; eg. Analysis,

Perform Analysis, indicates the menu choice "Perform Analysis" on the

"Analysis" menu and Cylinder Data indicates the "Cylinder Data" form.

2.3.2  HOT KEYS

Hot keys are keys which, when pressed anywhere within HydraCyl (except while

menus are active), will always carry out their associated action.

Some examples are:

[F1] opens Help related to the current activity;

[F3] exits HydraCyl; and

[F5] activates the pop-up calculator.

2.3.3  ACCELERATOR KEYS

Throughout HydraCyl there are "accelerator" keys which may be used to directly

carry out currently available actions by holding down the [Alt] key and then

pressing the desired accelerator (highlighted) key.

These keys are represented in HydraCyl by being the only bright white character

in the name of any control or group of controls.

Pressing the accelerator key for any control always causes that control to be

directly activated whether or not it is the currently selected control. For

example, pressing [Alt]  [O] while the Cylinder Dimensions form is on the

screen, will cause the [OK] button on that form to be activated.

Pressing an accelerator key that is in the name of a group of controls, will

always cause the first control in that group to become the currently selected

control. For example, pressing [Alt] [P] while the Operating Parameters form is

on the screen, will cause the cursor to jump to the first input field in the

Pressure category.

2.3.4  MENUS

At the very top of the HydraCyl working screen is a row of menu titles known

collectively as the menu bar. Each of these menu titles may be activated to

drop down a menu of items available under that title. These items may then be

selected in order to carry out the required action in the program.

Keyboard

Pressing the [Alt] key alone will activate the menu bar. The left and right

arrow keys can then be used to highlight the required menu and [Enter] then

pressed to drop that menu down.

Once a menu is dropped down the [up] and [down] arrow keys can be used to

highlight the required menu choice and [Enter] pressed to carry out the chosen

action, or the highlighted key in any menu item may be pressed to activate that

menu item directly.

Alternatively, the accelerator keys may be used to operate the menus as

follows:

- press [Alt] to highlight the accelerator keys on the menu bar;

- while still holding down [Alt], press the highlighted key for the

desired menu;

- then press the highlighted key in the menu for the required item.

For example pressing [Alt] [A] [A] will drop down the Analysis menu and select

Perform Analysis from that menu.

Mouse

Click on any menu title to drop down that menu. Then click on the desired menu

item.

Alternatively, point to the desired menu title with the mouse cursor, then

press and hold down the left mouse button. While continuing to hold down the

left mouse button, drag the highlight bar down until the required menu item is

highlighted. Then release the mouse button to activate the item.

2.3.5  COMMAND BUTTONS

Command buttons are controls on HydraCyl forms that may be "pressed" like

push-buttons in order to carry out particular actions.

The two most common command buttons (which are present on almost every form in

HydraCyl) are the [OK] and [Cancel] buttons. The [OK] button instructs the

program to close the form and accept any changes made. The [Cancel] button

closes the form without accepting any changes made.

Keyboard

A command button may be selected by pressing [Tab] (to go forward to the next

control) or [Shift] [Tab] (to go backward to the previous control) until the

required button is highlighted. Pressing [Enter] will then activate (or

"press") the selected button.

A command button may also be directly activated by using its accelerator key

(see above) if it has one.

On most forms the [Cancel] button can be activated by pressing the [Esc] key.

When a command button is the currently selected control on a form, pressing

[Enter] will always activate that button.

Mouse

To use a command button with the mouse simply click on the desired button.

2.3.6  RADIO BUTTONS

A radio button is a control that consists of a name or value displayed adjacent

to a pair of brackets that enclose a single space.

Radio buttons always occur in groups, where one, and only one, member must be

selected from among that mutually exclusive group of options (like the station

buttons on a push button car radio).

The selected member of a group of radio buttons is indicated by a dot that

appears inside the brackets beside that item.

Keyboard

Use the [Tab] or [Shift] [Tab] keys to move between controls on the form until

the desired group of radio buttons is selected.  Then use the [up] and [down]

arrow keys to select an item in the group.

If the name of the group of radio buttons has an accelerator key, that

accelerator key may be used to jump straight to that group.

Mouse

Click on the required radio button to select it.

2.3.7  INPUT FIELDS

Input fields are places on a form where the user may enter data from the

keyboard.

Keyboard

Use [Tab] or [Shift] [Tab] to select the required input field, then type in the

desired data.

If a named group of input fields has an accelerator key, using that key will

cause the cursor to jump to the first active input field in that group.

Remember that pressing [Enter] is only used in HydraCyl for activating a

selected command button or menu item.  Pressing [Enter] while an input field is

selected will simply cause a beep.  Use the [OK] button on the form to accept

any entered data.

Within an input field the left and right arrow and [Home] and [End] keys may be

used to move the cursor.  The [Insert], [Delete] and [Backspace] keys

(destructive backspace) may be used to edit the data.

Mouse

To select an active input field, click on it with the mouse.

See the "Keyboard" section above for information on the editing keys available

within an input field.

2.4  HELP

HydraCyl features context-sensitive help.  As the name suggests, relevant help

information is available for any screen, just by pressing the [F1] Help key.

For example, if you are in the Mounting Type screen ([Alt] [C] and [Alt] [M])

and press [F1] the Help window as shown in Figure 1 will appear.

By moving around this screen with the mouse or the [up] and [down] arrow keys

you will get extensive help for that screen.  Of course, you can access any

part of the Help file from any Help screen.  In the above example, you will see

buttons [Contents] and [Index]; these will give you instant access to any part

of the global help available for any screen or command.

Now, if this is not what you were looking for or if you wanted to get a list of

Help Topics, just press [Alt] [O] or click on the [Contents] button with a

mouse again to drop down the screen shown in Figure 2.

By activating [Index], the window as shown in Figure 3 will appear on your

screen.

From here, you can jump over to Help on other screens and/or commands or obtain

further information on how the Help system works.  Help remembers where you

have been, so you can retrace your steps back to the original screen by

pressing [Alt] [B].

The on-line help is not all-inclusive.  Although this context-sensitive help

has been made as detailed as possible, it is still no substitute for this

manual.

If [Alt] is pressed by mistake, use the [Esc] key to return to the previous

status line.

Pressing [Esc] will exit from all windows and HydraCyl functions without making

any changes.

PART  3        HydraCyl

3.1  DESCRIPTION

When you start HydraCyl for the first time you will be presented with the

initial working screen (Figure 4).

On the top left corner of your screen there are four menus for navigating

through program. They are:

Cylinder       (see also Figure 5)

Analysis       (see also Figure 17)

Tools          (see also Figure 21)

Help           (see also Figure 1)

The Cylinder menu is designed for manually entering all data inputs required by

program. The order in which you enter them is not important.

The Analysis menu invokes the module that analyses the entered inputs as well

as resetting all data (setting their values to zero).

Tools comprises a handy calculator for your convenience, allows viewing of a

previously saved Stroke - Braking Pressure Chart and contains a utility that

allow you to print generated outputs.

Help, as the world implies, is a complete on-line help system for this

software.  Remember that pressing the [F1] hot key from most places in HydraCyl

will call up the on-line help system.

3.1.1  CYLINDER

Use of a mouse to highlight Cylinder in the menu bar of the HydraCyl working

screen or pressing [Alt] [C] will generate the drop down menu shown in Figure

5.  This menu contains five data sets, these being:

Dimensions;

Operating Parameters;

Mounting Type;

Mounting Orientation; and

Hydraulics Pipe (Tube).

The menu also contains an Exit command.

The  purpose of this menu is to provide access to the data input screens.

HydraCyl will perform all necessary calculations at both the local level and

the global level, using the data entered into the program through each data

set.  Local level calculations relate to a specific data set and are based only

on the data entered into that particular data set; global level calculations

automatically take into account all information entered into the global

Cylinder menu by way of all five data sets.  Global level calculations are

initiated by invoking the Analysis command in the HydraCyl Working Screen menu.

If any information is missing or has an ambiguous meaning you will be warned by

an error message at the time of data entry or by the Warnings screen on the

fourth page of the Cylinder Data screen.

3.1.1.a  Dimensions

Dimensions provides a range of options detailing the type of cylinder you want

to use in your design.  Pointing and clicking the mouse on Dimensions will

display the screen shown in Figure 6.

The cylinder dimensions included in this program are based on data contained in

catalogues from the world's main cylinders manufacturers.

This menu is divided into three parts. They are:

Cylinder's Duty (a choice of Medium or Heavy duty)

Cylinder Bore Diameter (in inches)

Cylinder Rod Diameter (in inches)

To specify the cylinder's dimensions select, in this order, the Cylinder's

Duty, the Cylinder Bore Diameter and the Cylinder Rod Diameter.  This data is

validated by clicking on the [OK] button.

If for any reason you change your mind and decide to switch to another value,

just point and click mouse at that value.  No information on the cylinder

diameter will be saved until the [OK] button is pressed.

The cylinder dimensions available for both medium duty and heavy duty are shown

in Table I and Table II below.

After verifying the selected cylinder dimensions, by clicking the [OK] button,

you will be taken back one level to the previous menu.

3.1.1.b  Operating Parameters

The Cylinder Operating Parameters screen (as shown in Figure 7) allows you to

enter some of the hydraulic cylinder operating parameters which are necessary

for HydraCyl to perform its calculations and analysis.

There are four sub-sets to the Cylinder Operating Parameter screen.  They are:

Dimensions

Flow

Pressure

Other.

Only one parameter for each of the sub-sets labelled Flow and Pressure must be

entered, the values for the other parameters in these sub-sets will then be

calculated by HydraCyl as it performs its analysis.

Once you have entered your parameter of choice in the Flow and Pressure

sub-sets, HydraCyl prevents any further data being entered into both of these

sub-sets.  To re-activate all the input fields in one of the Flow or Pressure

sub-sets either delete the relevant input value or use the [Clear Values]

button (see below) to clear the values for that particular sub-set.

Dimensions

Under Dimensions there is only one parameter.  It is the Stroke, which is the

distance between the cylinder rod's fully extended and fully retracted

positions.

Flow

Flow comprises five components but data must be entered in only one of them.

The program will then calculate all other parameters and the results will be

presented in the output screen.

System Flow means the pump flow, which is the flow that is required by the

cylinder for it to fulfil its function during its working cycle.

Extending Speed means the speed of the movable part of the cylinder during

the cylinder's extension.

Extending Time means the time of cylinder travel between its fully

retracted and fully extended states.

Retracting Speed means the  speed of the movable part of the cylinder

during the cylinder's retraction.

Retracting Time means the time of cylinder travel between its fully

extended and fully retracted states.

It should be noted here that the value entered into the Flow sub-set will be

treated as the primary value and all other parameters will be determined by

HydraCyl on the basis of that entered value. For example, if you enter a value

for Extending Time, the values of System Flow, Extending Speed, Retracting

Speed and Retracting Time will be automatically  calculated.

Pressure

The logic of entering desired or required data in this sub-set is the same as

for Flow parameters. The entered value is the primary data and all other values

will be automatically calculated by HydraCyl.  For example if you enter

Retracting Force as the specified parameter, the System Pressure and the

Extending Force will be determined on the basis of that value.

System Pressure means the pressure generated by the pump, another source

external to the cylinder or the pressure required by the cylinder to fully

achieve its purpose during its working cycle.

Extending Force means the force that results from the action of the System

Pressure acting on the face of the piston, causing its extension

(Extending Force, Extending Speed and Extending Time are always

inextricably linked).

Retracting Force means the force that results from the action of the

System Pressure acting on the face of the piston, causing its retraction

(Retracting Force, Retracting Speed and Retracting Time are always

inextricably linked).

Others

The Applied Mass and Cyl-Valve distance are only the parameters in this

sub-set.  A value must be ascribed to both parameters, with zero being used if

necessary.

Applied Mass is the weight (not force) of the mass attached to the

cylinder during its rod travel.

Cyl-Valve distance is the distance between the cylinder's ports and the

command block ports (pipeline length).

Clearing Values

On the bottom of the screen you will notice four buttons for [OK], [Clear

Values], [Change Units] and [Cancel].

The [OK] button will validate all data entered and take you back to the

previous menu. [Cancel] will take you back to the previous menu but all newly

entered data will be ignored.  Any previously entered data will be retained.

If you want to change any data entered in one of the parameter options you will

be able to change it by clicking on the [Clear Values] button. This will

present you with a number of options in the form of a new button bar across the

bottom of the screen as shown in Figure 8.

The [Flow] button will delete the value of any parameter in the Flow

parameter sub-set and allow you to enter a new value for any other

parameter in that sub-set.

The [Pressure] button will delete the value of any parameter in the

Pressure parameter sub- set and allow you to enter a new value for any

other parameter in that sub-set.

The [Stroke] button will delete the value of stroke in the Dimensions

sub-set and allow you to enter a new value for the stroke.

The [Other] button will delete the value for the Applied Mass and the

Cyl-Valve distance in the Other parameter sub-set and allows you to enter

new values for the mass attached to the cylinder rod and the pipeline

length between cylinder and the command block.

The [All] button will delete all values entered in all four parameter

sub-sets.

The [Cancel] button, as the word implies, allows you the flexibility of

changing your mind and continuing to work with previously entered values

but without erasing any of those entered data.

It should be noted that clearing any value will take you back to the previous

menu.

Although there is only one parameter in the Dimensions sub-set, the same logic

as above has been followed for the sake of future software upgrading.

Changing Units

HydraCyl allows you the choice of working in metric or imperial units. The

[Change Unit] button provides the options available within the program.  Figure

9 shows the menu that appears when you click on the [Change Unit] button.

The dots show the default units for the program.  If the defaults are changed

the new defaults are saved in the HC.INI file in the working directory.  It is

emphasised here that if you change units after you have previously entered data

into the program HydraCyl will not convert that data to the new units.

For peace of mind always ensure that the units are in the desired system prior

to  entering any data.

The on-line calculator incorporated with the [F5] hot key will help you avoid

difficulties if you do not follow above mentioned recommendation.

The [OK] button will validate the new units (entered by you) as the defaults

and take you back to the previous screen.  The [Cancel] button will take you

back to the previous screen without setting new defaults.

A summary of the Cylinder Operating Parameters is given in Appendix A.

3.1.1.c  Mounting Type

This screen allows selection of the type of mounting or mounting style that

will be used to ensure that the cylinder is installed in such a manner that the

working forces act along the line of the piston rod axis.  There is a table in

this screen (Figure 10) which lists the range of possible mounting types. The

table is divided into columns labelled 'a' to 'p' (which represent the basic

mounting types) and rows labelled '0' to '3' (which represent variations within

each basic mounting type).  Selecting any field within the table generates the

display of a description of that particular mounting type.

Activating the [View Basic Mount Types (a - p)] button will cause HydraCyl to

display a diagram of the basic mounting types 'a' to 'p' (Figure 11), with

letters corresponding to the appropriate column in the table.

Note the code of the appropriate mounting and then press any key to return to

the Mounting Orientation form.

Activating the [View Detail of Selected Mount] button will generate a detailed

diagram of the specific variation to the basic mounting type which is

represented by the selected field.

A complete list of Mounting Type Codes is given in Appendix B.

A Mounting Type radio button that corresponds to the required basic mounting

type and the correct mounting type variation must be selected.

The procedure for specifying the type of mounting required is as follows:

*activate Mounting Type in the Cylinder drop down menu (Figure 5)

*switch to the high resolution diagram of the basic mounting types (Figure

11) by way of the [View Basic Mount Types (a - p)] button

*identify and note the letter code of the requisite mounting type and

return to the Mounting Type screen and its table (Figure 10) by pressing

any key

*mark the selected code in the table; details of the selected mounting

will appear in the Description box in that screen

*the selected mounting can be viewed by using the [View Detail of Selected

Mount] button.

Using the [View Detail of Selected Mount] button will also enable you to view

the variations in the different types of mounting.

The [OK] button will confirm the selected choice and automatically return you

to the previous menu. The [Cancel] button will ignore any new selection and

return you to the previous menu, retaining the previously selected values.

3.1.1.d  Mounting Orientation

The screen shown in Figure 12 relates to the range of possible mounting

orientations.

The Mounting Orientation menu provides access to a high resolution diagram that

allows you to see how the load is attached to the cylinder as well as the

spatial orientation of the hydraulic cylinder (defined by the angle between the

cylinder's axis and the horizontal).  Choosing one of the six Selection options

will generate details of that particular mounting orientation in the

Description box.

The angle between the cylinder's axis and the horizontal (see below) must be

entered. Any angle between 0 and 359 degrees inclusive is valid.  The

Coefficient of Friction between the fixed and the movable parts of the cylinder

- mass assembly system must also be entered.  Its value can range between 0 and

1.

By clicking on the [View Orientation Diagrams] button the high resolution

diagram as shown in Figure 13 is presented, showing the two basic methods of

load attachment.  These are:

*attaching the load to the cylinder's piston rod (as shown in the sketch

on the left hand side of Figure 13); and

*attaching the load to the cylinder's body (as shown in the sketch on the

right hand side of Figure 13).

In both cases the point of fixture is marked by O.  The cylinder axis OZ and

the horizontal line OX form the angle that defines the spatial orientation of

the cylinder.  The small red arrows indicate the mode of movement and the

method of attachment of the load to the cylinder.

The [OK] button will validate your choice and take you back to the previous

menu; the [Cancel] button will take you back to the previous menu without

accepting any new data input.  The Description box provides details of the

selected Mounting Orientation that corresponds to the chosen Selection.

A complete description of the Selection codes is given in Table III.

3.1.1.e  Hydraulics Pipe (Tube)

The pipe dimensions screen (Figure 14 and 15) displays a list of the

combinations of  nominal pipe diameters and wall thicknesses that are available

within HydraCyl. The [Metric] button will allow you to switch to metric tube

dimensions, given in millimetres; the [Imperial] button will take you back to

the imperial screen.

This list of pipe dimensions corresponds to the range of pipe (tube) sizes

available from manufacturers, with units in inches and/or millimetres are given

in Appendix D.

When the desired pipe (tube) is selected in the Nominal Pipe Size column

HydraCyl will present the corresponding available choices of wall thicknesses

in the Nominal Wall thickness column.

Mouse users may click on the desired diameters.  Keyboard users should use

[Tab] or the accelerator keys to choose the type of pipe (tube) to be used and

then use the up and down arrow keys to pick a particular value from the list.

Activate the [OK] button to validate your selections and record them on the

Cylinder Data form (Figure 4).

3.1.1.f  Exit

This switch allows you to terminate HydraCyl and go back to the DOS/WINDOWS

environment.  The [F3] hot key operates this function.  Use of this command

will bring up the screen shown in Figure 16.  Clicking on [Yes] with a mouse

terminates the HydraCyl program; the [No] button cancels your request to exit

HydraCyl and takes you back to the previous screen.

3.1.2  ANALYSIS

Highlighting Analysis in the top menu bar of the HydraCyl working screen

instructs the program to activate a drop down menu containing two options.

These options, as shown in Figure 17 are:

Perform Analysis

Reset All Data

3.1.2.a  PERFORM ANALYSIS

This command instructs HydraCyl to analyse previously entered data.  If you

have missed any data required by the program to carry out the analysis,

HydraCyl will prompt you to enter the missing information in the appropriate

menu (which is automatically displayed by the program).

HydraCyl is a very flexible program that will allow you to enter the required

inputs in any order you desire, but if you would like HydraCyl to lead you

through a "fast" analysis the first time you use it (or at any other time),

follow these steps:

*activate the Analysis menu and choose Perform Analysis. HydraCyl will

advise you if any of the required information is missing; if the [OK]

button is activated to continue, the program will present you with the

relevant form for entering the missing data.  For example, missing data on

Cylinders dimensions is shown in Figure 18.

*repeat the above steps until all the required inputs have been collected,

at which point HydraCyl will perform its calculations and present you with

a Calculations Complete message (the appearance of this message, as shown

in Figure 19, confirms that the data entry process has been correct)

*after selecting [OK], you may now browse through the pages of the

Analysis results by using the [Page Up] and [Page Down] buttons on the

Cylinder Data form or by pressing the [Page Up] and [Page Down] keys on

the keyboard (data inputs are dark blue, results are light blue)

*if desired, use the Cylinder menu to call up any of the different input

forms to change any data you wish, then run Analysis again to check on the

new results.

3.1.2.b  RESET ALL DATA

As the name implies, this command will reset all previously entered information

in HydraCyl.  In other words, it will have the same effect as exiting the

program and then starting your HydraCyl session again.  It is possible to

confirm the action of this switch by browsing through the five pages of

Cylinder Data (see 3.1.3.a PRINT ANALYSIS) as all information should have been

deleted.  Please note that if you perform Reset All Data it is not possible to

recall the deleted data.  Any data previously saved as a Stroke - Braking

Pressure Chart (PCX format) will be still available to you.

3.1.3  TOOLS

This menu provides access to the on-line calculator, contains the command to

generate a hard copy report containing the results of an analysis and allows

the presentation of a Stroke - Braking Pressure Chart.

The Tool screen is shown in Figure 20.

3.1.3.a  CALCULATOR

The on-line calculator has been included for your convenience, allowing any

quick calculations to be made if required.  The calculator can be invoked by

the [F5] hot key at almost any stage during while working within the HydraCyl

environment.  The [Hide] button will remove the calculator from the current

working screen and take you back to the previous screen, allowing you to

continue with your work.

The calculator screen is shown in Figure 21.

3.1.3.b  PRINT ANALYSIS

A hard copy of the analysis results can be obtained by clicking on Print

Analysis in the screen shown in Figure 22.

The [OK] switch will instruct HydraCyl to print the screens with computed data.

Always ensure that your Printer is properly connected to your system and set up

the printer defaults using the Printer Setup command.  Clicking on Printer

Setup generates the screen shown in Figure 23.

3.1.3.c  STROKE - BRAKING PRESSURE CHART

Cylinder cushions are often installed at one or both ends of a cylinder to slow

down the movement of the piston near the end of its stroke and prevent the

piston hammering against the end cap.

The main purpose of the Stroke-Braking Pressure Chart is to illustrate the need

for cushions in particular cylinder applications.  By studying this chart we

are able to determine rapidly the correlation between generated braking

pressure and associated braking stroke during cylinder decelerations at the end

of stroke.  The braking stroke (distance) is the cushion length.  The braking

pressure corresponds to the pressure generated by all external forces which

oppose the cylinder's moving part during slowing down movement.  Cylinder's

moving parts are defined earlier in Mounting Orientation. Under the term "all

external forces" we assume the inertial forces of the load attached load to the

cylinder and inertial forces of the cylinder moving parts.

This chart is dynamically drawn. Its typical presentation is shown on Figure

24. Whenever you point out and click on Braking Diagram button (screen 4 of 5),

the chart appears on the screen. On the left hand side of the screen, in the

vertical column, the colour legend is shown as follows:

Maximum Rated Pressure (in red)

Generated pressure at Head End compartment (in blue)

Generated pressure at Cap End compartment (in green)

Just below this information the column continues displaying:

Cylinder Bore Diameter

Cylinder Rod Diameter

Stroke Length

Code for Mounting Type

Attached Mass (inertial load)

System Pressure

System Flow

Code for Mounting Orientation

Slope

Friction Coefficient

and commands:

[E]xit (not shown on this figure)

[S]ave (not shown on this figure)

These two commands only appear when the Braking Diagram button is pressed,

giving you two options:

[E]xit -  is command for returning to program

[S]ave -  is command for saving generated chart as HC12D.PCX image and

returning to program.

If command for saving image on disk (in the subdirectory you are in) is issued,

on the top left side of the screen appears percent gauge (not shown on figure)

indicating the value of scanned and saved image.  That image can be invoked

back from Tools menu View Stroke-Braking Pressure Chart until it is replaced by

new image under the same name by new [S]ave command.

The purpose of saving the previously generated image comes from experience that

very often by comparing two charts (changed one or more parameters in cylinder

application) it is possible to minimise the requirement for cushions. Also it

allows us to examine the influence of some parameters on the behaviour of

generated pressure in cylinder, and as a consequence bring the cylinder into

the safe pressure rating zone (below the shown red line).

Axes on the diagram are:

X-axisThe Braking Stroke (distance) or cushion length

Y-axisThe Generated Pressure.

It should be emphasised that units for Braking Stroke and Generated Pressure

are defined in Cylinder menu under Operating Parameters as corresponding units

for Cylinder Stroke and System Pressure.

Interpreting the chart is 'intuitive'.  For any point that lies  on the line it

is possible to determine for particular Braking Stroke the corresponding

Generated Pressure.  If Generated Pressure goes over chart's limits (300 bar or

4350 psi) the appropriate message will be issued informing where that pressure

is generated.

The safe pressure zone lies below the red line (maximum pressure rating).  The

zone of possible damage to cylinder lies over the red line.  It is obviously

clear that any intersection between green or blue line with red line marks the

point of minimum allowable cushion length. In other words, for any point of

desirable braking distance that lies over the safe zone, the external braking

device must be used.

You can easily obtain information on standard cushion length (braking stroke)

and maximum pressure rating from your favourite supplier of cylinders.  Just

enter that data on the chart and you will get information on what would happen

in the cylinder.  Generally, for any value of braking stroke between 1/2 inch

and 2 inch (12.7 mm and 50.8 mm) you are able to determine actual braking

pressure.

3.1.4  HELP

On-line Help is very comprehensive and covers the topics of:

*Contents;

*Current Topic;

*Using Help;

*Keyboard/Mouse;

*Introduction; and

*About HydraCyl.

Help Screen is shown in Figure 25.

3.1.4.a  Contents

This command opens Help at the Contents page.

3.1.4.b  Current Topic

Calls up Help on the form currently in use.  This command has the same effect

as using the [F1] hot key.

3.1.4.c  Using Help

General explanations on how to use Help are generated by this command.

3.1.4.d  Keyboard/Mouse

Opens Help on keyboard and mouse use.

3.1.4.e  Introduction

This command displays the Introduction to HydraCyl.  It contains a brief

description of the purpose of the program, the standard disclaimer (which is

similar to that contained in this manual) and advice that the source code is

not available (to ensure product integrity).

3.1.4.f  About HydraCyl

The version number and details of the particular HydraCyl package being used

(DEMO or registered version) are shown in the [About] box.  If you have a full

registered version of HydraCyl your user details are also displayed in the

[About] box.

3.1.5  COMPUTED  DATA  SCREENS

Five pages of results of an Hydracyl analysis are available for browsing by

using the [Page Up] and [Page Down] keys.

3.1.5.a  Page 1 Screen

The first page comprises only data entered by the user.  It is split into two

groups, these being the cylinder's prime dimensions and the cylinder's mounting

details.

Detailed descriptions of Mounting Type and Mounting Orientation could be found

in Sections 3.1.1.c and 3.1.1.d.  Page 1 Screen is shown in Figure 26.

3.1.5.b  Page 2 Screen

This page comprises the Operating Parameters that are calculated in accordance

with the entered data.  The Operating Parameters have been defined previously

in Section 3.1.1.b.  Page 2 Screen is shown in Figure 27.

The length of stop tube required by the cylinder will be automatically

determined by HydraCyl. If a Stop Tube is not required this field will remain

blank.

Extending Return Flow is the flow generated at the cylinder's head end port

during the supply of the cylinder's cap end port by the  System Flow during

Extending Time.

Max Pressure Rating is directly determined by  cylinder's duty, bore and rod

dimensions, mounting type and orientation.

Retracting Return Flow is the flow generated at the cylinder's cap end port

during the supply of the cylinder's head end port by the System Flow during

Retracting Time.

Extending Force is the force generated by Supply Pressure applied at the

cylinder's cap end port.

Retracting Force is the force generated by Supply Pressure applied at the

cylinder's head end port.

3.1.5.c  Page 3 Screen

Page 3 Screen is shown in Figure 28.  This page comprises:

Column Strength based on cylinder's rod diameter and its length.  The value

that defines critical piston rod column strength when buckling occurs on push

load application.

Applied Mass specified by User in Cylinder Operating Parameters.

The Other Analysis Results that are calculated in accordance with the entered

data.  They are:

Stop Tube is length that defines minimum required distance between the gland

and piston when cylinder is fully extended.

Extending Return Flow is calculated theoretical value of return flow generated

by cylinder during its extending cycle.

Retracting Return Flow is calculated theoretical value of return flow generated

by cylinder during its retracting cycle.

System Power is calculated theoretical power required or generated by cylinder

for its full cycle.

Bore Area is calculated Cylinder's piston bore area.

Rod Area is calculated Cylinder's piston rod area.

Annulus Area is calculated difference between Cylinder's piston bore and rod

areas.

3.1.5.d  Page 4 Screen

This page comprises:

*Displacement Volume,

*Deceleration Device with Braking Diagram button and

*Warning message field.

Page 4 Screen is shown in Figure 29.

Cylinder Head is calculated theoretical cylinder's volume when piston rod is

totally retraced.

Cylinder Cap is calculated theoretical cylinder's volume when piston rod is

totally extended.

Total (Displacement Volume) is theoretical Cylinder's volume calculated as

difference between Cylinder's head and cap volume.  This value defines minimum

required oil volume for proper cylinder work when cylinder is previously filled

by oil.

Rod movement near the head and/or cap end is a critical part of the overall

cylinder movement. The braking device, or cushion, is a very important

component required for proper cylinder operation. HydraCyl will automatically

offer a [Braking Diagram] button which, if activated, will generate the high

resolution Stroke - Braking Pressure Chart as shown in Figure 24.

If HydraCyl's analysis of the input cylinder data results in any warnings or

errors they are displayed in red on this Screen.  Non-critical warnings or

errors result in partially complete analysis results and an advisory message.

Critical warnings and errors result in all analysis results being cleared and

an advisory message.

3.1.5.e  Page 5 Screen

This page comprises Cylinder's Dynamical Characteristics.  They are:

*Natural Frequency,

*Minimum Acceleration/Deceleration Time,

*Minimum Acceleration/Deceleration Distance,

*Minimum Required Dynamical Pressure for Acceleration/Deceleration,

Page 5 Screen is shown in Figure 30.

Natural Frequency Rod Extending is calculated value for undamped natural

frequency of system cylinder-attached load during rod extending.

Natural Frequency Rod Retracting is calculated value for undamped natural

frequency of system cylinder-attached load during rod retracting.

Minimum Ramp Time Rod Extending is calculated minimum time required for

cylinder to accelerate (decelerate) from zero to maximum speed (from maximum to

zero speed) during rod extending. The maximum speed is given by Extending

Speed.

Minimum Ramp Time Rod Retracting is calculated minimum time required for

cylinder to accelerate (decelerate) from zero to maximum speed (from maximum to

zero speed) during rod retracting.  The maximum speed is given by Retracting

Speed.

Minimum Travel Distance Rod Extending is calculated minimum distance required

for cylinder to accelerate (decelerate) from zero to maximum speed (from

maximum to zero speed) during rod extending.  The maximum speed is given by

Extending Speed.

Minimum Travel Distance Rod Retracting is calculated minimum distance required

for cylinder to accelerate (decelerate) from zero to maximum speed (from

maximum to zero speed) during rod retracting. Maximum speed is given by

Retracting Speed.

Minimum Total Pressure Rod Extending is calculated minimum total pressure

required for cylinder accelerating or decelerating during rod extending on the

distance defined by Minimum Travel Distance Rod Extending.  The negative value

of pressure is pointing to total pressure at cylinder's head end port generated

by attached load and friction forces.

Minimum Total Pressure Rod Retracting is calculated minimum total pressure

required for cylinder accelerating or decelerating during rod retracting on the

distance defined by Minimum Travel Distance Rod Retracting.  The negative value

of pressure is pointing to total pressure at cylinder's cap end port generated

by attached mass and friction forces.

PART  4    USING HydraCyl

4.1  EXAMPLE

We have included an example of a hypotetical design to ilustrate the step by

step procedure involved in HydraCyl.

Design specifications are:

*    Cylinder Duty                 Heavy

*    Cylinder Bore Dimension             2   inches

*    Cylinder Rod Dimension              1   inch

*    Cylinder Stroke                    34   inches

*    System Flow                   25   litres/min

*    Pressure                           60   bar

*    Attached Mass                       2300kg

*    Cylinder - Valve distance             150inches

(The length of pipes between the Cylinder's ports and the Solenoid valve)

*    Cylinder Mounting Type             Intermediate trunnion

*    Mounting Orientation               The applied massis permanently attached

to the piston rod stud and Cylinder rod

extends and retracts

*    Spacial orientation                35   deg

*    Coefficient of Friction             0.15

*    Hydraulic Line Dimensions     6    mm (outside diameter)

'L'  series

1.      Run HC.EXE

2.      Choose 'OK' on form 'About'

3.      Choose 'Analysis'

4.      Choose 'Perform Analysis'

You will get a message: 'Cylinder Dimensions must be specified'

5.      Choose 'OK'

Let's say you want a heavy duty 2"/1" cylinder

6.      Click on '( ) Heavy' in the 'Cylinder's Duty' box

7.      Click on Cylinder Bore Diameter 2.00

8.      Click on Cylinder Rod Diameter 1.00

9.      Click on 'OK'

10.     Choose 'Analysis'

11.     Choose 'Perform Analysis'

You will get a message: 'Cylinder Operating Parameters must be

specified'

12.     Choose 'OK'

Let's say the stroke of your cylinder is 34 inches, firstly you

need to et the correct units.

13.     Choose 'Change Units'

14.     Click on '( ) inch' in the 'Stroke' box

(this will be the default unit for stroke until you

change it back to 'mm')

15.     Click 'OK'

Now you can enter the stroke.

16.     Click in '[_     ] inch' under the 'Stroke' heading

17.     Type 43  (OOPS my typing mistake, should be 34)

18.     Click on 'Clear Values'

19.     Click on 'Stroke'

20.     Type 34 (correct value)

Let's say the system flow is 25 lit/min

21.     Click in '[_     ] lit/min' next to the 'System Flow' heading

22.     Type 25

Let's say the Pressure is 60 bar

23.     Click in '[_     ] bar' next to 'Pressure'

(if it is required change units to 'bar' in the simmilar

maner as stroke.)

24.     Type 60

Let's say the attached mass is 2300 kg

25.     Click in '[_     ] kg' next to 'Applied Mass'

26.     Type 2300

Let's say the pipeline length between Cylinder's ports and solenoid

valve is 150 inches

27.     Click in '[_     ] inch' next to 'Cyl-Valve distance'

28.     Type 150

29.     Click 'OK'

30.     Choose 'Analysis'

31.     Choose 'Perform Analysis'

You will get a message: 'Cylinder Mounting Type must be specified'

32.     Choose 'OK'

Let's say the cylinder mounting type is 'Intermediate trunnion'

33.     Click on 'View Basic Mount Types (a - p)'

34.     High resolution picture is shown of all possible mounting

types. Look for Intermediate trunnion. It is marked by 'b'

35.     Press any key to get out of diagram.

36.     Click on 'b0'

37.     Click 'OK'

38.     Choose 'Analysis'

39.     Choose 'Perform Analysis'

You will get a message: 'Cylinder Mounting Orientation Must be

specified'

40.     Choose 'OK'

Let's say a mass of 2300kg is permanently attached (cylinder rod

extending and retracting) to cylinder. Cylinder is mounted under 35

degrees slope, and mass is attached to Piston Rod Stud. Friction

coefficient is 0.15.

41.     Click on 'View Orientation Diagrams'

42.     High resolution picture is shown of all mounting orientation

types. Look for above description. It is marked as '3'

43.     Press any key to get out of diagram.

44.     Click on '3'

45.     Click in '[_     ]' next to 'Angle between OX and OZ axis'

46.     Type 35

47.     Click in '[_     ] kg' next to 'Coefficient of friction'

48.     Type 0.15

49.     Click 'OK'

50.     Choose 'Analysis'

51.     Choose 'Perform Analysis'

You will get a message: Hydraulic Line Dimensions must be specified

52.     Choose 'OK'

Let's say that Hudraulic line between Cylinder port's and Solenoid

valve are 6mm outside diameter and 'L' series.

53.     Choose 'Metric'

54.     Click on Outside Diameter Size 6

55.     Click on Series L

56.     Click on 'OK'

57.     Choose 'Analysis'

58.     Choose 'Perform Analysis'

You will get a message: 'Calculation Complete'

(this means that all relevant data has been entered and now you will

see the results of HydraCyl's analysis)

59.     Choose 'OK'

You get a Warning:  SUPPPLY PRESSURE  Required pressure for                              Cylinder's extending is low, consult Page 5.

60.     Observe top left corner of the screen. There is title:

Cylinder Data - Page 4 of 5.

61.     Choose 'Page Down'

62.     Observe Min.Total Pressure Rod Extending = 87.00 bar

Now we will make changes to our specifications to try to

overcome this design problem.

63.     Choose 'Cylinder' (on top left hand side of screen)

64.     Choose 'Operating Parameters'

65.     Change 'Supply pressure to 90 bar.

66.     Click 'OK'

67.     Choose 'Analysis'

68.     Choose 'Perform Analysis'

69.     Choose 'OK'

You will get a Warning: ROD BUCKLING Rod is buckling. Select larger

cylinder or use other engineering methods.

Consult pages 2 and 3.

70.     Observe top left corner of the screen. There is title:

Cylinder Data - Page 4 of 5.

71.     Choose 'Page Up'

72.     Browse pages until you get page 2 on the screen.

73.     Play between pages 2 and 3 and observe values of:

(page 2) Extending Force =  1824 daN

(page 3) Column Strength  =  585 daN

Obviously that actual extending (static) force is almost 4 times

higher than Piston Column strength. Now we will make some changes to

our specifications to try to overcome this design problem.

74.     Choose 'Cylinder' (on top left hand side of screen)

75.     Choose 'Dimensions'

76.     Click on Cylinder Rod diameter 1.375".

(we have just changed cylinder rod diameter from 1" to 1.375" to

try to eliminate buckling effects)

77.     Click 'OK'

78.     Choose 'Analysis'

79.     Choose 'Perform Analysis'

80.     Wait until you get a message 'Calculation complete'

81.     Choose 'OK'

At last NO Warnings. That means all parameters are acceptable and

properly entered. You have selected a suitable cylinder.

Let's look at the analysis results. Use 'Page Up' and 'Page Down' to

browse the inputs and results on the 'Cylinder Data' form, they are as

follows:

Dimensions:

Bore Diameter       2.00  inch       (This is the cylinder

Rod Diameter        1.375 inch        size you have now selected

Stroke             34.00  inch        for your application)

Hydr.Line Dimensions:

Pipe (Tube) Diameter         6      L

Cyl-Valve Distance       150.00  inch

Mounting:

Duty      Heavy

Type  b0  Intermediate trunnion

Orientation  3   Fixed Cylinder Body. Piston Rod retracting and

extending. Load attached to Piston Rod Stud.

Cylinder's Slope      35.00  deg

Friction Coefficient       0.15

Operating Parameters:

System Flow      25.00  lit/min

Extending Speed     205.58  mm/sec

Extending Time       4.20  sec

Retracting Speed     389.83  mm/sec

Retracting Time       2.22  sec

Supply Pressure      90.00  bar

Max Pressure Rating     138.00  bar

Extending Force    1824.00  daN

Retracting Force     962.00  daN

Column Strength    2091.00  daN

Applied Mass    2300.00  kg

Other Analysis Results:

Stop Tube       2.00  inch

Extending Return Flow      13.18  lit/min

Retracting Return Flow      47.41  lit/min

System Power       3.75  KW

Bore Area       3.14  sq.inch

Rod Area       1.48  sq.inch

Annulus Area       1.66  sq.inch

Displacement Volume:

Cylinder Head     0.9230  litre

Cylinder Cap     1.7504  litre

Total     0.8273  litre

Deceleration Device:        ��������������������������������Ŀ

�         Braking Diagram        �

����������������������������������

Warnings:

No Warnings.

Undamped Natural Frequency, Minimum Acceleration/Deceleration Time,

Distance and Pressure.

Natural Frequency Rod Extending            9.9    Hz

Natural Frequency Rod Retracting           9.9    Hz

Minimum Ramp Time Rod Extending            0.38   sec

Minimum Ramp Time Rod Retracting           0.38   sec

Min.Travel Distance Rod Extending          1.6    inch

Min.Travel Distance Rod Retracting         3.0    inch

Min.Total Pressure Rod Extending          87.00   bar

Min.Total Pressure Rod Retracting        -36.00   bar

Conclusions:

============

Stop tube of 2 inch must be mounted in your cylinder.

Although pump (System) delivered flow is only 25 lit/min, command

hydraulics block (solenoid valve and other components) should be

dimensioned on nominal value of flow 47.41 lit/min. If you neglect

this data you could not expect to get all speeds in your application.

When Cylinder's rod is retracted on that side you need 0.923 litre of

oil. When Cylinder's rod is extended on that side you need 1.7504

litre of oil. For proper work of cylinder you need only 0.8273 litre

of oil in your power unit (reservoir). That is total value of oil that

is required by cylinder for proper 'breathing'.

If you are in need to implement proportional device, you should

consider ramp time longer than 0.38 sec. Under those conditions, the

attached load requires at least 1.6 (respecively 3.0) inch travel

distance for its acceleration/deceleration from zero to maximum

(maximum to zero) corresponding speed.

On the page 5, you could read up:

Min.Total Pressure Rod Retracting = -36.00   bar

Due to fact that negative apsolute pressure is nonexistent one (in the

nature), the sign '-' is only pointing to absolute pressure of 36 bar

at the cylinder cap end port generated by attached mass and friction

forces only. In other words, you should have the relief valve in that

line during acceleration and deceleration and adjust its value to 36

bar. In such way you will be able to control acceleration and

deceleration of attached mass during rod retracting.

Stroke-Braking Pressure Chart

By observing this chart you are able to rapidly determine correlation

between generated braking pressure and accompanied braking stroke.

Due to fact that braking pressure at Cylinder's Head End is under 138

bar (maximal pressure rating for choosen cylinder), that side of

Cylinder should be supplied with standard cushion. On the other side,

Cap End should be supplied with standard cushion only if its length is

slightly over 3/4 inch (red and green lines intersection). For that

length, the maximum pressure rating (red line) and generated braking

pressure (green line) are at the same value.

You can easily obtain information on standard cushion length (braking

stroke) and maximal pressure rating from your favourite supplier of

cylinders. Just enter those data on the chart and you will get

information on what would happen in the cylinder. Generally, for any

value of braking stroke between 1/2 inch and 2 inch you are able to

determine actual braking pressure.

APPENDIX A CYLINDER OPERATING PARAMETERS

Dimensions

(A value must be entered)

Stroke

The distance between the cylinder's fully retracted and fully extended

positions.

Units:     mm,  inch

Flow

(Only one value to be entered)

System Flow

The pump flow required to operate the cylinder system at its optimum

potential.

Units:     lit/min,  gpm(US)

Extending Speed

The speed of the cylinder's movable part during the piston's extension.

Units:     mm/sec,  m/min,  inch/sec

Extending Time

The time required by the piston during extension to complete the stroke.

Units:     sec

Retracting Speed

The speed of the cylinder's movable part during the piston's retraction.

Units:     mm/sec,  m/min,  inch/sec

Retracting Time

The time required by the piston during retraction to complete the stroke.

Units:    mm/sec,  m/min,  inch/sec

Pressure

(Only one value to be entered)

Supply Pressure

The pump pressure required to operate the cylinder at its optimum level.

Units:     MPa,  bar,  psi

Extending Force

The total force applied to the head of the cylinder rod during extension.

Units:     kN,  dan,  N,  lbf

Retracting Force

The total force loaded onto the head of the cylinder rod during retraction.

Units:     kN,  daN,  N,  lbf

Other

(A value must be entered in both fields)

Applied Mass

The total mass attached to cylinder piston rod or body.

Units:     kg,  lb

Cyl-Valve distance

The length of pipeline between the cylinder's port and the control valve.

Units:     mm,  inch

APPENDIX B  MOUNTING TYPE CODES

APPENDIX C  Piston Rod-Stroke Charts (medium and heavy duty)

Long stroke fixed or pivot mounted cylinders tend to buckle on push loading

applications, resulting in high bearing loading at the rod gland or the piston.

In order to reduce these bearing loads, the use of a stop tube to lengthen the

distance between the gland and piston when cylinder rod is fully extended is

highly recommended.

The program automatically determines the need for the stop tube. If the stop

tube is required for particular cylinder application its length will be shown

on Cylinder Data - Page 3 of 5 screen. The value of Column Strength is

calculated and shown on the same page. The program compares values of Column

Strength and Extending Force (page 2 of 5) and generates warning message if

buckling is likely on push loading application.

On Figures 45 and 46 graphs represent the column strength of various piston

rods as a function of Stroke and Axial Thrust and show how buckling effects are

incorporated into the program.  Coloured bands are valid ones for corresponding

piston diameters whose sizes are marked with the same colour and shown below

the diagram.  Please note that not all values of piston rod are shown on

diagram, some of them are removed just for the sake of clarity.

Axes on the both figures are:

X - Stroke

Y - Axial Thrust Load

The Stroke is calculated using the equation:

Stroke = AS * SF

where:

AS - Actual Stroke (entered in Parameters screen of the program)

SF - Stroke Factor (based on cylinders Mounting Styles)

All data are extracted and combined from world's manufacturers of

hydraulic cylinders (Parker, Rexroth, Vickers & others).

APPENDIX D  Pipe (Tube) - Charts (Imperial and Metric sizes)

IMPERIAL SIZES according to USAS B36.10 (BS 1600)

METRIC SIZES