BIOS TYPES AND ISSUES

AMI (Quatel) BIOS

AMI Bios dated 4/91 or later will not have any of the

handshaking problems with IDE controllers and hard drives.

The problem seen with earlier versions of Bios and IDE hard

drives was that our tape software had to run in non-concurrent

mode to solve timing issues.

AT&T BIOS

To run the Jumbo tape drives you need to have AT&T Bios 1.21

or greater.

AWARD BIOS

Our tape software must run non-concurrent mode when using

Award Bios previous to version 3.05 with our tape software.

1.   Award Bios 3.05 solved the Conner hard drive/IDE

handshaking problem.

2.   Can run our tape software in concurrent mode.

3.   These problems are only relevant to DOS (No effect on

Unix/Xenix).

To purchase upgrades for Award Bios you may contact the

following:

1.   Pennacoe Sales -- 408-249-7400

2.   Komputer Werks -- 412-782-0384

3.   Unicore -- 800-686-6468 or 509-686-6468

4.   Award Canada -- 416-890-2727

DTK BIOS

There is a timing issue when DTK Bios version 4.26 rev. 4 or

earlier was used in conjunction with IDE hard drives.  This

showed up with our tape software and drives as "Error reading

drive C" during total backups.

DTK Bios can be identified by serial number.  Example:  After

the # sign, the first number designates the CPU type (2 is for

286, 3 is for 386, etc.).  The second number designates

revision number.  Revision 3 or higher is recommended for use

with IDE.  Revision 5 has corrections for the timing problems

seen with IDE hard drives.

Upgrades of this bios are available through Barlett Computers

of Chicago at 708-830-4222.

IBM PC BIOS

To run the Jumbo tape drives you will need IBM PC Bios dated

later than October 1984.  If earlier bios is used you will see

lockups during installation of software.

PHOENIX BIOS

Phoenix Bios dated previous to 1989 will have problems with

IDE controllers and tape drives.  386 version 1.10 runs from

January 1988 through December 1989 and was updated in May of

1989 to handle IDE's.  The 286 Bios versions are up to 3.10

but still falls into the same time constraints as the 236 Bios

versions.