CLEAN BOOT INSTRUCTIONS FOR DOS

The error you are experiencing during tape operations may be caused

by memory resident programs, or other devices, (mouse drivers,

etc.) that may be occupying memory which we may need for our

functions.  In order to eliminate this possibility, test your

system with a system bootable diskette.  To do this, please follow

these instructions.

Start with a clean (blank) diskette: FORMAT A: /S

Once the system has been transferred to the diskette, you will need

to add an AUTOEXEC.BAT & CONFIG.SYS to this diskette.  The

following commands need to be included:

AUTOEXEC.BAT                  CONFIG.SYS

prompt $P$g                   files=20

c:                            buffers=20

cd\tape

tape

Once these files are added to the boot diskette, you will need to

reboot your machine using this diskette.  When rebooting your

machine, DO NOT use CTRL-ALT- DELETE; instead, use your power

switch or the reset button.

CLEAN BOOT INSTRUCTIONS FOR NETWORK

The error you are experiencing during tape operations may be caused

by memory resident programs, or other devices, (mouse drivers,

etc.) that may be occupying memory which we may need for our

functions.  In order to eliminate this possibility, you will need

to test your system with a system bootable diskette.  To do this,

please follow these instructions. NOTE: you must have a local hard

drive.

Start with a clean diskette: FORMAT A: /S

Once the system has been transferred to the diskette, you will need

to add an AUTOEXEC.BAT & CONFIG.SYS to this diskette.  The

following commands need to be included:

AUTOEXEC.BAT                  CONFIG.SYS

prompt $P$g                   files=35

c:                            buffers=25

cd\tape

tape

Once these files are added to the boot diskette, you will need to

reboot your machine using this diskette.  When rebooting your

machine, DO NOT use CTRL-ALT-DELETE or the reset button.  Please

use your power switch instead.

Once you have verified the tape drive is working from the local

hard drive, include "IPX", "NETX" and "LOGIN" commands in the

autoexec.bat to test ability to backup the network hard drive, if

you are using Novell.  If you are using a peer to peer network such

as Lantastic, call your STARTNET batch file. (Please check this

batch file for loading of additional tars prior to calling the

batch file).  If you have a network which requires drivers (such as

IBM Token Ring), please insert the appropriate drivers in your

config.sys file.

If you have any further questions or concerns, please contact our

Customer Support department by telephone or fax for additional

support.