setbus : set AT bus data width for tseng based svga cards to

either 8 or 16 operation.  Most useful for Windows developers using

two monitors for CodeView debugging. By default, if there is a monochrome

(or other second monitor on the AT bus, the svga card operates at only

8 bits doubling video time.

This program allows the user to tell the svga card to use 16-bit data paths

which doubles video perforamce.  This improvement is nearly complete, as

verified by the PC Labs Windows Graphics benchmark test.

Some programs, such as pcANYWHERE IV and Microsoft's CV and CVW, will

only operate when the bus is in 8-bit mode.

Changing the mode within a DOS virtual session under Windows is not

sufficient to allow these programs to operate properly.  SETBUS must

be executed before Windwos for these programs to function.

The source for this code was downloaded from the STB section of the

GRAHICSVENDOR forum on Compuserv.  The original author is unknown.

This is offered as-is. No claims attached.

It has been tested with an Orchid Pro Designer II card, probably works with

STB's Tseng based cards, and may work with other Tseng cards.

Files in this .zip file:

setbus.c        source code.

mkbus.bat       creates setbus.com if you have the correct compiler

setbus.com      executable downloaded from original STB file

setbus.exe      executable created from MSC 6.0

readme.doc      this file

The mkbus.bat file contains the commands needed to complie the program

with some (unknown to me) compiler. I simply used Microsoft C6.00a with the

command:

cl setbus

both the .com and .exe versions appear to work properly.

Uploaded by Pat Farrell, aka pfarrell@gmuvax2.gmu.edu on the Internet.