Windows NT driver for VIPER VLB v1.01

Release Notes

***************************************************************************

Windows NT Driver Installation

------------------------------

To load the drivers for Microsoft NT you must do the following.

1. To customize the .DAT file for your particular card you must run SETMET.BAT

from the DOS prompt inside NT or DOS.  The options are as follows:

SETMEM 2  - For a Viper with 2MB of memory

SETMEM 1  - For a Viper with 1MB of memory (except for rev A4-A7)

SETMEM 1A - For a Viper with 1MB of memory and is rev A4-A7

(128Kx8 VRAM)

You can also get this listing by running SETMEM with out any command

switches

2. You must have Windows NT installed with the Standard VGA driver(640x480x16).

(If you are running a different graphics mode please re-configure for

Standard VGA).

3. Once you have confirmed that you are running in Standard VGA, place the

diskette contain these drivers into the floppy drive.

4. Open the Windows NT setup icon located in your Main program group.

5. Select Options | Change System Settings...

6. Scroll down to the bottom of the Display option and select

"Other Requires manufacturers Diskette."

7. You will be prompted for the location of this diskette. The location is

A:\

8. Change the drive name if you have the driver in other then A: drive

and select CLOSE.

9. Select your preferred resolution.

10. You will be prompted for the location of a OEM Disk, this is the Disk 1

and the path is A:\ (or B:\)

11. Close out of the Windows NT setup, you will need to restart Windows NT

for the changes to take effect.