02200000000801

1

2

9[...............................................................]0

MC 68000 MACHINE LANGUAGE COURSE PART IV by Mark van den Boer

What a pity!!  You missed the mega-surprise of part 3.  Next time

better luck! I am gonna take that holiday to Hawaii myself!

This time I will discuss the Integer Arithmetic Instructions. The

syntax used is of course the same as in part 3,  so when in doubt

refer  to  part  3.  This class of instructions  is  used  to  do

mathematical  calculations.  This  group is very  often  used  by

assembly  language programmers,  especially the instructions  for

adding and subtracting.

Integer Arithmetic Instructions

Instruction:   ADD

Syntax:        ADD Dn,<ea>

ADD <ea>,Dn

Data sizes:    byte, word, long

Condition codes affected:

X, C set by carry out of the most significant bit

N    set if the result was negative, cleared otherwise

Z    set if the result was zero, cleared otherwise

V    set if the result overflowed, cleared otherwise

Addressing modes:

Source: (destination is Dn)

Dn

An

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Add  source  to  destination  and  put  the  result  in

destination.

Examples:

Instruction              Before         After

ADD.W d0,d1              d0=00000011    d0=00000011

d1=0000FFFA    d1=0000000B

XNZVC=00000    XNZVC=11001

ADD.L (a0),d0            d0=00000022    d0=00000027

a0=12345678    a0=12345678

12345678 contains 5

XNZVC=00000    XNZVC=00000

Instruction:   ADDA

Syntax:        ADDA <ea>,An

Data sizes:    word, long

Condition codes affected: None

Addressing modes:

Source:

Dn

An

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#

Destination:

An

Function: Add a value to an address register. This operation does

not change any of the condition code values.  Note that

most  operations  that have an address  register  as  a

destination does not change the condition codes.

Example:

Instruction              Before         After

ADDA.L a0,a0             a0=00000002    a0=00000004

Notice  that this instruction has the same effect as  multiplying

the address register with two (if this was possible).

Instruction:   ADDI

Syntax:        ADDI #,<ea>

This instruction has exactly the same characteristics as the  ADD

instruction, except that the source can only be a constant.

Instruction:   ADDQ

Syntax:        ADDQ #,<ea>

Same story as for ADDI,  except that the immediate values in  the

source  field  can only range from 1 to 8.  Q stands  for  Quick,

since this instruction is the fastest way to add a number from  1

to 8 to a destination operand.

A note on ADD, ADDI, ADDQ:

Most assemblers accept the following instruction: ADD #1,Dn

and will translate it automatically to ADDQ #1,Dn    thus  saving

a few bytes of object code and some clock cycles execution time.

Instruction:   ADDX

Syntax:        ADDX Dn,Dn

ADDX -(An),-(An)

Data sizes:    byte, word, long

Condition codes affected: see ADD

Function: Add  X-bit  and  source to destination  and  store  the

result  in destination.  This instruction is  used  for

multiple  precision  operations and is  therefore  only

available with the two addressing modes mentioned.

Example:

Instruction              Before         After

ADDX.B -(a0),-(a1)       a0=10001001    a0=10001000

a1=10002001    a1=10002000

10001000 contains AA   the same

10002000 contains 5A   10002000 contains 4

X=0            X=1

ADDX.B -(a0),-(a1)        a0=10001000    a0=10000fff

a1=10002000    a1-10001fff

10000fff contains 0    the same

10001fff contains 0    10001fff contains 1

X=1            X=0

In this example the word that begins at 10000fff is added to  the

word that begins at 10001fff.  If one should try to do this  with

two  ADD.W instruction an address error would occur  since  words

always must be aligned to even addresses. This instruction can be

compared to the ADC instruction of the 6502 and 6809.

Instruction:   CLR

Syntax:        CLR <ea>

Data sizes:    byte, word, long

Condition codes affected:

N    always cleared

Z    always set

V    always cleared

C    always cleared

Addressing modes:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Set an effective address to zero. You will have noticed

that you can't CLR an address register.  However,  most

assemblers  allow  the  programmer to  CLR  an  address

register  by  substituting CLR a0 with SUB.L  a0,a0   .

This instruction has exactly the same result.

Example:

Instruction              Before         After

CLR.W d0                 d0=ffffffff    d0=00000000

NZVC=1011      NZVC=0100

Instruction:   CMP

Syntax:        CMP <ea>,Dn

Data sizes:    byte, word, long

Condition codes affected: NZVC (X is not affected)

Addressing modes (source):

Dn

An

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Function: compare an effective address with a data  register.  In

fact  all  condition codes are set as  if  Dn-<ea>  was

performed.  So CMP is kind of a subtraction which  only

affects the conditon codes.

Example:

Instruction              Before         After

CMP.L d0,d1              d0=00000001    d0=00000001

d1=00000002    d1=00000002

NZVC=1111      NZVC=0000

Instruction:   CMPA

Syntax:        CMPA <ea>,An

Data sizes:    word, long

Function: This  instruction  differs only from CMP  in  that  the

second  operand  is an address register and  that  byte

isn't allowed as a data size.

Instruction:   CMPI

Syntax:        CMPI #,Dn

Function: Yes,  it is nearly exactly the same as compare but  now

the first operand must be a constant.

Instruction:   CMPM

Syntax:        CMPM (An)+,(An)+

Function: Again, nearly exactly the same as CMP, but now both the

source  and  destination operand must  be  (An)+.  This

instruction  is used to compare areas  of  memory.  For

those of you who have a working knowledge of C:  strcmp

can be programmed easy with this instruction.

Note on all CMPx instructions.

Most assemblers accept instructions like:

CMP.W (a0)+,(a1)+

CMP.L #3,d0

Substitution of CMPM,  CMPI and CMPA are automatically  performed

by the assembler.

Instruction:   DIVS

Syntax:        DIVS <ea>,Dn

Data sizes:    word

Condition codes affected:

N    behaves normal; undefined on overflow

Z    behaves normal; undefined on overflow

V    behaves normal

C    always cleared

Addressing modes (source):

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#

Function: Guess  what?   This  instruction  performs  a  division

between two signed numbers. The

destination  register  is  always a  longword  and  the

source operand is always a word. After the division the

destination operand contains the result.  The  quotient

is always in the lower word and the remainder is always

in the high order word of the data register! This way a

modulo operation is also performed,  you just SWAP  the

data  register  and you have your result in  the  lower

word  of the data register.  Overflow occurs  when  you

attempt to divide a large number by a small number e.g.

ffffff divided by 1,  the result doesn't fit in a word.

Another error occurs when attempting to divide by zero.

In this case the 68000 generates an exception and  will

trap  to  a special routine which handles  division  by

zero erros.  On the Atari you must set up this  routine

yourself.  E.g.  FLOYD  (a  machine  language  monitor)

responds  to  a  division by zero  with  the  following

sentence "The answer is 42". Remember, don't panic when

you see such an answer.

Example:

Instruction              Before         After

DIVS #3,d0               d0=0000000B    d0=00020003

NZVC=1111      NZVC=0000

Instruction: DIVU

Function: Nearly  exactly the same as DIVS,  only this time  both

operands are assumed to be unsigned.

Instruction:   EXT

Syntax:        EXT Dn

Data sizes:    word, long

Condition codes affected:

N    behaves normal

Z    behaves normal

V    always cleared

C    always cleared

Function: turn  a byte into a word,  or turn a word into a  long.

This  instruction provides a convenient way to  turn  a

word into a long and still have the same value for that

register. If the high order bit of the data register is

0,  so the data register is positive, zeroes are padded

in, otherwise ones are padded in.

Example:

Instruction              Before         After

EXT.W d0                 d0=000000ff    d0=0000ffff

EXT.L d0                 d0=ffff0000    d0=00000000

Instruction:   MULS

Syntax:        MULS <ea>,Dn

Data sizes:    word

Condition codes affected:

N    behaves normal

Z    behaves normal

V    always cleared

C    always cleared

Addressing modes (source):

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#

Function: Ah!  another very handy instruction.  This  instruction

performs a multiplication of the source and destination

operand, putting the result in the destination operand.

Example:

Instruction              Before         After

MULS #3,d0               d0=0000000B    d0=00000021

NZVC=1111      NZVC=0000

Instruction: MULU

Function: Nearly  exactly the same as MULUS,  only this time  both

operands are assumed to be unsigned.

Instruction:   NEG

Syntax:        NEG <ea>

Data sizes:    byte, word, long

Condition codes affected: XNZVC (all behave normal)

Addressing modes:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: negate  an effective address operand.  In a high  level

language it would look like this: a = -a

Example:

Instruction              Before         After

NEG.L d0                 d0=00000001    d0=ffffffff

Instruction:   NEGX

Syntax:        NEGX <ea>

Data sizes:    byte, word, long

Condition codes affected: XNZVC (all behave normal)

Addressing modes:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: negate  an effective address operand and add the  X-bit

to  the  result.  This  is  another  instruction  which

provides a way to handle multi-precision  (e.g.  8-byte

integers).

Example:

Instruction              Before         After

NEGX.L d0                d0=00000001    d0=00000000

X=1            X=1

Instructions: SUB, SUBA, SUBI, SUBQ, SUBX

All these instruction perform subtractions.  They only differ  in

that   way   from   from  the   ADD   instructions,   all   other

characteristics are the same.

Instruction:   TAS

Syntax:        TAS <ea>

Data sizes:    byte

Condition codes affected:

N    evaluated before setting the byte

Z    evaluated before setting the byte

V    always cleared

C    always cleared

Addressing modes:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: First  test  the operand and set the  condition  codes,

then set the high-order bit to 1.  People who know what

semaphores (in programming of course...) are,  immedia-

tely  will love this instruction.  For those who  don't

know what semaphores are: M. Ben Ari has written a good

book  on the subject called "Principles  of  Concurrent

Programming".  Never,  I repeat never,  read a book  on

this subject written by a certain Ir.  E.H.H.  Dijkstra

(not the famous Dijkstra,  this Dijkstra will never  be

famous).

Example:

Instruction              Before         After

TAS $436                 $436=00        $436=80

NZVC=1111      NZVC=0100

TAS $436                 $436=FF        $436=FF

NZVC=1111      NZVC=1000

Instruction:   TST

Syntax:        TST <ea>

Data sizes:    byte, word, long

Condition codes affected:

N    behaves normal

Z    behaves normal

V    always cleared

C    always cleared

Addressing modes:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: test an effective address operand. This instruction can

be  seen as CMP <ea>,d0 where d0 is 0.  TST  is  nearly

always followed by a branch instruction (more on  these

later)

To  the people who also read the last lines (I hope  you've  also

read most of the preceeding ones):  Phone me and tell me how  you

like this course.

My phonenumber (in Holland) is: 013-422397

Originally published in ST NEWS Volume 2 Issue 3.