02200000000801

1

2

9[...............................................................]0

ASSEMBLY LANGUAGE COURSE PART VI by Mark van den Boer

Shift and Rotate Operations

Instruction:   ASL

Syntax:        ASL #,Dn (the immediate operand always modulo 8)

ASL Dn,Dn (the first operand always modulo 8)

ASL <ea>

Data sizes:    byte,  word,  long  except for ASL <ea> which  only

allows word and long as data sizes.

Condition codes affected:

X    set to the last bit shifted out

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    set  if  the most significant bit  is  changed

during the operation

C    see the X-bit

Addressing modes allowed with the ASL <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform  a shift left of the destination  operand.  This

instruction can be used as a fast form of multiplying an

operand  with a power of two.  On a processor  like  the

6502  this  instruction  is the only way  to  perform  a

multiply operation.  The lower bit of the destination is

always set to zero.

Examples:

Instruction              Before         After

ASL.L d0,d1              d0=33333333    d0=33333333

d1=00000005    d1=00000028

ASL.W $4ee               $4ee=0009      $4ee=0012

Instruction:   ASR

Syntax:        ASR #,Dn (the immediate operand always modulo 8)

ASR Dn,Dn (the first operand always modulo 8)

ASR <ea>

Data sizes:    byte,  word,  long  except for ASR <ea> which  only

allows word and long as data sizes.

Condition codes affected:

X    set to the last bit shifted out

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    set  if  the most significant bit  is  changed

during the operation

C    see the X-bit

Addressing modes allowed with the ASR <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform a shift right of the destination  operand.  This

instruction  can be used as a fast form of  dividing  an

operand  with a power of two.  On a processor  like  the

6502  this  instruction  is the only way  to  perform  a

divide  operation.  The upper bit (sign bit)  is  always

repeated.

Examples:

Instruction              Before         After

ASR.L d0,d1              d0=33333333    d0=33333333

d1=00000005    d1=00000002

ASR.W $4ee               $4ee=8009      $4ee=c004

Instruction:   LSL

See the ASL instruction.  The LSL instruction is exactly the same.

At the moment I haven't got the machine codes for the ASL and LSL

operations  but I think that even the machine codes are the  same.

E.g.  on the 6809 both ASL and LSL exist but translate to the same

machine code.

Instruction:   LSR

Syntax:        LSR #,Dn (the immediate operand always modulo 8)

LSR Dn,Dn (the first operand always modulo 8)

LSR <ea>

Data sizes:    byte,  word,  long  except for LSR <ea> which  only

allows word and long as data sizes.

Condition codes affected:

X    set to the last bit shifted out

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    set  if  the most significant bit  is  changed

during the operation

C    see the X-bit

Addressing modes allowed with the LSR <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform a shift right of the destination  operand.  This

instruction differs from ASR in that the high order  bit

is always cleared.

Examples:

Instruction              Before         After

LSR.L d0,d1              d0=33333333    d0=33333333

d1=00000005    d1=00000002

LSR.W $4ee               $4ee=0009      $4ee=0004

Instruction:   ROL

Syntax:        ROL #,Dn (the immediate operand always modulo 8)

ROL Dn,Dn (the first operand always modulo 8)

ROL <ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    always cleared

C    set to the last bit shifted out the operand

Addressing modes allowed with the ROL <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform  a  bitwise  rotate  left  of  the   destination

operand.

Examples:

Instruction              Before         After

ROL.L d0,d1              d0=00000001    d0=00000001

d1=88000001    d1=10000002  (C bit set)

ROL.W $4ee               $4ee=8009      $4ee=0012

Instruction:   ROR

Syntax:        ROR #,Dn (the immediate operand always modulo 8)

ROR Dn,Dn (the first operand always modulo 8)

ROR <ea>

Data sizes:    byte, word, long

Condition codes affected:

X    not affected

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    always cleared

C    set to the last bit shifted out the operand

Addressing modes allowed with the ROR <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform  a  bitwise  rotate  right  of  the  destination

operand.

Examples:

Instruction              Before         After

ROR.L d0,d1              d0=00000001    d0=00000001

d1=88000001    d1=c4000000  (C bit set)

ROR.W $4ee               $4ee=8009      $4ee=c004

Instruction:   ROXL

Syntax:        ROXL #,Dn (the immediate operand always modulo 8)

ROXL Dn,Dn (the first operand always modulo 8)

ROXL <ea>

Data sizes:    byte, word, long

Condition codes affected:

X    set to the last bit shifted out the operand

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    always cleared

C    set to the last bit shifted out the operand

Addressing modes allowed with the ROXL <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform  a  bitwise  rotate  left  of  the   destination

operand.  There  is very little difference with the  ROL

instruction.  By  the way,  it is very handy to  have  a

wordprocessor    with   cut/paste    and    find/replace

facilities.  All  I  did was cut out  the  complete  ROL

instruction and replaced all ROL's by ROXL's.

Examples:

Instruction              Before         After

ROXL.L d0,d1             d0=00000001    d0=00000001

d1=88000001    d1=10000002

ROXL.W $4ee              $4ee=8009      $4ee=0012

Instruction:   ROXR

Syntax:        ROXR #,Dn (the immediate operand always modulo 8)

ROXR Dn,Dn (the first operand always modulo 8)

ROXR <ea>

Data sizes:    byte, word, long

Condition codes affected:

X    set to the last bit shifted out the operand

N    set to the most significant bit of the result

Z    set if the result is zero, cleared otherwise

V    always cleared

C    set to the last bit shifted out the operand

Addressing modes allowed with the ROXR <ea> instruction:

Destination:

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Perform  a  bitwise  rotate  right  of  the  destination

operand.  There  is very little difference with the  ROR

instruction.  By  the way,  it is very handy to  have  a

wordprocessor    with   cut/paste    and    find/replace

facilities.  All  I  did was cut out the  complete  ROXL

instruction and replaced all ROXL's by ROXR's.

Examples:

Instruction              Before         After

ROXR.L d0,d1             d0=00000001    d0=00000001

d1=88000001    d1=10000002

ROXR.W $4ee              $4ee=8009      $4ee=0012

Originally published in ST NEWS Volume 2 Issue 5.