.HM A 1 1 1 1 1 1

.H 1 "Appendix for as6805 Frankenstein Assembler"

.H 2 "Pseudo Operations"

.H 3 "Standard Pseudo Operation Mnemonics"

.VL 40 5 1

.LI "End"

END

.LI "File Inclusion"

INCL

INCLUDE

.LI "If"

IF

.LI "Else"

ELSE

.LI "End If"

ENDI

.LI "Equate"

EQU

.LI "Set"

SET

.LI "Org"

ORG

.LI "Reserve Memory"

RESERVE

RMB

.LI "Define Byte Data"

BYTE

DB

FCB

.LI "Define Word Data"

DW

FDB

WORD

.LI "Define String Data"

FCC

STRING

.LI "Define Character Set Translation"

CHARSET

.LI "Define Character Value"

CHARDEF

CHD

.LI "Use Character Translation"

CHARUSE

.LE

.H 3 "Machine Dependent Pseudo Operations"

.H 4 "Instruction Set Selection"

.DS I N

CPU string

.DE

The instruction set can be specified in the source file with the CPU

pseudooperation.

The string, delimited by quotes or apostrophes, is scanned for a

substring which selects which instruction set is used.

When the program is invoked, this operation is performed on the name of

the program, then the -p optional arguement, if any, and then any CPU

statements.

The last one selects which subset of the instructions the assembler will

accept.

The instruction set can be changed at any place in the source file.

.VL 30 5 1

.LI "Instruction Set"

Substrings

.LI "68hc05"

hc

HC

.LI "146805"

4

.LI "6305"

3

.LI "6805"

05

l05

L05

.LE

.H 2 "Instructions"

.H 3 "Instruction List"

.TS H

;

l l l.

Opcode	Syntax	Selection Criteria

.sp

.TH

.sp

ADC	 '#' expr

ADC	 expr	DIRECT

ADC	 expr	EXTENDED

ADC	 indexed	INDEX0

ADC	 indexed	INDEX1

ADC	 indexed	INDEX2

.sp

ADD	 '#' expr

ADD	 expr	DIRECT

ADD	 expr	EXTENDED

ADD	 indexed	INDEX0

ADD	 indexed	INDEX1

ADD	 indexed	INDEX2

.sp

AND	 '#' expr

AND	 expr	DIRECT

AND	 expr	EXTENDED

AND	 indexed	INDEX0

AND	 indexed	INDEX1

AND	 indexed	INDEX2

.sp

ASL	 expr	DIRECT

ASL	 indexed	INDEX0

ASL	 indexed	INDEX1

.sp

ASLA

.sp

ASLX

.sp

ASR	 expr	DIRECT

ASR	 indexed	INDEX0

ASR	 indexed	INDEX1

.sp

ASRA

.sp

ASRX

.sp

BCC	 expr

.sp

BCLR	 expr ',' expr

.sp

BCS	 expr

.sp

BEQ	 expr

.sp

BHCC	 expr

.sp

BHCS	 expr

.sp

BHI	 expr

.sp

BHS	 expr

.sp

BIH	 expr

.sp

BIL	 expr

.sp

BIT	 '#' expr

BIT	 expr	DIRECT

BIT	 expr	EXTENDED

BIT	 indexed	INDEX0

BIT	 indexed	INDEX1

BIT	 indexed	INDEX2

.sp

BLO	 expr

.sp

BLS	 expr

.sp

BMC	 expr

.sp

BMI	 expr

.sp

BMS	 expr

.sp

BNE	 expr

.sp

BPL	 expr

.sp

BRA	 expr

.sp

BRCLR	 expr ',' expr ',' expr

.sp

BRN	 expr

.sp

BRSET	 expr ',' expr ',' expr

.sp

BSET	 expr ',' expr

.sp

BSR	 expr

.sp

CLC

.sp

CLI

.sp

CLR	 expr	DIRECT

CLR	 indexed	INDEX0

CLR	 indexed	INDEX1

.sp

CLRA

.sp

CLRX

.sp

CMP	 '#' expr

CMP	 expr	DIRECT

CMP	 expr	EXTENDED

CMP	 indexed	INDEX0

CMP	 indexed	INDEX1

CMP	 indexed	INDEX2

.sp

COM	 expr	DIRECT

COM	 indexed	INDEX0

COM	 indexed	INDEX1

.sp

COMA

.sp

COMX

.sp

CPX	 '#' expr

CPX	 expr	DIRECT

CPX	 expr	EXTENDED

CPX	 indexed	INDEX0

CPX	 indexed	INDEX1

CPX	 indexed	INDEX2

.sp

DAA		INSTDAA

.sp

DEC	 expr	DIRECT

DEC	 indexed	INDEX0

DEC	 indexed	INDEX1

.sp

DECA

.sp

DECX

.sp

EOR	 '#' expr

EOR	 expr	DIRECT

EOR	 expr	EXTENDED

EOR	 indexed	INDEX0

EOR	 indexed	INDEX1

EOR	 indexed	INDEX2

.sp

INC	 expr	DIRECT

INC	 indexed	INDEX0

INC	 indexed	INDEX1

.sp

INCA

.sp

INCX

.sp

JMP	 expr	DIRECT

JMP	 expr	EXTENDED

JMP	 indexed	INDEX0

JMP	 indexed	INDEX1

JMP	 indexed	INDEX2

.sp

JSR	 expr	DIRECT

JSR	 expr	EXTENDED

JSR	 indexed	INDEX0

JSR	 indexed	INDEX1

JSR	 indexed	INDEX2

.sp

LDA	 '#' expr

LDA	 expr	DIRECT

LDA	 expr	EXTENDED

LDA	 indexed	INDEX0

LDA	 indexed	INDEX1

LDA	 indexed	INDEX2

.sp

LDX	 '#' expr

LDX	 expr	DIRECT

LDX	 expr	EXTENDED

LDX	 indexed	INDEX0

LDX	 indexed	INDEX1

LDX	 indexed	INDEX2

.sp

LSL	 expr	DIRECT

LSL	 indexed	INDEX0

LSL	 indexed	INDEX1

.sp

LSLA

.sp

LSLX

.sp

LSR	 expr	DIRECT

LSR	 indexed	INDEX0

LSR	 indexed	INDEX1

.sp

LSRA

.sp

LSRX

.sp

MUL		INSTMUL

.sp

NEG	 expr	DIRECT

NEG	 indexed	INDEX0

NEG	 indexed	INDEX1

.sp

NEGA

.sp

NEGX

.sp

NOP

.sp

ORA	 '#' expr

ORA	 expr	DIRECT

ORA	 expr	EXTENDED

ORA	 indexed	INDEX0

ORA	 indexed	INDEX1

ORA	 indexed	INDEX2

.sp

ROL	 expr	DIRECT

ROL	 indexed	INDEX0

ROL	 indexed	INDEX1

.sp

ROLA

.sp

ROLX

.sp

ROR	 expr	DIRECT

ROR	 indexed	INDEX0

ROR	 indexed	INDEX1

.sp

RORA

.sp

RORX

.sp

RSP

.sp

RTI

.sp

RTS

.sp

SBC	 '#' expr

SBC	 expr	DIRECT

SBC	 expr	EXTENDED

SBC	 indexed	INDEX0

SBC	 indexed	INDEX1

SBC	 indexed	INDEX2

.sp

SEC

.sp

SEI

.sp

STA	 expr	DIRECT

STA	 expr	EXTENDED

STA	 indexed	INDEX0

STA	 indexed	INDEX1

STA	 indexed	INDEX2

.sp

STOP		INSTSTWA

.sp

STX	 expr	DIRECT

STX	 expr	EXTENDED

STX	 indexed	INDEX0

STX	 indexed	INDEX1

STX	 indexed	INDEX2

.sp

SUB	 '#' expr

SUB	 expr	DIRECT

SUB	 expr	EXTENDED

SUB	 indexed	INDEX0

SUB	 indexed	INDEX1

SUB	 indexed	INDEX2

.sp

SWI

.sp

TAX

.sp

TST	 expr	DIRECT

TST	 indexed	INDEX0

TST	 indexed	INDEX1

.sp

TSTA

.sp

TSTX

.sp

TXA

.sp

WAIT		INSTSTWA

.TE

.H 3 "Selection Criteria Keywords"

.VL 25 5

.LI DIRECT

The direct addressing mode can be used if the expression has a value

between 0 and 255.

The expression will be treated as a two byte long value if it is not

defined when the statement is processed in the first pass.

.LI EXTENDED

The expression can be a 2 byte long value.

.LI INDEX0

The Indexed addressing mode with no offset can be used.

.LI INDEX1

The Indexed addressing mode with a one byte offset can be used.

.LI INDEX2

The Indexed addressing mode with two byte offset can be used.

.LI INSTSTWA

The instruction is only available in the 146805, 6305, and 68hc05 instruction

sets.

.LI INSTMUL

The instruction is only available in the 68hc05 instruction set.

.LI INSTDAA

The instruction is only available in the 6305 instruction set.

.LE

.H 3 "Apostrophes"

The apostrophes in the syntax field are a notation used for the

parser generator and are not put in the assembler source statement.

.H 2 "Notes"

.H 3 "Bit Numbers"

The bit number expression in the BSET, BCLR, BRCLR, BRSET operations has

to have value defined when the instruction is read in the first pass.

The value must be between 0 and 7.

.H 3 "Indexed Addressing"

The indexed addressing mode is represented in two ways.

.P

First, with no offset, the form ",X" is used.

.P

Second, the form "expression, X" is used for both the one and two byte

offsets modes.

The expression will be treated as a two byte long value if it is not

defined when the statement is processed in the first pass.

.H 3 "Reserved Symbols"

.H 4 "Machine Dependent Reserved Symbols"

X

x

.H 4 "Standard Reserved Symbols"

AND

DEFINED

EQ

GE

GT

HIGH

LE

LOW

LT

MOD

NE

NOT

OR

SHL

SHR

XOR

and

defined

eq

ge

gt

high

le

low

lt

mod

ne

not

or

shl

shr

xor

.TC 1 1 7