.HM A 1 1 1 1 1 1

.H 1 "Appendix for as7000 Frankenstein Assembler"

.H 2 "Pseudo Operations"

.H 3 "Standard Pseudo Operation Mnemonics"

.VL 40 5 1

.LI "End"

END

.LI "File Inclusion"

INCL

INCLUDE

.LI "If"

IF

.LI "Else"

ELSE

.LI "End If"

ENDI

.LI "Equate"

EQU

.LI "Set"

SET

.LI "Org"

ORG

.LI "Reserve Memory"

RESERVE

RMB

.LI "Define Byte Data"

BYTE

DB

FCB

.LI "Define Word Data"

DW

FDB

WORD

.LI "Define String Data"

FCC

STRING

.LI "Define Character Set Translation"

CHARSET

.LI "Define Character Value"

CHARDEF

CHD

.LI "Use Character Translation"

CHARUSE

.LE

.H 2 "Instructions"

.H 3 "Differences with the TI defined Syntax"

.DS

.TS

;

l l.

TI syntax	as7000 syntax

.sp

op % immediate , register	op # immediate , register

op % immed , register, jump dest	op # immed , register , jump dest

op @ label	op label

op @ label ( B )	op expr ( B )

op *register	op [ register ]

movd % immed ( B ) , register	movd # immed (B), register

.sp

Constants

.sp

?0101010	%01010101

>89ab	$89ab

''''	"'"

.sp

Comments

.sp

after the last operand	everything following a semicolon

.sp

Symbols

.sp

R0-R255	equ with value between 0 and $ff

R0-R255	0 to 255, Not Recommended !

P0-P255	equ with value between $100 and $1ff

P0-P255	256 to 511, Not Recommended !

.sp

.TE

.DE

.H 3 "Instruction List"

.TS H

;

l l l.

Opcode	Syntax	Selection Criteria

.sp

.TH

.sp

ADC	 '#' expr ',' expr

ADC	 '#' expr ',' expr	DSTA

ADC	 '#' expr ',' expr	DSTB

ADC	 expr ',' expr

ADC	 expr ',' expr	DSTA

ADC	 expr ',' expr	DSTB

ADC	 expr ',' expr	SRCB DSTA

.sp

ADD	 '#' expr ',' expr

ADD	 '#' expr ',' expr	DSTA

ADD	 '#' expr ',' expr	DSTB

ADD	 expr ',' expr

ADD	 expr ',' expr	DSTA

ADD	 expr ',' expr	DSTB

ADD	 expr ',' expr	SRCB DSTA

.sp

AND	 '#' expr ',' expr

AND	 '#' expr ',' expr	DSTA

AND	 '#' expr ',' expr	DSTB

AND	 expr ',' expr

AND	 expr ',' expr	DSTA

AND	 expr ',' expr	DSTB

AND	 expr ',' expr	SRCB DSTA

.sp

ANDP	 '#' expr ',' expr

ANDP	 expr ',' expr	SRCA

ANDP	 expr ',' expr	SRCB

.sp

BR	 '[' expr ']'

BR	 expr '(' REG ')'	SRCB

BR	 expr

.sp

BTJO	 '#' expr ',' expr ',' expr

BTJO	 '#' expr ',' expr ',' expr	DSTA

BTJO	 '#' expr ',' expr ',' expr	DSTB

BTJO	 expr ',' expr ',' expr

BTJO	 expr ',' expr ',' expr	DSTA

BTJO	 expr ',' expr ',' expr	DSTB

BTJO	 expr ',' expr ',' expr	SRCB DSTA

.sp

BTJOP	 '#' expr ',' expr ',' expr

BTJOP	 expr ',' expr ',' expr	SRCA

BTJOP	 expr ',' expr ',' expr	SRCB

.sp

BTJZ	 '#' expr ',' expr ',' expr

BTJZ	 '#' expr ',' expr ',' expr	DSTA

BTJZ	 '#' expr ',' expr ',' expr	DSTB

BTJZ	 expr ',' expr ',' expr

BTJZ	 expr ',' expr ',' expr	DSTA

BTJZ	 expr ',' expr ',' expr	DSTB

BTJZ	 expr ',' expr ',' expr	SRCB DSTA

.sp

BTJZP	 '#' expr ',' expr ',' expr

BTJZP	 expr ',' expr ',' expr	SRCA

BTJZP	 expr ',' expr ',' expr	SRCB

.sp

CALL	 '[' expr ']'

CALL	 expr '(' REG ')'	SRCB

CALL	 expr

.sp

CLR	 expr

CLR	 expr	DSTA

CLR	 expr	DSTB

.sp

CLRC

.sp

CMP	 '#' expr ',' expr

CMP	 '#' expr ',' expr	DSTA

CMP	 '#' expr ',' expr	DSTB

CMP	 expr ',' expr

CMP	 expr ',' expr	DSTA

CMP	 expr ',' expr	DSTB

CMP	 expr ',' expr	SRCB DSTA

.sp

CMPA	 '[' expr ']'

CMPA	 expr '(' REG ')'	SRCB

CMPA	 expr

.sp

DAC	 '#' expr ',' expr

DAC	 '#' expr ',' expr	DSTA

DAC	 '#' expr ',' expr	DSTB

DAC	 expr ',' expr

DAC	 expr ',' expr	DSTA

DAC	 expr ',' expr	DSTB

DAC	 expr ',' expr	SRCB DSTA

.sp

DEC	 expr

DEC	 expr	DSTA

DEC	 expr	DSTB

.sp

DECD	 expr

DECD	 expr	DSTA

DECD	 expr	DSTB

.sp

DINT

.sp

DJNZ	 expr ',' expr

DJNZ	 expr ',' expr	DSTA

DJNZ	 expr ',' expr	DSTB

.sp

DSB	 '#' expr ',' expr

DSB	 '#' expr ',' expr	DSTA

DSB	 '#' expr ',' expr	DSTB

DSB	 expr ',' expr

DSB	 expr ',' expr	DSTA

DSB	 expr ',' expr	DSTB

DSB	 expr ',' expr	SRCB DSTA

.sp

EINT

.sp

IDLE

.sp

INC	 expr

INC	 expr	DSTA

INC	 expr	DSTB

.sp

INV	 expr

INV	 expr	DSTA

INV	 expr	DSTB

.sp

JC	 expr

.sp

JEQ	 expr

.sp

JGE	 expr

.sp

JGT	 expr

.sp

JHS	 expr

.sp

JL	 expr

.sp

JLT	 expr

.sp

JMP	 expr

.sp

JN	 expr

.sp

JNC	 expr

.sp

JNE	 expr

.sp

JNZ	 expr

.sp

JP	 expr

.sp

JPZ	 expr

.sp

JZ	 expr

.sp

LDA	 '[' expr ']'

LDA	 expr '(' REG ')'	SRCB

LDA	 expr

.sp

LDSP

.sp

MOV	 '#' expr ',' expr

MOV	 '#' expr ',' expr	DSTA

MOV	 '#' expr ',' expr	DSTB

MOV	 expr ',' expr

MOV	 expr ',' expr	DSTA

MOV	 expr ',' expr	DSTB

MOV	 expr ',' expr	SRCA

MOV	 expr ',' expr	SRCA DSTB

MOV	 expr ',' expr	SRCB

MOV	 expr ',' expr	SRCB DSTA

.sp

MOVD	 '#' expr '(' REG ')' ',' expr

MOVD	 '#' expr ',' expr

MOVD	 expr ',' expr

.sp

MOVP	 '#' expr ',' expr

MOVP	 expr ',' expr	DSTA

MOVP	 expr ',' expr	DSTB

MOVP	 expr ',' expr	SRCA

MOVP	 expr ',' expr	SRCB

.sp

MPY	 '#' expr ',' expr

MPY	 '#' expr ',' expr	DSTA

MPY	 '#' expr ',' expr	DSTB

MPY	 expr ',' expr

MPY	 expr ',' expr	DSTA

MPY	 expr ',' expr	DSTB

MPY	 expr ',' expr	SRCB DSTA

.sp

NOP

.sp

OR	 '#' expr ',' expr

OR	 '#' expr ',' expr	DSTA

OR	 '#' expr ',' expr	DSTB

OR	 expr ',' expr

OR	 expr ',' expr	DSTA

OR	 expr ',' expr	DSTB

OR	 expr ',' expr	SRCB DSTA

.sp

ORP	 '#' expr ',' expr

ORP	 expr ',' expr	SRCA

ORP	 expr ',' expr	SRCB

.sp

POP	  STATUS

POP	 expr

POP	 expr	DSTA

POP	 expr	DSTB

.sp

PUSH	  STATUS

PUSH	 expr

PUSH	 expr	DSTA

PUSH	 expr	DSTB

.sp

RETI

.sp

RETS

.sp

RL	 expr

RL	 expr	DSTA

RL	 expr	DSTB

.sp

RLC	 expr

RLC	 expr	DSTA

RLC	 expr	DSTB

.sp

RR	 expr

RR	 expr	DSTA

RR	 expr	DSTB

.sp

RRC	 expr

RRC	 expr	DSTA

RRC	 expr	DSTB

.sp

SBB	 '#' expr ',' expr

SBB	 '#' expr ',' expr	DSTA

SBB	 '#' expr ',' expr	DSTB

SBB	 expr ',' expr

SBB	 expr ',' expr	DSTA

SBB	 expr ',' expr	DSTB

SBB	 expr ',' expr	SRCB DSTA

.sp

SETC

.sp

STA	 '[' expr ']'

STA	 expr '(' REG ')'	SRCB

STA	 expr

.sp

STSP

.sp

SUB	 '#' expr ',' expr

SUB	 '#' expr ',' expr	DSTA

SUB	 '#' expr ',' expr	DSTB

SUB	 expr ',' expr

SUB	 expr ',' expr	DSTA

SUB	 expr ',' expr	DSTB

SUB	 expr ',' expr	SRCB DSTA

.sp

SWAP	 expr

SWAP	 expr	DSTA

SWAP	 expr	DSTB

.sp

TRAP	 expr

.sp

TSTA

.sp

TSTB

.sp

XCHB	 expr

XCHB	 expr	DSTA

XCHB	 expr	DSTB

.sp

XOR	 '#' expr ',' expr

XOR	 '#' expr ',' expr	DSTA

XOR	 '#' expr ',' expr	DSTB

XOR	 expr ',' expr

XOR	 expr ',' expr	DSTA

XOR	 expr ',' expr	DSTB

XOR	 expr ',' expr	SRCB DSTA

.sp

XORP	 '#' expr ',' expr

XORP	 expr ',' expr	SRCA

XORP	 expr ',' expr	SRCB

.TE

.H 3 "Selection Criteria Keywords"

.VL 25 5

.LI DSTA

The instruction will use a short form if the A register is used as the

destination register.

.LI DSTB

The instruction will use a short form if the B register is used as the

destination register.

.LI SRCA

The instruction will use a short form if the A register is used as the

source register.

.LI SRCB

The instruction will use a short form if the B register is used as the

source register.

.LE

.H 3 "Apostrophes"

The apostrophes in the syntax field are a notation used for the

parser generator and are not put in the assembler source statement.

.H 2 "Notes"

.H 3 "Reserved Symbols"

.H 4 "Machine Dependent Reserved Symbols"

A

B

ST

a

b

st

.H 4 "Standard Reserved Symbols"

AND

DEFINED

EQ

GE

GT

HIGH

LE

LOW

LT

MOD

NE

NOT

OR

SHL

SHR

XOR

and

defined

eq

ge

gt

high

le

low

lt

mod

ne

not

or

shl

shr

xor

.TC 1 1 7