.HM A 1 1 1 1 1 1

.H 1 "Appendix for as8051 Frankenstein Assembler"

.H 2 "Pseudo Operations"

.H 3 "Standard Pseudo Operation Mnemonics"

.VL 40 5 1

.LI "End"

END

.LI "File Inclusion"

INCL

INCLUDE

.LI "If"

IF

.LI "Else"

ELSE

.LI "End If"

ENDI

.LI "Equate"

EQU

.LI "Set"

SET

.LI "Org"

ORG

.LI "Reserve Memory"

RESERVE

RMB

.LI "Define Byte Data"

BYTE

DB

FCB

.LI "Define Word Data"

DW

FDB

WORD

.LI "Define String Data"

FCC

STRING

.LI "Define Character Set Translation"

CHARSET

.LI "Define Character Value"

CHARDEF

CHD

.LI "Use Character Translation"

CHARUSE

.LE

.H 3 "Machine Dependent Pseudo Operations"

.H 4 "Define Word Data"

The Define Word Data pseudo-operations generate the byte reversed form

for 16 bit constants.

The first byte is the low order half, followed by the high order byte.

.H 2 "Instructions"

.H 3 "Instruction List"

.TS H

;

l l l.

Opcode	Syntax	Selection Criteria

.sp

.TH

.sp

ACALL	expr

.sp

ADD	A ',' '#' expr

ADD	A ',' '@' REG	R01

ADD	A ',' REG	R07

ADD	A ',' expr

.sp

ADDC	A ',' '#' expr

ADDC	A ',' '@' REG	R01

ADDC	A ',' REG	R07

ADDC	A ',' expr

.sp

AJMP	expr

.sp

ANL	A ',' '#' expr

ANL	A ',' '@' REG	R01

ANL	A ',' REG	R07

ANL	A ',' expr

ANL	C ',' '/' bit

ANL	C ',' '/' expr

ANL	C ',' bit

ANL	C ',' expr

ANL	expr ',' '#' expr

ANL	expr ','A

.sp

CJNE	A ',' '#' expr ',' expr

CJNE	A ',' expr ',' expr

CJNE	'@' REG ',' '#' expr ',' expr	R01

CJNE	REG ',' '#' expr ',' expr	R07

.sp

CLR	A

CLR	C

CLR	bit

CLR	expr

.sp

CPL	A

CPL	C

CPL	bit

CPL	expr

.sp

DA	A

.sp

DEC	'@' REG	R01

DEC	A

DEC	REG	R07

DEC	expr

.sp

DIV	AB

.sp

DJNZ	REG ',' expr	R07

DJNZ	expr ',' expr

.sp

INC	'@' REG	R01

INC	A

INC	DPTR

INC	REG	R07

INC	expr

.sp

JB	bit ',' expr

JB	expr ',' expr

.sp

JBC	bit ',' expr

JBC	expr ',' expr

.sp

JC	expr

.sp

JMP	'@' A '+' DPTR

.sp

JNB	bit ',' expr

JNB	expr ',' expr

.sp

JNC	expr

.sp

JNZ	expr

.sp

JZ	expr

.sp

LCALL	expr

.sp

LJMP	expr

.sp

MOV	'@' REG ',' '#' expr	R01

MOV	'@' REG ',' A	R01

MOV	'@' REG ',' expr	R01

MOV	A ',' '#' expr

MOV	A ',' '@' REG	R01

MOV	A ',' REG	R07

MOV	A ',' expr

MOV	C ',' bit

MOV	C ',' expr

MOV	DPTR ',' '#' expr

MOV	REG ',' A	R07

MOV	REG ',' '#' expr	R07

MOV	REG ',' expr	R07

MOV	bit ',' C

MOV	expr ',' '#' expr

MOV	expr ',' '@' REG	R01

MOV	expr ',' A

MOV	expr ',' C

MOV	expr ',' REG	R07

MOV	expr ',' expr

.sp

MOVC	A ',' '@' A '+' DPTR

MOVC	A ',' '@' A '+' PC

.sp

MOVX	'@' DPTR ',' A

MOVX	'@' REG ',' A	R01

MOVX	A ',' '@' DPTR

MOVX	A ',' '@' REG	R01

.sp

MUL	AB

.sp

NOP

.sp

ORL	A ',' '#' expr

ORL	A ',' '@' REG	R01

ORL	A ',' REG	R07

ORL	A ',' expr

ORL	C ',' '/' bit

ORL	C ',' '/' expr

ORL	C ',' bit

ORL	C ',' expr

ORL	expr ',' '#' expr

ORL	expr ',' A

.sp

POP	expr

.sp

PUSH	expr

.sp

RET

.sp

RETI

.sp

RL	A

.sp

RLC	A

.sp

RR	A

.sp

RRC	A

.sp

SETB	C

SETB	bit

SETB	expr

.sp

SJMP	expr

.sp

SUBB	A ',' '#' expr

SUBB	A ',' '@' REG	R01

SUBB	A ',' REG	R07

SUBB	A ',' expr

.sp

SWAP	A

.sp

XCH	A ',' '@' REG	R01

XCH	A ',' REG	R07

XCH	A ',' expr

.sp

XCHD	A ',' '@' REG	R01

.sp

XRL	A ',' '#' expr

XRL	A ',' '@' REG	R01

XRL	A ',' REG	R07

XRL	A ',' expr

XRL	expr ',' '#' expr

XRL	expr ',' A

.TE

.H 3 "Selection Criteria Keywords"

.VL 25 5

.LI R01

Only the reserved symbols R0 and R1 (or r0, r1) are acceptable as

the REG operand.

.LI R07

All the reserved symbols R0 thru R7 are acceptable as the REG operand.

.LE

.H 3 "Bit Operands"

Bit Addresses can be in two forms.

.P

One form is "expression.constant" where expression gives an address for a

bit addressable register and constant (in the range of 0 to 7) gives the

bit number in the byte.

Expression values must be either in the range of 20h to 2fh, or

the multiples of 8 between 80h and f8h, and defined in the first pass.

.P

The second form is an symbolic expression with value between 0 and 255.

The value of the expression is placed in the operand field of the

instruction without modification.

.H 3 "Apostrophes"

The apostrophes in the syntax field are a notation used for the

parser generator and are not put in the assembler source statement.

.H 2 "Notes"

.H 3 "Address Spaces"

The Four address spaces, Program Memory, Internal Data Memory, Bit Numbers,

and External Data Memory are not differentiated in the symbol table.

Which address space a symbol refers to is determined by the context of

the instruction operands it appears in.

.H 3 "Reserved Symbols"

.H 4 "Machine Dependent Reserved Symbols"

A

AB

C

DPTR

PC

R0

R1

R2

R3

R4

R5

R6

R7

a

ab

c

dptr

pc

r0

r1

r2

r3

r4

r5

r6

r7

.H 4 "Standard Reserved Symbols"

AND

DEFINED

EQ

GE

GT

HIGH

LE

LOW

LT

MOD

NE

NOT

OR

SHL

SHR

XOR

and

defined

eq

ge

gt

high

le

low

lt

mod

ne

not

or

shl

shr

xor

.TC 1 1 7