Atari RGB to Composite Video Converter

RGB TO COMPOSITE VIDEO CONVERTER DOCUMENTATION

December 27, 1986

version 1.0

The following document is placed in the public domain.   You  may

make as many copies of it as you like and transmit it in any form

you want provided it is not sold commercially,  nor  any  product

derived from it is sold commercially. The author is not responsi-

ble for any damage, physical, mental or otherwise caused by  fol-

lowing  the  instructions  given  below.  Please mail corrections

and/or suggestions for improvement to the address given below.

Anees Munshi

58 York Road

Weston, Ontario

M9R 3E6

Canada.

(416) 246-0670

27 Dec 1986             (C) Anees Munshi                        1

Atari RGB to Composite Video Converter

1.  _#I_#N_#T_#R_#O_#D_#U_#C_#T_#I_#O_#N

The schematic described in this document converts the analog

RGB  video  signal  that is output by the ST to an NTSC composite

signal which can be displayed on a colour or monochrome composite

monitor,  or on a TV set by adding a modulator. Please be careful

when building the circuit.  Should anything in the schematics  or

this documentation seem suspicious, use your better judgement.  A

certain amount of experience at building electronic circuits will

be  very  helpful. Also, a good oscilloscope and knowledge of the

theory of RGB to composite conversion may be necessary  in  order

to  debug  the circuit (should you have to). I have tried to pro-

vide some background in this article.

2.  _#A_#N__#O_#V_#E_#R_#V_#I_#E_#W

The MC1377 RGB to composite video  converter  IC  used  does

most of the work in converting the red, green and blue signals to

composite video.  The red, green, blue, horizontal sync and vert-

ical  sync signals are taken from the ST and fed into the 1377. A

colour burst carrier (3.579545 Mhz) is fed  in  from  a  separate

oscillator [1].  The 1377 generates the R-Y,  B-Y  and  luminance

signals  by  passing  the  red,  green and blue signals through a

________

1.

The separate oscillator is not really necessary since 1377

contains a common-collector Colpitts oscillator which can be

used to generate the colour-burst on-chip. However, I found

it easier to generate the signal off-chip to make sure the

thing is indeed oscillating.

27 Dec 1986             (C) Anees Munshi                        2

Atari RGB to Composite Video Converter

matrix.  Then, the B-Y signal is modulated using the colour-burst

frequency  carrier,  and  the  R-Y signal is modulated using a 90

deg. phase shifted carrier.  This results in the  I  and  Q  (in-

phase  and  quadrature)  components of the chroma signal. The two

components of the chroma signal are added and amplified and  made

available  on  pin  13  of the chip. This allows the chroma to be

band-pass filtered [2] externally and then fed back into pin  10.

The  band-pass filter should be centered at the colour-burst fre-

quency and should have a bandwidth of about 1.6 Mhz. Not having a

very good colour TV to experment with, I chose to do an el-cheapo

filter since it wouldn't make any difference on  my  set  anyway.

(Besides,  I  plan to use my board on an old green-screen monitor

to run an occasional colour-only program). A simple second  order

LC  filter may be used. Set the resonant frequency of the tank to

the colour-burst frequency, and choose an appropriate R  so  that

you get the 1.6 Mhz bandwidth  [3].   Introducing  a  second  (or

higher  order) BPF to do chroma-filtering will probably result in

________

2.

If the chroma signal is not band-pass filtered, the low

frequency components it contains (those components having

frequency less than 2Mhz or so) will interfere with the

luminance signal since it is very hard to put the chroma

specral lines exactly in between the luminance spectral lines

without any interference between the two.

3.

Note Filter bandwidth = Wo/Q, where Wo is the resonant

frequency in radians/second (Wo=2*PI*3.15 Mhz) and Q is the

quality factor required. Q=R*sqrt(C/L). A few filters are

sketched in the Motorola Application Notes if you don't want

to design one. A reference to these notes is in the appendix.

27 Dec 1986             (C) Anees Munshi                        3

Atari RGB to Composite Video Converter

a visible delay on  the  chroma-signal.  So,  to  make  sure  the

colours  are  not  offset  from  the  B&W image (like in sloppily

coloured comic books), the luminance signal must  be  delayed  an

equal  amount  so  that the luminance information is not ahead of

the chrominance information.  To allow this, the luminance signal

is  looped  out  from  pin-6  to  pin-8. An approprate delay line

inserted between pin-6 and pin-8 will create the  required  delay

[4].  The luminance signal does not need any filtering.

3.  _#I_#N_#T_#E_#R_#F_#A_#C_#I_#N_#G

The horizontal and vertical synchs from the  Atari  must  be

combined  and  fed  into  the  composite  synch input of the 1377

(pin-2). The HSYNC and VSYNC are taken from the ST's monitor out-

put,  AND-gated and fed into the comp-sync input. This works fine

since the syncs are active low, TTL level signals and satisfy the

(-0.6V, 0.9V) active and (1.7V, 8.2V) inactive threshold levels.

The colour signals, red, green and blue must be capacitively

coupled  through  22uF capacitors and attenuated through 2-4 Kohm

resistors (in series with the input) so as to not interfere  with

the  chip's bias and satisfy the 1Vp-p signal requirement respec-

tively. All the three colour signals output  by  the  ST  have  a

________

4.

The Motorola Application notes show how to hook up a TDK

delay line if you need one.

27 Dec 1986             (C) Anees Munshi                        4

Atari RGB to Composite Video Converter

1.8Vp-p range with a 1.2V DC bias. 1377 inputs: Pin 3: red input;

Pin 4: green input; pin 5: blue input.

The colour-burst carrier signal is generated as shown in the

attached schematic and coupled to pin-17 through a 2.2Kohm resis-

tor and a 0.1uF capacitor in series.

The colour-burst is added after every sync pulse, (burst  is

not  suppressed  after VSYNC) approximately 5.5us (micro-seconds)

after the sync's leading edge and it lasts for approximately  3us

or  10.7  cycles  of  the  carrier. This timing is done by an R-C

timer in a fashion similar to an LM555 operating in astable mode.

A  0.001uF capacitor is connected between pin 1 and ground, and a

51Kohm resistor is connected between pin 1 and an 8.2V  reference

(available  at  pin  16).  When  a  sync occurs, the capacitor is

unclamped from ground and  begins  charging  through  the  51Kohm

resistor  from  8.2V  DC. When the capacitor voltage reaches 1.0V

(approx 5.5us after the sync), the colour-burst carrier is  gated

on.  When the capacitor voltage reaches 1.3V, the colour-burst is

gated off. The capacitor continues to charge  until  the  voltage

reaches  5.0V.  At this point, the capacitor is discharged to 0V.

Clearly, by changing the RC values (and hence the time-constant),

the burst can be made shorter or longer (8 cycles of colour-burst

is the NTSC spec, but I don't think a slightly longer period will

hurt (gives the PLLs in the receiver plenty of time to lock)). If

the time constant (Time const. = R*C) is increased the burst will

occur  later  and last for a longer time. The converse is true if

the time constant is decreased. If the time-constant is made  too

27 Dec 1986             (C) Anees Munshi                        5

Atari RGB to Composite Video Converter

long,  the  ramp  may not reach 5.0V before the next HSYNC, which

will result in the some missing bursts (not  too  good).  If  you

feel  like  it,  put a 50Kohm potentiometer instead of the 51Kohm

resistor. You may be able to get some cheap special effects while

calibrating the pot. :-)

A chroma band-pass filter must be introduced between pins 13

and  10  as  mentioned  before.   You may use the simple bandpass

filter shown in the schematic, or to get better results, use  one

of  the  many  shown in the Application Notes or design one your-

self.  As mentioned before, if you design your  own,  the  filter

should  have  a  bandwidth of about 1.6-2.0 Mhz and a center fre-

quency of 3.58 Mhz. Since most colour TV's have  a  BW  around  3

Mhz,  the BP filter will help reduce cross-talk between the lumi-

nance and chroma components (on some TVs, you would never see the

cross-talk  in all the noise!). If your TV has a comb filter, you

won't need a fancy BP filter (and the corresponding delay  line);

in  this  case,  the  simple  bandpass  filter  sketched  in  the

schematic will do.

Again, as mentioned before, if you do insert a  fancy  band-

pass  filter  and  see a noticeable shift between the outlines of

objects and their colour fill, you will need to put a delay  line

between  pin 6 and 8.  The Motorola Application Notes should help

in this department.

Pin 20 must be grounded to select NTSC operation [5].

________

5.

27 Dec 1986             (C) Anees Munshi                        6

Atari RGB to Composite Video Converter

Pin 19 provides  the  reference  voltage  for  the  voltage-

controlled phase shifter (needed for I-Q phase shifting). It must

be capacitively de-coupled to ground through a  0.01uF  capacitor

to  provide a stable voltage reference at the pin. By pulling the

pin up through a resistor to 8.2V, or by pulling it down  through

a  resistor  to ground, the axes can be tilted to get some colour

adjustment (about 7 degrees, for a slight effect,  according  the

application notes).

A 12V power supply is to be connected to pin 14.  The  power

supply need not be regulated.

The 8.2V DC reference voltage appears at pin  16.   A  0.1uF

cap between pin 16 and ground will provide adequate filtering for

this reference voltage.

Pin 15 is the ground connection.

Pins 11 and 12  are  coupled  through  0.1uF  capacitors  to

ground.   By sourcing or sinking some current through these pins,

whites may be made whiter and the blacks blacker (sounds  like  a

detergent  commercial) by compensating for the balanced modulator

feedthrough thus.

Pin 7 is coupled to ground via a 0.01uF capacitor.

_________________________________________________________________

If pin-20 is connected to the power supply, PAL operation is

selected.  For all you PAL hackers: If you replace the 3.58

Mhz crystal with 4.43 Mhz crystal, and if you have a properly

serrated vsync, you should be able to use this circuit to get

PAL composite. If you do hack and get it working, please

post.  (PAL is the colour system used in England, India and

some other countries.)

27 Dec 1986             (C) Anees Munshi                        7

Atari RGB to Composite Video Converter

Pin 9 is the  composite  video  output.  It  has  an  output

impedance  of about 50ohms. To drive a 75ohm monitor input termi-

nal, put a 25 ohm resistor in series with pin 9 and a 75 ohm  co-

axial  cable.  Connect the other end of the co-axial cable to the

monitor.

27 Dec 1986             (C) Anees Munshi                        8

Atari RGB to Composite Video Converter

4.  _#C_#O_#N_#S_#T_#R_#U_#C_#T_#I_#O_#N

You will need to buy a _#c_#o_#l_#o_#u_#r monitor cable from Atari  [6].

Start by cutting the cable in half. Then, strip about 2 inches of

insulation off the end of the cable.  Now  carefully  remove  the

shielding  without  cutting  the  wires  underneath.  This should

expose 7 wires. Four of the wires are shielded themselves;  these

are the red, green, blue and audio out wires. Strip the shielding

off these wires as well.  The following is a colour-code chart to

help you find the wires:

_#c_#o_#l_#o_#u_#r _#o_#f _#w_#i_#r_#e                  _#s_#i_#g_#n_#a_#l                  _#p_#i_#n#

white                           RED output              7

red                             GREEN                   6

black                           BLUE                    10

yellow                          HSYNC                   9

blue                            VSYNC                   12

green                           AUDIO out               1

brown                           ground                  13

________

6.

If this is not available, you may kludge one up as mentioned

in the Abacus _#I_#n_#t_#e_#r_#n_#a_#l_#s book.)

27 Dec 1986             (C) Anees Munshi                        9

Atari RGB to Composite Video Converter

The pins are labelled as follows (looking at the monitor

output plug from outside). Please test the cable with an

ohm-meter to see if my colour code is applicable to your

cable.

pin 4 -> 0 0 0 0 <- pin 1

0 0 0 0 <- pin 5

0 0 0 0 <- pin 9

0 <---- pin 13

Solder the cable onto the board  you  will  be  using,  then

solder  or  wire-wrap the circuit as shown in the schematic.  Try

to keep the construction as clean as possible. Keep  as  much  of

the  shielded  cables  shielded  as you can. Try to RF shield the

whole enclosure if you can. All the best.

27 Dec 1986             (C) Anees Munshi                       10

Atari RGB to Composite Video Converter

_#B_#i_#b_#l_#i_#o_#g_#r_#a_#p_#h_#y

o#+ Motorola Application Note AN-932

o#+ Motorola Linear Data Book

o#+ _#C_#o_#m_#m_#u_#n_#i_#c_#a_#t_#i_#o_#n _#S_#y_#s_#t_#e_#m_#s , Simon Haykin, Wiley 1983. pp. 168,

169.

27 Dec 1986             (C) Anees Munshi                       11