��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC   Supplement to TheRef(tm) Drive & Controller Listing ��

��������������������������������������������������������������������Ĵ�

�   In "publishing" TheRef(tm), I've often been asked the difference ��

� between the types of drive controllers and recording methods.  I'm ��

� not going to get into that in this document, as it would require a ��

� good sized doc. of it's own.  What I have supplied are diagrams of ��

� the different connectors associated with the technology today.     ��

�                                                                frf ��

����������������������������Ĵ CABLES ������������������������������Ĵ�

�                                                                    ��

�            Controller     Drive 2(or none)       Drive 1           ��

�                                                                    ��

�              1��ͻ ���������1��ͻ ����stripe�����1��ͻ Pins 10-16  ��

� FLOPPY cable  �::������������ô������������������ô� are twisted ��

� with twist    �::������������ô��������������XX��ô� before the  ��

� (control &    �::������������ô������������������ô� connector.  ��

� data, 34 pin) �::������������ô������������������ô�  (7 wires)  ��

�               ��ͼ           ��ͼ                 ��ͼ             ��

�              1��ͻ ���������1��ͻ ����stripe�����1��ͻ Pins 25-29  ��

� ST412 & ESDI  �::������������ô������������������ô� are twisted ��

� Hard Drive    �::������������ô������������������ô� before the  ��

� cable w/twist �::������������ô��������������XX��ô� connector.  ��

� (control)     �::������������ô������������������ô�  (5 wires)  ��

�               ��ͼ           ��ͼ                 ��ͼ             ��

�              1��ͻ �����������stripe�������������1��ͻ (no twists) ��

� ST412 & ESDI  �::���������������������������������ô� Each drive  ��

� Hard Drive    �::���������������������������������ô� has it's    ��

� (data, 20 pin)�::���������������������������������ô� own data    ��

�               ��ͼ                                ��ͼ cable       ��

�                                                                    ��

� IMPORTANT NOTE: Pin #1 on any drive cable SHOULD be indicated by a ��

�                 a colored stripe.  If you should find the stripe   ��

�                 by connector pin 34 (or 20), inspect the whole     ��

�                 cable VERY throughly!                              ��

�                                                                    ��

� DRIVE SELECT  For both Floppy and Hard drives, when the 34 pin     ��

� JUMPERS:      cable has a twist, the device number should be set   ��

�               to the second position.  Drives numbered 0-3, set to ��

�               1, those numbered 1-4, set to 2.  When cables with-  ��

�               out a twist are used, Floppy "A", and(or) Hard drive ��

�               "C" should be set to 1, and the second Floppy and    ��

�               (or) Hard drive should be set to 2.                  ��

�                                                                    ��

� TERMINATORS:  When using more than one drive on a cable (ie; 2FDs  ��

�               or 2HDs), the terminating resistor pack should be    ��

�               left on the drive furthest from the controller, and  ��

�               removed from the drive closest to the controller.    ��

�                                                                    ��

�        NOTE:  On SCSI drives, the Host Adapter also has resistors. ��

�               These are needed to terminate both ends of the bus.  ��

�               Since the SCSI bus can have up to 7 devices attached ��

�               to it, only the Host Adapter and the device farthest ��

�               from it will retain the resistors.  All devices in-  ��

�               between should have theirs removed.                  ��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC                                                     2 ��

��������������������������Ĵ CONNECTIONS ���������������������������Ĵ�

�                                                                    ��

� FLOPPY DRIVES                                      �����Ŀ         ��

�                                     HI/LO DENSITY >�2   1� GND     ��

� The connector on a floppy drive               N/C  �4 _ 3�  |      ��

� consists of 34 conductors.  Both              N/C  �6   5�  |      ��

� control and data use this same              INDEX <�8   7�  |      ��

� cable.  Most cables have a twist    MOTOR ENAB. A >�10  9�  |      ��

� that interchanges pins 10 through    DRIVE SEL. B >�12 11�  |      ��

� 16 at the end of the cable (on       DRIVE SEL. A >�14 13�  |      ��

� drive 1).  Most floppy connect-     MOTOR ENAB. B >�16 15�  |      ��

� ors have a "key" between pins      DIRECTION SEL. >�18 17�  |      ��

� 4 & 6, and 3 & 5, to prevent the        HEAD STEP >�20 19�  |      ��

� cable from being reversed.  At         WRITE DATA >�22 21�  |      ��

� the other end,  the dual row con-      WRITE GATE >�24 23�  |      ��

� nector that attaches to the con-         TRACK 00 <�26 25�  |      ��

� troller card will usually have a    WRITE PROTECT <�28 27�  |      ��

� set of ridges that coincide with        READ DATA <�30 29�  |      ��

� cutouts in the controller card's      HEAD SELECT >�32 31�  |      ��

� connector.  Note that old style       DISK CHANGE <�34 33� GND     ��

� floppy-only controllers used a                     �������         ��

� card-edge connector just like that        > Input   ( At the       ��

� of the drive.                             < Output   Drive Conn.)  ��

�                                                                    ��

� ST506/412 HARD DRIVE  (MFM & RLL)                                  ��

�                                                                    ��

� This standard drive system uses                    �����Ŀ         ��

� two cables; a 34 conductor control     HEAD SEL. 8 �2   1� GND     ��

� cable, and a 20 conductor data         HEAD SEL. 4 �4 _ 3�  |      ��

� cable.  The control cable contains      WRITE GATE �6   5�  |      ��

� a twist of the conductors going to   SEEK COMPLETE �8   7�  |      ��

� the farthest drive, which is drive         TRACK 0 �10  9�  |      ��

� "C" on most systems.  This twist       WRITE FAULT �12 11�  |      ��

� consists of conductors 25 through      HEAD SEL. 1 �14 13�  |      ��

� 29.  As with the floppy cable, the        RESERVED �16 15�  |      ��

� ST506/412 cables normally have a       HEAD SEL. 2 �18 17�  |      ��

� key to prevent reversal, and the             INDEX �20 19�  |      ��

� controller end has a pin-type con-           READY �22 21�  |      ��

� nector, while the drive end has a             STEP �24 23�  |      ��

� card-edge type connector.             DRIVE SEL. 1 �26 25�  |      ��

�                                       DRIVE SEL. 2 �28 27�  |      ��

�                   �����Ŀ             DRIVE SEL. 3 �30 29�  |      ��

�       DRIVE SEL'D �1   2� GND         DRIVE SEL. 4 �32 31�  |      ��

�          RESERVED �3 _ 4�  |          DIRECTION IN �34 33� GND     ��

�             |     �5   6�  |                       �������         ��

�             |     �7   8� GND                                      ��

�          RESERVED �9  10� RESERVED          Though control signals ��

�               GND �11 12� GND               go through a single 34 ��

�     * WRITE DATA+ �13 14� * WRITE DATA-     conductor cable, data  ��

�               GND �15 16� GND               flows through seperate ��

�      * READ DATA+ �17 18� * READ DATA-      20 conductor cables    ��

�               GND �19 20� GND               for each drive (C,D).  ��

� *(MFM or RLL)     �������                                          ��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC                                                     3 ��

��������������������������Ĵ CONNECTIONS ���������������������������Ĵ�

�                                                                    ��

� ESDI HARD DRIVES                                      �����Ŀ      ��

�                                           HEAD SEL. 3 �2   1� GND  ��

� Though ESDI and ST506/412 drives          HEAD SEL. 2 �4 _ 3�  |   ��

� share similar looking cables,              WRITE GATE �6   5�  |   ��

� even to the point of having a        CONFIG/STAT DATA �8   7�  |   ��

� twist, the actual data and con-         TRANSFER ACK. �10  9�  |   ��

� trol signals are very different.            ATTENTION �12 11�  |   ��

� One should never mix components           HEAD SEL. 0 �14 13�  |   ��

� from these two drive types.        SECT/ADD.MK. FOUND �16 15�  |   ��

� While the ST506/412 interface             HEAD SEL. 1 �18 17�  |   ��

� utilizes a standard pulse code                  INDEX �20 19�  |   ��

� to transmit data between the                    READY �22 21�  |   ��

� drive and controller, ESDI uses         TRANS.REQUEST �24 23�  |   ��

� a pulse code that does not require       DRIVE SEL. 1 �26 25�  |   ��

� the level to return to zero between      DRIVE SEL. 2 �28 27�  |   ��

� pulses.  This format is refered to       DRIVE SEL. 3 �30 29�  |   ��

� as NRZ, or Non Return to Zero.  By          READ GATE �32 31�  |   ��

� utilizing NRZ, the clock that data       COMMAND DATA �34 33� GND  ��

� is transfered by can be increased,                    �������      ��

� thereby increasing the troughput to                                ��

� and from the ESDI disk.                                            ��

�                                        �����Ŀ                     ��

�                            DRIVE SEL'D �1   2� SECT/ADD.MK. FOUND  ��

�                          SEEK COMPLETE �3   4� ADDRESS MARK ENABLE ��

�                   RESV'D FOR STEP MODE �5   6� GND                 ��

�                           WRITE CLOCK+ �7   8� WRITE CLOCK-        ��

�                      CARTRIDGE CHANGED �9  10� READ REF. CLOCK+    ��

�                       READ REF. CLOCK- �11 12� GND                 ��

�                        NRZ WRITE DATA+ �13 14� NRZ WRITE DATA-     ��

�                                    GND �15 16� GND                 ��

�                         NRZ READ DATA+ �17 18� NRZ READ DATA-      ��

�                                    GND �19 20� GND                 ��

�                                        �������                     ��

�                                                                    ��

� ��������������Ŀ And in this corner... Recording ����������������� ��

�                                                                    ��

�     Times were, you had a simple choice for type of disk drive...  ��

� Any kind, as long as it was ST506/412.  Those were the heydays of  ��

� MFM drives.  But many manufacturers weren't content with the 17    ��

� sectors/track that MFM provided.  They devised a newer encoding    ��

� scheme to pack data tighter, and called it RLL, or Run Length      ��

� Limited, as opposed to MFM, or Modified Frequency Modulation.  It  ��

� involves using groups of 16 bits rather than each individual bit,  ��

� thus achieving a sort of "compression" of the information as it is ��

� encoded.  Since the same information takes up less space as RLL    ��

� encoded data, more info can be writen to the disk.  The most com-  ��

� mon RLL technique, known as 2,7 RLL, can pack roughly 50% more on  ��

� a disk than MFM.  Of course, there is always a trade-off, and the  ��

� timing and media required for RLL is it.  RLL requires a higher    ��

� grade of media because of it's dense bit-packing, and timing is    ��

� more critical, since the data is flowing at 50% higher rate than   ��

� an MFM drive.  Also, the mechanics of the drive must have tighter  ��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC                                                     4 ��

��������������������������������������������������������������������Ĵ�

� tolerences because head positioning becomes more critical.  These  ��

� requirements kept RLL drives at a premium.  It has only been the   ��

� last two years, that RLL drives have outsold MFM, and have all but ��

� wiped them from the marketplace.  This turnabout has come from the ��

� need to increase disk capacity more and more.  Both ESDI, and SCSI ��

� type drives utilize RLL.(1*) encoding to achieve high capacity and ��

� transfer rates (from the disk).  And the newest interface, IDE, or ��

� Integrated Drive Electronics, is also based on this technology.    ��

� ������������������������������Ŀ ��������������������������������� ��

�                                                  �����Ŀ           ��

� SCSI HARD DRIVES                           DB0 <>�2   1� GND    5  ��

�                                            DB1 <>�4   3�  |     0  ��

� The normal internal cable for SCSI         DB2 <>�6   5�  |        ��

� is a 50 conductor ribbon, with all         DB3 <>�8   7�  |     P  ��

� odd numbered conductors grounded.          DB4 <>�10  9�  |     I  ��

� Two conductors, numbers 25 & 26, are       DB5 <>�12 11�  |     N  ��

� often left not-connected, as they          DB6 <>�14 13�  |        ��

� deal with Terminator power, and can        DB7 <>�16 15�  |     D  ��

� be easily shorted by cable reversals.      DBP <>�18 17�  |     U  ��

� There are no twists in this cable,         GND   �20 19�  |     A  ��

� and it's length may be a maximum of        GND   �22 21�  |     L  ��

� 6 meters.  But one is advised to use       GND   �24 23�  |        ��

� minimum lengths to improve timing.      TERM PWR �26 25�  |     R  ��

� Up to seven drives, or devices may be      GND   �28 27�  |     O  ��

� attached to an SCSI cable.  Each is        GND   �30 29�  |     W  ��

� daisy-chained on the cable, or, when       ATN  >�32 31�  |        ��

� a device has two connectors, another       GND   �34 33�  |     C  ��

� cable may be "spliced" into the chain      BSY <>�36 35�  |     O  ��

� starting at the second connector, and      ACK  >�38 37�  |     N  ��

� continued on.  Care must be taken to       RST <>�40 39�  |     N  ��

� insure that cables and connectors are      MSG < �42 41�  |     E  ��

� not reversed, as this would short pin      SEL <>�44 43�  |     C  ��

� 26 (TERMPWR) to ground, and likely         C/D < �46 45�  |     T  ��

� damage the drive or controller.  Also,     REQ < �48 47�  |     O  ��

� as explained earlier, the terminating      I/O < �50 49� GND    R  ��

� resistors should remain only on the              �������           ��

� controller (Host Adapter) and the LAST           ��Ŀ DB-25F CONN. ��

� drive on the cable, regardless of it's     GND   �1 ��Ŀ           ��

� address.                                   DB1 <>�2  14�<> DB0     ��

�    Most SCSI Host Adapters also have       DB3 <>�3  15�<> DB2     ��

� a connector for external drives in the     DB5 <>�4  16�<> DB4     ��

� form of a Centronics(tm) type 50 pin,      DB7 <>�5  17�<> DB6     ��

� or an "alternate", DB-25F connector.       GND   �6  18�<> PARITY  ��

� Only the internal 50-pin, and the          SEL <>�7  19�   GND     ��

� "alternate" external connector are         GND   �8  20�<  ATN     ��

� shown here.  (see also: MORE SCSI)       TMPWR   �9  21� > MSG     ��

�    Also, these diagrams refer to the       RST <>�10 22�<  ACK     ��

� single-ended SCSI connections, since       C/D < �11 23�<> BSY     ��

� this is the most common arrangement        I/O < �12 24� > REQ     ��

� for PCs today.  The Differential SCSI      GND   �13 25�   GND     ��

� requires balanced lines, and is used             �  ����           ��

� mostly on high-end workstations.                 ���� FUTURE DOMAIN��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC                                                     5 ��

����������������������������Ĵ CABLES ������������������������������Ĵ�

� SCSI (cont.)                                                       ��

�                          (T)                       ��(DC)�    (T)  ��

� On an SCSI cable, the  1��ͻ�stripe�1��ͻ��1��ͻ��1�����ͻ��1��ͻ  ��

� terminating resistors   �::����������::����::����::�::����::�  ��

� (T) remain at the END   �::����������::����::����::�::����::�  ��

� devices on the cable,   �::����������::����::����::�::����::�  ��

� even when 2 cables are  �::����������::����::����::�::����::�  ��

� "Daisy-Chained" (DC).   �::����������::����::����::�::����::�  ��

� Also, the external      �::����������::����::����::�::����::�  ��

� connector may be used,  ��ͼ         ��ͼ   ��ͼ   �����ͼ   ��ͼ  ��

� requiring the removal    (HA)          Drives 1-7 (in any order)   ��

� of the Host Adapter's                                              ��

� internal Term. resistors.                                          ��

��������������������������Ĵ CONNECTORS ����������������������������Ĵ�

�                                                                    ��

� IDE (AT) HARD DRIVES                       (<> AT THE DRIVE CONN)  ��

�                                                  �����Ŀ           ��

� IDE, or Integrated Drive Electronics       RST  >�1   2�   GND     ��

� is the most recent drive interface to      SD7 <>�3   4�<> SD8     ��

� gain popularity.  Often, the control       SD6 <>�5   6�<> SD9     ��

� circuitry is built into the mother-        SD5 <>�7   8�<> SD10    ��

� board, eliminating the requirement for     SD4 <>�9  10�<> SD11    ��

� a seperate Host Adapter.  There are 2      SD3 <>�11 12�<> SD12    ��

� types of IDE interfaces...those for the    SD2 <>�13 14�<> SD13    ��

� 8-bit XT bus, and those for the 16-bit     SD1 <>�15 16�<> SD14    ��

� AT bus (detailed here).  The cable for     SD0 <>�17 18�<> SD15    ��

� IDE contains 40 conductors and has no      GND   �19 20�N/C (KEY)  ��

� twists.  Like an SCSI cable, the IDE      RES.N/C�21 22�   GND     ��

� cable uses a Dual-row Pin connector for    IOW  >�23 24�   GND     ��

� both ends.  A single cable may be used     IOR  >�25 26�   GND     ��

� to connect two drives, or two cables      RES.N/C�27 28�N/C RES.   ��

� may be Daisy-Chained.  Most IDE Host      RES.N/C�29 30�   GND     ��

� Adapters will support two hard drives.    IRQ14 <�31 32�> I/O CS16 ��

� The first drive should be jumpered as      SA1 <>�33 34�<> PDIAG   ��

� the Master drive, and the second as the    SA0 <>�35 36�<> SA2     ��

� Slave drive.  Plug-in IDE Host Adapters    CS0  >�37 38�<  CS1     ��

� are often called Paddle-Boards, and      ACTIVE <�39 40�   GND     ��

� may contain a floppy controller, and             �������           ��

� serial and parallel ports.                                         ��

�                                                                    ��

����������������������������Ĵ CABLES ������������������������������Ĵ�

� Note:                                                              ��

�                        1��ͻ����stripe�����1��ͻ�����������1��ͻ   ��

� The IDE Host Adapter    �::�����������������::�������������::�   ��

� connector may be on     �::�����������������::�������������::�   ��

� a plug-in Paddle-Board  �::�����������������::�������������::�   ��

� or may be integrated    �::�����������������::�������������::�   ��

� on the Motherboard.     �::�����������������::�������������::�   ��

�                         ��ͼ                ��ͼ            ��ͼ   ��

�                         Host Adapter       Drives 1-2 (any order)  ��

��������������������������������������������������������������������Ĵ�

�  1*  There ARE some SCSI drives that utilize MFM, but very few.    ��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC                                                     6 ��

�����������������������Ŀ More on Recording ������������������������Ĵ�

�                                                                    ��

� WRITE PRECOMPENSATION                                              ��

�                                                                    ��

�    OK, so we've all seen it listed, and maybe even had to set it   ��

� in the CMOS.  So what IS it?  And what does it do?                 ��

�    PreComp. is the way in which the electronics compensates for    ��

� eventual "drift" of the magnetic domains written on the disk.  A   ��

� simple explaination is that it allows the head to space bits that  ��

� would attract each other, further apart, while it puts those that  ��

� repel each other, closer together.  It does this by analyzing the  ��

� data stream, and adjusting the timing for each bit, to allow it to ��

� be recorded earlier or later, if needed.                           ��

�    Not all disks require you to set their PreComp value.  Those    ��

� that do are asking for a cylinder to start PreComp. at.  Since the ��

� packing of the bits on a disk increases as you get closer to the   ��

� center of the disk (higher cylinders), the requirement for PreComp.��

� increases too.  The PreComp. value specified by the Manufacturer   ��

� for a disk is his way of insuring your long term data stability.   ��

�                                                                    ��

�         ��< THE EFFECT OF PRECOMPENSATION OVER TIME >��            ��

�                                                                    ��

�    When recorded (w/o PreComp)      When recorded (with PreComp)   ��

� ������������������������������Ŀ  ������������������������������Ŀ ��

� � +-   -+   +-   +-   -+   -+  �  �   +-  -+  +-   -+    -+    -+� ��

� ��������������������������������  �������������������������������� ��

�                                                                    ��

�     After time (w/o PreComp)          After time (with PreComp)    ��

� ������������������������������Ŀ  ������������������������������Ŀ ��

� �+-    -+    +- +-     -+ -+   �  �  +-   -+   +-   -+   -+   -+ � ��

� ��������������������������������  �������������������������������� ��

�                                                                    ��

�    From the figures above, we can see how a slight amount of Pre-  ��

� Compensation can insure long term stability.  The disk that didn't ��

� employ PreComp was eventually unreadable.  Of course, this would   ��

� take time to happen, but no one can give cold hard specs on how    ��

� much drift will occure.  (Of course, this example is a gross sim-  ��

� plification of the process, but, hey, who's counting?)             ��

�                                                                    ��

�����������������������Ĵ For Notes & Such �������������������������Ĵ�

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

�                                                                    ��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

� DIAGRAMS.DOC                                                     7 ��

��������������������������������������������������������������������Ĵ�

� APPLE SCSI                                                         ��

�                                                                    ��

�   Unlike in the PC world, the Apple          APPLE DB-25 SCSI      ��

� standardized on one drive interface,             ����Ŀ            ��

� SCSI.  Also, Apple standardized on         REQ < �1   �           ��

� a 25 pin connector for external con-       MSG < �2  14�   GND     ��

� nections.  However, Apple decided not      I/O < �3  15� > C/D     ��

� to implement the complete ANSI spec.,      RST <>�4  16�   GND     ��

� so one must be careful that peripherals    ACK  >�5  17�<  ATN     ��

� used are certified to work with Apple's    BSY <>�6  18�   GND     ��

� SCSI bus.                                  GND   �7  19�<> SEL     ��

�   Apple also developed it's own pin-       DB0 <>�8  20�<> PARITY  ��

� configuration.  The Apple and Future       GND   �9  21�<> DB1     ��

� Domain 25-pin SCSI connectors are as       DB3 <>�10 22�<> DB2     ��

� close to "Standards" as there are in       DB5 <>�11 23�<> DB4     ��

� the world of PCs.  But the real ANSI       DB6 <>�12 24�   GND     ��

� Standard called for a 50 pin connector     DB7 <>�13 25�  TMPWR    ��

� commonly referred to as a "Centronics"           �    ��           ��

� type (made popular by the Centronics             ������            ��

� printer company).  Instead of the 25                               ��

� staggered pins of the Apple & Future       50-PIN "CENTRONICS"     ��

� Domain type connectors, the Centronics         TYPE SCSI           ��

� type uses 2 parallel rows of 25 pins.            �����Ŀ           ��

� This arrangement allowed use of extra            �1  26�           ��

� grounds for better isolation.                    �2  27�           ��

�                                                  �3  28�           ��

�                                                  �4  29�           ��

�                                                  �5  30�           ��

�                                                  �6  31�           ��

�                                                  �7  32�           ��

�                                                  �8  33�           ��

�              (WORK IN PROGRESS)                  �9  34�           ��

�                                                  �10 35�           ��

�                                                  �11 36�           ��

�                                                  �12 37�           ��

�                                                  �13 38�           ��

�                                                  �14 39�           ��

�                                                  �15 40�           ��

�                                                  �16 41�           ��

�                                                  �17 42�           ��

�                                                  �18 43�           ��

�                                                  �19 44�           ��

�                                                  �20 45�           ��

�                                                  �21 46�           ��

�                                                  �22 47�           ��

�                                                  �23 48�           ��

�                                                  �24 49�           ��

�                                                  �25 50�           ��

�                                                  �������           ��

�                                                                    ��

�                                                                    ��

�                          THE END?                                  ��

���������������������������������������������������������������������ٱ

����������������������������������������������������������������������#