ZORG, Copyright Ludovic Rousseau 1993, 1994

FAT

The files are cut into clusters and not in sectors. The FAT (File

Allocation Table) is in fact a table of connected pointers to the clusters

of the files. For every cluster on the disk there is a corresponding

value in the FAT. This value can indicate several possible states: the

cluster is free, the cluster is part of a file, the cluster is the last

cluster of a file, or the cluster is damaged.

The files are not necessarily written contiguously on the disk but can

be in several pieces spread out randomly over the disk. On should be able

to 'trace' a file, and the FAT makes this possible. The fact that files

may be cut into clusters of several sectors makes it possible that files

cannot get too fragmented, which would cause the disk heads to move back

and forth a lot if clusters were only one

sector in size. At the same time it ensures that no more than 1023 bytes

are lost in the last cluster of a file, with a sector size of 512 bytes.

One must make a compromise between large clusters and smal clusters. The

remark can be made that RAM disks have clusters of 1 sector, because in

that case, the movement of the head has no importance whatsoever, because

there is no disk head.

The first cluster of a file is indicated in its directory entry. For

this cluster there is a corresponding value in the FAT. This value

indicates the next cluster of the file. Similarly, this cluster also

corresponds to a value in the FAT and so on until the last cluster.

On a floppy disk, a value in the FAT occupies 12 bits, which is 1.5

byte. Like the boot sector, the same rules as for MS-DOS floppies are

used, so it is in little endian format. The little endian format is the

format used by Intel processors (among others). In this format, the bytes

are written in the order least significant byte first and most significant

last. For example $1234 is written as $34 followed by

$12. The Motorola 680x0 processors use the big endian format in which

$1235 is written as $12 $34, which is much easier to read for human

beings.

Reading and writing 12 bits in little endian format is complicated. It

is best explained using 3 bytes, which is 2 entries in the FAT.

+---+---+---+---+---+---+

| A : B | C : D | E : F |

+---+---+---+---+---+---+

The bytes are cut into nibbles. Each character represents a nibble. The

characters A and B form the first byte.

These three bytes correspond with the entries formed by the nibbles 0D

AB and 0E FC.

+--------<--------+

|                 |

+---+---+---+---+  +---+---+  +---+---+---+---+

| 0 : D | A : B |  | C : D |  | 0 : E | F : C |

+---+---+---+---+  +---+---+  +---+---+---+---+

|                      |

+---------->-----------+

For a hard disk it is much simpler, because every entry occupies 2

bytes. It is sufficient to exchange the least significant byte with the

most significant byte to create a big endian value that can be understood

by a Motorola 68xxx processor.

The first 2 entries in the FAT are not used. On a floppy disk, there

values are $FF9 and $FFF and they occupy the first three bytes ($F9 $FF

$FF). $F9 indicates the format of the diskette, but it is never used. The

two $FF's have to point to the end of the disk, but are also not used. For

a hard disk, the first two entries occupy 4 bytes ($00 $00 $00 $00) and

don't have any particular function.

The first cluster of a disk is therefore cluster number 2. This cluster

starts on the disk, just after the end of the root directory.

Values of the entries in the FAT of a floppy :

- $000 indicates that the cluster does not belong to any file;

- $001 ???;

- from $002 to $FF0 : the cluster is part of a file and this value is

the number of the next cluster of the file;

- from $FF1 to $FF7 : the cluster is not used. In general this concerns

an error of the floppy media in one of the sectors in a cluster. This

cluster is unreliable and should not be used;

- from $FF8 to $FFF : the cluster is the last one of a file. The exact

end of the file can be calculated from the size of the file in bytes as

indicated in its directory entry.

For a hard disk, the values can range from $0000 to $FFFF

- $0000 the cluster is free;

- a value from $0002 to $FFF0 indicates the next cluster of the file;

- from $FFF1 to $FFF7 the cluster is damaged;

- from $FFF8 to $FFFF the cluster is the last one in a file.

ZORG has a function with which you can display a list of clusters used

by a file ('Information' / 'Tracer un Fichier'/'Trace a File').

ZORG can manage disks that have an arbitrary number of sectors per

cluster, so not just 1 or 2 sectors per cluster. So you could easily

imagine a partition of 128 Mb having 32768 clusters of 8 sectors each. The

problem is that the system does not read clusters but sectors and that

the system call used 'long Rwabs( int rwflag, void *buffer, int count, int

devno )' only allows you to indicate sectors with a 16 bit

value so a maximum of 65535 sectors for an unsigned value.

Therefore these two problems arise:

- all the routines must use unsigned values and I am not sure if all of

my routines do this.

- there is a limit to partitions of 65536 * 512 = 32 Mb or 32768 * 512 =

16 Mb with a 15 bit (signed) counter (used by TOS versions before 1.04).

The solution is to use sectors larger than 512 bytes. You can thus

create partitions upto 65536 * 32768 = 2 Gigabyte in size with sectors of

32768 bytes.

Despite this all, you can read ACSI disks of 1073741824 bytes (a giga-

byte) using direct access to the disk controller. This direct access

allows you to give a sector number of 21 bits. The problem is that this

access is not performed by the system, but by my own routines and

therefore they can't work with all disks. On TOS 2.0 and up a system call

'int DMAread(long sector, int count, void *buffer, int devno)' exists that

allow you to specify a sector in a 32 bit long value.

The solution to this problem of a limitation in the number of sectors is

to not use Rwabs() (limited to 16 bits) but to use either DMAread() on a

TOS 2.0 (TT or Falcon) or a custom routine. Then we have another problem.

DMAread() does not read a sector on a partition but on the disk as a

whole. It is therefore necessary to supply ZORG with the parameters of the

disks organization into partition so it can know where a partition on the

disk begins. These parameters are supplied with the ZORG.INF file.

I must caution you for the use of a custom routine. The routine I use

for sector reading is based on an example from the 'Disquette et Disque

Dur' book from Micro Application. There is no guarantee whatsoever that

this routine will work with all disks. It works flawlessly with my

MegaFile 30 but it could cause problems with bizarre disks. I can not

recommend you often enough to verify that everything works well, before

starting to really use ZORG.