ZORG, Copyright Ludovic Rousseau 1993, 1994

HARD DISK BOOT

The diskettes and the harddisk partitions have a boot sector. This boot

sector gives information about the format of the diskette or partition. On

a harddisk a 'super boot sector' exists, that gives information, not just

for a single partition, but for the whole harddisk.

This super boot sector can be found, as usual, on sector 0 of the disk.

Do not confuse sector 0 of the first partition with sector 0 of the disk.

Sector 0 of the first partition is in fact sector 2 (at least in my case)

of the disk.

One can read or write this super boot sector with the entries

'Lire'/'Read' and 'Ecrire'/'Write' of the 'Informations' menu or by using

direct access to the disk.

The boot sector is split up into bytes:

Byte  Size   Label

$1B6  1      hd_cc     number of cylinders

$1B8  1      hd_dhc    number of heads

$1B9  1      hd_unused unused

$1BA  2      hd_rwcc   reduce write current cylinder

$1BC  2      hd_wpc    write precomp cylinder

$1BE  1      hd_lz     landing zone

$1BF  1      hd_rt     seek rate code

$1C0  1      hd_in     interleave factor

$1C1  1      hd_spt    sectors per track

$1C2  4      hd_siz    total size of the disk in sectors

$1C6  1      p0_flg    partition 0 exists if pf_flg > 0

if bit 7 is set, the booting takes place

here

$1C7  3      p0_id     identification of the partition

$1CA  4      p0_st     sector number of the first sector of the

partition

$1CE  4      p0_siz    size of the partition in sectors

$1D2  1      p1_flg    ditto, for partition 1

$1D3  3      p1_id

$1D6  4      p1_st

$1DA  4      p1_siz

$1DE  1      p2_flg    ditto, for partition 2

$1DF  3      p2_id

$1E2  4      p2_st

$1E6  4      p2_siz

$1EA  1      p3_flg    ditto, for partition 3

$1EB  3      p3_id

$1EE  4      p3_st

$1F2  4      p3_siz

$1F6  4      bsl_st    starting sector of the "bad sector list"

$1FA  4      bsl_cnt   size of the list in sectors

This information comes from the book 'Disquette et Disque Dur'. This

book dates from 1986 and in those times disks of more than 4 partition

couldn't possibly exist (at least not at Atari :-) ).

With AHDI 5 and 6, disks of more than 4 partitions can be managed in the

following way:

There is no way to put a description for additional partitions in the

boot sector. Therefore one of the four partitions has the identification

code XGM instead of GEM which is used for a normal partition.

This XGM partition arranges several partitions (GEM and/or XGM). The

first sector and the size of a partition are given using 32 bits, so an

XGM partition can have more than 4 billion sectors and therefore contain

quite large partitions.

The first sector of an XGM partition has the same function as a 'super

boot sector'. The sector contains a description of 4 new partitions. These

can be either GEM or XGM. The starting sectors of the sub-partitions are

relative to the XGM partition that contains them. For example the first

sub partition starts at sector 1 so its real first sector is in fact 1 +

the first sector of the XGM partition.

This trick removes the limit of four partitions. The next limit is the

number of descriptors possible for the partitions. AHDI permits the

management of a maximum of 16 partitions and the GEMDOS does very well

upto 32.

Illustration with an example: if you have 5 partitions on your hard

disk, you will have 3 GEM partitions and 1 XGM partition. The XGM

partition contains 2 other GEM partitions. An XGM partition points to a

boot sector of the same type as the first boot sector, which means, a

sector that contains a description of the partitions.

Partition n� : type

0: GEM

1: GEM

2: GEM

3: XGM -> 0: GEM

1: GEM

Still another kind of mutant partition exists that has the

identification code BGM. This type of partition is used for partitions

larger than 16 Mb. Such a partition can no longer use sectors of 512

bytes, because the number of sectors has to fit in 15 bits. So, the

sectors must be bigger, ranging from 1024 bytes to 32768 bytes which means

you can have partitions as large as 1 Gb.

The size of the partition indicated in the boot sector is given in 512

byte sectors. In fact, if you use DMAread() and DMAwrite(), you always

work with 512 byte sectors, because these system calls count the sectors

in 32 bits. It is therefore always possible to access any partition by a

low level disk access and sectors of 512 bytes.

The 'bad sector list' is written on the disk after formatting. It

contains the list of defective sectors that couldn't be formatted. The

book tells us that this list is mostly found at the end of the disk, but

it can be found on sector 2 of my disk. The sectors marked in the list are

always used, in the sense that they appear in the FAT and can be read,

possibly with an error, but at least one can try.

With the variable pi_flg, the operating system checks of the partition

exists (pi_flg not equal to zero). The operating system boots from the

first partition that has bit 7 of pi_flg set.