1

HRAM

Version 1.3

User's Guide

2

COPYRIGHT

This HRAM documentation and the software are copyrighted with all

rights reserved.  Under the copyright laws, neither the

documentation nor the software may be copied, photocopied,

reproduced, translated, or reduced to any electronic medium or

machine-readable form, in whole or in part, without the prior

written consent of Biologic, except in the manner described in

this manual.  The unregistered version of HRAM and accompanying

documentation may be freely copied and distributed.

Copyright (C) Biologic 1990-1992

All rights reserved.  First edition printed 1990.  Printed in the

United States.

Software License Notice

Your license agreement with Biologic, which is included with the

product, specifies the permitted and prohibited uses of the

product.  Any unauthorized duplication or use of HRAM in whole or

in part, in print, or in any other storage and retrieval system

is forbidden.

Licenses and Trademarks

386MAX is a registered trademark of Qualitas, Inc.  QEMM is a

registered trademark of Quarterdeck Office Systems, Inc.  MS-DOS

is a registered trademark of Microsoft Corporation.  HRAM is a

trademark of Biologic.

Biologic

POB 1267

Manassas VA

22110

USA

703-368-2949

800-736-8088 (orders only)

703-361-8251 (fax)

70033,1324   (compuserve)

3

CONTENTS

License Agreement and Disclaimer of Warranty

Introduction

Notes on Memory and Memory Addresses

Testing Your Memory with Chkmem

Shadow Memory

Notes for 8088 and 80286 PCs

Notes for 80386 PCs

Condensed Instructions

hramdev.sys

hram.exe

hram.sys

Your Turn

4

LICENSE AGREEMENT AND DISCLAIMER OF WARRANTY

License agreement

The terms of this license agreement apply to you and to any

subsequent licensee of this HRAM software.  Biologic retains the

ownership of this copy of HRAM software.  This copy is licensed

to you for use under the following conditions.

You may use the HRAM software on any compatible computer,

provided the HRAM software is used on only one computer and by

one user at a time.

You may not provide use of the software in a computer service

business, network, timesharing, multiple CPU or multiple user

arrangement to users who are not individually licensed by

Biologic, except that you may designate any employee to use

such products on a one employee per license basis.

You may not disassemble or decompile the HRAM software.

Disclaimer of warranty

Biologic excludes any and all implied warranties, including

warranties of merchantability and fitness for a particular

purpose.  Biologic does not make any warranty of representation,

either express or implied, with respect to this software program,

its quality, performance, merchantability, or fitness for a

particular purpose.  Biologic shall not have any liability for

special, incidental, or consequential damages arising out of or

resulting from the use of this program.

5

INTRODUCTION

Description

HRAM is a powerful memory management program for 8088, 8086,

80286, 80386 and 80486 PCs that enhances the utilization of upper

memory (memory between 640K and 1024K).  It works in conjunction

with DOS 5 to create up to 96K of extra conventional memory and

up to 224K of upper memory for use by device drivers (such as

network drivers) and memory resident programs (TSRs).  In

addition, it provides many of the necessary memory management

features DOS 5 left out:

HRAM creates upper memory on 8088, 80286, 80386, and 80486 PCs

(DOS 5 creates upper memory on 80386 and 80486 PCs only.)

HRAM utilizes the shadow RAM capability of many chip sets to

create upper memory (the file chipset.doc contains a complete

listing of supported chip sets).  This may eliminate the need

to use the DOS 5 program, emm386.exe (or any other 80386

expanded memory manager), resulting in a speed increase of 10

to 40 percent!

On 80386 and 80486 PCs, HRAM provides more upper memory than

DOS 5 by thoroughly searching the area between 640K and 1024K

and converting ALL unused regions to upper memory--memory that

DOS 5 often overlooks.

HRAM gives you an extra 64K of upper memory for program

initialization by using expanded memory for loading drivers

and TSRs--this means you'll often load programs you couldn't

load before.

HRAM optimizes your upper memory by  automatically determining

which order and in which memory region your drivers and TSRs

should be loaded. This one step can easily double the

utilization of your upper memory. (DOS 5 suggests that you

"experiment with different combinations and orders of

programs".  When you consider that your upper memory may

consist of several separate regions and that you probably use

up to a dozen drivers and TSRs, this process, without HRAM,

could require rebooting your PC thousands of times!)

Overview

HRAM is a set of programs that consists of:

hramdev.sys, a device driver that manages upper memory and can

6

fill unused upper memory areas with expanded memory or shadow

RAM.

hram.exe, a program that works in conjunction with the DOS

LOADHIGH command to optimally load TSRs into the upper memory

created by hramdev.sys.  hram.exe also provides a status

report of upper memory and lists the programs that have been

loaded into it.

hram.sys, a special driver that works in conjunction with the

DOS DEVICEHIGH command to optimally load device drivers into

upper memory.

A utility program, Chkmem, is included in the package and

provides information about the status of memory in your PC.

Requirements

A PC with a 8088, 8086, 80286, 80386 or 80486 microprocessor.

IBM Personal Computer Disk Operating System (PC-DOS) or

Microsoft Disk Operating System (MS-DOS) version 5.0 or

greater.

On 8088 and 80286 PCs, expanded memory version 4 or shadow

RAM.

Contents of the HRAM disk

The following files are included in the root directory of the

distribution disk.

hramdev.sys

hram.exe

hram.sys

chkmem.exe

read.me

There may also be a \freeware directory which contains

unregistered versions of our other software products.

7

NOTES ON MEMORY

AND MEMORY ADDRESSES

Conventional Memory

Conventional memory is located between 0K and 1024K and is the

maximum amount of memory that can be addressed by the 8088

microprocessor on which the IBM PC is based.(1)  The designers

of the original PC divided conventional memory space into a 640K

block of memory to be used by MS-DOS programs and a 384K block of

upper memory for system hardware and ROM.  The term, conventional

memory, is sometimes used to refer just to memory from 0 to 640K.

Upper Memory

Upper memory is located between 640K and 1024K was originally

intended to be reserved for system hardware and ROM.  Since the

amount of free conventional memory between 0K and 640K is

critical to the performance of your PC, DOS 5 utilizes upper

memory blocks (UMBs) and provides a way of moving device drivers

and TSRs out of conventional memory and into upper memory (with

the DEVICEHIGH and LOADHIGH commands).  DOS 5 will use upper

memory only if a program which provides UMBs (like hramdev.sys or

emm386.sys) has been loaded and "dos=umb" is in your config.sys

file.

High Memory Area

The high memory area (HMA) is a 64K region starting at 1024K

(1M).  Usually, part of DOS is loaded into this area to free up

low DOS memory (if "dos=high" is in your config.sys file).  This

region is not upper memory, and DEVICEHIGH and LOADHIGH do not

load programs into it.

Expanded Memory

Expanded memory is "paged" memory that can exist in 8088, 80286,

and 80386(2) based PCs.  As the memory requirements of programs

grew, expanded memory was developed to support up to 32M of

memory by swapping small blocks or "pages" of extra memory into

the 1M address space of the 8088 processor so that only a few

pages are addressable at a time.  Memory pages that are not in

use are stored as deactivated pages on an expanded memory board.

The original expanded memory specification, EMS version 3.2, was

developed jointly by Lotus, Intel, and Microsoft.  Soon after,

AST and Ashton-Tate developed a similar, but enhanced,

____________________

(1) 1K equals 1,024 bytes. 1024K equals 1,048,576 bytes or 1

megabyte.

(2) Throughout the remainder of this documentation anything

said about 80386 PCs applies equally to 80486 PCs.

8

specification called EEMS.  These two specifications were

replaced by EMS version 4.0, which incorporated features of both

specifications.

Below is a summary of a few important features:

An expanded memory page is 16K in size.

Expanded memory that conforms to the version 3.2 specification

can be mapped only into a 64K region in upper memory called

the page frame.  EMS version 4.0 supports the mapping of

expanded memory into any location in conventional memory.

A software control program, called an expanded memory manager,

manages the placement of memory pages, and is typically loaded

by the config.sys file when your PC is turned on or restarted.

Extended Memory

Intel's newer processors, starting with the 80286, are capable of

addressing memory above 1M--extended memory.  Unfortunately, this

memory is not addressable when these processors are in a

processor state called real mode (a mode designed to maintain

compatibility with the 8088) and cannot be fully utilized when

using a real mode operating system such as MS-DOS.

The problem of accessing extended memory while running MS-DOS has

spawned several solutions:

On PCs that support extended memory, a feature of the ROM BIOS

allows programs to copy data between conventional and extended

memory.  Although programs cannot execute in extended memory,

they can use it as storage space.  Programs that take

advantage of this feature include RAM disks, print spoolers,

and 80286 expanded memory simulators.

Protected mode run-time environments which allow an

application program to execute in protected mode under MS-DOS.

80386 memory managers, like VRAM/386, that use the page

mapping capability of the 80386 processor to convert extended

memory into expanded memory and provide additional

conventional memory for use by MS-DOS.

Memory Addresses

Memory addresses and ranges are typically specified using

hexadecimal (hex) numbers.  Hex numbers include the digits 0-9

and the letters A-F, giving 16 possible values for each hex

digit.

Conventional memory can be divided into 64 pages of 16K each.

These pages can be referred to as page 0, page 1, page 2, and so

on, or, as is the case in this manual, by their hexadecimal

segment addresses.  Using this notation, page 0 is at segment 0,

9

page 1 is at segment 0400, page 2 is at segment 0800, etc.  The

following table lists some page numbers and their corresponding

segment address and linear address:

Segment      Linear

Page         Address     Address               ________________________________

0         0000             0K

1         0400            16K

2         0800            32K

3         0c00            48K

4         1000            64K

5         1400            80K

8         2000           128K

12         3000           196K

40         a000           640K

44         b000           704K

63         fc00         1,008K

The first 40 pages (640K) of conventional memory are used by MS-

DOS and the other 24 pages are reserved for system hardware and

ROM.  The exact usage of the upper 384K of conventional memory

depends on the hardware configuration of your machine; typically,

several blocks of this area are not used and are simply empty

space.

The table below lists the memory map of a typical PC:

Address

Range        Description                             ________________________________________

0000-9fff    low DOS memory (640K)

a000-bfff    VGA display adapter (128K)

c000-c7ff    hard disk controller (32K)

c800-cfff    unused address space (32K)

d000-dfff    expanded memory page frame (64K)

e000-efff    unused address space (64K)

f000-ffff    ROM

10

TESTING YOUR MEMORY WITH CHKMEM

The Chkmem program is a memory utility which displays the types

and amounts of memory in your PC.  With it, you can determine the

current status of the upper memory area.  Chkmem can also

identify the chip set in your computer.

Enter the command [chkmem] (don't type the brackets) to display

the amount of upper memory that can be created on your PC.

Example output is shown below:

Biologic Chkmem, version 1.3

copyright (c) Biologic 1990-1992. all rights reserved.

655360 bytes conventional memory

0 bytes extended memory

351232 bytes available XMS memory

655360 bytes expanded memory

0 bytes upper memory

0 bytes upper memory blocks (UMBs) can be converted to

upper memory

0 bytes expanded memory can be converted to upper memory

163840 bytes unused areas or shadow ram can be converted to

upper memory

163840 bytes upper memory can be created

If the last line in the listing, "bytes upper memory can be

created", is not 0, then HRAM can create and/or manage upper

memory on your PC.

8088 and 80286 PCs:  If the listing shows that the only

available source of upper memory is "unused areas or shadow

RAM", as in the example above, then your computer must have a

chip set that is supported by HRAM or you must add expanded

memory to your system.  The file, chipset.doc, contains a list

of chip sets that are supported and the section, "Shadow

memory", describes how to determine which chip set you have.

Read the section, "Notes for 8088 and 80286 PCs" for more

information on installing expanded memory.

8088 and 80286 PCs:  If your chip set is not supported by HRAM

and only 65536 bytes (64K) of expanded memory can be converted

to upper memory, your expanded memory may not be fully

compatible with the EMS version 4.0 specification or it may

need to be configured differently.  Read the section, "Notes

for 8088 and 80286 PCs" for more information on configuring

your expanded memory.

11

SHADOW MEMORY

Most PCs are designed so that a 384K segment of memory, located

between 640K and 1024K, is reserved for shadowing ROM.  When your

PC is started, the shadow memory at ROM locations is activated

and the ROM code is copied to it.  The remaining shadow memory is

not activated and is not used.

HRAM locates these regions of unused shadow memory, activates

them, and converts them to upper memory.  This means:

HRAM can create upper memory on 80286 PCs.

HRAM can create upper memory on 80386 PCs without using

emm386.exe or any other expanded memory manager.  This is

an important advantage because 80386 memory managers (like

emm386, 386MAX, and QEMM) must switch the 80386 chip to a

mode which causes your PC to run slower than normal.  If

you don't need expanded memory, you can use HRAM to create

upper memory, and your PC will run at full speed.

Determining your chip set

The amount of upper memory created from shadow memory, and how it

is activated, depends on which chip set is in your PC.  Since

HRAM cannot automatically determine which chip set you have, you

must determine this yourself and specify the chip set on the

hramdev.sys command line (using the /t<chip set number> option).

There are several ways to determine your chip set:

Look for it on the setup screen or in the documentation for

your PC.

Use the [chkmem /t] command.  The chkmem program will

attempt to identify the chip set.  It is possible that

chkmem will not be successful. This does not mean your

shadow memory cannot be utilized--you just need to identify

it another way.  (Your PC should be rebooted after using

this command).

Ask the manufacturer of your PC.

Take off the cover and look.  The chip set will consist of

one or more chips (large or small) identified by names

and/or numbers.  Try to match the number(s) with those

listed in the file 'chipset.doc'.

12

NOTES FOR 8088 AND 80286 PCS

1. HRAM can utilize the shadow RAM capability of many chip sets

to create upper memory blocks; expanded memory is not

required.  The file, chipset.doc, contains a list of chip sets

HRAM currently supports.

2. HRAM can also convert expanded memory to upper memory,

provided your expanded memory is hardware compatible with EMS

version 4.0.  If the Chkmem program reported that only 65536

bytes of upper memory can be created from expanded memory,

then your expanded memory is either not hardware compatible

with EMS 4 or it needs to be reconfigured.  Consult the

documentation for the board or contact your computer dealer to

make this determination (note: many memory boards are software

compatible with EMS 4 but are not fully compatible on a

hardware level).

3. If your expanded memory is hardware compatible with EMS 4 and

Chkmem reports that only 65536 bytes of upper memory can be

created from expanded memory, then you need to reconfigure

your expanded memory by changing the line in your config.sys

file that loads the expanded memory manager for the board.

Run the command [chkmem /c] to display a recommended command

line for your expanded memory manager.

4. If your expanded memory is not hardware compatible with EMS 4,

you can still create upper memory by using the hramdev /f

option.  This option will give you 64K of upper memory, but

will make your expanded memory unavailable to other programs.

13

NOTES FOR 80386 PCS

1. HRAM can utilize the shadow RAM capability of many chip sets

to create upper memory blocks.  If you do not need to convert

extended memory to expanded memory, the expanded memory

manager, emm386.exe, is not required.  The file, chipset.doc,

contains a list of chip sets HRAM currently supports.

2. If HRAM does not support the shadow RAM capability of your

chip set or if you need expanded memory, the program,

emm386.exe, should be loaded in your config.sys file.

14

CONDENSED INSTRUCTIONS

Creating Upper Memory

Processor:  80386, 80486

From:       shadow memory

When:       HRAM supports your chip set and you do not need expanded

memory

1. Add the line [dos=umb] to your config.sys file (its location

in your config.sys file is not important).

2. Add the line [device=hramdev.sys /t<chipset>] to your

config.sys file (<chipset> is the number identifying the chip

set in your computer).  This line should be located after the

line [device=himem.sys] and near the beginning before any

other [device=] lines.

3. Reboot your PC and run the Chkmem program.  Chkmem should

report a non-zero value for "bytes upper memory".  Enter the

command [chkmem /h] to display the size and location of the

upper memory regions in your PC.

Processor:  80386, 80486

From:       expanded memory manager (emm386.exe)

When:       HRAM does not support your chip set or you need expanded

memory

1. Add a [device=] line to your config.sys file which loads your

expanded memory manager.  An expanded memory manager is a

single file which, typically, has "EMM" in its name.  If this

line already exists in your config.sys file, you may need to

change the parameters so that your expanded memory is

configured correctly.

DOS 5 provides an expanded memory manager, emm386.exe, for use

on 386 PCs.  You should run the [chkmem /c] command to list

the recommended command line for emm386.exe, and add this line

to your config.sys file (or modify it if it already exists).

This line should be located immediately after the line

[device=himem.sys].

2. Add the line [dos=umb] to your config.sys file (its location

in your config.sys file is not important).

3. Add the line [device=hramdev.sys] to your config.sys file.

This line should be located after the line [device=emm386.exe]

and near the beginning before any other [device=] lines.

4. Reboot your PC and run the Chkmem program.  Chkmem should

report a non-zero value for "bytes upper memory".  Enter the

command [chkmem /h] to display the size and location of the

upper memory regions in your PC.

15

Processor:  8088, 80286

From:       shadow memory

When:       HRAM supports your chip set

1. Add the line [dos=umb] to your config.sys file (its location

in your config.sys file is not important).

2. Add the line [device=hramdev.sys /t<chipset>] to your

config.sys file (<chipset> is the number identifying the chip

set in your computer).  This line should be located after the

line [device=himem.sys] and near the beginning before any

other [device=] lines.

3. Reboot your PC and run the Chkmem program.  Chkmem should

report a non-zero value for "bytes upper memory".  Enter the

command [chkmem /h] to display the size and location of the

upper memory regions in your PC.

Processor:  8088, 80286

From:       expanded memory

When:       HRAM does not support your chip set and you have EMS 4

expanded memory in your computer.

1. Add a [device=] line to your config.sys file which loads your

expanded memory manager.  An expanded memory manager is a

single file which, typically, has "EMM" in its name.  If this

line already exists in your config.sys file, you may need to

change the parameters so that your expanded memory is

configured correctly.

Run the command [chkmem /c] to list a recommended command line

for the expanded memory manager that was provided with your

memory board.  You should add this line to your config.sys

file (or modify it if it already exists).  Since expanded

memory managers are different for every board, the syntax of

this command may differ slightly from what is displayed by

[chkmem /c].  You should consult the documentation for your

memory board for information on the exact syntax of this

command.

2. Add the line [dos=umb] to your config.sys file (its location

in your config.sys file is not important).

3. Add the line [device=hramdev.sys /e] to your config.sys file.

This line should be located after the line that loads your

expanded memory manager and near the beginning before any

other [device=] lines.

4. Reboot your PC and run the Chkmem program.  Chkmem should

report a non-zero value for "bytes upper memory".  Enter the

command [chkmem /h] to display the size and location of the

upper memory regions in your PC.

16

Optimizing upper memory

Upper memory is utilized in two ways: (1) programs that recognize

this area will automatically take advantage of it, and (2) TSRs

and device drivers can be loaded into it with the DOS commands

DEVICEHIGH and LOADHIGH.

The process of loading programs high is complicated by the fact

that upper memory is a relatively small area, which may consist

of several regions of different sizes, and that, when loaded,

programs have an initialization size which may be much larger (or

smaller) than their resident size.  Loading programs in a

different order or into different memory regions can

significantly increase the number of programs you are able to

load high.

HRAM provides features which allow you to gain optimum use of

your upper memory: it automatically determines the resident size

and initialization size of each of your drivers and TSRs, it

calculates the best configuration for your upper memory, it

allows you to load a program into a specific region, and it

provides an extra 64K of memory for program initialization by

temporarily adding expanded memory to upper memory.

Follow the steps below to optimize your memory:

1. Add the /s option to the [device=hramdev.sys] line in your

config.sys file and add a new line as shown below:

device=hramdev.sys /s

device=hram.sys on

2. Reboot your PC.

3. Type the command [hram /l] to list the analysis of your

drivers and TSRs.

4. Type the command [hram /c] to list recommended commands for

loading your programs high and add these lines to your

config.sys and autoexec.bat files.

5. Remove the /s option from the [device=hramdev.sys] line in

your config.sys file and reboot your PC.

6. Use the [hram /m] command to list the programs that have been

loaded into upper memory.

17

HRAMDEV.SYS

Command reference

The format of the hramdev.sys command is

device=d:\path\hramdev.sys [options]

[options]

specifies the optional hramdev.sys parameters described in the

following section.

The following options can appear in the hramdev.sys command.

/e

Convert expanded memory to upper memory.  HRAM will fill

unused region between 640K and 1024K with expanded memory and

convert them to upper memory blocks.  Your expanded memory

must be hardware compatible with EMS 4 and be capable of being

mapped to these regions.  The command [chkmem /e] will display

a list of mappable memory segments (16K).  The page frame is

not used for upper memory (see /f option).

/f

Use the expanded memory page frame for upper memory.  The page

frame is a 64K buffer in the upper memory area into which

expanded memory pages are mapped.  Although EMS version 4.0

supports mapping of expanded memory to areas outside the page

frame, most software programs, at a minimum, require the page

frame to be available.  This option will create an additional

64K of upper memory, but will, in most cases, prevent other

programs from utilizing expanded memory.

/i<addressrange>

Include address range.  The memory range specified by

<addressrange> will be converted to upper memory.  It must be

above a000 (hex).  <addressrange> is specified with

hexadecimal segment addresses which must be multiples of 16K

(0000, 0400, 0800, 0c00, 1000, 1400, ...).  For example, to

include the range c000 to c800 use the parameter [/ic000-

c800].  Multiple include ranges should be separated by commas.

For example, [/ic000-c800,e000-f000].

If /i is used, only the memory areas with the include range(s)

will be available as upper memory.

This option is useful only if there are mappable areas in

upper memory that should not be under the control of

hramdev.sys.  Normally, this option should not be used;

hramdev.sys will automatically convert all available memory

(excluding the page frame) into upper memory.

/k

Pause after loading (so you can read memory map screen).

18

/n

No pause on error.  If hramdev.sys reports an error, it waits

for you to press a key.  This option causes hramdev.sys to

continue without waiting for a key.

/p

Do not use expanded memory for program initialization.

Normally, HRAM provides extra memory for use by programs

during their initialization by temporarily adding 64K of

expanded memory to upper memory.

/r

Use previously activated RAM.  HRAM will convert any RAM it

finds into upper memory (except a000-afffh and b000-b7ffh).

Use this option with caution since there may be RAM present in

the upper memory area (such as a network card buffer) that

should not be converted to upper memory.

This option might be useful if you have shadow memory and

hramdev.sys fails to load after a warm boot.

/s

Get size of programs.  HRAM will calculate the resident size

and initialization size of every driver and TSR that is loaded

after it and write this information to the file

"\hram0000.dat" for use by the [hram /l] command (The HRAM

status must be "on"--see "Condensed Instructions").  Programs

will not be loaded high when this option is used.

/t<chipset number>

Use shadow memory.  HRAM will convert shadow memory to upper

memory.  Since HRAM cannot automatically determine which chip

set you have, you must specify the chip number on the

hramdev.sys command line (e.g. [hramdev.sys /t1] for the C&T

NEAT Chip set).  See the section, "Shadow Memory", for

instructions on how to determine which chip set you have.

/x<addressrange>

Exclude address range.  This option prevents hramdev.sys from

using a particular range of addresses.  <addressrange> is

specified with hexadecimal segment addresses which must be

multiples of 16K.  For example, to exclude the range c000 to

c800 use the parameter [/xc000-c800].  Multiple exclude ranges

should be separated by commas.  For example, [/xc000-

c800,e000-f000].

19

HRAM.EXE

Displaying regions and programs in upper memory

The command, [hram /m], will display a status report of the upper

memory created by hramdev.sys.  As illustrated in the following

example, the report lists the location and size of each upper

memory region as well as the TSRs and drivers that have been

loaded:

Biologic HRAM, version 1.3

copyright (c) Biologic 1990-1992. all rights reserved.

region  address                size

------  ---------  ----------------

0       b002-b7fd    32704 ( 31.9k)

1       c802-dfff    98272 ( 96.0k)

region  address                size  program

------  ---------  ----------------  -------

0       b003-b01b      400 (  0.4k)  (character device) setverxx

0       b01d-b122     4192 (  4.1k)  (character device) con

0       b124-b67d    21920 ( 21.4k)  (character device) smartaar

0       b684-b6e0     1488 (  1.5k)  c:\util\unblink.com

0       b6e7-b7d9     3888 (  3.8k)  c:\util\calc.com

0       b7db-b7fd      560 (  0.5k)  (avail)

0                      128 (  0.1k)  other allocated blocks

1       c803-cba0    14816 ( 14.5k)  (character device) ms$mouse

1       cba7-cdcc     8800 (  8.6k)  c:\util\anarkey.com

1       cdd3-cdfe      704 (  0.7k)  c:\util\fastkey.com

1       ce05-ce27      560 (  0.5k)  c:\util\scrnsave.com

1       ce29-dfff    73072 ( 71.4k)  (avail)

1                      192 (  0.2k)  other allocated blocks

expanded memory page frame located at: e000

hram: off

Note that each region is identified by a number, starting with 0,

and that the report indicates into which region each TSR or

device driver has been loaded.  The amount of available space

remaining in each region is listed also.

The command, [hram /a], will list the raw memory allocations in

upper memory.

Loading TSRs into upper memory

hram.exe, itself, does not load programs into upper memory, it

works in conjunction with DOS to improve the function of the

LOADHIGH command.  hram.exe provides a way to load a program into

a specific memory region and provides an extra 64K for program

initialization by temporarily converting expanded memory to upper

memory.  To illustrate, suppose you wish to load a TSR program,

20

called Notepad, into upper memory.  The command you normally use

to load it into low memory is:

notepad /i

The command to load it into upper memory, without using HRAM,

would be:

loadhigh notepad /i

The commands to load the program into the second memory region in

upper memory (regions are numbered starting with 0) and provide

an additional 64K of memory would be:

hram on /r1

loadhigh notepad /i

hram off

Although the LOADHIGH command alone will load notepad into upper

memory, it will load it into the first available upper memory

region; the advantage to using HRAM is that it will force

LOADHIGH to load it into a specific region.  This is an important

feature, since in order to gain optimum use of upper memory, your

programs should be loaded into the region recommended by HRAM

(with the [hram /l] command).  In addition, suppose you have 100K

of upper memory, but the notepad program requires 150K for

initialization.  Without HRAM, there would not be enough upper

memory and LOADHIGH would load it into low memory.  Since HRAM

provides an additional 64K for initialization, the program would

be successfully loaded high.

The HRAM command can be executed at the DOS prompt or it can be

used within a batch file (usually your autoexec.bat file).  When

the HRAM command is executed without any parameters, [hram], it

reports the current status of HRAM--on or off.

Command reference

The format of the HRAM command is

hram [on|off] [options]

[on|off]

specifies whether HRAM should be "on" or "off".  When HRAM is

on, programs can only be loaded into the memory region

specified on the HRAM command line (with the /r option) and

64K of expanded memory is temporarily converted to upper

memory.

[options]

specifies the optional HRAM parameters described in the

following section.

The following options can appear in the HRAM command.

21

/?

Display help.

/7

Increase low DOS memory size to 704K or 736K. If a upper

memory region exists at the address a002h (use the command

[hram /m] to display these regions), the command [hram /7]

will add this region to the low dos memory pool and increase

its size to 704K or 736K. The /7 option can only be used if no

programs have been loaded into the region at a002h. Programs

cannot be loaded high after this option has been used.

/a

List raw memory allocations in upper memory.  Use this option

to display the memory control blocks that have been allocated

from upper memory.

/c

List recommended config.sys and autoexec.bat files.  When the

/s option is added to the hramdev.sys command and your PC is

rebooted, information about the memory requirements of your

drivers and TSRs is written to the file "\hram0000.dat".  The

command [hram /c] analyzes this information and displays a

list of recommended commands for your config.sys and

autoexec.bat files.

/i

Display registration information.  This option causes hram.exe

to list information about registering your copy of HRAM.  If

you have not purchased a registered copy of HRAM, and you

continue to use it after a reasonable testing period, you are

required to register your copy.

/l

List analysis of programs.  When the /s option is added to the

hramdev.sys command and your PC is rebooted, information about

the memory requirements of your drivers and TSRs is written to

the file "\hram0000.dat".  The command [hram /l] analyzes this

information and displays the optimum load order and memory

region for each program.

/m

List regions and programs in upper memory.  [hram /m] lists a

status report of upper memory, including the size and location

of each memory region and program or device driver.  Read the

previous section, "Displaying regions and programs in upper

memory", for more information.

/n

No pause on error.  If hram.exe reports an error, it waits for

you to press a key.  This option causes hram.exe to continue

without waiting for a key.

/o<order>

Set program load order.  This option is useful only if you

22

have a program which must be loaded before one or more other

programs.  For example, to ensure that program1.exe is loaded

before program2.exe and program3.exe, include these commands

in your autoexec.bat file:

hram on /o1

program1

hram on /o2

program2

program3

hram on

To ensure that program1.exe is loaded before program2.exe and

that program2.exe is loaded before program3.exe, use these

commands:

hram on /o1

program1

hram on /o2

program2

hram on /o3

program3

hram on

This option is valid only if the /s option is used on the

hramdev.sys command line.  Otherwise, it is ignored.

/p

Do not temporarily convert 64K of expanded memory to upper

memory for use during program initialization.

/r<region>

Load into memory region number <region>.  This option causes

LOADHIGH to load one or more programs into the specified

memory region.  For example, the following commands would load

the programs, notepad.exe and calc.exe, into memory region 1

(memory regions are numbered starting with 0):

hram on /r1

loadhigh notepad

loadhigh calc

hram off

Without this option, LOADHIGH loads programs into the first

memory region in which they fit.

23

HRAM.SYS

Like hram.exe, hram.sys does not load programs into upper memory,

it works in conjunction with DOS to improve the function of the

DEVICEHIGH command.  hram.sys provides a way to load a device

driver into a specific memory region and provides an extra 64K

for driver initialization by temporarily converting expanded

memory to upper memory.  To illustrate, suppose you wish to load

the device driver, ansi.sys, into upper memory.  The command you

normally use to load it into low memory is:

device=ansi.sys

The command to load it into upper memory, without using HRAM,

would be:

devicehigh=ansi.sys

The commands to load the program into the second memory region in

upper memory (regions are numbered starting with 0) and provide

an additional 64K of memory would be:

device=hram.sys on /r1

devicehigh=ansi.sys

device=hram.sys off

Command reference

The format of the hram.sys command is

device=hram.sys [on|off] [options]

[on|off]

specifies whether HRAM should be "on" or "off".  When HRAM is

on, programs can only be loaded into the memory region

specified on the HRAM command line (with the /r option) and

64K of expanded memory is temporarily converted to upper

memory.

[options]

specifies the optional HRAM parameters described in the

following section.

The following options can appear in the HRAM command.

/n

No pause on error.  If hram.sys reports an error, it waits for

you to press a key.  This option causes hram.sys to continue

without waiting for a key.

/o<order>

Set program load order.  This option is useful only if you

have a program which must be loaded before one or more other

programs.  For example, to ensure that program1.sys is loaded

24

before program2.sys and program3.sys, include these commands

in your config.sys file:

device=hram.sys on /o1

device=program1.sys

device=hram.sys on /o2

device=program2.sys

device=program3.sys

device=hram on

This option is valid only if the /s option is used on the

hramdev.sys command line.  Otherwise, it is ignored.

/p

Do not temporarily convert 64K of expanded memory to upper

memory for use during program initialization.

/r<region>

Load into memory region number <region>.  This option causes

DEVICEHIGH to load one or more programs into the specified

memory region.  For example, the following commands would load

the programs, notepad.sys and calc.sys, into memory region 1

(memory regions are numbered starting with 0):

device=hram.sys on /r1

devicehigh=notepad.sys

devicehigh=calc.sys

device=hram.sys off

Without this option, DEVICEHIGH loads drivers into the first

memory region in which they fit.

25

YOUR TURN

We value you opinions.  Please let us know what you like and

dislike about the HRAM program and user's guide.  We're dedicated

to developing the most innovative and useful software available--

and selling it at reasonable prices.  Your support helps make

this possible.

Please send all comments and suggestions to

Biologic

POB 1267

Manassas VA

22110

USA

THANK YOU.