MyComm  Version 2.31

Unregistered Version Documentation

Printed November 5, 1995

Copyright (c) 1992 - 1995 - Ken Dorshimer

All Rights Reserved

MyComm is NOT Public Domain or free software.  The MyComm

software, support files, and documentation are Copyright (c) 1992

- 1995 Ken Dorshimer, all rights reserved.

MyComm is being marketed under the "Shareware" concept.  You

may use MyComm for a 30 day evaluation period to determine its

usefulness in your particular application.  After the 30 day

evaluation period you must register MyComm if you continue to use

it.

Registration is as follows:

Send $25 (twenty five U.S. dollars drawn on a U.S. bank),

plus $2 ($3 outside of U.S.) shipping and handling to:

Ken Dorshimer

P.O. Box 6881

San Mateo, Ca. 94403

California residents: please add 8.25% sales tax to the order.

(Please send check or money order, do not send cash.  People

who want to order MyComm from outside the U.S. should obtain a

money order in U.S. dollars from a local bank.)

For your convenience there is a file called ORDER.FRM which is

an order form for MyComm.  An order form is also included in

Appendix F of this documentation.  Please note that if you

downloaded MyComm from a BBS that charges a subscription fee,

obtained a copy of MyComm from a "shareware disk copying"

service, or CD ROM library the registration fee for MyComm is not

paid for or collected by those services!

1.0  Introduction to MyComm                                1

1.1  Registration and Licensing                        2

1.2  Disclaimer and Warranty                           5

1.3  Definitions, Acronyms                             5

1.4  General System Requirements                       6

1.5  MyComm Files                                      6

1.6  MyComm Support and Technical Assistance           7

2.0  MyComm Installation and Running                       8

2.1  System Requirements                               8

2.2  MyComm Installation Instructions                  8

2.3  Running MyComm                                   10

2.4  Command Line Parameters                          11

2.4.1 Join Script                                 11

2.4.2 Dial Entries  (Registered version only)     11

2.4.3 Timed Start  (Registered version only)      12

3.0  MyComm Screens and Menus                             13

3.1  Terminal Screen                                  13

3.2  Help Menu - F1                                   13

3.3  Chat Mode - Alt A                                14

3.4  Send Break - Alt B                               14

3.5  Clear the Screen - Alt C                         15

3.6  Dialing Functions - Alt D                        15

3.6.1  Add Phone List Item                        15

3.6.2  Edit Phone List Item Screen                15

3.6.2.1  Name                                 16

3.6.2.2  Number                               16

3.6.2.3  Baud Rate                            16

3.6.2.4  Data Bits                            16

3.6.2.5  Data Parity                          17

3.6.2.6  Stop Bits                            17

3.6.2.7  Default Protocol                     17

3.6.2.8  Dial Prefix                          17

3.6.2.9  Logon Name                           17

3.6.2.10 Password                             18

3.6.2.11 Terminal Type                        18

3.6.2.12 Local Echo                           18

3.6.2.13 Received BS (BackSpace) Destructive  19

3.6.2.14 BS Key Sends BS or DELete            19

3.6.2.15 Strip Hi Bits                        19

3.6.2.16 Add LF after incoming CR             19

3.6.2.17 Clear last date called               19

3.6.2.18 Script                               19

3.6.2.19 Save Changes                         20

3.6.3  Delete an Item                             20

3.6.4  Call Numbers (Dial)                        20

3.6.5  Phone Item Info Screen                     21

3.6.6  Manual Dial Screen                         21

3.6.7  Mark Phone List Items                      22

3.6.8  Find Phone List Items                      22

3.6.9  Load Phone List Screen                     22

3.6.10 Sort Phone List                            22

3.6.11 Move Phone List Item                       22

3.6.12 Print Phone List                           23

3.7  Local Echo On/Off - Alt E                        23

3.8  Line Feeds On/Off - Alt F                        23

3.9  Hang Up Modem - Alt H                            23

i

3.10  Send Modem Initialization Strings - Alt I       23

3.11 Join Script - Alt J                              24

3.12 Present Line Status - Alt L                      24

3.13 Scroll Back - Alt K                              24

3.14 Manual Dial - Alt M                              25

3.15 Outside to DOS - Alt O                           25

3.16 Change Port Parameters - Alt P                   25

3.17 Quick Re-Dial - Alt Q                            26

3.18 Run External Program - Alt R                     26

3.19 Session Capture On/Off/Pause - Alt S             26

3.20 Terminal Emulation - Alt T                       27

3.21 Update Configuration - Alt U                     27

3.21.1  Modem Setup Screen                        27

3.21.1.1  COM Port                            28

3.21.1.2  Modem Initialization String         28

3.21.1.3  Modem Attention String              29

3.21.1.4  Modem Hang Up String                29

3.21.1.5  Modem Answer String                 29

3.21.1.6  Modem Cancel Dial String            29

3.21.1.7  Modem Dial Suffix String            30

3.21.1.8  Dialing Wait Time                   30

3.21.1.9  Pause Between Calls                 30

3.21.1.10 Local Echo on by Default            30

3.21.1.11 Use DTR to Hang Up Modem            30

3.21.1.12 Use XON/XOFF Flow Control           31

3.21.1.13 Use CTS/RTS Flow Control            31

3.21.1.14 Use DSR/DTR Flow Control            31

3.21.1.15 Dialing Prefixes                    31

3.21.1.16 Connect Strings                     32

3.21.2  Paths Setup                               32

3.21.2.1  Download Directory                  33

3.21.2.2  Upload Directory                    33

3.21.2.3  Path\File for Screen Capture        33

3.21.2.4  Session Capture Path\File           33

3.21.2.5  Phone File Path\File                33

3.21.2.6  Editor Path\Filename                33

3.21.2.7  External Programs Path\Filename     33

3.21.3  Protocols Setup                           34

3.21.3.1  ASCII Transfers Setup               34

3.21.3.1.1  Strip High Bits               34

3.21.3.1.2  Remote Abort Char             35

3.21.3.1.3  CR Translation                35

3.21.3.1.4  LF Translation                35

3.21.3.1.5  Local Echo On During Uploads  35

3.21.3.1.6  Expand Blank Lines to Spaces  35

3.21.3.1.7  Line Pace Time                35

3.21.3.1.8  Character Pace Time           36

3.21.3.1.9  Pacing Char                   36

3.21.3.2  Zmodem Transfers Setup              36

3.21.3.2.1  Zmodem 32 Bit CRC             36

3.21.3.2.2  Zmodem Auto Download Feature  36

3.21.3.2.3  Zmodem Send Crash Recovery    36

3.21.3.2.4  Zmodem Receive Crash Recovery 37

3.21.3.3  Kermit Transfers Setup              38

3.21.3.4  External Protocols Setup            38

3.21.4  Miscellaneous Options Setup               40

3.21.4.1  Screen Background Color             40

ii

3.21.4.2  Screen Text Color                   40

3.21.4.3  Date Format Style                   40

3.21.4.4  12 or 24 Hour Time Format           40

3.21.4.5  Make Noise When Calls Connect       41

3.21.4.6  Make Noise After File Transfers     41

3.21.4.7  Use Direct Video                    41

3.21.4.8  Your Name for sign on               41

3.21.4.9  Password for sign on                41

3.21.4.10 Clever quip for TagLine             41

3.21.4.11 Guess Up/Download Filename          42

3.21.4.12 Dialing Screen After Calls          42

3.21.5  COM Port Addresses                        42

3.21.6  Restore to Defaults                       43

3.21.7  Save Setup                                43

3.22 Send Screen to File - Alt W                      43

3.23 Exit MyComm - Alt X                              43

3.24 Help Screen - F1                                 43

3.25 Send Name - F2                                   43

3.26 Send Password - F3                               43

3.27 Send Tagline  - F4                               44

3.28 Transferring Files                               44

3.28.1  Starting a File Transfer                  45

3.28.2  File Transfer Hints                       46

3.28.3  Download Files - PgDn                     47

3.28.4  UpLoad Files - PgUp                       47

3.28.5  ASCII File Transfers                      47

3.28.6  SEAlink Derived File Transfers            47

3.28.7  Xmodem File Transfers                     48

3.28.8  1K-Xmodem File Transfers                  48

3.28.9  Ymodem Batch File Transfers               48

3.28.10  Ymodem-G Batch File Transfers            48

3.28.11 Zmodem File Transfers                     49

3.28.12 Kermit File Transfers                     49

3.28.13 External Protocol File Transfers          49

3.28.14 MyComm as a Host                          50

Appendix A - MyComm Command Summary                       52

Appendix B - MyComm Line Editing Functions                53

Appendix C - Terminal Emulation                           54

1.0  TTY Terminal Emulation                           54

2.0  ANSI Terminal Emulation                          54

3.0  VT-52 and VT-100 Terminal Emulation              55

4.0  3270 Terminal Emulation                          56

Appendix D - Troubleshooting                              57

Appendix E - MyComm Script Language                       61

1.0  Script Language Introduction                     61

1.2  Creating a Script                            61

2.0  Script Language                                  62

2.1  Special Script Characters                    63

2.2  Script Functions                             63

2.2.1  Labels                                 63

2.2.2  WAITFOR()                              63

2.2.3  XMIT()                                 64

2.2.4  GOTO()                                 64

2.2.5  SETFLAG()                              64

2.2.6  INCFLAG()                              64

2.2.7  DECFLAG()                              65

2.2.8  IFFLAG()                               65

iii

2.2.9  IFSCAN()                               65

2.2.10 SENDNAME()                             66

2.2.11 SENDPASS()                             66

2.2.12 SENDFILE()                             66

2.2.13 RECVFILE()                             67

2.2.14 SNDBREAK()                             67

2.2.15 ALARM()                                67

2.2.16 PAUSE()                                67

2.2.17 HANGUP()                               68

2.2.18 IGNORECD()                             68

2.2.19 SETSTR()                               68

2.2.20 GOSUB()                                69

2.2.21 RETURN()                               69

2.2.22 EXIT()                                 69

2.2.23 QUIT()                                 69

2.2.24 COMPSTR()                              69

2.2.25 LOCLECHO()                             70

2.2.26 IF()                                   70

2.2.27 PORTGET()                              70

2.2.28 DIAL() (Registered version only)       70

2.2.29 CHDIR() (Registered version only)      71

2.2.30 RUNDOS() (Registered version only)     71

2.2.31 FILEHERE() (Registered version only)   71

2.2.32 JUMPSCP() (Registered version only)    71

2.2.33 CONSGET() (Registered version only)    71

2.2.34 CONSLOC() (Registered version only)    72

2.2.35 CONSCLR() (Registered version only)    72

2.2.36 CONSMSG() (Registered version only)    72

2.2.37 KBDFLSH() (Registered version only)    72

2.2.38 PORTFLSH() (Registered version only)   72

2.2.39 SESCAP() (Registered version only)     73

3.0  Script Syntax Checker                            74

4.0  Script Sample                                    75

Appendix F - MyComm Order Form                            76

Appendix G - MyComm Bug Report Form                       77

Appendix H - Control Characters                           78

Appendix I - MyComm Strip ANSI (Registered version only)  79

Index                                                     80

iv

MyComm Version 2.31 Documentation                 Page 1

1.0  Introduction to MyComm

MyComm is a small, yet powerful, data communications program

designed to enable users to access public bulletin board systems

(BBS's) and other telecommunication services.  MyComm is designed

to run under the PC/MS-DOS operating system using DOS version 3.3

or higher.  MyComm has a variety of built in file transfer

protocols including Zmodem, Xmodem, Xmodem-1K, Ymodem Batch,

Ymodem-G Batch, SEAlink, Kermit, and of course ASCII.  There are

also slots to install up to six external file transfer protocols.

The driving principle behind MyComm is to provide a reliable,

powerful, and versatile communications package which is easy to

use, and small in size.  Great care has been taken to optimize

MyComm for operating speed and to keep the executable code size

small so that it takes up less of your valuable disk space.

MyComm is designed to be easy to use for both novice and

experienced users, and employs a menu system that is friendly and

simple to navigate through.

MyComm features include:

�    A full featured communications package which is small in

size.

�    Wide variety of popular file transfer protocols.

�    Powerful, yet easy to use "script" language.

�    Small code size which will run with approximately 200K of

RAM.

�    Intelligent ANSI character translation which filters out

potentially harmful ANSI keyboard re-mapping sequences.

�    Intelligent DOS shell function that brings you back to the

disk drive and directory that you started from when you

return to MyComm.

�    Detection and use of the 16550 UART and FIFO buffers.

�    Locates program and configuration files no matter what

directory MyComm is started from (DOS 3.0 or greater).

�    Support for COM ports 1 through 8.

�    Simple, and easy to use.  Some may even call it "fun"!

MyComm Version 2.31 Documentation                 Page 2

1.1  Registration and Licensing

MyComm is NOT Public Domain or free software.  The MyComm

software, support files, and documentation are Copyright (c) 1992

- 1995 Ken Dorshimer, all rights reserved.

MyComm is being marketed under the "Shareware" concept.  You

may use MyComm for a 30 day evaluation period to determine its

usefulness in your particular application.  After the 30 day

evaluation period you must register MyComm if you decide to

continue to use it.

Registration is as follows:

Send $25 (twenty five U.S. dollars drawn on a U.S. bank),

plus $2 ($3 outside of U.S.) shipping and handling to:

Ken Dorshimer

P.O. Box 6881

San Mateo, Ca. 94403

California residents: please add 8.25% sales tax to the order.

(Please send check or money order, do not send cash.  People

who want to order MyComm from outside the U.S. should obtain a

money order in U.S. dollars from a local bank.)

For your convenience there is a file called ORDER.FRM which is

an order form for MyComm.  The order form is also included in

Appendix F of this document.  Please note that if you downloaded

MyComm from a BBS that charges a subscription fee, obtained a

copy of MyComm from a "shareware disk copying" service, or CD ROM

library the registration fee for MyComm is not paid for or

collected by those services!

MyComm represents many, many hours of work on my part in

research, design, coding, testing, revisions, enhancements and

writing the documentation.  Writing a piece of software like

MyComm is not trivial.  Your registration not only gives me the

incentive to continue producing and upgrading shareware products

like this one, but it is the right thing to do.

Let's face it.  $25 is not a lot to spend on a program that

does everything MyComm does.  Comparable communications packages

are generally much more expensive.  The Shareware market is an

"all risk" venture for the author, while the user has everything

to gain.  Your registration tells me that MyComm was worth

creating, and that people like it.  I like to know that people

appreciate the effort I put into a project, don't we all?  Enjoy

using the MyComm software, and I'm looking forward to hearing

from you.

When you register MyComm you will receive the most recent

version of MyComm and the documentation on floppy disk (standard

3.5" or 5.25" disk, 3.5" preferred).  At this time a printed

manual is not being offered in order to keep the cost of the

software reasonable.  Registered users will also be offered a

MyComm Version 2.31 Documentation                 Page 3

discount on future upgrades.  Please bear in mind that the

registered versions have the latest revisions and enhancements,

and generally have additional features or functions (another

registration incentive).

Multiple registrations to the same individual or site will be

discounted as follows:

1  to 5    copies      -  no discount

6  to 10   copies      -  5% discount

11 to 25   copies      - 10% discount

26 to 50   copies      - 20% discount

51 to 100  copies      - 30% discount

over 100 copies        - inquire for quote

Inquiries for site licenses and multiple registrations of

MyComm should be directed to the above address, or via Compuserve

(Tm) 73113,3536 (Internet: 73113.3536@compuserve.com).  In the

case of site licenses and multiple registrations a maximum of two

(2) diskettes will be sent with the order unless otherwise

negotiated.  Site licenses will have the same serial number on

each diskette.  If you require site licences or multiple

registrations with multiple serial numbers please feel free to

inquire about a price quote.

BBS sysops and operators of similar on-line services are

encouraged to order the "BBS Custom" version of MyComm.  MyComm

BBS Custom is similar to the registered version of MyComm and may

be distributed to users after paying a licensing fee.

MyComm BBS Custom allows the BBS sysop to place up to three

telephone numbers for their BBS or on-line service into the

dialing directory and to present a customized ANSI "logo" to the

user when MyComm initializes.  Inquires regarding MyComm BBS

Custom should be directed to the above mailing or Internet

address.

Registered users will be granted license to use the MyComm

software as they deem fit and within the law.  The registered

version of MyComm may be duplicated for backup purposes only.

The registered version of MyComm may not used on more than one

computer at a time with the exception that registered users may

install MyComm on their computer at work and their computer at

home providing that the two computers are not operated

simultaneously.

Businesses, corporations, organizations, government agencies,

or schools who intend to use MyComm on more than one computer at

a time must contact the author for a site license.  The

registration of an individual copy of MyComm is not to be

construed a site license (let's be serious).  Feel free to

contact the author regarding multiple registrations and site

licenses.

MyComm Version 2.31 Documentation                 Page 4

All users are granted a limited license to make copies of the

unregistered "shareware" version of MyComm for the purpose of

allowing others to try it out, and the unregistered version may

also be uploaded to public bulletin board systems subject to the

above restrictions as well as the following:

�    MyComm must be distributed in completely unmodified

form, and include all support files and documentation.

�    MyComm may not be included or bundled with any other

product for any reason without a license from the

author, Ken Dorshimer.

�    No charge or payment may be levied or accepted for the

MyComm software except by the author, Ken Dorshimer.

MyComm may not be sold or distributed through a third party

(this includes copying services and shareware distribution

outlets which distribute software for a copying fee) without the

expressed written permission of the author, Ken Dorshimer.

Shareware distribution services and bulletin boards which charge

a fee for software distribution should contact me for written

permission to distribute MyComm.

The MyComm executable file(s) and documentation, both the

"registered" and "unregistered" versions are copyrighted material

and may not be modified or reverse engineered in any manner.  If

you require modified or customized versions of MyComm for

whatever reason feel free to contact the author at the address

above.

MyComm Version 2.31 Documentation                 Page 5

1.2  Disclaimer and Warranty

Disclaimer and Warranty:

MyComm was designed and coded on a best efforts basis in

accordance with the standard of care usually and reasonably

expected in the performance of such computer software.

THIS SOFTWARE AND DOCUMENTATION IS PROVIDED "AS IS".  THE

AUTHOR, KEN DORSHIMER, MAKES NO WARRANTY OF ANY KIND, EITHER

EXPRESSED OR IMPLIED, INCLUDING BUT NOT LIMITED TO IMPLIED

WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE

OR THE SOFTWARE'S USEFULNESS IN A PARTICULAR APPLICATION.  IN NO

EVENT SHALL THE AUTHOR, KEN DORSHIMER, BE LIABLE FOR ANY DAMAGES

(INCLUDING DAMAGES FOR LOSS OF BUSINESS OR PERSONAL PROFITS,

BUSINESS OR PERSONAL INTERRUPTION, LOSS OF BUSINESS OR PERSONAL

INFORMATION, OR OTHER PECUNIARY LOSS) ARISING OUT OF THE USE OF

OR INABILITY TO USE THIS PROGRAM, EVEN IF THE AUTHOR, KEN

DORSHIMER, HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

THE USER ASSUMES FULL RESPONSIBILITY AND LIABILITY FOR THE PROPER

USE OF THE MYCOMM SOFTWARE AND ACCOMPANYING DOCUMENTATION.

<Whew>, glad the legal stuff is out of the way.  If I hadn't

included that my attorney would have had me drawn and quartered.

1.3  Definitions, Acronyms, Acknowledgements, and Abbreviations

� BBS       -  Bulletin Board System

� IBM       -  Trademark, International Business Machines

� Hayes     -  Hayes (trademark) compatible modems have

become the defacto standard in modem

communications. The Hayes command set are

modem commands preceded by "AT"

� DOS       -  Disk Operating System

� MS-DOS    -  Microsoft Disk Operating System, trademark of

Microsoft Corporation.

� Shareware  - A marketing system where users are allowed to

evaluate software before purchase and

registration.

� XMODEM    -  A file transfer protocol developed by Ward

Christensen in the summer of 1977.

� YMODEM    -  A file transfer protocol based on XMODEM

developed by Chuck Forsberg.

� ZMODEM    -  A file transfer protocol developed by Chuck

Forsberg/Omen Technology, Inc.

MyComm Version 2.31 Documentation                 Page 6

� SEAlink   -  A file transfer protocol based on XMODEM

developed by System Enhancement Associates,

Inc. Copyright 1986, 1987 by System

Enhancement Associates, Inc.

� Kermit    -  A file transfer protocol developed by

Columbia University, New York, NY.

� WordStar  -  Trademark (company unknown) of an editor

which has been around since the early days of

personal computers.

� Unix      -  Trademark of operating system designed by

Bell Labs/Western Electric.

� VT-nnn    -  As in VT-52, VT-100, etc. are trademarks of

the Digital Equipment Corporation.

� 3270      -  3270 and 7171 are designations used by IBM

for terminal emulation and protocol

conversion respectively.

Any other names, trademarks, etc. not mentioned here belong to

whomever they belong to.  If there are errors or omissions, and

you let me know, I will make an effort to correct them in the

next version of this documentation.

1.4  General System Requirements

MyComm is designed to run under the PC/MS-DOS operating system.

MyComm may be run either from a hard drive or a floppy drive,

although a floppy drive is not recommended when performing file

transfers (just too slow).  Great care has been taken to keep the

code size small so MyComm will run with as little as 200K of main

memory (RAM) available.  MyComm will run using a Monochrome

Graphics Adapter (MGA), a true Hercules (Tm) monochrome card,

EGA, and VGA.

MyComm is designed to use modems which use the Hayes (Tm) "AT"

command set.  Your modem should support this standard or a subset

of it.  MyComm supports modem speeds of 300, 1200, 2400, 4800,

9600, 19200, 38400, 57600, and 115200 baud and a wide variety of

data bit, parity bit, and stop bit settings.

1.5  MyComm Files

The files included in the MyComm package are:

� MYCOMM.EXE  -     MyComm main executable program.

� MYCOMM.DOC  -     Documentation for MyComm (you're reading

it now).

� ORDER.FRM   -     Order form for MyComm.

MyComm Version 2.31 Documentation                 Page 7

� BUGRPT.FRM  -     Form to describe any problems or

bugs you may have found, or to make

suggestions for future versions of

MyComm.

� SCP_CHK.EXE -     Program to check the syntax of script

file commands. See Appendix-E "MyComm

Script Language" for more information.

� MCSTPANS.EXE -    Program to remove ANSI escape sequences

from text files - See Appendix I. (In

registered version only)

� *.SCP       -     Example MyComm "script" language files.

MyComm also creates and modifies the following files as part of

its normal operation:

� MYCOMM.CFG   -    MyComm configuration file.

� MYCOMM.PHN   -    MyComm phone list file.

� MYCOMM.OLD   -    Backup of the MyComm phone list file.

� MYCOMM.SCR   -    File created when you capture the screen

to a file (see Alt W).

� MYCOMM.SES   -    File created when capturing your current

session to a file (see Alt S).

� Other Files  -    phone lists that you create, script

files, and any files that were

downloaded.

1.6  MyComm Support and Technical Assistance

MyComm technical assistance is offered in the following manner:

�    Contact the author through Compuserve at 73113,3536 or

Internet 73113.3536@compuserve.com  (Please note that many

on-line services have links to Compuserve and Internet).

�    Write to:

MyComm - Tech Help

P.O. Box 6881

San Mateo, Ca. 94403

Before writing for technical assistance please read through the

MyComm documentation and Appendix D - "Troubleshooting".  The

preferred method for technical assistance is through one of the

e-mail systems listed above which will generally get a faster

response.

MyComm Version 2.31 Documentation                 Page 8

2.0  MyComm Installation and Running

MyComm is amazingly simple to install and use.  That's the

whole idea!  The instructions in this section will have you up

and running in no time.  Pay special attention to the information

that should be in your CONFIG.SYS file, and the dissertation

about the COM ports.

2.1  System Requirements

MyComm is designed to run on IBM or compatible personal

computers under the MS-DOS operating system.  Machines may be of

the PC, XT, AT, 386, 486 or above class of personal computers.

MyComm has the following system requirements:

�   at least 200K of free memory (RAM).

�   MS-DOS version 3.0 or greater or similar "after market"

Disk Operating System.

�   Hayes compatible modem which accepts the " AT " command

set.

�   Your CONFIG.SYS file should contain the statements:

Device=ANSI.SYS

Files=40     (or greater)

Buffers=40   (or greater)

If your CONFIG.SYS file does not contain these

statements then you will need to modify the CONFIG.SYS

file.  The CONFIG.SYS file may be edited with an

ordinary text editor.  Simply type in a line that reads

"Device=ANSI.SYS", "Files=40", and one that reads

"Buffers=40" (without the quotes) and save the file.

Refer to your DOS manual if you need more information

on CONFIG.SYS.

2.2  MyComm Installation Instructions

Before you start the MyComm installation you should know which

COM port your modem is set for.  Consult your modem manual if

you're not sure.  Modems vary too much from one manufacturer to

another to describe the differences in this manual.

Step 1:   Modify your CONFIG.SYS file to make any of the required

changes described in section 2.1 above.

MyComm Version 2.31 Documentation                 Page 9

Step 2:   Create a directory on your hard drive (or a floppy

drive) called "MYCOMM" and change directory into the

new "MYCOMM" directory.

Example:

C:

CD\

MD MYCOMM

CD MYCOMM

Step 3:   Copy the MYCOMM files into this new directory.

Step 4:   Run MyComm.  At the DOS command prompt type "MYCOMM"

then press the <Enter> key.

Step 5:   MyComm will start to load.  The first time MyComm is

run it will tell you that it can't find the

"Configuration File".  It will then create a new

configuration file called MYCOMM.CFG.  MyComm will also

create an empty phone dialing list called MYCOMM.PHN.

Step 6:   MyComm has a default COM port setting of COM Port 1.

If your modem is set up for COM 1 already, don't sweat

it.  Otherwise, MyComm will ask you which COM port to

use.  Choose the COM port that your modem is set on.

MyComm will then try to initialize the COM port.  If

all goes well you should see the letters "OK" on the

screen under the copyright notice.  If the modem

doesn't appear to be responding, try repeating Step 6

using another COM port (if you have more than one) or

see Appendix D "Troubleshooting".

You can see if the modem is responding correctly by

typing "AT" (without the quotes) and then press

<Enter>.  The modem should respond with "OK".  You may

test the phone line connection by typing "ATM1DT"

(again, without the quotes) then press <Enter>.  This

command tells the modem to turn its' speaker on, then

dial nothing.  You should hear a dial tone from the

modem speaker.  Press <Enter> again and the dial tone

should stop and the modem should respond with "NO

CARRIER".

Step 7:  Press the <Alt> and "U" key.  This will bring up

MyComm's Update Configuration Menu.  Go through each of

the menu options and make any changes required.  The

remainder of this documentation describes each of the

MyComm options and functions.

MyComm Version 2.31 Documentation                 Page 10

Step 8:  You may also need to modify the "Modem Initialization

String".  This is done in the "Modem Setup" screen.

MyComm needs to see text response codes from the modem

rather than numeric codes.  The general purpose

initialization string that MyComm uses as by default

is:

AT M1 X4 Q0 V1 S0=0 S11=50 S7=50 &C1 &D2^M

Broken down this means:

AT     - "Hi modem! I'm sending you commands now".

M1     - Turn on modem speaker (M0 turns it off).

X4     - Select highest level of responses.

Q0     - Always send responses.

V1     - Send text responses.

S0=0   - Do not answer the phone.

S11=50 - Use fast dialing speed.

S7=50  - Wait time for Carrier Detect in seconds.

&C1    - Keep Carrier Detect (CD) low until a

connection is made.

&D2    - Use RTS/CTS flow control (important on

high speed modems).

^M     - Send Carriage Return character (MyComm

internal command - see also Appendix H).

2.3  Running MyComm

Section 3 of this document covers all the options and functions

that MyComm offers.  There are some additional items that should

be covered first.

MyComm may be started in a number of ways.  Below are three

different methods to start the MyComm program.

Method 1: Start MyComm from the directory that you installed

MyComm in. For example:

CD\MYCOMM

then type "MYCOMM" and press the <Enter> key.  This

will start the MyComm program.

Method 2: MyComm will also load from any directory if you have

set a "Path" with the DOS path command that points to

the directory MyComm lives in.  For example to set the

path type:

PATH=C:\MYCOMM

providing of course that you installed MyComm in the

directory C:\MYCOMM.  Just substitute the appropriate

path in that case.

MyComm Version 2.31 Documentation                 Page 11

Method 3: You may also specify the "fully qualified" DOS path to

start MyComm.  For example:

C:\MYCOMM\MYCOMM

will also cause MyComm to start.

If you have questions about the DOS Path commands or statements

please refer to your DOS manual.

If you are using DOS version 3.0 or higher MyComm will

automatically find it's configuration file.  This means that

MyComm may be started from any drive or directory on your system

without having to always start in the MyComm directory  This will

save you the trouble (and typing) of changing directories when

you want to use MyComm.

MyComm will however try to start a new phone file in the

directory you are in unless you have included the fully qualified

path to the phone directory file in the Setup options.  This

allows you to keep multiple phone files in different directories

depending on your needs.

2.4  Command Line Parameters

MyComm will take several "command line" parameters.  Command

line parameters are commands that are typed after the program

name at the DOS prompt. For example typing

MYCOMM -J MYSCRIPT.SCP

on the command line will cause MyComm to start then to

immediately (after initial logo screen) "join" the script

MYCSCRIPT.SCP.  A description of the command line parameters that

MyComm understands follows.

2.4.1 Join Script

The -J <script name> parameter causes MyComm to start the named

script immediately after MyComm starts up.  See the section "Join

Script" for more details on MyComm script functions.  Please note

that the "Join Script" function takes precedence over the "Dial"

command described below.

2.4.2 Dial Entries  (Registered version only)

The -D <n1, n2, ... nx> function tells MyComm to mark the

dialing entries that correspond to the numbers after the -D and

begin dialing them.  This would be the same as if you had gone

into the dialing directory and manually marked number for dialing

then started the Call command.  For example

MYCOMM -D 2 4 14 15

will mark dialing entries 2, 4, 14, and 15 and then begin to call

those entries after MyComm initializes.  Please note that entries

MyComm Version 2.31 Documentation                 Page 12

are separated by a space.  If the dialing entry is invalid it is

ignored.

2.4.3 Timed Start  (Registered version only)

The -T <HH:MM> function starts a timer that waits until the

specified time before MyComm begins to do anything else.  The

time may be entered as follows:

2:00          2:00 Am

14:00          2:00 Pm  (military time)

2:00P         2:00 Pm

For example

MYCOMM -T 6:00P

would tell MyComm that you want to wait until 6:00 Pm before it

does something.  This is especially useful for setting a timer

for the -J or -D commands (described above).  While MyComm is

waiting for the start time a message box is displayed.  You may

quit the timer by pressing the <ESC> key.

MyComm Version 2.31 Documentation                 Page 13

3.0  MyComm Screens and Menus

MyComm has several screens and menus to allow you to change

various options and to perform various functions.  Most screens

and menus are brought up by pressing the <ALT> key and then a

letter.  Pressing <F1> while in the terminal screen shows you

Help Menu which lists all of the MyComm functions (see also

section 3.2).

3.1  Terminal Screen

When MyComm is first loaded, and after the configuration file

has been read, it defaults to the terminal screen which looks

like this:

MyComm <Version Information>

Copyright (c) 1992 - 1995 Ken Dorshimer  -  All rights reserved.

Press <F1> for Help

Press <Alt-D> for Dial Screen

You will see the version number, and copyright notice.

In the terminal screen you may enter regular Hayes - AT modem

commands, or choose one of the many MyComm functions.  The MyComm

functions are listed in the Help Menu which is displayed when you

press the <F1> key on your keyboard.  The MyComm functions are

described in more detail in the following sections of this

manual.

3.2  Help Menu - F1

Pressing the <F1> key brings up the MyComm Help Screen which

looks like this:

(NOTE: many of the MyComm screens shown in this documentation are

smaller than actual size so that they will fit on a printed

page.)

MyComm Version 2.31 Documentation                 Page 14

+------------------------------------------------| MyComm Help |+

|                                                               |

|                     MyComm Commands                           |

|                                                               |

|Alt A  Activate Chat Mode         Alt J  Join Script           |

|Alt B  Send Break                 Alt M  Manual Dial           |

|Alt C  Clear the Screen           Alt O  Outside to DOS        |

|Alt D  Dialing Functions          Alt P  Change Port Parameters|

|Alt E  Toggle Local Echo On/Off   Alt Q  Quick Re-Dial         |

|Alt F  Toggle Line Feeds On/Off   Alt R  Run External Program  |

|Alt H  Hang up modem              Alt S  Session Capture On/Off|

|Alt I  Send Modem Init. String    Alt T  Terminal Emulation    |

|Alt K  Scroll BacK                Alt U  Update Configuration  |

|Alt L  Current Line Status        Alt W  Write Screen to File  |

|                                                               |

|                                                               |

|F1     Help With Commands         Alt X  EXit MyComm           |

|F2     Send Name                  PgDn   Download Files        |

|F3     Send Password              PgUp   Upload Files          |

|F4     Send Tagline                                            |

|                                                               |

|                                                               |

|  MyComm <Version Information>                                 |

| Copyright (c) 1992 - 1995 Ken Dorshimer - All rights reserved.|

|                                                               |

+---------------------------------------------------------------+

Pressing <Esc> or any other key closes the MyComm Help Screen

window.

3.3  Chat Mode - Alt A

Pressing the <ALT> and the "A" key brings you into the MyComm

Chat window.  The chat screen is useful when you want to send

text messages or "chat" interactively with someone while on-line.

If you were not using the Chat Mode the "conversation" would

eventually end up looking like a garbled mess on your screen.

The Chat window is divided horizontally.  The top half of the

screen is where received characters are written, the bottom half

is where the characters you type are written.

To exit Chat Mode, press the <Esc> key.

Please note: Some BBS "chat" systems send special control

characters to emulate a split screen chat or colors.  Since there

is no filtering done by MyComm while in Chat Mode this may cause

strange effects in your receive window.  If this occurs, exit

from Chat Mode and allow the BBS software to control the chat

session.

3.4  Send Break - Alt B

Pressing the <ALT> and the "B" key sends a special modem signal

called "BREAK".  Some host systems use the Break signal to

perform special functions such as ending a session.

MyComm Version 2.31 Documentation                 Page 15

3.5  Clear the Screen - Alt C

Pressing the <ALT> and the "C" key clears the screen and resets

the current color attributes.  Many host systems send color

information which stays in place even after you log off. Clear

Screen resets the screen colors to the ones you chose.

3.6  Dialing Functions - Alt D

Pressing the <ALT> and the "D" keys displays the MyComm Dial

Screen.  From this screen you may add, delete, edit, view, find,

and mark phone list entries, and of course dial phone list

entries.  You will see the options available from this screen

written into the little window at the bottom of the screen. The

letter which activates an option will appear highlighted or in a

different color.  A description of the dialing functions follows.

You will see a bar highlighting the current phone list item.

The highlight bar may be moved up and down by using the Up-Arrow,

Down-Arrow, Page Up, Page Down, Home and End keys.  The highlight

bar appears as reverse video bar on monochrome screens.

+------------------------------------------------| Dial Screen |+

|                                                               |

|   #   Name                     Phone Number         Last Call |

|                                                               |

|   1   Your Favorite BBS       555-4657               01/26/92 |

|   2   The Music Board         555-8944               01/19/92 |

|   .           .                  .                      .     |

|   .           .                  .                      .     |

|   .           .                  .                      .     |

|  15                                                  00/00/00 |

|---------------------------------------------------------------|

|                                                               |

| Add  Edit  Delete  Call  Info  Manual  (Un)MarK  Find  Load   |

|                    Sort  moVe  Print                          |

+---------------------------------------------------------------+

3.6.1  Add Phone List Item

Pressing the "A" key in the Dial Screen window adds a blank

entry after the currently highlighted entry.

3.6.2  Edit Phone List Item Screen

Pressing the "E" key in the Dial Screen window brings up the

Dial Edit window.  This is where you set up the phone list entry

for each system you will call.  There are several options to

choose from, and each is described.  You may also use the Up-

Arrow and Down-Arrow keys to move between items.  Items such as

Name, Number, etc use MyComm's Line Editor described in Appendix

B.

MyComm Version 2.31 Documentation                 Page 16

+----------------------------------------| Edit |+

|                                                |

| Name:                 Your Favorite BBS        |

| Number:               555-4657                 |

| Baud Rate: 2400       Data Bits: 8             |

| Data Parity: None     Stop Bits: 1             |

| Default Protocol:     1K-Xmodem                |

| Dial Prefix: 1        ATDT*70                  |

| Logon Name:           Your Name                |

| Password:             Password                 |

| Terminal Type: ANSI                            |

| Local Echo: No                                 |

| Received BS Destructive: Yes                   |

| BS Key Sends BS or DELete: BS                  |

| Strip Hi Bits? (Pure ASCII): No                |

| Add LF after incoming CR?: No                  |

| Script                                         |

| Clear last date called and number of calls?    |

|                                                |

+------------------------------------------------+

The following describes each phone list item and how to configure

it.

3.6.2.1  Name

Just type in the name of the bulletin board system or person

you will call here.

3.6.2.2  Number

The telephone number of the system you will call goes in here.

The telephone number may include spaces or hyphens for

readability (modems usually ignore spaces and hyphens).

Characters such as letters or other gibberish will probably

confuse your modem.  MyComm will assume that whatever you put

here is a valid telephone number.

3.6.2.3  Baud Rate

Choose a baud rate (300, 1200, 2400, 4800, 9600, 19200, 38400,

57600, or 115200) that you want MyComm to use when connected to

the system.  For a more accurate reading of the data rate during

file transfers (described elsewhere in this manual) and the on-

line status (also described elsewhere in this manual) the baud

rate should be set to the rate of the system that you will be

calling.

3.6.2.4  Data Bits

Choose the data bits (5, 6, 7, 8) that you want MyComm to use

when connected to the system.  8 data bits is the most common

setting for public bulletin board systems.  7 data bits are

MyComm Version 2.31 Documentation                 Page 17

common on mainframes and many Unix (Tm) systems.  5 and 6 data

bits are rarely used by many systems any longer, but are included

for completeness.

Most host systems in their advertisements, etc. tell you what

Data, Parity, and Stop bits they expect.  You will often see them

listed as 8-N-1 or similar.  8-N-1 means 8 data bits, no parity

(None), and 1 stop bit. 8-N-1 is the most common setting used.

If in doubt, try 8-N-1 first.

3.6.2.5  Data Parity

Choose the data parity (None, Even, Odd, Mark, Space) that you

want MyComm to use when connected to the system.  None is usually

used on most systems, especially with 8 data bits.  Even parity

is common when using 7 data bits.

3.6.2.6  Stop Bits

Choose the stop bits (1, 2) that you want MyComm to use when

connected to the system.  1 stop bit is the most common.

3.6.2.7  Default Protocol

MyComm allows you to choose the file transfer protocol that you

will use by default for each system you call.  Choose the

protocol which best suits your needs.  A more complete

description of the various protocols is found later in this

manual in the section on Uploading and DownLoading Files.

3.6.2.8  Dial Prefix

Choose the dialing prefix that you want MyComm to use when

calling the number for this system.  MyComm has slots for three

different dialing prefixes.  Choose one by moving the highlight

bar to the selection you want.  The dialing prefixes may be

changed to suit your needs in the Update Configuration menu

option.

3.6.2.9  Logon Name

This option lets you enter the name that you log on with when

you call this system.  When bulletin board systems answer your

call they usually ask for your name and password.

Putting the characters "^M" at the end of the Logon Name causes

an <Enter> character to be sent.  Some BBS's don't like seeing an

<Enter> character immediately after the Logon Name string when

logging on, in which case you should leave the "^M" out.

If Logon Name is filled in MyComm will send your logon name

when you push the <F2> key to save you some typing.

MyComm Version 2.31 Documentation                 Page 18

3.6.2.10 Password

This option lets you enter the password that you log on with

when you call this system.  When bulletin board systems answer

your call they usually ask for your name and password.

If Password is filled in MyComm will send your logon password

when you push the <F3> key to save you some typing.  If you are

really security conscious, then you may leave this option blank.

Putting the characters "^M" at the end of the Password causes

an <Enter> character to be sent.  Some BBS's don't like seeing an

<Enter> character immediately after the Password string when

logging on, in which case you may leave the "^M" out.

3.6.2.11 Terminal Type

This option lets you choose the type of terminal emulation to

use when calling this system.  The choices are TTY, ANSI, VT-52,

VT-100 and 3270 (registered version only).

TTY (TeleTYpe) gives no emulation what-so-ever. The characters

appear on your screen just as they come in with no formatting.

ANSI emulation provides functions similar to your ANSI.SYS

driver to provide color attributes (except on mono monitors of

course), blinking characters and other goodies.  MyComm uses

intelligent ANSI character translation which filters out any ANSI

sequences which cause "keyboard re-mapping".  This prevents the

unscrupulous from sending ANSI sequences to your personal

computer system that could potentially cause damage or annoyance.

VT-52 and VT-100, and 3270 are most commonly used on mainframe

hosts.  Some bulletin boards also support these.  Both of these

emulations support a subset of the DEC protocols including a

mapping to the "keypad mode" used to generate commands on DEC

terminals.  It should be noted that VT-52 is an "ANSI

incompatible" emulation, and VT-100 is ANSI compatible.

A listing of the keys supported for the ANSI, VT-52, VT-100,

and 3270 emulations is in Appendix C - Terminal Emulations.

3.6.2.12 Local Echo

Local Echo is used when the other system does not send the

characters typed on your terminal back to you.  Most systems do

echo characters back, so Local Echo is normally off ("No").  If

you find that you can't see what you are typing when connected to

a system, set Local Echo on ("Yes").  On the other hand if you

see double characters when you hit a key (for example you see

"xx" when you type an "x") then Local Echo should be off ("No").

MyComm Version 2.31 Documentation                 Page 19

3.6.2.13 Received BS (BackSpace) Destructive

This option tells MyComm whether received BackSpace or DELete

characters should back up the cursor and erase what was there.

Usually this On.

3.6.2.14 BS Key Sends BS or DELete

This option tells MyComm whether to send a BS (BackSpace - 8

decimal) or DEL (Delete - 127 decimal) character when the

backspace key is pressed.  Usually this is set to BS.  Mainframe

hosts usually like to see DEL rather than BS.

3.6.2.15 Strip Hi Bits

This option tells MyComm whether to strip the high bit from

received and sent characters. Usually this is set to "No".

Just for your own information, bytes on the personal computer

are 8 bits long.  So, the bit-map of a byte might be 10011101.

The left-most bit is the high bit.  So if you were to strip the

high bit, the byte above would become 00011101.  Pure ASCII only

covers bytes in the range of 00000000 (0 decimal) to 01111111

(127 decimal).  The ANSI extended set includes values from

10000000 (128 decimal) to 11111111 (255 decimal).  So by

stripping the high bit, a character is guaranteed to be in the

range of the pure ASCII character set.

3.6.2.16 Add LF after incoming CR

This option tells MyComm whether to add a LF (Line Feed - 10

decimal) character after a CR (Carriage Return - 13 decimal)

character is received.  Usually this is set to "No".

If you notice that the lines on the screen seem to look odd or

do not scroll up properly, you will want to turn this on (Yes).

3.6.2.17 Clear last date called and number of calls

Each time you connect with a system MyComm updates the last

date and time called portion the dialing entry record.  If you

want to reset the date and time the system was last called and

the number of times you have called it, answer "Yes" here.

3.6.2.18 Script

This is where you will put the name of the script file that you

want to run ("Join") when you connect with this system.  For a

complete discussion of the MyComm script language see Appendix -

E "MyComm Script Language".

MyComm Version 2.31 Documentation                 Page 20

3.6.2.19 Save Changes (not shown in example)

MyComm asks if you want to save the changes you just made.  If

you answer "Yes" MyComm updates the phone file on your disk

drive.  When MyComm updates the phone file a backup of the phone

file is created and named PHONE.OLD.

3.6.3  Delete an Item

Pressing the "T" key allows you to delete the item that is

currently at the dialing highlight bar.  MyComm asks you to

confirm the delete request before proceeding.  MyComm also

updates the phone file on your disk drive.

3.6.4  Call Numbers (Dial)

Pressing the "C" key tells MyComm that it's time to start

calling numbers in the Dialing List  (See also section 3.6.7

(Un)MarK).  Pressing the <Enter> key also will start the dialing.

If no Dialing List Items are "marked" MyComm will dial the

number that the highlight bar is currently resting on.  If there

are "marked" Dialing List Items, MyComm will start dialing the

marked numbers in sequence until a connection is made.  During

the dialing attempts you may cancel the dialing by pressing the

<Esc> key.

While MyComm is calling numbers and waiting for a connection, a

Dial Status Window is displayed (see example below).  The status

window shows who is being called and the telephone number, how

many call tries have been made so far, the status of the last

dialing try, the time in seconds until this try "times out", the

time since you started calling, and the time that the current

dialing attempt started.

+-------------------------------------------------------| Dial |+

|                                                               |

|  Calling: Your Favorite BBS        At: 555-4657               |

|                                                               |

|  Try # 1       Last Status: None Yet        Dial Time:  42    |

|                                                               |

|  Dialing started at:  3:01:20   This try started at:  3:01:21 |

|                                                               |

|                                                               |

|          Hit ESC to quit dialing, 'N' for next number,        |

|          or 'D' to delete number from list.                   |

|                                                               |

+---------------------------------------------------------------+

Pressing the "N" key causes MyComm to stop dialing  the number

it is currently dialing and to skip to the next marked number.

Pressing the "D" key causes MyComm to stop dialing the number

it is currently calling and to remove it from the list of

"marked" numbers.  If you press "D" and the number is the only

MyComm Version 2.31 Documentation                 Page 21

one left in the "marked" list, MyComm will give you a warning and

stop dialing.

The Dial Time and Pause Time between calls is a user

configuration option which may be set up in the Update

Configuration menu described later in this document.

3.6.5  Phone Item Info Screen

The Phone Item Info Screen looks very much like the Edit Phone

List Item Screen with the exception that the Last Call Date, Last

Call Time, and Number of Calls fields are shown.  If you see

something you want to change, pressing "E" will bring you to the

Edit Phone List Item Screen.

+------------------------------| Phone Item Info |+

|                                                 |

| Name:                 Your Favorite BBS         |

| Number:               555-4657                  |

| Baud Rate: 2400       Data Bits: 8              |

| Data Parity: None     Stop Bits: 1              |

| Default Protocol:     1K-Xmodem                 |

| Dial Prefix: 1        ATDT*70                   |

| Logon Name:           Your Name                 |

| Password:             Password                  |

| Terminal Type: ANSI                             |

| Local Echo: No                                  |

| Received BS Destructive: Yes                    |

| BS Key Sends BS or DELete: BS                   |

| Strip Hi Bits? (Pure ASCII): No                 |

| Add LF after incoming CR?: No                   |

|                                                 |

| Last Call Date:       01/26/92                  |

| Last Call Time:        4:26:12                  |

| Number of Calls:            28                  |

|                                                 |

|          Press "E" to Edit Record               |

+-------------------------------------------------+

3.6.6  Manual Dial Screen

Pressing "M" brings up the Manual Dial Screen.  The Manual Dial

Screen lets you enter a single phone number to dial.  The dialing

prefix used is always Dialing Prefix number 1.

Pressing <Esc> cancels the function.  Pressing <Enter> accepts

the telephone number and the usual Dialing Status Window appears.

+-------------------------------------| Manual Dial |+

|                                                    |

|  Enter phone number: 555-1234                      |

|                                                    |

+----------------------------------------------------+

MyComm Version 2.31 Documentation                 Page 22

3.6.7  Mark Phone List Items

Pressing "K" marks or un-marks phone list items for later

dialing.  As a convenience, pressing the <Space> bar also marks

and un-marks items.  When a phone list item is marked a chevron

("�") will appear next to the item.

3.6.8  Find Phone List Items

Pressing "F" brings up the Find Phone List Items screen (not

shown).  Simply type in the characters in the name, or a partial

phone number that you want to find.  The search is started at the

top of the list.  If the search fails, a message is displayed.

3.6.9  Load Phone List Screen

Pressing "L" brings up the Load Phone List Screen.  You may

keep more than one phone dialing list.  Each list will hold up to

100 telephone numbers.  If the dialing list you select does not

yet exist, MyComm will display an empty dialing list.

Again, when changes are made to the phone file, MyComm will

create or update the file PHONE.OLD as a backup.  There is only

one PHONE.OLD backup file kept, so all new backups overwrite the

last PHONE.OLD backup file.

3.6.10 Sort Phone List

You may sort the Phone List in one of four ways:

- by BBS name

- by Telephone number

- by last date called

- by number of times called

The sort option will also ask you for Ascending or Descending

sort order.  Simply select the sort option that best suits your

needs.  The list will be saved in the new sort order.  Pressing

<Esc> exits the Sort menu.

3.6.11 Move Phone List Item

This option allows you to move a phone list item to another

location in the list.  This will save you some retyping if you

want to customize your phone list.

To use this option move the highlight bar to the phone list

item that you want to move.  Press the "V" key.  Next use the up

and down arrow keys to move the highlight bar to the position

that you would like the item moved to.  Press the <Enter> key to

make the move permanent, or <Esc> to quit the move.

MyComm Version 2.31 Documentation                 Page 23

3.6.12 Print Phone List

This option allows you to print the phone list.  The phone list

may also be "printed" to a text file.  The printed output shows

the BBS name, phone number, and last date called.

To use this option press "P".  You will then be asked for a

file name to "print" to.  To print to your printer (PRN) simply

press <Enter> (PRN is a standard DOS device name for the

printer).  A status message is displayed while the phone list is

being printed.  The printing may be stopped by pressing the <Esc>

key.

3.7  Local Echo On/Off - Alt E

Local Echo is used when the other system does not send the

characters you typed on your terminal back to you.  Most systems

do echo characters back, so Local Echo is normally off ("No").

If you find that you can't see what you are typing when connected

to a system, set Local Echo on (Yes).  On the other hand if you

see double characters when you hit a key (for example you see

"xx" when you type an "x") then Local Echo should be off ("No").

3.8  Line Feeds On/Off - Alt F

This function toggles the "Line Feeds" on or off.  When "on"

Line Feed characters are appended to the display in the terminal

window each time a Carriage Return character is received.  When

"off" no Line Feed character is added.

This is useful when the host system is not sending Line Feed

characters to you and the display does not drop down to a new

line when each new line of text is received.

3.9  Hang Up Modem - Alt H

Pressing the <ALT> and the "H" key tells MyComm that you want

to disconnect from your current telephone connection.  MyComm

will try to hang up the phone first by dropping the modem signal

called DTR, and if that fails by sending the Hang Up String

defined in the Modem Setup Screen (see section 3.17.1  Modem

Setup Screen).  If you don't want MyComm to use DTR to hang up

the modem set the option "Use DTR to Hang Up Modem" in Modem

Setup Screen to "No".

3.10  Send Modem Initialization Strings - Alt I

Pressing the <ALT> and the "I" key brings up the Send Init.

String Menu shown below.  Choosing one of these options will send

the Modem Initialization String, or the Hang Up String (same as

<Alt> + H), or the Modem Attention String, or the Phone Answer

String.  These strings are defined in the Modem Setup Screen (see

section 3.17.1  Modem Setup Screen).

MyComm Version 2.31 Documentation                 Page 24

+-------| Send... |+

| Init. String     |

| Hang Up String   |

| Attention String |

| Phone Answer     |

+------------------+

3.11 Join Script - Alt J

This option will allow you to "Join" in or run a script file.

You will be asked to supply the name of the script file which

MyComm will then load.  If the script file can not be opened you

will see an error message.  You may quit the script by pressing

the <Esc> key until you see the "Yes/No" window that asks if you

want to quit the script.  See Appendix-E "Script Language" for

more information on script files.

3.12 Present Line Status - Alt L

Pressing the <ALT> and the "L" key brings up the Present Line

Status Window.  This window tells you the current COM Port

Status, i.e. what Port, Baud Rate, Data Bits, Parity Bits, and

Stop Bits are in effect, whether you are on-line and for how

long.  If you are on-line MyComm tells you the name of the system

you are logged onto, and the default file transfer protocol.

Also shown is the present Local Echo status and whether Session

Capture is active.  Pressing <Esc> closes the window.

+--------------------------| Present Line Status |+

|                                                 |

|  COM Port Status COM2, 2400, 8, N, 1           |

|    ----------------------------------------     |

|                                                 |

| On-line: Yes          Time On-line:  0:00:00    |

| Logged on to:         My Favorite BBS           |

| Current Protocol:     None                      |

|    ----------------------------------------     |

|                                                 |

| Local Echo On:      No                          |

| Screen Capture On:  Yes                         |

|    ----------------------------------------     |

|                                                 |

| Current Time: 12:34:56          Date: 01/23/93  |

+-------------------------------------------------+

3.13 Scroll Back - Alt K

Pressing the <ALT> and the "K" key brings up the "scroll back"

buffer.  MyComm saves the last few screens of received text for

you.  This option allows you to see what has been recently

received in case there was something that went by too quickly

that you would like to look at again.  The scroll back buffer

saves about two to three screens worth of information depending

MyComm Version 2.31 Documentation                 Page 25

on how much actual text was in the previous screens (about 4K's

worth).  The scroll back buffer does not display ANSI or high bit

characters, they are stripped out to leave more room for more

useful information.

When the scroll back buffer is first shown it displays the very

beginning of the buffer information.  The up-arrow, down-arrow,

Page Up, Page Down, Home and End keys allow you to move through

the text.  You may also write what is in the Scroll Back Buffer

to a file by pressing the "W" key.  To exit press the <Esc> key.

3.14 Manual Dial - Alt M

Pressing the <ALT> and the "M" key brings up the Manual Dial

Screen.  The Manual Dial Screen lets you enter in a single phone

number to dial.  The dialing prefix used is always Dialing Prefix

number 1.  The parity setting is always 8-N-1.

If you press <Esc> you may cancel the selection.  Pressing

<Enter> accepts the telephone number and the usual Dialing Status

Window appears.  This is exactly the same function as in the

Dialing Screen, selection "M", described above.

3.15 Outside to DOS - Alt O

Pressing the <ALT> and the "O" key lets you shell out of MyComm

to DOS.  You may use the Outside to DOS even when you are on-

line.  Typing "EXIT" on the DOS command line brings you back to

MyComm.

One point of interest here: if you switch to a different drive

or directory while in the DOS shell, MyComm will automagically

bring you back to the drive and directory you started from when

you type "EXIT" to return to MyComm.

3.16 Change Port Parameters - Alt P

Pressing the <ALT> and the "P" key brings up the Change Port

Parameters Screen (see example below).  MyComm will let you

change the port configuration, even while on-line.  Please note

however that changing the COM Port while on-line will probably

disconnect the current session.  However Data, Parity, and Stop

Bits may be changed.

Please note that the changes done here are not permanent.  To

make permanent changes to the COM port settings, you must use the

"Update Configuration" which is brought up by pressing <Alt> and

"U" then the "M" key.

MyComm Version 2.31 Documentation                 Page 26

+-----------------------| Change Port Parameters |+

|                                                 |

| Present settings: COM2, 2400, 8, N, 1           |

|                                                 |

| COM Port:2                                      |

|                                                 |

|           +---------+                           |

|           | 1: COM1 |                           |

+-----------| 2: COM2 |---------------------------+

| 3: COM3 |

| 4: COM4 |

| 5: COM5 |

| 6: COM6 |

| 7: COM7 |

| 8: COM8 |

+---------+

3.17 Quick Re-Dial - Alt Q

Pressing the <ALT> and the "Q" key allows you to redial the

last number that MyComm dialed.  This is especially useful for

re-dialing manually entered telephone numbers.

3.18 Run External Program - Alt R

Pressing the <ALT> and the "R" key brings up the Run External

Program Screen (not shown).  MyComm lets you define an editor

program plus up to three other programs that you may run without

leaving your MyComm session.  The external programs are defined

in Paths Setup (see 3.19.2  Paths Setup).

If there is not enough available memory to run the external

program, the external program will not run and control passes

directly back to MyComm.

3.19 Session Capture On/Off/Pause - Alt S

Pressing the <ALT> and the "S" key opens the MyComm Session

Capture Screen (not shown).  You may either accept the default

name that MyComm provides for the capture file or enter a new

name.  If the Drive:\Path\Filename is in any way invalid, MyComm

will issue an error message.  You may also press <Esc> to leave

the Session Capture Screen without starting the Session Capture.

After a Session Capture has begun pressing the <ALT> and the

"S" key again will allow you to Close, or Pause/Resume the

capture file.

If you exit MyComm and forget to Close the Session Capture

file, MyComm will do it for you.

Please note that often while Session Capture is active, that

the display may appear jerky, or start and stop from time to

time.  This is normal during Session Capture as it is the time

that MyComm is writing the file to disk.

MyComm Version 2.31 Documentation                 Page 27

It may also be important to know that the Session Capture does

not do any ANSI filtering.  What you see is what you get.

Generally you will read the Session Capture file using a text

editor or word processor when you are no longer "on line".  Text

editors and word processors often perform ANSI filtering for you.

3.20 Terminal Emulation - Alt T

MyComm allows you to reset the Terminal Emulation even while

on-line.  The options are: TTY, ANSI, VT-52, VT-100, and 3270.

Further information on the emulation may be found in Appendix C -

"Terminal Emulations".

3.21 Update Configuration - Alt U

Pressing the <ALT> and the "U" key brings up the Update

Configuration Menu (see below).  From this menu you may choose

various options to allow you to customize the MyComm

configuration.  You may either use the Up and Down-Arrow keys,

then press <Enter> to choose an option, or simply press the

highlighted letter of the option you want.  A description of the

options that may be configured follows.

+---| Update Configuration |+

| Modem Setup               |

| Paths and Filenames       |

| File Transfer Options     |

| Other Stuff for MyComm    |

| COM Port Addresses        |

| Save Configuration        |

| Exit                      |

+---------------------------+

3.21.1  Modem Setup Screen

The Modem Setup Screen (see below) allows you to make changes

to the various setting that control the modem.  You may move from

item to item by using the Up and Down-Arrow keys and then

pressing <Enter>.  Some of the items use MyComm's built in line

editor to modify the item, others use little pop-up Yes/No

windows.  If you want to exit without making any changes, just

press the <Esc> key.  A description of each item in the Modem

Setup Screen follows.

MyComm Version 2.31 Documentation                 Page 28

+------------------------------------------------| Modem Setup |+

|                                                               |

| COM Port:                        COM2, 2400, 8, N, 1          |

| Modem Initialization String:     ATM0X4Q0V1S0=0S11=50S7=50^M  |

| Modem Attention String:          +++                          |

| Modem Hang Up String:            ATH0^M                       |

| Modem Answer String:             ATA^M                        |

| Modem Cancel Dial String:        ^M                           |

| Modem Dial Suffix String:        ^M                           |

| Dialing Wait Time, in Seconds:    45                          |

| Pause Between Calls, in Seconds:   4                          |

| Local Echo on by Default?        No                           |

| Use DTR to Hang Up Modem?        Yes                          |

| Use XON/XOFF Flow Control?       Yes                          |

| Use CTS/RTS Flow Control?        No                           |

| Use DSR/DTR Flow Control?        No                           |

| Dialing Prefix #1:               ATDT*70                      |

| Dialing Prefix #2:               ATDT 9,                      |

| Dialing Prefix #3:               ATDT 1                       |

|                                                               |

|                                                               |

|          Use Up and Down Arrows to move between items         |

|               Hit <Enter> to edit, <Esc> to Quit              |

|                                                               |

+---------------------------------------------------------------+

3.21.1.1  COM Port

The COM Port selection allows you to set various COM port

settings.

When you first installed your modem you probably set the COM

port up for COM1, COM2, etc. use the setting correct for your

modem.  If you set the wrong COM port in this area, MyComm

doesn't mind, but won't let you dial numbers later on.

MyComm does not re-initialize the COM port when you make

changes in this menu.  The new COM port settings will be used the

next time that you run MyComm.  If you need to change the COM

port settings now, use the Alt-P function from the main terminal

screen.

The baud rate should be set to whatever the top speed of your

modem is.  Refer to your modem user manual if you're not sure.

The most common settings for the Data, Parity, and Stop bits

are 8-N-1 (8 data bits, no parity, 1 stop bit).  If you require

different settings, this is the place to change them.

3.21.1.2  Modem Initialization String

This option lets you set up the Modem Initialization String.

This is the string which is sent to the modem to "wake it up" and

set options when MyComm is first loaded.  The initialization

string is made up of Hayes "AT" commands.  The command line

always starts with the letters "AT".  The rest of the line

MyComm Version 2.31 Documentation                 Page 29

contains various options for your modem.  Your modem manual

should have a list of the commands that are valid for your modem.

MyComm always likes to have AT LEAST the Q0, V1, and X4

commands.  Q0 tells the modem to ALWAYS send result codes, V1

tells the modem to ALWAYS send the text, not numeric, result

codes.  MyComm needs the result codes in text format to monitor

call progress, and to send you other useful information that the

modem produces.  X4 (your modem may use X1, X2, etc) tells the

modem to send "verbose" modem status information to MyComm.

You should place "^M" at the end of the Initialization String

so that MyComm will send a Carriage Return at the end of the

line.

High speed modem users: If you need to set "flow control" options

for your modem in the setup string this is the place to do it.

For example, if your modem needs to use RTS/CTS flow control the

Hayes AT command "&K3" should be added to the initialization

string.

3.21.1.3  Modem Attention String

This is the Attention signal that tells the modem that you want

it's undivided attention for something.  Most Hayes compatible

modems use three plus signs "+++" as the attention string.  Refer

to your modem manual if you use a different Attention string.

3.21.1.4  Modem Hang Up String

This is the string that tells your modem to hang up the

telephone line.  On most Hayes compatible modems this is ATH0.

You may use the characters "^M" to have MyComm send a Carriage

Return after it sends the Hang Up string (recommended).  Refer to

your modem manual if you use a different Hang Up string.

3.21.1.5  Modem Answer String

This is the string that tells your modem to answer the

telephone.  On most Hayes compatible modems this is ATA.  You may

use the characters "^M" to have MyComm send a Carriage Return

after it sends the Answer string (recommended).  Refer to your

modem manual if you use a different Answer string.

3.21.1.6  Modem Cancel Dial String

This is the string that tells your modem to cancel dialing a

telephone number.  On most Hayes compatible modems this just a

Carriage Return (13 decimal).  You may use the characters "^M" to

send a Carriage Return after it sends the Cancel Dial string

(recommended).  Refer to your modem manual if you use a different

Cancel dialing string.

MyComm Version 2.31 Documentation                 Page 30

3.21.1.7  Modem Dial Suffix String

This is the string that tells your modem what to terminate

telephone numbers with when dialing.  On most Hayes compatible

modems this just a Carriage Return.  You may use the characters

"^M" to have MyComm send a Carriage Return as the Dial Suffix

string (recommended).  Refer to your modem manual if you use a

different Dial Suffix string.

3.21.1.8  Dialing Wait Time

After MyComm dials a number it waits for this amount of time

for a connection.  Waiting stops automatically if MyComm detects

a BUSY signal.  The valid range for Dialing Wait Time is 1 to 240

seconds.

In general a wait time of 45 seconds is good for most local and

long-distance calls, 60 seconds for international calls.

You will also want to add "S7=nn" (without the quotes), to your

modem Initialization String (see above). Replace "nn" with a

number greater than or equal to the time set in Dialing Wait

Time.

3.21.1.9  Pause Between Calls

If a connection is not made after dialing due to a BUSY signal

or a time-out, MyComm resets the modem and waits the amount of

seconds specified in the Pause Between Calls before trying to

dial again.  The valid range for Pause Between Calls is 0 to 60

seconds.

In most cases a pause time of 2 to 5 seconds is sufficient for

the modem to reset.

3.21.1.10 Local Echo on by Default

Local Echo is used when the other system does not send what you

typed on your terminal back to you.  Most systems do echo

characters back, so Local Echo is normally off ("No").  If you

find that you can't see what you are typing when connected to a

system, set Local Echo on (Yes).  On the other hand if you see

double characters when you hit a key (for example you see "xx"

when you type an "x") then Local Echo should be off ("No").

3.21.1.11 Use DTR to Hang Up Modem

This option allows you to set whether MyComm should use the

modem signal called DTR to terminate connections.  Using DTR to

hang up is usually faster than just sending the Hang Up String

(above).  This option is usually set to "Yes".

MyComm Version 2.31 Documentation                 Page 31

However, some modems don't like to have DTR messed with. You

can usually tell if the modem doesn't like you to fool with DTR

as it will cause your computer system to lock up (not a bad thing

really, just terribly annoying - re-booting always fixes this).

Refer to your modem manual if you have questions about whether to

use DTR to hang up the telephone connection.

NOTE: some modems require that you add "&D2" to your

initialization string (see above) to enable the DTR line

monitoring.  Check your modem manual for further details.

3.21.1.12 Use XON/XOFF Flow Control

This tells MyComm whether it should use the XON/XOFF flow

control while in terminal mode and during ASCII file transfers.

This default for this option is "Yes".

3.21.1.13 Use CTS/RTS Flow Control

This tells MyComm whether to use CTS/RTS flow control.

This is used with high speed modems and null modem cable links to

regulate the flow of data. If the modem supports CTS/RTS flow

control this should be on (refer to the user manual for your

modem).  If a null modem cable is being used and these signals

are to be sent over the cable, this should be on.

You may also want to add "&K3" to your initialization string

(see above) to enable your modem to follow CTS/RTS status.  This

option is most often used on high speed modems.

3.21.1.14 Use DSR/DTR Flow Control

This tells MyComm whether to use DSR/DTR flow control.

Hardware handshaking is used with high speed modems and

sometimes null modem cable links to regulate the flow of data.

If the modem supports DSR/DTR flow control this should be "Yes"

(refer to the user manual for your modem).  If a null modem cable

is being used and these signals are to be sent over the cable,

this should be on.

You may also want to add "&D2" to your initialization string

(see above) to enable your modem to follow DTR status.  This

option is most often used on high speed modems.

3.21.1.15 Dialing Prefixes

Dialing Prefixes are dialing codes such as area codes, that get

sent before a telephone number is dialed.  You may set up three

different dialing prefixes.  Each dialing prefix may be up to 25

character/digits long.

For example, if you have Call Waiting (Tm) you can disable it

by dialing *70 (that's asterisk - 70) before the number you're

calling (this works in the my dialing area, your dialing area may

MyComm Version 2.31 Documentation                 Page 32

differ).  If you were to set one of your dialing prefixes to *70

MyComm would "prefix" every number you called with *70.  The

dialing prefixes may also be used to store long distance codes,

or whatever dialing codes you would want to have dialed before a

number.

3.21.1.16 Connect Strings

MyComm will use the strings defined in Connect and No Connect

strings to determine the dialing status.  From this option you

will be able to change the strings that MyComm will use as

responses from your modem.

There is one "Connect" string and four "No Connect" strings.

The Connect string should be the same as the one your modem

generates when a connection has been made.  The default is

"CONNECT" which is fairly standard.  Please note: you must define

this string so that MyComm will know when a connection has been

made.  MyComm does not rely on the Carrier Detect signal to

determine when it has connected during dialing!

The "No Connect" strings should be set to whatever strings your

modem generates when a connection is not made during dialing.

When MyComm sees one of these responses from the modem during

dialing it stops waiting for a connection and moves on to the

next number to be dialed.  The default strings are "BUSY", "NO

CARRIER", "NO DIALTONE", and "VOICE".

3.21.2  Paths Setup

The Paths Setup Screen (see below) is where you set up paths to

directories that you want MyComm to know about.  A description of

the various system paths follows.

+---------------------------------------------| Paths Setup |+

|                                                            |

| Directory to Download files to:  C:\PUBLIC\                |

| Directory to Upload files from:  C:\PUBLIC\                |

| Path\File for Screen Capture:    C:\PUBLIC\MYCOMM.SCR      |

| Path\File for Session Capture:   C:\PUBLIC\MYCOMM.SES      |

| Path\File for Phone File:        C:\MYCOMM\MYCOMM.PHN      |

| Path\Filename.Ext for Editor:    C:\UTIL\EDITOR.EXE        |

| Path\Filename.Ext of Xter Prog1: C:\SOMEPRGM.EXE           |

| Path\Filename.Ext of Xter Prog2: C:\                       |

| Path\Filename.Ext of Xter Prog3: C:\                       |

|                                                            |

|                                                            |

|      Use Up and Down Arrows to move between items          |

|           Hit <Enter> to edit, <Esc> to Quit               |

|                                                            |

+------------------------------------------------------------+

MyComm Version 2.31 Documentation                 Page 33

3.21.2.1  Download Directory

This option tells MyComm which directory you want MyComm to put

files that are downloaded from another system.  Notice that the

path always ends in a backslash ( "\" ) character.

3.21.2.2  Upload Directory

This option tells MyComm which directory you want MyComm to get

the files from that will be uploaded to other systems.  Notice

that the path always ends in a backslash ( "\" ) character.

3.21.2.3  Path\File for Screen Capture

This option tells MyComm which directory and the filename that

you want MyComm to use for the Screen Capture file.  Screen

Capture is activated by <Alt> + W.

3.21.2.4  Session Capture Path\File

This option tells MyComm which directory and the filename that

you want MyComm to use for the Session Capture file.  Session

Capture is activated by <Alt> + S.

3.21.2.5  Phone File Path\File

This option tells MyComm which directory and the filename that

you want MyComm to use for the Phone File which contains your

dialing directory.  You may have multiple Phone Files, but this

is the one that MyComm will load when it starts up.

3.21.2.6  Editor Path\Filename

If you want to use an editor program without leaving MyComm,

this option lets you tell MyComm the path and filename of the

editor.  Notice that the filename of the editor must include not

only the filename, but the extension (.EXE or .COM) as well.

3.21.2.7  External Programs Path\Filename

You may define up to three other external programs to run

without leaving your MyComm session.  Use these options to tell

MyComm the paths and filenames of the programs.  Notice that the

filename of the programs must include not only the filename, but

the extension (.EXE, .COM, or .BAT) as well.

MyComm Version 2.31 Documentation                 Page 34

3.21.3  Protocols Setup

The Protocols Setup Menu (see example below) allows changes to

parameters for ASCII file transfers, Kermit file transfers,

Zmodem file transfers, and the external protocols.

The usage of these options follows.

+--| Protocol Setup |+

| ASCII Transfers    |

| Zmodem Transfers   |

| Kermit Transfers   |

| EXternal Protocols |

| Exit               |

+--------------------+

3.21.3.1  ASCII Transfers Setup

The ASCII Transfers Setup screen (see example below) lets you

set up the parameters for ASCII file transfers.  ASCII file

transfers are only used to transfer text files.  Program files

and archived files, also known as binary files, should not be

transferred as ASCII files.  Most of the options for the ASCII

transfers work well with their default settings.  A description

of the different options follows.

+----------------------------| ASCII Transfers Setup |+

|                                                     |

| Strip High Bits?:                 No                |

| Remote Abort Char (0 for none):    0 (ASCII Value)  |

|                                                     |

| ASCII Upload Options                                |

| CR Translation:                 None                |

| LF Translation:                 None                |

| Local Echo On During Uploads?:   Yes                |

| Expand Blank Lines to Spaces?:    No                |

| Line Pace time in 1/10 seconds:    0                |

| Char Pace time in 1/10 seconds:    0                |

| Pacing Char (0 for none):          0 (ASCII Value)  |

|                                                     |

| ASCII Download Options                              |

| CR Translation:                 None                |

| LF Translation:                 None                |

|                                                     |

|    Use Up and Down Arrows to move between items     |

|          Hit <Enter> to edit, <Esc> to Quit         |

+-----------------------------------------------------+

3.21.3.1.1  Strip High Bits

This option tells MyComm whether to strip the high bit from

received and sent characters during ASCII uploads and downloads.

Usually this is set to "No".  See also section 3.6.2.15 "Strip

High Bits" for a discussion of "high bits".

MyComm Version 2.31 Documentation                 Page 35

3.21.3.1.2  Remote Abort Char

If the system you're performing the ASCII file transfer with

uses an abort char to cancel transfers you should set this value.

The value is in the range of 1 to 255.  A value of zero (0), the

default, means MyComm won't look for the abort character during

ASCII file transfers.

3.21.3.1.3  CR Translation Upload and Download

Some systems require a CR (Carriage Return) character placed

before the LF (Line Feed) character during ASCII file transfers.

If the lines of the file look odd after the transfer, this may be

the case.

3.21.3.1.4  LF Translation Upload and Download

Some systems require a LF (Line Feed) character placed after

the CR (Carriage Return)  character during ASCII file transfers.

If the lines of the file look odd after the transfer, this may be

the case.

3.21.3.1.5  Local Echo On During Uploads

During ASCII uploads if you want to watch the file transfer set

this option to "Yes".  This is often used when uploading a

message into a bulletin boards message base.  When this option is

set to "No", the usual file transfer window is shown as the

upload progresses.  Please note that when this option is set to

"No" a Ctrl-Z (end of file marker) character is sent at the end

of the file transfer.  When set to "Yes" the Ctrl-Z is not sent.

3.21.3.1.6  Expand Blank Lines to Spaces

This option tells MyComm to expand lines that are empty except

for a CR/LF into a line of spaces that end with a CR/LF.  This is

useful when you are uploading a message to a bulletin board.

Some boards interpret a blank line with only a CR/LF as the end

of message signal.  Expanding blank lines allows you to upload

the entire message.

3.21.3.1.7  Line Pace Time

This options tells MyComm to how much time to wait between

sending each line of text from the ASCII file.  Some systems

can't handle high speed ASCII transfers, this gives the other

system time to "think".  The value entered from 0 (the default)

to 255 are in increments of 1/10 second (approximately).

MyComm Version 2.31 Documentation                 Page 36

3.21.3.1.8  Character Pace Time

This options tells MyComm how much time to wait before sending

each character from the ASCII file.  Some systems can't handle

high speed ASCII transfers, this gives the other system time to

"think".  The value entered from 0 (the default) to 255 is in

increments of 1/10 second (approximately).

3.21.3.1.9  Pacing Char

Some systems send a Pacing Character to limit throughput on

ASCII transfers.  MyComm will wait for the Pacing Char before

sending each line of the file.  The Pacing Char value ranges from

1 to 255 which represents an ASCII value.  A value of zero (0),

the default, means MyComm will not look for the Pacing Char.

3.21.3.2  Zmodem Transfers Setup

MyComm lets you define certain special functions used in the

Zmodem file transfer protocol.  A description of these follows.

3.21.3.2.1  Zmodem 32 Bit CRC

Zmodem uses either a 16 bit or a 32 bit CRC for error detection

during file transfers.  The 32 bit CRC provides a much higher

degree of accuracy than its 16 bit cousin.  However the

calculation involved in the 32 bit CRC may slow the file transfer

rate (by only a few CPS though).  If you want a high degree of

error detection this option should be set to "Yes".  For higher

transfer speed, set this option to "No".

3.21.3.2.2  Zmodem Auto Download Feature

When this option is set to "Yes" MyComm will automatically

start a Zmodem download file transfer for you.  The Zmodem

protocol at the host's end sends some special characters when it

starts up a Zmodem transfer.  MyComm looks for these characters

and will save you some typing by starting up the Zmodem download

for you.  If you don't want to use this feature, set this option

to "No".

3.21.3.2.3  Zmodem Send Crash Recovery

Sometimes a file transfer will fail after a file has been

partially sent.   A file transfer could fail due to excessive

line noise for example.  Zmodem has a rather nifty feature that

will attempt to restart a failed file transfer.

For normal use this feature should be set to "No".  If you want

to attempt to restart a failed Zmodem transfer set this option to

"Yes" before beginning the upload again.  If the system you are

MyComm Version 2.31 Documentation                 Page 37

trying to upload the file to supports the "Crash Recovery"

feature the file transfer will resume where it left off.

3.21.3.2.4  Zmodem Receive Crash Recovery

Sometimes a file transfer will fail after a file has been

partially received.  Zmodem has a rather nifty feature that will

attempt to restart a failed file transfer.

For Normal use this feature should be set to "No".  If you want

to attempt to restart a failed Zmodem transfer then set this

option to "Yes" before beginning the download again.  If the

system you are trying to download the file from supports the

"Crash Recovery" feature the file transfer will resume where it

left off.

THIS OPTION SHOULD BE USED WITH CARE.  If this option is set to

"Yes" and you download a file from another system that has the

same name as a file you already have in your download directory,

the end of the existing file will be overwritten with the

information from the new file.  This option should only be set to

"Yes" when you are attempting to restart a previously failed

Zmodem transfer.

NOTE: the Zmodem Crash Recovery option has some rudimentary

idiot proofing.  The option will not perform the transfer if the

file you have has a different time/date stamp than the other

system, or if the file that you have is larger than the file on

the other system.

MyComm Version 2.31 Documentation                 Page 38

3.21.3.3  Kermit Transfers Setup

Kermit was designed to allow many different types of machines

to send data to each other.  In order to do this Kermit places a

special "escape" character before any control characters

encountered.  However this compatibility does have its price,

Kermit is one of the slowest protocols in use today.  There are

several parameters that may be adjusted. Most people will never

need to change any of these, but for those who may a brief

description follows.

File Format:             Binary or ASCII. Generally 8 bit or 8th

bit stripped (sometimes called "Text"

mode) respectively

Packet Size:             Allowable range is 10 to 94

Number of Pad Chars:     0 - none, if needed valid range 1 to 94

Pad Char:                0 - ignored, if needed valid range 1 to

94

Quote Char:              35 ASCII, 23 hex, # symbol by default

Packet Start Char:       01 ASCII, SOH by default

EOLN Char:               13 ASCII, 0D hex, carriage return by

default

3.21.3.4  External Protocols Setup

Up to six external file transfer protocols may be defined.

Once defined the external file transfer protocols will appear in

the Upload and Download protocol selection menus.

Use the Up and Down-Arrow keys to move from item to item.  The

<Enter> key selects an item for editing and saves the completed

edit.  Pressing the <Esc> key quits the edit.

+-----------------------------------| External Protocols Setup |+

|                                                               |

| Protocol  Name    Upload.Bat    Download.Bat     Ask FileName |

|                                                               |

|   1:   BIGFAST    BIGFASTU.BAT  BIGFASTD.BAT     No           |

|   2:                                             Yes          |

|   3:                                             Yes          |

|   4:                                             Yes          |

|   5:                                             Yes          |

|   6:                                             Yes          |

|                                                               |

|         Use Up and Down Arrows to move between items          |

|               Hit <Enter> to edit, <Esc> to Quit              |

+---------------------------------------------------------------+

MyComm Version 2.31 Documentation                 Page 39

The name of the external protocol should be typed in under the

"Name" heading as you would like it to appear in the MyComm

Upload and Download protocol selection menus.

In the fields for Upload.Bat and Download.Bat you will enter

the names of the DOS batch files that will be used to run the

external protocol.  External file transfer protocols are run as

DOS batch files from MyComm (see your DOS manual for information

on batch files).

In the selection marked "Ask FileName" tells MyComm whether you

should be asked the name of the file when performing downloads

(some file transfer protocols automatically get the filename for

you).

MyComm sends the COM Port number you are using, the Baud Rate,

and the Drive:\Path\ and filename as parameters for the batch

file.  Using the example above, let's say you are using an

external protocol called "BigFast", your COM Port is 2, the Baud

Rate is 2400, and your upload directory is "C:\PUBLIC\" and you

are uploading a file called MYSTUFF.ARC. The batch file for

Uploads with BigFast would look something like this:

REM BigFast Upload batch file

BigFast Send Port: %1  Baud: %2 %3

the %1 would be replaced by a "2", the %2 by "2400", and the %3

would be replaced with the name of the upload directory and the

filename: "C:\PUBLIC\MYSTUFF.ARC" when MyComm calls the .BAT file

to run.

External file transfer protocols often differ quite a bit, but

they usually come with enough documentation of their own to get

you going.

NOTE: Experience has shown that some external protocol drivers

may not release the COM port properly when they have completed

their business.  This causes the terminal to appear "locked" (no

characters come across the scree).  MyComm attempts to restore

the COM port after the external protocol has been called.  If

your terminal appears locked the COM port can be reset by using

the <ALT> + P function from the terminal window.

MyComm Version 2.31 Documentation                 Page 40

3.21.4  Miscellaneous Options Setup

The Miscellaneous Options Setup screen (see example below) lets

you set up special options for MyComm.

+------------------------------------| Misc. Options Setup |+

|                                                           |

| Screen Background Color:         Black                    |

| Screen Text Color:               Gray                     |

| Date Format Style:               MMDDYY                   |

| 12 or 24 Hour Time Format:       12 Hour                  |

| Make Noise When Calls Connect?:  Yes                      |

| Make Noise After File Transfers: Yes                      |

| Use Direct Video? (Faster):      Yes                      |

| Your Name for sign on :          Your Name^M              |

| Password for sign on :           Password^M               |

| Clever quip for TagLine:         Witty quote goes here... |

| Guess Up/Download Filenames:     Yes                      |

| Dialing Screen After Calls:      Yes                      |

|                                                           |

|         Use Up and Down Arrows to move between items      |

|            Hit <Enter> to edit, <Esc> to Quit             |

+-----------------------------------------------------------+

3.21.4.1  Screen Background Color

This option tells MyComm what color to use for the screen

background color.  This option only affects the screen of the

terminal window.

3.21.4.2  Screen Text Color

This option tells MyComm what color to use for the screen

foreground (the text) color.  This option only affects the screen

of the terminal window.

3.21.4.3  Date Format Style

You may choose three date formats for MyComm to use when it

displays dates:  MM/DD/YY,  DD/MM/YY, or YY/MM/DD.  Choose

whichever one best fits your mood.

3.21.4.4  12 or 24 Hour Time Format

MyComm will display the time in either 12 hour or 24 hour

format. Choose the one that bests reflects how you identify with

time.

MyComm Version 2.31 Documentation                 Page 41

3.21.4.5  Make Noise When Calls Connect

MyComm sounds a short alarm noise when a connection is made

during dialing.  If you don't want MyComm to sound the alarm, set

this option to "No".

3.21.4.6  Make Noise After File Transfers

MyComm sounds a short alarm noise after a file transfer has

completed.  If you don't want MyComm to make the noise, set this

option to "No".

3.21.4.7  Use Direct Video

MyComm by default uses a system called "direct video" when

displaying information to the screen.  Unfortunately, direct

video will often "bleed through" to other programs in a

multitasking environment.  If you are using a multitasker and

getting bleed through from MyComm, set the Direct Video option to

"No".

3.21.4.8  Your Name for sign on

This option lets you enter the name that you log on with when

you call systems.  When bulletin board systems answer your call

they usually ask for your name and password.  This name becomes

the default for "Logon Name" for new numbers you add to the

dialing directory.  If Logon Name is filled in MyComm will send

your logon name to systems when you push the <F2> key to save you

some typing.

3.21.4.9  Password for sign on

This option lets you enter the password that you log on with

when you call systems.  When bulletin board systems answer your

call they usually ask for your password.  This password becomes

the default for "Logon Password" for new numbers you add to the

dialing directory.  If Logon Password is filled in MyComm will

send your logon password to systems when you push the <F3> key to

save you some typing.

3.21.4.10 Clever quip for TagLine

Many BBS's have message areas where you can converse with other

people.  If you want to be able to sign your name, leave a witty

quote or whatever without typing it over and over for each

message fill this area in.  When you press the <F4> key MyComm

will send the message you have entered here.  The message can be

up to 40 characters long.  You may also send ANSI "Escape"

sequences (see DOS manual) from the Tagline.

MyComm Version 2.31 Documentation                 Page 42

If you want to send the ASCII "ESCape character (27 decimal, 1B

hex) which usually precedes ANSI escape sequences, MyComm uses

the characters "^[ ". ("caret" sign left bracket).  For example,

the ANSI.SYS driver recognizes the string

<Esc>[5mThis is blinking<Esc>[0m

as a string that blinks. (note: <Esc> represents the 27 decimal

(1B hex) character here).  So, if your Tagline looked like

^[[5mThis is blinking^[[0m

the other system would display "This is blinking" as a blinking

message line.

3.21.4.11 Guess Up/Download Filename

By default MyComm tries to "guess" the filename that you want

to upload or download.  MyComm will make a best guess as to what

you most recently typed that looked like a filename.  Having this

option set to "Yes" can save you time and typing when performing

file transfers.

3.21.4.12 Dialing Screen After Calls

This option instructs MyComm to display the Dialing Directory

rather than the Terminal Window as the default screen when MyComm

is first loaded, and after a call completes.

Please note that in order for this feature to operate correctly

the modem must be set so that Carrier Detect is only active when

you are actually on-line.  See also the Modem Init String

section.

3.21.5  COM Port Addresses

This option enables changes to the COM port addresses.  By

default MyComm uses standard addresses and Interrupt Request

Lines (IRQ) to communicate with your modem.  Most users will

never have a need to use this option.  It is most commonly used

when there is a conflict with a mouse or other serial device and

the modem.  By changing the base address and IRQ line this

problem can often be alleviated.  Modifying the COM port address

table and IRQ settings can be tricky.  If you are not entirely

certain that you know what you are doing this option is best left

alone.

You will see a table with the COM port number, the base address

it is using, and the IRQ line.  You move from field to field by

pressing the <Enter> key.  The base address is displayed in and

should be entered as a hexadecimal number.  The IRQ line will

take numbers between 2 and 7.

MyComm Version 2.31 Documentation                 Page 43

After you make changes you will have to reinstall the COM port.

This is done by going back to the main terminal window and

pressing <Alt> and 'P' and following the "Change Port" menu.

3.21.6  Restore to Defaults

This option will reset the MyComm configuration information to

it's defaults.  A message will appear warning you of this, and a

second "Yes/No" screen will ask if you want to proceed.  The

changes take effect immediately, but are not saved until you use

the "Save Setup" option.

3.21.7  Save Setup

This option saves the changes you made to the various

configuration items to the file MYCOMM.CFG.  If you make changes,

but do not save them, the changes will only remain active until

you exit from MyComm.

3.22 Send Screen to File - Alt W

Pressing the <ALT> and the "W" key takes a "snap shot" of the

currently displayed terminal screen and writes it out to the file

MYCOMM.SCR.  The resulting file will contain the text currently

on the screen.

3.23 Exit MyComm - Alt X

Pressing the <ALT> and the "X" key brings up a message box that

takes a Yes/No response.  Press "Y" to exit MyComm, or "N" or

<Esc> to cancel.

3.24 Help Screen - F1

When you are in the main terminal window pressing the F1 key

will show you a list of MyComm commands that are available to

you.

3.25 Send Name - F2

When bulletin board systems answer your call they usually ask

for your name and password.  This option sends the name that you

log on with when you call a system if your name is filled in for

the dialing directory entry for the system.

3.26 Send Password - F3

This option sends the password that you log on with when you

call a system. See also "Send Name" above.

MyComm Version 2.31 Documentation                 Page 44

3.27 Send Tagline  - F4

This is used to sign your name, or put a short quote, up to 40

characters at the end of a message.  This is set up in the

"Miscellaneous Options Setup" screen.

3.28 Transferring Files

MyComm has a wide variety of built in file transfer protocols

such as  Zmodem, Xmodem, Xmodem-1K, Ymodem Batch, Ymodem-G Batch,

SEAlink, Kermit, and of course ASCII.  There are also slots to

include up to six external file transfer protocols if you would

like to add more.

You may Upload (send) files or Download (receive) files.  Most

host systems will have you start the file transfer at their end

first, and then wait for you to set up on your end.  Host systems

usually wait about a minute or so for you to set up, so don't

panic.

All file transfer protocols on MyComm use 8-N-1 (8 data bits,

No parity, and 1 stop bit).  MyComm automatically sets these

settings for you before the file transfer begins, then switches

back to what ever settings were in place before the transfer

began.

Except for ASCII file transfers, MyComm turns off the XON/XOFF

flow control switch if you have it set on.  This is done to

prevent the XON/XOFF codes as being mistaken as part of the

incoming file.  The original state of the XON/XOFF flow control

is restored at the end of the file transfer.  If your modem has

its own built in XON/XOFF flow control setting, you should turn

it off during file transfers (refer to your modem manual -

usually the &K option).

File transfers differ slightly from one host system to another.

Basically you need to know three things:

�    whether you want to Upload (send) or Download (receive)

a file

�    the name of the file you want to send or receive

�    which file transfer protocol you want to use

Probably you can figure out for yourself whether you are

uploading or downloading from a system, and what the file name is

that you want to transfer.  However you and the host system MUST

use the same file transfer protocol or the transfer will fail.

The tricky part sometimes is deciding which file transfer

protocol to use.  There are so many!

Actually it's easy to decide which file transfer protocol you

want to use.  When you request a file transfer from the host

system, they usually show you a list of file transfer protocols

that their system supports.  Just pick one that both MyComm and

the host system support, and you're on your way!

MyComm Version 2.31 Documentation                 Page 45

The two most frequently used protocols are Xmodem and Zmodem.

A little experimentation on your part will help you decide which

of the file transfer protocols best meets your needs.

A description of each of the file transfer protocols that

MyComm supports will be found later in this section.

3.28.1  Starting a File Transfer

The steps to start the file transfer from your end with MyComm

are simple:

�    Press PgUp to send files, or PgDn to receive files

�    Choose the file transfer protocol you want to use from

the menu

�    Type in the filename (if needed)

�    The file transfer will start and display a status

window

�    When the file transfer completes MyComm makes a noise

MyComm's file transfer menus and screens look basically the

same for both uploads and downloads.  When you press PgUp or PgDn

a menu pops up showing you the different file transfer protocol

options you have (see example below).  Pick a protocol, or press

the <Esc> or "Q" key to cancel.

Download Protocol Menu Example

+------| Down Load |+

| ASCII             |

| SEAlink Derived   |

| Xmodem            |

| 1K-Xmodem         |

| Real Ymodem Batch |

| Ymodem Batch - 1K |

| Zmodem            |

| Kermit            |

external protocol ==>   | 1: BIGFAST        |

(example)               | Quit/None         |

+-------------------+

After you pick a file transfer protocol, MyComm will ask you

for the filename that you want to send or receive.  Some of the

file transfer protocols for receiving files, such as Ymodem Batch

and Zmodem, don't need to ask you the file name so you may not

always be asked for a filename.  MyComm ALWAYS asks for the

filename when you upload files.

Note:

When you upload files with the "batch" protocols you may also

use the DOS "wildcard" characters for filenames such as *.* or

ALL*.ARC, or FILE??.*, etc.

MyComm Version 2.31 Documentation                 Page 46

If the filename, or the path to the file (the directory) are

invalid or can't be found you will see an error message and you

may re-enter the filename.  If you want to exit without

performing the file transfer press the <Esc> key.

When the filename is entered correctly MyComm will start the

file transfer and the File Transfer Status Window shown below

will pop up.

Download/Upload Status Window

+------------------------------------------| Ymodem Download |+

|                                                             |

| Receiving: D:\PUBLIC\SOMEFILE.EXT                           |

| ----------------------------------------------------------- |

|                                                             |

| Xfer Time:     0:04:18        Bytes Received:   14567       |

| Elapsed Time:  0:01:18        Bytes Expected:   58645       |

| Approx. CPS:       186        Blocks Sent:        114       |

| ----------------------------------------------------------- |

|                                                             |

| Error Count: 0                Error Correction: CRC-16      |

| Last Error Message: Receiving File                          |

|                                                             |

+-------------------------------------------------------------+

The status window displays information about the file transfer

such as the file name being sent/received, an estimate of how

long it will take to do the transfer, the elapsed time, the CPS

rate (Characters Per Second), the bytes sent/received, the blocks

sent, error correction type, and error status.

Note:

Some protocols will not tell you the Xfer Time or the Bytes

Expected during downloads.  This is because some protocols do not

send an information "header" packet that tells MyComm how many

bytes to expect from the sender.

When MyComm calculates the amount of time a transfer will take

it does the calculation based on a "worst case" rate of 80%

efficiency. This is to take into consideration possible line

noise, packet resends, and other considerations.  Actual file

transfers may be faster than what MyComm calculates.

The CPS rate reflects only the number of bytes actually

sent/received from/to the file.  Overhead bytes such as CRC and

"escape" characters are not included in the calculation.  This

provides a more realistic picture of the file transfer progress.

3.28.2  File Transfer Hints

�    Both the sender and receiver MUST use the same file

transfer protocol

MyComm Version 2.31 Documentation                 Page 47

�    If a file transfer keeps aborting using one protocol on

a host system, try using a different one.

�    It is not a good idea to upload or download from floppy

disk.  Floppy drives are slow, so the read and write

access time will slow down the transfer.  Floppy drives

do not hold very much data.  If the floppy disk were to

fill up during a download, the transfer would be

aborted.

3.28.3  Download Files - PgDn

Pressing the PgDn (Page Down) key starts the file download

procedures previously discussed.

3.28.4  UpLoad Files - PgUp

Pressing the PgUp (Page Up) key starts the file upload

procedures previously discussed.

3.28.5  ASCII File Transfers

ASCII file transfers are only used to transfer text files.

Program files, also known as binary files, should not be

transferred as ASCII files.  ASCII uploads do not show the File

Transfer Status Window unless you have set the "Show Upload"

option to "No" in the ASCII Xfer configuration menu.  ASCII

transfers have no error correction, but just plough along as

though the characters were actually being typed from the

keyboard.  This is commonly referred to as a "send and pray"

protocol.

During ASCII downloads, the download continues until either a

Ctrl-Z (end of file marker) is received, or no characters have

been received for 10 seconds.  You may also stop the transfer at

any time by pressing the <ESC> key.

3.28.6  SEAlink Derived File Transfers

SEAlink is an advanced version of Xmodem developed by System

Enhancement Associates.  SEAlink uses a "sliding window" protocol

which sends ahead several packets while waiting for an

acknowledgement from the receiver.  This can result in transfers

that are 10 to 20 percent faster than plain Xmodem.  SEAlink

sends the file name and file size when the transfer starts up,

and can be used to send more than one file at a time.

The version of SEAlink that MyComm uses is known as "Derived

SEAlink" as MyComm does not support the "overdrive" option which

is only used in half-duplex links.

MyComm Version 2.31 Documentation                 Page 48

3.28.7  Xmodem File Transfers

Xmodem is probably the most widely used protocol today.  Xmodem

was originally developed by Ward Christensen in the summer of

1977 for use on CP/M machines, and was soon ported to the MS-DOS

environment.  Xmodem sends data in 128 byte blocks.  The block

count seen in the File Transfer Status Window shows the number of

128 byte blocks that have been transferred.  This has

euphemistically been called "Xmodem blocks" and has become

somewhat of a standard in P.C. data communications.

Xmodem has two flavors; Check Sum and CRC-16 mode.  MyComm

supports both Check Sum and CRC-16 mode, and automatically

selects the correct one to use.  The check sum mode is not nearly

as reliable as CRC-16, and isn't used as often.

3.28.8  1K-Xmodem File Transfers

Xmodem-1K is a variation of Xmodem which uses a 1K (1024 byte)

block to send data.  Under most conditions Xmodem-1K is slightly

faster than plain Xmodem.  If there is a lot of line noise

however, the throughput can degrade due to having to perform a

resend on the larger packet.

Some older host systems may refer to Xmodem-1K as "Ymodem" so

read the host system menu carefully.

3.28.9  Ymodem Batch File Transfers

The Ymodem Batch protocol can perform either single or batch

file transfers.  That is, it can send or receive one or more

files in a transfer session.  Ymodem Batch starts sessions by

sending the file name and size to the receiver, then sends the

file data.  Ymodem uses both a 128 byte or 1024 byte (1K) packet

size and MyComm chooses the correct packet size to use during the

transfer.

3.28.10  Ymodem-G Batch File Transfers

The Ymodem-G Batch protocol is similar to the regular Ymodem

Batch protocol. The difference is that Ymodem-G performs no error

correction what-so-ever and as such it is generally only used

with error correcting modems, or Null modem cables.  If an error

is detected during a transfer, the transfer is immediately

aborted.

MyComm Version 2.31 Documentation                 Page 49

3.28.11 Zmodem File Transfers

Zmodem is another widely used file transfer protocol.  It was

developed by Chuck Forsberg/Omen Technology, Inc.  Zmodem is an

extremely robust protocol as it recovers from errors well and

will try to keep the file transfer session going under adverse

conditions.

Zmodem will perform both single file and batch (multiple) file

transfers.  Like Ymodem, when receiving a file using Zmodem, you

don't have to type in the file name on your end.  Zmodem sends

the filename as part of a special "header" that MyComm

recognizes.

Zmodem uses either a 16 bit or 32 bit CRC (Cyclic Redundancy

Check) when transferring the file.  You will see the CRC type

displayed in the File Transfer Status Window.  The CRC is used

for error checking during the transfer.  MyComm automatically

selects the proper CRC type to use so you don't have to worry

about it.  The 16 bit CRC transfers often go a little faster than

32 bit CRC transfers.  However the 32 bit CRC is much more

reliable for error correction, and well worth the few seconds

added to the transfer time.

3.28.12 Kermit File Transfers

The Kermit file transfer protocol is widely used in many

environments in particular mainframes and mini mainframes.

Kermit was designed to allow many different types of machines to

send data to each other.  In order to do this Kermit places a

special "escape" character before any control characters

encountered.  However this compatibility does have its price,

Kermit is one of the slowest protocols in use today.  Because of

the slow file transfers Kermit should be used when there is no

other protocol available.  The Kermit version in MyComm supports

the "basic" Kermit protocol.

3.28.13 External Protocol File Transfers

MyComm lets you define up to six external file transfer

protocols.  The external file transfer protocols will also appear

in the Upload and Download protocol selection menus.

External protocols are set up in the External Protocols Setup

(see section 3.20.3.3 "External Protocols Setup").  External

protocols differ from vendor to vendor, so you should refer to

the particular protocol's documentation for more detailed

installation instructions.  MyComm invokes the external protocols

through the use of DOS "batch" files.  Parameters are passed to

the batch file in the following order: COM Port number (0 to 7),

BPS rate, and Drive:\Path\Filename which translate to batch

parameters %1, %2, and %3.

MyComm Version 2.31 Documentation                 Page 50

3.28.14 MyComm as a Host

At this point in time MyComm does not have a "host" mode per

Se.  This was a design decision in order to keep the code size

small. However, there is a file called MCHOST.SCP which is a

MyComm script that will perform very simple host functions.  The

MCHOST.SCP file may be used as an example to create your own host

scripts as well. Please note that MyComm is not intended for use

as a BBS, but the MCHOST.SCP should provide enough functionality

for simple file transfers.  See also Appendix E - Script

Language.

It is also possible to transfer files between two MyComm

programs (or MyComm and another communication program) without

the use of a special script file.  To do this one person acts as

the "host", and the other as caller.  As the caller you would

just call the other person as you would for any other bulletin

board system.

For this example, let's say you're the host since that's the

complicated part.

Step 1:   Prearrange a time for the other person to call you.

Step 2:   Get MyComm up and running.

Step 3:   When the other person calls you will hear the phone

ringing or see the word "RING" appear on the MyComm

terminal screen.  When you see "RING", type ATA then

press the <Enter> key.  This tells your modem to answer

the phone and initiate the carrier tones so the two

modems will "talk" to each other.  Alternatively you

could press <Alt> and the "I" key, then press "A" for

the "Send Answer String" command.

Step 3a:  Alternative to Step 3: before the other person calls

type ATS0=1 then press <Enter>.  This tells your modem

to answer the modem on the first ring and to initiate

the carrier tones.

Step 4:   When transferring files from one MyComm to another

remember that when they are uploading, you are

downloading and vice versa.  So if the other person

wants to download a file from you they would start the

download function at their end, and you would start an

upload from your end.  The most important things to

decide before doing the transfer are:

- who is uploading/downloading to and from who

- which file transfer protocol you are using (they

MUST be the same)

- and finally, what the filename is.

MyComm Version 2.31 Documentation                 Page 51

Step 4a   This is where the MyComm "chat" mode can be handy.  It

can help you establish stuff while you're on-line with

the other person.  If you would like to "chat" on-line

with the other person, Press <Alt> and the "A" key to

bring up the MyComm split screen chat mode (see section

3.3 "Chat Mode").

Step 5:   If all goes smoothly the file transfer will be

successful.  In fact for the most part it will be as

though you had simply called another BBS, except this

time you're interacting with a live person rather than

the BBS software.

Appendix A - MyComm Command Summary

Alt A - Chat Mode

Alt B - Send Break

Alt C - Clear the Screen

Alt D - Dialing Functions

Alt E - Local Echo On/Off

Alt F - Toggle Line Feed On/Off

Alt H - Hang Up Modem

Alt I - Send Modem Initialization Strings

Alt J - Join Script (Run a Script)

Alt L - Present Line Status

Alt K - Scroll Back

Alt M - Manual Dial

Alt O - Outside to DOS

Alt P - Change Port Parameters

Alt Q - Quick Re- Dial

Alt R - Run External Program

Alt S - Session Capture On/Off/Pause

Alt T - Terminal Emulation

Alt U - Update Configuration

Alt W - Send Screen to File

Alt X - Exit MyComm

F1    - Help Screen

F2    - Send Name

F3    - Send Password

F4    - Send Tagline

PgUp  - Upload Files

PgDn  - Download Files

Appendix B - MyComm Line Editing Functions

Many of the user configuration options and the dialing list

have sections that require a line of text to be input and edited.

MyComm has a built in line editor to perform simple editing on

these text lines.  The MyComm line editor uses a tiny subset of

WordStar type commands.  A description of the line edit commands

follows.

� Enter       -     Terminates editing the line and keeps it

� Esc         -     Terminates editing the line and does not keep

it

� Right Arrow -     Moves the cursor right.

� Left Arrow  -     Moves the cursor left.  This is "non-

destructive", the character is not erased as

the cursor moves

� BackSpace   -     Moves the cursor left.  This is

"destructive", the character is erased as the

cursor moves

� End         -     Positions the cursor at the end of the line

� Home        -     Positions the cursor at the beginning of the

line

� Ctrl + T    -     When the Ctrl key and the letter T are

pressed simultaneously, the line is erased

from the current cursor position to the end

of the line

� Ctrl + Y    -     When the Ctrl key and the letter Y are

pressed simultaneously, the entire line is

erased and the cursor is positioned at column

one

Appendix C - Terminal Emulation

This is a list of the functions that MyComm supports in the

terminal emulators.

1.0  TTY Terminal Emulation

TTY does not provide any character translation what-so-ever.

The characters are simply displayed as they come in.

2.0  ANSI Terminal Emulation

ANSI provides mapping similar to that found in your ANSI.SYS

driver.  The color and screen attributes as described in the

"MS-DOS 5.0 User's Guide and Reference" are supported, except for

the keyboard mapping functions.  The following keys also generate

codes recognized by other systems operating in ANSI mode:

Direction (Arrow) Keys  - move cursor

Delete                  - sends DEL (127 decimal)

End                     - erase to end of line

Home                    - homes cursor

Ctrl-Home               - clears screen, and homes cursor

Function Keys           - do not generate a character

3.0  VT-52 and VT-100 Terminal Emulation

The VT-52 and VT-100 emulations are typically used with

mainframe environments.  These emulations provide a subset of the

"application keyboard" functions defined in the "VT-200

Programmer Guide", Digital #EK-VT220-HR-001.  These emulations

will work in either a 7 or 8 bit environment.  A standard key pad

for these terminals looks like this:

+-----------------------+

| PF1 | PF2 | PF3 | PF4 |

|-----+-----+-----+-----|

|  7  |  8  |  9  |  -  |

|-----+-----+-----+-----|

|  4  |  5  |  6  |  ,  |

|-----+-----+-----+-----|

|  1  |  2  |  3  |  R  |

|-----------+-----|  T  |

|     0     |  .  |  N  |

+-----------------------+

MyComm defines these keys as follows:

PF1 through PF4     - <SHIFT> + F1 through F4

1 through 0         - <ALT> + F1 through F10

Minus Sign "-"      - <SHIFT> + F5

Comma ","           - <SHIFT> + F6

Decimal Point "."   - <SHIFT> + F7

RTN (Enter key)     - <SHIFT> + F8

NOTE: the above keys are the function keys on a standard PC

keyboard.  The PC numeric keypad has not been mapped to these

keys.

Also defined:

Direction (Arrow) Keys  - move cursor

Delete                  - sends DEL (127 decimal)

End                     - erase to end of line

Home                    - homes cursor

Ctrl-Home               - clears screen, and homes cursor

4.0  3270 Terminal Emulation (Registered version only)

This emulation is most commonly used when connecting to IBM

mainframe hosts.  This emulation should be used instead of VT-100

emulation when connecting through a 7171 protocol converter.

This emulation works best with 101 key PC keyboards (to get the

PF11, PF12, PF23, and PF24 keys).  The mapping to the PC keyboard

follows.

PC Keypad         Alternate            3270 emulation

<Shift> + F1                           PF1

...                                 ...

...                                 ...

<Shift> + F10                          PF10

<Shift> + F11    <Esc> + -             PF11

<Shift> + F12    <Esc> + =             PF12

<Alt> + F1                             PF13

...                                 ...

...                                 ...

<Alt> + F10                            PF22

<Alt> + F11      <Esc> + _             PF23

<Alt> + F12      <Esc> + +             PF24

<Ctrl> + F1                            PA1

...                                 ...

...                                 ...

<Ctrl> + F4                            PA4

Arrow Keys                         Arrow Keys

Insert                             Toggle Insert

<Shift> + <Tab>                    Back Tab

<Ctrl> + <Left Arrow>              BackSpace

Home                               Home

Delete                             DEL

<Ctrl> + Home                      CLEAR

End                                Erase to End of field

<Ctrl> + End                       Line Feed

Appendix D - Troubleshooting

This section should answer common problems that you might have

when using MyComm.

Problem:  - Modem doesn't respond at all, responds

intermittently, or behaves strangely.

Possible Causes:

-  Is the COM port the right one?  i.e COM 1, 2, 3, 4, 5, 6, 7,

or 8.

-  Is the modem installed correctly?

-  If this is an external modem, are the cables hooked up? Is

it plugged in and switched on?

-  Is the modem really Hayes (Tm) compatible?

-  Is the initialization string correct for the modem?  The

manual that came with your modem will list the "AT" commands

that are valid.

-  Is the baud rate entered in the "Modem Setup" screen less

than or equal to the top speed of the modem?  For example, a

2400 bps modem will not initialize correctly if the baud

rate is set to 19200.

-  Is there another serial device such as a mouse, multi-I/O

card, or a sound card that is sharing the same IRQ line as

the modem?  This is a very common problem. So common in fact

that several paragraphs are devoted to it.

COM1 and COM3 both share IRQ4, and COM2 and COM4 both

share IRQ3. You can not hook up a serial device such as a

mouse to COM1 and the modem to COM3 and expect them to work

correctly (the same is true for COM2 and COM4). Often a

multi-I/O cards will come with COM2 enabled which will

conflict with your modem if you attempt to use the modem on

either COM2 or COM4.

The solution is to set the offending card or the modem to

a different IRQ line than one that is currently in use.

This is generally performed by re-configuring the jumper

settings on the card or modem.  The store where you bought

your card from or the manual(s) that came with the card or

modem will describe how to do this.  Many newer modems can

be re-jumpered for IRQ2 or IRQ5. If you change the IRQ line

settings on the modem be sure to change the configuration in

MyComm as well (see below and the section titled "COM Port

Addresses").

-  MyComm uses default address and interrupt vectors for COM

Ports of:

COM Port   Address  IRQ Line

1        3F8        4

2        2F8        3

3        3E8        4

4        2F8        3

5        3E8        2

6        2F8        2

7        3E8        5

8        2F8        5

If you have set these to something different, MyComm

probably won't like it.  Reset the interrupt vectors to their

standard settings.  If you don't know what this means, you

probably didn't fool with it in the first place, so don't mess

with it now.

MyComm allows user configuration of addresses or interrupt

vectors on the COM ports.  See the section "COM Port Addresses"

for more information.

Problem:  - Modem won't hang up the phone line.

Possible Causes:

-  Some modems have a DIP switch setting that keeps Carrier

Detect always on.  MyComm uses the Carrier Detect signal

(CD) to see if it's on-line, and to see when the modem has

released the phone line.  You should set your modem to let

software, in this case MyComm, to handle the Carrier Detect

signal.

-  Try adding the Hayes command "&C1" (without the quotes) to

your initialization string.  This tells the modem to raise

Carrier Detect only when connected.

Problem:  - Modem dials, but doesn't know when the connection has

been made.

Probable Causes:

-  Make certain that the modem is sending text responses rather

than numeric.  Add "Q0" and "V1" to your modem

initialization string.

-  Go into the "Modem Setup" screen and check the "Connect

Strings" setting.  Make sure that they look like the ones

your modem usually generates.

Problem:  - The result codes from the modem are numbers instead

of text.

Possible Causes:

-  Try adding "Q0" and "V1" to the modem initialization string.

MyComm MUST see the result codes, and they have to be the

English Text.  Also try adding "X4" to the initialization

string.

Problem:  - When using a multitasker such as Windows (Tm),

DoubleDOS (Tm), etc, the MyComm displays bleed

through to other applications.

Possible Causes:

-  Set the option "Direct Screen Writes" in the "Miscellaneous

Options" screen to "No".

Problem:  - When I call some services, the characters look weird,

or just garbage.

Possible Causes:

-  Make sure that the Baud Rate, Data Bits, etc. are set

correctly. Most public BBS's use 8-N-1, but some may use 7-

E-1 (often true with Unix (Tm) based systems and

mainframes).

-  Are you using the correct terminal emulation? See <ALT> +T

command.

-  This is sometimes caused by a COM port or IRQ line conflict.

If another serial card or device is on the same COM port or

IRQ line you may experience dropped characters, weird things

happening during file transfers and other problems. See also

question # 1 above.

Problem:  - File transfers seem to always time out or abort.

Possible Causes:

-  MyComm will abort a transfer if there are too many errors in

the transfer due to line noise, or other unpredictable

circumstances.

-  Are you and the other guy are using the same file transfer

protocol?

Problem:  - On some host systems the Word wrap or the linefeeds

look odd.

Possible Causes:

-  Is the "LF After CR" set? It is possible that the other

system is sending Line Feeds after Carriage Return and you

are getting double linefeed characters.  You can also turn

the Line Feeds off (or on) by pressing Alt + F in the main

terminal window.

-  MyComm uses a screen width of 80 columns by 25 lines. As

incoming text is displayed MyComm will scroll to the next

line when either a linefeed character is received or when

the cursor reaches column 80. Some host systems will send

characters all the way out to column 79 with column 80 being

a linefeed. In this case the text will appear to "wrap" too

soon and you may see the line scroll down twice. In this

case tell the host system through it's "user setup" that you

want a screen width of only 79 characters to prevent the

premature wrapping.

Question: - Why isn't a 14400 or 28800 setting in the baud rate

list?

Answer:

-  A portion of the modem hardware called the "baud rate

divisor" determines the "baud" setting for the modem.  The

baud rate divisor requires a number which will divide evenly

into the clock speed which all of the baud settings in the

baud rate list do.  For 14400 modems the best setting is

19200, for 28800 modems one of the higher speeds is

recommended.

Appendix E - MyComm Script Language

1.0  Script Language Introduction

This Appendix describes the format and syntax of the script

language used by MyComm.  The MyComm script language is a series

of commands in a text file that is read by MyComm to perform

certain functions.  For example, a script function could send

your name and password automatically when you log onto a system,

then perform steps to navigate through the host system menus to

gather your mail for you and then log off.

The script language used in MyComm consists of 22 commands that

are used to build scripts.  The script language is very simple,

yet very powerful.  It is designed in such a way that ordinary

people rather than programmers or technicians will find it easy

to use and understand.  MyComm's script language should be no

more difficult to use than writing DOS batch files.

1.2  Creating a Script

Scripts are created using a line editor, text editor, or word

processing software that can save files as plain unformatted

text.  It is similar to creating DOS "batch" files.

To Create a Script

Step 1:   Log onto the BBS or system that you want to create a

script for.  As you log on write down the sequence of

questions and responses that you receive and send.  For

example if the host system has a prompt that says

"Enter your Name:" and you type in your name at that

point, write down the question asked by the host system

and your response.  Optionally you may turn on the

Session Capture (ALT + "S") to save your responses for

later referral.

Step 2:   Log off the BBS or system. No sense in tying up their

line.

Step 3:   Use your text editor to write the script file.  The

script commands that MyComm understands are described

later in this Appendix.  The script file must be saved

to the same directory that the MyComm program is in

(for example C:\MYCOMM) so that MyComm can find it

later.

Step 4:   Test your script with the MyComm Script Checker,

SCP_CHK.EXE (described later in this Appendix).  Revise

the script and re-test as necessary.

Step 5:   Go to the Dialing Directory and edit the dialing entry

for this BBS.  There is an option called "Scripts".

Add the name of your script file here.

Step 6:   Call the BBS and test your script.  Sometimes it may

take more than one try to get it all right.  Don't

worry, it happens to us all.

2.0  Script Language

The MyComm script language consists of 22 commands which will

be described shortly.  The script files that you create must have

the file name extension .SCP, and must be saved in the same

directory that MyComm is in.  For example if you created a script

for MyBbs, the script file would be named MYBBS.SCP.

This manual will use the following conventions to describe the

functions and the parameters that they take:

FUNCNAME()     Function name is all CAPS. Please note that

no function name is over eight characters

long. All functions have the parameters

enclosed with parenthesis.  The parameters

are separated with commas.

<int>          Integer parameter.

<"string">     String parameter.  Please note that all

incoming and outgoing strings may be a

maximum of 60 characters in length.  Strings

are case sensitive, "PassWord" and "password"

are different strings!  Also note that the

strings are enclosed in quotes.  Since commas

are used as parameter delimiters, strings

must not contain embedded commas!

<op1|op2|...>  Operator parameter.  Some functions use an

operator.

<"label">      Label name.

<Sn>           Parameter may be one of the string flags S0

through S9.

<In>           Parameter may be one of the integer flags I0

through I9.

(optional)     Indicates the next parameter is optional

<"string"|Sn>  Parameter may either be a string constant or

one of the string flags.

:label         Label within the script file.  All labels are

preceded by a colon ':' character.  Labels

may be up to eight (8) characters.

comments       Comments are preceded by a semi-colon ';'.

The comments are free form and may be placed

anywhere.  The rest of the line after the

semi-colon is ignored by the script.

The script functions are described in the following sections.

The format of the descriptions is the function name, the

parameter it takes, and a description. For example:

WAITFOR(int waitsecs, "string"|Sn, (optional) "label"|Sn)

indicates that the WAITFOR function takes an integer parameter, a

string or "S" flag parameter, and an optional string or "S" flag

parameter.

2.1  Special Script Characters

There are several special characters that MyComm will interpret

for you when they are put into strings that you are sending to

the host system:

^M ("caret" sign and capital "M")       This character sequence

sends a "carriage

return", as though you

had pressed <Enter> on

the keyboard.

^[ ("caret" sign and left bracket "[")  This character sequence

sends <Esc> as though you

had pressed <Esc> on the

keyboard.

~ (tilde) -                             This character causes a

100 millisecond delay.

2.2  Script Functions

The next section describes the various script functions that

are available.

2.2.1  Labels

Labels are used to produce logical execution branches within

the script file.  Labels are used with the GOTO(<label>) function

as well as several other functions which will branch when a

condition is met.

2.2.2  WAITFOR()

format: WAITFOR(int waitsecs, "string"|Sn, (optional) "label"|Sn)

WAITFOR() will wait up to <int> seconds for <string> and goto

<label> if the <string> is received.  If the <string> is not

received the next line in the script is executed.  Note that

<label> is optional in which case the next line in the script

would be executed regardless.

For example if you wanted to wait 30 seconds for the string

"PassWord" then jump to the label :GotPass

WAITFOR(30, "PassWord", "GotPass")

would do it.

2.2.3  XMIT()

format: XMIT("string"|Sn)

Transmit a "string".  For example:

XMIT("password")

would send the string "password".

If the LOCLECHO() function has been set to "Yes" the characters

sent by XMIT() will also be shown on your local screen.  This may

be useful when the other system is not echoing characters back to

you.

2.2.4  GOTO()

format: GOTO("label"|Sn)

Jump to :label.  For example, if you have the label ":Lab001" in

your script, GOTO("Lab001") would jump to that label.  Script

execution resumes at the line after the label.

2.2.5  SETFLAG()

format: SETFLAG(In, int value)

Set integer flag to a value.  The flags are referenced as flag 0

through 9.  The flags can take on values between -999 and 9999.

This function has the following format:

For example to set flag 1 to 42:

SETFLAG(I1, 42)

2.2.6  INCFLAG()

format: INCFLAG(In)

The flags that were set (see SETFLAG() above) may be

incremented by 1.  To increment flag 1:

INCFLAG(I1)

2.2.7  DECFLAG()

format: DECFLAG(In)

The flags that were set (see SETFLAG() above) may be

decremented by 1.  To decrement flag 1:

DECFLAG(I1)

2.2.8  IFFLAG()

format: IFFLAG(In, <EQ|GT|LT|GE|LE|NE>, int value, "label"|Sn)

The integer flags may be tested for logical conditions.  If the

condition is met script execution continues at the statement

following the label.  The test condition operators are:

EQ - equal

GT - greater than

LT - less than

GE - greater than or equal

LE - less than or equal

NE - not equal

If you wanted to test to see if flag 1 was EQual to 42, then jump

to label :Lab001 you would enter:

IFFLAG(I1, EQ, 42, "Lab001")

NOTE: Quote characters are not used around the operators.

2.2.9  IFSCAN()

format: IFSCAN(Sn, "string"|Sn, "label"|Sn)

You may set up to ten scan strings to continuously scan for.

When the scan condition is met script execution continues at the

statement following the label.  For example if you wanted to scan

for the string "John Doe" and then jump to the label :Lab001 when

it was found:

IFSCAN(S1, "John Doe", "Lab001")

IFSCAN() recognizes S0 through S9 as valid flag names.  The

IFSCAN() function can be reset to scan for a new string by re-

using the string variable. For example:

IFSCAN(S1, "Bob Public", "Lab001")

2.2.10 SENDNAME()

format: SENDNAME()

The SENDNAME() function will send the name that you have

defined in your dialing directory entry for the BBS that the

script is "Joined" to.  SENDNAME() take no parameters.  If the

name entry in the dialing directory is blank nothing is sent.

2.2.11 SENDPASS()

format: SENDPASS()

The SENDPASS() function will send the password that you have

defined in the dialing directory entry for the BBS that the

script is "Joined" to.  SENDPASS() take no parameters.  If the

password entry in the dialing directory is blank nothing is sent.

2.2.12 SENDFILE()

format: SENDFILE(<op> A|S|X|K|Y|G|Z, "filename"|Sn,

optional "path"|Sn)

The SENDFILE() function is used to upload a file to the host

system.  You will have to take care of any preparatory steps

within the script, such as telling the host that you are sending

a file.

The parameters are a one letter designator for the protocol,

and a "string" or string flag (S0..S9) for the filename.  By

default MyComm looks for the file that you are sending in the

"Upload Directory" that you have defined in MyComm.  You may

specify a different directory path as the third parameter.

The valid operators for <op> are:

A - ASCII

S - SEAlink

X - Xmodem

K - Xmodem-1K

Y - Ymodem Batch

G - Ymodem-G

R - Kermit

Z - Zmodem

For example if you wanted to send the file "MYCOMM.ARC" using

Zmodem:

SENDFILE(Z, "MyComm.ARC")

- or -

SENDFILE(Z, "MyComm.ARC", "MYPATH")

2.2.13 RECVFILE()

format: RECVFILE(<op> A|S|X|K|Y|G|Z, optional "filename"|Sn,

optional "path"|Sn)

RECVFILE() allows you to receive files from the host system.

You will have to take care of any preparatory steps within the

script, such as telling the host that you want to receive a file.

See SENDFILE() (above) for the list of valid operators.  By

default the received files are put into the "Download" directory

defined in MyComm, however files may be put into a different

directory by using the third optional parameter.

For example if you wanted to download the file "STUFF.ARC" using

Xmodem:

RECVFILE(X, STUFF.ARC)

- or -

RECVFILE(X, "MyComm.ARC", "MYPATH")

Please note that SEAlink, Ymodem, Ymodem-G, and Zmodem do not

require that a filename be specified.  These protocols send the

filename in a special header.  The ASCII, Xmodem, and Xmodem-1K

protocols always require that a filename is specified.

NOTE: if you have the Zmodem "Auto Download" feature turned on

there may be conflicts between the script Zmodem and the MyComm

Zmodem.  If you have the Zmodem "Auto Download" feature turned on

in MyComm the Zmodem file transfer will start automatically, and

you will not need to use this function during Zmodem downloads.

2.2.14 SNDBREAK()

format: SNDBREAK(optional <int ms>)

SNDBREAK() sends a "Break" signal for 1500 milliseconds.  It

will also take an optional parameter for the number of

milliseconds that the "Break" signal will be sent.

2.2.15 ALARM()

format: ALARM(optional <int ms>)

ALARM() will make a noise for <int> milliseconds.  For example

ALARM(1000) would sound a beep for one second.  The default alarm

duration is 500 milliseconds (1/2 second).

2.2.16 PAUSE()

format: PAUSE(int seconds)

PAUSE() will cause a delay for "seconds" seconds.  For example

PAUSE(5) would pause the script for 5 seconds.

2.2.17 HANGUP()

format: HANGUP()

The HANGUP() function causes the modem to disconnect.  HANGUP()

uses the information from the MyComm configuration file to send

the correct hang up string.  This function sets the SUCCESS flag

which can be tested with the IF() function.

2.2.18 IGNORECD()

format: IGNORECD("Yes"|"No"|Sn)

This function "Ignore Carrier Detect" tells the script

processor to ignore the state of the "Carrier Detect" modem

signal.  By default if the carrier signal is lost the script

exits.  In some cases, such as while waiting for an incoming

call, it is useful for the script to continue execution even when

there is no carrier signal.

This function takes one parameter: a string of "Yes" or "No",

or one the string variables (S0..S9).  A value of "Yes" turns off

carrier detect monitoring within the script, "No" turns

monitoring back on.

2.2.19 SETSTR()

format: SETSTR(Sn, "string"|Sn)

SETSTR() sets one of the string variables S0 through S9 to a

string constant or another string variable.  For example:

SETSTR(S2, "This String")

would set string variable S2 to "This String".

Please note that the SETSTR() function resets the "goto" label

of the string variable if that same string variable was

previously assigned with the IFSCAN() function.

2.2.20 GOSUB()

format: GOSUB("label"|Sn)

The GOSUB() function is used to branch execution in the script.

It is similar to the GOTO() function except that later in the

script a RETURN() function may be used which will transfer script

execution to the statement following the last GOSUB().  For

example:

GOSUB("gosub1")     ; go to label "gosub1"

<statement1>        ; execution resumes here after RETURN()

<statement2>

...

<statement-n>

:gosub1             ; label for GOSUB() to go to

<statement1>

<statement2>

...

<statement-n>

RETURN()            ; return to statement after GOSUB()

2.2.21 RETURN()

format: RETURN()

The RETURN() function returns script execution to the statement

following the most recent GOSUB().  See the description of the

GOSUB() function above for details.

2.2.22 EXIT()

format: EXIT()

The EXIT() function exits the script and return control back to

MyComm.

2.2.23 QUIT()

format: QUIT()

The QUIT() function exits from the script and MyComm

completely.  Please note: QUIT() does not hang up the modem. If

you would like the modem to disconnect the HANGUP() function

should be used before QUIT().

2.2.24 COMPSTR()

format: COMPSTR("string"|Sn, "string"|Sn, "label"|Sn)

The COMPSTR() function compares two strings or string variables

and jumps to a label if the strings are equal.  The comparison is

not case sensitive.

2.2.25 LOCLECHO()

format: LOCLECHO("Y"|"N")

This function turns the local echo on during script execution.

When the local echo is on anything sent with the XMIT() function

is echoed back to the local console.  Additionally the PORTGET()

function (see below) echoes characters back to the other system.

2.2.26 IF()

format: IF("TRANSTAT"|"CARDET"|"NOCARDET"|"SUCCESS", "label"|Sn)

The IF() function tests one of the MyComm internal script flags

for a TRUE condition and then jumps to a label if TRUE.  The

internal script flags are:

TRANSTAT - True if last file transfer was successful

CARDET   - True if carrier detected

NOCARDET - True if no carrier detected

SUCCESS  - Set by some script functions

2.2.27 PORTGET()

PORTGET()

format: PORTGET(Sn, len, waittime)

The PORTGET() function waits up to "waittime" seconds to get up

to "len" characters from the COM port and put them into a string

variable.  If the LOCLECHO() function has been activated the

characters are echoed back to the other system.  This would be

useful for getting responses from someone who is calling you for

example.  Note: only printable ASCII characters in the range of

20 hex to 7E hex (Space char through tilde) will be placed into

the string variable.  Also note that receiving a "carriage

return" character (0D hex) will cause PORTGET() to exit with

whatever characters were received up to that point.

2.2.28 DIAL() (Registered version only)

format: DIAL("number to dial", optional <tries>

The DIAL() function will dial a telephone number.  The optional

parameter "tries" allows you to set a finite number of dialing

attempts, otherwise the number is dialed continuously until a

connection is made.

For example:

DIAL("555-1212", 10)

would dial the number 555-1212 until either a connection was made

or there were 10 unsuccessful dialing attempts.

This function sets the SUCCESS flag which can be tested with

the IF() function.

2.2.29 CHDIR() (Registered version only)

format: CHDIR("string"|Sn)

This function changes the directory that you are currently in

to another one if the new directory is valid.  This function sets

the SUCCESS flag which can be tested with the IF() function.

2.2.30 RUNDOS() (Registered version only)

format: RUNDOS("string"|Sn)

This function runs a DOS command.  If the command is invalid,

or there is not enough memory to run the command, execution

returns immediately to MyComm.

2.2.31 FILEHERE() (Registered version only)

format: FILEHERE("filename"|Sn)

This function checks to see if the named path\file exists.  The

MyComm internal variable SUCCESS is set to True if the file

exists. The state of SUCCESS may be later tested with the IF()

script function.

2.2.32 JUMPSCP() (Registered version only)

format:  JUMPSCP("string"|Sn)

The JUMPSCP() function closes execution of the current script

and starts execution in a new script.  The current state of the

string and flag variables is preserved.  Note: this is a one way

ticket into the new script.  Execution is not returned to the

calling script.

2.2.33 CONSGET() (Registered version only)

format: CONSGET(Sn, optional <int len>)

CONSMGET(Sn,<int len>)

The CONSGET() and CONSMGET() functions place input from your

local keyboard up to "len" characters into a string variable.

The CONSMGET() "masks" the input (for passwords for example).

2.2.34 CONSLOC() (Registered version only)

format: CONSLOC(<int row>, <int col>)

The CONSLOC() function moves the cursor on the local console

(your screen) to a new location. The valid range for screen

locations is:

row - 1 to 25

col - 1 to 80

Note: CONSLOC(0,0) clears the screen and homes the cursor.

2.2.35 CONSCLR() (Registered version only)

format: CONSCLR(<int textcolor>, <int backgroundcolor>)

The CONSCLR() function allows you to change the screen colors

on the local console (your screen). The valid colors are:

Foreground     Background

BLACK           - 0       Y              Y

BLUE            - 1       Y              Y

GREEN           - 2       Y              Y

CYAN            - 3       Y              Y

RED             - 4       Y              Y

MAGENTA         - 5       Y              Y

BROWN           - 6       Y              Y

LIGHTGRAY       - 7       Y              Y

DARKGRAY        - 8       Y              Y

LIGHTBLUE       - 9       Y              N

LIGHTGREEN      - 10      Y              N

LIGHTCYAN       - 11      Y              N

LIGHTRED        - 12      Y              N

LIGHTMAGENTA    - 13      Y              N

YELLOW          - 14      Y              N

WHITE           - 15      Y              N

2.2.36 CONSMSG() (Registered version only)

format: CONSMSG("string"|Sn)

The CONSMSG() function displays a message on the local console.

2.2.37 KBDFLSH() (Registered version only)

format: KBDFLSH()

The KBDFLSH() function clears the keyboard buffer.

2.2.38 PORTFLSH() (Registered version only)

format: PORTFLSH()

The PORTFLSH() function clears the COM port buffers.

2.2.39 SESCAP() (Registered version only)

format: SESCAP(<O|C|P|R>, optional <"filename"|Sn>)

The SESCAP() function operates on the Session Capture file.  By

default the session capture file from the MyComm configuration is

used as the filename.  The optional parameter allows you to use a

different filename.  The following functions are available:

O    - Open

C    - Close

P    - Pause

R    - Resume

This function sets the SUCCESS flag which can be tested with

the IF() function.  Please note that it will cause an error to

attempt to open a Session Capture file that is already open, so

care should be taken with your script logic.

See also the Session Capture section in this manual.

3.0  Script Syntax Checker

The program SCP_CHK.EXE is used for checking the syntax of

script files.  To use the program to check a script file type:

SCP_CHK SCRIPT.SCP

Replace SCRIPT.SCP with your file name.  Also the .SCP extension

is assumed and you may leave it off if you like.  The output from

SCP_CHK will be display on the screen, or you may redirect the

output to a file.  For example

SCP_CHK MYSCRIPT > MYSCRIPT.OUT

would create the file MYSCRIPT.OUT with all the error

information.  This might prove useful if you have a long error

list.

The output from SCP_CHK will look something like this:

Line 3  ALARM  - Error  : too few arguments

This shows you the line number where the error occurred, the

function that caused the error, and the reason why it was an

error.  The error messages are self explanatory.  Simply re-edit

the script file and correct any errors.  Run SCP_CHK again to

make sure that you caught all the errors.

4.0  Script Sample

This is a sample of a working script file that picks up mail at

a local BBS.

waitfor(180, "=no change?")              ;get past opening screen

xmit("^M")

waitfor(30, "Do you want graphics")

xmit("Y^M")

waitfor(30, "your first name?")          ;login

sendname()

waitfor(30, "Is this correct?")

xmit("Y^M")

waitfor(30, "Password")

sendpass()

waitfor(180, "Scan Message Base")        ;get past local msgs

xmit("N^M")

waitfor(15,"Press (Enter) to continue?", jump1)

goto("nojump1")

:jump1

xmit("^M")

:nojump1

;at main menu now...

waitfor(60, "Command?")

xmit("open^M")                           ;get to door section

waitfor(30, "Enter the DOOR # to Open")  ;open mail door

xmit("3^M")

waitfor(60, "mail Command?")

xmit("D^M")                              ;start mail gathering

waitfor(600, "receive these messages")

xmit("G^M")                              ;start mail d/l

;zmodem will auto start here

;download complete

waitfor(60, "Thank you for visiting")    ;now logoff

hangup()

exit()                                   ;exit the script

Appendix F - MyComm Order Form

MyComm Version 2.31  Order Form

Copyright (c) 1992 - 1995 - Ken Dorshimer

All Rights Reserved

Mail to:

Ken Dorshimer

P.O. Box 6881

San Mateo, Ca. 94403

Your Name: _________________________________________________

Address:   _________________________________________________

_________________________________________________

City:      _________________________________________________

State:    _________      Zip Code: _________________________

Disk Format: 5.25" floppy ____   OR  3.5" semi-floppy ____

Number of Copies of MyComm  _______  X $25.00 =   __________

Number of Copies of MyComm  _______  X $2 S&H = + __________

(NOTE: S&H is $3 outside of the United States)

California residents please add 8.25% Sales Tax + __________

(CA Sales Tax on $27.00 is $2.23)

Total:      __________

Please make check or Money Order payable to Ken Dorshimer, and

send to the address above (mine not yours).

Funds should be in U.S dollars, and drawn on a U.S. bank.

(Please send check or money order, do not send cash.  People

who want to order MyComm from outside the U.S. should obtain a

money order in U.S. dollars from a local bank).

Please note: I am not able to process credit card orders at

this time (several folks have asked me about this).

Appendix G - MyComm Bug Report Form

MyComm Version 2.31  Bug Report Form

This form may be used to describe any problems, or bugs that

you may have found with MyComm.  You may also use it to describe

any enhancements you would like to see added to MyComm, or

suggestions for future versions.

Bug reports and suggestions are always welcome.  Of course all

bug reports and suggestions get much closer scrutiny when

accompanied by a registration check <grin>.

----------------------------------------------------------

Machine Type __________________________     DOS Version ____

Machine BIOS Type _____________________     Monitor ________

Modem Brand __________________________      Speed     ______

Modem Initialization String _______________________

Describe problem, bug, or suggestion:

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

Mail To:

Ken Dorshimer

P.O. Box 6881

San Mateo, Ca. 94403

Appendix H - Control Characters

MyComm supports the sending of standard data communications

control characters.  Control characters are preceded by the caret

character '^'.  The caret denotes that the next character will be

a control character.  For example ^M is the symbol for Carriage

Return or <Enter> which could be used to terminate your "Modem

Initialization String".  The '^' stands for Ctrl, so sending one

of these sequences would be similar to pressing the <Ctrl> key

and the character 'M' simultaneously.  The control characters are

especially useful when using the script language functions. For

example ^[  represents the ESCape character which is often used

to start a session on a BBS.

Please note that these control sequences are only interpreted by

MyComm when you are sending a string such as your "Modem

Initialization String", "Hangup String", etc. or with the XMIT()

function in the script language.  A listing of commonly used

control characters follows.

Sequence       Meaning

^@             NUL

^A             SOH

^B             STX

^C             ETX

^D             EOT

^E             ENQuire

^F             ACKnowledge

^G             BELL

^H             BackSpace

^I             TAB

^J             Line Feed

^L             Form Feed

^M             Carriage Return (Enter key)

^P             DLE

^Q             XON

^S             XOFF

^U             NAK

^V             SYN

^X             CANcel (often used for aborting file transfers)

^Z             SUB (also end of file)

^[             ESCape

Appendix I - MyComm Strip ANSI (Registered version only)

The MCSTPANS.EXE program is for stripping ANSI sequences out of

text files which were created using the "Session Capture"

function.  During the normal operation of Session Capture all

characters that are received while Session Capture is active are

written to the Session Capture file.  Some of these characters

may be ANSI escape sequences which can make the text file hard to

read later.  The MCSTPANS.EXE program removes the ANSI sequences.

The usage of MCSTPANS is simple:

MCSTPANS <input file> (optional) > <output file>

For example if you wanted to strip the ANSI sequences from

MYCOMM.SES:

MCSTPANS MYCOMM.SES

would read MYCOMM.SES and display the resulting output to the

screen.  MCSTPANS may also be used with DOS "pipes" and

"redirection".  For example:

MCSTPANS MYCOMM.SES | MORE

would read in MYCOMM.SES and "pipe" it to the DOS "MORE" command

and display it to the screen.  Or:

MCSTPANS MYCOMM.SES > SESSION.TXT

would read in MYCOMM.SES and "redirect" it to the file

SESSION.TXT which could be viewed later with a text editor.

MCSTPANS is particularly useful for stripping out ANSI screen

color sequences.  Please note that MCSTPANS will not convert ANSI

drawings or a file with a lot of cursor movement "escape"

sequences into anything that is humanly readable due to the usage

of the cursor movement "escape" sequences that originally placed

characters to a specific screen location.

Index

-D (Dial Command Line Parameter)  9

-J (Join Script Command Line Parameter)  9

-T (Timed Start Command Line Parameter)  10

1K-Xmodem File Transfers  46

3270 Terminal Emulation  54

Add LF after incoming CR  17

Add Phone List Item  13

ALARM()  65

Alt A  12

Alt B  12

Alt C  13

Alt D  13

Alt E  21

Alt F  21

Alt H  21

Alt I  21

Alt K  22

Alt L  22

Alt M  23

Alt O  23

Alt P  23

Alt Q  24

Alt R  24

Alt S  24

Alt T  25

Alt U  25

Alt W  41

Alt X  41

ANSI Terminal Emulation  52

ASCII File Transfers  45

ASCII Transfers Setup  32

ASCII Transfers Setup Screen  32

Baud Rate  14

BS Key Sends BS or DELete  17

Call Numbers (Dial)  18

Caret Sign  76

Change Port Parameters - Alt P  23

Change Port Parameters Screen  24

Character Pace Time  34

Chat Mode - Alt A  12

CHDIR()  69

Clear last date called  17

Clear the Screen - Alt C  13

Clever quip for TagLine  39

COM Port  26

COM Port Addresses  40

Command Line Parameters  9

Command Summary  50

COMPSTR()  67

Connect Strings  30

CONSCLR()  70

CONSGET()  69

CONSLOC()  70

CONSMGET()  69

CONSMSG()  70

Control Characters  76

CR Translation Upload and Download  33

Data Bits  14

Data Parity  15

Date Format Style  38

DECFLAG()  63

Default Protocol  15

Definitions, Acronyms  3

Delete an Item  18

Dial Entries  9

Dial Prefix  15

Dial Screen  13

DIAL()  68

Dialing Functions - Alt D  13

Dialing Prefix #1  29

Dialing Prefix #2  29

Dialing Prefix #3  29

Dialing Prefixes  29

Dialing Screen After Calls  40

Dialing Status Window  18

Dialing Wait Time  28

Disclaimer and Warranty  3

Download Directory Path  31

Download Files - PgDn  45

Download Protocol Menu  43

Download/Upload Status Window  44

Edit Phone List Item Screen  13, 14

Editor Path\Filename  31

Exit MyComm - Alt X  41

EXIT()  67

Expand Blank Lines to Spaces  33

External Programs Path\Filename  31

External Protocol File Transfers  47

External Protocols Setup  36

External Protocols Setup Screen  36

F1 key  11, 41

F2 key  41

F3 key  41

F4 key  41

File Transfer Hints  44

FILEHERE()  69

Filename Guessing  40

Find Phone List Items  20

General System Requirements  4

GOSUB()  67

GOTO()  62

Guess Filename  40

Guess Up/Download Filename  40

Hang Up Modem - Alt H  21

HANGUP()  66

Help Menu - F1  11

Help Screen  12

Help Screen - F1  41

High speed modem  27

IF()  68

IFFLAG()  63

IFSCAN()  63

Ignore Carrier Detect  66

IGNORECD()  66

INCFLAG()  62

Installation  6

Introduction to MyComm  1

Join Script  9

Join Script - Alt J  22

JUMPSCP()  69

KBDFLSH()  70

Kermit File Transfers  47

Kermit Transfers Setup  36

LF Translation Upload and Download  33

Line Editing Functions  51

Line Feeds On/Off - Alt F  21

Line Pace Time  33

Load Phone List Screen  20

Local Echo  16

Local Echo on by Default  28

Local Echo On During ASCII Uploads  33

Local Echo On/Off - Alt E  21

LOCLECHO()  68

Logon Name  15

Make Noise After File Transfers  39

Make Noise When Calls Connect  38

Manual Dial - Alt M  23

Manual Dial Screen  19

Manual Dial Window  19

Mark Phone List Items  20

MCSTPANS.EXE  77

Miscellaneous Options Setup  37

Miscellaneous Options Setup Screen  38

Modem Answer String  27

Modem Attention String  27

Modem Cancel Dial String  27

Modem Dial Suffix String  28

Modem Hang Up String  27

Modem Initialization String  26

Modem Setup Screen  25, 26

Move Phone List Item  20

MyComm as a Host  48

MyComm Bug Report Form  75

MyComm Command Summary  50

MyComm Files  4

MyComm Installation  6

MyComm Line Editing Functions  51

MyComm Order Form  74

MyComm Registration and Licensing  2

MyComm Screens and Menus  11

MyComm Strip ANSI  77

Name  14

Number  14

Order Form  74

Outside to DOS - Alt O  23

Pacing Char  34

Page Down  45

Page Up  45

Password  16

Password for sign on  39

Paths Setup  30

Paths Setup Screen  30

Pause Between Calls  28

PAUSE()  65

PgDn  45

PgUp  45

Phone File Path\File  31

Phone Item Info Screen  19

PORTFLSH()  70

PORTGET()  68

Present Line Status - Alt L  22

Present Line Status Window  22

Print Phone List  21

Protocols Setup  32

Protocols Setup Menu  32

Quick Re-Dial - Alt Q  24

QUIT()  67

Received BS (BackSpace) Destructive  17

RECVFILE()  65

Remote Abort Char  33

Restore to Defaults  41

RETURN()  67

Run External Program - Alt R  24

RUNDOS()  69

Save Changes  18

Save Setup  41

SCP Files  60

SCP_CHK.EXE  72

Screen Background Color  38

Screen Capture Path\File  31

Screen Text Color  38

Screen Width  58

Screens and Menus  11

Script - Dialing Entry  17

Script, creating  59

Script Labels  61

Script Language  59, 60

Script Sample  73

Script Syntax Checker  72

Scripts, Special Characters  61

Scroll Back - Alt K  22

SEAlink Derived File Transfers  45

Send Break - Alt B  12

Send Init. String Menu  22

Send Modem Initialization Strings - Alt I  21

Send Name - F2  41

Send Password - F3  41

Send Screen to File - Alt W  41

Send Tagline  - F4  41

SENDFILE()  64

SENDNAME()  64

SENDPASS()  64

SESCAP()  71

Session Capture On/Off/Pause - Alt S  24

Session Capture Path\File  31

SETFLAG()  62

SETSTR()  66

SNDBREAK()  65

Sort Phone List  20

Starting a File Transfer  43

Stop Bits  15

Strip Hi Bits  17

Strip High Bits  32

System Requirements  6

Terminal Emulation - Alt T  25

Terminal Screen  11

Terminal Type  16

Time Format - 12 or 24 Hour  38

Timed Start  10

Transferring Files  42

Troubleshooting  55

TTY Terminal Emulation  52

Update Configuration - Alt U  25

Update Configuration Menu  25

Upload Directory Path  31

UpLoad Files - PgUp  45

Use CTS/RTS Flow Control  29

Use Direct Video  39

Use DSR/DTR Flow Control  29

Use DTR to Hang Up Modem  28

Use XON/XOFF Flow Control  29

VT-100 Terminal Emulation  53

VT-52 Terminal Emulation  53

WAITFOR()  61

Word wrap  58

XMIT()  62

Xmodem File Transfers  46

Xmodem-1K  46

Ymodem  46

Ymodem Batch - 1K  46

Ymodem Batch File Transfers  46

Ymodem-G Batch  46

Your Name for sign on  39

Zmodem 32 Bit CRC  34

Zmodem Auto Download Feature  34

Zmodem File Transfers  47

Zmodem Receive Crash Recovery  35

Zmodem Send Crash Recovery  34

Zmodem Transfers Setup  34