�����������������������������������Ŀ

�  �����������������������������͸  ���

�  �         ATPlus.Exe          �  ���

�  �        Version  1.1         �  ���

�  �  �������������������������  �  ���

�  �   (c) 1993 Alien Archives!  �  ���

�  �     All Rights Reserved     �  ���

�  �����������������������������;  ���

������������������������������������ٱ�

�������������������������������������

October 03, 1993

������������������������������������������������������������������������͸

���� Introduction ��������������������������������������������������������

������������������������������������������������������������������������;

The purpose of ATPlus is to send commands directly to a modem from the

DOS command line. The program will send AT "Plus whatever you tell it"

to your modem.

The program will read the necessary information from its configuration

file, or directly via command line switches.  (You can always override

the configuration file via the command line if you wish.)

This can be handy if you've got a single command to send to your modem

often and do not want to be bothered loading up your favorite terminal

program to do it. Works great in batch files too!

������������������������������������������������������������������������͸

���� What it does ��������������������������������������������������������

������������������������������������������������������������������������;

ATPlus will send any command you like to a Hayes compatible modem. You

can use a configuration file to store default values, (which is useful

if you are sending the same command most of the time),  or you can use

the command line to enter values. You can always override the defaults

you set in the configuration file via the command line.

The program will execute simple commands (such as taking the line "off

hook") and simply return the "OK" returned from the modem, or in cases

where you want to enter a more specialized command (such as having the

modem display NVRAM settings) you can instruct the program to give you

a verbose listing of the modem results that are returned.

The program can be operated with, or without a FOSSIL driver loaded.

ATPlus returns ERRORLEVELS as follows:

255 = unspecified error (error code will be displayed)

255 = invalid screen mode (will be specified)

255 = no timer available (will be specified)

121 = FOSSIL initialization failure - FOSSIL not loaded

120 = FOSSIL initialization failure - wrong baud rate specified

119-110 = invalid value specified (error will be specified)

109 = configuration file not found (and no command was specified)

102 = program terminated - modem returned "ERROR"

101 = program terminated - key pressed to abort

80-1 = operating error (error will be specified)

0 = normal program termination

If an error occurs ATPlus will display the specific error to you so it

can be easily identified and corrected.

������������������������������������������������������������������������͸

���� What it needs �������������������������������������������������������

������������������������������������������������������������������������;

The program will operate on an IBM (or compatible) PC with an 80286 or

higher CPU; DOS 3.0 (or above); and approximately 64,000 bytes of free

RAM. This is a text mode application, so graphics capabilities are not

required.  Note: If you run this on less than an AT (ie 80286) it WILL

hang your system.

������������������������������������������������������������������������͸

���� Command line options ������������������������������������������������

������������������������������������������������������������������������;

There are a number of command line options. The program will first try

to open its configuration file (which must be named ATPLUS.CFG) in the

current directory, and if it is not found there, your DOS PATH will be

searched for the file.  If the configuration file is found, the values

in the file will be loaded (replacing the internal default values). If

a conflicting command is entered on the command line, it will be taken

over the configuration file. Hint: By placing both the program and its

configuration file in a directory that is in your PATH statement,  you

will be able to execute the program from any directory on your system.

The command line options available are as follows:

(a) -P[Port]  --  default value = 1

This is the base address of the port your modem is connected to.

If you are operating in FOSSIL mode, and you have *NOT* remapped

the ports, then ATPlus will compensate for the port address. (ie

if the modem is on COM1 and you are using a FOSSIL then you have

probably told the FOSSIL to use Port 0.  In this case, you would

tell ATPlus to use Port 1, and the necessary adjustments will be

made for you.

Valid values for -P are 1-4.

(b) -I[IRQ]  --  default value = 0 (standard IRQ for Ports 1-4)

This specifies which interrupt to use for the port selected. For

COM1 & COM3 the default interrupt that will be used is IRQ4, and

for COM2 & COM4 the default that will be used is IRQ3. If you're

using a port with a non-standard IRQ, use this option to specify

which IRQ is to be used. Note: You should be very careful before

you override the default values here,  as you may cause problems

with other devices in your system.

The following lists the default interrupts used:

COM        Base Address        IRQ (Default)

1           3F8  hex              4

2           2F8  hex              3

3           3E8  hex              4

4           2E8  hex              3

Valid values for -I are 0 (use IRQ default for Port) to 15.

(c) -L[Word Length]  --  default value = 8

This specifies the word length of the serial data.

Valid values for -L are 7 or 8.

(d) -T[Parity]  --  default value = 0

This specifies the parity to use. It can be  NONE (0),  ODD (1),

or  EVEN (2).

Valid values for -T are 0-2.

(e) -B[Baud Rate]  --  default value = 1200

This specifies the baud rate to be used for the port.  If a rate

of 0 is specified the port will be used "AS-IS". This will leave

the baud rate, parity, and word length unchanged. That might NOT

be the case if you were operating the program in FOSSIL mode (as

this is an undefined condition in the FOSSIL specification),  so

it is always best to provide a speed.

Valid values for  -B  are 0, 300, 1200, 2400, 4800, 9600, 19200,

38400, 57600, and 115200. (Note: 57600 & 115200 not supported if

you are using the program in FOSSIL mode.)

(f) -H[Handshake]  --  default value = 0

This specifies the type of handshake you want to use between the

CPU and the port (modem). You can select NONE (0), XON/XOFF (1),

CTS/RTS (2), or XON/XOFF and CTS/RTS (3).

Valid values for -H are 0-3.

(g) -F[FOSSIL Mode]  --  default value = 0

This specifies whether or not you will be using a FOSSIL driver.

Valid values for -F are 0 (no FOSSIL), or 1 (with FOSSIL).

(h) -C[Modem Command]  --  no default value

This is the command you would like ATPlus to send to your modem,

and it can be any valid command. The "AT" prefix is added to the

command automatically, so you only need to enter the actual com-

mand. If you are going to enter a command (such as one to return

the modem's NVRAM settings) that will generate a response great-

er than "OK" from the modem, you must make sure that you set the

-V(erbose) switch to 1 in order to get the proper display. There

cannot be any spaces in the command you enter (ie H 1 will *NOT*

work, but H1 will).

Valid values for -C are any valid modem commands.

(i) -V[Verbose]  --  default value = 0

This tells ATPlus how it should display the modem response. If a

command will only result in an "OK" message from the modem, then

the default mode (verbose OFF) is appropriate. In this mode your

command is sent to the modem,  and ATPlus waits for the modem to

respond with "OK". If there is no response from the modem within

8 seconds, or if a key is pressed to abort the process, then the

port will be closed and ATPlus will terminate.

If you are going to give the modem a command that will result in

more than an "OK" message (or no "OK" message as would result if

you were using ATPlus to dial a number),  then you would set the

verbose mode ON. In this mode your command is sent to the modem,

and ATPlus will display the entire results from the modem rather

than waiting for an "OK" to be returned. Just like the OFF mode,

if a key is pressed the port will be closed and the program will

terminate.  ATPlus will also monitor the time in this mode,  but

instead of waiting 8 seconds, it will wait 10 seconds. (This may

be needed to give the modem time to finish sending the requested

data.) This mode can be used with commands that will only result

in an "OK" message, however, the display will not be as "clean",

and the additional timeout delay is wasteful. (Hint: Output from

the program can be redirected using DOS redirection. This can be

a handy way to save results to a file for later reference.)

If you are going to use the program in a BATch file, you can set

the verbose mode to BATCH. In this mode ATPlus will suppress the

display unless an error occurs.

Valid values for -V are 0 (OFF), 1 (ON), and 2 (BATCH).

The command line is not case sensitive (ie -p is the same as -P),  and

options do not have to be given in a specific order.  You must include

the leading dash ("-") before each switch,  and the value entered must

follow the switch immediately. (Eg: -p2 is acceptable but -p 2 isn't.)

All of the options here can be entered in a configuration file so that

none need to be specified on the command line when you run the program

unless you want to override one or more of the values. The configurat-

ion file MUST be named ATPLUS.CFG and it must either be in the current

directory,  or in a directory listed in your DOS PATH statement (refer

to your DOS manual for details on the PATH statement if needed).

The configuration file is a plain ASCII text file, nine (9) lines long

with one option per line. All options must start at the beginning of a

line, with no leading spaces. The order of the file DOES matter (refer

to the sample configuration file that was included in the archive).

The order of the configuration file is as follows:

Line No.     Option                Example

Line 1 -->  Modem Command              H1

Line 2 -->  Port                       1

Line 3 -->  IRQ                        0

Line 4 -->  Word Length                8

Line 5 -->  Parity                     0

Line 6 -->  Baud Rate                  1200

Line 7 -->  Handshake                  0

Line 8 -->  FOSSIL Mode                0

Line 9 -->  Verbose Mode               0

Below you will see a few sample command lines, (with an explanation as

to how these would run the program).

Eg: ATPlus

(this runs the program using the defaults set in ATPLUS.CFG)

Eg: ATPlus -CZ

(this runs the program using the defaults set in ATPLUS.CFG,  but

overrides the Modem Command in the file and sends ATZ instead)

Eg: ATPlus -p3 -i15 -b38400 -c&V -v1

(this runs the program using COM3 & IRQ15 at 38400 baud, with the

verbose mode on, for the modem command AT&V -- all other options

are left at the default value)

������������������������������������������������������������������������͸

���� What you should have received ���������������������������������������

������������������������������������������������������������������������;

The following files should have been included in the "AV" stamped file

ATPLUS11.ZIP you received:

ATPLUS.CFG  --  example configuration file

ATPLUS.DOC  --  documentation in text format

ATPLUS.EXE  --  the basic executable

ATPLUS.HST  --  a brief history of versions

ATPLUS.PRN  --  this file

FILE_ID.DIZ  --  sample description

PACKING.LST  --  file list (with size/time details)

README.1ST  --  any last minute updates

REGISTER.FRM  --  registration form

VENDOR.DOC  --  information on distribution

������������������������������������������������������������������������͸

���� License �������������������������������������������������������������

������������������������������������������������������������������������;

ATPlus is being distributed as Shareware. Both the program and its re-

lated files are copyrighted, and NOT in the public domain, NOR is this

free software. Ownership is retained by the author.

Under the Shareware concept you are permitted to evaluate software for

a period of time to determine its usefulness to you.  This also offers

you a chance to test the software and ensure it will operate as stated

on your computer. If you continue to use the software after this eval-

uation period, then you MUST pay for it or remove it from your system.

You are granted a license to use this software for evaluation purposes

for a period of 30 (thirty) days. After 30 days you must either remove

the software from your system, or send in the registration fee for the

program.  The registration fee for ATPlus is $15.00 for non commercial

users, and $25.00 for commercial users.

For the purpose of this license, commercial includes any use in a "for

profit" organization (whether or not a profit is being made), and also

includes any governmental agency or educational institution. (The fees

stated are in Canadian dollars for Canadian users and U.S. dollars for

all others.)

Registration is a one time thing.  By registering the program now, you

are entitled to use any further versions of the program without having

to pay any additional registration fees. (This remains true even if in

the future "unlimited updates" are not included in the initial regist-

ration fee.)

This license applies only to version 1.1 of ATPlus, and will remain in

effect for one (1) year from the date the program is released or until

another version is released.  Future versions (if any) may not contain

the same license as stated here.

ATPlus is NOT crippled in any way. You have received a fully function-

al version of the program.  Registration merely gives you the right to

continue using the program legally after the 30 day evaluation period.

You are encouraged to share this program with your friends, and anyone

else you think might find the program beneficial. Please make sure you

only pass along the complete set of files. (It would be unfair to give

anyone an incomplete package, including all the documentation and sup-

port files.)

Refer to the REGISTER.FRM file for details on how to register, and the

bonus programs you could receive with your registration.

Please, help support the Shareware concept by registering your copy of

this program. Remember that even the software used to create this pro-

gram cost the author hundreds of dollars, not to mention the time that

was spent developing and testing this software. Your support is needed

to continue the development of this, and other software programs.

������������������������������������������������������������������������͸

���� Warranty ������������������������������������������������������������

������������������������������������������������������������������������;

THERE IS NONE WHATSOEVER. Every effort has been made to ensure no harm

will be caused by this program, however, by using it the risk is yours

entirely. ATPlus is used on a regular basis by the author and found to

operate as stated, however,  this is NO GUARANTEE that it will work on

your system. In no event will the author assume ANY responsibility for

any damages of any kind as a result of your use of this program.

THIS SOFTWARE IS SUPPLIED "AS IS". THERE ARE NO WARRANTIES OF ANY KIND

EXPRESS OR IMPLIED, INCLUDING THOSE OF MERCHANTABILITY, PERFORMANCE OR

FITNESS FOR A PARTICULAR PURPOSE.  IN *NO* EVENT WILL ALIEN ARCHIVES!,

ITS OWNERS, THE AUTHOR, OR ANY AFFILIATES BE LIABLE FOR DIRECT, INCID-

ENTAL, INDIRECT, SPECIAL,  OR CONSEQUENTIAL DAMAGES (INCLUDING DAMAGES

FOR LOSS OF PROFITS, BUSINESS INTERRUPTION, LOSS OF DATA AND THE LIKE)

RESULTING FROM ANY DEFECT IN THIS  SOFTWARE  OR ITS  DOCUMENTATION  OR

ARISING OUT OF THE USE OF  OR  INABILITY TO USE THIS  SOFTWARE  OR ITS

DOCUMENTATION EVEN IF THE AFOREMENTIONED HAS BEEN ADVISED OF THE POSS-

IBILITY OF SUCH DAMAGE.

������������������������������������������������������������������������͸

���� Miscellaneous �������������������������������������������������������

������������������������������������������������������������������������;

The program was written in Visual Basic for MS-DOS v1.00 (Professional

Edition), and was written on a 486/33C IBM compatible computer operat-

ing under MS-DOS 5.0.

Serial I/O routines were provided from QBserial v3.10 by Jeff Sumberg.

Trade names mentioned in this document are the property of their resp-

ective owners.

������������������������������������������������������������������������͸

���� Contacting the author �����������������������������������������������

������������������������������������������������������������������������;

The author can be contacted via one of the following:

--  BBS    : Alien Archives! (905)949-9437 -- HST/v32bis/v42/v42bis

--  FidoNet: 1:259/501

--  IMExNet: 89:480/310

--  Mail   : Mark Allan

3-2325 Hurontario Street, Suite 173

Mississauga, Ontario L5A 2E8

Canada

������������������������������������������������������������������������͸

���� Distribution ��������������������������������������������������������

������������������������������������������������������������������������;

The program may be freely distributed *PROVIDED* it is distributed the

way it was released, and with all files intact.  You are NOT permitted

to convert the archive to another format. Only if ATPLUS11.ZIP has the

"-AV" code from  Alien Archives!  on each file in the ZIP may you dis-

tribute the file - ANYTHING ELSE IS A COPYRIGHT INFRINGEMENT!  Version

2.04g of PKZIP was used in creating the original archive.

You may archive the original  ZIP  file "inside" an archive of another

format if you prefer, but the original file must remain intact.

See the included VENDOR.DOC file for complete details on the terms and

conditions for distributing this software.

������������������������������������������������������������������������͸

���� End of Document �����������������������������������������������������

������������������������������������������������������������������������;