DRVTEST

DRVTEST tests SCSI disk drives in a PC compatible system. In

order to function correctly on a system, an ASPI (Advanced SCSI

Programming Interface) driver must be available and installed on

the system. DRVTEST communicates with the SCSI drive though the

ASPI driver. Many manufacturers provide ASPI drivers for their

controllers. LOMAS DATA PRODUCTS, Inc. provides ASPI drivers for

their line of advanced caching/mirroring controllers for AT bus

systems. This program was developed and tested on the LDP CACHE

II PLUS controller and its ASPI driver and has been tested on the

world's fastest AT bus caching controller, the LDP CACHE IV.

It is assumed that the person using this test is familiar with

SCSI peripherals and error conditions and codes. Using this

utility, the SCSI disk drive can be rigorously tested and any bad

sectors found can be remapped using the SCSI REASSIGN BLOCK

command.

ERROR REPORTING

When an error is found, an error message is printed on the screen

that identifies the ASPI MANAGER error code along with the

information provided by a SCSI SENSE command. This information

will identify the error that occurred along with the record that

caused the error. In order to properly respond to error

conditions, you should have a list of the error codes your drive

may provide. A basic list of SENSE KEYs codes are provided at the

end of this document. If your drive did not come with a manual

defining what SENSE KEYs and EXTENDED SENSE CODES, the drive

provides in error circumstances, you may use these codes as a

guide. It is unlikely that any error with a SENSE KEY other than

'3' will benefit from having the record remapped.

An example of the error report follows:

ASPI error reading id 1, error code 4

Error code = 70

Sense key = 03

Extended sense code = 11

Block number = 34565

Should the failing block be reassigned?(y/n)

STARTING DRVTEST

DRVTEST requires one parameter, the SCSI device id of the drive

to be tested, and may accept one additional parameter which

defines how DRVTEST responds to error conditions.

The test is begun as follows:

DRVTEST SCSI_DEVICE_ID <nN,rR>

n,N is an optional non destructive specifier.

r,R automatically reassigns bad blocks when encountered during

testing

The following are examples of acceptable command lines:

DRVTEST 1

Tests the drive attached at SCSI address 1. Any

read/write testing will destroy data on the  drive

and users are prompted before reassigning a bad

record.

DRVTEST 1 R

Test the drive attached at SCSI address 1.

Read/write tests will destroy any existing data on

the drive. Any blocks found to be failing will be

automatically reassigned.

DRVTEST 1 N

Tests the drive attached at SCSI device 1.

Nondestructive testing will be performed. Data on

the drive will be preserved by saving the data

before testing the records under tests and restoring

them when the test is complete. The user will be

prompted before any bad records are reassigned.

DRVTEST 1 NR

Tests the drive attached at SCSI device 1. Non

destructive testing will be performed. Any failing

records will be automatically reassigned.

The test will verify that the device is a random access device

before proceeding.

The following menu will then be displayed:

1. READ TEST

2. SIMPLE READ/WRITE TEST

3. EXHAUSTIVE READ/WRITE TEST

4. RANDOM READ TEST (SEEK TEST)

5. LOW LEVEL FORMAT THE SCSI DRIVE

6. EXIT

READ TEST:

Read test will read all the records on the drive and if an error

occurs, it will be reported to the user. If the option 'R' was

specified, the bad record will automatically be reassigned.

Otherwise, the user will have the option of remapping the failing

sector if appropriate. The test will continue unless aborted by

the user. The test may be aborted at any time by typing ESC.

SIMPLE READ/WRITE TEST

Each record will have a '11100110' test pattern written to it

with a write-verify command. If the option 'R' was specified, any

bad records will automatically be reassigned otherwise the user

will have the option of remapping the failing sector if

appropriate. If the 'N' option is specified on the command line,

the test is done non-destructively. The test may be aborted at

any time by typing ESC.

EXHAUSTIVE READ/WRITE TEST

Each record will have several patterns written to it and

verified. The test patterns used will be:

00000000

11111111

01010101

10101010

11001100

If the option 'R' was specified, the bad record will

automatically be reassigned otherwise the user will have the

option of remapping the failing sector if appropriate. If the 'N'

option is specified on the command line, the test is done non-

destructively. The test may be aborted at any time by typing ESC.

RANDOM READ TEST

The random read test will randomly read sectors from the harddisk

drive specified. An error count will be kept and reported to the

user. No attempt will be made to reassign errors encountered. The

user will be prompted for how many read operations should be done.

Each read will consist of 1 sector only. This test is designed to

exercise the drive head positioning mechanism.

LOW LEVEL FORMAT THE SCSI DRIVE

This menu option will perform a low level format on the specified

drive. This option uses the SCSI FORMAT UNIT command to perform

the format and no additional testing is performed. This low level

format will format the drive in a manner that will remove any

records reassigned by any of the previous operations. After

performing this format, the drive should be tested with the

EXHAUSTIVE READ/WRITE TEST before being used.

SCSI ERROR CODES

SENSE KEY       Description

0h              NO SENSE: Indicates that there is no specific

sense key information to be reported for the

designated unit.

1h              RECOVERY ERROR: Indicates that the last command

was successfully completed, with some recovery

action performed by the drive. Details can be

determined by examining the additional sense

bytes in the information bytes.

2h              NOT READY: Indicates that the drive cannot be

accessed. Operator intervention may be required

to correct this condition.

3h              MEDIUM ERROR: Indicates that the command

terminated with a non-recovered error condition

which was probably caused by a flaw in the

medium, or an error in the recorded data.

4h              HARDWARE ERROR: Indicates that the drive detected

a non-recoverable hardware failure while

performing the command, or during the self test.

5h              ILLEGAL REQUEST: Indicates that there was an

illegal parameter in the command descriptor block

or in the additional parameters suplied as data

for some commands.

6h              UNIT ATTENTION: Indicates that reset occured

since the last selection by the initiator.

7h              DATA PROTECTION: Indicates the drive cannot be

written because it has been write protected.

8h thru Fh      For DRVTEST these codes have no meaning.