FAX DISPLAY

for the

KANTRONICS KAM and KPC3

Copyright (c) 1993, 1994 by Richard R. Hensel N8WLC

All Rights Reserved

This program is distributed as SHAREWARE and the author retains

all copyrights.  You are encouraged to give copies to all of your

friends and upload this program to any BBS as long as this document

is included.  If you like the program, please support further

development by sending $5.00 to

Richard Hensel N8WLC

715 Joselyn Avenue

Lexington, Ohio USA 44904

This program will interface with the fax capabilities built into

the KAM and KPC3 TNC's from Kantronics Inc.  The command are as

follows:

Keys        use

1-0         set line skipping ie 1 = 1 to 1   2 = 2 to 1

F1-F10      picture skew left,  skip (1 - 10) pixels to align fax on screen

AF1-AF10    picture skew right, skip (1 - 10) pixels to align fax on screen

ALT X       exit

ESC         Start fax recieve and leave terminal mode

i           invert picture ie black on white to white on black

t           return to terminal mode.  Stops fax receive

c           clear screen

s           save incoming fax to FAX.FIL

r           copy the saved fax to annother file name

p           playback

1-0         These set the aspect ratio of the displayed fax picture.

0 displays all lines of the incomming fax, and 9 displays

every 9th line.  I have found the most natural aspect is

found using 1.

F1-F10     The fax must be centered on the monitor screen.  If you

AF1-AF10   need to move the picture left use the F1 - F10 keys.  If

instead it would be better to move right use Alt F1 to

Alt F10.  These keys shift the picture 1 to 10 pixels

in the intended direction.

ALT X      Exits the program and re sets the TNC.

ESC        The Escape key causes the program to leave terminal

mode and start recieving a fax.

i          Invert the picture, that is change from white dots on a

black background to balck dots on a white background.

I have found that the best pictures come from using

upper side band and when revieving a satalite foto,

inverting the picture.

t          Return to terminal mode from fax recieve.  This will

put you into a simple terminal emulation connected

to the TNC.

c          Clear the screen, this will clear the screen of the

picture while in FAX recieve mode.

s          Save the incoming fax.  This will start saving the

fax to a file called FAX.FIL for later playback.

r          Will copy the FAX.FIL to another file so multiple

images can be saved.

p          Playback will cause the program to play FAX.FIL back

to the screen.

Coments and sugestions are wellcome.  I can be reached at the address

at the beginning of this doc file or on packet at

N8WLC @ W8WE.OH.USA.NA

Hardware Requirements

1      IBM PC/AT 80286 or equivelant with 640 Kbytes of menory.

2      Microsoft or IBM PC DOS 3.1 or higher.

3      EGA or VGA adapter card and monitor

Comming Attractions:

1     Printing the fax to you dot matrix printer.

2     Enhancements you request.

Bugs

1     Due to the COM port interface routines I am currently using

the program will not run if your computer has a 16550 uart

with the buffering turned on.  So i have provided a short

program "16550.exe" that will turn the buffering off.

just issue the command "16550 x off" where x is the COM port

1 or 2 that you are using.

###############################################