Personal Communications Library

For Windows

(PCL4W)

USERS MANUAL

Version 1.3

July 8, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1996

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : info@marshallsoft.com

web : www.marshallsoft.com

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4W Users Manual                                                Page 1

C O N T E N T S

Chapter                                                   Page

1.0 Introduction................................................3

1.1 User Support............................................4

1.2 ASP Ombudsman...........................................4

1.3 A Typical Application...................................5

1.4 Determining UART Address & IRQ Settings.................6

1.5 Disabling Windows Ports.................................6

1.6 Installation............................................7

2.0 Library Organization........................................8

2.1 Configuration...........................................8

2.2 Initialization & Termination............................8

2.3 Modem Control & Status..................................9

2.4 Serial I/O..............................................9

2.5 Error Detection........................................10

2.6 General Support........................................10

3.0 Library Overview...........................................11

3.1 Dynamic Link Libraries.................................11

3.2 DOS Protected Mode Interface...........................11

3.3 Memory Models..........................................11

3.4 Using the Library......................................11

3.5 Using a MAKEFILE.......................................12

3.6 Using an IDE...........................................12

3.7 Making the Library.....................................12

4.0 Talking to Your Modem......................................13

4.1 Modem Standards........................................13

4.2 Flow Control...........................................14

4.3 Modem I/O Functions....................................15

4.4 Modem Initialization...................................17

5.0 Problems...................................................18

6.0 Serial Communications......................................19

6.1 Communications Basics..................................19

6.2 Standard Port Addresses................................20

6.3 Running 3 or 4 Ports Concurrently......................21

6.4 Using Multiport Boards.................................22

6.4.1 Using the Digiboard..............................22

6.4.2 Using the BOCA board.............................22

6.5 Transmitter Interrupts.................................23

6.6 RS232 Signals..........................................24

6.7 National INS8250, INS16450, and INS16550 UARTs.........25

6.8 Register Summary.......................................26

7.0 Example Programs...........................................28

7.1 SIMPLE.................................................28

7.2 LOGIN..................................................28

7.3 SELFTEST...............................................28

7.4 TERM...................................................28

8.0 Legal Issues...............................................29

8.1 Registration...........................................29

8.2 License................................................30

8.3 Warranty...............................................30

9.0 Summary....................................................31

9.1 Revision History.......................................31

9.2 Function Summary.......................................32

9.3 Further Reading........................................33

10.0 Other MarshallSoft Computing products for C/C++............33

10.1 The Personal Communications Library for C/C++.........33

10.2 Libraries for Other Languages.........................33

PCL4W Users Manual                                                Page 2

1.0 Introduction

The  Personal  Communications Library for Windows (PCL4W) is an asynchronous

communications dynamic link library (DLL) for software developers using  the

Microsoft  Windows  C/C++  compiler,  the  Borland  Windows compiler, or the

WATCOM Windows compiler. Other Windows compilers may  also  work.   A  80386

machine or higher is recommended.

o 30+ communications functions.

o Supports the high performance INS16550 UART.

o Supports the PC/4 and PC/8 DigiBoard.

o Supports the BOCA BB1004, BB1008, and BB2016 boards.

o Supports hardware (RTS/CTS) flow control.

o Use any IRQ (IRQ0-IRQ15) with any UART address.

o Interrupt driven receiver & (optionally) transmitter.

o Supports 300 baud to 115,200 baud.

o Supports COM1 through COM20.

o Adjustable queues from 8 bytes to 32 KB.

o Control-BREAK error exit.

o 18 communications error conditions trapped.

o Allows 4 ports to run concurrently (more with Digi/BOCA board).

o Complete modem control & status.

o Written in assembly language for small size & high speed.

o Terminal program featuring ASCII (with XON/XOFF), XMODEM,

YMODEM, & YMODEM-G.

Why should you buy PCL4W? Consider the following:

COMPLETE  :  PCL4W is complete since it provides absolute control of the

serial ports (including the high performance INS16550).

COMPACT  :  PCL4W is very compact at less than 6 KB.  Your  application

doesn't carry a lot of excess code.

FAST  :  PCL4W is fast.  It will run at 115200 baud on all 80386 and

up machines.

SUPPORT   : If  you  get  stuck,   you will talk to the programmer that

wrote the code, not a person hired to answer the phone.

BBS  :  A user support BBS is available (2400 to 14400 baud, N81) in

order to provide immediate support as necessary.

NEWSLETTER  :  Our newsletter "COMM TALK" is published quarterly (available

online)   which   discusses   communications   problems  and

solutions.

PRICE  :  You get PCL4W for a very reasonable price !

UPGRADES  :  Once  you  buy  PCL4W,  you can always update  to  the most

recent version for a very reasonable cost.

COMPATIBLE  :  PCL4W  is  call  for call compatible with the MSDOS based

version PCL4C except for two functions in which delay  times

are no longer necessary.

Our goal is to provide a robust serial communications library that that  you

and your customers can depend upon.

PCL4W Users Manual                                                Page 3

1.1 User Support

We want you to be successful in developing your applications using PCL4W! We

depend on our customers to let us know what they need  in  a  communications

library.   This  means we are committed to providing the best communications

library that we can. If you have any suggestions or comments, please let  us

know.

If you are having a problem using PCL4W, call  us  at  205-881-4630  between

1:30PM  and  9:30PM  CST  Monday  through Friday. You can also call at other

times and leave a message, and call back  later  for  a  reply.   Registered

users (ONLY) can also FAX us at 205-880-0925 at any time (24 hours).

However,  we  can  only  answer  questions  with  respect to using the PCL4W

library.  We cannot help you program your application.

You  may  also  call  our User Support BBS (2400 to 14400 baud, no parity, 8

data bits, 1 stop bit) at 205-880-9748 and leave a message  (address  it  to

the SYSOP).  We will usually have a reply ready for you within 24 hours.

The BBS  is  available  24  hours  per  day  except  at  2  PM  Sundays  for

maintenanace. All files are in standard ZIP format. The BBS will contain the

latest  shareware  version  of  all MarshallSoft products as well as related

files such as:

BUGS.ZIP:  Bug report.

NEWS.ZIP:  Latest news regarding our products.

If you are on the Internet, you can email us at info@marshallsoft.com.   You

can  also  get  the  latest  versions of our products from our anonymous ftp

site:

FTP:   ftp.marshallsoft.com

PATH:   marshallsoft

The MarshallSoft  Computing,  Inc.   newsletter  "Comm  Talk"  is  published

quarterly.  It discusses various communications problems and solutions using

PCL4W  as  well  as related information.

The   latest   copy   of our newsletter can be found on User Support BBS (in

file area "Newsletters"), our anonymous ftp site  (directory  /marshallsoft)

as well as our web site.

Web site:  www.marshallsoft.com

1.2 ASP Ombudsman

MarshallSoft  Computing,  Inc.   is a member of the Association of Shareware

Professionals (ASP).  ASP wants to make sure that  the  shareware  principle

works  for  you.   If  you are unable to resolve a shareware-related problem

with an ASP member by contacting the member directly, ASP  may  be  able  to

help.  The  ASP  Ombudsman can help you resolve a dispute or problem with an

ASP member, but does not provide technical support  for  members'  products.

Please  write  to  the  ASP  Ombudsman  at 545 Grover Road, Muskegon, MI USA

49442-9427, Fax 616-788-2765, or send a CompuServe  message  via  CompuServe

Mail to ASP Ombudsman 70007,3536.

PCL4W Users Manual                                                Page 4

1.3 A Typical Application

In general, there are two classes of applications that use a  communications

library  like  PCL4W: those that use a modem to connect to the outside world

and those that connect directly to a peripheral device. In  either  case,  a

typical   application  program  using  PCL4W  might  consist  of  3  phases:

initialization, execution, and  termination.   The  following  code  segment

assumes a knowledge of the windows API.

In  the  following  code  segment,  SioRxBuf is called to set up a 1024 byte

receive buffer; SioParms is called to set up the parity, stop bit count, and

word length; SioReset is called to set the baud rate to 9600 and  reset  the

UART (Universal Asynchronous Receiver / Transmitter).

case WM_CREATE:

/* initialization */

SioRxBuf(Port,RxBuffer,Size1024);

SioParms(Port,NoParity,OneStopBit,WordLength8);

SioReset(Port,Baud9600);

If you are using the version of  the  library  with  transmitter  interrupts

enabled  (PCL4W2.LIB),  then  SioTxBuf()  must  be  called  to  set  up  the

transmitter buffer. The transmit buffer must be distinct  from  the  receive

buffer.

case WM_CREATE:

/* initialization */

SioRxBuf(Port,TxBuffer,Size1024);

SioTxBuf(Port,RxBuffer,Size1024);

SioParms(Port,NoParity,OneStopBit,WordLength8);

SioReset(Port,Baud9600);

Characters can be  transmitted  and  received  once  the  required  port  is

initialized.   Before leaving your application, SioDone is called to restore

the prior state of the serial communications  system.   Forgetting  to  call

SioDone  before  exiting your application will leave the interrupt system in

an unuseable state and the machine will have to be re-booted.

case WM_DESTROY:

/* termination */

SioDone(Port);

If you are using a modem, you also need to be concerned  about  initializing

your  modem  correctly  and  handling  any  required flow control.  Refer to

Chapter 4.0, "Talking to Your Modem" for detailed information.

PCL4W Users Manual                                                Page 5

1.4 Determining UART Address & IRQ Settings

Both Windows 3.X and Windows 95 maintain a list of serial port settings.  On

Windows  3.X  systems, choose the "Ports" icon in the "Control Panel" in the

"Main Group".  Select "Settings", and then "Advanced" to view the  COM  port

UART address and IRQ settings.

In Windows 95, choose "My Computer" icon ("or whatever your computer icon is

named), select the "Control Panel" folder and then the "System" icon.  Click

on  the "Device Manager" tab.  Click "Computer" and then click "Properties".

Click the "View Resources" tab.   To  view  reserved  resources,  click  the

resource  type at the top of the dialog box (i.e., "Interrupt request (IRQ)"

or "Input/output (I/O)" ).

1.5 Disabling Windows Ports

The Windows operating system virtualizes all serial ports that it  controls.

This  means  that  Windows  intercepts  reads  and writes to hardware ports,

resulting in significant slowdown. When Windows virtualizes a port, the UART

FIFO control register (FCR) can not be modified.

Since  PCL4W  also  controls the hardware directly, it is best (although not

absolutely necessary) to disable Windows control of the serial ports and the

IRQs that will be used by PCL4W applications.

The four standard COM ports which Windows typically controls are:

Port  Address  IRQ

COM1   &H3F8    4

COM2   &H2F8    3

COM3   &H3E8    4

COM4   &H2E8    3

Since COM1 and COM3 share an IRQ, if either is to be used by PCL4W then both

should be disabled from Windows control. Similarly for COM2 and COM4.

To begin, exit from Windows completely. To disable the  serial  ports,  edit

the  [386Enh] section of the WINDOWS.INI file in the WINDOWS directory.  For

example, to disable COM1 and COM3,  add  the  following  to  the  [386  Ehn]

section:

COM1Base=0

COM1Irq=-1

COM3Base=0

COM3Irq=-1

Restart Windows after modifying SYSTEM.INI. This procedure does not apply to

multiport boards or any other serial ports unknown to Windows.

16550 UARTS are really necessary when running under Windows, particularly at

baud  rate  above 19200.  The Microsoft Diagnostics (MSD) program can be run

which will report the type of UARTs installed on your machine.

If a 16550 port MUST be shared with Windows, and you want the FIFO  enabled,

look for statements such as

COM1FIFO=0

in  the  [386Ehn] section of SYSTEM.INI and delete it.  However, the Windows

communications driver may have trouble with the FIFO, which is why there  is

a statement to disable it. This problem does not occur with PCL4W.

PCL4W Users Manual                                                Page 6

1.6 Installation

(1) Before installation of PCL4W , your Windows C compiler should already be

installed on your system and tested. Examine the file "FILES.LST" for a list

of the distribution files.

(2) Make a backup  copy  of  your  distribution  disk.   Put  your  original

distribution disk in a safe place.

(3) Create a work directory on your hard drive. For  example,  to  create  a

work directory named PCL4W, type from DOS:

MKDIR PCL4W

(4)  Copy  all  the  files from your backup copy of the distribution disk to

your work directory.  For example, to copy from the A: drive  to  your  work

directory, we type:

CD PCL4W

COPY A:*.*

(5)  Compile SIMPLE.C and link with the PCL4W library (PCL4W.LIB).

Makefiles  and  project  files  are  provided  for  each  of  the  supported

compilers.

Microsoft command line compiler:   NMAKE SIMPLE._M_

Borland command line compiler:     MAKER -fSIMPLE._B_

Watcom command line compiler:      WMAKE -f SIMPLE._W_

NOTES: (1) Use NMAKE, not MAKE with Microsoft C

(2) Use MAKER, not MAKE with Borland C.

Alternately, SIMPLE can be compiled in an Integrated Development Environment

(IDE)  such  as supplied by Microsoft, Borland, and Watcom. Refer to section

3.5 "Using an IDE" for more information.

(6) The recommended way to test  SIMPLE  is  to  run  it  on  two  computers

connected  by  a null modem cable.  Whatever is typed on one computer should

be displayed on the other. If you don't have  two  computers,  you  can  use

SIMPLE  to talk to your modem. Sending an "AT" to the modem should result in

an "OK" being sent back. See section 7.1 "SIMPLE" for more information.

(7) Compile and link LOGIN. Use LOGIN to call a local BBS. See  section  7.1

"LOGIN" for more information.

(8)  Compile  and  link  SELFTEST.   Use SELFTEST to test one of your serial

ports. See section 7.3 "SELFTEST" for more information.

PCL4W Users Manual                                                Page 7

2.0 Library Organization

The  PCL4W  library  is organized into six categories of functions. Refer to

the PCL Reference Manual (PCL4W.REF) for details on individual functions.

2.1 Configuration

There  are three functions in the configuration category.  SioPorts sets the

number of PC and DigiBoard / BOCA ports.  SioUART is used to change the UART

base address for a communications port  to  a  non-standard  address,  while

SioIRQ  is  used  to  assign a nonstandard IRQ line to a port.  (See Section

6.2, Standard Port Addresses for more details on standard UART addresses and

IRQ lines).

The  configuration  functions  SioPorts,  SioUART  and SioIRQ must be called

before calling any other library functions.   Be  very  careful  when  using

these  functions.   Remember that your serial hardware must support the UART

and IRQ that you specify.  Always test any new configuration immediately.

SioPorts : Sets number of PC and DigiBoard / BOCA ports.

SioUART  : Sets the UART base address.

SioIRQ   : Assigns an IRQ line to a port.

THE IRQ GOLDEN RULE: You may open (via  SioReset)  only  one  port  per  IRQ

(except for the DigiBoard / BOCA board).

2.2 Initialization & Termination

There  are  nine  functions  in the initialization and termination category.

Together, SioGetDiv, SioFIFO, SioRxBuf, SioTxBuf,  and  SioReset  initialize

your  serial communications system.  Your application must call SioParms and

SioRxBuf before calling SioReset, and SioReset must  be  called  before  any

serial I/O processing can be done.

After initialization, SioParms and SioBaud can be called again to change the

communications  parameters without resetting the serial port. SioFlow can be

called to enable hardware flow control.

Before exiting from your application, SioDone must  be  called.  Failure  to

call SioDone can crash your system later.

SioRxBuf   : Sets up receive buffer.

SioTxBuf   : Sets up transmitter buffer.

SioFIFO    : Sets the interrupt level for the INS16550.

SioParms   : Sets parity, stop bits, and word length.

SioReset   : Initialize a serial port for processing.

SioDone    : Terminates further serial processing.

SioBaud    : Sets the baud rate of the selected port.

SioFlow    : Enables / disables flow control.

SioGetDiv  : Gets the baud rate divisor.

PCL4W Users Manual                                                Page 8

2.3 Modem Control & Status

There  are  eight  functions  in the modem control and status category which

provide your application with complete control over the status  and  control

bits of your modem.

There  are  two modem control bits, "Data Terminal Ready" (DTR) and "Request

To Send" (RTS). These bits can be  read,  set,  or  cleared  by  SioDTR  and

SioRTS.

There  are  four  modem status bits, "Data Set Ready" (DSR), "Clear To Send"

(CTS), "Ring Indicator" (RI), and "Data Carrier Detect" (DCD). SioModem  can

read  any  of  the modem status bits.  SioDSR, SioCTS, SioRI, and SioDCD can

only read their respective modem status bit.

Refer to the chapter entitled "RS232 Signals" for a discussion  of  each  of

the control and status bits.

SioDTR   : Set, clear, or read the Data Terminal Ready (DTR) bit.

SioRTS   : Sets, clears, or reads the Request to Send (RTS) line.

SioModem : Reads the modem status register.

SioDSR   : Reads the Data Set Ready (DSR) modem status bit.

SioCTS   : Reads the Clear to Send (CTS) modem status bit.

SioDCD   : Reads the Data Carrier Detect (DCD) modem status bit.

SioRI    : Reads the Ring Indicator (RI) modem status bit.

SioRead  : Reads the contents of the 7 UART registers.

2.4 Serial I/O

There  are  eight  library  functions  in the serial I/O category. Together,

these functions give the programmer complete control over serial I/O. Higher

level functions such as protocols and  smart  modem  communications  can  be

completely  implemented  in  terms  of these functions. Refer to the example

code.

SioGetc and SioPutc perform all the actual serial I/O.   SioUnGetc  "ungets"

the  last  serial  byte  read.  SioRxClear  clears  the  receive queue while

SioTxClear clears the transmit queue. SioLine can be used to test  for  UART

errors.   SioRxQue  returns  the  number of bytes in the receive queue while

SioTxQue returns the number of bytes in the transmit queue.

SioGetc    : Reads the next character from the serial line.

SioPutc    : Transmit a character over a serial line.

SioUnGetc  : "Un:gets" (puts back) a specified character.

SioRxClear : Flush (clears) the receive buffer.

SioRxQue   : Returns the number of characters in the RX queue.

SioTxClear : Flush (clears) the transmit buffer.

SioTxQue   : Returns the number of characters in the TX queue.

SioLine    : Reads the line status register.

PCL4W Users Manual                                                Page 9

2.5 Error Detection

There  are  three  functions  in  the  error  detection  category.  They are

concerned with detecting or reporting communications errors.  Use  of  these

functions can make your application significantly more robust.

SioBrkSig can read or  modify  the  UART  break  bit.  This  is  useful  for

signalling   the   remote  system  that  a  fatal  condition  has  occurred.

SioLoopBack can be used to test the integrity of your UART.

There is also a file SIOERROR.C which contains the function  SioError  which

displays  a  error  message  corresponding  to an error code returned from a

PCL4W function (every PCL4W function returns a code).

SioBrkSig   : Asserts, cancels, or detects the RS232 BREAK  signal.

SioError    : Displays  error  in  text.

SioLoopBack : Performs a UART loopback test.

2.6 General Support

There is one function in the general support category. SioInfo  returns  the

version  number  of  the  library  and  whether  transmitter  interrupts are

enabled.

SioInfo      : Returns the library version, memory model, the number of

transmitter interrupts, and receiver interrupts.

PCL4W Users Manual                                                Page 10

3.0 Library Overview

3.1 Dynamic Link Libraries

PCL4W is provided as a dynamic link library (DLL). A DLL is characterized by

the fact that it need not be loaded until required by an application program

and that only one copy of the DLL is necessary regardless of the  number  of

application  programs  that  use it. Contrast this to the traditional static

library which is bound to each and every application that uses  it  at  link

time.

Since  PCL4W is a DLL, only one copy of the PCL4W code & data is loaded into

memory regardless of the number of applications programs that use it.

For example, more than one instance  of  the  test  program  SIMPLE  can  be

started.   All  copies of SIMPLE can run concurrently as long as each uses a

different COM port.

3.2 DOS Protected Mode Interface

Windows itself uses what is known as  the  "DOS  Protected  Mode  Interface"

(DPMI)  to  control  changing  states  (protected  vs real), controlling the

interrupt system, etc.  This is necessary because Windows  itself  typically

runs  in  a  protected  state  and must change to real state in order to use

MSDOS services.

The  PCL4W  library  uses  this same DPMI mechanism. In particular, the file

USE_DPMI.ASM contains the necessary functions for using DPMI. The  developer

should not modify USE_DPMI.ASM in any way.

3.3 Memory Models

Most windows programs are small or medium memory model programs since  these

memory models are limited to one data segment.  However, all PCL4W functions

are FAR functions, so they can be used with any memory model. Thus, there is

just one PCL4W.DLL and PCL4W.LIB regardless of the memory model used.

3.4 Using the Library

The  PCL4W has been tested on a Gateway 2000 25 MHZ 80386-DX, a Gateway 2000

66MHZ 80486-DX2 (all running Windows  3.1),  and  a  Mid-West  Micro  150MHZ

Pentium running Windows 95.

PCL4W  has  been  testing  with  the  Microsoft Windows SDK compiler, Visual

C/C++, Borland C/C++, and Watcom C/C++.

Please examine the PCL4W.H file. Note that COM1 is defined as port zero, not

port  one.   The  user must assume the responsibilty for passing the correct

information when calling PCL4W functions.

If there are any conflicts between PCL4W  definitions  and  those  in  other

libraries,  the PCL4W definitions can be changed in the PCL4W.H file and any

file that uses the definition. There is no change necessary for the  library

code itself.

If  you  write  an  application  using  the PCL4W library, don't run another

application that uses the same communications port.

PCL4W Users Manual                                                Page 11

3.5 Using a MAKEFILE

Makefiles  originated  on UNIX systems. They are the standard way that C/C++

programs are constructed in command line environments. Windows programs  can

be  constructed  with  makefiles  running  DOS  using  command  line Windows

compilers, such as provide by Borland and Microsoft.

Makefiles are provided for Microsoft and Borland command line compilers.

3.6 Using an IDE

All windows comilers have an Integrated Development  Environment  (IDE)  for

building  application programs in the Windows environment. Since there is no

standard format for IDE project files, file names must be entered  into  the

IDE  from  the  keyboard.  Each example program has a project text file (eg:

SIMPLE.PRJ) which contains the list of filenames that must be  entered  into

the IDE.

The  example  programs  have been build using Microsoft, Borland, and Watcom

IDEs. Refer to IDE_INFO.DOC for more information on using specfic IDEs.

3.7 Making the Library

Registered  users  may  wish  to  assemble PCL4W.ASM.  Use the /MX switch in

order to disable automatic conversion from lower case to upper case.  If the

/MX switch is not used, then all PCL4W function references in C code must be

in upper case. To assemble using the Microsoft assembler, use  the  provided

makefile "PCL4W.MAK". From the DOS command line, type

nmake pcl4w.mak

Two  libraries  are  made, one with transmitter interrupts disabled (PCL4W1)

and one with transmitter interrupts enabled (PCL4W2). Most users should  use

the  library  without transmitter interrupts enabled since they require less

overhead.

To use the PCL4W qlibrary without transmitter interrupts enabled, type

copy pcl4w1.lib pcl4w.lib

copy pcl4w1.dll pcl4w.dll

To use the PCL4W library with transmitter interrupts enabled, type

copy pcl4w2.lib pcl4w.lib

copy pcl4w2.dll pcl4w.dll

After creating PCL4W.LIB and PCL4W.DLL, the example programs can be compiled

and linked. Refer to Chapter 7.0, "Example Programs".

PCL4W Users Manual                                                Page 12

4.0 Talking to Your Modem

A  modem  is  used  to  extend  the distance over which you may communicate.

Without a modem, your RS232 cable is limited to a maximum  of  approximately

50 feet.  But with a modem, you can communicate literally around the world.

4.1 Modem Standards

Two modems can communicate over a telephone line only if they are both using

the  same  signaling frequencies and modulation, which are determined by the

the modem standards used.  Modem standards can be divided into  three  sets:

(1) speed, (2) data compression used, and (3) error control.

The  Bell  standards  (103  &  212A)  are  those  of  AT&T.   The CCITT (The

International Consultative Committee for Telephone and Telegraph)  standards

are designated as "V. ".

Speed

Bell 103  :   300 baud

Bell 212A :  1200 baud

V.21      :   300 baud

V.22bis   :  1200 & 2400 baud

V.32      :  4800 & 9600 baud

V.32bis   :  4800, 7200, 9600, 12000, and 14400 baud

Data Compression

MNP 5     :  Microcom Networking Protocol (proprietary).

V.42bis   :  International data compression standard.

Error Control

MNP 2,3,4 :  Three level error correction (public domain).

V.42      :  International error correction standard.

PCL4W Users Manual                                                Page 13

4.2 Flow Control

With  modems  using data compression, the modem to modem connection will run

at various speeds depending on the quality of the  line.   The  computer  to

modem  connection  will be at a fixed baud rate. Therefore, a protocol (flow

control) is necessary to synchronize the data flow between a modem  and  the

computer  to  which  it  is  connected.   Refer  to  your  modem  manual for

information on flow control protocols supported.

Two flow control protocols are used by most all modems  which  require  flow

control.  Software  flow  control  is called "XON/XOFF" (other software flow

control character pairs are defined but operate the same  as  XON/XOFF)  and

hardware  flow  control is called "RTS/CTS".  Most modems which require flow

control enable hardware flow control by default.

In XON/XOFF (software) flow control, the computer suspends transmitting data

if it receives a XOFF character (13  hex)  from  the  modem,  and  continues

transmitting  when  it  receives  a XON character (11 hex).  Similiarly, the

computer can signal the modem not to send any more data  by  transmitting  a

XOFF  to  it,  and  can tell the modem to continue transmission be sending a

XON.

In RTS/CTS (hardware) flow control, the RTS line is used by the computer  to

signal  the  modem  ,  while the CTS line is used by the modem to signal the

computer.  The RTS line is set OFF by the computer  to  tell  the  modem  to

suspend  transmission,  and  set  to  ON  to  tell  the  modem  to  continue

transmission.  The CTS line is set to OFF by the modem to tell the  computer

to  stop  transmitting,  and  set  to  ON  to  tell the computer to continue

transmitting.

Given the choice, always choose hardware flow  control  over  software  flow

control  so  that  all  data  transmission is transparent.  If hardware flow

control is not the default (which it almost always is),  you  should  modify

your modem initialization string to turn hardware flow control on.

PCL4W Users Manual                                                Page 14

4.3 MODEM I/O (MIO)

4.3.1 MIO Introduction

The file MIO.H contains declarations for using the  Modem  I/O  DLL.   These

functions  ease  communicating  with  modems  using  AT commands.  The LOGIN

example program code has examples of using the MIO functions.

The Windows "operating system" is what is called "cooperative multitasking".

This  means  that  the  executing  Windows  task  must voluntarily "give up"

control before another Windows task (or Windows itself) can execute.

The difficulty  is  how  to  wait  for  a  number  of  seconds  while  still

relinquishing control periodically before the wait is up. One solution is by

making  functions that need to wait (such as the MIO functions) into "state"

machines. A function is broken down into parts called states, and control is

returned to Windows between executing each state.

For example, to send the string "ATDT1,205,880,9748" to the modem  with  one

tenth second delay between transmitted characters, the following sequence is

used:

(1) Send the string to the MIO driver by executing:

Code = mioSendTo(Port,100,"!ATDT1,205,880,9748")

The '!' characters are converted to carriage returns.  The  text  string  is

copied into the driver's data area.

(2)  Call mioDriver (typically based on a timer) until MIO_IDLE is returned.

Each time mioDriver is called, it will send another character to  the  modem

provided  the  required  delay  (since  the previous character was sent) has

passed.  If the delay has not passed, the driver simply returns MIO_RUNNING,

but without actually sending a character to the modem.  Once all  characters

have been sent, mioDriver will return MIO_IDLE, indicating it is done and is

ready to accept another function.

mioDriver  will return MIO_RUNNING if it is still processing.  Anything else

received indicates that it is still processing and the returned value  is  a

character from the modem that can be displayed if wanted.

(3) Once mioDriver returns MIO_IDLE, call mioResult to get the result of the

mioSendTo call.

The above is rather straight forward in practice.  See the LOGIN application

for an example of its use.

4.3.2 MIO Functions

4.3.2.1 mioDriver

Syntax: int mioDriver(int Port)

mioDriver  executes  the  next  state of any previously started MIO function

such as mioSendTo, mioWaitFor, and mioQuiet. It returns:

MIO_IDLE    : if the driver is ready for the next mioSendTo, mioWaitFor, or

mioQuiet.

MIO_RUNNING : if the driver is not idle.

<otherwise> : if the driver is not idle,  and  the  returned  character  was

received from the modem.

PCL4W Users Manual                                                Page 15

4.3.2.2 mioBreak

Forces the MIO driver to the IDLE state, abandoning any work in progress.

4.3.2.3 mioSendTo

Syntax:  int mioSendTo(int Port, long Pace, char *String)

The mioSendTo function sends the characters in the string 'Text'  to  serial

output. There is a delay of 'Pace' milliseconds between characters.

4.3.2.4 mioWaitFor

Syntax: int mioWaitFor(int Port, long Wait, int Case, char *String)

The  mioWaitFor  function  waits for characters from serial input that match

the string 'Text'. A total of 'Wait' milliseconds are allowed before  timing

out  and  returning  FALSE (0).  If the 'Case' flag is TRUE, then the string

comparison is NOT case sensitive.

For example, to wait up to one minute for "CONNECT", "NO CARRIER", or "BUSY"

from the modem after dialing a number (on COM1):

Code = mioWaitFor(COM1,60000,1,"CONNECT|NO CARRIER|BUSY")

The function mioDriver() must be called until MIO_IDLE  is  returned.   Then

mioResult()  is  called  to  get the result of the mioWaitFor.  A value of 0

indicates  that  neither "CONNECT", "BUSY", nor "NO CARRIER" was received. A

non-zero value indicates that one of the three sub-strings was received.   A

ASC("0")  is  returned  if  "CONNECT"  was seen, ASC("1") is returned if "NO

CARRIER" was seen, and ASC("2") is returned if "BUSY" was seen.

4.3.2.5 mioQuiet

Syntax: int mioQuiet(int Port,long Wait)

The mioQuiet function waits for continuous quiet [no incoming  serial  data]

of  'Wait'  milliseconds  before  returning.   Any  incoming character while

mioQuiet us running is flushed.

4.3.3 MIO Summary

mioDriver   : Allows  the  execution  of  mioSendTo,  mioWaitFor, or

mioQuiet once they have been  started.  Returns  MIO_IDLE

(defined in MIO.H) if ready not running,   MIO_RUNNING if

running,  and anything else is a character from the modem

that can be displayed if wanted.

mioBreak    : Forces the MIO driver to IDLE state.

mioSendTo   : Sends string (including control chars) to the modem.

mioWaitFor  : Waits for a particular string from the modem, passing

all else through.

mioQuiet    : Waits for continuous quiet of specified duration.

mioBreak    : Breaks further modem I/O activity.

PCL4W Users Manual                                                Page 16

4.4 Modem Initialization

If your application uses a modem (as opposed to using a null  modem  cable),

then  you should always send an initialization string to your modem if it is

a programmable modem such as those made by  Hayes.   Communication  programs

such as PROCOMM and TELIX always send such a string automatically as soon as

they start up.

The particular initialization string depends on the make of your modem.  For

Hayes  and  Hayes  AT  command  set  compatible modems, the following string

(followed by a carriage return) should work:

AT E1 S7=60 S11=60 V1 X1 Q0 S0=0

Refer to your Modem User's Guide for a full discussion of these commands.  A

brief description is as follows:

AT     Modem attention command.

E1     Modem will echo what you send to it.

S7=60  Wait 60 seconds for carrier and/or dial tone.

S11=60 Use 60 milliseconds for tone dialing duration & spacing.

V1     Display result code as words (not numbers).

X1     Use the extended result message (CONNECT XXXX) set.

Q0     Modem returns result codes.

S0=0   Do not answer RING.

If  your application will answer incoming calls, then set the S0 register to

the ring on which to automatically answer.

If you send the above codes by using SioPutc (as opposed to typing them from

the keyboard), then follow these guidelines:

(1) Send an initial carriage return before the initialization string.

(2) Pause at least 150 milliseconds after each character sent as your  modem

needs the time to perform its own internal processing. Pause a little longer

if your modem is not accepting your initialization string.

(3)  Pause  one  and a half seconds after sending any initialization command

such as ATZ or AT&F since your modem must do quite a bit of processing.

If you experience any problems in initializing your Hayes modem, you  should

first reset it to factory settings by sending:

AT&F

Refer to the LOGIN program (function SendToModem in the file MODEM_IO.C)  for

an example of sending an initialization string to a Hayes compatible modem.

PCL4W Users Manual                                                Page 17

5.0 Problems

If  you  cannot  get your application to run properly, first compile and run

the terminal emulator program SIMPLE provided  on  your  distribution  disk.

Test  SIMPLE  by  connecting  two  computers  with  a null modem cable or by

commanding a Hayes AT command set compatible modem.

Once  SIMPLE  runs, compile and run the SELFTEST program.  This program will

test your serial ports functionality.

If your application does not run but  SIMPLE  and  SELFTEST  run  correctly,

then  you  have  most likely made a programming mistake in your application.

MarshallSoft Computing cannot debug your application,  especially  over  the

telephone!  However,  consider  each  of the following when searching for an

error in your application.

1.  Have you included the file PCL4W.H in your application ?

2.  Is your receive buffer large enough ? If you are using 1K data blocks in

YMODEM,  then  your  receive  buffer should be at least 1K (2K if baud rates

above 19200 are to be used).

3.  Have you selected too high a baud rate? Windows can multitask many tasks

at once. You may have to lower your baud rate (or get 16550 UARTS).

4.  If you are running two COM ports simultaneously, are you using separate

receive and transmits buffers ? (you should).

5.  Did SioReset return a zero value ?  If not, then you must call  SioReset

again. See LOGIN.C for an example.

6.   Did  you  send the proper initialization string to your modem ? Did you

set DTR and RTS ? (you should).

7.  Do you have more than one COM1 port?  For example, if you  have  a  COM1

port  on  your  motherboard, you cannot add another COM1 port or modem board

that uses COM1 without first disabling the COM1 on the motherboard.

8.   Is  your  IRQ set correctly? If you can transmit data but can't receive

using the PCL4W library with transmitter interrupts disabled (PCL4W2),  then

this usually means that interrupts are not working.

9. If -19 is returned from SioReset, this means that  you  have  not  called

PostMainHandle() from within your code first. See example in SIMPLE.C.

We recommend the following steps if you believe that you have  discovered  a

bug  in  the  library:  (1)  Create  the  smallest,  simpliest  test program

possible that demonstrates the problem.  (2)  Document  your  exact  machine

configuration  and  what error the test program  demonstrates.   (3)  Upload

the example source to our user support BBS, email it or mail us a disk.

If  the  problem can be solved with an easy work-around, we will publish the

work-around.  If the problem requires a modification to the library, we will

make the change and make the modified library  available  to  our  customers

without charge.

PCL4W Users Manual                                                Page 18

6.0 Serial Communications

6.1 Communications Basics

The  heart  of  serial  communications  is  the UART (Universal Asynchronous

Receiver Transmitter).  The IBM  PC/XT/AT  and  compatibles  use  the  8250,

16450, or the 16550 UART.  The purpose of the UART is:

(1) To convert bytes from the CPU (Central Processing Unit), into  a  serial

format  by  adding  the  necessary start, stop, and parity bits to each byte

before transmission, and to then transmit each bit at the correct baud rate.

(2) To convert the incoming stream (at a specified baud rate) of serial bits

into bytes by removing the start, stop, and parity bits  before  being  made

available to the CPU.

The  UART  is part of the serial interface circuitry which allows the CPU to

send and receive signals over the RS232 lines. This  can  be  diagrammed  as

follows:

Serial Interface

+-------------------+

|                   |

+-----+  Data Bus  |     +------+      |    RS232 Signals

| CPU +------------+     | UART |      +----------------*

+-----+            |     +------+      |

|                   |

+-------------------+

The  8250/16450/16550  UART  is  capable  of  operating in one of two modes,

"polled" and "interrupt driven".  The serial communications functions in the

BIOS uses the polled method.  In this approach, the CPU is  typically  in  a

loop  asking  the  UART  over  and over again if it has a byte ready. If its

does, the polling code returns the byte.  But, if the  next  byte  comes  in

before the polling code is executing again, then that byte is lost.

In the interrupt driven approach (used by PCL4W), when a byte is received by

the  UART,  an  "Interrupt  Service  Routine" (ISR) is executed immediately,

suspending temporarily whatever else is executing. The ISR  then  moves  the

byte to a buffer so that your application program can later read it.

If transmitter interrupts are enabled, then  bytes  are  queued  up  waiting

transmission.   When  a  byte  is  moved  from  the UART transmitter holding

register to the UART transmitter shift register, an interrupt  is  generated

and  the  next  byte is taken from the library transmitter buffer by the ISR

and written to the UART holding register.

Up to 16 bytes can be taken from the transmitter buffer while processing one

transmitter  interrupt  if an 16550 UART is used. The 16550 UART is strongly

recommended for computers doing serial communications under Windows.

Refer to Sections  6.6  and  6.7  entitled  "RS232  Signals"  and  "National

INS8250,   INS16450,   and   INS16550   UARTs",   respectively  for  further

information on these topics.

PCL4W Users Manual                                                Page 19

6.2 Standard Port Addresses

There  are  a  few  things to know about how serial communications ports are

used by IBM PC/XT/AT and compatible computers.  The  standard  IBM  PC/XT/AT

configuration values are as follows:

Port     Reg Base   IRQ Line   Vector

COM1        3F8H         4        12

COM2        2F8H         3        11

COM3        3E8H         4        12

COM4        2E8H         3        11

(Refer  to  your  DigiBoard  manual  for  DigiBoard addresses, and your BOCA

manual for BOCA board addresses).

PCL4W  assumes the above values.  If necessary, the UART base address can be

changed by SioUART, and IRQ lines can be re-assigned  by  SioIRQ.   Remember

that  each  port to be used concurrently must have a unique IRQ line.  Refer

to the PCL4W Reference Manual for specific details.

When installing new  communications  cards,  the  following  guidelines  are

recommended:

(1)  Be  sure to read the documentation for the hardware you are installing.

Pay special attention to UART base addresses and IRQ lines, particularly  if

trying to set up a non-standard configuration.

(2)  If  you  have  a  choice in base addresses and IRQ lines, always choose

standard values as defined above.

(3) The first port should be COM1, the second COM2, etc.

(4)  Use SioUART to zero all unused ports (for example, call SioUART(COM4,0)

if there is no COM4 port installed).

(5) Be carefull not to configure two ports for the  same  address.  This  is

easier to do than you may believe.

(6)  Choose  an  external  modem over an internal one.  It is much easier to

debug problems with an external modem than an internal one.

(7) Select hardware flow control (RTS/CTS) if flow control is  required  and

hardware flow control is not the default.

(8)  Always  test your port as soon as it is installed. Try several programs

that use the communications ports.

PCL4W Users Manual                                                Page 20

6.3 Running 3 or 4 Ports Concurrently

PCL4W supports up to 4 serial ports running concurrently (more if you have a

DigiBoard or BOCA board).  One free interrupt for  each  port  is  required.

Refer to the next section if you have a DigiBoard or BOCA board.

Interrupts IRQ4 and IRQ3 are dedicated to  the  communications  ports  in  a

standard  IBM  PC/XT/AT configuration.  IRQ4 is shared between COM1 and COM3

while IRQ3 is shared between COM2 and COM4.  This means that you can run two

ports simultaneously provided that they don't share an interrupt.

Suppose that you wish to run 3 ports simultaneously. To begin, you must have

3 serial UARTs installed on your computer.  Assume,  for  purposes  of  this

discussion,  that  COM1  is installed on your motherboard, and that you have

purchased a new 2 port serial communications board.

You should be able to configure the first serial board port as  COM2,  which

uses IRQ3.  Refer to the manual that came with your serial board.

In  order  to  run the third serial port concurrently with the first two, an

unused interrupt must be found.  If your serial card can use only  IRQ3  and

IRQ4,  then there is no way to run a third line since IRQ4 and IRQ3 are used

for COM1 and COM2.

However, many serial cards can use other IRQs, typically IRQ2 through  IRQ7.

Since  IRQ5  is  normally  used  for  a  second  printer  port, it is a good

candidate for COM3. To use IRQ5 for the third serial port,  first  set  your

serial card to use IRQ5 for COM3 (refer to your serial card manual) and then

add the following line to your applications code before calling SioReset:

SioIRQ(COM3,IRQ5);

Don't  forget  to  disable  any device that might use IRQ5, such as a second

printer port or a music card.   Unfortunately,  there  is  no  easy  way  to

determine  that  you have no conflicts until you actually attempt to use the

IRQ. If there are conflicts, your system will probably  hang  and  you  will

have to reboot.

To  run  a  fourth  serial  port,  another  free IRQ must be found.  On some

systems, IRQ7 can be used. You must first disable IRQ7 on your parallel port

card first (your printer doesn't need an IRQ). To use IRQ7  for  the  fourth

serial port, first set your serial card to use IRQ7 for COM4 and then add:

SioIRQ(COM4,IRQ7);

To  summarize,  your  serial  card must be able to generate the correct IRQ,

which is not already being used. Refer to the entry for the SioIRQ  function

in the PCL4W Reference Manual.

PCL4W Users Manual                                                Page 21

6.4 Using Multiport Cards

The  PCL4W  library supports the dumb Digiboard (PC/4 and PC/8) and the dumb

BOCA board (BB1004, BB1008, and BB2016).   Most  multiport  boards  will  be

compatible  with either the DigiBoard (the status register contains the port

number), or the BOCA board (the status register is bit mapped).

6.4.1 The DigiBoard

In order to use the DigiBoard, (not the Intelligent Digiboards such  as  the

PC/Xe  and  PC/Xi) you must configure PCL4W using the SioPorts(), SioUART(),

and SioIRQ() functions.

Your  PCs  ports  must be partitioned into "standard" PC ports and dumb card

ports.  Remember that standard PC ports cannot share IRQs like the DigiBoard

(or BOCA board) can. If you are using IRQ4 and IRQ3 for  standard  PC  ports

COM1  and  COM2, then you cannot use either for DigiBoard ports (try IRQ5 or

IRQ7).

Suppose that COM1 through COM2 are standard PC ports (using IRQ4  and  IRQ3)

and  you  have  installed  a  PC/8  DigiBoard  that you wish to use for COM3

through COM10 using interrupt line IRQ5.  You choose to use the  recommended

DigiBoard UART addresses starting at 0x100:

SioPorts(10,COM3,0x140,DIGIBOARD);/* COM3 = 1st DigiBoard port */

Address  = 0x100;                 /* 1st DigiBoard UART address */

for(Port=COM3;Port<=COM10;Port++) /* look at each port    */

{SioUART(Port,Address);         /* set the UART address */

Address += 8;                  /* compute next address */

SioIRQ(Port,IRQ5);             /* set the DigiBoard IRQ */

}

The DigiBoard uses 0x140 for the status address for odd interrupts and 0x141

for even interrupts.

Digiboard  may  be  contacted  at  6400 Flying Cloud Drive, Eden Prairie, MN

55344.  Telephone 612-943-9020 or FAX 612-943-5398.

6.4.2 The BOCA Board

In  order  to  use  the dumb BOCA Boards, you must configure PCL4W using the

SioPorts(), SioUART(), and SioIRQ() functions.

For example, to configure the BOCA BB2016 to use COM1 to  COM16,  with  base

addresses starting at 0x100 and IRQ5:

SioPorts(16,COM1,0x107,BOCABOARD);/* COM3 = 1st BOCA board port */

Address = 0x100;                  /* 1st BOCA UART address */

for(Port=COM1;Port<=COM16;Port++) /* look at each port    */

{SioUART(Port,Address);         /* set the UART address */

Address += 8;                  /* compute next address */

SioIRQ(Port,IRQ5);             /* set the BOCA IRQ */

}

BOCA may be contacted at BOCA Research, Inc., 6413  Congress  Avenue,  Suite

130, Boca Raton, FL 33487.  Phone 407-241-8088, FAX 407-997-0918.

PCL4W Users Manual                                                Page 22

6.5 Transmitter Interrupts

Transmitter interrupts are supported by the library.  Separate libraries are

provided, one with transmitter interrupts enabled  and  one  without.   When

transmitter  interrupts  are  NOT  enabled, the following logic occurs every

time you call SioPutc():

1. Wait for transmit buffer to become empty. The transmit  buffer  may  not

be empty if the previous transmit is not completed (the UART breaks down the

byte  &  sends  1  bit at a time). 2. When the transmit buffer is empty, the

byte from the SioPutc() call is loaded into the transmit buffer and  control

is returned to the caller.

Note that you can not write to the UART any faster the the UART baud rate.

When transmitter interrupts are enabled, the byte from SioPutc() is put into

a previously prepared (by SioTxQue) transmitter queue. The interrupt service

routine fetches bytes from this queue as soon as the previous byte has  been

sent.

While you can now call SioPutc() faster than the baud rate, bytes are  still

transmitted at the given baud rate.

The   above   sounds   like  transmitter  interrupts  are  the  way  to  go.

Unfortunately, this is usually NOT the case.  Most applications will perform

better if transmitter interrupts are NOT enabled.

The reason is that transmitter interrupts double the amount of code  in  the

time  critical interrupt service routine.  While the library is processing a

transmitter interrupt (which can take a while), incoming bytes  can  not  be

processed.  What this means is that a given machine can run at a higher baud

rate without transmitter interrupts. This problem is compounded when running

multiple ports simultaniously.

However,  there are a few application areas where transmitter interrupts are

preferable.  If your application will be  transmitting  blocks  of  data  at

fairly slow baud rates you might profit from enabling transmitter interrupts

provided  that there is something else for the processor to do (which is NOT

the case in most protocols).

However,  if  you are using 16550 UARTS (which have 16 byte on-chip transmit

and receive buffers rather that the 1 byte buffers on  the  8250  and  16450

UARTS),  you  may  want  to  use the library version with interrupts enabled

provided that you enable the 16550 UART with the SioFIFO() function.

PCL4W Users Manual                                                Page 23

6.6 RS-232 Signals

RS-232 is the name of the serial data interface  standard  used  to  connect

computers  to modems.  Most IBM compatible computers are built with at least

one serial port and use either DB9 (9 pin) or DB25 (25 pin) connectors.

A summary of these pins and  their  function  follows.   For  more  detailed

information, refer to one of the many books dealing with RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The  SG  line  is  used  as  the  common  signal  ground, and must always be

connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The DTR line is used by the computer to signal the modem that it  is  ready.

DTR should be set high when talking to a modem.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer that it is ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The  RTS  line  is used to "turn the line around" in half duplex modems, and

for hardware flow control in most modems that require flow control.  RTS  is

controlled by the computer and read by the serial device (modem).

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The  CTS  line  is used to "turn the line around" in half duplex modems, and

for hardware flow control in most modems that require flow control.  CTS  is

controlled by the serial device (modem) and read by the computer.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the computer that a data carrier

signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

PCL4W Users Manual                                                Page 24

6.7 National INS8250, INS16450, and INS16550 UARTs

The  Personal  Communications  Library  is  based  on  the standard National

INS8250, INS16450, and INS16550 UARTs. The 8250 was the original  UART  used

in  the IBM PC, whereas the 16450 is a faster version found on most 286 & up

machines. The 16550 contains a 16 byte FIFO to further reduce communications

overhead. These UARTs consists of 8 register ports as follows:

Offset R/W Register

0   R/W    Receiver  (read)  /  Transmitter  (write)

1   R/W    Interrupt Enable (read)

2   R      Interrupt Identification

2   W      FIFO control (INS16550 only)

3   R/W    Data Format (Line Control)

4   R/W    RS-232 (Modem) Control

5   R/W    Line Status

6   R/W    RS-232 (Modem) Status

7   R/W    Not used.

For  the  standard  PC  ports  (not DigiBoard ports), the UART registers are

based at 3F8h (COM1), 2F8h (COM2), 3E8h (COM3), and 2E8h (COM4).   COM1  and

COM3  share  interrupt  request  line IRQ4 while COM2 and COM4 share request

line IRQ3.  This means that  COM1  and  COM3  can't  be  used  concurrently.

Similarly for COM2 and COM4.

However,  standard PC ports may be re-configured to use other UART addresses

and IRQ assignments.  Refer  to  the  SioPorts(),  SioUART(),  and  SioIRQ()

functions.

If  you  have  a  DigiBoard PC/4 (or PC/8) installed, you will have 4 (or 8)

additional ports using INS16450 or INS16550 UARTS.   The  default  DigiBoard

ports  are  located  at 100h, 108h, 110h & 118h for the PC/4 continuing with

120h 128h, 130h & 138h for the PC/8. IRQ3 is the default for all ports.

Four sources of interrupts  are  possible  with  the  8250  and  16550:  (1)

receiver error or BREAK, (2) receiver data ready, (3) ready to transmit, and

(4)  RS232  input.   These  four  sources  of  interrupts  are summarized as

follows:

Source of Interrupt        Action Required to Clear

Receiver error or BREAK.   Read Line Status register.

Receiver data.             Read data from data register.

Transmitter Buffer Empty.  Write to data register or read IID reg.

RS232 Input.               Read Modem Status register.

If  you are not familiar with UARTS, several good books are available. Refer

to  Chapter  6.0,  Serial  Communications   chapter   for   recommendations.

Although  a knowledge of the 8250/16450/16550 is not necessary to use PCL4W,

a general knowledge of the theory of asynchronous serial  communications  is

helpful.

PCL4W Users Manual                                                Page 25

6.8 Register Summary

REG 0 : Data Register

Reading from the data register fetches the  next  input  byte,  once  it  is

ready.   Writing  to the data register transmits the byte written to it over

the serial line.

REG 1 : Interrupt Enable

The Interrupt Enable register enables each of four types of interrupts  when

the appropriate bit is set to a one.

bit 3 :  Enable interrupt  on  RS232  input.

bit 2 :  Enable interrupt on receiver error or break.

bit 1 :  Enable interrupt on transmitter buffer empty (TBE).

bit 0 :  Enable interrupt on received data (RxRDY).

REG 2 : Interrupt Identification (IID)

Reading the Interrupt Identification (read only) register once an  interrupt

has occurred identifies the interrupt as follows:

Bit 2  Bit 1  Bit 0  Priority   Interrupt

0     0      1      none      none

1     1      0      0         (high) Serialization or break.

1     0      0      1         Received data.

0     1      0      2         Transmitter Buffer Empty.

0     0      0      3         (low) RS232 Input.

In  the  INS16650,  REG  2  (write  only) is also the FIFO control register.

Writing bits 6 & 7 will set the FIFO trigger level (number of bytes received

before an interrupt is generated).

Bit 7  Bit 6   Trigger             Bit 7  Bit 6   Trigger

0      0      1 byte               1      0     8 bytes

0      1      4 bytes              1      1     14 bytes

REG 3 : Line Control

RS232 line parameters are selected by writing to this register.

bit 7   : DLAB = 0

bit 6   : BREAK on(1), off(0).

bits 5-3: Parity None(000),ODD(001),EVEN(011),MARK(101),SPACE(111)

bit 2   : One stop bit(0), two stop bits(1).

bits 1-0: Data bits = 5 (00), 6(01), 7(10), 8(11).

When  the Divisor Latch Access Bit (DLAB) is 1, registers 0 and 1 become the

LS and MS bytes of the Baud Rate Divisor registers.

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

PCL4W Users Manual                                                Page 26

REG 4 : Modem Control

RTS, DTR, loopback testing, and  General  Purpose  Outputs  #1  and  #2  are

controlled by the Modem Control register as follows:

bit 4 : Enable local loopback.

bit 3 : Enable GP02. Necessary for 8250 interrupts.

bit 2 : Enable GP01.

bit 1 : Set / clear RTS.

bit 0 : Set / clear DTR.

REG 5 : Line Status

Reading  the  Line Status register provides status information as follows (1

for TRUE, 0 for FALSE) :

bit 6 : Transmitter Empty.

bit 5 : Transmitter Buffer Empty (TBE).

bit 4 : BREAK detect.

bit 3 : Framing error.

bit 2 : Parity error.

bit 1 : Overrun error.

bit 0 : Data Ready.

REG 6 : Modem Status

Reading the Modem Status register provides the following status  information

(1 for TRUE, 0 for FALSE) :

bit 7 : DCD status.

bit 6 : RI status.

bit 5 : DSR status.

bit 4 : CTS status.

bit 3 : Delta DCD status.

bit 2 : Delta RI status.

bit 1 : Delta DSR status.

bit 0 : Delta CTS status.

The delta bits (bits 0 through 3) are set whenever one of  the  status  bits

(bits  4 through 7) changes (from 0 to 1 or from 1 to 0) since the last time

the Modem Status register was read. Reading the Modem Status register clears

the delta bits.

REG 7 : Scratch Register

There  is  no  function  associated  with  register 7.  It does not exist in

earlier versions of the 8250.

PCL4W Users Manual                                                Page 27

7.0 Example Programs

The  example programs can be built either using the command line makefile or

directly from your compiler's IDE (see section 3.6 "Using and IDE").

7.1 SIMPLE

SIMPLE  is a very simple communications programming using PCL4W.  Everything

that is typed on the keyboard is sent to  the  serial  port,  and  everthing

incoming from the serial port is displayed on the screen.

Using the Microsoft command line compiler: NMAKE SIMPLE._M_

Using the Borland command line compiler:   MAKER -fSIMPLE._B_

Using the Watcom command line compiler:    WMAKE -f SIMPLE._W_

If you are using  the  Borland  IDE,  be  sure  and  turn  off  LINKER  case

sensitivies:  Choose  OPTIONS,  PROJECTS,  LINKER,  GENERAL and turn off the

"case sensitive link" and "case sensitive exports & imports" boxes.

The SIMPLE makefiles use the small memory model. If a different memory model

is  required,  the  Microsoft  (or  Borland)  runtime  library  must also be

changed to match the memory model. No  change  is  required  for  the  PCL4W

library.

For example, to change the makefile from using the  small  memory  model  to

using  the  medium  memory  model,  the following changes are necessary: For

Microsoft, change -AS to -AM and change SLIBCEW to  MLIBCEW.   For  Borland,

change -sm to -mm and change C0WS, CWS, and MATHWS to C0WM, CWM, and MATHWM.

7.2 LOGIN

LOGIN  is the same program as SIMPLE but with the addition of "Modem" on the

menu bar.  Under "Modem", select "Start", then "Handshake" in order to  send

an "AT" to the connected modem, or "Dial" to send a dial string to the modem

(which  dials  our  user  support  BBS). Once the dial sequence is sent, the

program will wait for up to 60 seconds for the  "CONNECT"  string  from  the

modem.  This  wait  can  be terminated at any time by choosing "BREAK" under

"Modem".

To  test LOGIN, you need a AT command set compatible modem and a BBS to call

up. LOGIN will dial our BBS at 205-880-9748, or  edit  the  dial  string  in

LINE.C to call up a local BBS.

7.3 SELFTEST

SELFTEST performs a serial port I/O functionality test. A single port can be

tested  if  a  loopback adapter is connected to the end of the serial cable.

Refer to LOOPBACK.DOC for an explanation of how to make a loopback adapter.

7.4 TERM

TERM  is  an  example  communications  program suitable for calling bulletin

board systems (BBS).

TERM features modem initialization, hardware flow control, and state  driven

file  transfer  using  ASCII,  XMODEM, and YMODEM, communications protocols.

TERM can be used to  call  up  any  bulletin  board  system,  including  the

MarshallSoft Computing BBS.

In  an  attempt  to  minimize the size of the shareware version of the PCL4W

library,  the  TERM  program  files  are  distributed  as  a  separate  file

(WTERMxx.ZIP,  where  xx  is the current version). WTERMxx.ZIP can always be

downloaded from the MarshallSoft support BBS.

PCL4W Users Manual                                                Page 28

8.0 Legal Issues

8.1 Registration

If you wish to register the PCL4W library, please send $75 plus $5 S&H  ($10

outside of North America) to:

MarshallSoft Computing, Inc.

Post Office Box  4543

Huntsville AL 35815-4543

Multiple  copies  are  available: $60 each for 2 to 5, $45 each for 5 to 10,

and $30 each for 20 or more.  A site license  is  also  available  for  $495

(includes 5 sets of printed documentation). We pay shipping.

We accept American Express, VISA, MasterCard, checks in US dollars drawn  on

a   US   bank, International Postal Money Orders, purchase orders (POs) from

recognized US schools and companies listed in  Dun  &  Bradstreet,  and  COD

(street  address and phone number required) within the USA (plus a $4.50 COD

charge).

For  credit  card  orders,  be  sure  to  include  the  account  number, the

expiration date, the exact name on the card, and the complete  card  billing

address (the address to which the credit card bill is mailed).

You can also order PCL4W from The Public Software Library  (PSL)  with  your

MC,  Visa,  AmEx,  or  Discover card by calling 800-242-4PSL (from overseas:

713-524-6394) or by FAX at 713-524-6398.  THESE  NUMBERS  ARE  FOR  ORDERING

ONLY. The product number for PCL4W is 11171.

Print the file PCL4W.INV if an invoice is needed.

If you wish to update from an older version of PCL4W, send $25 plus  $5  S&H

($10  outside  of  North  America).   Updates  must be ordered directly from

MarshallSoft Computing.

The registered package includes:

o  PCL4W Library w/o shareware screens.

o  Assembler source code for the library.

o  SCRIPT language V2.1 (compiler and interpreter).

o  Laser printed Users and Reference Manuals.

o  Telephone, BBS, and email support for one year.

The  registered  user will receive the latest version of PCL4W shipped by US

second day priority mail (packet airmail  overseas).   A  3.5"  diskette  is

provided unless a 5.25" diskette is requested.

All  prices  are guaranteed for one year from the release date as printed on

the title page.

PCL4W Users Manual                                                Page 29

8.2 License

MarshallSoft  Computing,  Inc. grants the registered user of PCL4W the right

to use one copy of the PCL4W library (in object form) on a  single  computer

in  the  development  of  any software product (other than libraries such as

PCL4W).  The user may not use the library on more than one computer  at  the

same  time.   The  source code for the library (PCL4W.ASM) is copyrighted by

MarshallSoft Computing and may not be released in whole or in part. Products

developed using PCL4W may be distributed without any royalty.

8.3 Warranty

MARSHALLSOFT COMPUTING, INC.  DISCLAIMS  ALL  WARRANTIES  RELATING  TO  THIS

SOFTWARE,  WHETHER  EXPRESSED  OR  IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR  PURPOSE,

AND  ALL  SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY DISCLAIMED. NEITHER

MARSHALLSOFT COMPUTING, INC. NOR ANYONE ELSE WHO HAS BEEN  INVOLVED  IN  THE

CREATION,  PRODUCTION,  OR DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY

INDIRECT, CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING OUT  OF  THE  USE  OR

INABILITY  TO  USE  SUCH  SOFTWARE EVEN IF MARSHALLSOFT COMPUTING, INC.  HAS

BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS. IN NO EVENT SHALL

MARSHALLSOFT COMPUTING, INC.'S LIABILITY FOR ANY SUCH  DAMAGES  EVER  EXCEED

THE  PRICE  PAID FOR THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM

OF THE CLAIM. THE PERSON USING THE SOFTWARE BEARS ALL RISK AS TO THE QUALITY

AND PERFORMANCE OF THE SOFTWARE.

Some states do not allow  the  exclusion  of  the  limit  of  liability  for

consequential  or  incidental damages, so the above limitation may not apply

to you.

This agreement shall be governed by the laws of the  State  of  Alabama  and

shall  inure  to  the  benefit  of  Marshallsoft  Computing,  Inc.   and any

successors, administrators, heirs and  assigns.  Any  action  or  proceeding

brought  by either party against the other arising out of or related to this

agreement shall be brought only in a STATE or  FEDERAL  COURT  of  competent

jurisdiction  located in Madison County, Alabama. The parties hereby consent

to in personam jurisdiction of said courts.

PCL4W Users Manual                                                Page 30

9.0 Summary

9.1 Revision History

Version 1.0: 1 February, 1994.

The  initial  release  of  PCL4W uses a considerable amount of the code from

version 4.0 of PCL4C (the DOS version of the comm library).

Version 1.1:  August 1, 1994

o A flow control bug was fixed.

o COM11 through COM16 defined.

o Supports the BB1004, BB1008, and BB2016 BOCA boards.

o The new function SioGetDiv() was added.

o A script compiler & interpreter was added to the registration package.

Version 1.2:  August 8, 1995

o Defined ports extended to 20.

o Support for all IRQs [IRQ0 to IRQ15].

o Bug in BREAK detection corrected.

o Bug in SioRxClear fixed.

o SioInfo also returns # interrupts.

Version 1.3: July 8, 1996

o SioRxFlush renamed SioRxClear.

o SioTxFlush renamed SioTxFlush.

o Added more choices in SioInfo.

o Added LOGIN & SELFTEST example programs.

o Simplified SIMPLE example program.

PCL4W Users Manual                                                Page 31

9.2 Function Summary

Refer  to the PCL4W Reference Manual (PCL4W.REF) for detailed information on

the communications and  support  functions.  A  one  line  summary  of  each

function follows:

+--------------+-----------------------------------------------------------+

|  SioBaud     |  Sets the baud rate of the selected port.                 |

|  SioBrkSig   |  Asserts, cancels, or detects BREAK signal.               |

|  SioCTS      |  Reads the Clear to Send (CTS) modem status bit.          |

|  SioDCD      |  Reads the Data Carrier Detect (DCD) modem status bit.    |

|  SioDone     |  Terminates further serial processing.                    |

|  SioDSR      |  Reads the Data Set Ready (DSR) modem status bit.         |

|  SioDTR      |  Set, clear, or read the Data Terminal Ready (DTR) bit.   |

|  SioFIFO     |  Sets the interrupt level for the INS16550.               |

|  SioFlow     |  Enables / disables hardware flow control.                |

|  SioGetc     |  Reads the next character from the serial line.           |

|  SioGetDiv   |  Gets the baud rate divisor.                              |

|  SioInfo     |  Returns library ver number, memory model, # interrupts   |

|  SioIRQ      |  Assigns an IRQ line to a port.                           |

|  SioLine     |  Reads the line status register.                          |

|  SioLoopBack |  Performs a UART loopback test.                           |

|  SioModem    |  Reads the modem status register.                         |

|  SioParms    |  Sets parity, stop bits, and word length.                 |

|  SioPorts    |  Sets # ports, 1st DigiBoard port & status register.      |

|  SioPutc     |  Transmit a character over a serial line.                 |

|  SioRead     |  Reads any of 7 UART ports.                               |

|  SioReset    |  Initialize a serial port for processing.                 |

|  SioRI       |  Reads the Ring Indicator (RI) modem status bit.          |

|  SioRTS      |  Sets, clears, or reads the Request to Send (RTS) line.   |

|  SioRxBuf    |  Sets up receive buffer.                                  |

|  SioRxClear  |  Clears the receive buffer.                               |

|  SioRxQue    |  Returns the number of characters in the receive queue.   |

|  SioTxBuf    |  Sets up transmit buffer.                                 |

|  SioTxClear  |  Clears the transmit buffer.                              |

|  SioTxQue    |  Returns the number of characters in the transmit queue.  |

|  SioUART     |  Sets the UART base address.                              |

|  SioUnGetc   |  "Un-gets" (puts back) a specified character.             |

+--------------+-----+-----------------------------------------------------+

PCL4W Users Manual                                                Page 32

9.3 Further Reading

The  best way to learn about serial communications is to read a good book on

the subject. Several good texts are available.  Two that I like are:

(1) C Programmers's Guide to Serial Communications by  Joe  Campbell  (SAMS)

(2) Mastering Serial Communications by Peter Gofton (SYBEX).

10.0 Other MarshallSoft Computing Products

The following shareware  libraries  are  also  available  from  MarshallSoft

Computing.   They  can  be  registered  directly  through us.

10.1 The Personal Communications Library for C/C++

The Personal Communications Library for the C  Language  (PCL4C)  is  a  DOS

based  asynchronous communications library designed for experienced software

developers programming  in  C.   Four  compilers  are  supported:  Microsoft

Optimizing  C,  Microsoft Quick C, Borland Turbo C, and MIX Power C.  An IBM

PC/XT/AT or compatible is required.  PCL4C is the DOS equivalent of PCL4W.

The Personal Communications Library for C (PCL4C) is available for $75  plus

$5  S&H ($10 S&H overseas). PCL4C and PCL4W can be ordered together for $120

plus $5 S&H ($10 overseas).

10.2 Libraries for Other Languages

We  also have communications libraries for Turbo Pascal, Visual Basic (DOS),

and PowerBASIC.  A Visual BASIC library for Windows is also available.

PCL4W Users Manual                                                Page 33