Windows Standard Communications

Library for C/C++

(WSC4C)

REFERENCE MANUAL

Version 1.0

September 9, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1996

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : info@marshallsoft.com

web : www.marshallsoft.com

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

WSC4C Reference Manual                                                Page 1

C O N T E N T S

Chapter                                     Page

Table of Contents.............................2

List of Functions.............................3

SioBaud....................................3

SioBrkSig..................................3

SioCTS.....................................4

SioDCD.....................................4

SioDone....................................5

SioDSR.....................................5

SioDTR.....................................6

SioFlow....................................6

SioGetc....................................7

SioGets....................................7

SioInfo....................................8

SioParms...................................8

SioPutc....................................9

SioPuts....................................9

SioReset..................................10

SioRI.....................................10

SioRTS....................................11

SioRxClear................................11

SioRxQue..................................12

SioStatus.................................12

SioTxClear................................13

SioTxQue..................................13

SioUnGetc.................................14

Function Summary.............................14

Error Code Summary...........................15

WSC4C Reference Manual                                                Page 2

+-------------+------------------------------------------------------------+

|   SioBaud   |  Sets the baud rate of the selected port.                  |

+-------------+------------------------------------------------------------+

Syntax    int SioBaud(int Port,int BaudCode)

// Port     : Port selected.

// BaudCode : Baud code or actual baud rate.

Remarks    The  SioBaud  function sets the baud rate without resetting  the

port. It is used to change the baud rate after calling SioReset.

SioBaud  may be called with either the actual baud rate value or

one of the baud rate codes as follows:

Code   Rate    Name             Code   Rate   Name

0     300    Baud300            5    9600   Baud9600

1     600    Baud600            6   19200   Baud19200

2    1200    Baud1200           7   38400   Baud38400

3    2400    Baud2400           8   57600   Baud57600

4    4800    Baud4800

Returns       IE_BADID : No such port.

IE_BAUDRATE : Unsupported baud rate.

See Also    SioReset

+-------------+------------------------------------------------------------+

|  SioBrkSig  |  Asserts, cancels, or detects BREAK signal.                |

+-------------+------------------------------------------------------------+

Syntax    int SioBrkSig(int Port, int Cmd)

// Port : Port selected.

// Cmd  : ASSERT, CANCEL, or DETECT

Remarks    The SioBrkSig function controls the BREAK bit in the line status

register. The legal commands are:

ASSERT_BREAK : 'A' to assert BREAK

CANCEL_BREAK : 'C' to cancel BREAK

DETECT_BREAK : 'D' to detect BREAK

ASSERT_BREAK, CANCEL_BREAK,  and  DETECT_BREAK  are  defined  in

WSC4C.H. See LOGIN.C for an example of the use of SioBrkSig.

Returns     IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

WSC_RANGE : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected (DETECT command only)

See Also    SioBrkKey

WSC4C Reference Manual                                                Page 3

+-------------+------------------------------------------------------------+

|   SioCTS    |  Reads the Clear to Send (CTS) modem status bit.           |

+-------------+------------------------------------------------------------+

Syntax    int SioCTS(int Port)

// Port : Port selected.

Remarks    The SioCTS function is used to detect if CTS (Cleat To Send) has

changed since the last call to SioCTS.

The  CTS  line  is  used  by  some  error  correcting  modems to

implement hardware flow control.  CTS is dropped by the modem to

signal the computer not to send data  and is  raised  to  signal

the computer to continue.

Refer to the User's Manual for a discussion of flow control.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

0 : CTS has not changed.

>0 : CTS has changed.

See Also    SioFlow, SioDSR, SioRI, SioDCD, and SioModem.

+-------------+------------------------------------------------------------+

|   SioDCD    |  Reads the Data Carrier Detect (DCD) modem staus bit.      |

+-------------+------------------------------------------------------------+

Syntax    int SioDCD(int Port)

// Port : Port selected.

Remarks    The  SioDCD  function is  used to read the Data Carrier Detect

(DCD) modem status bit. Also see SioModem.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

0 : DCD is clear.

>0 : DCD is set.

See Also    SioDSR, SioCTS, SioRI, and SioModem.

WSC4C Reference Manual                                                Page 4

+-------------+------------------------------------------------------------+

|   SioDone   |  Terminates further serial processing.                     |

+-------------+------------------------------------------------------------+

Syntax    int SioDone(int Port)

// Port : Port selected.

Remarks    The  SioDone  function  terminates  further  serial  processing,

allowing other applications to use the port.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

See Also    SioReset.

+-------------+------------------------------------------------------------+

|    SioDSR   |  Reads the Data Set Ready (DSR) modem status bit.          |

+-------------+------------------------------------------------------------+

Syntax    int SioDSR(int Port)

// Port : Port selected.

Remarks    The  SioDSR  function is used to detect if DSR (Data Set Ready)

has changed since the last call to SioDSR.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

0 : DSR has not changed.

>0 : DSR has changed.

See Also    SioCTS, SioRI, SioDCD, and SioModem

WSC4C Reference Manual                                                Page 5

+-------------+------------------------------------------------------------+

|    SioDTR   |  Set, clear, or read Data Terminal Ready (DTR) status bit. |

+-------------+------------------------------------------------------------+

Syntax    int SioDTR(int Port, int Cmd)

// Port : Port selected.

// Cmd  : DTR command (see below).

Remarks    The SioDTR function controls the Data Terminal Ready (DTR) bit

in the modem control register.  DTR should always  be  set  when

communicating with a modem.  Commands  (defined  in  WSC4C.H)

are:

SET_LINE   : 'S'  to set DTR (ON)

CLEAR_LINE : 'C'  to clear DTR (OFF)

READ_LINE  : 'R'  to read DTR

Returns    IE_NOPEN  : Port not opened. Call SioReset first.

IE_BADID  : No such port.

WSC_RANGE : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ_LINE Command).

>0 : DTR is ON (READ_LINE Command).

See Also    SioRTS.

+------------+-------------------------------------------------------------+

|   SioFlow  |  Sets hardware (RTS/CTS) flow control.                      |

+------------+-------------------------------------------------------------+

Syntax    int SioFlow(int Port,int Cmd)

// Port : Port selected.

// Cmd  : Class of flow control (see below).

Remarks    The SioFlow function is used to enable or disable hardware  flow

control.  Hardware flow control uses RTS and CTS to control data

flow  between  the  modem  and the computer.  To enable hardware

flow control, call SioFlow with Cms set to one of:

Cmd     Flow Control

'H'  Hardware (RTS/CTS) flow control.

'S'  Software (XON/XOFF) flow control.

'N'  No flow control.

In order for hardware flow control to work correctly, your modem

must also be configured to work with hardware flow control,  and

your  computer  to  modem  cable  must  have  RTS  and CTS wired

straight through. If you enable hardware flow  control,  do  not

modify  the  RTS  line  (by  calling  SioRTS).

Flow control is disabled after resetting a port.  To  explicitly

disable hardware flow control, call SioFlow with Tics=-1.

Returns     IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

WSC_RANGE : Cannot recognize Cmd.

-1  : Flow control disabled.

>0  : Flow control enabled.

See Also    SioPutc

WSC4C Reference Manual                                                Page 6

+------------+-------------------------------------------------------------+

|   SioGetc  |  Reads the next character from the serial line.             |

+------------+-------------------------------------------------------------+

Syntax    int SioGetc(int Port)

// Port : Port selected.

Remarks    The SioGetc function reads a byte from the selected serial port.

A WSC_NO_DATA (-100) is returned if no byte is available.

Returns    IE_NOPEN    : Port not opened. Call SioReset first.

IE_BADID    : No such port.

WSC_NO_DATA : If timed out.

>= 0 : Character read.

See Also    SioUnGetc and SioGets.

+------------+-------------------------------------------------------------+

|   SioGets  |  Reads the next byte buffer from the serial line.           |

+------------+-------------------------------------------------------------+

Syntax    int SioGetc(int Port, char *String, int Cnt)

// Port   : Port selected

// Buffer : Receive buffer

// Cnt    : Number bytes wanted

Remarks    The SioGets function reads the smaller of the  number  of  bytes

wanted  (Cnt)  and  the number of bytes in the recieve buffer. A

zero is return if no bytes are read.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

>= 0 : Number of characters actually read.

See Also    SioUnGetc and SioPutc.

WSC4C Reference Manual                                                Page 7

+-------------+-----------------------------------------------------------+*

|   SioInfo   |  Returns WSC4C library information.                       |

+-------------+-----------------------------------------------------------+*

Syntax    int SioInfo(int Cmd)

// Cmd : Command (See below)

Remarks    The  SioInfo  function returns an integer code  corresponding to

the Cmd as follows.

Command   SioInfo returns

'V'  Library version number.

Returns    See remarks above.

-1 : Cannot recognize Cmd.

+-------------+------------------------------------------------------------+

|  SioParms   |  Sets parity, stop bits, and word length.                  |

+-------------+------------------------------------------------------------+

Syntax     int SioParms(int Port, int Parity,

int StopBits, int DataBits)

// Port     : Port selected.

// Parity   : Parity code [0,1,2].

// StopBits : Stop bits code [0,1].

// DataBits : Word length code [0,1,2,3].

Remarks    The SioParms function sets  the  parity,  stop  bits,  and  word

length.   If  the  default parity (none), stop bits (1), or word

length (8) is not  acceptable,  then  they  can  be  changed  by

calling SioParms.  SioParms can be called either before or after

calling SioReset. See file WSC4C.H.

Use  the  constant  values defined in WSC4C.H to minimize the

chance of passing an incorrect parameter value.

Value   Description    WSC4C.H Name

ParityCode:    *0      no parity      NoParity

1      odd parity     OddParity

2      even parity    EvenParity

StopBitsCode:  *0      1 stop bit     OneStopBit

2      2 stop bits    TwoStopBits

WordLengthCode: 5      5 data bits    WordLength5

6      6 data bits    WordLength6

7      7 data bits    WordLength7

*8      8 data bits    WordLength8

Returns        IE_BADID : No such port.

IE_BYTESIZE : Word length not supported.

WSC_RANGE : Parameter out of range.

See Also    SioReset.

WSC4C Reference Manual                                                Page 8

+-------------+------------------------------------------------------------+

|   SioPutc   |  Transmit a character over a serial line.                  |

+-------------+------------------------------------------------------------+

Syntax    int SioPutc(int Port, int Ch)

// Port : Port selected.

// Ch   : Character to send.

Remarks    The SioPutc function copies the character to the transmit queue

for subsequent transmission.

Returns     IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

0 : No error.

See Also    SioGetc and SioFlow.

+-------------+------------------------------------------------------------+

|   SioPuts   |  Transmits a byte buffer over a serial line.               |

+-------------+------------------------------------------------------------+

Syntax    int SioPutc(int Port, char *String, int Cnt)

// Port   : Port selected.

// Buffer : String of bytes to transmit.

Remarks    The SioPuts function copies the characters to the transmit queue

for subsequent transmission.

Returns     IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

>= 0 : The number of bytes actually transmitted.

See Also    SioGetc and SioFlow.

WSC4C Reference Manual                                                Page 9

+-------------+------------------------------------------------------------+

|  SioReset   |  Initialize a serial port for processing.                  |

+-------------+------------------------------------------------------------+

Syntax    int SioReset(int Port, int RxQueSize, int TxQueSize)

// Port      : Port selected.

// RxQueSize : Receive queue size.

// TxQueSize : Transmit queue size.

Remarks    The SioReset function  initializes  the  selected  serial  port.

SioReset  should  be  called  before  making  any other calls to

WSC4C.  SioReset uses the parity, stop  bits,  and  word  length

value  previously  set  if  SioParm  was  called,  otherwise the

default values (see SioParm) are used.

Recall that COM1 and COM3 share the same  interrupt  vector  and

therefore  cannot  operate  simultaneously. Similiarly, COM2 and

COM4 cannot operate simultaneously. Any other combination of two

ports can be used.

Returns       IE_BADID : No such port.

IE_OPEN : Already open.

IE_MEMORY : Cannot allocate memory.

IE_HARDWARE : Hardware error.

See Also    SioBaud, SioParms, and SioDone.

+-------------+------------------------------------------------------------+

|    SioRI    |  Reads the Ring Indicator (RI) modem status bit.           |

+-------------+------------------------------------------------------------+

Syntax    int SioRI(int Port)

// Port : Port selected.

Remarks    The SioRI function is used to read the Ring  Indicator

(RI) modem status bit. Also see SioModem.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

0 : RI is clear.

>0 : RI is set (RING has occurred).

See Also    SioDSR, SioCTS, SioDCD, and SioModem.

WSC4C Reference Manual                                                Page 10

+-------------+------------------------------------------------------------+

|    SioRTS   |  Sets, clears, or reads the Request to Send (RTS) line.    |

+-------------+------------------------------------------------------------+

Syntax    int SioRTS(int Port, int Cmd)

// Port : Port selected.

// Cmd  : RTS command (SET, CLEAR, or READ)

Remarks    The SioRTS function controls the Request to Send (RTS bit in the

modem control register.

The  RTS  line  is  used  by  some  error  correcting  modems to

implement hardware flow control.  RTS is dropped by the computer

to signal the modem not to send data, and is  raised  to  signal

the modem to continue. RTS should be set when communicating with

a modem unless Flow Control is being used.

Refer  to  the  User's  Manual for a discussion of flow control.

Commands (defined in WSC4C.H) are:

SET_LINE     'S'   - set RTS (ON)

CLEAR_LINE   'C'   | clear RTS (OFF)

READ_LINE    'R'   - read RTS

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

WSC_RANGE: Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ_LINE Command).

>0 : RTS is ON  (READ_LINE Command).

See Also    SioFlow and SioDTR.

+------------+-------------------------------------------------------------+

| SioRxClear |  Clears the receive buffer.                                 |

+------------+-------------------------------------------------------------+

Syntax    int SioRxClear(int Port)

// Port : Port selected.

Remarks    The SioRxClear  function  will  delete  any  characters  in  the

receive  buffer  for  the  specified port.  After execution, the

receive buffer will be empty.  Call SioRxClear after resetting a

port in order to delete any spurious characters.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

See Also    SioRxQue.

WSC4C Reference Manual                                                Page 11

+------------+-------------------------------------------------------------+

| SioStatus  |  Returns the serial port status.                            |

+------------+-------------------------------------------------------------+

Syntax    int SioStatus(int Port, int Mask)

// Port : Port selected.

// Mask : Error mask.

Remarks     The  SioStatus  function  returns  the serial port error status

corresponding to the Mask argument.

WSC_RXOVER  : The receive queue overflowed.

WSC_OVERRUN : An  incoming  byte  was  overwritten.

WSC_PARITY : A parity error was detected (incoming byte)

WSC_FRAME : A framing error was detected (incoming byte)

WSC_BREAK : A break signal was detected.

WSC_TXFULL : The transmit queue is full.

For example, to test for receive queue overflow:

if(SioStatus(Port, WSC_RXOVER)) printf("RX Overflow");

The  nummerical  values  of  the  mask  constants are defined in

WSC4C.H.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

+-------------+------------------------------------------------------------+

|  SioRxQue   |  Returns the number of bytes in the receive queue.         |

+-------------+------------------------------------------------------------+

Syntax    int SioRxQue(int Port)

// Port : Port selected.

Remarks    The  SioRxQue  function  will return the number of characters in

the receive queue at the time of the call. It  can  be  used  to

implement XON/XOFF flow control.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

See Also     SioRxFlush.

WSC4C Reference Manual                                                Page 12

+------------+-------------------------------------------------------------+

| SioTxClear |  Clears the transmitter buffer.                             |

+------------+-------------------------------------------------------------+

Syntax    int SioTxClear(int Port)

// Port : Port selected.

Remarks    The SioTxClear  function  will  delete  any  characters  in  the

transmit  buffer  for the specified port, provided that you will

Once  this  function  is  called,  any character in the transmit

buffer (put there by SioPutc) will be  lost  and  therefore  not

transmitted.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

See Also    SioTxQue & SioTxFlush.

+------------+-------------------------------------------------------------+

|  SioTxQue  |  Returns the number of bytes in the transmit queue.         |

+------------+-------------------------------------------------------------+

Syntax    int SioTxQue(int Port)

// Port : Port selected.

Remarks    The SioTxQue function will return the number  of  characters  in

the  transmit  queue  at the time of the call.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

See Also     SioTxFlush.

WSC4C Reference Manual                                                Page 13

+------------+-----+-------------------------------------------------------+

| SioUnGetc  |  "Un|gets" the last character read with SioGetc().          |

+------------+-----+-------------------------------------------------------+

Syntax    int SioUnGetc(int Port, char Ch)

// Port : Port selected.

// Ch   : Character to unget.

Remarks    The SioUnGetc function returns  ("pushes")  the  character  back

into  the serial input buffer.  The character pushed will be the

next character returned by SioGetc.  Only one character  can  be

pushed back. This function works just like the "ungetc" function

in the C language.

Returns    IE_NOPEN : Port not opened. Call SioReset first.

IE_BADID : No such port.

See Also    SioReset.

WSC4C Reference Manual                                                Page 14

Function Summary

Refer to the WSC4C Reference Manual (WSC4C.REF) for detailed information  on

the  communications  and  support  functions.   A  one  line summary of each

function follows:

+---------------+-----------------------------------------------------------+

|  SioBaud      |  Sets the baud rate of the selected port.                 |

|  SioBrkSig    |  Asserts, cancels, or detects BREAK signal.               |

|  SioCTS       |  Reads the Clear to Send (CTS) modem status bit.          |

|  SioDCD       |  Reads the Data Carrier Detect (DCD) modem status bit.    |

|  SioDone      |  Terminates further serial processing.                    |

|  SioDSR       |  Reads the Data Set Ready (DSR) modem status bit.         |

|  SioDTR       |  Set, clear, or read the Data Terminal Ready (DTR) bit.   |

|  SioFlow      |  Enables / disables hardware flow control.                |

|  SioGetc      |  Reads the next character from the serial line.           |

|  SioGets      |  Reads a character string from the serial line.           |

|  SioInfo      |  Returns information such as library version.             |

|  SioParms     |  Sets parity, stop bits, and word length.                 |

|  SioPutc      |  Transmit a character over a serial line.                 |

|  SioPuts      |  Transmit a character string over a serial line.          |

|  SioReset     |  Initialize a serial port for processing.                 |

|  SioRI        |  Reads the Ring Indicator (RI) modem status bit.          |

|  SioRTS       |  Sets, clears, or reads the Request to Send (RTS) line.   |

|  SioRxClear   |  Clears the receive buffer.                               |

|  SioRxQue     |  Returns the number of characters in the receive queue.   |

|  SioStatus    |  Returns the serial line status.                          |

|  SioTxClear   |  Clears the transmit buffer.                              |

|  SioTxQue     |  Returns the number of characters in the transmit queue.  |

|  SioUnGetc    |  "Un-gets" (puts back) a specified character.             |

+---------------+-----+-----------------------------------------------------+

Error Code Summary

+---------------+---------------------------------------------+

|  WSC_NO_DATA  |  No incoming serial data is available.      |

|  WSC_RANGE    |  A parameter is out of range.               |

|  WSC_ABORTED  |  The shareware version of WSC has expired.  |

+---------------+---------------------------------------------+

|  IE_BADID     |  No such port.                              |

|  IE_OPEN      |  Port already opened.                       |

|  IE_NOPEN     |  Port not opened. Call SioReset first.      |

|  IE_MEMORY    |  Cannot allocate memory for queues.         |

|  IE_DEFAULT   |  Error in default parameters.               |

|  IE_HARDWARE  |  Hardware not present.                      |

|  IE_BYTESIZE  |  Unsupported byte size.                     |

|  IE_BAUDRATE  |  Unsupported baud rate.                     |

+---------------+---------------------------------------------+

WSC4C Reference Manual                                                Page 15