XE 8  EDITOR

by

Nigel Stanley

NB This is a shareware program for the Atari ST with mono monitor.

It may be freely copied and distributed but users must register

with the author. This costs 8 or 15$. It should be sent to Nigel

Stanley, 96 Geere Rd. London E15 3PW. If you also send a blank

disk I will be happy to send you the GFA Basic listing. The author

takes no responsibility for any damage use of this program may

cause to any equipment.

To get going you need this text file (XE8.DOC), the program it-

self, XE8.PRG, which should be in the root directory, a folder

called XE8 containing a file called XE8.SCR. You may also find a

CHANNEL.DAT file, and various files ending in either a VCE or SND

suffix, within the XE8.

This manual assumes a basic knowledge of using the Atari ST, of

MIDI and of Akai's XE 8 drum sample player.

INTRODUCTION

Akai's XE8 Midi Drum Expander is a drum machine with a difference.

Unlike most drum machines it has no facility for playing back the

sounds it contains. It must therefore be used with a sequencer or

other Midi controller. Its strength is that it contains, with its

expansion cards, 3 megabytes of high quality 16-bit drum samples.

These basic sounds can be manipulated in various ways giving so-

mething of the flexibility of a sampler. Although it's a high

quality piece of equipment, it seems to have failed to catch on

and has been heavily discounted. This means a new model can be

picked up for less than 150 which is excellent value for money.

Getting at the parameters that provide this degree of control is

relatively straightforward from the front panel, but as with all

such devices, a great deal of button pushing is required. The ad-

vantages of an editing program are therefore clear. A few opera-

tions with the mouse substitute for much button pushing, and the

values of all the parameters can be seen at a glance.

STARTING OFF

You will obviously need to connect the ST midi-ports to the XE8.

As an XE8 is almost certainly part of more extensive Midi set up,

this manual assumes you know how to do this. The memory protect on

the XE8 need not be changed to 'off', as the memory protect seems

not to affect changes secured via Midi.

You will also need to set the Midi channel on the XE8 to 16 (the

XE8 manual explains this) unless there is a CHANNEL.DAT file on

the disk. This keeps a record of which Midi channel the program

last used. If it's not there the program assumes you share the

author's preference for drum machines to be on channel 16. A first

time user is probably best advised to delete any CHANNEL.DAT file

on the disk and start off with the XE8 set to Midi channel 16. It

is a simple process to write a new channel.dat later if you are

used to using another Midi channel for drums.

Double clicking on the XE8 program icon will first of all bring up

the title screen, which will flash prettily at you. A click with a

mouse button or a touch on the keyboard will get rid of the

flashing and allow you to choose whether to load the initial pro-

gram from disk or the XE8 itself, by clicking on the option

required.  Unfortunately program has two meanings in this manual.

Not only does it have its obvious use, but Akai use the term to

signify one of the 32 banks of 16 drum sounds the XE8 contains.

If this is the first time you are using this program, you will be

best advised to load a program from the XE8. Loading a program

from disk is dealt with below. You can choose which of the 32

programs by pointing and clicking the mouse on the program number

displayed. As with all change to parameters in the program, the

left button causes the value to increase and the right decrease.

ck this box you will then be asked to choose which program you

wish to load into the editor. Once you have done this hit return

or point and click with the mouse on the OK button. The program

will then load from the XE8. This takes some time, and the screen

with stripe across to let you know something is happening.

If this fails and a message telling that nothing has been received

the program goes straight to the main edit screen. The failure

will mean that the midi leads are not corrected properly or that

the program and the XE 8 are not using the same Midi channel.

CHANGING THE MIDI CHANNEL

Select System from the GEM Menu bar and then Midi channel from the

pop-up menu. The program's message box will then appear and invite

you to select a Midi channel. This must be between 1 and 16. Press

return. The message box will disappear and you will hear the ST

writing a new CHANNEL.DAT file. As long as you keep this file with

the main program in a folder called XE8 every time you boot up the

program it will start with your chosen channel. It can be changed

at any time.

PROGRAM NUMBER

There are two important features to the program. These are the

buttons at the top left hand corner of the screen - 'PROG NUMBER'

and 'SOUND NUMBER'. It's vital to understand how these features

work to get a grasp of how the program operates.The editor only

deals with a single program of 16 drum sounds at a time. Putting

the mouse pointer over the program number, and pressing a mouse

button will change the number, (with the left button increasing

the number and the right button decreasing it)

.

But although this box can be used to change the program number, it

does not in any way alter the sounds. If for example you loaded

program 11 from the XE8 when you booted up the editor, and you now

change the program number to 12, this does not mean that you now

have the data from the XE8's program 12 in the editor. It simply

means that if you were now to load this program back into the XE8

it would send the data for program 11 into program 12. In other

words you have copied program 11 to program 12 so that they are

now both the same. You will also have lost program 12, as this has

now been overwritten by program 11. So beware!

In other words the program number only tells you which of the 32

memory slots in the XE8 the editor is communicating with. Select-

ing a program number is only a first step - a preliminary to

choosing, by means explained below, to either load a program from

the XE8 to the editor, thus losing any information already held in

the editor, or to overwrite an XE8 program by the one currently

held in the editor.

If you set the XE8 to display the program number (by moving the

Play/Edit front panel switch to 'Program') you will see as you

change the program number with the editor the XE8 follows. This is

because the editor sends a program change Midi message to the XE8

each time the program number is changed within the editor.

SOUND NUMBER

The sound number parameter is changed in the same way as the pro-

gram number, ie by clicking with a mouse button over the number.

If you change the sound number, you will see the screen rewrite

itself to display the parameters of the sound you have chosen. XE

8 programs consist of 16 sounds.

EDITING THE PARAMETERS

Editing the parameters of each sound is achieved by clicking with

the mouse and should be fairly intuitive, especially if you have

ever used another editor.

For example to change the sample that is being sounded, point the

mouse at the box containing the drum sound name, eg 'Kick 0' and

press the left button. You will see the editor cycle through the

drum sounds available, until you get the highest numbered sound on

the second card. If you then press the right mouse button, it will

cycle backwards until it reaches the lowest numbered sound in the

internal memory.

This procedure is used to alter all the parameters, whether re-

prepsented by text, symbol or number. Although the envelope and

pitch warp parameters are shown graphically they are altered from

the parameter number box, not by dragging parts of the graph as on

some other editors.

WARNING - BUG ALERT

The only special cases are the high and low note boxes. At least

on my XE8 it is not possible to alter these parameters via midi.

Although the display on the XE8 will change, it does not actually

alter the parameters. This has to be done manually from the XE8

front panel. This is very irritating but is down to a bug in the

operating system of the XE8, not this program.

The output channel and mode box also requires a word of explana-

tion. As the XE8 manual explains there are two output modes. Mode

1 sends a sound as it is played to the lowest numbered free output

channel of the 8 available. Mode 2 allows you to choose which of

the 8 channels each sound is assigned to. If you select Mode 1 the

channel select box is 'blanked off' as altering this parameter now

makes no difference. In mode 2 the channel numbers reappear as the

sound now goes to the channel selected, cutting off any sound al-

ready being output through that channel.

You will notice each time you change a parameter, the XE8 will

play a note. It sends a midi note-on message with velocity of 100

and the pitch you have chosen for the midi-note low, unless you

are altering the midi-note high in which case it send this value.

PLAYING NOTES

Although the editor sends a single note-on message each time you

alter most of the parameters, there are three more sophisticated

ways you can hear the sounds you are editing.

First if you put the mouse pointer over the 'Voice' circle and

press a button.

Secondly you can select the 'Program' option in the same way. This

cycles through all the sounds you have created from the highest

'midi-note high' in your program to the lowest 'midi-note low' in

semi-tone and velocity steps.

The third way of playing sounds is by means of a midi controller

connected to the Midi-in socket of the Atari. To select the Midid

through mode press both mouse buttons at any time and the program

will go into thru mode.

You should now be able to play the XE8 from a controller, such as

a keyboard, set to the same channel as the XE8.

Disk Storage

You can save whole programs or single sounds to disk and load them

again with the editor. To save a program to disk select the 'pro-

gram save' option from the file menu. You can give it any name

normally allowed by the Atari but programs must have a .SND suf-

fix.

Loading a program from disk is just as easy. Select the 'Program

load' option from the file menu and then choose the program you

want from the file selector box.

Saving and loading individual sounds is done in the same way,

using the voice save and load options from the file menu, except

that sounds are given a .VCE suffix. The sound currently displayed

is the one that is saved, and when you load a sound from disk it

will replace the sound currently displayed on the screen AND au-

tomatically load into the XE8, thus erasing the sound previously

selected. Be careful!

PRINTING

If you select the print option from the file menu, the editor will

print out the most important parameters for each sound within your

program, namely the sample, the output channel and the low and

high notes for each sound. This can be extremely useful when

you're programming a drum part into a sequencer.

LOWEST NOTE

The "Lowest Note" option in the System Menu allows you to choose

the lowest note that can be selected by the lowest note parameter.

the default value for this is Midi note 36 or C3, as that is the

lowest note that many keyboards can use.

Desk Accessories

GFA BASIC V2.0 is not at its best with desk accessories. The pro-

gram will work with them, but be careful to save all work before

activating a DA, as the ATARI may crash or freeze up. After you

have finished with the DA, you may find it has punched a hole in

your screen. You can get rid of this by selecting the rewrite op-

tion from the system menu.

HINTS AND TIPS

The XE8 is a relatively straightforward device to use, and there

are no arcane mysteries that I have discovered to pass on. Here

however are a few ideas that might not have occured to you.

Using more than one sound.

You can set more than one sound to be triggered by the same note.

Although you probably don't want a snare to sound every time you

hit a hi-hat, you can make some interesting compound sounds by

using two similar samples or even the same sound slighly detuned

or swept differently. Another possibility is to beef up the attack

of a sound by adding just the attack of a second sample, by

chopping it off using the envelope controls.

Humanising your drums

Although the XE8 contains high quality and realistic samples, it

will still sound like a drum machine as each sound will always

sound exactly the same. This may be what you want, but in real

life hitting a drum with a slightly different velocity or in a

different place will change the way it sounds in a subtle way that

tells you a human being is playing, not a micro-chip. But parti-

cularly by using the ability to modulate the XE8's parameters by

velocity with low levels of modulation depth set, quite subtle

changes to the sound can be achieved if you ensure that your

sequencer is sending slighly different velocities each time.

Special Effects

Similar techniques can be used to get some fairly weird effects.

And if you have a drum machine, rather than a live drummer in your

spare bedroom, why not use it to do things you can never get a

real drummer to do? (other than staying sober). If you set high

levels of modulation depth by either key or velocity the same drum

sound can be made to vary dramatically. If you now record a one

bar or two bar pattern into your sequencer and then copy it a

number of time you can achieve some interesting cyclical rhythms.

Drum sounds that sound individually somewhat odd can sound effec-

tive in a pattern.

Sounds that pan across the stereo field can be achieved by putting

the same sample played forwards through one channel panned hard

one way and backwards through a channel panned hard the other way.

You will need to use the envelope controls to get a realistic

sweep effect and you may want to cut out the second attack. The

two sounds can either be triggered by the same note, or a sequen-

cer can be used to copy the notes triggering the forwards sound to

another track, transposed to the note that triggers the backwards

sound and the timing can then be varied, by moving the second

track either forwards or backwards in time, to get the rhythmic

effect required.

Building a kit

It's probably a good idea to use the same notes for similar sounds

in each program. That means that you can use the same sequence to

audition different programs from your disk or the XE8's memories,

choose the one nearest to what you want and then individually

tweak each sound.

To do this effectively you will need to spread your sounds across

the available midi notes, leaving gaps between each sound so that

you can set some programs up to modulate by key.