This is NOT a id software product!   Do not call id software for support!

---------------------------------------------------------------------------

This is a hack of id's serial device driver.  It allows arbitrary baudrate,

hardware handshaking, arbitrary irq, and arbitrary port addresses.  This will

allow you to run your V32bis modem at higher than 9600 baud.  Also, some

problems with dialing and answering the modem have been fixed (some modems

can not accept command strings at full speed).  The new structure of the

modem.cfg file is described in the accompanying modem.cfg file.

*Important: disable MNP5 compression, but leave V42bis running.  Check your

modem manual for the required init string.  With a 8250, do not lock the baud

rate higher than 19200 or doom will grind to a halt.

Many portions of the code were rewritten with optimization in mind.  Two

machines with 8250s running a null-modem and a 19200 baud rate will yield

a fast game.  The routine also provides diagnostics at the exit of doom which

indicate how many characters have been sent and how many have been received, as

well as how many TX and RX interrupts have occurred.  With a 16550, the TX and

RX FIFO utilization statistics should be around 9-11%, if not something may

be wrong with your 16550.  For anyone interested, ID had a crucial bug in their

code, they did not initialize the outque and inque pointers; this would cause

random slowdowns.

To use this fix, rename the original sersetup.exe in your doom 1.2 directory.

Copy the new file into your doom 1.2 directory, along with the sample modem.cfg.

Use as you would the normal setup.exe to launch a modem game. Play doom!

Modified from original ID code by:

Tony da Costa, U.B.C. Electrical Engineers, (cdacosta@ee.ubc.ca)

March 2, 1994

If you would like to see some other controls added to the modem.cfg file, drop

me a line at my Internet address.

-----------------------------------------------------------------------------

Command Lines for serial Setup:

-DIAL xxx-xxxx = This will cause Serial setup to Dial the number xxx-xxxx

-ANSWER        = This will cause Serial setup to answer.

-PLAYER1       = Use this for the slowest machine if you have problems.

-VECTOR 0x??   = Use this vector, Doom will automaticly search 0x60-0x66 for

= a unused vector.  If it can't find one it will quit.

= Use this to override, or if it reports that it can't find any

= free vectors.

-COMx          = Use COMx for the Com port  (Com1, Com2, Com3, Com4)

-PORT          = Use this Port if your Com port is non Standard.  Use -IRQ also.

-IRQ           = Use this if you IRQ is non standard. Use -PORT also.

-SKILL x       = Use Skill level (1-5)

-DEATHMATCH    = Use if you want to play a Deathmatch game. (Coop is Default)

-EPISODE x     = Use to play a Episode (1-3)

Hope this helps all of you out there.

Remember, Turn OFF MNP5 on your modem. This will cause the game to be SLOW!