������������������������������������������������������������������������������Ŀ

��Ŀ  ���Ŀ ��Ŀ � ���Ŀ ���Ŀ ����Ŀ ڿ  �

���� ���   ô � � ���   ���Ĵ   ô   ���Ĵ

����� ����� �� ��� ����� ��  �   ��   ��  �

����Ŀ ڿ  � ���Ŀ

ô   ���Ĵ ���

��   ��  � �����

���Ŀ ���Ŀ ����Ŀ ���Ŀ ڿ ��Ŀ � ���Ŀ

����� ���   ô � � ���Ĵ ô ô � � ���¿

�� �  ����� �� � � ��  � �� �� ��� �����

Magazine

This phile dated 06/22/1994

Volume 3, Issue 4

You can always find BTR on PsychoTron BBS

��������������������������������������������������������������������������������

I found this and just put it in, it's good.

-------------------------------------------

You remember the little "bug" installed on the bad guys car in the James Bond

flicks that allowed Bond to follow the car from a distance? Well this file is

a tutorial on them.

First, they do exist, I've built my own, but even the best commercial units

intended for law enforcement purposes wont do what the Bond model purports

to do, that is, give a printout on a moving map showing the route driven by

the bug toter.

The basics of the unit are the transmitter, which is about the size of

a pack of cigarettes and is held on via a magnet. And a receiver, using 2

identical antennas, coupled to a center zero meter which gives a heading

towards the transmitter. More on these later.

The receiver/display unit is used by pilots, amateur radio operators, and

law enforcement and security personnel to track the movements of the

transmitter

usually at short ranges, the civil air patrol uses these units to find

downed aircraft by tracking the emergency beacon, activated by the

impact of the crash. Amateurs play "hunt the fox" to keep illegal transmitters

out of the ham bands. Law enforcement personnel track the movement of people,

drugs, and weapons by attaching a transmitter to the object (or suspect's

car) to be followed.

The transmitter is usually a small VHF or UHF battery operated package

dangling a 19" flexible antenna (about the thickness of piano wire). The

transmitter does not "beep" per say, but transmits a continual carrier.

The FBI uses 167.xxx mhz for theirs and the local DA uses the intercounty

police freq of 155.37. I have seen military models that use 149.xxx mhz

around here (air force).

Now the receiver:

Two identical antennas mounted on the chase vehicle (usually magnetic mounts)

feed a pair of PIN diodes that feed a phase detector which samples the

receiver's IF output. When the received signal is directly in front of you,

signals arrive at exactly the same time at each antenna. This is calibrated to

read center 0 on the meter. (Incidentally the unit can't tell if the signal is

in front or in back of you, so the need to make sure you follow the subject

reasonably closely is apparent). If the bug travels say to 10 o'clock on the

compass rose, the needle will swing to 4 o'clock on the meter. The object here

is to always drive towards zero and you follow the bug in the most direct

direction. With a little practice,you can follow a subject on an adjcent

street without loosing him.

The meter swings because the signal arrives later at one antenna than the

other, causing a voltage change in the phase detector (an Exar Radio-teletype

decoder chip in my model).

Some recent units ive seen have Light emitting diodes in a 360 degree circle

and use 4 antennas. This gives you full circle detection capabilitys as the

phase between pairs of antennas is calculated also.

Now, prevention:

The easiest way to detect if you've been planted with one of these little

transmitters, is to walk around the car or whatever with a portable frequency

counter and check for an alien RF signal. This is also the recommended

method to de-bug your home. A small freq counter with 1.2 ghz capabilitys

sells for around $100. today. If you do find a transmitter, have fun with it.

Stick it on a train heading out of town, a Greyhound bus, or a over the road

tractor-trailer rig....my favorite is to stick it on one of their own

vehicles and watch them chase themselves....hehehe.

-+\+-

A BTR production

All rights lost.

-rk