ZIPDISK.COM is a command processor that will  make  the  diskette

drives on an IBM PC work faster and more quietly.

PC-DOS uses a diskette head settling time which is longer than it

needs to be for the diskette drives used in a PC.  It is designed

that  way  because  the  drives  used  in  a PCjr (and some other

drives) require a long head  settling  time.  When  the  diskette

drive seeks from track to track,  it must pause at each track for

this time.

ZIPDISK.COM replaces the value for head settling time in a  table

in  low storage.  The table is called the DISK BASE.  It consumes

no RAM.

ZIPDISK.COM  should  be  executed  only  once,  usually  in  your

AUTOEXEC.BAT  file,  but  alternatively  by  entering the command

ZIPDISK.

After you enter the  ZIP  command,  you  will  notice  that  your

diskette  drives  make  much less noise,  and careful measurement

will reveal that disk accesses are speeded up.