This page covers notes for PrintCAD 1.14 that  are not included in the

v1.10 manual.

/Q sets the  number of  copies  of each printed page  (default /Q1).

Page  printers  produce  multiple copies faster using the "N copies"

prefix code (see Useful Prefix and Suffix Codes).

/DX1,..,/DX4 provide xon/xoff serial port drive.

/DH1,..,/DH4 provide hardware handshaking serial port drive.

The Canon  BJ130  driver (/FJ) is faster, but sets the printer to 10

pitch text.

/FR drives the Tektronix ColorQuick.

You can specify the nominal resolution of  PostScript output with  a

prefix code  that  includes  the dots/inch  followed by '  XX'.  For

example  /J 54 48 48 32 88  88 sets 600 dpi.  The actual  resolution

should be set to match, for example /FS 600,600. You can specify the

number of  copies similarly with  the XC macro, for example /J 50 32

88 67 makes 2 (ASCII 050) copies.

The shading patterns are revised, but have the same coverage.

Forty pens are allowed instead of fifteen.

Color dot matrix output uses true black instead of CMY black.

/FZi,j outputs PCX format at ixj dots/inch.

PrintCAD  only beeps  on the first  data sent following a file open.

/XS turns on the old sound mode.