PrintCAD 1.14

resident ADI/HP-GL pen plotter emulator

for use as a printer driver

with MS-DOS CAD packages

including AutoCAD and Microstation

(C) Copyright Ravitz Software Inc. 1991,1992

Ravitz Software Inc.                    BBS/FAX 606-268-0577

P.O. Box 25068

Lexington, KY  40524-5068

USA                          Compuserve 70431,32 Cary Ravitz

p-1

Contents -------------------------------------------------------------

Introduction ..................................................... 2

License and Registration                                           3

What Is New ...................................................... 3

Operation                                                          4

Installation ..................................................... 5

Suggested Startup .BAT Files                                       5

Syntax ........................................................... 6

Overview of Options                                                7

/1, /2, /3, /4, /5 - Alternate Configurations .................... 7

/A - Location Option                                               7

/B - Tiling Option ............................................... 8

/C - Pen Color Option                                              8

/D - Output Destination Option ................................... 9

/F - Output Format Option                                         10

/J, /K - Printer Code Prefix, Suffix Options .................... 12

/L - Page Layout Option                                           13

/M - Magnification Option ....................................... 14

/O - Origin and Orientation Option                                14

/P - Intercept Mask Option ...................................... 14

/Q - Number of Copies Option                                      16

/R - Internal Resolution Option ................................. 15

/S - Pen Shading Option                                           15

/T - Temporary File Option ...................................... 15

/W - Pen Width Option                                             16

/X - Control Option ............................................. 16

/Z - Chord Angle and Enhanced Font Option                         16

Large Paper ..................................................... 17

Answers                                                           18

Useful Prefix and Suffix Codes .................................. 19

Printers                                                          20

PCX and Fax Output .............................................. 24

Compatibility and Technical Information                           24

User Support .................................................... 25

p-2

Introduction ---------------------------------------------------------

PrintCAD is a pen plotter emulator that acts as a printer driver for

most IBM/DOS based CAD packages. It lets you print drawings directly

from CAD programs, via the CAD program's pen plotter driver - either

AutoCAD's ADI plotter driver or an HP-GL driver.

PrintCAD works with AutoCAD, AutoSketch, Microstation, Generic CADD,

Draft Choice, PC-Draft-CAD, and other CAD programs. It will print on

Epson and IBM 9 pin printers, Epson, Fujitsu, IBM, NEC, and  Toshiba

24  pin  printers, HP  LaserJet, DeskJet,  DeskJet  500C,  PaintJet,

QuietJet,  and plotters,  Canon  LBP,  BJ,  and  BJC  printers,  IBM

LaserPrinter,  ExecJet,  and Quietwriter  2  and 3, Kodak  Color  4,

Tektronix  ColorQuick, and PostScript printers, and it will generate

PCX files.

PrintCAD outperforms most  native CAD printer  drivers and  provides

additional  flexibility, such as pen width, shading, and color (with

color  mixing), and  paper  size,  orientation,  magnification,  and

positioning.  PrintCAD  uses the best graphics modes  available  for

each printer that it supports.

PrintCAD  is a swapping resident program that uses 5K of base memory

and  400K of EMS or XMS memory.  Either 400K of EMS or XMS memory is

required.  EMS and  XMS are expanded and extended  memory management

protocols that are typically set  up with memory management software

such as HIMEM/EMM386, QEMM386, or  386MAX.  If you have  a  computer

with extended  or expanded memory hardware, you need to install such

a driver to use it as EMS or XMS memory.

PrintCAD  automatically loads high if a 5K DOS  or XMS  upper memory

block is available. You do not need to use LOADHI or LOADHIGH.

For  use with DOS extender based  programs such as  AutoCAD  386 and

Microstation  4, a VCPI handler  needs to be present.  VCPI handlers

are built into MS-DOS  5's  EMM386.EXE and  recent  versions of QEMM

386, 386MAX, and other 386 memory managers.  If PrintCAD notes "VCPI

not  present"  then you  need  to  upgrade  your  memory  management

software.

PrintCAD has been  tested  in  a  variety of environments.  However,

because  it is a  swapping resident program, unusual  situations may

cause it to crash  the  system.  Please  test it thoroughly on  your

system  before  using it with unsaved data.  PrintCAD  has not  been

tested under Microsoft Windows, and you should not expect it to work

in this environment.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintCAD is  provided  as is.  There are  no warranties expressed or

implied.  PrintCAD  is  a  swapping resident  program that may  have

problems with Microsoft Windows and unusual DOS environments.

PrintCAD  is  copyrighted  by  Ravitz Software  Inc.  You  may  copy

PrintCAD only for use under this license agreement.

PrintCAD is distributed as shareware.  You may use  PrintCAD without

charge on a trial basis to determine its suitability for you. If you

continue to use  it  after  your evaluation,  a $49 registration  is

requested.  This  registration  covers  use by  a  single person (on

multiple computers)  or installation  on  a single  computer (may be

used by a group of people).  There is no  cost to  use a later  1.**

version after registering any 1.** version.

You may distribute PrintCAD as a stand alone product if you keep the

entire   package  together,  unchanged,   including   this   license

explanation, and do not charge more than $10. You may not distribute

PrintCAD as part of another product.

To  register your copy of PrintCAD, send your name, address, and $49

check to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You  will be sent a printed manual and  current disk (specify 3.5 or

5.25).  If you previously  registered PrintCAD, you may send  $15 to

get a current disk and manual.

Multiuser licenses are $49 for the first copy (with disk and manual)

and thereafter $40  per  copy  (with  disk and  manual) or  $25  per

license (without disk or manual).

For users outside the USA, please make checks payable in US dollars,

and except  for  Canada,  please  use  a  check  from  a US bank  if

possible. Don't forget to include your country in your address.

If you register through a business, be sure to include your name.

What Is New ----------------------------------------------------------

v 1.14 - handles 40 pens, color dot matrix output uses true black,

PCX output, quiet (/XS turns on sound mode), multiple

copies (/Q)

v 1.12 - Tektronix ColorQuick support, revised shading patterns

p-4

Operation ------------------------------------------------------------

When PrintCAD  is run  for the first  time,  it installs itself  and

swaps most  of  its  code  to EMS (expanded memory) or XMS (extended

memory). Subsequent runs look for the resident copy and send control

information to it, updating the parameters.

When resident, PrintCAD intercepts calls to DOS that the CAD package

makes to output  HP-GL.  This data is used to drive the printer.  To

make this work, you must tell  PrintCAD how  to recognize that HP-GL

data is being output from your CAD packages.  Choose a filename mask

that you can match with your  CAD  package, such as *.PLT.  Set this

with the /P option (/P*.PLT).  Inside your CAD package, output HP-GL

(HP 7475, HP 7440/17440, or HP 7550) to a file name that matches the

mask. PrintCAD intercepts the data and prints the picture.

For  AutoCAD and  AutoSketch, you can use  ADI for the filename mask

(/PADI).  Configure AutoCAD for an ADI plotter on interrupt hex 78 -

up to 40 pens, 1016 dots/inch.  Then all output  to this driver goes

to  PrintCAD  via interrupt  78.  You can  also  use AutoCAD's HP-GL

driver - setup is the same as noted in the previous paragraph.

After making PrintCAD resident, you can verify that it is functional

by copying an HP-GL file to "pcad" with the DOS COPY command.

For color output, the CAD package should be configured for a plotter

with multiple pens  (up to 40),  the CAD  package's  pen  assignment

table  should be  set  up match  screen colors to  pen  numbers, and

PrintCAD's /C option should set up to match.

PrintCAD beeps  the first time  that  it is sent  data  after a file

open.  If you turn on sound (/XS), PrintCAD causes a high beep after

swapping in its code and a low  beep before swapping it out,  to let

you know that it is processing.

PrintCAD can be used with DESQview (DV). If you load PrintCAD inside

a  DV window, it will only be visible from that  window.  No special

steps are needed for this.  If you load PrintCAD  before running DV,

there may be problems, depending on your configuration.  If you have

problems, instead of running PCAD  to  install PrintCAD, run PCADDV.

This  loads PrintCAD at the top  of lower memory, preventing it from

overwriting DV  code.  You  must  have  a 5K upper memory  block for

PCADDV  to  work.  Do not use the /XL option with PCADDV as  it will

cause PCAD's resident block to split lower memory.

Internally PrintCAD uses two  byte coordinates  on a 1016  dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger  than  an HP  7475  can handle.  However, if  you  are  using

PrintCAD to emulate  a large plotter, you may  overflow the internal

coordinate system.  This condition is not checked, and it will cause

garbage output.  You can change the  internal resolution to 508 dots

per  inch (giving  60 inch maximum  plot sizes) with the  /R  option

(/R508).  This only helps  store the original  plot internally.  The

next  step in PrintCAD converts these  coordinates  to the requested

size in the resolution  of the output device,  and these coordinates

must also fit in the two byte coordinate list.

p-5

Installation ---------------------------------------------------------

The PrintCAD package includes these files:

PCAD.EXE  program          PCADDV.EXE  special loader for DESQview

PCAD.DOC  documentation    README.*    quick information

To install PrintCAD copy the distribution files to your working disk

or directory.

PCAD.EXE  is  a stand  alone executable  file.  There  are no  extra

printer driver files or overlays.

PCAD.DOC is this manual. You can print it with "COPY PCAD.DOC PRN".

PCADDV.EXE is a loader for setting up PCAD underneath DESQview.

The  README.*  files are not needed to  run  PrintCAD, but they must

accompany the rest of the files if you distribute the package.

Suggested Startup .BAT Files -----------------------------------------

Below  are some sample .BAT files for various situations.  Such .BAT

files may be used  to start PrintCAD or  change its parameters.  You

should not start PrintCAD from under a DOS shell, but you can change

parameters in this situation.

AutoCAD or AutoSketch, DeskJet or LaserJet IIP/III, 1/1 prints

PCAD /padi/fi

AutoCAD or AutoSketch, DeskJet 500C, 1/1 prints

PCAD /padi/fa/crgbcmyk

In AutoCAD configure  a  seven pen ADI  plotter and set up the pen

assignment table so that pens  1..7  have colors red, green, blue,

cyan, magenta, yellow, black.

AutoCAD, DeskJet (printer 1), PaintJet (LPT2), plotter (COM1)

PCAD /p#prn.plt/fi

PCAD /2/fp/ckrgbcmyk/dlpt2

PCAD /3/f8/dcom1

From AutoCAD plot to 1prn, 2prn, or 3prn.

Microstation, Canon BJ330, 1/1 print, 11x17 paper

PCAD /p*.000/f7/l10,16,.25,.25/j27 67 0 17

PC-Draft-CAD, LaserJet II, legal paper, best fit

PCAD /xa/p*.plt/mf1/fl/j27 38 108 51 65/l7.97,13.25,0,0

(/XA and/or /XL may be needed for use with PC-Draft-CAD.)

p-6

Syntax ---------------------------------------------------------------

The calling syntax of PrintCAD is:

PCAD /option1 /option2 ...

Spaces between parameters are not required.

Except for the /X and  /P options, if a given option is entered more

than once, only the last entry is used.  And if an option is invalid

(but not incorrect), it is ignored. Only one /X and one /P option is

allowed.

If you do  not enter any  parameters, PrintCAD  displays  its option

reminder screen.

Below, [  and ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that  multiple  numeric suboptions are  ordered  and numeric

suboptions must follow character suboptions where the characters may

be digits.  "c"  refers to  a suboption character,  "x" and  "y"  to

suboption  numbers,  "i"  and "j" to suboption  integers, "b"  to  a

suboption byte, and "f" to a DOS file or device name.

For numbers (x, y), a decimal point  is allowed but is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For  integer  input (i, j),  only  base ten  integers from -32768 to

32767 are allowed. "+" is not allowed. A byte (b) must be a base ten

integer from 0 to  255 or a hexadecimal  number prefixed with $ from

$00 to $FF.  Further range checking is done for most options to flag

unreasonable input.  If an option  has multiple  numeric suboptions,

these may be separated with commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

p-7

Overview of Options --------------------------------------------------

PrintCAD's options fall into several categories.  The first is input

and output  choices.  The input option  (/Pmask) lets you  choose an

intercept filename mask, or you may select ADI interrupt 78 by using

"adi". The output options are /F, the output format (printer model),

and /D, the destination port, device, or file.

The second category is  the page layout.  /L  controls the size  and

location of  the print  window (hard  clip limits) and the option to

box the window. Tiling is set with /B and copies with /Q.

The third category defines  how the  plot  looks  within  the  print

window.  /M,  the  magnification,  sets  the size  of  the plot.  /A

determines the location of the plot within the print window. /O sets

the  origin  and  orientation, and  is used to rotate or  mirror the

plot.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

The fifth category determines how PrintCAD processes the plot.  This

includes the temporary file /T, the default and minimum chord  angle

/Z, the internal resolution /R,  and prefix and suffix printer codes

/J and /K.

/1, /2, /3, /4, and /5 let you set up multiple configurations.

/1, /2, /3, /4, /5 - Alternate Configurations ------------------------

These options select up to 5 alternate configurations, labeled 1..5.

/1 is  the  default.  The configuration used  for printing is chosen

based on the file name and mask (/P option).

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /A

The A option determines the location of the plot in the print window

by specifying a point on the plot that is to be aligned with a point

in the print window.  You can specify the  plot alignment point with

x,y  in   inches   from  the   plot's   origin  (before   PrintCAD's

magnification is applied), or let it default to the plot center. For

the print window alignment point, you can default  to the  center or

specify the origin with  O.

/A      puts the center of the plot at the print window center

/AO0,0  puts the plot 0,0 at the print window origin

/A0,0   puts the plot 0,0 at the print window center

p-8

/B - Tiling Option ---------------------------------------------------

/B[i,j,x] - tiling parameters - default /B1,1,0

/B  lets  you  specify tiling options that break a  large  plot into

pages. Each page is defined by the page layout option (/L).  I and j

specify  the number of tiles (pages) horizontally and vertically.  X

specifies the overlap in inches.

/B2,2,.5  breaks the plot into 4 pages with .5 inch overlap

/B        is the same as /B1,1,0 and produces no tiling

/C - Pen Color Option ------------------------------------------------

/C[O|T]c.. - color - default /CK

/C specifies the color  of each of  40 pens.  The colors  are  B for

blue, C for cyan, G for green, K for  black,  M  for magenta,  R for

red, W for white, and Y  for  yellow.  Unspecified pens use the last

selected color  (/CRGB  is the same as  /CRGBBBBBB).  If  the output

format  does not  allow  color then  nonwhite  colors are changed to

black.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix  the  eight pure colors.  The format for a

mixed color is  .??  where each ?  represents any of  the eight pure

colors.  The first  color is printed  with the pen's shading pattern

(/S option)  and the  second  is printed  with  the  inverse shading

pattern.  If  the shade value is  0 (solid) it is replaced by 2 (one

pel checkerboard) which produces the most uniform mixed colors.

Mixed colors need line  widths  of at least  2 (4  for 2 pel shading

patterns) to look good, and they may have  visual artifacts on edges

near a 45 degree angle.  Multipass dot matrix formats may not handle

color mixing well.

By default, colors are  overwritten into the print,  so they  appear

opaque.  You can  switch to translucent colors (ored into the print)

with the  T suboption, and then back to opaque with the O suboption.

Make  this  mode switch in front of the pen that you want to change.

For PostScript devices and plotters, the color processing is done by

the device, not PrintCAD.  PostScript  devices  use opaque lines and

plotters use translucent lines.

" means repeat the previous color.

/CK             sets pen 1..8  to black

/CKOR.MB""T.KY  sets pen 1     to black

pen 2     to opaque red

pen 3..5  to opaque magenta on blue

pen 6..40 to translucent black on yellow

p-9

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption causes files to  be appended rather than overwritten (it

has no effect on device output). 1, 2, and 3 select the BIOS printer

routines.  X1, X2, X3, and X4 select xon/xoff controlled serial port

drive.  H1,  H2,  H3, and H4 select hardware controlled serial  port

drive. You can also use the DOS devices LPT1, COM1, etc.

PrintCAD cannot set up the serial ports. This is usually done in the

AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

/D2          sends output to the second parallel port via BIOS

/DLPT3       sends output to the DOS device LPT3

/D+TEMP.PRN  appends output to file TEMP.PRN

p-10

/F - Output Format Option --------------------------------------------

/Fc[-|+|*][!][~][x,y] - output format - default /FN~

The F option specifies the output format. -, +, and * are resolution

modifiers, !  selects a  compression mode  driver,  and ~  turns off

color processing on  color  printers.  -, +, *, !, and ~ are allowed

even where they have no effect.

You  may  enter the horizontal and vertical resolution of the output

device in dots/inch.  This overrides PrintCAD's normal output device

resolution, but does not affect the  structure of the output  format

(-,+, and * generally do alter the output structure).  These numbers

must follow the format character. This lets you compensate for print

size  error.  For example,  a 360x360 dpi printer consistently under

indexes so that a  6  inch high  box  comes out 5.94  inches.  Using

360,363.6 (6/5.94*360 = 363.6) removes the error.

Drivers marked below with ! have a compression mode driver, selected

with "!".  This may be  very useful, but it depends on your printer.

These  drivers compress the  output data by breaking  bit map strips

with printhead positioning commands.  This  saves data  transmission

time, but some  printers spend more  time positioning the  printhead

than is saved by not sending the data.

A  number of  printers use  compression  mode by default.  It can be

turned off with � (ASCII 173).

Drivers marked below with ~  are  color devices.  Using ~ suppresses

color processing, substituting black for all colors except white.

/F1     ~ IBM 9 pin ....................................... 120x72

/F1+    ~ IBM 9 pin (2 pass)                                120x144

/F1*    ~ IBM 9 pin (4 pass) .............................. 240x144

/F2       IBM Quietwriter 2                                 240x240

/F3       IBM Quietwriter 3 ............................... 240x240

/F4-      IBM LaserPrinter (PPDS mode)                      150x150

/F4       IBM LaserPrinter (PPDS mode) .................... 300x300

/F5    !~ IBM 24 pin alternate 1                            180x180

/F5+   !~ IBM 24 pin alternate 1 (2 pass) ................. 360x180

/F5*   !~ IBM 24 pin alternate 1 (4 pass)                   360x360

/F6    !  IBM Proprinter X24 .............................. 180x182

/F6+   !  IBM Proprinter X24 (2 pass)                       360x182

/F6*   !  IBM Proprinter X24 (4 pass) ..................... 360x364

/F7-      Canon BJ (Proprinter mode), IBM ExecJet           180x180

/F7       Canon BJ (Proprinter mode), IBM ExecJet ......... 360x360

/F8       HP, IBM plotter                                  1016x1016

/F9    !~ NEC 24 pin ...................................... 180x180

/F9+   !~ NEC 24 pin (2 pass)                               360x180

/F9*   !~ NEC 24 pin (4 pass) ............................. 360x360

/FA-    ~ HP DeskJet 500C, Mannesmann Tally MT92C           150x150

/FA     ~ HP DeskJet 500C, Mannesmann Tally MT92C ......... 300x300

/FB-    ~ Canon BJC 800 (Epson LQ mode)                     180x180

/FB     ~ Canon BJC 800 (Epson LQ mode) ................... 360x360

p-11

/FD-      Canon LBP (ISO mode) ............................ 150x150

/FD       Canon LBP (ISO mode)                              300x300

/FF    !~ Fujitsu 24 pin .................................. 180x180

/FF+   !~ Fujitsu 24 pin (2 pass)                           360x180

/FF*   !~ Fujitsu 24 pin (4 pass) ......................... 360x360

/FI-      HP LaserJet 3, DeskJet                            150x150

/FI       HP LaserJet 3, DeskJet .......................... 300x300

/FJ-      Canon BJ (BJ130 mode)                             180x180

/FJ       Canon BJ (BJ130 mode) ........................... 360x360

/FK     ~ Kodak Diconix Color 4                             192x192

/FL-      HP LaserJet ..................................... 150x150

/FL       HP LaserJet                                       300x300

/FN     ~ Epson 9 pin ..................................... 120x72

/FN+    ~ Epson 9 pin (3 pass)                              120x216

/FN*    ~ Epson 9 pin (6 pass) ............................ 240x216

/FO    !~ Toshiba 24 pin                                    180x180

/FO+   !~ Toshiba 24 pin (2 pass) ......................... 360x180

/FO*   !~ Toshiba 24 pin (4 pass)                           360x360

/FP    !~ HP PaintJet ..................................... 180x180

/FQ       HP QuietJet (PCL mode)                            192x192

/FR     ~ Tektronix ColorQuick ............................ 216x216

/FS     ~ PostScript                                       1016x1016

/FT    !~ Epson 24 pin .................................... 180x180

/FT+   !~ Epson 24 pin (2 pass)                             360x180

/FT*   !~ Epson 24 pin (4 pass) ........................... 360x360

/FW     ~ Epson 9 pin alternate                             120x72

/FW+    ~ Epson 9 pin alternate (3 pass) .................. 120x216

/FW*    ~ Epson 9 pin alternate (6 pass)                    240x216

/FX    !~ IBM 24 pin alternate 2 .......................... 180x180

/FX+   !~ IBM 24 pin alternate 2 (2 pass)                   360x180

/FX*   !~ IBM 24 pin alternate 2 (4 pass) ................. 360x360

/FZ     ~ ZSoft .PCX                                        100x100

/FZi,j  ~ ZSoft .PCX ........................................ ixj

Most 9 pin  printers, including the IBM Graphics Printer,  work best

with  the /FN  drivers, even  if  they claim Proprinter rather  than

Epson compatibility.  9 pin printers with 1/144  inch indexing, such

as the IBM  Proprinter and 2380,  work better with the /F1  drivers.

And some not fully compatible 9 pin printers only work with /FW, the

Epson 9 pin alternate drivers.

Most   Proprinter  X24   compatible  printers,  including   the  IBM

Quickwriter and 2390, work best with /F5 or /FX. /F6 is only for the

IBM Proprinter X24, XL24, X24E, XL24E, and 24P.

p-12

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets  you send codes to the printer before PrintCAD's usual  data

is sent and /K lets  you send  codes after PrintCAD's usual  data is

sent.  This  lets  you  set  up and reset  the  printer  for special

situations. These are not needed in the normal use of PrintCAD.

For  plotter  and  PostScript  output,  the  /J  codes   follow  the

initialization commands.

The  codes  are  specified  with  byte  values  (0..255  or $0..$FF)

separated with blanks or commas.  There is  a limit of 80 characters

for any option, so very long  sequences must  be handled outside  of

PrintCAD.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

/K10 10   sends two line feeds after the plot

p-13

/L - Page Layout Option ----------------------------------------------

/L[B][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 8.

The first optional x and  y are the print window width and height in

inches rounded to  .01.  These define  the hard  clip limits.  These

values are not checked against  your printer's capabilities.  If you

specify a print  window  that exceeds the output  device limits, the

results are  unknown.  You may need to reset your printer's margins,

maximum line  length, or  page length to get large plots  (see Large

Paper). The size needed for a given width and height exceeds x and y

by 8 pels to allow for line widths.

The  second optional x and y are the left and top margins,  measured

from the printer's left margin and current  vertical position to the

print  window.  The top margin  is  modified  to match the printer's

native indexing,  but  is  accurate  to  .02 inch.  The  margins for

HP-GL  and PostScript  are measured  from the  device's  0,0  (left,

bottom).

THE PRINT  WINDOW PLUS MARGINS MUST FIT WITHIN THE PRINTABLE AREA OF

YOUR PRINTER.

Each printer has a default page layout.  The default print window is

7.54x10.14  for   printers  and   10.14x7.54  for   plotters.   This

corresponds to  HP  7440  A size  paper.  The  margins  are  printer

specific to compensate for different paper  handling.  In evaluating

user  options,  PrintCAD  processes each  option  and  when finished

reprocesses the default page  layout for the chosen printer and then

reprocesses  the  last user /L  option.  This procedure  gives  good

default  values so that your /L option can change the letter or size

suboptions and still have good margins.

/LB7.5,3,.5,0  makes  the  print window 7.5 inches wide and 3 inches

high,  with  a .5 inch left margin and no top margin,

and boxes the print window

/L7.54,10.14   sets window equivalent to HP 7440 with A  paper

/L7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/L7.84,10.20   sets window equivalent to HP 7475 with A  paper

/L7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/L10.20,16.38  sets window equivalent to HP 7475 with B  paper

/L10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-14

/M - Magnification Option --------------------------------------------

/M[F]x - magnification - default /M1

The M option specifies  the magnification.  F fits the  plot to  the

print window size.  The default, /M1, gives the same size plot as an

HP 7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    prints at the same size as an HP 7475

/M.5   prints at half the size of an HP 7475

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O1

The O  option  sets the  print  window origin and  orientation.  The

origin is one of  four  corners numbered 1..4 for  upper left, lower

left, lower right, and upper right. The L suboption specifies a left

handed coordinate system (this should never be needed).

Here are pictures of PrintCAD's view of the output media compared to

an HP 7475's view, to help with choosing a /O option.

HP 7475                                 PrintCAD

A size           B size                 printer       plotter

y��������Ŀ    0,0��������Ŀy         ul�����Ŀur   ul��������Ŀur

�        �       �        �           1�     �4     1�        �4

�        �       �        �            �     �       �        �

0,0����������x      �        �            �     �     ll����������lr

�        �            �     �      2          3

�        �          ll�������lr

�        �           2       3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

/P - Intercept Mask Option -------------------------------------------

/PADI|mask - intercept mask

/PADI sets up PrintCAD as an AutoCAD ADI plotter driver. /Pmask lets

you  choose  a DOS file mask as PrintCAD's intercept mask.  The mask

should not include disk  or directory information - PrintCAD ignores

this information when looking at  file names.  The mask may  contain

the character #.  This is the same as ?, but when a file matches the

mask   the  character  that  matches  the  #  selects  the  PrintCAD

configuration - 2..5  selects configurations 2..5  and anything else

selects configuration 1.

/Padi       sets up PrintCAD to take data from AutoCAD interrupt 78

/P*.plt     intercepts data written to *.plt

/P#prn.plt  intercepts data written to 1prn.plt, 2prn.plt, etc.

p-15

/Q - Number of Copies Option -----------------------------------------

/Qi - number of copies - default /Q1

/Q  sets the number of copies of  each printed page.  The default is

one.  Page printers  produce multiple  copies  faster using  the  "N

copies" prefix code (see Useful Prefix and Suffix Codes).

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R508.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S  assigns a shading pattern to each pen.  Unspecified pens use the

last specified shade.  The shade values are 0..8.  0  is the default

solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%) and need

line widths of at least three to give good looking  lines.  5..8 are

2x2 dot patterns  (75%, 50%, 25%,  12.5%) and need line widths of at

least five to give good looking lines. Some printers, especially dot

matrix printers  in  multipass modes,  may wash  out shading because

they have a dot size that is large relative to the dot spacing.

1     2     3     4       5         6         7         8

����  ����  ��       �    ������    ��  ��    ��              ��

����  ����    ��   �      ��  ����    ��  ��    ��

����  ����  ��       �    ����  ��  ��  ��          ��    ��

����  ����    ��   �        ������    ��  ��      ��

/S024  sets pen 1 to solid, pen 2 to 50%, pen 3..8 to 12.5%

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPRINTGL.TMP

PrintCAD  uses  a temporary file when it does not have enough memory

to handle  a plotfile.  You can use /T to put it on a RAM  disk  for

extra speed.  You can force the use of a temporary file, if the plot

has over  1024 data points, with +.  If you use a RAM  disk this may

speed printing.

p-16

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns pen line widths. Each width may be 0..9, .10 ..

.16 pels or A..I for 2..10 units of 4/1016 inch (.1 mm).  A..I gives

a  device  independent width, but it is an approximation, limited by

the device technology. Unassigned pens use the last assigned width.

For  nonsquare  matrix  printers,  PrintCAD  uses  "pens"  that  are

optimized for an x/y ratio of .67, 1, or 1.5. The widths are figured

in the direction of  higher resolution, and are  not accurate in the

other direction.

Line widths of over eight pels are  slightly distorted at  the print

window edge to fit within the four pel border.

/WA      sets pen 1..40 to 0.2 mm

/WB2.16  sets pen 1     to 0.3 mm

pen 2     to 2 pels

pen 3..40 to 16 pels

/X - Control Option --------------------------------------------------

/XQ - queries PrintCAD installation

/XR - removes PrintCAD if allowed

/X[X][L][A][S] - controls PrintCAD's setup

/XQ  checks to see  if  PrintCAD is installed.  /XR removes PrintCAD

from memory, if allowed. /XQ and /XR disallow all other options.

The  other suboptions  control PrintCAD's setup, and  are  effective

only during initial  setup.  X forces the  use  of XMS  for swapping

instead of EMS.  L forces PrintCAD  to load low  instead  of using a

UMB.  A sets an alternate handle (use with PC-Draft-CAD and Drafix).

S adds progress  beeps.  These may be used in any combination.  Each

one, except S, may or may not improve compatibility.

/XL  sets up PrintCAD in low memory

/Z - Chord Angle Option ----------------------------------------------

/Z[E]i,j - default, min chord angle, enhanced font - default /Z10,5

The  Z  option  specifies the default  and  minimum chord angles for

circles and arcs. The HP 7475 defaults to a chord angle of 5 with no

minimum. But using chord angles of less than 10 returns little print

quality  improvement and takes a lot  of  processing  time.  So  the

default Z option is /Z10,5. For better emulation use /Z5,2.

The  E suboption  says to use the enhanced font,  which  doubles the

number of chords on curves in text, improving the  curve smoothness.

This is only effective on character heights above .5 cm. And for CAD

packages  (including  AutoCAD)  that  use  vectors  to  create  text

(instead of the HP-GL label command), this option is ineffective.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-17

Large Paper ----------------------------------------------------------

PrintCAD can handle printing on  paper larger than the usual 8.5x11,

but  this  takes  some  extra  work.  First  you  need  to  increase

PrintCAD's print window to cover the large paper.  This is done with

the /L option.  The  print  window plus the  margins  (/L)  must fit

within the printer's printable area.

And  then you must make sure that your printer recognizes the larger

paper  height.  For  continuous forms  paper  this is best  done  by

setting skip perforation to  off,  either by printer switch or  with

PrintCAD prefix codes.  For Canon, Epson, Fujitsu, IBM, and NEC line

printers (dot matrix, ink jet, thermal) use /J27 79.

For  cut sheet paper, on  Canon,  Epson, Fujitsu,  IBM, and NEC line

printers use /J27 67 0 n, where n is the page length in inches.

For  page printers,  legal  size paper  may be specified with  these

printer  code  prefixes: HP-PCL printers /J27 38 108 51  65 (/K27 38

108 50 65 to reset  to 11 inch paper), Canon LBP printers /J27 91 51

50 59 59 112 (/K27  91 51 48 59 59 112  to  reset), IBM LaserPrinter

/J27 91 70 5 0 3 1 2 1 (/J27 91 70 5 0 3 1 1 1 to reset).

To  get a 10x16  print  window on  11x17  paper with a Canon, Epson,

Fujitsu, IBM, or NEC line printer, use these options: /L10 16 .4 .25

/J27 67 0 17.  You will probably need to adjust the margins (.4 .25)

for best centering.

The  maximum  width  of  the print  window on  most  narrow carriage

printers is 8  - 8/(horizontal dpi).  For 300 dpi printers,  this is

7.97.  At 120 dpi it is 7.93.  The maximum width of the print window

on most wide carriage printers is 13.6 - 8/(horizontal dpi).  At 180

dpi this is 13.55.  At 360 dpi  it is  13.57.  The  horizontal print

margin should be set to 0 to get this width.

p-18

Answers --------------------------------------------------------------

To get data from your CAD package to PrintCAD you must PLOT.  Do NOT

print.

If you are getting system  crashes or nothing from PrintCAD, try the

options /XA, /XX, /XL, /XAX, /XAL, /XXL, and /XAXL.

If  your plot is messed up, with wraparound lines covering the page,

you have probably  exceeded the PrintCAD  internal coordinate limit.

This happens when the HP-GL coordinates exceed 32 inches. Reduce the

internal resolution  with /R762 for  coordinates up to 42  inches or

/R508 for coordinates up to 64 inches.

To get color output from PrintCAD, you must:

Define a multipen plotter in your CAD package.

Set  up  the pen assignment table in your CAD package so that each

color is assigned to a pen.

Use the PrintCAD /C option to assign the same colors to PrintCAD's

pens, for example /CRGBCMYK.

Plot spoolers are not  generally compatible with PrintCAD, and would

be of little value if they were.  If it worked, a plot spooler would

send  data  to  PrintCAD the background,  but PrintCAD's  processing

would take place in the foreground.

Print buffers  may or may not be compatible with PrintCAD.  You must

install the buffer first.

If your prints are coming out not quite to scale:

For ADI plotters,  check  that your  plotter  is  defined  as 1016

steps/inch.

Check that no calibration is being done by the CAD package.

See  /F  Output  Format  Option  for  information  on  print  size

correction.

For the Canon  BJC-800, the right margin defaults to  8 inches.  Use

/J27 81 114 to set it to 11.4 inches. Because the margin can only be

set once  between printer resets, you may need to use /J27  64 27 81

114 (27 64 is a printer reset).

For the Canon BJ-330 and most  other wide  carriage PC printers, the

maximum  print  window width  is 13.57  inches.  This is  a  printer

limitation that you cannot get around.

p-19

Useful Prefix and Suffix Codes ---------------------------------------

Below,  n  represents any number 0..255, nA represents any number in

ASCII format (1 becomes 49, 2 becomes 50, 43 becomes 52 51, etc.)

Canon, Epson, Fujitsu, IBM, NEC dot matrix and inkjet printers

N inch paper          27 67 0 n

17 paper              27 67 0 17

Skip perforation off  27 79

Reset                 27 64

HP-PCL printers

Reset                 27 69

Portrait mode         27 38 108 48 79

Top margin 0          27 38 108 48 69

Skip perforation off  27 38 108 48 76

11 inch paper         27 38 108 50 65

14 inch paper         27 38 108 51 65

HP LaserJet

N copies  27 38 108 nA 88

HP DeskJet 500C

50% shingling  27 42 111 49 81  (1/2 speed, reduced banding)

No depletion   27 42 111 49 68  (uses more ink, better quality)

PostScript

N copies           nA 32 88 67

Nominal dots/inch  nA 32 88 88  (use with /FS nA nA)

IBM LaserPrinter

11 inch paper, tray 1, n copies  27 91 70 5 0 3 1 1 n

14 inch paper, tray 1, n copies  27 91 70 5 0 3 1 2 n

14 inch paper, manual, n copies  27 91 70 5 0 1 1 2 n

N copies                         27 91 70 5 0 0 0 0 0 n

Canon LBP

11 inch paper          27 91 51 48 59 59 112

14 inch paper          27 91 51 50 59 59 112

Manual feed            27 91 49 113

Automatic feed         27 91 48 113

Full page memory mode  27 59 27 91 50 38 122

N copies               27 91 nA 118

Cursor to top of page  27 91 48 100

Diablo mode            27 58

ISO mode               27 59

HP QuietJet

HP-PCL mode      27 37 65

DIP switch mode  27 37 64

p-20

Printers -------------------------------------------------------------

Canon BJ and BJC Printers, IBM ExecJet

For  the BJ130e,  BJ10e, BJ10ex,  BJ300,  BJ330,  and  ExecJet  in

Proprinter mode, use /F7.  This is the fastest way to  drive these

printers at 360 dpi.

/FJ gives 360 dpi graphics for any Canon BJ printer in BJ130 mode.

The printer is set to 10 pitch text.

You can use /FT*! with the BJ or ExecJet printers in Epson LQ mode

to get 360 dpi print, but it is very slow.

These printers  have an image  density option (switch  1-9 on  the

BJ130,  1-4  on  the  BJ300,  BJ330,  and  ExecJet).  Low  density

eliminates  every second pel.  To get the full 360 dpi resolution,

use high density.  For the BJ300, BJ330, and ExecJet, also be sure

that the front panel FONT HQ (high quality) light is on.

For Canon BJC printers, use /FB in Epson LQ mode. This driver uses

a graphics mode that is not supported by the BJ printers.

Canon LBP Printers

/FD  works  with any Canon LBP printer in ISO mode.  At 300 dpi it

takes up to one  meg  of  printer memory to print a full  page  of

graphics. At 150 dpi (/FD-) 512K handles a full page.

It  may help to set the printer memory mode with /J27 59 27 91  50

38 122.

If the printer  is in Diablo mode, PrintCAD switches to  ISO mode.

To switch back to Diablo mode on completion, use /K27 58.

Epson, IBM 9 Pin Printers

/FN, /FW, and /F1 cover a wide range of Epson/IBM compatible 9 pin

dot matrix printers.  You need a  cyan/magenta/yellow/black ribbon

to get color prints. The * drivers use 1/240 graphics. /FN+, /FN*,

/FW+,  and  /FW*  use  1/216  indexing  and  do  three  vertically

interleaved passes.  /F1+  and /F1* use  1/144 indexing and do two

vertically interleaved passes. /FW sets line feeds to 1/6 inch.

For Epson printers,  the IBM Graphics  Printer, and other printers

with 1/216 indexing hardware, /FN is preferred. For some not quite

compatible  printers, /FW works.  For  IBM  Proprinters  and other

printers with 1/144  indexing hardware, /F1  gives better results.

Some Proprinters do not handle 1/240 graphics correctly.

These drivers use the following escape codes:

esc 3  set n/216 line feeds for all /FW

esc J  index n/216 (n=2 for 1/144 index) for all /FN and /F1

esc L  graphics command for /FN, /FN+, /FW, /FW+, /F1, and /F1+

esc Z  graphics command for /FN*, /FW*, and /F1*

esc r  set ribbon color (only if color is specified)

p-21

Epson, Fujitsu, NEC, Toshiba 24 Pin Printers

/FT  covers  all  Epson LQ printers and  many  compatible printers

(Panasonic in LQ mode and others).  /FF works with  Fujitsu 24 pin

printers in Fujitsu mode.  /F9 works with NEC 24 pin printers. /FO

works  with Toshiba 24  pin printers.  /FO  resets the  line  feed

distance to 1/6 inch on completion.

For all of  these  drivers, you  need  a cyan/magenta/yellow/black

ribbon to get color prints. The + and * drivers use 1/360 graphics

mode,  and  the * drivers use 1/360  (1/120 for /FO*) indexing and

reset the line  feed distance to 1/6 inch on completion.  A  clean

paper path with  equal tension on  each side  is  needed  for good

360x360 graphics.

Many 24 pin printers are not capable of 360x360 graphics and a few

cannot handle 360x180 graphics.

The compression mode drivers  (! suboption)  may or  may not be an

improvement with these printers.

The /FT drivers use the following printer escape codes:

esc $    absolute tab

esc * '  graphics command for /FT

esc * (  graphics command for /FT+, /FT*

esc +    set n/360 indexing for /FT*

esc 2    set 1/6 indexing for /FT*

esc J    index n/180 for /FT, /FT+

esc r    set ribbon color (only if color is specified)

/FT does  not work with IBM Proprinter  X24s in alternate graphics

mode because it uses the esc $ command.

HP DeskJet, Mannesmann Tally MT92C

/FI works with any black/white DeskJet in portrait mode.

/FA drives the DeskJet 500C or MT92C, and any color 300 dpi HP-PCL

printer that supports data transfer mode 2.

HP LaserJet

/FL works with any LaserJet  compatible printer in portrait  mode.

Portrait mode  may be set with /J 27 38 108 48 79 or you can reset

the printer with /J 27 69.

/FI works with the LaserJet 3 and any HP-PCL 5 compatible printer.

LaserJets with under one meg of memory may not be able to handle a

full page of graphics.  /FL uses less printer  memory than /FI, so

it may be preferred on LaserJet 3s with limited memory.

p-22

HP PaintJet

/FP drives PaintJet  printers.  PaintJet  printers  are  sold with

either a serial or parallel interface.  The  parallel interface is

faster.

The  compression mode  driver (/FP!)  uses  data  transfer  mode 2

(instead of 1) and works only with a PaintJet XL.

HP QuietJet

/FQ works with QuietJets  in HP-PCL mode.  /J27  37 65  /K27 37 64

sets HP-PCL  mode and  resets  to  the DIP switch selected mode on

completion.

IBM LaserPrinter

/F4 drives any LaserPrinter in PPDS (native)  mode.  /FL will work

if the LaserPrinter is in HP LaserJet mode, but this may be slower

than  PPDS mode.  At  300 dpi it takes up to  one meg  of  printer

memory  to print a full page of graphics.  At  150 dpi (/F4-) 512K

handles a full page.

IBM 24 Pin Printers

PrintCAD has  three drivers that  cover Proprinter  X24 compatible

printers. The printers should be in native mode (not AGM).

The first driver, /F6, is for all IBM Proprinter X24s and the 24P.

These printers do  1/144 inch indexing, and using them at 180  dpi

results  in horizontal  white streaks  every  2/3  inch.  The  /F6

drivers  compensate  for  this by indexing  19/144 inch  per line,

giving  182 dpi.  To work properly, the printer must be on a  1/72

boundary when the graphics are printed.  If you keep the line feed

distance  a multiple of 1/72 (1/6, 1/8, 1/9) then this will always

be the case.

The second X24 compatible driver is /F5 and  is referred to as X24

alternate 1.  This is for printers that are  X24 compatible except

that they  do 1/180 inch indexing.  This is probably the case with

most  X24 compatible printers, including the  IBM  Quickwriter and

IBM 2390  and 2391.  /F5* works only with printers that can handle

1/360  indexing,   including  the  2390  and  2391   but  not  the

Quickwriter.

The third  X24 compatible  driver is /FX and is referred to as X24

alternate  2.  This is  the same  as  /F5 except that the vertical

units command uses  a  different  format  that is not strictly IBM

compatible.  This driver works  with Panasonic 24 pin  printers in

X24 mode.

The compression mode drivers (!  suboption)  may or  may not be an

improvement with these printers.

Proprinter X24 and XL24 printers are very slow with graphics.  The

X24E and XL24E are significantly faster.

p-23

IBM Quietwriter 2 and 3

/F2 and /F3 drive  the Quietwriter 2 and 3.  Do not use  1 for pen

widths because  Quietwriters do not  print consistent 1 pel lines.

These  drivers  reset  the  line  feed distance to  1/6  inch upon

completion.

Kodak Diconix Color 4

/FK drives the Color 4.

PostScript Printers

/FS outputs PostScript.  All characters, arcs, fill, and clips are

converted to simple vectors.

PrintGL's pen color  and  shading parameters  are mapped into gray

levels or RGB color levels.

You  can  specify the nominal resolution of  the PostScript with a

prefix code that includes  the  dots/inch followed  by ' XX'.  For

example  /J 54 48 48 32 88 88 sets 600 dpi.  The actual resolution

should be set to match, for example /FS 600,600.

You can specify the number of copies similarly with the  XC macro,

for example /J 50 32 88 67 makes 2 (ASCII 050) copies.

Tektronix ColorQuick

/FR drives the ColorQuick.

HP and IBM Pen Plotters

/F8  outputs low level HP-GL, converting characters,  arcs,  fill,

and clips to vectors. The output is HP 7440 compatible, containing

only IN, SP, IW, LT, PU, PD, and PG commands.

PrintCAD uses its width parameter  (/W) as a  pen selector, so for

each pen in the  original plot,  you may specify a new pen  in the

output. The default /W option is /W12345678 for this case. Shading

and color parameters are ignored.

p-24

PCX and Fax Output ---------------------------------------------------

Use /FZ to  output a Zsoft PCX bit map.  The horizontal and vertical

dots/inch  may  be appended  to  override  the default 100x100,  for

example /FZ80,80. Such output should generally be directed to a file

with the /D option.

Since most fax boards  can take  PCX files as input, this is  useful

for faxing drawings. Use /FZ200,196 to match the fax resolution.

Compatibility and Technical Information ------------------------------

PrintCAD  will  run on any IBM compatible DOS computer with  400K of

EMS or 400K of XMS and a base memory region of 304K. Because it is a

swapping  resident   program,  it   may   have   problems  in   some

environments.  PrintCAD  can handle  VCPI (Virtual  Control  Program

Interface) based DOS extenders, but DPMI based programs (MS Windows)

and stand alone DOS extenders may cause problems.

When  using  a DOS extender based  program such  as  AutoCAD  386 or

Microstation  4,  use   an  extended/expanded  memory  manager  that

provides VCPI, such as MS-DOS 5's EMM386.EXE/HIMEM.SYS, QEMM 386, or

386MAX.

PrintCAD  is a TSR that takes over interrupt hex 21 and  watches for

DOS 2.0+ file management open functions.  If the filename on an open

command matches either "pcad" or  the mask specified with  /P,  then

file  handle dec 99 (or dec  15 if  /XA  is  used) is  returned  and

PrintCAD is awakened. It then responds to write, device information,

close functions, seek, and set date functions.

When awake,  PrintCAD stores HP-GL commands (from the write command)

until  its  buffer limit (16K) is reached and then  it swaps  in its

304K memory space, processes the data, and swaps the original memory

back.

PrintCAD  was  written by Cary Ravitz and  compiled  with  Borland's

Turbo Pascal 6.0.  PrintGL Printer Driver  is used  as  the graphics

engine.

p-25

User Support ---------------------------------------------------------

The  quickest  way to  resolve  problems with PrintCAD is to use the

support bulletin board.  The Ravitz Software support  bulletin board

is at:

606-268-0577  1200/2400,N,8,1  24 hours/day.

To ask a question (or make a comment or suggestion), first prepare a

file with the details of your question. Please be very specific, and

include  any related information  on  your  hardware,  software, and

operating environment.  Then call the bulletin board  and choose the

upload question  option.  XMODEM file  transfer protocol is required

for this.  Your question will be assigned  a number (such as 1015) -

remember this number. An answer file (with the assigned number) will

be posted, usually within 24 hours.

To  get an  answer,  call the  bulletin  board, choose the  download

option, then the answer option, and then enter  the  answer  number.

XMODEM is also required for downloading answers.

All question and answer files are public.

You may send questions to Compuserve - 70431,32  Cary Ravitz.

You may send questions by mail to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

Please send  a diskette  (360K, 1.2M, 720K,  or 1.44M, 720K is best)

with your question  so that example  files or newer  versions of the

software may  be  included  with the  response.  Return  postage  is

appreciated.