Date: 04-12-92 (19:51)

From: TOM HANLIN

Subj: QB PATCHES

------------------------------------------------------------------------

I spent some time today digging through the BCOM45.LIB code.  Funny way

to get your kicks, I know.  I took the opportunity to develop a few

fixes for things that irritate me about QuickBASIC.  I've tested these

patches on my own system, but of course I can't guarantee 'em.  They

appear to work fine with BCOM45.LIB (QuickBASIC 4.5 stand-alone .EXE

runtime support).  They will quite probably also work with QuickBASIC

4.0-4.5 and BASCOM/PDS 6.0-7.1, but I haven't tried it.  Anyway...

1) MAKE A BACKUP COPY of your BCOM45.LIB or whatever you plan to patch,

just in case.

2) Dig out your favorite debugger or disk editor.  Haven't got one?

Pick up FED (File EDitor) at your local BBS.

3) Install patches of choice.

To prevent the COMMAND$ function from capitalizing the command line,

SEEK: 3C 61 72 06 3C 7A 77 02 34 20

MAKE: EB 08 90 90 90 90 90 90 90 90

To prevent OPEN "COM..." from destroying the BIOS comm data area,

SEEK: 1E 33 D2 8E DA 87 95 00 04 1F

MAKE: 1E 33 D2 8E DA 8B 95 00 04 1F

SEEK: 1E 33 D2 8E DA 89 9D 00 04 1F

MAKE: 90 33 D2 90 90 90 90 90 90 90

To prevent CLOSE or END from lowering the DTR on comm ports,

SEEK: B0 00 E3 01 40 83 C2 04 EE

MAKE: B0 00 90 90 40 83 C2 04 EE

Be advised that if you install that last patch, you will need to drop

the DTR manually if you want it lowered.  My PBClone library can do

that for you.