PMEM: Protected Mode Monitor                           Version 2.4A

PMEM: A Protected-Mode Memory Monitor

Introduction

PMEM  is   a  protected-mode   environment   for   interactively

examining/changing/testing the  entire 4  Gigabyte memory space and

64K I/O  ports of 80386 or 80486 processors.  PMEM is not a program

debugger.   Rather, PMEM  is a  very useful  tool for examining and

debugging hardware  systems.   PMEM  is particularly useful for the

following applications:

-    For  system   hardware  designers,   PMEM  allows  system

integrity  to  be  verified  very  early  in  the  design

process.

-    For PC board and peripheral designers, PMEM allows you to

exercise your  product to  verify that  it is  functional

even before software drivers have been written.

-    For programmers, PMEM allows you to "peek under the hood"

to see  how your program is using memory and other system

resources.   Thus,  even  though  PMEM  isn't  a  program

debugger, it  can be  very  useful  for  system  software

authors.  Also, PMEM's special I/O capabilities allow you

to test  your hardware  initialization sequences  without

writing a line of code!

-    For technicians and engineers, PMEM can be used to create

"scope loops" which can greatly simplify finding hardware

problems.

-    For general  use, PMEM  is a  useful 32-bit  programmer's

calculator,  with   arithmetic  and   logical  operators,

variables, and more.

-    PMEM is  also extremely  useful as a learning tool, since

it allows  you to  examine areas of your system which are

normally inaccessible.

All of PMEM's commands can directly read and write memory or I/O

ports  8-bits,  16-bits,  or  32-bits  at  a  time.    This  unique

capability is  one of  PMEM's most  powerful features  -  very  few

inexpensive tools exist which have this capability.

Caveats

PMEM is not a simple program.  While a fair degree of effort has

been spent  trying to  make it an easy-to-use tool, PMEM works at a

level of  detail which  is beyond  most users.    If  you  are  not

familiar with bits, bytes, or bus cycles, PMEM probably won't be of

much help  to you.   Also, PMEM runs in 80386 protected mode, so if

you don't have an 80386 or an 80486 CPU, it's time to upgrade!

IF YOU  ARE RUNNING  PROTECTED MODE  PROGRAMS SUCH AS THE EMM386

DRIVER THAT  IS SUPPLIED  WITH MS-DOS VERSION 5, PMEM WILL NOT RUN!

IN ADDITION,  PMEM WILL  NOT  RUN  UNDER  WINDOWS!    To  run  PMEM

properly, you  should create  a "simple"  MS-DOS system, which does

not use EMM386, QEMM, 386^Max or similar programs.

Page 1               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

Overview of PMEM

The commands supported by PMEM include:

S:   Set the  command defaults.   You can set the DEFAULT word

size to  bytes (8-bits),  words (16-bits), or dwords (32-

bits),  and   the  numeric   input  base  to  decimal  or

hexadecimal.

":   The quote  command allows  you to  display specific  text

prompts.

C:   Compare two different regions of memory

D:   Display Memory location(s)

E:   Examine Memory (command-line and interactive forms)

F:   Fill memory locations with an arbitrary string

H:   Hex Math  calculator:  displays  the  result  of  a  math

expression.  PMEM has a broad range of math capabilities,

including arithmetic and logical operators.

I:   Input from I/O port(s)

M:   Move a block of memory to another location

O:   Output to an I/O port

R:   Set new refresh rate, or compute current rate

V:   Verify a block of RAM against an arbitrary string

T:   Test Memory with a predefined pattern sequence

Q:   Quit back to MS-DOS

?:   Show a brief program help message

In  addition,  PMEM  provides  facilities  to  combine  multiple

commands on  a single  command line,  along with  the capability of

repeatedly executing  individual commands and/or the entire command

line in order to perform "scope loops".

Overview of PMEM

In general,  PMEM operates very much like program debuggers like

DEBUG.   One major  difference between  DEBUG and PMEM is that PMEM

normally utilizes  linear addresses  (since there  is  no  standard

segment value for addresses beyond 1 megabyte).  However, PMEM will

permit you  to enter  addresses in segment:offset form if you wish.

In this  format, the  ':' simply tells PMEM to multiply the segment

number you  enter by  16, and  to add  in the offset which follows.

For example,  entering FE00:1234 will translate to a linear address

of 000FF234H.   Under  any circumstances, every address you specify

will range  from 0 to FFFFFFFF (4 gigabytes-1).  All numbers may be

entered in  hexadecimal,  decimal,  or  as  expressions  with  both

logical and mathematical operators.

Another feature  unique to  PMEM  is  that  it  understands  the

difference between  Bytes, Words, and Dwords.  All commands in PMEM

have the  capability of  performing their unique function in any of

these modes.   In  other words,  the Display  command can  not only

display memory as Bytes, Words, or DWords, it will also perform the

actual memory and I/O bus cycles in this form.  This will allow you

Page 2               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

to check  out unique  hardware problems which may only occur during

32-bit memory reads, etc.

Beyond the  default word  size (set  by  the  S  command),  each

command can  optionally have its own override attached to it.  This

will permit  you to  normally use, say, Words, but still be able to

execute a single command with Bytes or DWords.

The general form of PMEM's commands is as follows:

<Command>[<Override>] <parameters>

...where the  optional word-size  override is  one of the following

characters:

B    (For Bytes)

W    (For Words)

D    (For DWords)

Note that  there is  NO space  between the command and its optional

word-size override,  while there is a space between the command and

its parameters.   This  can  lead  to  some  confusion,  but  is  a

necessary evil.   Consider  the following.   The command to display

memory is 'D'.  Let's say that you want to display memory at D0000.

Typing DD0000  will look  to PMEM like a request to display double-

words at  0000.   Instead, you can type D D0000 to get the job done

(in this  second case,  PMEM will display the memory at D0000 using

the word-size specified by the last Setmode command).

The following examples may help confuse you...

DB0  Display bytes at 0

D B0 Display using the defaults at B0

DBB  Display bytes starting at 0B

D0   Display memory at 0 using the default

Note from  the last  example that  when the  first character  of  a

command's parameters  is not  'B', 'D',  or 'W',  no problems  will

arise.  However, to prevent confusion, your best bet is to either:

1)   Always enter  a space following every command, before its

parameters, when  you want  to use the default word size,

or...

2)   Always enter a word-size override on every command, as in

DB, DW, or DD.  Then, the space isn't necessary.

When  entering   numbers,  PMEM   normally  accepts  hexadecimal

numbers.   However, PMEM  also has  the capability  to allow you to

specify numbers in decimal, optionally in megabytes or Kbytes (this

saves you  from having  to type  in all those zeroes).  To do this,

just follow  any decimal  number with  a 'K' for Kbytes, an 'M' for

Megabytes, or  a 'T'  (for base  Ten) for  simple decimal  numbers.

Again, note  that a  suffix of  'K', 'M'  or 'T' indicates that the

number you  are entering is decimal, not hex.  In other words '64K'

Page 3               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

is 64  * 1K,  not 64H  * 1K.   If no suffix is entered, the current

input base  (which is  normally hexadecimal)  is  assumed  for  all

numbers.   Ergo, if  you've left the input base set to hexadecimal,

entering plain '64' is equal to 100 decimal, not 64.  If you try to

enter hex  digits in  a number, followed by 'M', 'K', or 'T' (as in

1F00T), PMEM will complain about an invalid number.

If you  are entering  lots of decimal numbers (particularly when

using PMEM  as a calculator), you may wish to set the default input

base to  decimal.   The SetMode  command, in addition to specifying

Bytes/Words/DWords as  a default,  can also  set the  input base to

either decimal  or Hex.   Once  the input  base  has  been  set  to

decimal, you  can still enter Hex numbers by following them with an

'H' (ie,  FF3CH).  In fact, all of the input base specifiers (T, K,

M, and  H) can  be used regardless of whether PMEM is in decimal or

Hex mode  - it simply isn't necessary to specify the base specifier

for numbers  being entered  with the default base.  For example, in

decimal mode,  adding a  'T' after  a number has no effect.  In Hex

mode, adding an 'H' after a number has no effect.

One final  point about  entering numbers.    PMEM  now  has  the

capability to  allow  you  to  enter  arbitrary  math  expressions.

Anywhere a  number is  required, PMEM  will allow  you to  enter an

expression which includes the following operators:

+  Addition         -  Subtraction

*  Multiplication   /  Division

%  Remainder        &  Bit-wise AND

|  Bit-wise OR      ~  Bit-wise Invert

^  Bit-wise XOR     !  Logical NOT

== Equal to         != Not Equal

<  Less than        >  Greater than

++ Increment var.   -- Decrement variable

:  Segment:offset input format for numbers

Each of these operators may be combined in the usual manner, and

parentheses may  be used  to delineate subexpressions.  With regard

to precedence,  negation has  the highest precedence, then multiply

and divide,  then addition  and subtraction,  then  binary  logical

operations (AND, OR, and XOR).  Examples include:

100000+64K     1M*100+7T      4M%317

1K+100-5       1B7C0/2ACD     500*(2-16)

FFE0:27B&!3C   87|127&F0      22^15T

Page 4               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

NOTE! Be  careful with the segment:offset operator (':'), which has

the same  precendence as  multiply and  divide.  If you want to use

multiply/divide/modulus   with   a   segment:offset   number,   use

parenthesis to eliminate any confusion, as is: "(12C0:33FB) * 2" or

"12C0:(33FB *  2)".   With other operators such as addition, typing

"F000:400+C6C7" is the same as typing "F000:(400+C6C7)"

Command Summary

S    Setmode -  sets  the  default  modes  used  by  PMEM  for  all

subsequent commands.   When you first invoke PMEM, the default

word size is Bytes, and the default input base is Hex.  If you

just want  to use a command or two in Words or DWords, you can

specify an override for the command.  Likewise, if you want to

enter just  one or  two numbers in decimal, you can follow the

numbers with  a 'T'.  If, on the other hand, you want to semi-

permanently work in Word/DWord mode, or in decimal input mode,

the Setmode  command will  change the  default mode so that it

isn't necessary  to keep  entering overrides for each command.

The syntax is:

S <mode>

<Mode> can  be used  to set  the default  word size  to B (for

Bytes), W  (for Words), or D (for DWords).  <Mode> can also be

used to set the input base: T for decimal (base Ten), or H for

Hexadecimal.  Examples:

SB        Set Byte Mode

SW        Set Word Mode

SD        Set DWord Mode

ST        Set Decimal input mode

SH        Set Hexadecimal input mode

C    Compare memory.   This  command performs  a direct  comparison

between two  different regions of protected memory (which must

be the  same length).    As  the  regions  are  compared,  any

differences between  the source  address (the first address on

the command  line) and  the destination  address  (the  second

address on  the line)  are displayed  on the  screen, with the

contents of  the failing  addresses.    The  syntax  for  this

command is:

C[<override>] <range>,<address>

If, instead  of comparing  two different  memory regions,  you

want to  verify that  memory holds a specific sequence of data

values, see the Verify command, below.  Examples:

C 1M L1M,4M         Compare the 2nd megabyte to the 5th

CD1000,1FFF,300C    Compare dwords between the two areas

Page 5               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

D    Display Memory.   This  command can  display any  area in  the

4-gigabyte 386/486  memory space.   Used just like the monitor

ROM, the syntax is:

D[<override>] [<range>]

The range can be specified in one of four different ways:

1)   <Start>,<End>       Start address to end

2)   <Start> L <BYTEcnt> Start address w/length

3)   <Start>             Start only, length is 128

4)   -nothing-           Continue from last Display

Examples:

DB0            Display bytes at 0

DW FFFFFF80    Display last 128 bytes of ROM

D 0,3FFF       Display first 16K RAM w/default

DD90L10T       Display 10 bytes worth of DWords @90

D F000:800C    Display the ZDS Machine ID in slushware

E    Examine Memory.   This command takes two forms.  In the first,

you specify  a data  string to  be written  to memory.  In the

second, you  just specify  an address,  and can  interactively

examine and change memory.  The first form is:

E[<override>] <address>,{<number>|"<string>"}, ...

In this  form, the  specified data  is written directly to the

memory address  you specify.   This is equivalent to filling a

memory block which is exactly as long as the data you specify.

For more  details on entering strings, see the 'Fill' command,

below.   In the second form of the Examine command, the syntax

is:

E [<override>] <address>

When you  use this form, PMEM will display the contents of the

specified memory  location, and will prompt you for a new data

value.  If you want to change the contents as shown, type in a

new value,  followed by  Enter.   To move  forward to the next

location in  memory, just  press Enter,  or a  space.  To move

backwards to the previous location, press BackSpace.  When you

are finished examining a block of memory, press Escape.

Examples:

ED100000       Examine the DWord at 100000 (interactive)

E 32K-17       Examine w/default at 7FE9 (interactive)

EW1234,5678    Write 5678H at location 1234H

EB9000,"ABC"   Write bytes of 41H,42H,43H to 9000H-9002H

Page 6               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

F    Fill Memory with a user-specified string.  The fill command is

used to  write a  block of memory with a distinct memory value

or values.  The syntax is:

F[<override>] <range>,{<number>|"<string>"}, ...

The string  can be  any mixture  of hex  numbers and/or  ASCII

characters.   When  numbers  are  entered,  they  are  entered

according  to   the  appropriate  word  size.    For  example,

'FD 100000L100,1' would  fill the  256 bytes  starting at  the

base of  the second  megabyte with the value 00000001.  If you

then   display    this    as    bytes,    you    would    see:

01H,00H,00H,00H,01H,00H,00H,00H..., since  the 386/486  places

bytes in reverse order.

Whenever you  enter ASCII  characters, they  are automatically

byte-packed to  correspond to  the correct  word  size.    For

instance, 'FD 200000L10000 "Hi!",D' will fill the 64K starting

at 200000  with 48H,69H,21H,0DH,00H,00H,00H,00H which would be

displayed in  double-word mode  as 4869210DH,00000000H.   Note

that ALL  entities are  byte packed,  so that  if you enter an

ASCII string  which is  five bytes long, followed by a number,

the number  will be shifted to align with the string (in other

words, if you want numbers to appear with the proper alignment

following ASCII  strings, make sure the strings evenly fit the

word size you are using if you are in Word or DWord mode).

Regardless of  what you  enter, if  the sum total of the bytes

you have  entered does not evenly match the current word size,

PMEM will pad trailing 0s into the fill string.  For instance,

if you  do a  DWord Fill with "ABC", PMEM will write 00434241H

to each DWORD.

Examples:

F  8000:0,8FFFF,"Hello"  Fill 64K with "Hello"

FD 80000L100,12345678    Fill 256 bytes w/12345678

FW 1M,1M+1M/2,9999,7A+CD Fill 1.5M+2 w/9999H,0147H

FB B800:0 L 16K,"1",17H  Fill screen w/white on blue '!'s

H    Hex Math.   PMem  can directly display the results of a simple

math expression  in both hexadecimal and decimal.  To do this,

just type in the expression to be displayed.  Syntax:

H <number>[+|-<number>]...

Examples of this command include:

H 1M+64K+4K+256T+10+1    Displays: 00111111H  1118481T

H 3FCD0+FFE0             Displays: 0004FCB0H  326832T

Page 7               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

H 1M-16T                 Displays: 000FFFF0H  1048560T

H 2900/17T+FF*120        Displays: 00012149H  74057T

H 55 | 37 ^ (2C & !15)   Displays: 0000005FH  95T

I    Input from  one or  more  I/O  ports.    As  with  the  memory

commands, I/O  can also be done using Bytes, Words, or DWords.

Unlike the  monitor ROM,  the Input command can also display a

range of I/O ports.  The syntax is:

I[<override>] <range>

The range  is very  much like  that specified  for the display

function,  except   that  if  no  end  address  or  length  is

specified, the  result is  that a single I/O port is displayed

(rather than 128).

Examples:

IB 0,3FF       Display the 1K PC/AT ports as bytes

I80            Read w/default from I/O port 80H

IW80           Read I/O ports 80H,81H as 1 word

M    Move a  block of  memory to  another location.   This  command

operates identically  to the  monitor ROM, except that it does

NOT check  for overlapping  moves.  In other words, if you are

moving a  block of  memory to  another location which overlaps

with the source block, data will be lost.  The purpose of this

choice was to ensure that what you type, you get! Syntax:

M[<override>] <range>,<destination>

Examples:

MD B000:0L64K,1M         Move all video RAM to 1M

M  320K,640K-1,100000    Copy the top 320K to Extended

MW 0,16M,0               Rewrite the base 16M with self

O    Output to  a single I/O port.  This command operates just like

the monitor  ROM (except for the override option).  The syntax

for this command is as follows:

O [<override>] <port>,<value>

Examples:

OBFB,0              Enable the ZDS Privy port

OD 80H,12345678     Write ports 80-83 w/78,56,34,12.

OFFF0,1             Write a 1 to default width at FFF0

Page 8               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

R    Refresh rate  set/determine.   This command allows you to slow

down or  speed up  the refresh  rate  to  help  find  refresh-

associated failures.   To  set the refresh rate, enter the new

desired refresh  rate in  microseconds (from  3-50000).  If no

number is  entered, the  routine will  attempt to  compute the

current approximate refresh rate.  Syntax:

R [<uSec>]

Examples:

R                   Display the current refresh rate

R10                 Set refresh to 16T (Normal rate)

R128T               Set slow-refresh mode

T    Test a  block of memory.  PMEM now uses a "March Test" to test

a block  of memory,  by making  sequential passes  which  both

verify memory  and write  new contents  in each pass (the test

makes a  total of  nine passes over each block).  To help show

test  progress,   PMEM  will  display  a  single  digit  which

increments to  show you  the phase of the memory test (from 0-

9). This  new test  procedure is  very  effective  at  finding

read/write errors, as well as addressing errors.

Another option  offered by  the test  command  is  to  perform

refresh testing.   If  specified, the  routine will wait for a

user-entered number  of seconds  between test passes, with the

CPU halted.   This  will help "encourage" any potential memory

refresh failures  in the system being tested.  To specify this

option, enter 'W' after the range.  Syntax:

T[<override>] <range> [W<waitcount_in_seconds>]

Examples:

T100000L100000           Test second megabyte of RAM

TW2M,3M-1,W5             Test 3rd Meg as words w/5 second

delay between passes

TB100000,1C2FF0          Test specified block as bytes

V    Verify a  block of  memory against  an arbitrary string.  This

command is  just like  Fill, except  that it verifies that the

data in  the memory  block is correct, rather than writing the

data.  The syntax is:

V[<override>] <range>,<string>

This command will display any discrepancies between the string

and the  memory block  by printing the address of the failure,

along with the good data (what SHOULD have been there) and the

Page 9               (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

bad data  (what WAS  there).  If the entire block verifies OK,

the routine won't print anything.

Examples:

VD1ML5M,0                Is 1st 5 Meg of extended RAM=0?

VB80000L100,12345678     Verify 64 DWords are 12345678

(Remember, L specifies # BYTEs)

W    Wait for  a specified  number of  seconds.  This option uses a

'hard' delay,  where  the  entire  system  will  be  quiescent

throughout the delay.  This is accomplished by placing the CPU

in a  halt state,  and then  using the real time clock's alarm

output to  wake up  the CPU once the requested time period has

elapsed.  Syntax:

W <seconds>

Example:

W5                       Wait for 5 seconds, with the

CPU halted.

"    The Quote  command (")  is used  to display  a prompt message.

The syntax is:

"<ascii string>",[<number>|"string"]...

Examples include the following:

"Result is ";D@0L1  Display msg followed by memory at @1

"",0D,0A            Display a blank line

"2+2=";2+2          Display 2+2=4!

Repeating Command Execution

One of  the most  powerful features  of PMEM  is its  ability to

repeat both  individual commands,  as well  as repeating the entire

command line.   To  repeat an  individual command, follow it with a

backslash ('\').   Optionally,  you may follow the backslash with a

repeat count - if you do not enter a repeat count, PMEM will repeat

the requested command forever (until you press ESCape).  Examples:

I41\                Repeatedly display the refresh timer

T1ML1MW5\10         Test the 2nd meg of RAM, with a 5

second delay between passes, for a

total of 16 passes.

It is  also possible to string multiple commands together on the

same command  line by separating them with a semicolon (';').  This

allows multiple functions to be performed at the same time.  Alone,

Page 10              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

this isn't  a very  useful capability,  but in conjunction with the

ability to repeat the entire command line, it can be quite helpful.

To repeat  a command line, enter either a forward slash ('/') or

a backslash  ('\') command at the end of the command line.  Using a

slash will simply repeat the command, while a backslash will repeat

the command  and then  print a pass number on the screen after each

repetition (this  will enable  you to verify that the loop is still

executing).

Note here  an important  syntax difference  between repeating  a

command, and  repeating the  command line.    To  repeat  a  single

command, follow the command directly with a backslash (and optional

repeat count).   To repeat an entire command line, enter the '\' or

'/' as a separate command (in other words - follow the last command

on the  line with  a semicolon,  then with  a slash  or backslash).

Examples include:

FD1ML1M,0\          Fill the 2nd meg of RAM w/0

forever, displaying a pass count.

If, instead,  you just  want to  write a pattern and continually

look for read failures, you can type:

FD1ML1M,0           Fill the memory

VD1ML1M,0\          Verify it forever

Another takeoff on this concept permits you to continually write

memory with different patterns, as follows:

F512KL4K,55;F80000,80FFF,AA;/

This command  will repeatedly  write the same block of memory first

with 55H,  then with  0AAH  (note  the  two  different  methods  of

specifying the  same memory  block, and the usage here of repeating

the entire command line, rather than a single command.

Whenever you  are performing  a repeated  command such  as those

noted above,  PMEM will  loop forever  (or until  you  stop  it  by

pressing ESCape).   If you simply want to execute a loop a specific

number of times, just follow the repeat command ('/' or '\') with a

count value.  PMEM will then repeat the command the number of times

you specify and stop automatically.

To abort  any command,  press ESCape.   If  ESCape doesn't work,

you've probably hosed the machine - PMEM makes no effort whatsoever

to prevent  you from  blowing important things away.  Keep in mind,

however, that  some commands do take quite a while to execute.  For

example, testing big blocks of extended memory (multiple megabytes)

can take tens of seconds or more; you may have to be patient.

Page 11              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

Variables

For some  more complex  tests, it is sometimes necessary to have

additional flexibility.   To support these more complex tests, PMEM

provides fifty numeric variables, which can be assigned, displayed,

used in  any command,  etc.   These variables are specified with an

'@', sollowed by a variable index.  Legal variable names include @0

to @49T  (equivalently, @00H to @31H).  These variables can be used

anywhere a number would be used, and will serve the same purpose as

a number.   Specify a variable name alone (as in @1), and the value

used will  be the  current contents of the variable.  Additionally,

you can  follow a  variable name  with a  equals sign to assign the

result of an expression to the variable.

When you  specify a  variable index,  you will  normally type  a

simple number, as described above.  However, it is also possible to

enter a  parenthesized subexpression  following the '@' sign.  This

will allow  you to  specify a variable name indirectly (essentially

providing the same function as a CPU index register).  For example,

"@(5) =  1" is  simply another  way of  setting variable  @5 to  1.

However, "@(@5)  = 1" is a very different beast.  This means to set

the variable  pointed to  by @5  to 1.  In this case, if @5 was 17,

then "@(@5) = 1" would set @17 to 1 (@5 remains unchanged).

The final  capability offered  for variables  is the  ability to

increment and  decrement them,  as a  convenient shorthand.   To do

this, follow  a variable  name with  ++ (to add 1 to it), or -- (to

subtract 1  from it).   Note  that this  is  equivalent  to  typing

@var=@var+1, or @var=@var-1, respectively.

Some examples of variable usage include:

@0++                      Increment variable 0

@0                        Displays the contents of variable 0

D@0L@1                    Displays memory starting at (@0),

for length (@1)

@0*@1                     Show result of multiplying @0 and @1

@0=1                      Set variable 0 to 1

@(@0)=55H;@0++;/10T       Set variables 1 to 10 to 55H

@0=@1=1                   Set @0 and @1 to 1

@0;@2=@0+@1;@0=@1;@1=@2;/ Display the Fibonacci numbers!

@0=1M                     Set @0 to the start of extended RAM

E@0,@0&FF;@0++;/4K        Fill 4K with 0,1,2...FF,0,1,2...etc.

As a  final example of the power of PMEM, consider the following

test:

@0=16T

R@0;R;M1ML512K,1M+512K\200;T1ML1MW3;@0=(@0*110T)/100T;\

Page 12              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

This incredibly  complicated looking  pair of  commands  can  be

broken down  as follows.   The first command simply sets variable 0

to 16  - which  is the  normal memory refresh rate in microseconds.

The next line can be interpreted as follows:

R@0                   Set the refresh rate to the contents of @0

(initially, this is 16 microseconds)

R                     Display the current refresh rate

M1ML512K,1M+512K\200  This command will move memory from 1M to

1.5M to the area from 1.5M to 2M, and will

repeat the move 200H (512T) times.

T1ML1MW3              Test the second megabyte with a wait-for-

refresh test of 3 seconds between phases.

@0=(@0*110T)/100T     Increase the value of @0 by 10%.  Since

PMEM doesn't have floating point, the use

of a multiply/divide pair provides the

same capability.

In total,  then, this  pair of  commands will first run memory very

hard to ensure as much heat build-up as possible occurs (this helps

to induce  memory refresh  errors), then it tests to see if refresh

errors will  actually occur.   After  doing this  test, the refresh

rate is  increased by  10% and  the entire  process repeats.   This

single  command  provides  a  very  simple  way  of  verifying  how

resistant a system is to refresh failures.

Memory and I/O Pseudo-Arrays

One final  capability offered  by PMEM (and potentially the most

powerful) is  that it  supports direct  access to  memory  and  I/O

within expressions.   PMEM  uses built-in pseudo-arrays which allow

you to  specify any memory or I/O location in a manner very similar

to the  @xx variables  mentioned above.   This  is much  easier  to

demonstrate than to explain:

M[0]           Memory location 0, in the current mode

IW[80]         The word-sized input port at 80H

OB[F2]         Byte-sized output port at F2H

These pseudo-arrays  may be  treated just  like variables in any

expression.   For instance,  you can  see the  contents  of  memory

location 0  just by  typing M[0].  In addition, these pseudo-arrays

may also  be written to.  To change the contents of memory at 1M to

55H, type:  M[1M]=55.  Write to an I/O port in the same manner, ie,

O[F2]=AA.

The real power of the memory and I/O arrays comes from their use

in repeated expressions.  Some examples include:

@0=0;@1=1M                    Set @0 to 0, @1 to 1 Megabyte

M[@1]=I[@0];@0++;@1++;\400    Copy all I/O ports to 1M RAM!

O[F2]=M[1ABCD];/              Copy memory data to output port

Page 13              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

Summary

In summary,  I hope that you find PMEM useful.  Just remember to

run it  only on  386 or 486-based systems, and be somewhat cautious

until you've learned the program.  PMEM has been around awhile, but

no doubt  still has  a few  bugs lurking  in it.   If you happen to

encounter one, please drop me a line as to what went wrong, and how

you made  it happen.  Have fun with it, and please let me know what

you think!

Mark J. Foster

4284 White Pine Circle

St. Joseph, Michigan  49085

Compuserve ID: [76505,2436]

Page 14              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

Change History

07/14/92: Version 2.4A

- Added ability to repeat individual commands

07/14/92: Version 2.4

- Corrected expression evaluator errors in complex expressions

- Added the 'Wait' command to help find refresh errors

- Changed syntax of the Test Refresh command to Test Wait

- Modified Test Wait to use the new full-halt WAIT for

more complete refresh error testing

07/13/92: Version 2.3

- Added comparison operators: <, ==, >, !=

- Added tertiary if-then-else: <cond> ? <expr1> ` <expr2>

- Added memory pseudo-array variables

- Added I/O pseudo-array variables

- Added Increment and Decrement commands

07/12/92: Version 2.2

- Added Refresh Test option to the memory test

- Added ability to set new refresh rate for memory testing

- Added "display refresh rate" capability

07/11/92: Version 2.1A

- Fixed error on the last byte of a block while doing a

Test or Verify command.  In this situation, PMEM would

continue verifying past the end of the block.

- Improved error handlers to show pass count where errors

occurred, if the passcount feature is being used.

07/11/92: Version 2.1

- Added Fast Block Clear to memory test during phase 0

- Added Fast Block Clear during Fills with a string length

of just one entity.

07/10/92: Version 2.0A

- Corrected problem with repeat counts following a '\'

07/10/92: Version 2.0

- Modified memory test to make 10 phases with worst-case data

- Added memory phase display to indicate progress of testing.

07/09/92: Version 1.9

- Rewrote memory test to use predefined data, in 7 phases.

This march test is much better at finding errors than the

prior PMEM algorithms.

- Modified '\' to display a pass counter instead of a period.

- Modified Compare, Verify, and Test NOT to display 'OK'

after testing a block, to prevent errors from scrolling off

the screen.

03/16/92: Version 1.8B

Page 15              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

- Fixed exit-to-DOS bug with MS-DOS 5

- Adapted source to compile under Borland's BCC Compiler

- Updated and improved documentation

07/24/90: Version 1.8A

- Modified help message to include variables, repeat counts

- Fixed error message on missing quotes (used to say missing

parenthesis).

07/20/90: Version 1.8

- Added variable support (@0-@50T)

- Added assignment to variables (@xxx = <expression>)

- Added quote (") command for text prompts

- Limited input line length to 128 characters

- PMEM now allows negation of parenthesized subexpressions

- Added ability to use parenthesized subexpressions in strings

07/19/90: Version 1.7B

- Corrected problem with long input lines overwriting internal

variables (caused various weird behaviors).

07/18/90: Version 1.7A

- Corrected problem w/V1.7 which displayed the current modes

every time an override was used.

07/18/90: Version 1.7

- PMEM now uses SCP Gate-A20, instead of Fast-A20.  This lets

it run on all ATs, not just ZDS machines.

07/18/90: Version 1.6B

- Added decimal/Hex default input modes

- Added 'H' to specify Hex numbers in decimal mode

- Added current mode display to setmode, and at startup

- Added divide-by-zero check

07/17/90: Version 1.6A

- Corrected problem where a compare error on the last byte

of a block would compare past the end of the block

- Added a CPU Type Check so PMem will complain about the

wrong CPU, rather than just die.

- Added optional repeat count following a repeat command

(either '/' or '\').

07/13/90: Version 1.6

- Added segment:offset format numeric input

07/12/90: Version 1.5B

- Corrected problem with Verify Bytes w/length > 64K

- Fixed spurious error message after interactive Examine

07/11/90: Version 1.5A

- Added logical binary operators (&:AND, |:OR, ^:XOR)

- Added unary negation operator (!:NOT)

Page 16              (C) 1992, Mark J. Foster         July 14, 1992

PMEM: Protected Mode Monitor                           Version 2.4A

07/11/90: Version 1.5

- Added multiply, divide, and modulus operators

- Added parenthesis handling for subexpressions

- Added decimal display to the hex math command

- PMEM once again requires a ';' before a repeat command

('/') to prevent the slash  from  being misinterpreted

as a division request.

07/09/90: Version 1.4A

- Added ability to accept math expressions

- Added <expression> 'command'

- Display speed substantially increased

- PMEM has been reduced in size by ~3.5K

07/06/90: Version 1.4

- Added the Compare Memory command

07/06/90: Version 1.3B

- Disabled NMIs to prevent reboots while in protected mode

- Added '\' (loop with '.' echo on each pass)

- Eliminated need for ';' before '/' or '\'

07/05/90: Version 1.3

- Added Move command

07/05/90: Version 1.2B

- Added 'M' and 'K' suffixes for numerical entries

- Changed input base on 'M' and 'K' numbers to decimal

- Fixed verify count problem on long blocks

07/03/90: Version 1.2A

- Corrected inability to use lower-case in strings

- Fixed problem with non-aligned fills

- Corrected documentation bug w/FD200000L10000,"Hi!",D

Page 17              (C) 1992, Mark J. Foster         July 14, 1992