Viper - the Visual DOS Command Line Editor

Table of Contents                   Page

Introduction To Viper........................................ 01

Installing Viper For DOS..................................... 02

Installing Viper For Windows................................. 02

Uninstalling Viper........................................... 03

VP.DAT File.................................................. 04

Command Line Switches........................................ 06

The Visual Window............................................ 14

Editing Commands............................................. 15

Function Key Buffer Strings.................................. 16

Copy and Paste............................................... 16

KEEP and PLAYBACK Records.................................... 17

AUTO PLAYBACK Feature........................................ 18

Alarm Feature................................................ 20

Mouse Support................................................ 21

Battery Management Tips For Laptops and Notebooks............ 21

Registration Form............................................ 22

Technical Support............................................ 23

Hardware and Software Requirements........................... 23

License Agreement............................................ 23

Index........................................................ 24

Viper

by

Freshour Software

531 Main Street #306

El Segundo, CA 90245

310-640-1649

INTRODUCTION                     Page 1

Welcome to Viper, "The Visual DOS Command Line Editor".  Viper (VP)

remembers keystrokes you type in at the DOS prompt and by pressing the

up or down arrow key (or the left mouse button if you have a mouse

installed), Viper pops up a visual window to allow you to view, edit

and select these previous DOS commands for re-execution.

But you'll find that Viper has many other features and capabilities:

o Pop-up window for better visual orientation

o BATTery Watch digital readout for portable computers

o Multiple commands executed from one record or command line

o Full command line/window editing capabilities

o Variable number of records in visual window (10 to 22)

o Commands can be assigned to Function keys F1 - F10

o DEL command can first save files a into \TRASHCAN subdirectory

o VP can be altered via command line switches

o User can configure Viper's memory usage

o VP.DAT file: permits pre-loaded commands you use most often

o Tag command(s) for manual (group) playback

o Tag command(s) for automatic unattended (group) playback

o Built-in alarm for appointments

o 386MAX, QEMM and DOS 5.0 compatible (to run in high memory)

o Full mouse support

When you use Viper, don't continue using your previous DOS history

utility; DOSKEY, DOSEDIT, etc.  You only need to use one of these types

of utilities at a time.

Viper can save you time and increase your daily productivity by

relieving you of the task of re-typing the same DOS commands over and

over again.  With Viper, you can set and keep any DOS command you wish

to use again.

You can also use Viper to execute multiple DOS commands (including the

running of other programs) if you have a series of commands that you

need to run from the DOS prompt.  Viper can do all of this without you

having to re-type each one of the commands in each time you want to run

them.

If you already use a DOS history utility, you'll appreciate Viper's

visual pop-up window.  It's this visual window that makes Viper so easy

and convenient to use, among it's many other features.

INSTALLING VIPER FOR DOS             Page 2

Copy all the files on the master diskette down into the root section of

your hard disk.  For example, from the DOS C:\> prompt, type COPY

A:\*.* <RETURN>.  This will copy the VP.EXE, VP.DOC and VP.DAT files

down into the root section of your hard disk.  From here, to start

Viper, simply type VP <RETURN>.  You may also type VP /? <RETURN> to

see all of the switches available to alter Viper's operational

behavior.

Viper is a TSR (Terminate but Stay Resident) or memory resident program

and therefore, must first be installed, or, loaded into memory, before

it can be used.  To install Viper into memory type VP <RETURN> at the

DOS prompt.  You may want to control some of the operational features

by also including some command line switches when you install it.  See

page 6 on COMMAND LINE SWITCHES for a complete description on how these

options alter Viper's operational behavior.

Since you will most likely use some command line switches each time you

start Viper, this task can be automated by placing the start-up

commands in your AUTOEXEC.BAT file or another .BAT file.  You could,

for example, make a batch file called VIPER.BAT containing the line VP

/D /F1 /R22 /MM.  Then simply typing VIPER <RETURN> will install it

with these command line switches.

INSTALLING VIPER FOR WINDOWS

1. In the MAIN Group window, click on the MS-DOS icon to shell out to

the DOS prompt.

2. You will usually be shelled out at the C:\WINDOWS subdirectory.

Change back to the root subdirectory by typing CD\ <RETURN>.

3. Install Viper with a .BAT file such as VIPER.BAT that contains all

of your start-up switches you wish to start Viper with.  Or, you can

manually start Viper like this: VP <RETURN>.  Note that Windows is

one of those applications that use the spare video pages, therefore,

on color video systems, with Windows, Viper must not be started with

the /VB1 or /VB2 switches.

4. Now we want to return to Windows but we don't want to close the DOS

Prompt session we just created and started Viper in.  Therefore, DO

NOT type 'EXIT' <RETURN> at this point!

5. Instead, press CTRL-ESC to display the Windows Task List window.

You will see the 'MS-DOS Prompt' listed as one of the current tasks

that is running.  Select 'Program Manager' or any other task from

this list to switch you back into Windows.

6. Now, whenever you need to do some work at the DOS prompt, simply

press CTRL-ESC to display the Windows Task List and select the

'MS-DOS Prompt' task.  Windows will return you to your previous DOS

Prompt session with Viper already loaded into memory.

UNINSTALLING VIPER                 Page 3

Viper can be uninstalled or, removed from memory, by simply typing VP

<RETURN> at the DOS prompt.  It will make a number of internal checks

to determine if it is safe to uninstall itself.  If it is safe, Viper

will remove itself from memory.  If it is not safe, Viper will instead

simply deactivate itself and display the message 'Viper deactivated'.

When deactivated, Viper is still in memory but not operational.  Typing

Viper again will reactivate it back to it's normal functionality and

you'll see a 'Viper reactivated' message displayed.

Viper is a well-behaved TSR.  If Viper can safely remove itself from

memory, it will honor your uninstall request.  If it can't, it won't.

Don't blame Viper.  The problem is being caused by other software

running on your system.  Well-behaved TSR's won't allow you to

uninstall them if it isn't safe to do so.

VP.DAT FILE                     Page 4

There is an optional file (VP.DAT) Viper will use if it finds it where

VP.EXE is located.  VP.DAT is simply an ASCII text file that allows you

to setup default DOS commands that will appear in the visual window

each time Viper is loaded.  Thus, if you reboot your computer many

times a day, you won't have to manually setup all of your personal DOS

commands that you wish to use.  If Viper does not find VP.DAT,

installation proceeds as normal and the visual window will contain all

records blanked out initially.

You may make multiple copies of the VP.DAT file and keep different

commands in each one.  Then when starting Viper, you can simply enter

Viper "VP3.DAT" <RETURN> and Viper will load into memory using the

commands from the VP3.DAT file.

If you wish to tag a record in this VP.DAT file as a KEEP record or

PLAYBACK record, use the following ASCII byte codes:

o To mark as a KEEP record.......: ASCII 04 decimal

o To mark as a PLAYBACK record...: ASCII 17 decimal

Place the code in column 1 of the VP.DAT file.  Most ASCII text editors

allow entering of these codes by holding down the ALT key while

pressing the decimal value on the numeric key pad.  For example,

holding down the ALT key and pressing the 4 key on the numeric key pad,

then releasing the ALT key will produce the ASCII 04 code at the cursor

position.  See page 17 on tagging records for more information.

You may have from 10 to 22 lines of text in Area 1 of the VP.DAT file.

You may have less than 10 lines and more than 22 lines are ignored.

Note that 1 space must precede the actual text.  This is because you

must allow for a TAG character in this first position even if you don't

use one.

Notice the # characters in the file.  These are special characters and

MUST always be in the file, flush left, column 1.

Here is a sample of some of the commands you might have in your VP.DAT

file.  The (---------- Area 1 ----------) text is for your

clarification only and would not actually be in the VP.DAT file:

VP.DAT File Format:

del a:\*.*                           (---------- Area 1 ----------)

copy *.dat a:\

format A: /U /F:1.2

chkdsk

# Next Lines: Function Key string commands F1 - F10

dir /w                               (---------- Area 2 ----------)

cls

format A: /U /F:1.2

# Next Lines:  Battery Watch strings  (---------- Area 3 ----------)

02:30

C:\VPBATT.DAT

# Next Line: /TrashCAN command string: COPY or PKZIP

copy                                  (---------- Area 4 ----------)

pkzip /a c:\trashcan\trash

VP.DAT FILE                     Page 5

(---------- Area 1 ----------): Commands entered into this area will

appear in the visual window itself as pre-loaded commands.  A minimum

of 10 lines, a maximum of 22.

(---------- Area 2 ----------): Commands entered into this area will be

assigned to function keys F1 through F10, in descending order as they

appear in this file.  For example:

F1 receives the string assignment: "dir /w"

F2 receives the string assignment: "cls"

F3 receives the string assignment: "format A: /U /F:1.2"

(---------- Area 3 ----------): Battery time and name of the file where

the battery time remaining string will be stored are entered in this

area.  See page 9 for more information.

(---------- Area 4 ----------): If you enable VP's TrashCAN feature by

invoking the /TCAN command line switch at Viper's start up, you may

choose the method of how the DELeted file is moved or copied into the

C:\TRASHCAN subdirectory.  Only one line is read by VP upon start up in

Area 4, thus in this case, the COPY command would be used.  For

convenience, you may keep other line commands after the top line in

Area 4 such as shown; the PKZIP utility.  You can swap these two lines

and make PKZIP the active trashcan method when DELeting files.

Remember, you may use any utility you wish for this feature that does

what you want it to do with the file BEFORE VP actually invokes DOS's

DELete command.  See page 4 for more /TCAN information.

COMMAND LINE SWITCHES                Page 6

Viper's operational behavior may be modified to suit your own personal

style via switches during installation.  Listed below are the switches

you may use to alter Viper's operational behavior:

NOTE: Don't type in the optional bracket [] symbols.

VP.................... install/uninstall Viper.

VP "FILE"............. use this FILE instead of the VP.DAT file.

VP /D................. Don't keep visual window up after playback.

VP /C[L][R]........... set Cursor Left/Right flush on record: /CL.

VP /T................. enable sliding Tone when visual window pops up.

VP /F[1-4]............ set Fast cursor speeds: /F1 /F2 /F3 /F4.

VP /B................. keep lightbar on Bottom record position.

VP /R[10-22].......... set number of visual window Records: /R22.

VP /U[00-12].......... set visual window Upper row position: /U00.

VP /Wbg/fg............ set Window background/foreground colors: /W02/15.

VP /Lbg/fg............ set menu Lightbar background/foreground colors.

VP /S................. turn Snow removal on (eliminate screen flicker).

VP /G................. force b&w screen attributes (for Gas plasma/LCD).

VP /A................. use loud Alarm tone.

VP /M[F][M][S]........ use Mouse support. Double-click rate:Fast/Medium/Slow.

VP /X[1-9]............ eXclude 1-9 characters from being saved: /X2.

VP /BATT[1-5]......... enable BATTery watch in visual window: /BATT2.

VP /TCAN.............. copy deleted files to C:\TRASHCAN subdirectory.

VP /VB[1-2]........... use Video Buffer area 1 or 2 for visual window.

VP /On1/n2:n3/n4...... set Overtype/insert cursor sizes. See VP.DOC file.

VP /SI[N-F]........... Start up Insert mode: oN or ofF.

VP /? or /H........... this Help screen.

You may use the / \ or - character in front of each command line letter.

Viper will accept either one of these.

"FILE"............ Allows you to have different VP.DAT type files

available.  Just start Viper like this: VP

"VP2.DAT" <RETURN> and the visual window will use

data from file VP2.DAT.  BOTH DOUBLE QUOTE MARKS

ARE REQUIRED.

/D................ This switch will cause Viper to return you to the

DOS prompt instead of popping up the visual window

after automatically running any F6 Tagged records.

/C[L][R].......... This switch controls where the cursor positions

itself when the visual window is displayed and you

move the lightbar up and down.  L sets the cursor

flush left within the record.  R sets it flush

right.  No setting, (the default) causes the

cursor to remain in the same relative position.

COMMAND LINE SWITCHES cont...          Page 7

/T................ If you like the sound of the sliding tone each

time you pop-up the visual window, use the /T

switch to enable it.  Otherwise, there will be no

sound each time the visual window is popped-up.

/F[1-4]........... The /F switch will cause Viper to increase the

cursor speed when Viper is operating (at the DOS

prompt or when the visual window is up).  When the

/F switch is used, each time a program is started

by Viper, it will first restore the keyboard speed

to what it was before Viper took control and then

start the program.

There are 4 speed settings available:

F1 - 10 CPS 500 ms delay  F2 - 15 CPS 500 ms delay

F3 - 20 CPS 250 ms delay  F4 - 30 CPS 250 ms delay

If the /F switch is not used, Viper will have no affect

on cursor movement speed at all.

/B................ This switch will keep the lightbar on the bottom

record position each time you pop-up the visual

window, regardless of which record it was on the

last time the window was displayed.

/R[10-22]......... This switch allows you to determine the number of

visual window Records to be displayed.  The number

must be two digits (10, 12, 18) and cannot be less

than 10 or greater than 22.  The default number of

visual window records is 12.

/U[00-12]......... This switch allows you to place the visual window

vertically on the screen (within limits).  The

number must be two digits (04, 08, 12) and cannot

be greater than 18.  The default placement row is

08.

Viper will automatically adjust for incorrect /U

and /R settings.

/Wbg/fg........... This switch allows you to change the colors of the

visual Window.

/Lbg/fg            'bg' refers to the background color.  You must use

2 characters, for example, 00 or 07.  Background

colors must be limited to values between 00 thru

07.  'fg' means foreground and these color values

can range between 00 thru 15.

Viper automatically sets the attributes on systems

with monochrome video cards, thus, these switches

(/W and /L) are intended for use on systems with

color video cards only.

COMMAND LINE SWITCHES cont...          Page 8

Listed below is a color table for your reference:

Value  Color   Value  Color

----------------------------------------------

00    black    08    bright black (gray)

01    blue     09    bright blue

02    green    10    bright green

03    cyan     11    bright cyan

04    red      12    bright red

05    magenta  13    bright magenta

06    brown    14    yellow

07    white    15    bright white

/S................ Snow removal.  If you have an older CGA color

monitor, you can eliminate the screen flicker, or

snow as it is sometimes called, by using this

switch.  Note that while enabling the snow removal

code in Viper does eliminate the snow, it also

causes the screen display routines to operate

slower, therefore, don't use this switch unless

you have to.

/G................ Force black and white screen operation.  Use this

switch if you're running Viper on a Gas plasma or

LCD screen.  These type of screens (found on

laptops and notebook computers, among others) are

driven by a color video card but are attached to a

non-color screen.  This switch will force Viper to

operate in a B&W video mode thus allowing you to

easily see Viper on the screen.

/A................ Force use of the louder alarm tone when you use

the alarm-only feature (no records tagged as

PLAYBACK).  Different computer systems produce

different sound levels for the same type of tone,

thus you get some control and flexibility via this

switch.  The default setting is the lower tone.

/M[F][M][S]....... Force Viper to use internal mouse support code if

the mouse software driver is loaded before

starting Viper for the first time.  If this switch

is not used, regardless of whether or not a mouse

driver is loaded, Viper will not make use of the

mouse.  Immediately following the /M switch is an

optional setting for controlling the double-click

rate.  Fast/Medium/Slow.  No switch specifies the

Medium double-click rate.

COMMAND LINE SWITCHES cont...          Page 9

/X[1-9]........... This switch determines how many characters are

eXcluded from being saved in the visual window as

DOS commands are executed.  For example, /X2 would

cause all commands entered at the DOS prompt that

were 3 or more characters in length, to be saved

within the visual window.  Thus, ME or B or TC or

DC would not get saved.  The allowable range is 1

thru 9 characters.

/BATT[1-5]........ The BATTery Watch feature of Viper is in the form

of a digital readout on the top row of the visual

window when it's popped-up.  The digital reading

represents battery time remaining in hours and

minutes.

The BATTery Watch feature is an approximation

display only.  It is a manually set system,

initially set by you, based on the information

contained in your computer owner's manual on

expected battery life for your specific machine.

There are no DOS or BIOS calls for software to

"tap into" to determine accurate remaining battery

life in 90% of the portable computers manufactured

today.

If your computer manual indicates that a full

battery charge "should" last for 3 hours, you

could set "03:00" hours in the VP.DAT file (Area

3).  Viper in turn, based on this information,

will start it's battery watch display with 03:00

as hours remaining on the the digital BATTery

readout.

As you use your portable, you'll get a better feel

as to the "real world" expectations of battery

life for your specific machine and how you use it.

Many items affect battery life.  For example, you

may find that despite what your manual says, you

can't get 3 hours out of your batteries.  So, you

can learn to "adjust" and start setting Viper's

read out to 02:30 to reflect a more realistic

expectation of battery life for your machine.

Remember, you determine the battery setting for

Viper.  You can give yourself an extra margin of

safety by setting a slightly lower battery time.

For example, you could set 02:15 to provide an

extra 15 minute margin of safety.

[1-5]

The optional values 1 thru 5 represent how often,

in minutes, the "C:\VPBATT.DAT" file is written to

(updated) on the disk.  The default (no option) is

1 minute.  For example, /BATT3 would enable the

BATTery Watch feature and update the

"C:\VPBATT.DAT" file every 3 minutes on the disk.

COMMAND LINE SWITCHES cont...          Page 10

Viper keeps track internally of how much time has

passed each time you use Viper.  Once every minute

(or more if you've used the optional /BATTx

switch), Viper updates the BATTery Watch file

(C:\VPBATT.DAT) with current time remaining

information.  When you turn off your machine, the

information is already saved.  Upon starting Viper

the next time, Viper checks this battery file and

continues on from there.  So, over time, the

BATTery Watch readout provides a general estimate

of the time remaining for your batteries.

The following paragraphs instruct you how to

correctly set up the BATTery Watch feature:

SETTING THE BATTERY WATCH FEATURE THE FIRST TIME:

To initially set the BATTery readout feature of

Viper the first time, edit the VP.DAT file and set

the time you want the digital readout to reflect,

in this format: HH:MM.  Thus, 02:30 would

represent 2 hours and 30 minutes.

The next line contains the text string of the

filename Viper will save the time information in,

once every minute.  It's defaulted to

"C:\VPBATT.DAT" but you can use any valid DOS file

name.  Be sure to use a fully qualified path name

(include drive and path).

Now, when you start Viper by typing VP /BATT

<RETURN> (or an optional /BATT2 or /BATT3), Viper

will know that you want to use the BATTery Watch

readout feature.  It will try to find the file

"C:\VPBATT.DAT".  When it fails to find it, Viper

will assume you are requesting a fresh start on

it's digital display and you will see the initial

time string at the top of the visual window.

Each time you start Viper, if it finds this

"C:\VPBATT.DAT" file, it will check it and see if

the time remaining is greater than 0 minutes.  If

it is, Viper will just continue to monitor the

time as if Viper was never shut off.  If the time

remaining is down to 0, Viper will display 00:00,

indicating "no battery life" remaining.

COMMAND LINE SWITCHES cont...          Page 11

SETTING A NEW BATTERY WATCH TIME:

After you have re-charged your computer's

batteries, you'll want to re-set Viper's digital

BATTery readout to "full" again.  To do this,

simply re-check the time setting in the VP.DAT

file to make sure the time is what you want to

start with.  Then delete or erase the

"C:\VPBATT.DAT" file from the disk (NOT THE VP.DAT

FILE).  Now, when you start Viper, it will NOT

find the "C:\VPBATT.DAT" file and assume you are

starting from a full, fresh battery charge.

/TCAN............. TCAN invokes the "TrashCAN" feature of Viper.

This will copy file(s) to a subdirectory named

TRASHCAN on your C: drive when you use the DEL

command.  AFTER the file(s) are copied into this

C:\TRASHCAN subdirectory, VP will pass the DELete

command on to DOS.  Thus, TCAN is an additional

safety feature for not losing files by

accidentally deleting them.  If your disk space is

short, you may elect to use the PKZIP or some

other file compression utility to pack the deleted

file(s) into the C:\TRASHCAN subdirectory.

/VB[1-2].......... use normally spare Video Buffer areas 1 or 2:

MONOCHROME VIDEO SYSTEMS:

Viper will normally use space within its own memory image as

internal buffer storage.  The visual window records and

function key strings are also stored in this internal buffer

area.  However, this internal buffer area adds to the overall

memory requirements of Viper.

ON COLOR VIDEO SYSTEMS:

Occasionally, on a color system, you'll notice that when an

application uses these spare video pages you'll pop up Viper

and all of your commands will be gone.  The visual window

will have the video characters displayed from the video page

that the application made use of.  These characters can be

almost anything, but will always be a combination of readable

characters and "junk" characters.  You can now do one of two

things:

1. Re-boot your machine to re-read the VP.DAT

strings back into Viper and don't use the /VB1 or

/VB2 switches if you plan on using the same

application.

or

2. Press CTRL-ALT-V to re-clear and reset to all blanks the

visual window and function key string buffers and continue

to work as usual.

NOTE: To minimize Viper's memory requirements, when the above

mentioned Video Buffer switch is used, this forces

Viper to use the available memory areas contained

within the color video card itself.  This reduces

Viper's memory requirements on a color system by

approximately 8+K.

COMMAND LINE SWITCHES cont...          Page 12

/On1/n2:n3/n4..... In order to make use of the 'cursor sizing'

switch, a basic understanding of cursor scan lines

are in order.

If you look closely enough, you'll see that the

cursor is actually made up of very tiny horizontal

lines.  These lines are called scan lines.  The

following diagram shows a close-up view all of the

scan lines available on color and monochrome

monitors:

Monochrome System               Color System

xxxxxxxxxxx  scan line 00     xxxxxxxxxxx  scan line 00

xxxxxxxxxxx  scan line 01     xxxxxxxxxxx  scan line 01

xxxxxxxxxxx  scan line 02     xxxxxxxxxxx  scan line 02

xxxxxxxxxxx  scan line 03     xxxxxxxxxxx  scan line 03

xxxxxxxxxxx  scan line 04     xxxxxxxxxxx  scan line 04

xxxxxxxxxxx  scan line 05     xxxxxxxxxxx  scan line 05

xxxxxxxxxxx  scan line 06     xxxxxxxxxxx  scan line 06

xxxxxxxxxxx  scan line 07     xxxxxxxxxxx  scan line 07

xxxxxxxxxxx  scan line 08

xxxxxxxxxxx  scan line 09

xxxxxxxxxxx  scan line 10

xxxxxxxxxxx  scan line 11

xxxxxxxxxxx  scan line 12

xxxxxxxxxxx  scan line 13

Notice that the scan lines are numbered from 00 at

the top thru 07 or 13 at the bottom.  The normal

default cursor scan lines for a color monitor are

scan lines 06 and 07 being visible.  For

monochrome, it's 12 and 13.  If you look closely

at your current cursor, you may see these tiny two

scan lines.  Line 06 or 12 is called the STARTING

scan line and line 07 or 13 is called the ENDING

scan line.

'n1' represents the starting scan line for

overtype mode.  'n2' represents the ending scan

line for overtype mode.  The default values for

color are 'n1' = 06 and 'n2' = 07.  The default

values for mono are 'n1' = 12 and 'n2' = 13.

If you don't like the normal cursor size (size

actually refers to the scan lines) you can change

them in Viper.

COMMAND LINE SWITCHES cont...          Page 13

For example, if you want a bigger cursor, perhaps

a 1/2 block size, then for COLOR systems, you

would specify it like this:  /O04/07 (for

overtype).  This example would have the starting

cursor scan line begin at line 04 and include scan

lines thru scan line 07, which is the ending scan

line value.

If you wanted the cursor as big as possible, you

would do it this way: /O00/07.  In this case, all

scan lines would be visible.

Viper uses this last example as it's default

overtype cursor size.  This means that when you

press the INS key to toggle insert OFF, you'll see

the cursor change to it's maximum size.  'n3' and

'n4' represent the starting and ending scan lines

respectively, for the insert mode.

/SI[N-F].......... Start up Insert mode: You can control what insert

state Viper starts up with: use N for insert oN or

use F for insert ofF.

/? or /H.......... This switch will cause Viper to display a help

screen showing every command line switch available

to start Viper with.

THE VISUAL WINDOW                  Page 14

The visual window is what pops-up when you press the up/down arrow

keys, or the PgUp/PgDn keys or click the left mouse button when you're

at the DOS prompt.  This visual window contains 12 records as the

default number of records.  Each record holds a DOS command (or can

hold a DOS command, since a record can be blank).

Pressing the Up or Down arrow key simply pops-up Viper's visual window

with the lightbar on the last selected record position.  If you pop-up

with the PgUp key the lightbar will be positioned to the top record

position.  If you pop-up using the PgDn key, the lightbar will be

positioned to the bottom record position.

Whenever this visual window is displayed, it is always in the edit mode

for the record the lightbar is on.  This means that you may immediately

have full use of all editing capabilities Viper has (described on page

15).  You can move the lightbar up and down by simply pressing the up

and down arrow keys to position the lightbar on the record containing

the DOS command you are interested in.  You may move the mouse cursor

to a record, click the left mouse button and the lightbar will position

itself to that record with the editing cursor placed at the mouse

cursor position.  PgUp and PgDn will quickly move the lightbar to the

top and bottom record, respectively.

Pressing the <RETURN> key or double-clicking the left mouse button will

cause the command the lightbar is on to be executed.  You can also

press the ESC key or the right mouse button to return to the DOS prompt

without executing any commands.

The visual window can also be used to type in your DOS commands

directly.  In other words, you don't have to always be at the DOS

prompt to type in DOS commands.  You can pop-up the visual window, move

the lightbar to a blank record, enter a DOS command and then press the

<RETURN> key to execute it.  The command will stay in the visual window

for future use.

Viper also has the ability to group multiple commands in a single

visual record.  To do this, simply separate the commands with a

semi-colon character or use CTRL-T.  For example:

cls;dir /w;chkdsk

When you press the <RETURN> key on this record, Viper will clear the

screen, do a DIR /W command and then execute the CHKDSK program.  This

feature permits you to group commands on the same record for execution

and not have to use additional records for each single command by

themselves.  You may also run programs this way and/or .BAT files if

other "chores" need to be run first or afterwards.  To actually use the

semicolon within a command and not have Viper interpret it as a command

separator, use it back to back like this: ;;

In support of laptop and notebook computers, Viper has the ability to

turn the Print Screen capability ON and OFF.  This feature prevents

Print Screen dumps from accidentally occurring.  You'll see the

"PTRSCRN" characters in the top part of the visual window.  ALT-P

(while the visual window is up) will toggle the Print Screen feature On

and OFF.  "PTRSCRN" indicates ON and "ptrscrn" indicates OFF.

EDITING COMMANDS                  Page 15

o Visual Window Record Editing:

The visual window always pops-up in the edit mode.  You may

immediately edit the text (commands) in any record the lightbar

is on.

All editing commands are the same as when editing at the DOS

prompt.

o right arrow   - moves the cursor to the right one character.

o left  arrow   - moves the cursor to the left one character.

o TAB           - moves the cursor to the right one word at a time.

o shift TAB     - moves the cursor to the left one word at a time.

o CTRL RT Arrow - moves the cursor to the right one word at a time.

o CTRL LF Arrow - moves the cursor to the left one word at a time.

o CTRL-B        - blank  all text from the current record

o CTRL-Y        - delete all text from the current record

o CTRL-D        - delete text from cursor position to end of line

o ALT-D         - delete text from cursor position to end of line

o CTRL-T        - multiple command separator.  Optionally used in place

of the semicolon character.

o DEL           - deletes the character the cursor is on.

o BS            - backspace.  Deletes the character to left of cursor.

o INS           - toggles the insert mode ON and OFF.

o ESC           - clears current command, resets cursor, DOS prompt only.

FUNCTION KEY BUFFER STRINGS              Page 16

o "F2=Funct Keys":

Pressing the F2 key will cause the 10 function key strings to

be displayed in the first 10 visual window record positions.

At the top of the visual window, "FUNCTION KEYS" will be

displayed as a reminder that you have the function key strings

displayed in the visual window.  Along the left side of the

window you'll see numbers that maps each string to their

associated function key.

You may edit any of function key strings while they are in the

visual window.  After editing or viewing them, press F2 or

ESC to swap out the function key strings with the normal

visual window record strings.  You may display (only) the

function key strings by pressing ALT-F or ALT-Z from the DOS

prompt.  This will list all ten function key assignment

strings.  Note that you do not have to make use of all 10

function keys.  You may set and use whatever number you need.

Note: As a closer key, ALT-Z will do the same thing as ALT-F

from the DOS prompt.  The ALT-Z keys are closer together on the

keyboard and may be much faster to use on laptop and notebook

computers.

COPY AND PASTE

o "F3=Copy" Function Key:

Pressing the F3 key will copy the lightbar record text or the

function key text into an internal clipboard buffer.  A beep

will confirm that the text got copied into Viper's clipboard

buffer.

o "F4=Paste" Function Key:

Pressing the F4 key will "paste" the text from the clipboard

buffer into the current lightbar record or the function key

number where the lightbar is positioned, thus allowing you to

copy text from one area into another.

The same operation can be done via the mouse:

1. Move the mouse cursor to the record to be copied.

Click the left mouse button.  The lightbar now positions

to this record.

2. Move the mouse cursor to the "F3=Copy" inverse video box.

Click the left mouse button.  Viper's internal clipboard

buffer now holds a copy of the text.

3. Move the mouse cursor to the record you want to paste

to.  Click the left mouse button.  The lightbar now

positions to this record.

4. Move the mouse cursor to the "F4=Paste" inverse video

box.  Click the left mouse button.  The text in the

clipboard buffer will now get copied into the lightbar

record.

KEEP AND PLAYBACK RECORDS             Page 17

o "F5=Keep" Function Key:

Pressing the F5 key will toggle the highlighted record between

a tagged and untagged KEEP record state.  Each time you enter a

command from the DOS prompt, that command is copied into the

bottom record position within the visual window.  Except if you

started Viper with a valid /X (eXclude) command line switch.

Each previous record is "bumped up" one position within the

window.  Thus, at some point, the record at the top will get

"pushed off" and lost.

If you have a command that you wish to always KEEP (never to

get pushed off the top), tag that record as a KEEP record by

pressing the F5 key or click the left mouse button on the lower

inverse video box "F5=Tag Keep".  A diamond symbol will appear

in the left portion of that record, visually indicating that

this record is now tagged as a "KEEP me around record" and it

will not get pushed off the top.  Remember that you may

pre-load commands and TAG them by adding them to the VP.DAT

file with an editor.  Note that you cannot tag function key

text strings since that serves no purpose.

Pressing "ALT-F5" will clear ALL F5 tagged records at once.

o "F6=Play" Function Key:

Pressing the F6 key or clicking the left mouse button in the

lower inverse video box "F6=Tag Play" will toggle the

highlighted record between a tagged and untagged PLAYBACK

record state.  PLAYBACK records are records that you may

manually start executing when you press either the ALT-F6 keys

or click the left mouse button on the lower inverse video box

"ALT F6=Playback" within the visual window.

NOTE: F6 tagged records also have the same characteristics that

"F5 Keep" records have.  That is, an F6 tagged record

will also be kept around and not get pushed off the top

if it is tagged as a Playback record.  The difference is

that an F6 tagged record can additionally be manually

played back by using the ALT F6 keys.  A record can only

have one tagged state: by either the F5 or the F6 key.

o "ALTF6=Playback (MANUAL)":

Combined with F6 tagged PLAYBACK records, Viper can be used to

execute a series of DOS commands (manually) by simply pressing

the ALT-F6 keys.  When you do this, all F6 tagged PLAYBACK

records will be executed starting with the top tagged record in

the visual window.

Viper can also be used to execute a series of DOS commands

while your computer is un-attended.  In the upper right corner

of the visual window is a Date and Time field.  At start-up,

this field displays the string "MM-DD-YY HH:MM" to indicate the

format expected.

AUTO PLAYBACK FEATURE                Page 18

The AUTO PLAYBACK feature uses the date and time set by you to

tell it when to start executing any record(s) tagged as

PLAYBACK records.  AUTO PLAYBACK uses DOS's date and time, so

be sure you have your DOS date and DOS time set correctly.

Here's how to use AUTO PLAYBACK:

(1) Move the lightbar to each record you want Viper to execute

and press the F6 key or click the left mouse button on the

lower inverse video box "F6=Playback".

(2) Move the lightbar to a blank record (or a record already

containing some text.  It's okay to overwrite the text.

Viper reads only the first 14 characters).

We make use of a record as a "scratch pad" to provide the

string input for the date and time format.

(3) Enter the Date and Time you want AUTO PLAYBACK to start

executing.  Be sure to use the format: MM-DD-YY HH:MM

Notice there is no provisions for entering seconds.

You must use a 24-hour time format, listed here for your

reference:

When it's      It's         When it's      It's

(12 hour     (24 hour       (12 hour     (24 hour

time here)   time here)     time here)   time here)

----------------------------------------------------

00:00 AM .... 00:00         12:00 PM .... 13:00

01:00 AM .... 01:00         01:00 PM .... 13:00

02:00 AM .... 02:00         02:00 PM .... 14:00

03:00 AM .... 03:00         03:00 PM .... 15:00

04:00 AM .... 04:00         04:00 PM .... 16:00

05:00 AM .... 05:00         05:00 PM .... 17:00

06:00 AM .... 06:00         06:00 PM .... 18:00

07:00 AM .... 07:00         07:00 PM .... 19:00

08:00 AM .... 08:00         08:00 PM .... 20:00

09:00 AM .... 09:00         09:00 PM .... 21:00

10:00 AM .... 10:00         10:00 PM .... 22:00

11:00 AM .... 11:00         11:00 PM .... 23:00

(4) "F7=Timer" Function Key:

Pressing the F7=Timer key will cause Viper to read the date

and time string in the highlighted record and set it's

internal timer.  The date and time you entered will now

appear in the upper right corner of the visual window.

This verifies that this date and time is now set internally

in Viper.

If there were any syntax errors in your string, the message

" SYNTAX ERROR " would have appeared in the upper right

corner.  No syntax errors means the date and time string

format was correct and is now set.

AUTO PLAYBACK FEATURE cont...          Page 19

(5) AUTO PLAYBACK will only execute from the DOS prompt, so

press the ESC key or click the right mouse button to remove

the visual window and return to the DOS prompt.  Now, at

the date and time set, all records tagged with a PLAYBACK

symbol will get executed.

(6) CANCELING: To cancel a date and time already set, move the

lightbar to a blank record and press F7 again or click the

left mouse button on the lower inverse video box

"F7=Timer".  This will clear the date and time string from

the upper right corner of the window and turn off the

internal timer.  Verification of the timer going off is the

format string re-appearing in the upper right corner:

"MM-DD-YY HH:MM".

ADDITIONAL NOTES:

o You can tell if the internal timer is set because the date

and time it will trigger on will appear in the upper right

corner.  If the timer is not set, there will be only one of

the following strings displayed:

o MM-DD-YY HH:MM   (format reminder)

o SYNTAX ERROR

o RUN WAS MISSED   (the visual window was up)

o RUN FINISHED

o ALARM SOUNDED    (alarm only - no records tagged)

o After you set a date and time for AUTO PLAYBACK, nothing will

ever happen until you ESC back to the DOS prompt.  Remember,

AUTO PLAYBACK only triggers into action from the DOS prompt.

If you have a program running during the trigger time or you

have the visual window up, Viper will not execute your tagged

records.

You may continue to use Viper as you normally would after

setting a date and time, but as the trigger time nears, be

sure your machine is idle at the DOS prompt so the PLAYBACK

records can be executed.

o "F8=DOS Date":

This key is used to set DOS's current DATE into the AUTO

PLAYBACK display when you load a Date/Time string from the

VP.DAT file.

EXAMPLE: Suppose you have the following string in the top

record of the VP.DAT file - "06-15-92 21:00".  Every day that

you start up Viper, this record is initially prepared to set

the AUTO PLAYBACK date and time for any records you have

tagged with the "F6=Play" key.  But since this date comes out

of the VP.DAT file, this date would have to be changed each

day to the current DOS date.  You can either type in today's

date into the lightbar record or the easier way to do it is

to just press the F8 key (or click in the "F8=DOS Date" box)

and the date in the upper right-hand window will change to

today's DOS date automatically.  Regardless of the date in

the string coming out of the VP.DAT file, pressing F8 will

set TODAYS DATE and the string time.

ALARM FEATURE                    Page 20

Note that "F8=DOS Date" can be used in lieu of "F7=Timer" to

automatically set the AUTO PLAYBACK feature using today's

date instead of the date contained in the record string from

the VP.DAT file.  The time will come from the lightbar record

format that appears in the VP.DAT file (if you set one).

o You can have a date and time set and no records tagged for

PLAYBACK.  In this case, Viper will just activate a special

"alarm tone".  You can use this feature as an appointment

alarm clock or other reminder.

To use the Alarm feature, simply set your date and time as

you normally would but don't tag any records with the F6

Playback key.

Viper will sense that no PLAYBACK records are tagged and

sound the alarm tone instead.  The next time you pop-up the

visual window, you'll see "ALARM SOUNDED " in the upper right

corner of the visual window.  Move the lightbar to a blank

record and press the F7 key or click the left mouse button on

the lower inverse video box "F7=Timer".  This will clear this

message and reset the alarm.

Alarm Hint:

Since only the first 14 characters of a record are read by

Viper when you press the F7 key, use the rest of the record

to make a note to yourself of what the appointment time is

for.

For example, your alarm record might look like this:

06-15-92 11:30  Meet with Ralph on the sales meeting.

MOUSE SUPPORT                    Page 21

To prepare Viper for mouse use, load your mouse software driver first.

Then install Viper using the /MF, /MM or /MS switch.

Buttons available at the DOS prompt:

1. Click left mouse button to pop-up the visual window.

Buttons available from the visual window:

1. Move the mouse cursor to the record row you want and click

the left button.  The lightbar will move to that record

row and the editing cursor will move to the same column

position that the mouse cursor is on.

2. Double-click the left mouse button to execute the command

the lightbar is on.

3. With the record already highlighted by the lightbar:

Click the left mouse button of any one of the inverse

video boxes at the bottom of the visual window to cause

that action to occur.

4. Click the right mouse button to exit the visual window.

BATTERY MANAGEMENT TIPS FOR LAPTOPS AND NOTEBOOKS

1. Screen: The screen on your portable computer is the biggest drain on

your batteries.  Try to run the screen in the normal mode vs inverse

mode.  The inverse mode (black characters on a white background) can

decrease battery life by as much as 25%.

Turn the screen brightness down as low as possible for comfortable

viewing.  The brighter the screen, the more battery power it

requires.

2. Hard Disk: If you have an option to control the power down interval

for your hard disk, you may have to try different intervals to

achieve a good balance between when the hard disk should power down

vs keeping it running.  This will depend mostly on the type of

software you're using (and thus how often it goes to the disk).

Generally, it takes almost 3 times as much power to start your hard

disk up again as it does to just keep it running.

The best solution is to try and make as much use of a RAM disk if

you can.  Using any RAM disk takes less power than using the hard

disk.

3. CPU Speed: It varies by quite an amount, depending on which portable

computer you have, but, if you can slow down your CPU speed, you

will gain some additional battery life.  This increase in battery

life can range from 5 percent up to 30 percent.

4. Modem and Speaker: If possible, turn off your internal modem and the

speaker, when you can.  This will save battery power.

5. Battery Charging: Whenever possible, let your batteries charge over-

night.  "Quick charging" your batteries in most cases will only

bring them up to 80-90 percent of their rated capacity.

REGISTRATION FORM                  Page 22

Viper is being distributed as a Shareware product. Shareware is a

marketing method only.  It allows you to "try before you buy".

Shareware is not Freeware or Public Domain software.  If you use Viper,

you are expected to pay for it.

We don't believe in releasing special versions of Shareware until you

decide to register for the "real" product.  Examples of this type of

software are: limited or demo versions of a product, Bannerware

versions and other types of nuisance reminders that you haven't

registered your copy yet.  We trust that if you find Viper to be useful

to you, you will support our efforts in bringing Viper to the

marketplace by registering your copy.  The version you have now is a

full release version of Viper with no shareware reminders other than

what's contained in this VP.DOC file.

Viper is a fully copyrighted, commercial quality product.  The only way

for us to keep improving Viper and to advise you of future changes

(enhancements, added features, etc.) is for you to support it by

registering your copy.  The registration fee is $20.00. Thank you.

+--------------------------- REGISTRATION FORM ------------------------------+

|                                                                            |

|Name:_______________________________________________________________________|

|                                                                            |

|Title:______________________________________________________________________|

|                                                                            |

|Address:____________________________________________________________________|

|                                                                            |

|City:__________________________________ State:_____________ Zip:____________|

|                                                                            |

|Work Phone:____________________________ Home Phone:_________________________|

|                                                                            |

|Viper Version:____________  Date Received:________________                  |

|                                                                            |

|                                                                            |

|Comments:___________________________________________________________________|

|                                                                            |

|____________________________________________________________________________|

|                                                                            |

|____________________________________________________________________________|

|                                                                            |

|____________________________________________________________________________|

|                                                                            |

|                                                                            |

+----------------------------------------------------------------------------+

Please mail this form to:

Freshour Software

Viper Registration

531 Main Street #306

El Segundo, CA 90245

310-640-1649

TECHNICAL SUPPORT                  Page 23

We provide unlimited non-toll free telephone technical support.  This

means that when you call, you are paying for the call.  Call

310-640-1649, west coast time.  We'll be happy to answer your

questions.  Note: Technical support is provided for unregistered users

as well.  We strongly believe in supporting Viper.

HARDWARE AND SOFTWARE REQUIREMENTS

Hardware: IBM-PC/XT/286/386/486/PS2 computer systems or compatible clone.

Video system: Mono/CGA/EGA/VGA.

Software: PC-DOS/MS-DOS 3.10 or higher.

Viper runs in text video modes 2, 3 and 7, in 80 columns.

VIPER LICENSE AGREEMENT

GRANT OF LICENSE:

Freshour Software grants to the legal purchaser of Viper the right to

use this copy of Viper on a single CPU at a time.  Viper is copyrighted

(C) and owned by Freshour Software and is protected by United States

copyright laws.  You may make backup copies of Viper to protect your

purchase.

DISCLAIMER:

Viper and any associated support files, including documentation files,

are provided "as is" without warranty of any kind.  The entire risk as

to the results and performance of Viper and any associated files is

assumed by you.

Freshour Software disclaims all warranties, either express or implied,

including but not limited to implied warranties of merchantability and

fitness for a particular purpose, with respect to the Viper software

product itself, the diskette it came on and any associated files and/or

materials.

In no event shall Freshour Software be liable for any damages

whatsoever (including without limitation damages for loss of business

profits, business interruption, loss of business information, and the

like), arising out of the use or the inability to use, this product,

even if Freshour Software has been so advised of such damages.

INDEX                        Page 24

ALT-CTRL-V (clear/reset video buffer area)................... 11

ALT-F5 (clear all tagged lightbar records)................... 17

ALT-F6 (execute auto playback)............................... 17

ALT-F (display function key strings at DOS prompt)........... 16

ALT-P.(Toggle Print Screen ON/OFF)........................... 14

ALT-Z (display function key strings at DOS prompt)........... 16

Automatic Playback, F7....................................... 18

Alarm Feature................................................ 20

BATTery Watch Feature........................................ 09

Battery Management Tips For Laptops and Notebooks............ 21

Canceling Automatic Playback................................. 19

Command Line Switches:

"FILE".................................................... 06

/D........................................................ 06

/C[L][R].................................................. 06

/T........................................................ 07

/F[1-4]................................................... 07

/B........................................................ 07

/R[10-22]................................................. 07

/U[00-12]................................................. 07

/Wbg/fg................................................... 07

/Lbg/fg................................................... 07

/S........................................................ 08

/G........................................................ 08

/A........................................................ 08

/M[F][M][S]............................................... 08

/X[1-9]................................................... 09

/BATT[1-5]................................................ 09

/TCAN..................................................... 11

/VB[1-2].................................................. 11

/On1/n2:n3/n4............................................. 12

/SI[N-F].................................................. 13

CTRL-ALT-V (clear/reset video buffer area)................... 11

Color Settings............................................... 07

Copy Operations, F3.......................................... 16

Cursor Shape Options......................................... 12

Cursor Left/Right Positions Within Visual Window............. 06

DOS Date, F8................................................. 20

Don't Keep Visual Window Up After Playback................... 06

Editing Commands............................................. 15

eXcluding Characters To Be Saved............................. 09

Fast Cursor Speed Settings................................... 07

F2-Funct Keys................................................ 16

F3-Copy...................................................... 16

F4-Paste..................................................... 16

F5-Keep...................................................... 17

F6-Playback.................................................. 17

F7-Timer..................................................... 18

F8-DOS Date.................................................. 19

Function Keys................................................ 16

Gas Plasma/LCD Screens: Laptops and Notebooks................ 08

Hardware/Software Requirements............................... 23

Help Screen (VP /?).......................................... 13

Installing Viper For DOS..................................... 02

Installing Viper For Windows................................. 02

Internal Timer Messages (automatic playback)................. 19

INDEX                        Page 25

KEEP Tag, F5................................................. 17

Keep Lightbar On Bottom Record Position...................... 07

VP.DAT File.................................................. 04

"Viper Deactivated" Message.................................. 03

"Viper Activated" Message.................................... 03

Viper Help Screen (VP /?).................................... 13

License Agreement............................................ 23

MM-DD-YY HH:MM Internal Timer Format Syntax.................. 18

Manual Playback, ALT F6...................................... 17

Mouse Support................................................ 21

Number of Lightbar Records................................... 07

Other VP.DAT Files........................................... 04

Paste Operations, F4......................................... 16

PLAY Tag, F6................................................. 17

Portable Computer Features of Viper

o BATTery Watch System................................... 09

o Gas Plasma/LCD Screen Settings......................... 08

o Cursor Sizing Options.................................. 12

o Print Screen ON/OFF (ALT-P)............................ 14

Registration Form............................................ 22

Semicolons In Visual Window (multiple commands executed)..... 14

Software/Hardware Requirements............................... 23

Snow (screen flicker) Removal................................ 08

Sound Tone /T................................................ 07

Tagged Records:

KEEP Tags F5............................................. 17

PLAYBACK Tags F6......................................... 17

Automatic Playback of Tagged Records ALT-F6.............. 17

Manual Playback of Tagged Records........................ 17

Technical Support............................................ 23

Upper Row Positioning For Visual Window...................... 07

Uninstalling Viper........................................... 03

Visual Window:

ALT-F5 Clear All Tagged Records.......................... 17

ALT F6-Playback (Manual)................................. 17

ALT-P  Toggle Print Screen ON/OFF........................ 14

Audible Sliding Tone Sound............................... 07

Cursor Positioning Within Record (left/right flush)...... 06

Editing Commands......................................... 15

F2-Funct Keys............................................ 16

F3-Copy.................................................. 16

F4-Paste................................................. 16

F5-Keep.................................................. 17

F6-Playback.............................................. 17

F7-Timer................................................. 18

F8-DOS Date.............................................. 19

General Visual Window Information........................ 14

Lightbar Colors.......................................... 07

Semicolons............................................... 14

Upper Visual Window Row Positioning...................... 07

Visual Window Colors..................................... 07

Windows...................................................... 02

Windows is a tm of Microsoft