i

CHAPTER 0.1 - NUMBERS AND ARITHMETIC

You don't habitually use the base two system to balance your

checkbook, so it would be counterproductive to teach you machine

arithmetic on a base two system. What number systems have you had

a lot of experience with? The base 10 system springs to mind. I'm

going to show you what happens on a base 10 system so you will

understand the structure of what happens with computer

arithmetic.

BASE 10 MACHINE

Each place inside the microprocessor that can hold a number is

called a REGISTER. Normally there are a dozen or so of these. Our

base 10 machine has 4 digit registers.  They can represent any

number from 0000 to 9999. They are exactly like an industrial

counters or the counters on your tape machines.{1} If you add 27

to a register, the microprocessor counts forward 27; if you

subtract 153 from a register, the microprocessor counts backwards

153.   Every time you add 1 to a register, it increments by 1 -

that is 0245, 0246, 0247, 0248. Every time you subtract 1 from a

register, it decrements by 1 - that is 3480, 3479, 3478, 3477.

Let's do some more incrementing.  9997, 9998, 9999, 0000, 0001,

0002. Whoops! That's a problem. When the register reaches 9999

and we add 1, it changes to 0000, not 10,000. How can we tell the

difference between 0000 and 10,000? We can't without a little

help from the CPU.{2}  Immediately after an arithmetical

operation, the CPU knows whether you have gone through 10,000

(9999->0000). The CPU has something called a carry flag. It is

internal to the CPU and can have the value 0 or 1. After each

arithmetical operation, the CPU sets the CARRY FLAG to 1 if you

went through the 9999/0000 boundary, and sets the carry flag to 0

if you didn't.{3}

Here are some examples, showing addition, the result, and the

carry flag. The carry flag is normally abbreviated by CF.

number 1       number 2        result     CF

0289           4782           5071      0

4398           2964           7382      0

8177           5826           4003      1

____________________

1. Exactly like industrial counters that have several hundred

thousand parts, that is.

2. The CPU (central processing unit) is the chip(s) that does

all the arithmetic. In the case of the PC, it is the 8086.

3. When you set a flag to 0, it is called CLEARING the flag.

______________________

The PC Assembler Tutor - Copyright (C) 1989 Chuck Nelson

The PC Assembler Tutor                                         ii

______________________

6744           4208           0952      1

Note that you must check the carry flag immediately after the

arithmetical operation. If you wait, the CPU will reset it after

the next arithmetical operation.

Now let's do some decrementing. 0003, 0002, 0001, 0000, 9999,

9998. Golly gosh! Another problem. When we got to 0000, rather

than getting -1, -2, we got 9999, 9998. Apparently 9999 stands

for -1, 9998 stands for -2. Yes, that's the system on this, on

the 8086, and on all computers. (Back to that in a moment.) How

do we tell that the number went through 0 ; i.e. 0000->9999? The

carry flag comes to the rescue again. If the number goes through

the 9999/0000 boundary in either direction, the CPU sets the CF

to 1; if it doesn't, the CPU sets the CF to 0. Here's some

subtraction, with the result and the carry flag.

number 1       number 2       result     CF

8473           2752           5721      0

2836           4583           1747      1

0654           9281           8627      1

9281           0654           8627      0

Look at examples 3 and 4. The numbers are reversed. The results

are the same but they have different signs. But that is as it

should be. When you reverse the order in a subtraction, you get

the same absolute value, only a different sign (15 - 7 = 8 but

7 - 15 = -8). Remember, the CF is reliable only immediately after

the operation.

NEGATIVE NUMBERS

The negative numbers go 9999=-1, 9998=-2, 9997=-3, 9996=-4,

9995=-5 etc. A more negative number is denoted by a smaller

number in the register; -5 = 10,000 -5 = 9995; -498 = 10,000 -498

= 9502, and in general, -x = 10,000 -x. Here are some negative

numbers and their representations on our machine.

number     machine no              number     machine no

-27          9973                -4652          5348

-8916          1084                -6155          3845

As you will notice, these numbers look exactly the same as the

unsigned numbers. They ARE exactly the same as the unsigned

numbers. The machine has no way of knowing whether a number in a

register is signed or unsigned. Unlike BASIC or PASCAL which will

complain whenever you try to use a number in an incorrect way,

the machine will let you do it. This is the power and the curse

of machine language. You are in complete control. It is your

responsibility to keep track of whether a number is signed or

unsigned.

Which signed numbers should be positive and which negative? This

has already been decided for you by the computer, but let's think

Chapter 0.1 - Numbers and Arithmetic                          iii

____________________________________

out what a reasonable solution might be. We could have from 0000

to 8000 positive and from 9999 to 8001 negative, but that would

give us 8001 positive numbers and 1999 negative numbers. That

seems unbalanced. More importantly, if we take -(3279) the

machine will give us 6721, which is a POSITIVE number. We don't

want that. For reasons of symmetry, the positive numbers are

0000-4999 and the negative numbers are 9999-5000.{4} Our most

negative number is -5000 = 10,000 -5000 = 5000.

10'S COMPLEMENT

It's time for a digression. If we are going to be using negative

numbers like -(473), changing from an external number to an

internal number is going to be a bother: i.e. -473 -> 9527. Going

the other way is going to be a pain too: i.e. 9527 -> -473. Well,

it would be a problem except that we have some help.

0000 =    10,000    =     9999     +1

- 473

result                        9526     +1   = 9527

Let's work this through carefully. On our machine, 0000  and

10000 (9999+1) are the same thing, so 0 - 473 is the same as

9999+1-473 which is the same as 9999-473+1. But when we have all

9s, this is a cinch. We never have to borrow - all we have to do

is subtract each digit from 9 and then add 1 to the total. We may

have to carry at the end, but that is a lot better than all those

borrows. We'll do a few examples:

(-4276)

0000 =    10,000    =     9999     +1

-4276

result                        5723     +1   = 5724

(-3982)

0000 =    10,000    =     9999     +1

-3982

result                        6017     +1   = 6018

(-2400)

0000 =    10,000    =     9999     +1

-2400

result                        7599     +1   = 7600

(-1989)

0000 =    10,000    =     9999     +1

____________________

4. That way, if we tell the machine that we are working with

signed numbers, all it has to do is look at the left digit. If

the digit is 5-9, we have a negative number, if it is 0-4, we

have a positive number. Note that 0000 is considered to be

positive. This is true on all computers.

The PC Assembler Tutor                                         iv

______________________

-1989

result                        8010     +1   = 8011

This is called 10s complement. Subtract each digit from 9, then

add 1 to the total. One thing we should check is whether we get

the same number back if we negate the negative result; i.e. does

-(-1989)) = 1989?  From the last example, we see that -1989 =

8011, so:

(-8011)

0000 =    10,000    =     9999     +1

-8011

result                        1988     +1   = 1989

It seems to work. In fact, it always works. See the footnote for

the proof.{5} You are going to use this from time to time, so you

might as well practice some. Here are 10 numbers to put into 10s

complement form. The answers are in the footnote. (1) -628, (2)

-4194, (3) -9983, (4) -1288, (5) -4058, (6) -6952, (7) -162, (8)

-9, (9) -2744, (10) -5000.{6}

The computer keeps track of whether a number is positive or

negative. After an arithmetical operation, it sets a flag to tell

whether the result is positive or negative. This flag has no

meaning if you are using unsigned numbers. The computer is

saying, "If the last arithmetical operation was with signed

numbers, then this is the sign of the result." The flag is called

the sign flag (SF). It is 0 if the number is positive and 1 if

the number is negative. Let's decrement again and look at both

the sign flag and carry flag.

NUMBER    SIGN     CARRY

3         0         0

2         0         0

1         0         0

0         0         0

9999         1         1

____________________

5. Let x be any number. Then:

-x     = ( 10,000 - x)     = ( 9999 + 1 - x ) ;

-(-x)  = ( 10,000 - (-x) ) = ( 9999 + 1 - (-x) )

= ( 9999 + 1 - ( 9999 + 1 - x ) )

= ( 9999 + 1 - 9999 - 1 + x )

= x

6.   (1) -628 = 9372 , (2) -4194 = 5806 , (3) -9983 = 0017,

(4) -1288 = 8712 , (5) -4058 = 5942 , (6) -6952 = 3048

(7) -162 = 9838 , (8) -9 = 9991 , (9) -2744 = 7256,

(10) -5000 = 5000. This last one is a little strange. It

changes 5000 into itself. In our system, 5000 is a negative

number and it winds up as a negative number. This happens on all

computers. If you take the maximum negative number and take its

negative, you get the same number back.

Chapter 0.1 - Numbers and Arithmetic                            v

____________________________________

9998         1         0

9997         1         0

9996         1         0

That worked pretty well. The sign flag changed from 0 to 1 when

we went from 0 to 9999 and the carry flag was set to 1 for that

one operation so we could see that we had gone through the

9999/0000 boundary.

Let's do some more decrementing.

NUMBER    SIGN     CARRY

5003         1         0

5002         1         0

5001         1         0

5000         1         0

4999         0         0

4998         0         0

4997         0         0

4996         0         0

This one didn't work too well. 5000 is our most negative number

(-5000) and 4999 is our most positive number; when we crossed the

4999/5000 boundary, the sign changed but there was nothing to

tell us that the sign had changed. We need to make another flag.

This one is called the overflow flag. We check the carry flag

(CF) for the 0000/9999 boundary and we check the overflow flag

for the 5000/4999 boundary. The last decrementing example with

the overflow flag:

NUMBER    SIGN     CARRY     OVERFLOW

5003         1         0         0

5002         1         0         0

5001         1         0         0

5000         1         0         0

4999         0         0         1

4998         0         0         0

4997         0         0         0

4996         0         0         0

This time we can find out that we have gone through the boundary.

We'll come back to how the computer sets the overflow flag later,

but let's do some addition and subtraction now.

UNSIGNED ADDITION AND SUBTRACTION

Unsigned addition is done the same way as normally. The computer

adds the two numbers. If the result is over 9999, it sets the

carry flag and drops the left digit (i.e. 14625 -> 4625, CF = 1,

19137 -> 9137 CF = 1, 10000 -> 0000 CF = 1). The largest possible

addition is 9999 + 9999 = 19998. This still has a 1 in the left

digit. If the carry flag is set after an addition, the result

must be between 10000 and 19998.

The PC Assembler Tutor                                         vi

______________________

Since this is unsigned addition, we won't worry about the sign

flag or the overflow flag for the moment. Here are some examples

of unsigned addition.

NUMBER 1       NUMBER 2       RESULT         CF

5147           2834          7981           0

6421           8888          5309           1

2910           6544          9454           0

6200           6321          2521           1

Directly after the addition, the computer has complete

information about the number. If the carry flag is set, that

means that there is an extra 10,000, so the result of the second

example is 15309 and the result of the fourth example is 12521.

There is no way to store all that information in 4 digits in

memory so that extra information will be lost if it is not used

immediately.

Subtraction is similar. The machine subtracts, and if the answer

is below 0000, it sets the carry flag, borrows 10000 and adds it

to the result. -3158 -> -3135 + 10000 -> 6842 CF = 1 ; -8197 ->

-8197 + 10000 -> 1803  CF = 1. After a subtraction, if the carry

flag is set, you know the number is 10000 too big. Once again,

the carry flag information must be used immediately or it will be

lost. Here are some examples:

NUMBER 1       NUMBER 2       RESULT         CF

3872           2655          1217           0

9826           5967          3859           0

4561           7143          7418           1

2341           4907          7434           1

If the carry flag is set, the computer borrowed 10000, so example

3 is 7418 - 10000 = -2582 and example 4 is 7434 - 10000 = -2566.

MODULAR ARITHMETIC

What the computer is doing is modular arithmetic. Modular

arithmetic is like a clock. If it is 11 o'clock and you go

forward 1 hour it's now 12 o'clock; if it's 11 and you go

backwards 1 hour it's now 10. If it's 11 and you go forward 4

hours it's not 15, it's 3. If it's 11 and you go backward 15

hours it's not -4, it's 8.

The clock is doing  mod 12  arithmetic.{7}

(A+B) mod 12

(A-B) mod 12

From the clock's viewpoint, 11 o'clock today, 11 o'clock

yesterday and 11 o'clock, June 8, 1754 are all the same thing. If

____________________

7. To be a perfect analogy 12 o'clock should be 0 o'clock.

Chapter 0.1 - Numbers and Arithmetic                          vii

____________________________________

you go forward 200 hours (that's 12X16 + 8) you will have the

same result as going forward 8 hours. If you go backwards 200

hours (that's -(12X16 + 8) = -(12X16) -8) you get the same result

as going backwards 8 hours. If you go forward 4 hours from 11

(11+4) mod 12 = 3 you get the same result as going backwards 8

hours (11-8) mod 12 = 3. In fact, these come in pairs. If A + B =

12, then going forward A hours gives the same result as going

backwards B hours. Forwards 9 = backwards 3; forwards 7 =

backwards 5; forwards 11 = backwards 1.

In the mod 12 system, the following things are equivalent:

(+72 + 4)      (+72 - 8)

(+60 + 4)      (+60 - 8)

(+48 + 4)      (+48 - 8)

(+36 + 4)      (+36 - 8)

(+24 + 4)      (+24 - 8)

(+12 + 4)      (+12 - 8)

(  0 + 4)      (  0 - 8)

(-12 + 4)      (-12 - 8)

(-24 + 4)      (-24 - 8)

(-36 + 4)      (-36 - 8)

(-48 + 4)      (-48 - 8)

(-60 + 4)      (-60 - 8)

They form what is known as an equivalence class mod 12. If you

use any one of them for addition or subtraction, you will get the

same result (mod 12) as with any other one. Here's some

addition:{8}

(+48 + 4) + 7 = (48 + 11) mod 12 = 11

(-48 - 8) + 7 = (48 - 1 ) mod 12 = 11

(  0 - 8) + 7 = ( 0 - 1 ) mod 12 = 11

(-60 + 4) + 7 = (-60 +11) mod 12 = 11

And some subtraction:

(+48 + 4) - 2 = (48 + 2 ) mod 12 = 2

(-48 - 8) - 2 = (48 - 10) mod 12 = 2

(  0 - 8) - 2 = ( 0 - 10) mod 12 = 2

(-60 + 4) - 2 = (-60 + 2) mod 12 = 2

Our pretend computer doesn't cycle every 12 numbers, it cycles

every 10,000 numbers - it is a mod 10,000 machine. On our

machine, the number 6453 has the following equivalence class:

(+30000 + 6453)               (+30000 - 3547)

(+20000 + 6453)               (+20000 - 3547)

(+10000 + 6453)               (+10000 - 3547)

(     0 + 6453)               (     0 - 3547)

(-10000 + 6453)               (-10000 - 3547)

(-20000 + 6453)               (-20000 - 3547)

(-30000 + 6453)               (-30000 - 3547)

____________________

8. (-10) mod 12 = 2 ;   (-11) mod 12 = 1

The PC Assembler Tutor                                       viii

______________________

Any one of these will act the same as any other one. Notice that

10000 - 3547 is the subtraction that we did to get the

representation of -3547 on the machine.

-3547    = 9999 + 1

3547

6452 + 1 = 6453

6453 and -3547 act EXACTLY the same on this machine. What this

means is that there is no difference in adding signed or unsigned

numbers on the machine. The result will be correct if interpreted

as an unsigned number; it will also be correct if interpreted as

a signed number.

6821 + 3179 = 10000  so  -3179 = 6821   and  3179 = -6821

5429 + 4571 = 10000  so  -4571 = 5429   and  4571 = -5429

Since -3179 and 6821 act the same on our machine and since -4571

and 5429 act the same, let's do some addition. Take your time so

you understand why the signed and unsigned numbers are giving the

same results mod 10000:

---------------------------------------------------------

6821 + 497 = 7318

-3179 + 497 = (10000 - 3179) + 497 = 10000 -2682  = -2682

7318 + 2682 = 10000      so    -2682 = 7318

----------------------------------------------------------

5429 + 876 = 6305

-4571 + 876 = (10000 - 4571) + 876 = 10000 - 3695 = -3695

6305 + 3695 = 10000      so    -3695 = 6305

----------------------------------------------------------

Here's some subtraction:

-----------------------------------------------------------

6821 - 507 = 6314

-3179 - 507 = (10000 - 3179) - 507 = 10000 - 3686 = -3686

6314 + 3686 = 10000     so     -3686 = 6314

----------------------------------------------------------

5429 - 178 = 5251

-4571 - 178 = (10000 - 4571) - 178 = 10000 - 4749 = -4749

5251 + 4749 = 10000    so      -4749 = 5251

-----------------------------------------------------------

It is the same addition or subtraction. Interpreted one way it is

Chapter 0.1 - Numbers and Arithmetic                           ix

____________________________________

signed addition or subtraction; interpreted another way it is

unsigned addition or subtraction.

The machine could have one operation for signed addition and

another operation for unsigned addition, but this would be a

waste of computer resources. These operations are exactly the

same. This machine, like all computers, has only one integer

addition operation and one integer subtraction operation. For

each operation, it sets the flags of importance for both signed

and unsigned arithmetic.

For unsigned addition and subtraction, CF, the carry flag tells

whether the 0000/9999 boundary has been crossed.

For signed addition and subtraction, SF, the sign flag tells the

sign of the result and OF, the overflow flag tells whether the

result was too negative or too positive.

SIGN EXTENSION

Although our base 10 machine is set up for 4 digit numbers, it is

possible to use it for numbers of any size by writing the

appropriate software. We'll use 12 digit numbers as an example,

though they could be of any length. The first problem is

converting 4 digit numbers into 12 digit numbers. If the number

is an unsigned number, this is no problem (we'll write the number

in groups of 4 digits to keep it readable):

4816      ->   0000 0000 4816

9842      ->   0000 0000 9842

127      ->   0000 0000 0127

what if it is a signed number? The first thing we need to know

about signed numbers is, what is positive and what is negative?

Once again, for reasons of symmetry, we choose positive to be

0000 0000 0000  to  4999 9999 9999 and negative to be 5000 0000

0000 to 9999 9999 9999.{9}  This longer number system cycles from

9999 9999 9999 to 0000 0000 0000. Therefore, for longer numbers,

0000 0000 0000 = 1 0000 0000 0000. They are equivalent.

0000 0000 0000 = 9999 9999 9999 + 1.

If it is a positive signed number, it is still no problem (recall

that in our 4 digit system, a positive number is between 0000 and

4999, a negative signed number is between 5000 and 9999). Here

are some positive signed numbers and their conversions:

1974      ->   0000 0000 1974

1      ->   0000 0000 0001

3909      ->   0000 0000 3909

____________________

9. Once again, the sign will be decided by the left hand

digit. If it is 0-4 it is a positive number; if it is 5-9 it is a

negative number.

The PC Assembler Tutor                                          x

______________________

If it is a negative number, where did its representation come

from in our 4 digit system? -x -> 9999 + 1 -x = 9999 - x + 1.

This time it won't be 9999 + 1 but 9999 9999 9999 + 1. Let's have

some examples.

4 DIGIT SYSTEM       12 DIGIT SYSTEM

-1964

9999     + 1        9999 9999 9999 + 1

-1964                         -1964

8035   -> 8036      9999 9999 8035 + 1 -> 9999 9999 8036

-2867

9999     + 1        9999 9999 9999 + 1

-2867                         -2867

7132   -> 7133      9999 9999 7132 + 1 -> 9999 9999 7133

-182

9999     + 1        9999 9999 9999 + 1

-182                          -182

9817   -> 9818      9999 9999 9817 + 1 -> 9999 9999 9818

As you can see, all you need to do to sign extend a negative

number is to put 9s to the left.

Can't those 9s on the left become 0s when we add that 1 at the

end?  No. In order for that to happen, the right four digits must

be 9999. But that can only happen if the number to be negated is

0000:

9999 9999 9999 + 1

-0000

9999 9999 9999 + 1 -> 0000 0000 0000

In all other cases, adding 1 does not carry anything out of the

right four digits.

It is impossible to truncate one of these 12 digit numbers to a 4

digit number without making the results unreliable. Here are two

examples:

(number)      0000 0168 7451 ->   7451  (now a negative number)

(actual value)     +168 7451     -2549

(number)      9999 9643 2170 ->   2170  (now a positive number)

(actual value)     -356 7830     +2170

We now have 12 digit numbers. Is it possible to add them and

subtract them? Yes but only 4 digits at a time. When you add with

pencil and paper you carry left from each digit. The computer can

carry left from each group of 4 digits. We'll do the following

addition:

0138 6715 6037

+ 2514 2759 7784

Chapter 0.1 - Numbers and Arithmetic                           xi

____________________________________

Do this with pencil and paper and write down all the carries. The

computer is going to do this in 3 parts:

1) 6037 + 7784

2) 6715 + 2759 + carry (if any)

3) 0138 + 2514 + carry (if any)

The first addition is our regular addition. It will set the carry

flag if the 0000/9999 boundary was crossed (i.e. the result was

larger than 9999). In our case CF = 1 since the result is 13821.

The register holds 3821. We store 3821. Next, we need to add

three things: 6715 + 2759 + CF (=1). There is an instruction like

this on all computers. It adds two numbers plus the value of the

carry flag. Our first addition was ADD (add two numbers). This

time the machine instruction is ADC (add two numbers and the

carry). The result of our second addition is 9475. The register

holds 9475 and CF = 0. We store 9475. Finally, we need to add

three more things: 0138 + 2514 + CF (=0). Once again we use ADC.

The result is 2652, CF = 0. We store the 2652. That is the whole

result:

2652 9475 3821

If CF = 1 at this point, the number has crossed the

9999,9999,9999/0000,0000,0000 boundary. This will work for signed

numbers also. The only difference is that at the very end we

don't check CF, we check OF to see if the

4999,9999,9999/5000,0000,0000 boundary has been crossed.

Just to give you one more example we'll do a subtraction using

the same numbers:

0138 6715 6037

2514 2759 7784

Notice that in order for you to do this with pencil and paper

you'll have to put the larger number on top before you subtract.

With the machine this is unnecessary. Go ahead and do the

subtraction with pencil and paper.

The machine can do this 4 digits at a time, so this is a three

step process:

1) 6037 - 7784

2) 6715 - 2759 - borrow (if any)

3) 0138 - 2514 - borrow (if any)

The first one is a regular subtraction and since the bottom

number is larger, the result is 8253, CF = 1. (Perhaps you are

puzzled because that's not the result that you got. Don't worry,

it all comes out in the wash). Step two subtracts but also

subtracts any borrow (We had a borrow because CF = 1). There is a

special instruction called SBB (subtract with borrow) that does

just that. 6715 - 2759 - 1 = 3955, CF = 0. We store the 3955 and

go on to the third part. This also is SBB, but since we had no

The PC Assembler Tutor                                        xii

______________________

borrow, we have 0138 - 2514 - 0 = 7624, CF = 1. We store 7624.

This is the end result, and since CF = 1, we have crossed the

9999,9999,9999/0000,0000,0000 boundary. This is going to be the

representation of a negative number mod 1,0000,0000,0000. With

pencil and paper, your result was:

-2375 6044 1747

The machine result was:

7624 3955 8253

But CF was 1 at the end, so this represents a negative number.

What number does it represent? Let's take its negative to get a

positive number with the same absolute value:

9999 9999 9999  + 1

7624 3955 8253

2375 6044 1746  + 1  = 2375 6044 1747

This is the same thing you got with pencil and paper. The reason

it looked wierd is that a negative number is always stored as its

modular equivalent. If you want to read a negative number, you

need to take its negative to get a positive number with the same

absolute value.

If we had been working with signed numbers, we wouldn't have

checked CF at the very end, we would have checked OF to see if

the 4999,9999,9999/5000,0000,0000 boundary had been crossed. If

OF = 1 at the end, then the result was either too negative or too

positive.

OVERFLOW

How does the machine decide that overflow has occured? First,

what exactly is overflow and when is it possible for overflow to

occur?

Overflow is when the result of a signed addition or subtraction

is either larger than the largest positive number or more

negative than the most negative number. In the case of the 4

digit machine, larger than +4999 or more negative than -5000.

If one number is negative and the other is positive, it is not

possible for overflow to occur. Take +32 and -4791 as examples.

If we start with the positive number (+32) and add the negative

number (-4791), the result can't possibly be too positive.

Similarly, if we start with the negative number (-4791) and add

the positive number (+32), the result can't be too negative.

Therefore, the result can be neither too positive nor too

negative. Make sure you understand this before going on.

What if both are positive? Then overflow is possible. Here are

some examples:

Chapter 0.1 - Numbers and Arithmetic                         xiii

____________________________________

(+3500) + (+4500) = 8000 = -2000

(+2872) + (+2872) = 5744 = -4256

(+1799) + (+4157) = 5956 = -4044

In each case, two positive numbers give a negative result. How

about two negative numbers?

(7154) + (6000) = 3154 = +3154

(actual value)     -2946    -4000

(5387) + (5826) = 1213 = +1213

(actual value)     -4613    -4174

(8053) + (6191) = 4244 = +4244

(actual value)     -1947    -3809

The numbers underneath are the negative numbers that the numbers

above them represent. In these cases, adding two negative numbers

gives a positive result.

This is what the machine checks for. Before the addition, it

checks the signs of the numbers. If the signs are the same, then

the result must also be the same sign or overflow has

occurred.{10}  Thus + and + must have a + result; - and - must

have a - result. If not, OF (the overflow flag) is set (OF = 1).

Otherwise OF is cleared (OF = 0).

MULTIPLICATION

Unsigned multiplication is easy. The machine simply multiplies

the two numbers. Since the result can be up to 8 digits (the

maximum result is 9999 X 9999 = 9998 0001) the machine uses two

registers to hold the result. We'll call them R1 and R2.

5436 X 174     R1   0094

R2   5864

2641 X 2003    R1   0528

R2   9923

You need to know which register holds which half of the result,

but besides that, everything is straightforward. On this machine

R1 holds the left four digits and R2 holds the right four digits.

Notice that our machine has changed the modular base from N to

N*N (from 1 0000 to 1 0000 0000). What this means is that two

things which are modularly equivalent under addition and

subtraction are not necessarily equivalent under multiplication

and division.  6281 and -3719 will not work the same.

____________________

10. The machine checks something considerably more obscure

because it is easier to implement in semiconductor logic, but

what it is actually doing is checking to see if the two numbers

being added have the same sign. If they do, the result must be

the same sign or overflow has occurred.

The PC Assembler Tutor                                        xiv

______________________

The machine can't do signed multiplication. What it actually does

is convert the numbers to positive numbers (if necessary),

perform unsigned multiplication, and then do sign adjustment of

the results (if necessary). It uses 2 registers for the result.

SIGNED MULTIPLICATION      REGS         RESULT

(number)           (5372) X (3195)     R1   8521  =  -1478 6460

(actual value)     -4628  X +3195      R2   3540

(number)           (9164) X (8746)     R1   0104  =   +104 8344

(actual value)      -836  X -1254      R2   8344

(number)           (9927) X (0013)     R1   9999  =        -949

(actual value)      -73  X   +13       R2   9051

Looking at the last example, if we performed unsigned

multiplication on those two numbers, we would have

9927 X 0013 = 0012 9051, a completely different answer from the

one we got. Therefore, whenever you do multiplication, you have

to tell the machine whether you want unsigned or signed

multiplication.

DIVISION

Unsigned division is easy too. The machine divides one number by

the other, puts the quotient in one register and the remainder in

another. Once again, the only problem is remembering which

register has the quotient and which register has the remainder.

For us, the quotient is R1 and the remainder is R2.

6190 / 372          R1   0016           16  remainder 238

R2   0238

9845 / 11           R1   0895           895 remainder 0

R2   0000

As with multiplication, signed division is handled by the machine

changing all numbers to positive numbers, performing unsigned

division, then putting back the appropriate signs.

SIGNED DIVISION         REGS            RESULT

(number)      (7192) / (9164)     R1   0003      +3  rem. -300

(actual value)-2808  /  -836      R2   9700

(number)      (3753) / (9115)     R1   9996      -4  rem. +213

(actual value)+3753  /  -885      R2   0213

Looking at the last example, 3753 / 9115, if that were unsigned

multiplication the answer would be 0 remainder 3753, a completely

different answer from the signed division. Every time you do a

division, you have to state whether you want unsigned or signed

division.