A UTILITY FOR LOADING TSR'S SOMEPLACE ELSE

HILOAD is a utility program designed to load your normal TSR

(Terminate-and-Stay-Resident) programs somewhere else in memory.

Usually this is a place outside the normal DOS region; that is, outside

the first 640K of system memory.

At this time, it does not include either extended or expanded

memory.

BACKGROUND

The 8088 used in the PC-XT is capable of addressing up to 1024K

of memory.  The designers of the PC and DOS decided that only the first

640K would be used by the operating system, DOS, and the rest was reserved

for other uses.

Because of the way the 8088 forms addresses, it is customary to

refer to addresses by a segment:offset combination. The 'actual' final

address is formed by multiplying the segment value by 16 (i.e., shifting

it left 4 places) and adding the result to the offset address.

Thus, the very first address in the machine is called 0000:0000.

An address 1024 bytes from this start (hex 400) may be variously

indentified as 0000:0400, 0040:0000, 0020:0200, 0038:0080, etc.

For convenience, we may speak of the various segments as:

0000:0000       to      0000:FFFF       Block 0

1000:0000       to      1000:FFFF       Block 1

and so on

E000:0000       to      E000:FFFF       Block E

F000:0000       to      F000:FFFF       Block F

It is easy to see that there are 16 contiguous segments of 64K

each, yielding (16 X 64 =) 1024K, or one megabyte, and that DOS can

only use the first 10 segments, or blocks 0 through 9.

What is in the other 6 segments?  Is that 384K of potential

memory just sitting there idle?

WHO'S USING WHAT?

Well, actually not. Some of these blocks have been set aside,

and are probably being used, for specific purposes.

Block F is where the ROM BIOS lives and, if you have a true-

blue IBM PC, where the ROM BASIC lives.  So the F block is pretty well

spoken for.

Block B is where the memory for the video sits.  If you have

a Monochrome Display Adapter (MDA), it uses 4K of memory at addresses

B000:0000 to B000:1000, and the remainder of the B block, 60K, is

unused. In fact, if you have a straight monochrome card like IBM

supplies, that's all the memory you have available.

If you have a Color Graphics Adapter (CGA), it occupies

memory from B800:0000 to B800:7FFF (or equivalently B000:8000 to B000:FFFF).

Like the MDA, when the CGA is in text mode it only uses the first 4K of

this memory, but it can support up to 8 'pages' of 4K each; when in

graphics mode, all 32K are in use. Like the MDA, true-blue CGA cards, and

most clone CGA cards, have only that 32K available.

Enhanced Graphics Adapters (EGA) are even bigger consumers of

memory, at maximum using the A block and part of the C block!

If you have a hard disk, its BIOS is normally located on the

hard disk controller card, and is ususlly resident at C000:8000, or

as it is commonly referred to, C800:0000.

SO WHAT?

Astute observers will notice that the D and E blocks were pretty

much left out of the above description.  It is believed that these areas

of memory were set aside by IBM for program cartridges, but few, if any,

of these were actually produced, and this area is normally wide open and

available for use.

If you are like most PC users, you have only a monochrome or CGA

display, and the A block is available to you also.

DOWN TO BUSINESS

The '640K barrier' people talk about is caused by two reasons:

programs are getting more sophisticated and user-friendly, meaning they

take up more memory.  Secondly, there has been a proliferation of many

handy programs that reside in memory (TSR's) which 'pop up' with useful

information at the touch of a key or perform some other action.  Examples

are SideKick from Borland International, various printer buffers which are

almost a necessity, and so on.  It is not too unusual to have half one's

DOS memory taken up with various TSR's.

While there's little we can do about fat programs, we can move

these TSR's out of the way. This was the driving force behind the

development of HILOAD.

WHERE DO I GET THIS MEMORY?

Some PC clone makers map their memory into the D and E blocks;

Kaypro for one does, others may also. Constructing a board to fill in

these holes is not difficult, especially if static RAMS (SRAM) are used;

these are now available in 32K X 8 sizes at attractive prices.

An old trick is to use commercial memory cards (now obsolete

but available) that allow themselves to be mapped into the desired

regions.

Finally, users of Hercules Graphics cards and their clones usually

have the full 64K in the B block available, but only 4K is used in text

mode.  That 60K looks mighty attractive, as does the 'spare' memory on

a CGA card...BUT...because one may easily forget there are programs in

that space, switching to Herc graphics, or simulating a CGA, or using

a CGA in graphics mode will really screw up the works, so this technique

is not recommended.  It may be used to test out the program, though, and

see if it's worth the small trouble to get memory for the 'safe' areas.

HOW THE PROGRAM WORKS

HILOAD grabs the program you want, as well as any command tail

required, and loads it into the memory area you specify. After loading

the candidate TSR program, it revectors several DOS interrupts and then

executes the TSR program in the new location. When the TSR program

attempts to 'go resident', HILOAD traps that request, either from INT 27H

or function 31H of INT 21H, resets the appropriate interrupt vectors,

examines the interrupt vector table to see which ones point to the new

program, and then terminates normally.

Thus, when HILOAD exits, zero DOS memory is used up, and the

TSR is snugly nestled in another part of memory, acting just like it

normally does.

HOW TO SPECIFY VARIOUS THINGS

HILOAD requires various pieces of information to operate properly.

Addresses - HILOAD requires a starting address for placing TSR's.

After it places the first one, it keeps track of where

it should place the next one by using the Intra-application

Communication Area (ICA) at 0040:00F0 through 0040:00FF.

It checks this area and computes an address from the

information there, so the initial starting address has

to be specified only once (until you change blocks).

As long as the loading area is contiguous, the starting

address need not be re-specified.

The starting address is specified by an environment

variable, which is discussed below.

Program names - The program name to be loaded must be specified

with the full path name.  Sorry about this, but it

wasn't worth the effort to program the logic to chase

down the paths to find the programs.  This is not

considered (by me, anyhow) to be a serious objection,

as the typical use of this program is in your AUTOEXEC

file where it is only typed in once.

Starting off - Executing HILOAD without any program specified

causes it to zero out the ICA.  This is the easiest

way to begin a loading sequence after changing the

start address.

SPECIFYING A START ADDRESS (ENVIRONMENT VARIABLES)

When DOS loads a program, it also provides that program with a

copy of what it calls the Environment.  To see what the contents of

your environment are, do the following:

At the DOS prompt (e.g., C:\>), type SET <CR>

The response will be a list of things, starting with COMSPEC and continuing

with your PATH and PROMPT values (if you have any).  After these values, you

may or may not have additional entries.  If you do, they will be of

the form:

name=value

To add a value to the environment, type:

set abc=ABCabc <CR>

and then type

set <CR>

Notice that a new environment variable has been added.  Notice also

that the name, abc, has been capitalized as ABC, but the value remains as

it was typed in, i.e., ABCabc.

To change an environment value, type

set name=NewValue<CR>

and NewValue will replace the old value.

To delete an environment variable, type

set name=<CR>

and the value will disappear from the environment.

HILOAD requires an environment variable with a name the same as the

program name.  The value is the hex starting address.

So, if the program is named HILOAD, as it is on this disk, and

the desired starting value is A000, then you would set the environment

variable by typing:

set hiload=A000<CR>

This can also be done as an entry in your AUTOEXEC.BAT file.

It is poorly documented, but DOS provides the name of the program

it loads, and where it came from, right after the environment.  I have

used this quirk of DOS to determine the name of the environment variable.

If you change the program name to, say, SHIFTSR.COM, then you would set

the environment variable as:

set shiftsr=A000<CR>

This feature (?) allows you to have multiple copies of the program

under different names, each with its own environment variable, if you should

so desire.

NOTE: As far as I can tell, this feature is only available in DOS 3.X.  The

program determines the DOS version in use and, if less than 3.0, it uses a

default program name of HILOAD.  This is what the name of the environment

variable would have to be, no matter what you rename the program to.

INITIALIZING THE PROCESS

As stated above, the program must first initialize the ICA. It

does this by being invoked with no parameters.  The 'standard' starting

sequence is therefore:

SET HILOAD=A000

HILOAD

etc.

SPECIFYING PROGRAMS TO LOAD

Also as stated above, full path names must be given.  Suppose you

have four TSR's you wish to load, as follows:

NAME            SIZE            PARAMETERS (TSR command tail)

TSR1             14K                 none

TSR2             22K                 /X /U /M=22

TSR3             1.4K                none

TSR4             26K                 'text string'

Adding them up, we see that they will occupy less than 64K, so

they can all be loaded in one block. Suppose also we have the A block

available for use.  The series of commands in the AUTOEXEC file to

perform the desired action might be:

SET HILOAD=A000                         starting address

HILOAD                                  initialize

HILOAD \DOS\TSR1                        load first tsr

HILOAD \SUB1\SUB2\TSR2 /X /U /M=22      load next tsr

HILOAD \TSR3                            load next tsr

HILOAD \UTILITY\TSR4 'text string'      load last tsr

It is assumed the directories shown are the complete path names

to the various TSR's.

Suppose now you have another TSR you want loaded, but it's too

big to fit in the A block; you have memory available in D block.

You would follow the above sequence with:

SET HILOAD=D000                         reset start address

HILOAD                                  re-initialize

HILOAD \SUB3\SUB4\TSR5                  load it

PROGRAM OUTPUT

HILOAD issues various progress and error messages as it chugs along.

It tells you the starting address it uses, the name of the program it is

loading, what it found for a command tail to the TSR, and the amount of

memory the TSR has reserved for itself.

It also lists the interrupts the TSR has captured, along with

their new addresses.  This can be handy for debugging purposes or

just finding out who captures what. Many TSRs do not go through Int 21H

functions 35H and 25H to get and set the interrupt vectors; they directly

manipulate the DOS interrupt table at the bottom of memory.  This is not

a recommended technique but many do so. HILOAD will capture these villians

(SideKick being the most prominent one) and show what they trap.

Incidentally, it appears that SideKick traps interrupt 21H for

itself, and any TSR loaded after it, who attempts to grab Int 9H, is

pushed down the list by SideKick so he (SideKick) still has first grab

at the keyboard. Very clever programming; a pain in the kazoo when you're

groping around to figure out what's actually happening!

The above comments are based on an older version of SideKick (1.56)

which I have been using. Later versions may act in a more civilized manner.

KEEPING THE OUTPUT

Since the information from HILOAD can whiz off the screen before

you have a chance to read it, it is a good idea to redirect the output to

a file instead of the screen.  The program output can then be perused at

your leisure. The following sequence is an example of how to do this:

DEL TSRLIST.LST                           delete old file

SET HILOAD=A000                           set start address

HILOAD >>TSRLST.LST                       initialize

HILOAD \TSR1 >>TSRLST.LST                 load first tsr

HILOAD \SUB1\TSR2 /X /Z >>TSRLST.LST      load next tsr

SET HILOAD=D000                           reset start address

HILOAD >>TSRLST.LST                       initialize

HILOAD \UTILITY\BIGTSR /PARMS >>TSRLST.LST  load big tsr

After everything is loaded, the file TSRLST.LST may be

browsed to see what went on, get addresses, check vectors, etc.

Of course, the output may also be redirected to the printer

by putting a >PRN instead of >>filename.ext

ERROR MESSAGES

If something doesn't go right, the program tries to tell you

and exit safely.  The error messages are:

No address given ... ABORTING          Something's wrong with the

environment variable. Check

spelling of variable name and

make sure it agrees with the

program name.

The value given must be 4 hex

digits, upper or lower case.

Can't find file PROGNAME ... ABORTING   HILOAD was unable to locate

the specified program. Check to

see that the path is correct.

Also, only .COM files can be

loaded this way; .EXE files

will bomb!

Requires DOS 2.0 ... ABORTING           Program requires DOS version

2.0 or above.

The program also issues a message when it initializes the ICA.

DISCLAIMER

TSRs are tricky things; SideKick is notorious for being sensitive

to the order in which other TSR's are loaded. Experiment with loading

your TSRs in the 'normal' fashion until you get a loading sequence that

works. Use that order to HILOAD them and all should be well.

LEGAL STUFF

This program is being placed in the public domain.  It may be copied

and distributed freely for non-commercial uses.  Both this file and the

executable code must be distributed together and unmodified.

Contents of both files are copyright 1989 by the author.

Any brand names mentioned are copyright by their respective owners.

SHEEPISH ADMISSION

This program was developed over a period of time as I was learning

about TSRs and revectoring interrupts, etc. Also, though the objective of

the program remained firm, the little niggly details kept changing on me

as I thought of alternate ways to accomplish things and more frills were

added.

Consequently, the source code is comprehensible to me but I'm sure

looks like a real mess to the 'seasoned professional'.  I am thus a little

ashamed to release it for public scrutiny and scorn.

However, it is included and commented as best as I can recall what

each little piece does.  Don't bother to tell me there are 'dead code' areas

and unused variables.

Anybody who wants can modify it to their hearts' content, but please

take note of the distributon restriction noted above.

Larry Shannon

5615 Truscott Terrace

Lakeview, NY  14085

April l989