RAMCLEAR

--------------------

by  Dennis L. Dias

Here's a short utility for anyone who uses a compiled language.

When a program is compiled, code and data are separated according

to methods determined by the individual compiler. Code usually

occupies a contiguous memory space. Individual routines cannot

have un-initialized memory within them because that would lead

to unpredictable results. Data, on the other hand, does quite often

contain un-initialized blocks of memory. Successive compiles of the

same source code will generate program files that are not identicle

because the contents of those locations within the disk file will

be whatever was in memory at compile time. Usually, even though the

files are not identicle they will be functionally equivalent and

error free. Usually.

Some Clipper users have reported bad compiles. Source code that

compiles correctly most of the time will, for no apparent reason,

produce garbage. The most obvious effect is a screen being painted

completely wrong. The actual error appears in storage reserved for

numeric constants.

Although this is relatively rare, I have experienced it myself. The

normal solution is to compile the same source code again, but on one

occasion I encountered a source file that would only compile correctly

the first time. Each successive compile would be filled with this kind

of error and the only solution was a cold boot to clear memory. I

didn't like that at all and proceeded to write a short utility that

would clear all user memory to zero. For the first time since I

purchased Clipper I was able to generate program files that are

absolutely identicle and the utility that I wrote now appears as

the first command in many of my batch files.

The program itself is short and simple. What may not be obvious

to many programmers is how to create a .COM file (or why).

An .EXE file contains a header structure that describes all the

relocation information needed to run the same program at different

absolute addresses in memory. Each time the program is loaded

adjustments must be made before the program is given control.

A .COM file does not have any such header structure. Therefore,

program files take less space on disk and load time is reduced.

This is possible because: (1) .COM files are restricted to one

segment only and (2) the first executable instruction is always

at offset 100H. The first 256 bytes (100H) are reserved for the

program segment prefix (PSP).

The first condition is met by declaring only one segment (called

CODE in this program). Notice that even though we are limited to

one segment we are easily able to access all of memory.

The second condition is met by deliberately setting our program

entry point to 100H (ORG 100H and END START). Here is the basic

format:

CODE        SEGMENT PARA              ;one segment

ASSUME  CS:CODE,DS:CODE

ORG     100H              ;first instruction at 100H

START:      JMP     BEGIN             ;jump over data area

DB      "Optional data area"

BEGIN:      .

.

.                         ;program code

.

.

CODE        ENDS                      ;end of segment

END     START             ;indicate program entry point

Data can go anywhere as long as the program branches around it, but

it is best to declare data before it is accessed. In the program

presented here there is no data.

Programs formatted this way will assemble correctly without error.

However, when linking the object file, a warning message such as

Warning:  No STACK segment

will appear. This message is only a warning and must be ignored.

The linker will generate an .EXE file which cannot be run as is. It

must first be converted to a .COM file with the DOS utility EXE2BIN.

This will remove the header structure from the file. Here are the

steps to follow:

C>MASM  RAMCLEAR;

C>LINK  RAMCLEAR;

C>EXE2BIN  RAMCLEAR.EXE  RAMCLEAR.COM

C>ERASE  RAMCLEAR.EXE

To run the program enter:

C>RAMCLEAR

I should mention that the source file that would not compile has

been given to Nantucket development and that that particular bug

has since been fixed for the next version of Clipper. In the

meantime, I think that this utility will help.

;------------------------------------------------------:

;   (C) Copyright 1986 Dennis L. Dias                  :

;        All Rights Reserved.                          :

;                                                      :

;   Filename....:  RAMCLEAR.ASM                        :

;   Author......:  Dennis L. Dias                      :

;   Source ID...:  NAN449                              :

;   CompuServe..:  73765,302                           :

;   Date........:  06/22/86                            :

;                                                      :

;   Purpose.....:  Fill RAM with zeros from the lowest :

;                  user segment to the top of memory.  :

;                                                      :

;   Note........:  Convert to .COM file.               :

;------------------------------------------------------:

CODE        SEGMENT PARA

ASSUME  CS:CODE,DS:CODE

ORG     100H              ;.COM file format

;     the amount of installed memory is stored at offset 02H

START:      MOV     DX,CS             ;reference lowest user seg

MOV     BX,02H            ;get top of memory from

MOV     AX,[BX]           ;   PSP and place in BX

MOV     BX,AX             ;   for segment counter

XOR     AX,AX             ;zero used for fill

CLD                       ;forward direction

;     all but the last segment is cleared with this loop

CLEAR_LOOP: SUB     BX,1000H          ;next lower 64k seg

CMP     BX,DX             ;lower than lowest?

JBE     LAST_SEG

MOV     ES,BX             ;load seg reg

XOR     DI,DI             ;initial offset

MOV     CX,8000H          ;word count for 64k

REP     STOSW             ;fill with zeros

JMP     CLEAR_LOOP        ;next segment

;     the last segment contains fewer than 64 kilobytes

LAST_SEG:   PUSH    CS                ;set ES to local seg

POP     ES

LEA     DI,RAM_START      ;first byte after program

AND     DX,0FFFH          ;clear high nibble

MOV     CL,4              ;shift count

SHL     DX,CL             ;convert to offset value

;     DX now contains the offset from the next lower 64k boundary

ADD     DX,DI             ;plus offset of end of program

XOR     CX,CX             ;set CX = # of bytes above

SUB     CX,DX             ;   prog not yet cleared

REP     STOSB             ;fill what's left to clear

INT     20H               ;terminate program

RAM_START   LABEL   BYTE              ;first byte after program

CODE        ENDS                      ;end of segment

END     START             ;indicate program entry point

#

#######