��������������������������������������������������������������������������ͻ

�               HTimer Hi-Resolution Microsecond Timing Class              �

�                       for use with Borland C++ 3.x                       �

�                                                                          �

�                     Version 1.2 - November 16,  1992                     �

��������������������������������������������������������������������������Ķ

�                          Betz Associates, Inc.                           �

�                          120 North Shore Road                            �

�                           Derry, N.H.  03038                             �

�                              603-898-8806                                �

�                         email @ CIS: 76605,2346                          �

�                                                                          �

�       Contract Development for the Entertainment Software Industry       �

��������������������������������������������������������������������������ͼ

Contents:     Topic                                     Section

����������������������������������������������������������������������������

Description and Definition -------------- A

Class Description -------------------- A1

Problem Definition ------------------- A2

Class Specification ------------------ A3

Adherence to Specification ----------- A4

Class Operation ------------------------- B

Hardware Interface ------------------- B1

Description of 8253/8254 timers --- B1a

Hardware Initialization ----------- B1b

Programming Considerations -------- B1c

Application Programming Interface ---- B2

Timer Restrictions ------------------- B3

Changes in this Version ----------------- C

File List ------------------------------- D

Copyrights and Disclaimers -------------- E

References ------------------------------ F

About the Programs ---------------------- G

TMRTEST ------------------------------ G1

TMREXAM ------------------------------ G2

����������������������������������������������������������������������������

A. Class Description, Problem Definition, and Class Specification

-----------------------------------------------------------------

A1. Description

---------------

The HTimer class is a single class which provides a hi-resolution event

timer. Up to 255 timers can be instanced and run concurrently with no

additional overhead. The HTimer class handles hardware interface actions

transparently in almost all applications, and provides a maximum timing

resolution of +/- 1 microsecond at an approximate accuracy of +/- 0.05

percent of "true" time. The class can be used in any of the memory

models supported by Borland C++ 3.1. The class has not been tested in

any processor mode other than real.

A2. Problem Definition

----------------------

Often during the development of high-performance or realtime software it

is necessary to know the duration of events with a high degree of accu-

racy. The PC/AT/80x86 architecture provides several levels of hardware

and software driven event timing, but the PC-BIOS and MS-DOS provide

system support for low-resolution timing only. BIOS subfunctions of

function 1Ah can read and write the real-time clock, but the maximum

resolution obtainable is +/- 1 second. By reading the 32-bit time counter

maintained by the BIOS at memory location 0040h:006Ch a program can

obtain a "time tick" once every 54.94 milliseconds (approx.). Higher

resolution is available with DOS function 21h, subfunction 2Ch, which can

provide timing down to 1/100 second, but still this is too slow for many

purposes, and the overhead of the call to DOS is high. Fortunately the

Intel 8253/8254 timer chip used in the PC/AT provides access to a clock

pulse running at 1.193180 Mhz, through which we can obtain a timing

resolution of 1 microsecond or less. In order to take advantage of this

hardware capability a program must provide it's own driver code for the

timer chip.

A3. Specification

-----------------

The HTimer class is an attempt to encapsulate the necessary functions

for hi-resolution timing on the PC. During it's design the following

specification was used as a baseline goal for the class:

- minimum timing resolution of 1 microsecond

- maximum deviation +/- 0.05 percent of true time

- transparent encapsulation of hardware interface activity

- support for multiple concurrently running timers

- class extensibility

- useable in large, medium, and small model programs

- low code space requirements/simplicity

A4. Adherence to Specification

------------------------------

This class fulfills these requirements in most cases. The following are

areas in which the class design does not fully satisfy the specification:

- When running the included testbed program the timer exhibits a

maximum deviation of +/- 0.04 percent. This is just within the

specified range. To determine accuracy the test program times

a single tick of the bios system time counter. In theory this

event should require 1,000,000/18.2 second, 54.94 milliseconds,

or 54945.05 microseconds. Since the system time tick frequency

can vary by more than .05 percent without affecting the operation

of the system it is impossible to guarantee this as a standard

against which to test.

- Encapsulation of the hardware interface activity is to some extent

transparent to the user. However, if the using program intends to

alter the timer frequency or the handler for IRQ 0 (interrupt 8),

then it must follow certain conventions described below in order

to maintain correct operation of the timer class.

B. Class Operation

------------------

B1. Hardware Interface

----------------------

B1a. Description of the Intel 8254 Programmable Timer

-----------------------------------------------------

The Intel 8253/8254 Programmable Timers provide software controlled

clock pulses to the PC/AT system board. The package consists of three

programmable timer elements, each of which is made up of a 16-bit

Counter Element (CE), latches, and various control elements. Timer 0

provides the system time-of-day clock pulse, timer 1 is programmed by

the BIOS to request RAM refresh, and timer 2 is hard-wired for control

of the internal speaker.

All three of the internal timers are programmable Countdown Timers.

Each timer is provided with a 1.193180 Mhz input pulse from SYSCLK.

Depending on the mode of operation the timer decrements it's internal

counter on each pulse. The Initial Count value loaded into the CE at

the start of a timing period is programmed into the timer, and held

in it's internal latches for reuse. In the case of timer 0 the default

mode of operation is mode 3 (Square Wave Generator). In this mode timer

0 operation is as follows: CLKOUT starts high. On each pulse of the

input signal the CE is decremented by 2. At terminal count (0) CLKOUT

is lowered and the Initial Count is reloaded into CE, and is then dec-

remented by 2 until terminal count is reached again. At this point,

CLKOUT is raised to generate the time-of-day interrupt on IRQ level 0,

the Initial Count is reloaded into the CE, and the procedure repeats

indefinitely.

Mode 2 (Pulse Rate Generator) is a preferable mode for hi-resolution

timing, since in this mode the counter is decremented by 1 on each

pulse of the input signal, and there is no need to divide by two to

get a raw elapsed counter value.

For details on programming the 8253/8254 Programmable Timers please

consult the references listed at the end of this document.

B1b. HTimer Hardware Initialization

-----------------------------------

In order to perform accurate, long-duration timing the HTimer class

must at some point perform some reprogramming of the 8253/8254 timer

chip. The first requirement is that timer channel 0 be programmed to

operate in the more accurate Mode 2. Secondly, in order to exceed the

16-bit capacity of the counting element and provide for long-duration

timing it is necessary to hook into the IRQ 0 interrupt vector and

track the number of interrupts generated. Finally, on shutdown the

timer must be reprogrammed to it's defaults, and the original IRQ 0

interrupt vector must be restored.

The implementation of these housekeeping chores in HTimer is nearly

transparent to the using program. Exceptions to this will be discussed

below. Whenever an instance of an HTimer is created it checks a static

member variable of the class to see if it is the first instance. If

so, it performs the reprogramming of the timer, and sets up the ISR

for the timer interrupt. When the instance is destroyed it again checks

the static member variable, this time to see if it is the last instance.

If so, it reprograms the timer chip to it's defaults, and restores the

interrupt vector.

B1c. Considerations for Programs Using the Timer Hardware

---------------------------------------------------------

The exception to the above statement involves programs which are using

the timer for purposes other than running the HTimer class. If the

using program needs to alter the frequency of the timer interrupt it

MUST DO SO USING THE SUPPLIED MEMBER FUNCTION HTimer::setCount(). If

this convention is not followed the HTimer class will produce incorrect

results. This is due to the fact that the class _must_ know the number

of pulses per interrupt in order to calculate elapsed time. Similarly,

if the using program requires the timer interrupt vector for it's

own use it MUST HOOK THE VECTOR BEFORE INSTANCING AN HTimer OBJECT. The

HTimer class maintains it's own short handler for IRQ 0 (int 8) in

order to count raw interrupt frequency. To ensure accurate operation

this handler must be the FIRST HANDLER IN THE CHAIN FOR IRQ 0.

B2. Application Programming Interface

-------------------------------------

Access to the functions of the HTimer class is through the defined member

function interface. The following functions are available to application

programs:

void HTimer::timerOn()

The timerOn() function starts a new timing process. This function

must be called at least once after instancing an HTimer object.

Parameters: None

Returns:    Nothing

dword HTimer::timerOff()

The timerOff() function stops the current timing process, resets

the internal timer parameters, and returns the elapsed time count.

Parameters: None

Returns:    32-bit elapsed time count in microseconds

dword HTimer::getElapsed()

The getElapsed() function calculates and returns the current elapsed

time without disrupting the current timing process. After the func-

tion returns the timer continues to run, the internal count is not

reset, and the next call to either getElapsed() or timerOff() will

return the TOTAL accumulated microsecond count for the run, inc-

luding totals returned by earlier calls to getElasped().

Parameters: None

Returns:    32-bit elapsed time count in microseconds

void HTimer::setCount(word regCount)

The setCount() function is provided in order to allow applications

programs to alter the Counter Element Initial Count, and therefore

the frequency of IRQ 0 interrupts. Access to the CE MUST be made

through this member function, or the HTimer class will not provide

accurate timing results.

Parameters: regCount, 16-bit Initial Count value for timer 0 CE

Returns:    Nothing

B3. Timer Restrictions

----------------------

In addition to the hardware restrictions discussed above, the HTimer

class is also restricted in terms of the maximum duration of a timing

run, based on the frequency of the timer interrupts generated on IRQ 0.

The minium value is always 0, and the maximum can be calculate with the

following formula:

maxT = (2^32 * (1,000,000/f))

Where maxT is the maximum elapsed time in microseconds, and f is the

frequency of the IRQ 0 interrupt in hz. In the case of the standard

interrupt frequency of 18.2 hz the timer can accumlate up to 2.358 E14

microseconds. In the case of the maximum theoretical interrupt rate of

1,193,180 hz. the timer is capable of accumulating 3,599,597 microsecs

for a maximum timing duration of only 3.59 seconds. However, since the

PC architecture will not support this theoretical maximum interrupt rate

the effective minimum duration is much higher.

Note that, in order to avoid overflowing the 32-bit elapsed time accum-

ulators, it is necessary to use a different method of microsecond

conversion for raw pulse counts over 4,294,967. This involves basically

reversing the order of division and multiplication, and results in a

small loss of precision for timing runs over 4 seconds in duration. See

the implementation of HTimer::calcElapsed() in HTIMER.CPP for details.

C. Changes in this Version

--------------------------

Please see the file HTIMER.HST for version notes

Problems corrected in, and changes made to version 1.1

- Included TYPES.H file missing from version 1.0 archive.

- Added a function to poll the timer for the elapsed time without dis-

rupting the current timing run.

- Implemented an interrupt handler to collect raw interrupt counts. The

previous version could be confused if the interrupt rate was changed

from the default.

- Added code to handle wrap in the interrupt counter.

- Added code to allow changing the interrupt rate by altering the counter

value for timer channel 0.

- Removed the elapsed time calculation to a seperate function.

- Tested compilation in all memory models.

- Created a testbed program.

- Created an example program.

- Added documentation.

- Fixed bug in elapsed time calculation.

Version 1.2 is a complete replacement for version 1.1. You have version

1.1 if the archive is dated earlier than Nov. 16, 1992. Problems corrected

in, and changes made to, this version are listed below:

- Corrected subtle bug in timer start count latching operation which

caused the HTimer::calcElapsed() member function to operate incorrectly

when timing began immediately after an interrupt (i.e., the latched

start value was very high, near 65535).

- Corrected improper use of tick counter in HTimer::calcElasped()

D. List of Files in Archive HTIMER.ZIP

--------------------------------------

HTIMER.H      - Header file for HTimer class definition

HTIMER.CPP    - Function definitions for HTimer class

HTIMER.OBJ    - HTimer large model object module

HTIMER.DOC    - HTimer documentation

HTIMER.HST    _ HTimer version notes/history list

TMRTEST.CPP   - HTimer testbed program

TMRTEST.PRJ   - Borland C++ 3.1 project file for test program

TMRTEST.EXE   - HTimer testbed program executable

TYPES.H       - Header file containing generic types

TMREXAM.CPP   - HTimer example program

TMREXAM.PRJ   - Borland C++ 3.1 project file for example program

TMREXAM.EXE   - HTimer example program executable

E. Copyrights and Disclaimer

----------------------------

The HTimer class, source code, and documentation is released to the

Public Domain. This source code and associated object modules are offered

without warranty or guarantee of any kind, express or implied. Also,

the author(s) and Betz Associates, Inc. expressly disclaim any resulting

consequences of the use of this source code and related object modules

in any product, public domain, shareware, or commercial.

USE OF THIS PRODUCT IS UNDERTAKEN SOLELY AT THE RISK OF THE USER.

F. References

-------------

Software engineers desiring to learn more about the Intel 8253/8254

programmable timers, and their implementation in the IBM PC/AT comp-

atible architecture, are directed to the following sources:

Intel Peripheral Components Handbook 1991

ISBN# 1-55512-127-6

Intel order# 296467-002

IBM Technical Reference, Personal Computer AT

IBM Personal Computer Hardware Reference Library# 6280070

The New Peter Norton Programmer's Guide to the IBM PC & PS/2

Microsoft Press, by Richard Wilton

ISBN# 1-55615-131-4

PC System Programming, Abacus, by Michael Tischer

ISBN# 1-55755-035-2

G. About the Programs

---------------------

G1. TMRTEST

-----------

TMRTEST is a simple test program for the HTimer class. The program

performs three tests in sequence, and reports the results to stdio.

All three tests time the duration of one bios time tick. On the PC

and AT this interval should be very close to 54,945 microseconds.

The first test times the interval by starting and stopping the timer.

The second test starts the timer, then polls the timer for the current

elapsed time, leaving the timer running. The third test checks the

ability of the timer class to handle a faster interrupt rate. Before

calling this test function the test program hooks the vector for

interrupt 8 to a short handler which ensures that the _system_ handler

for this interrupt continues to be called at a frequency of 18.2 hz.

It then calls the test function, which cranks the timer interrupt up

to 76.2 hz, and times the duration of a bios time tick.

G2. TMREXAM

-----------

TMREXAM is a simple example program demonstrating how to instance

three timers, time several actions, and report the results.